3GPP TSG-RAN5 Meeting #90-e	R5-210289r1

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0143 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of common implementation conformance statements for R16 NR SON and MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE, MediaTek Inc, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of common implementation conformance statements for new test cases for NR SON and MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Common implementation conformance statements for new test cases for NR SON and MDT are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR SON and MDT are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: File name update |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

A.4.4	Additional information

Table A.4.4-1: Additional information

| Item | Additional information | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of ICMP or ICMP IPv6 | RFC 792 OR RFC 4443, RFC 4884 | NA | pc_IP_Ping | UE supports ICMP or ICMPv6 protocol to enable IP Ping Operation |
| 2 | Support of IMS | 24.229, Annex U | Rel-15 | pc_IMS_5GS |  |
| xx | Support of rachReport | 38.306, 4.2.17 | Rel-16 | pc_rachReport | UE supports delivery of rachReport upon request from the network. |
| xx1 | Support of GNSS | 38.306, 4.2.18 | Rel-16 | pc_GNSS_location | UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON or MDT related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE. |
| xx2 | Support of UL PDCP Packet Average Delay | 38.306, 4.2.18 | Rel-16 | pc_PDCP_Delay | UE supports UL PDCP Packet Average Delay measurement and reporting in RRC_CONNECTED state |
| xx3 | Supprt logged MDT | 38.306, 4.2.18 | Rel-16 | pc_Logged_MDT | UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. |
| xx4 | Support of uncompensated barometeric pressure measurement reporting | 38.306, 4.2.18 | Rel-16 | pc_barometer | UE supports uncompensated barometeric pressure measurement reporting upon request from the network. |
| xx5 | Support of orientation information reporting | 38.306, 4.2.18 | Rel-16 | pc_orientation | UE supports orientation information reporting upon request from the network. |
| xx6 | Support of speed information reporting | 38.306, 4.2.18 | Rel-16 | pc_speed | UE supports speed information reporting upon request from the network. |
| xx7 |  | Support of Bluetooth measurements in RRC_CONNECTED state |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_immMeasBT |  | UE supports Bluetooth measurements in RRC_CONNECTED state. |  |
| xx8 |  | Support of WLAN measurements in RRC_CONNECTED state |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_immMeasWLAN |  | UE supports WLAN measurements in RRC_CONNECTED state. |  |
| xx9 |  | Support of Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_loggedMeasBT |  | UE supports Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state. |  |
| xxa |  | Support of WLAN measurements in RRC_IDLE and RRC_INACTIVE state |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_loggedMeasWLAN |  | UE supports WLAN measurements in RRC_IDLE and RRC_INACTIVE state. |  |

<<< END OF CHANGES >>>