3GPP TSG-RAN5 Meeting #90-e	R5-210462r2

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1703 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align TDD UL DL Common for RRM with TS 38.533 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The message contents in Table 4.6.3-192 TDD UL/DL ConfigCommon with Condition RRM are not aligned with the RRM configuration as per TS 38.533 A.1.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The message contents in Table 4.6.3-192 for RRM have been corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The default configuration for TDD UL / DL ConfigCommon for RRM will remain incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: there was already a Table for the RRM exceptions in 7.3.1, but wrongly named. This table has been corrected and the contents for RRM have been removed from 4.6.3 R2: fixed leftover RRM condition for numberOfDownlinkSlots |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### –	TDD-UL-DL-ConfigCommon

Table 4.6.3-192: TDD-UL-DL-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1 |
|  | ms0p625 |  | SIG AND FR2 |
|  | ms2 |  | RF AND FR2 |
| nrofDownlinkSlots | 7 |  | (FR1 AND SCS30) OR (RF AND FR2 AND SCS120) |
|  | 3 |  | (FR1 AND SCS15) OR (FR2 AND SCS60) OR (SIG AND FR2 AND SCS120) |
|  | 14 |  | FR1 AND SCS60 |
| nrofDownlinkSymbols | 6 |  | FR1 AND SCS30 |
|  | 10 |  | (FR1 AND SCS15) OR (SIG AND FR2 AND (SCS60 OR SCS120)) |
|  | 12 |  | (FR1 AND SCS60) OR (RF AND FR2 AND SCS120) |
|  | 4 |  | RF AND FR2 AND SCS60 |
| nrofUplinkSlots | 2 |  | FR1 AND SCS30 |
|  | 1 |  | (FR1 AND SCS15) OR (SIG AND FR2 AND (SCS60 OR SCS120)) |
|  | 4 |  | (FR1 AND SCS60) OR (RF AND FR2 AND SCS60) |
|  | 8 |  | RF AND FR2 AND SCS120 |
| nrofUplinkSymbols | 4 |  | FR1 AND SCS30 |
|  | 2 |  | (FR1 AND SCS15) OR (SIG AND FR2 AND (SCS60 OR SCS120)) |
|  | 8 |  | FR1 AND SCS60 |
|  | 0 |  | RF AND FR2 AND (SCS60 OR SCS120) |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

# < Unchanged sections omitted >

### 7.3.1	Radio resource control information elements for RRM

As defined in clause 4.6.3 with the following exceptions:

#### –	TDD-UL-DL-ConfigCommon

Table 7.3.1-1: TDD-UL-DL-ConfigCommon

| Derivation Path: Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms0p625 |  | TDDConf.3.1 |
|  | ms4 |  |  |
| nrofDownlinkSlots | 3 |  | TDDConf.2.1, TDDConf.3.1 |
|  | 1 |  | TDDConf.1.1 |
| nrofDownlinkSymbols | 10 |  | TDDConf.1.1, TDDConf.3.1 |
|  | 6 |  | TDDConf.2.1 |
| nrofUplinkSlots | 2 |  | TDDConf.1.1 |
|  | 1 |  | TDDConf.3.1 |
|  | 4 |  | TDDConf.2.1 |
| nrofUplinkSymbols | 4 |  | TDDConf.2.1 |
|  | 2 |  | TDDConf.1.1, TDDConf.3.1 |
| dl-UL-TransmissionPeriodicity-v1530 | Not present |  |  |
|  | ms4 |  | TDDConf.1.1, TDDConf.2.1 |
| } |  |  |  |
| pattern2 | Not present |  |  |
| pattern2 SEQUENCE { |  |  | TDDConf.1.1, TDDConf.2.1 |
| dl-UL-TransmissionPeriodicity | ms1 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofDownlinkSlots | 1 |  | TDDConf.1.1 |
|  | 2 |  | TDDConf.2.1 |
| nrofDownlinkSymbols | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofUplinkSlots | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofUplinkSymbols | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TDDConf.1.1 | TDD UL/DL configuration for SCS=15kHz |
| TDDConf.2.1 | TDD UL/DL configuration for SCS=30kHz |
| TDDConf.3.1 | TDD UL/DL configuration for SCS=120kHz |

< End of changes >