3GPP TSG-RAN5 Meeting #90-e	R5-210736r1

# Electronic, 22nd February - 5th Marth, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0875 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting EN-DC A-MPR test requirements for non-overlapping test points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For UEs supporting DPS, the requirements for EN-DC A-MPR non-overlapping test points shall refer back to SA requirements. For UEs not supporting DPS, the requirements for EN-DC A-MPR non-overlapping test points shall refer to 38.101-3 requirements. Currently only one test requirement table captures non-overlapping test points, without mentioning UE capability of DPS, which might lead to confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indicating the table is for UE supporting DPS. Correcting the requirement reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE might fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3.1, 6.2B.3.2, 6.2B.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Removing redundant rows in Table 6.2B.3.2.5-1 |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2B.3.1	UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC

6.2B.3.1.1	Test purpose

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2B.1.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2B.3.1.2 	Test applicability

The requirements of this test apply in test case 6.5B.2.1.2 Additional spectrum emission mask for network signalled values NS_04 and NS_35 to all types of E-UTRA power class 3 and power class 2 UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.2B.3.1.3	Minimum conformance requirements

For intra-band contiguous EN-DC band combinations with additional requirements the allowed A-MPR is specified in table 6.2B.3.1.3-1 for UEs configured with EN-DC and combinations of network signalling values indicated in the E-UTRA and NR cell groups.

Unless otherwise stated the A-MPR specified insubclause 6.2B.3.1 for intra-band contiguous EN-DC configurations is the total power reduction allowed including MPR.

Table 6.2B.3.1.3-1: Additional maximum power reduction for Intra-band contiguous EN-DC

| DC configuration | Requirement (subclause) | E-UTRA network signalling value | NR network signalling value | A-MPR (clause) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA | 6.5B.2.1.2.3.1 | NS_35 | NS_35 | 6.2B.3.1.3.13 |
| DC_(n)41AA1 | 6.5B.2.1.2.3.2 | NS_01 or NS_04 | NS_04 | 6.2B.3.1.3.24 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The additional emission requirement is indicated when the combination of network signalling values in the two CGs is set (only for UEs configured with EN-DC). NOTE 3: The A-MPR is applied as MPR if NS_35 is not signalled. NOTE 4: Void. |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.3.1.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA overlap in time with NR.LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that doesn't support dynamic power sharing. LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

No exception requirements for NR or E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that supports dynamic power sharing. LTE anchor agnostic approach is not applied for this case.

6.2B.3.1.3.1	A-MPR for DC_(n)71AA

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with TS 36.101 [5]

-	for the SCG, A-MPR'c = [A-MPRDC]

-	for the total configured transmission power, A-MPRtot = A-MPRDC

with A-MPRDC as defined in this subclause.

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = A-MPRE-UTRA

-	for the SCG,

A-MPR'c = A-MPRNR

with A-MPRE-UTRA and A-MPRNR as defined in this subclause.

For DC_(n)71AA with configured with network signaling values as per Table 6.2B.3.1.0-1 the allowed A-MPR is defined by

-	for UE indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE

A-MPRDC = CEIL{ MA,DC (A), 0.5}

where A-MPRDC is the total power reduction allowed (dB),

-	for OFDM:

MA,DC =	11.00 - 11.67*A;		0.00 < A ≤ 0.30

8.10 - 2.00*A;					0.30 < A ≤ 0.80

6.50;								0.80 < A ≤ 1.00

-	for DFT-S-OFDM:

MA,DC =	11.00 - 13.33*A;		0.00 < A ≤ 0.30

8.00 - 3.33*A;					0.30 < A ≤ 0.60

6.00;								0.60 < A ≤ 1.00

where:

with LCRB, E-UTRA and NRB, E-UTRA the number of allocated PRB and transmission bandwidth for MCG, LCRB,NR and NRB,NR the number of allocated PRB and transmission bandwidth for SCG with SCS = 15 kHz.

-	for UE not indicating support of dynamicPowerSharing

A- MPRE-UTRA = CEIL{ MA, E-UTRA, 0.5}

A-MPRNR = CEIL{ MA,NR, 0.5}

where A-MPR is the total power reduction allowed per CG with

MA,E-UTRA=MA,DC(AE-UTRA,wc)-1-E-UTRA

MA,NR=MA,DC(ANR,wc)-1-NR

AE-UTRA,wc=LCRB,E-UTRA+1NRB,E-UTRA+NRB,NR

ANR,wc=1+LCRB,NRNRB,E-UTRA+NRB,NR

Δ E-UTRA=10 log10NRB,E-UTRANRB,E-UTRA+NRB,NR

ΔNR=10 log10NRB,NRNRB,E-UTRA+NRB,NR

Where LCRB,NR and NRB,NR the number of allocated PRB and transmission bandwidth for SCG with SCS = 15 kHz.

6.2B.3.1.3.2	A-MPR for NS_04

6.2B.3.1.3.2.0	General

When the UE is configured for B41/n41 intra-band contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this clause. The A-MPR for EN-DC defined in this clause is used instead of MPR defined in 6.2B.2.2, not additively, so EN-DC MPR = 0 when NS_04 is signaled. For UEs scheduled with single uplink transmission, AMPR in subclause 6.2.4 of [5] and 6.2.3 of [2] apply.

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with 36.101 [5]

-	for the SCG,

A-MPR'c = A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3)

-	for the total configured transmission power,

A-MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - A-MPRE-UTRA)/10) + 10^((PPowerClass,NR - A-MPRNR)/10))

where

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA + MPRsingle,E-UTRA, A-MPRIM3 )

with

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 36.101 [5]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [5]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = MAX( A-MPRsingle, E-UTRA + MPRsingle,E-UTRA, A-MPRIM3 )

-	for the SCG,

A-MPR'c = MAX( A-MPRsingle,NR, A-MPRIM3 )

where

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 36.101 [5]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [5]

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If FIM3,low_block,low < 2490.5 MHz

Channel Configuration Case B. A-MPRIM3 defined in clause 6.2B.3.1.3.2.2.

Else

Channel Configuration Case A. A-MPRIM3 defined in clause 6.2B.3.1.3.2.1.

where

-	FIM3,low_block,low = (2 * Flow_channel,low_edge) – Fhigh_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

Where the transmission bandwidth configuration for NR is the maximum frequency span covering all the configured SCSSpecificCarrier for scenarios that carrier bandwidths with different SCS can be fully overlapped.

6.2B.3.1.3.2.1	A-MPRIM3 for NS_04 to meet -13 dBm / 1MHz for 26dBm UE power

A-MPR in this subclause is relative to 26 dBm for a power class 2 Cell Group. The same A-MPR is used relative to 23 dBm for a power class 3 Cell Group. For the UE is configured with channel configurations Case A or Case C (defined in Clause 6.2B.3.2.3.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 15 ; 0 ≤ B < 0.5

10 ; 0.5 ≤ B < 1.0

8 ; 1.0 ≤ B < 2.0

6 ; 2.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR =15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA =15 kHz is assumed in calculation of B

and MA is reduced by 1 dB for B < 2.0.

6.2B.3.1.3.2.2	A-MPR for NS_04 to meet -25 dBm / 1MHz for 26 dBm UE power

A-MPR in this subclause is relative to 26 dBm for a power class 2 Cell Group. The same A-MPR is used relative to 23 dBm for a power class 3 Cell Group. For the UE is configured with channel configurations Case B or Case D (defined in clause 6.2B.3.2.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 15 ; 0 ≤ B < 1.0

14 ; 1.0 ≤ B < 2.0

13 ; 2.0 ≤ B < 5.0

12 ; 5.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR =15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA =15 kHz is assumed in calculation of B

and MA is reduced by 1 dB.

6.2B.3.1.4	Test description

6.2B.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on NR operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.2B.3.1.4.1-1 through 6.2B.3.1. 4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.3.1.4.1-0: Void

Table 6.2B.3.1.4.1-1: Test configuration table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | NC |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  | Low range, High range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  | Lowest, Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (Note 1) | Power config (Note 8) |
| 1 | Default | N/A | 16QAM | Outer_Full | DFT-s-OFDM Pi/2 BPSK | Outer_Full | B |
| 2 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | B |
| 3 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 4 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | B |
| 6 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 8 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 9 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 10 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 11 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 12 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 13 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 14 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 15 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 16QAM | Outer_Full | B |
| 16 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | Edge_1RB_Right | B |
| 17 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | N/A | A |
| 18 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Right | A |
| 19 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | Edge_1RB_Left | B |
| 20 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Left | A |
| 21 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | N/A | A |
| 22 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 64QAM | Outer_Full | B |
| 23 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 64QAM | Edge_1RB_Right | B |
| 24 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 64QAM | Edge_1RB_Left | B |
| 25 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 256QAM | Outer_Full | B |
| 26 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 256QAM | Edge_1RB_Right | B |
| 27 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 256QAM | Edge_1RB_Left | B |
| 28 | Default |  | 16QAM | Outer_Full | CP-OFDM QPSK | Outer_Full | B |
| 29 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right | B |
| 30 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | N/A | A |
| 31 (Note 3) | High |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Right | A |
| 32 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Left | B |
| 33 (Note 4) | Low |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Left | A |
| 34 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | N/A | A |
| 35 | Default |  | 16QAM | Outer_Full | CP-OFDM 16QAM | Outer_Full | B |
| 36 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | Edge_1RB_Right | B |
| 37 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | N/A | A |
| 38 (Note 3) | High |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Right | A |
| 39 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | Edge_1RB_Left | B |
| 40 (Note 4) | Low |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Left | A |
| 41 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | N/A | A |
| 42 | Default |  | 16QAM | Outer_Full | CP-OFDM 64QAM | Outer_Full | B |
| 43 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 64QAM | Edge_1RB_Right | B |
| 44 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 64QAM | Edge_1RB_Left | B |
| 45 | Default |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 46 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 47 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 48 (Note 4) | Default |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Edge_Full_Right is defined as 2 RBs allocated at the right edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 7: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. NOTE 8: Power config as specified in Table 6.2B.3.1.4.3-1-1 to 6.2B.3.1.4.3-2 (PC3) or Table 6.2B.3.1.4.3-3 to 6.2B.3.1.4.3-4 (PC2). |  |  |  |  |  |  |  |

Table 6.2B.3.1.4.1-2: NR test configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | NC |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  | Low range, High range (Note 7) |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  | Lowest, Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (Note 1) | Power config (Note 8) |
| 1 | Default | N/A | 16QAM | Outer_Full | DFT-s-OFDM Pi/2 BPSK | Outer_Full | B |
| 2 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | B |
| 3 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 4 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM PiI/2 BPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | B |
| 6 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 8 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 9 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 10 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 11 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 12 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 13 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 14 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 15 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 16QAM | Outer_Full | B |
| 16 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | Edge_1RB_Right | B |
| 17 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | N/A | A |
| 18 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Right | A |
| 19 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | Edge_1RB_Left | B |
| 20 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Left | A |
| 21 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | N/A | A |
| 22 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 64QAM | Outer_Full | B |
| 23 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 64QAM | Edge_1RB_Right | B |
| 24 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 64QAM | Edge_1RB_Left | B |
| 25 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM 256QAM | Outer_Full | B |
| 26 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 256QAM | Edge_1RB_Right | B |
| 27 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 256QAM | Edge_1RB_Left | B |
| 28 | Default |  | 16QAM | Outer_Full | CP-OFDM QPSK | Outer_Full | B |
| 29 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right | B |
| 30 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | N/A | A |
| 31 (Note 3) | High |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Right | A |
| 32 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Left | B |
| 33 (Note 4) | Low |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Left | A |
| 34 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | N/A | A |
| 35 | Default |  | 16QAM | Outer_Full | CP-OFDM 16QAM | Outer_Full | B |
| 36 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | Edge_1RB_Right | B |
| 37 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | N/A | A |
| 38 (Note 3) | High |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Right | A |
| 39 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | Edge_1RB_Left | B |
| 40 (Note 4) | Low |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Left | A |
| 41 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | N/A | A |
| 42 | Default |  | 16QAM | Outer_Full | CP-OFDM 64QAM | Outer_Full | B |
| 43 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 64QAM | Edge_1RB_Right | B |
| 44 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 64QAM | Edge_1RB_Left | B |
| 45 | Default |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 46 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 47 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 48 (Note 4) | Default |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Edge_Full_Right is defined as 2 RBs allocated at the right edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 7: Additional IM3 test frequencies may apply. NOTE 8: Power config as specified in Table 6.2B.3.1.4.3-1 to 6.2B.3.1.4.3-2 (PC3) or Table 6.2B.3.1.4.3-3 to 6.2B.3.1.4.3-4 (PC2). NOTE 9: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. |  |  |  |  |  |  |  |

Table 6.2B.3.1.4.1-3: Additional IM3 Test Frequencies for NS_04 intra-band contiguous EN-DC

| Additional Initial Condition |  |
| --- | --- |
| Additional IM3 Test Frequencies | if SCS 15 kHz, 15 kHz NR raster, and NR ChBw 40 MHz or 50 MHz then IM3 test frequencies as specified in Table 6.2B.3.1.4.1-4. if SCS 30 kHz, 30 kHz NR raster, and NR ChBw 40 MHz, 50 MHz or 60 MHz then IM3 test frequencies as specified in Table 6.2B.3.1.4.1-5. if SCS 60 kHz, 15 kHz NR raster, and NR ChBw 40 MHz, 50 MHz or 60 MHz then IM3 test frequencies as specified in Table 6.2B.3.1.4.1-6. |

Table 6.2B.3.1.4.1-4: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 15 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2600.400 | 40694 | - | - | - | - | - | - |  |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2570.400 | 514080 | 2550.96 | 510192 | 0 | 15 | 6384 | 514080 |  |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2620.400 | 40894 | - | - | - | - | - | - |  |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2585.400 | 517080 | 2561.1 | 512220 | 0 | 15 | 6408 | 517080 |  |

Table 6.2B.3.1.4.1-5: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 30 kHz, 30 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2600.400 | 40694 | - | - | - | - | - | - |  |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2570.400 | 514080 | 2550.96 | 510192 | 0 | 15 | 6384 | 514080 |  |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2620.400 | 40894 | - | - | - | - | - | - |  |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2585.400 | 517080 | 2561.1 | 512220 | 0 | 15 | 6408 | 517080 |  |
| E-UTRA: 20MHz + NR: 60MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2640.500 | 41095 | - | - | - | - | - | - |  |
|  | NR CC1 | 60 | 162 | Downlink & Uplink | IM3 | 2600.500 | 520100 | 2571.34 | 514268 | 0 | 30 | 6438 | 520100 |  |

Table 6.2B.3.1.4.1-6: EN-DC combination DC_(n)41AA, intra-band contiguous, SCS 60 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2600.400 | 40694 | - | - | - | - | - | - |  |
|  | NR CC1 | 40 | 51 | Downlink & Uplink | IM3 | 2570.400 | 514080 | 2552.04 | 510408 | 0 | 15 | 6387 | 514080 |  |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2620.400 | 40894 | - | - | - | - | - | - |  |
|  | NR CC1 | 50 | 65 | Downlink & Uplink | IM3 | 2585.400 | 517080 | 2562 | 512400 | 0 | 15 | 6411 | 517080 |  |
| E-UTRA: 20MHz + NR: 60MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2640.500 | 41095 | - | - | - | - | - | - |  |
|  | NR CC1 | 60 | 79 | Downlink & Uplink | IM3 | 2600.500 | 520100 | 2572.06 | 514412 | 0 | 15 | 6435 | 520100 |  |

Editor's note:	The following lines belong at the end of clause 6.2B.3.1.4.1. As new tables are added to this clause, these lines should always follow the tables.

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for E-UTRA the cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set according to TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG link respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.1.4.3.

8.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 6.2B.3.1.4.1-1 or 6.2B.3.1.4.1-2 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers for the EN-DC configuration, which shall meet the requirements described in table 6.2B.3.1.5.1-1 through to 6.2B.3.1.5.2-6. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms). For TDD, only slots consisting of only UL symbols are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2B.3.1.4.1-1 or 6.2B.3.1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1, with the following exceptions.

Table 6.2B.3.1.4.3-1: RRCConnectionReconfiguration: nr-Config-r15 for PC3

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 | Power config A (NOTE 1) |  |
|  | 20 | Power config B (NOTE 2) |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.3.1.4.3-2: PhysicalCellGroupConfig for PC3

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 | Power config A (NOTE 1) |  |
|  | 20 | Power config B (NOTE 2) |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.3.1.4.3-3: RRCConnectionReconfiguration: nr-Config-r15 for PC2

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 26 | Power config A (NOTE 1) |  |
|  | 23 | Power config B (NOTE 2) |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

Table 6.2B.3.1.4.3-4: PhysicalCellGroupConfig for PC2

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 26 | Power config A (NOTE 1) |  |
|  | 23 | Power config B (NOTE 2) |  |
| NOTE 1: Applies when E-UTRA UL transmission not overlapping with NR UL transmission in time. NOTE 2: Applies when E-UTRA UL transmission overlapping with NR UL transmission in time. |  |  |  |

6.2B.3.1.4.3.1	Message contents exceptions (network signalled value "NS_04")

Message contents are according to TS 38.508-1 [6] clause 4.6.1 with the following exceptions for NS_04:

Table 6.2B.3.1.4.3.1-1: AdditionalSpectrumEmission for MCG and "NS_04"

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_04) |  |  |

Table 6.2B.3.1.4.3.1-2: AdditionalSpectrumEmission for SCG and "NS_04"

| Derivation Path: TS 38.508-1 [6] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_04) |  |  |

6.2B.3.1.4.3.2	Message contents exceptions (network signalled value "NS_35")

Message contents are according to TS 38.508-1 [6] clause 4.6.1 with the following exceptions for NS_35:

Table 6.2B.3.1.4.3.2-1: AdditionalSpectrumEmission for MCG and "NS_35"

| Derivation Path: 36.508 [11] clause 4.6.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_35) |  |  |

Table 6.2B.3.1.4.3.2-2: AdditionalSpectrumEmission for for SCG "NS_35"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_35) |  |  |

6.2B.3.1.5	Test requirement

Table: 6.2B.3.1.5-1: Test Tolerance for UE maximum output power (LTE, NR TX separately)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| LTE | BW ≤ 20MHz | 0.7 | 1.0 | 1.3 |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 |

6.2B.3.1.5.1	Test requirement for network signalled value "NS_35"

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in table 6.2B.3.1.5.1-1. The allowed A-MPR values specified in table 6.2B.3.1.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.1.1.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2B.1.1.3-1 apply.

Table 6.2B.3.1.5.1-1: UE Power Class test requirements for network signalled value "NS_35"for UEs not supporting dynamic power sharing

| Test ID | Test freq. range | E-UTRA BW | NR BW | Modulation | PPowerClass (dBm) | ΔPPowerClass (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 6.0 | 0 | 17.0 | 5 | +2/-3 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 6.0 | 0 | 17.0 | 5 | +2/-3 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 6.0 | 0 | 17.0 | 5 | +2/-3 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 6.0 | 0 | 17.0 | 5 | +2/-3 | 25+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 10.5 | 0 | 12.5 | 6 | +2/-3 | 25+TT | 6.5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 10.5 | 0 | 12.5 | 6 | +2/-3 | 25+TT | 6.5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 3, 10, 17 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 12.5 | 0 | 10.5 | 6 | +2/-3 | 25+TT | 4.5-TT |
| 3, 14, 17 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 4, 11, 18 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 12.5 | 0 | 10.5 | 12.5 | +2/-3 | 25+TT | 4.5-TT |
| 4, 11, 18 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 16.0 | 0 | 7.0 | 16.0 | +2/-3 | 25+TT | 0-TT |
| 6, 13, 20 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 6, 13, 20 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 7, 14, 21 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 7, 14, 21 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 28, 35, 42, 45 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 28, 35, 42, 45 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 28, 35, 42, 45 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 28, 35, 42, 45 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 29, 35, 42, 45 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6.5-TT |
| 29, 35, 42, 45 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 29, 35, 42, 45 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6.5-TT |
| 29, 35, 42, 45 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 30, 34, 37, 41 | Low | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 30, 34, 37, 41 | Low | 5 | 15 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 31, 33, 38, 40 | High | 5 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 31, 33, 38, 40 | High | 15 | 5 | E-UTRA/NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |

Table 6.2B.3.1.5.1-1A: UE Power Class test requirements for network signalled value "NS_35" for UEs supporting dynamic power sharing

| Test ID | Test freq. range | E-UTRA BW | NR BW | Modulation | PPowerClass (dBm) | ΔPPowerClass (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25 | Low | 5 | 5 | E-UTRA | 23 | 0 | 8.5 | 0 | 14.5 | 5 | +2/-3 | 25+TT | 9.5-TT |
| 1, 8, 15, 22, 25 | Low | 5 | 5 | NR | 23 | 0 | 8.5 | 0 | 14.5 | 5 | +2/-3 | 25+TT | 9.5-TT |
| 1, 8, 15, 22, 25 | Low | 5 | 15 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 1, 8, 15, 22, 25 | Low | 5 | 15 | NR | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 1, 8, 15, 22, 25 | High | 5 | 5 | E-UTRA | 23 | 0 | 8.5 | 0 | 14.5 | 5 | +2/-3 | 25+TT | 9.5-TT |
| 1, 8, 15, 22, 25 | High | 5 | 5 | NR | 23 | 0 | 8.5 | 0 | 14.5 | 5 | +2/-3 | 25+TT | 9.5-TT |
| 1, 8, 15, 22, 25 | High | 15 | 5 | E-UTRA | 23 | 0 | 6.5 | 0 | 16.5 | 5 | +2/-3 | 25+TT | 11.5-TT |
| 1, 8, 15, 22, 25 | High | 15 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 5 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | Low | 5 | 15 | NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 5 | 5 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 15 | 5 | E-UTRA | 23 | 0 | 11.5 | 0 | 11.5 | 6 | +2/-3 | 25+TT | 5.5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 2, 26, 27 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 3, 10, 17 | Low | 5 | 5 | E-UTRA/ | 23 | 0 | 12.5 | 0 | 10.5 | 6 | +2/-3 | 25+TT | 4.5-TT |
| 3, 10, 17 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 4, 11, 18 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 4, 11, 18 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 6, 17, 20 | Low | 5 | 5 | E-UTRA/ | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 6, 17, 20 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.5 | 0 | 6.5 | 7 | +2/-3 | 25+TT | -0.5-TT |
| 7, 14, 21 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 7, 14, 21 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 28, 35, 42, 45 | Low | 5 | 5 | E-UTRA | 23 | 0 | 9.5 | 0 | 13.5 | 5 | +2/-3 | 25+TT | 8.5-TT |
| 28, 35, 42, 45 | Low | 5 | 5 | NR | 23 | 0 | 9.5 | 0 | 13.5 | 5 | +2/-3 | 25+TT | 8.5-TT |
| 28, 35, 42, 45 | Low | 5 | 15 | E-UTRA | 23 | 0 | 13.5 | 0 | 9.5 | 6 | +2/-3 | 25+TT | 3.5-TT |
| 28, 35, 42, 45 | Low | 5 | 15 | NR | 23 | 0 | 7.0 | 0 | 16.0 | 5 | +2/-3 | 25+TT | 11-TT |
| 28, 35, 42, 45 | High | 5 | 5 | E-UTRA | 23 | 0 | 9.5 | 0 | 13.5 | 5 | +2/-3 | 25+TT | 8.5-TT |
| 28, 35, 42, 45 | High | 5 | 5 | NR | 23 | 0 | 9.5 | 0 | 13.5 | 5 | +2/-3 | 25+TT | 8.5-TT |
| 28, 35, 42, 45 | High | 15 | 5 | E-UTRA | 23 | 0 | 7.0 | 0 | 16.0 | 5 | +2/-3 | 25+TT | 11-TT |
| 28, 35, 42, 45 | High | 15 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | Low | 5 | 5 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | Low | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 0-TT |
| 29, 36, 43, 46 | Low | 5 | 15 | NR | 23 | 0 | 11.0 | 0 | 12.0 | 6 | +2/-3 | 25+TT | 6-TT |
| 29, 36, 43, 46 | High | 5 | 5 | E-UTRA | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 | +2/-3 | 25+TT | 4-TT |
| 29, 36, 43, 46 | High | 15 | 5 | E-UTRA | 23 | 0 | 11.5 | 0 | 11.5 | 6 | +2/-3 | 25+TT | 0-TT |
| 29, 36, 43, 46 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 | +2/-3 | 25+TT | 5.5-TT |
| 30, 37 | Low | 5 | 5 | E-UTRA/ | 23 | 0 | 13.0 | 0 | 10.0 | 6 |  | 25+TT | 4-TT |
| 30, 37 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.0 | 0 | 7.0 | 7 |  | 25+TT | 0-TT |
| 31, 38 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 |  | 25+TT | 4-TT |
| 31, 38 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 |  | 25+TT | 0-TT |
| 34, 41 | High | 5 | 5 | NR | 23 | 0 | 13.0 | 0 | 10.0 | 6 |  | 25+TT | 4-TT |
| 34, 41 | High | 15 | 5 | NR | 23 | 0 | 16.0 | 0 | 7.0 | 7 |  | 25+TT | 0-TT |
| 33, 40 | Low | 5 | 5 | E-UTRA/ | 23 | 0 | 13.0 | 0 | 10.0 | 6 |  | 25+TT | 4-TT |
| 33, 40 | Low | 5 | 15 | E-UTRA | 23 | 0 | 16.5 | 0 | 6.5 | 7 |  | 25+TT | -0.5-TT |
| NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2B.3.1.5.2	Test requirement for network signalled value "NS_04"

Table 6.2B.3.1.5.2-1: UE Power Class 3 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 3 | 20 | 23 | 6 | 2 | 25+TT | 14-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 9 | 14 | 23 | 6 | 2 | 25+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 11 | 12 | 23 | 6 | 2 | 25+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 6 | 17 | 23 | 5 | 2 | 25+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 11 | 12 | 23 | 6 | 2 | 25+TT | 6-TT |
| 48 (Note 3, 4) | 15 | 7 | 16 | 23 | 5 | 2 | 25+TT | 11-TT |
| 48 (Note 3, 4) | 30, 60 | 12 | 11 | 23 | 6 | 2 | 25+TT | 5-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 7 | 16 | 23 | 5 | 2 | 25+TT | 11-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-2: UE Power Class 3 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission not overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | MPR (dB) | A-MPR (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (Note 1) | 15, 30, 60 | 0 | 3 | 20 | 23 | 4 | 2 | 25+TT | 16-TT |
| 4 (Note 2) | 15, 30, 60 | 0.5 | 0 | 22.5 | 23 | 2 | 2 | 25+TT | 20.5-TT |
| 6 (Note 2) | 15, 30, 60 | 0 | 3.5 | 19.5 | 23 | 3.5 | 2 | 25+TT | 16-TT |
| 11 (Note 2) | 15, 30, 60 | 1 | 0 | 22 | 23 | 2 | 2 | 25+TT | 20-TT |
| 13 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 (Note 2) | 15, 30, 60 | 2 | 0 | 21 | 23 | 2 | 2 | 25+TT | 19-TT |
| 20 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5-TT |
| 33 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12-TT |
| 38 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5-TT |
| 40 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12-TT |
| NOTE 1: Test configuration IDs without transmission overlap, 1RB E-UTRA allocation, A-MPR requirements in TS 36.101 [5] apply NOTE 2: Test configuration IDs without transmission overlap, 1RB NR allocation, A-MPR requirements in TS 38.101-1 [2] apply. NOTE 3: Void. NOTE 4: Void. NOTE 5: Void. NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-3: UE Power Class 3 test requirements for NS_04, not supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 6 | 17 | 23 | 5 | 2 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 12 | 11 | 23 | 6 | 2 | 25+TT | 5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 14 | 9 | 23 | 7 | 2 | 25+TT | 2-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 9 | 14 | 23 | 6 | 2 | 25+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 14 | 9 | 23 | 7 | 2 | 25+TT | 2-TT |
| 48 (Note 3, 4) | 15 | 10 | 13 | 23 | 6 | 2 | 25+TT | 7-TT |
| 48 (Note 3, 4) | 30, 60 | 15 | 8 | 23 | 7 | 2 | 25+TT | 1-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 10 | 13 | 23 | 6 | 2 | 25+TT | 7-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-4: UE Power Class 2 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 3 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 9 | 17 | 26 | 5 | 2 | 28+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 11 | 15 | 26 | 6 | 2 | 28+TT | 9-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 6 | 20 | 26 | 6 | 2 | 28+TT | 14-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 11 | 15 | 26 | 6 | 2 | 28+TT | 9-TT |
| 48 (Note 3, 4) | 15 | 7 | 19 | 26 | 5 | 2 | 28+TT | 14-TT |
| 48 (Note 3, 4) | 30, 60 | 12 | 14 | 26 | 6 | 2 | 28+TT | 8-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 7 | 19 | 26 | 5 | 2 | 28+TT | 14-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-5: UE Power Class 2 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission not overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | MPR (dB) | A-MPR (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (Note 1) | 15, 30, 60 | 0 | 3 | 23 | 26 | 2 | 2 | 28+TT | 21-TT |
| 4 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 6 (Note 2) | 15, 30, 60 | 0 | 5.5 | 20.5 | 26 | 2.5 | 2 | 28+TT | 18-TT |
| 11 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 13 (Note 2) | 15, 30, 60 | 0 | 6 | 20 | 26 | 2.5 | 2 | 28+TT | 17.5-TT |
| 18 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 20 (Note 2) | 15, 30, 60 | 0 | 6 | 20 | 26 | 2.5 | 2 | 28+TT | 17.5-TT |
| 31 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 33 (Note 2) | 15, 30, 60 | 0 | 7.5 | 18.5 | 26 | 4 | 2 | 28+TT | 14.5-TT |
| 38 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 40 (Note 2) | 15, 30, 60 | 0 | 7.5 | 18.5 | 26 | 4 | 2 | 28+TT | 14.5-TT |
| NOTE 1: Test configuration IDs without transmission overlap, 1RB E-UTRA allocation, A-MPR requirements in TS 36.101 [5] apply. NOTE 2: Test configuration IDs without transmission overlap, 1RB NR allocation, A-MPR requirements in TS 38.101-1 [2] apply. NOTE 3: Void. NOTE 4: Void. NOTE 5: Void. NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-6: UE Power Class 2 test requirements for NS_04, not supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 6 | 20 | 26 | 6 | 2 | 28+TT | 14-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 12 | 14 | 26 | 6 | 2 | 28+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 14 | 12 | 26 | 6 | 2 | 28+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 9 | 17 | 26 | 5 | 2 | 28+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 14 | 12 | 26 | 6 | 2 | 28+TT | 6-TT |
| 48 (Note 3, 4) | 15 | 10 | 16 | 26 | 5 | 2 | 28+TT | 11-TT |
| 48 (Note 3, 4) | 30, 60 | 15 | 11 | 26 | 6 | 2 | 28+TT | 5-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 10 | 16 | 26 | 5 | 2 | 28+TT | 11-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

6.2B.3.2	UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC

Editor's note:	This test case is incomplete. The following aspects are either missing or not yet determined:

-	Test frequencies for the Minimum WGAP

6.2B.3.2.1	Test purpose

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2B.1.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2B.3.2.2	Test applicability

The requirements of this test apply in test case 6.5B.2.2.2 Additional spectrum emission mask for network signalled values NS_04 to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.2B.3.2.3	Minimum conformance requirements

For intra-band non-contiguous EN-DC band combinations with additional requirements the A-MPR allowed are specified in table 6.2B.3.2.3-1 for UEs configured with EN-DC and combinations of network signalling values indicated in the E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR specified in subclause6.2B.3.2 for intra-band non-contiguous EN-DC configurations is the total power reduction allowed including MPR. For UEs scheduled with single uplink transmission, AMPR in subclause 6.2.4 of [4] and 6.2.3 of [2] apply.

Table 6.2B.3.2.3-1: Allowed power reduction for intra-band non-contiguous EN-DC

| DC configuration | Requirement (clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (clause) |
| --- | --- | --- | --- | --- |
| DC_41A_n41A1 | 6.6.3.3.19 and 6.6.2.2.2 of TS 36.101 [5] and 6.5.2.3.2 and 6.5.3.3.1 of TS 38.101-1 [2] | NS_01 or NS_04 | NS_04 | 6.2B.3.2.3.1 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The requirement applies when the combination of network signalling values in the two CGs is set (only for UEs configured with EN-DC). |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.3.2.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA overlap in time with NR.LTE anchor agnostic approach is not applied for this case and referred to as sub-test 1. E-UTRA test point analysis is included and E-UTRA measurements are performed.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that doesn't support dynamic power sharing. LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

No exception requirements for NR or E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that supports dynamic power sharing. LTE anchor agnostic approach is not applied for this case.

6.2B.3.2.3.1	A-MPR for NS_04

When the UE is configured for B41/n41 intra-band non-contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this clause. The A-MPR for EN-DC defined in this clause is used instead of MPR defined in 6.2B.2.2, not additively, so EN-DC MPR=0 when NS_04 is signalled.

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with TS 36.101 [5]

-	for the SCG,

A-MPR'c = A-MPRNR = MAX( A-MPRsingle,NR, A-MPREN-DC)

-	for the total configured transmission power,

A-MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - A-MPRE-UTRA)/10) + 10^((PPowerClass,NR - A-MPRNR)/10))

where

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA + MPRsingle,E-UTRA, A-MPREN-DC )

A-MPREN-DC = MAX(A-MPRIM3, A-MPRACLRoverlap )

with

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 38.101-3 [4]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 38.101-3 [4]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = MAX( A-MPRsingle, E-UTRA + MPRsingle,E-UTRA, A-MPRIM3, A-MPRACLRoverlap)

-	for the SCG,

A-MPR'c = MAX( A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap)

where

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 36.101 [5]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [5]

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If AND( FIM3,low_block,high < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

Channel Configuration Case C. A-MPRIM3 defined in Clause 6.2B.3.1.3.2.1

Else

Channel Configuration Case D. A-MPRIM3 defined in Clause 6.2B.3.1.3.2.2

where

-	FIM3,low_block,high = (2 * Flow_channel,high_edge ) – Fhigh_channel,low_edge

-	FIM3,high_block,low = (2 * Fhigh_channel,low_edge) – Flow_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Flow_channel,high_edge is the uppermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,low_edge is the lowermost frequency of upper transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.6.2.2.2 in [5] and Subclause 6.5.2.3.2 in [2] respectively.

Where the transmission bandwidth configuration for NR is the maximum frequency span covering all the configured SCSSpecificCarrier for scenarios that carrier bandwidths with different SCS can be fully overlapped.

The UE determines the value of A-MPRACLRoverlap as specified in Table 6.2B.3.2.3.1-1:

Table 6.2B.3.2.3.1-1: A-MPRACLRoverlap

| Wgap | A-MPRACLRoverlap |
| --- | --- |
| < BWchannel,E-UTRA + BWchannel,NR | 4 dB |
| ≥ BWchannel,E-UTRA + BWchannel,NR | 0 dB |
| NOTE 1: Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge |  |

6.2B.3.2.4	Test description

6.2B.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2B.3.1-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in test configuration table 6.2B.3.1.4.1-1 through 6.2B.3.1.4.1-2 with additional IM3 test frequencies for NS_04 in 6.2B.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A2. Configurations of PDSCH and PDCCH before measurement are specified in Annex TS 36.521-1 [10] Annex C and in Annex C2 for LTE link and NR link respectively.

Table 6.2B.3.2.4.1-0: E-UTRA test configuration table

| E-UTRA Test Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency (Note 1) | Downlink | Uplink |  |
|  |  | N/A for A-MPR testing | Modulation | RB allocation |
| 20 MHz | Low range and High range (Note 2) |  | QPSK | 100 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [6] clause 4.3.1 NOTE 2: NR carrier shall be the outermost carrier during test. |  |  |  |  |

Table 6.2B.3.2.4.1-1: Additional IM3 Test Frquencies for NS_04 intra-band non-contiguous EN-DC

| Additional Initial Condition |  |
| --- | --- |
| Additional IM3 Test Frquencies | if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 15 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-2. if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 30 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-3. if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 60 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-4. |

Table 6.2B.3.2.4.1-2: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 15 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2528.86 | 505772 | 0 | 15 | 6327 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2657.26 | 531452 | 0 | 15 | 6648 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2665.000 | 533000 | 2640.7 | 528140 | 0 | 15 | 6606 | 533000 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519 | 503800 | 0 | 15 | 6303 | 508660 |

Table 6.2B.3.2.4.1-3: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 30 kHz, 30 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 106 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2529.22 | 505844 | 0 | 30 | 6333 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 106 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2657.62 | 531524 | 0 | 30 | 6654 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 133 | Downlink & Uplink | IM3 | 2665.000 | 533000 | 2641.06 | 528212 | 0 | 30 | 6612 | 533000 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 133 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519.36 | 503872 | 0 | 30 | 6309 | 508660 |

Table 6.2B.3.2.4.1-4: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 60 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 51 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2529.94 | 505988 | 0 | 15 | 6330 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 51 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2658.34 | 531668 | 0 | 15 | 6651 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 65 | Downlink & Uplink | IM3 | 2665.000 | 535340 | 2653.3 | 530660 | 0 | 15 | 6639 | 535340 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 65 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519.9 | 503980 | 0 | 15 | 6306 | 508660 |

Editor's note:	The following lines belong at the end of clause 6.2B.3.2.4.1. As new tables are added to this clause, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set according to TS 36.521-1 [10] Annex A.2 and Annex A for LTE link and NR link respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.2.4.3.

8.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.3.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format [0_1] for C_RNTI to schedule the UL RMC according to table 6.2B.3.2.4.1-1 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers for the EN-DC configuration, which shall meet the requirements described in table 6.2B.3.2.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms).

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2B.3.1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1, with the following exceptions.

Table 6.2B.3.2.4.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 | Apply if run test points with E-UTRA UL transmission not overlapping with NR UL transmission in time for PC3 UE |  |
|  | 20 | Apply if run test points with E-UTRA UL transmission overlapping with NR UL transmission in time for PC3 UE, and UE doesn’t support dynamic power sharing. |  |
| P-Max | 23 | Apply if run test points with E-UTRA UL transmission not overlapping with NR UL transmission in time for PC3 UE |  |
|  | 20 | Apply if run test points with E-UTRA UL transmission overlapping with NR UL transmission in time for PC3 UE, and UE doesn’t support dynamic power sharing. |  |

6.2B.3.2.4.3.1	Message contents exceptions (network signalled value "NS_04")

For "NS_04" see A-MPR test case in table 6.2B.3.1.4.3.1-1 and table 6.2B.3.1.4.3.1-2.

6.2B.3.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in table 6.2B.3.2.5-1. The allowed A-MPR values specified in table 6.2B.3.2.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.1.1.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2B.1.1.3-1 apply.

Table 6.2B.3.2.5-0: Test Tolerance for UE maximum output power (LTE, NR TX separately)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| LTE | BW ≤ 20MHz | 0.7 | 1.0 | 1.3 |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 |

Table 6.2B.3.2.5-1: UE Power Class 3 test requirements for network signalled value "NS_04"

| Test ID | Modulation | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | A-MPREN-DC (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 1 | 0 | 2 | 0 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 2 | 0 | 2 | 0 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 25+TT | 6-TT (4.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 1 | 0 | - | Note 5 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 2 | 0 | - | Note 5 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 25+TT | 6-TT (4.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 3 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 4 | 0 | 1 | Note 6 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 25+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 3 | 0 | - | Note 5 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 4 | 0 | - | Note 5 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 25+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 2 | 0 | - | Note 5 | 11 | 11 | 0 (1.5) | 9 (7.5) | 6 (6) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 | E-UTRA Note 1 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37 and 41 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 4 | NR | 0 | 0.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 25+TT | 20.5-TT (19-TT) |
| 6 | NR | 0 | 0 | Note 9 | 3.5 | 3.5 | 0 (1.5) | 19.5 (18) | 3.5 (4) | 2 (3.5) | 25+TT | 16-TT (14-TT) |
| 11 | NR | 0 | 1 | Note 9 | 0 | 0 | 0 (1.5) | 22 (20.5) | 2 (2.5) | 2 (3.5) | 25+TT | 20-TT (18-TT) |
| 13 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 18 | NR | 0 | 2 | Note 9 | 0 | 0 | 0 (1.5) | 21 (19.5) | 2 (3.5) | 2 (3.5) | 25+TT | 19-TT (16-TT) |
| 20 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 31 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 33 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| 38 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 40 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| NOTE 1: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 2: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 3: When NR SCS = 15kHz. NOTE 4: When NR SCS = 30 kHz or 60 kHz. NOTE 5: NR A-MPR values for NS_04 are defined in Table 6.2.3.3.2-1. NOTE 6: E-UTRA A-MPR= 3 dB for 1 RB and fc < 2517.5 MHz, otherwise 0 dB. NOTE 7: ΔTC,c = 1.5 dB for transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, otherwise 0 dB. NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.2.5-0. NOTE 9: Apply for UE supporting dynamic power sharing. NR A-MPR values for NS_04 are defined in TS 38.101-1 [2]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.2.5-2: UE Power Class 2 test requirements for network signalled value "NS_04"

| Test ID | Modulation | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | A-MPREN-DC (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 1 | 3 | 2 | 0 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 2 | 3 | 2 | 0 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 1 | 3 | - | Note 5 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 3 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 4 | 3 | 1 | Note 6 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 2 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 3 | 3 | - | Note 5 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 4 | 3 | - | Note 5 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 9 (7.5) | 6 (6) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 | E-UTRA Note 1 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37 and 41 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 4 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 6 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 20.5 (19) | 2.5 (3.5) | 2 (3.5) | 28+TT | 18-TT (15.5-TT) |
| 11 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 13 | NR | 0 | 0 | Note 9 | 6 | 6 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 28+TT | 17.5-TT (14.5-TT) |
| 18 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 20 | NR | 0 | 0 | Note 9 | 6 | 6 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 28+TT | 17.5-TT (14.5-TT) |
| 31 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 33 | NR | 0 | 0 | Note 9 | 7.5 | 7.5 | 0 (1.5) | 18.5 (17) | 4 (5) | 2 (3.5) | 28+TT | 14.5-TT (12-TT) |
| 38 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 40 | NR | 0 | 0 | Note 9 | 7.5 | 7.5 | 0 (1.5) | 18.5 (17) | 4 (5) | 2 (3.5) | 28+TT | 14.5-TT (12-TT) |
| NOTE 1: When FIM3,low_block,low ≥ 2490.5 MHz (Case A). NOTE 2: When FIM3,low_block,low < 2490.5 MHz (Case B). NOTE 3: When NR SCS = 15kHz. NOTE 4: When NR SCS = 30 kHz or 60 kHz. NOTE 5: NR A-MPR values for NS_04 are defined in Table 6.2.3.3.2-1. NOTE 6: E-UTRA A-MPR= 3 dB for 1 RB and fc < 2517.5 MHz, otherwise 0 dB. NOTE 7: ΔTC,c = 1.5 dB for transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, otherwise 0 dB. NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.2.5-0. NOTE 9: Apply for UE supporting dynamic power sharing. NR A-MPR values for NS_04 are defined in TS 38.101-1 [24]. |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Changes >