3GPP TSG-RAN WG5 Meeting #93-e	R5-216672

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0196 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2.8 - SIP NOTIFY |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For MCPTT px_MCPTT_PublicServiceId_A has been replaced by tsc_MCPTT_PublicServiceId_A; similar changes are required for MCVideo and MCData - MCPTT missing as condition |  |  |  |  |  |  |  |  |
| Summary of change: |  | - px_MCVideo_PublicServiceId_A replaced by tsc_MCVideo_PublicServiceId_A; px_MCData_PublicServiceId_A replaced by tsc_MCData_PublicServiceId_A - MCPTT as condition added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.8	SIP NOTIFY

This message is sent by the SS.

Table 5.5.2.8-1: SIP NOTIFY

| Derivation Path: TS 24.229 [16] clause A.2.1.4.8, A2.2.4.8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "NOTIFY" |  |  |  |
| Request-URI | same URI as the UE has provided earlier in the Contact header of the SUBSCRIBE |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] |  |
| sent-protocol[1] | "SIP/2.0/TCP" |  |  |  |
| sent-by[1] |  |  |  |  |
| host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS |  |  |  |
| via-branch[1] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[2] | "SIP/2.0/UDP" |  |  |  |
| sent-by[2] |  |  |  |  |
| host | “scscf.3gpp.org“ |  |  |  |
| port | not present |  |  |  |
| via-branch[2] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[3] | "SIP/2.0/UDP" |  |  |  |
| sent-by[3] |  |  |  |  |
| host | host name of the MC server |  |  |  |
|  | tsc_MCX_CMS_Hostname |  |  | CONFIG |
|  | tsc_MCX_GMS_Hostname |  |  | GROUPCONFIG |
| port | not present |  |  |  |
| via-branch[3] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same URI as received in the To header of the SUBSCRIBE message | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | same tag as in the To-header of the response which has established the dialog | Remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same URI as received in the From header of the SUBSCRIBE message | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | same value as received in From tag of the SUBSCRIBE message | Local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | same as value received in SUBSCRIBE message |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |  |
| method | "NOTIFY" |  |  |  |
| Contact |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCData |
|  | "sip:" & tsc_MCX_CMS_Hostname |  |  | CONFIG |
|  | "sip:" & tsc_MCX_GMS_Hostname |  |  | GROUPCONFIG |
| port | not present |  |  |  |
| Event |  |  | RFC 6665 [39] RFC 3842 [61] |  |
| event-type | "presence" |  |  | PRESENCE-EVENT |
|  | "xcap-diff" |  |  | CONFIG. GROUPCONFIG |
|  | "poc-settings" |  |  | POC-SETTINGS-EVENT |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "67" | The recommended initial value is 70 in RFC 3261. Assuming 3 hops as according to the Via header this results in a value of 67 in the message sent to the UE |  |  |
| Subscription-State |  |  | RFC 6665 [39] |  |
| substate-value | "active" |  |  |  |
| expires | "7200" |  |  |  |
| Content-Type |  |  | RFC 3261 [22] RFC 3842 [61] |  |
| media-type | “multipart/mixed" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | PIDF |  | PRESENCE-EVENT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/pidf+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the PIDF XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | PIDF as described in Table 5.5.3.5-1 |  | TS 24.379 [9] clause 9.3.1 | MCPTT |
|  | PIDF as described in Table 5.5.3.5-2 |  | TS 24.281 [86] clause 8.3.1 | MCVIDEO |
|  | PIDF as described in Table 5.5.3.5-3 |  | TS 24.282 [87] clause 8.4.1 | MCDATA |
| MIME body part |  | xcap-diff |  | CONFIG, GROUPCONFIG |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/xcap-diff+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the xcap-diff XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | xcap-diff document as described in Table 5.5.3.12-1 |  |  | CONFIG |
|  | xcap-diff document as described in Table 5.5.3.12-2 |  |  | GROUPCONFIG |
| MIME body part |  | PoC-Settings |  | POC-SETTINGS-EVENT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/poc-settings+xml" |  | RFC 4354 [103] |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the PoC-Settings XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | PoC-Settings document as described in Table 5.5.3.11-1 |  |  |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-2 |  | TS 24.379 [9] |  |