3GPP TSG-RAN WG5 Meeting #93-e	R5-216673

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0197 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.1 - SDP Message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Pics pc_MCX_MediaRequestQueueing was introduced to reflect optional queueing support from UE - First-to-answer test case(s) need to force end-to-end security to be started by sending "a=key-mgmt" attribute in SDP answer - Session Name shall be single space when a session has no meaningful name |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Pics pc_MCX_MediaRequestQueueing applied - Condition WITH_SECURITY introduced - Session Name corrected in SDP Message from the SS for MCVideo and MCData |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading message contents may result in incorrect MCPTT test case implementation, correct implemented UE could fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-4 CR 0018 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.1	SDP Message

##### 5.5.3.1.0	Common conditions for SDP Message

The following conditions apply throughout clause 5.5.3.1:

Table 5.5.3.1.0-1: Conditions

| Condition | Explanation |
| --- | --- |
| INITIAL_SDP_OFFER | SDP message is an initial offer |
| SDP_OFFER | SDP message is an offer |
| SDP_ANSWER | SDP message is an Answer |
| FIRST_SDP_FROM_UE | First SDP message sent by the UE within the session |
| FIRST_SDP_FROM_SS | First SDP message sent by the SS within the session |
| IMPLICIT_GRANT_REQUESTED | An implicit grant is requested by the user |
| IMPLICIT_FLOOR_GRANTED | An implicit grant shall be granted by the SS |
| PRE_ESTABLISHED_SESSION | SDP message during establishment or modification of a pre-established session NOTE: The condition shall be applied for all SDP messages of preestablished session test cases and it is not explicitly mentioned in specific message content for these test cases |
| WITHOUT_MEDIACONTROL | SDP message shall not contain a media description for media control (e.g. in case on-demand private call without floor control) |
| WITHOUT_SECURITY | In case of private call: SDP message shall not contain any "a=key-mgmt" attribute for end-to-end security |
| WITH_SECURITY | End-to-end security to be applied independent from other conditions like PRIVATE-CALL, SDP_OFFER (e.g. for first-to-answer call) |

##### 5.5.3.1.1	SDP Message from the UE

##### -	MCPTT

Table 5.5.3.1.1-1: SDP Message from the UE for MCPTT

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | any allowed value |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute | present only if there is no key-mgmt media attribute in the media description for audio | a= line attribute = key-mgmt (NOTE 2) |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Session attribute | optional (NOTE 3) | a=line attribute=”ice-lite” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |
| Media description[1] |  | Media description for audio |  |  |
| media description |  | m= line media = audio | RFC 4867 [59] |  |
| media | "audio" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "RTP/SAVP" |  |  |  |
| fmt | any allowed value(s) | Indicating RTP payload type numbers |  |  |
| media title | "speech" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| bwtype | "RS" |  | RFC 3556 [113] |  |
| bandwidth | any value if present |  |  |  |
| bwtype | "RR" |  | RFC 3556 [113] |  |
| bandwidth | any value if present |  |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | same value as format parameter of the "fmtp" attribute |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | a value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec NOTE: In addition to the parameters below the UE may provide further parameters |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks | RFC 4867 [59] clause 8.2 |  |
| max-red | "0" | No redundancy will be used | RFC 4867 [59] clause 8.2 |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | any allowed value | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | any allowed value | maximum packet time |  |  |
| media attribute | optional | a= line attribute =sendrecv Indicates send and receive mode being activated |  |  |
| sendrecv |  | Parameter has no value |  |  |
| media attribute | one or several attribute lines if present | a=line attribute=ssrc | RFC 5576 [116] |  |
| ssrc |  |  |  |  |
| ssrc-id | any allowed value but all the same if there is more than one ssrc attribute for audio |  |  |  |
| attribute | any source attribute according to RFC 5576 [116] (NOTE 1) |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in speech media's c= line or in the session's c= line if the speech media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for speech |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for RTCP |  |  |
| foundation | any value |  |  |  |
| component-id | 2 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in speech media's c= line or in the session's c= line if the speech media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for speech incremented by 1 |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Media description[2] |  | Media description for media control |  | NOT WITHOUT_MEDIACONTROL |
| media description |  | m= line media = application SDP media-level section for a media-floor control entity (NOTE 2) |  |  |
| media | "application" |  |  |  |
| port | any allowed value | The port for the media-floor control entity |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCPTT" |  |  |  |
| format specific parameters |  |  |  | SDP_OFFER, INITIAL_SDP_OFFER |
| mc_queueing | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  | present | Parameter has no value |  | pc_MCX_MediaRequestQueueing |
| mc_priority | any allowed value | Any integer value in the range of 1..255 | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_granted | not present |  |  |  |
|  | present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 | INITIAL_SDP_OFFER |
| mc_implicit_request | not present |  |  |  |
|  | present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 | IMPLICIT_GRANT_REQUESTED |
| format specific parameters |  |  |  | SDP_ANSWER |
| mc_queueing | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
|  | present | Parameter has no value |  | pc_MCX_MediaRequestQueueing |
| mc_priority | same value as in the offer |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_granted | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_implicit_request | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| mc_ssrc | not present |  | TS 24.380 [10] cl. 12.1.2.3 |  |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for Media Control messages |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "UDP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in application media's c= line or in the session's c= line if the application media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line for application |  |  |  |
| cand-type | "host" |  |  |  |
| NOTE 1: If "ssrc" media attributes are included, then at least one "ssrc=" line shall contain a "cname" source attribute according to RFC 5576 [116] clause 6.1. NOTE 2: Even though there is no clarity in core specs it is assumed that a key-mgmt attribute at session level does not affect the media control security, i.e. the key-mgmt attribute is not applicable for the "application" media description for which still the CSK is used as security key. This is in contrast to RFC 4566 [27] clause 5 saying "In general, session-level values are the default for all media unless overridden by an equivalent media-level value." NOTE 3: If the UE is configured as lite implementation according to RFC 5245 [115], it shall include "a=ice-lite" session-level attribute; nevertheless this is not a test requirement unless specified otherwise in a test case. |  |  |  |  |

##### -	MCVideo

Table 5.5.3.1.1-2: SDP Message from the UE for MCVideo

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | px_MCVideo_User_A_ID | Username of client |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media description[1] |  | Media description for audio |  |  |
| media description |  | m= line media = audio | RFC 4867 [59] |  |
| media | "audio" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "RTP/SAVP" |  |  |  |
| fmt | any allowed value(s) | Indicating RTP payload type numbers |  |  |
| media title | "speech" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| bwtype | "RS" |  | RFC 3556 [113] |  |
| bandwidth | any value if present |  |  |  |
| bwtype | "RR" |  | RFC 3556 [113] |  |
| bandwidth | any value if present |  |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | same value as format parameter of the "fmtp" attribute |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  |  |  |
| format | a value given in fmt in the audio media description |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec NOTE: In addition to the parameters below the UE may provide further parameters |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks | RFC 4867 [59] clause 8.2 |  |
| max-red | "0" | No redundancy will be used | RFC 4867 [59] clause 8.2 |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | any allowed value | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | any allowed value | maximum packet time |  |  |
| media attribute | optional | a= line attribute =sendrecv Indicates send and receive mode being activated |  |  |
| sendrecv |  | Parameter has no value |  |  |
| media attribute |  | a= line attribute = key-mgmt |  | PRIVATE-CALL |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A | Use condition MCVIDEO | RFC 4567 [44] |  |
| Media description[2] |  | Media description for video |  |  |
| media description |  | m= line media = video SDP media-level section for a media-transmission control entity |  |  |
| media | "video" |  |  |  |
| port | any allowed value | The port for the media-transmission control entity |  |  |
| proto | "udp" | User Datagram Protocol. With UDP, computer applications can send messages to other hosts on an Internet Protocol (IP) network. Time-sensitive applications often use UDP because dropping packets is preferable to waiting for packets delayed due to retransmission, which may not be an option in a real-time system. |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| bwtype | "RS" |  | RFC 3556 [113] |  |
| bandwidth | any value if present |  |  |  |
| bwtype | "RR" |  | RFC 3556 [113] |  |
| bandwidth | any value if present |  |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "" |  |  |  |
| encoding name | "H.264" |  |  |  |
| clock rate |  |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "" if present | Channel number |  |  |
| media attribute |  | a= line attribute = key-mgmt |  | PRIVATE-CALL |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A | Use condition MCVIDEO | RFC 4567 [44] |  |
| Media description[3] |  | Media description for media control |  |  |
| media description |  | m= line media = application SDP media-level section for a media-floor control entity |  |  |
| media | "application" |  | 3GPP TS 24.581 [88] clause 12 |  |
| port | any allowed value | The port for the media-floor control entity |  |  |
| proto | "udp" | User Datagram Protocol. With UDP, computer applications can send messages to other hosts on an Internet Protocol (IP) network. Time-sensitive applications often use UDP because dropping packets is preferable to waiting for packets delayed due to retransmission, which may not be an option in a real-time system. |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  | SDP_OFFER, INITIAL_SDP_OFFER |
| mc_queueing | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
|  | present | Parameter has no value. |  | pc_MCX_MediaRequestQueueing |
| mc_priority | not present or any allowed value | Any integer value in the range of 1..255 Shall include the "mc_priority" fmtp attribute when a transmission priority different than the default priority is required. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | not present or any allowed value | Any integer value in the range of 0…255 Shall include the "mc_reception_priority" fmtp attribute when a reception priority different than the default reception priority is required. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | INITIAL_SDP_OFFER |
| mc_implicit_request | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | IMPLICIT_GRANT_REQUESTED |
| format specific parameters |  |  |  | SDP_ANSWER |
| mc_queueing | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
|  | present | Parameter has no value |  | pc_MCX_MediaRequestQueueing |
| mc_priority | same value as in the offer |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority | not present or any allowed value | Any integer value in the range of 0…255 Shall include the "mc_reception_priority" fmtp attribute when a reception priority different than the default reception priority is required. | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_implicit_request | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |

##### -	MCData

Table 5.5.3.1.1-3: SDP Message from the UE for MCData

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin |  | o= line |  |  |
| username | px_MCDATA_ID_User_A | Username of client |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Session Information | any allowed value | i=<session description> The "i=" field is intended to provide a free-form human-readable description of the session or the purpose of a media stream. It is not suitable for parsing by automata. |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS:" | bwtype:bandwidth |  |  |
| bandwidth | any allowed value |  | TS 26.114 [64] Table K.6 |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = message | RFC 4867 [59] TS 24.282 [31] |  |
| media | "message" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "TCP/MSRP " |  |  |  |
| fmt | “*” |  |  |  |
| media title | "message" | i= line |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = sendonly |  |  |
| sendonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = path |  |  |
| path | px_MSRP_URI_A_ID | attribute containing its own MSRP URI. An example: msrp://mcdata.example.com:7654/abcde1; tcp | TS 24.282 [31] |  |
| media attribute |  | a= line attribute = accept-types |  |  |
| accept-types | "application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload" |  |  |  |
| media attribute |  | a= line attribute = setup |  |  |
| role | “actpass” |  |  |  |
| media attribute |  | a= line attribute = key-mgmt |  | MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A | Use condition MCDATA | RFC 4567 [44] |  |

##### 5.5.3.1.2	SDP Message from the SS

##### -	MCPTT

Table 5.5.3.1.2-1: SDP Message from the SS for MCPTT

| Derivation Path: RFC 4566 [27] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| Session description: |  |  |  |  |  |  |  |  |  |
| Protocol Version |  | "0" |  | v= line |  |  |  |  |  |
| Origin |  | Same o=line as in the previous SDP message sent by the SS except that sess-version is incremented by one |  | o= line |  |  |  |  |  |
| Origin |  |  |  | o= line |  |  |  | FIRST_SDP_FROM_SS |  |
| username |  | "-" |  | "-" indicating the concept of user IDs not being supported |  |  |  |  |  |
| sess-id |  | "12345678" |  | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |  |  |  |
| sess-version |  | "12345678" |  |  |  |  |  |  |  |
| nettype |  | "IN" |  |  |  |  |  |  |  |
| Addrtype |  | "IP4" or "IP6" depending on IP address" |  | This depends on the unicast address of the UE |  |  |  |  |  |
| unicast-address |  | IP address of the SS |  |  |  |  |  |  |  |
| Session Name |  | "" |  | s= line single empty space indicating no session name |  |  |  |  |  |
| Bandwidth |  |  |  | b= line |  |  |  |  |  |
| bwtype |  | "AS" |  | bwtype:bandwidth |  |  |  |  |  |
| bandwidth |  | "38" |  | kilobits per second; Maximum AMR-WB at 23.85 kbps but limit to 12.65 kbps plus overhead |  | TS 26.114 [64] Table K.6 |  |  |  |
| Time description |  |  |  |  |  |  |  |  |  |
| Timing |  |  |  | t= line |  |  |  |  |  |
| start-time |  | "0" |  |  |  |  |  |  |  |
| stop-time |  | "0" |  |  |  |  |  |  |  |
| Session attribute |  |  |  | a=line attribute=”ice-lite” |  | RFC 5245 [115] |  | PRE_ESTABLISHED_SESSION |  |
| ice-lite |  |  |  |  |  |  |  |  |  |
| Media description[1] |  |  |  | Media description for audio |  |  |  |  |  |
| media description |  |  |  | m= line media = audio |  | RFC 4867 [59] |  |  |  |
| media |  | "audio" |  |  |  |  |  |  |  |
| port |  | port number assigned by the SS (even integer) |  | The transport port to which the media stream is sent |  | RFC 6335 [63] clause 6 |  |  |  |
| proto |  | "RTP/SAVP" |  |  |  |  |  |  |  |
| fmt |  | "99" |  | RTP/SAVP payload type for AMR-WB is dynamic |  |  |  | INITIAL_SDP_OFFER |  |
|  |  | value for AMR-WB as used in initial offer |  |  |  |  |  |  |  |
| media title |  | "speech" |  | i= line |  |  |  |  |  |
| Connection Data |  |  |  | c= line |  |  |  |  |  |
| nettype |  | "IN" |  |  |  |  |  |  |  |
| Addrtype |  | "IP4" or "IP6" depending on IP address" |  | This depends on the connection address |  |  |  |  |  |
| connection-address |  | IP address of the SS |  |  |  |  |  |  |  |
| Bandwidth |  |  |  | b= line |  |  |  |  |  |
| bwtype |  | "AS" |  | bwtype:bandwidth |  |  |  |  |  |
| bandwidth |  | 38 |  |  |  | TS 26.114 [64] Table K.6 |  |  |  |
| bwtype |  | "RS" |  |  |  | RFC 3556 [113] |  |  |  |
| bandwidth |  | 0 |  |  |  |  |  |  |  |
| bwtype |  | "RR" |  |  |  | RFC 3556 [113] |  |  |  |
| bandwidth |  | 2000 |  |  |  |  |  |  |  |
| media attribute |  |  |  | a= line attribute = rtpmap |  |  |  |  |  |
| rtpmap |  | "rtpmap" |  |  |  |  |  |  |  |
| payload type |  | "99" |  |  |  |  |  | INITIAL_SDP_OFFER |  |
|  |  | value for AMR-WB as used in initial offer |  |  |  |  |  |  |  |
| encoding name |  | "AMR-WB" |  |  |  |  |  |  |  |
| clock rate |  | 16000 |  |  |  | RFC 4867 [59] clause 8.3 |  |  |  |
| encoding parameter |  | "1" |  | Channel number |  |  |  |  |  |
| media attribute |  |  |  | a= line attribute = fmtp |  |  |  |  |  |
| fmtp |  |  |  |  |  |  |  |  |  |
| format |  | "99" |  |  |  |  |  | INITIAL_SDP_OFFER |  |
|  |  | value for AMR-WB as used in initial offer |  |  |  |  |  |  |  |
| format specific parameters |  |  |  | Parameters of WB-AMR codec |  |  |  |  |  |
| mode-change-capability |  | "2" |  | To be able to interoperate fully with gateways to circuit switched networks |  | RFC 4867 [59] clause 8.2 |  |  |  |
| max-red |  | "0" |  | No redundancy will be used |  | RFC 4867 [59] clause 8.2 |  |  |  |
| media attribute |  |  |  | a= line attribute =ptime |  |  |  |  |  |
| ptime |  | "20" |  | packet time |  |  |  |  |  |
| media attribute |  |  |  | a= line attribute =maxptime |  |  |  |  |  |
| maxptime |  | "240" |  | maximum packet time |  |  |  |  |  |
| media attribute |  |  |  | a= line attribute = key-mgmt |  |  |  | WITH_SECURITY OR (PRIVATE-CALL AND SDP_OFFER AND NOT WITHOUT_SECURITY) |  |
| key-mgmt |  |  |  |  |  | TS 24.379 [9] clause 6.2.1 |  |  |  |
| mikey |  | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 |  |  |  | RFC 4567 [44] |  |  |  |
| media attribute |  |  |  | a=line attribute=”candidate” |  | RFC 5245 [115] |  | PRE_ESTABLISHED_SESSION |  |
| candidate |  |  |  | candidate for RTP |  |  |  |  |  |
| foundation |  | 1234 |  | arbitrarily selected |  |  |  |  |  |
| component-id |  | 1 |  | according to RFC 5245 [115] clause 4.1.1.1 |  |  |  |  |  |
| transport |  | "UDP" |  |  |  |  |  |  |  |
| priority |  | 2130706431 |  | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |  |  |  |
| connection-address |  | IP address of the SS (same IP address as in the c=line for speech) |  | default candidate |  |  |  |  |  |
| port |  | same port number as in the m= line for speech |  |  |  |  |  |  |  |
| cand-type |  | "host" |  |  |  |  |  |  |  |
| media attribute |  |  |  | a=line attribute=”candidate” |  | RFC 5245 [115] |  | PRE_ESTABLISHED_SESSION |  |
| candidate |  |  |  | candidate for RTCP |  |  |  |  |  |
| foundation |  | 1234 |  | same as for RTP |  |  |  |  |  |
| component-id |  | 2 |  | according to RFC 5245 [115] clause 4.1.1.1 |  |  |  |  |  |
| transport |  | "UDP" |  |  |  |  |  |  |  |
| priority |  | 2130706430 |  | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |  |  |  |
| connection-address |  | IP address of the SS (same IP address as in the c=line for speech) |  | default candidate |  |  |  |  |  |
| port |  | same port number as in the m= line for speech incremented by 1 |  |  |  |  |  |  |  |
| cand-type |  | "host" |  |  |  |  |  |  |  |
| Media description[2] |  |  |  | Media description for media control |  |  |  | NOT WITHOUT_MEDIACONTROL |  |
| media description |  |  |  | m= line media = application SDP media-level section for a media-floor control entity |  |  |  |  |  |
| media |  | "application" |  |  |  |  |  |  |  |
| port |  | port number assigned by the SS being different than the port number of the audio channel (RTP) and its associated control channel (RTCP)" |  | The port for the media-floor control entity |  |  |  |  |  |
| proto |  | "udp" |  |  |  |  |  |  |  |
| fmt |  | "MCPTT" |  |  |  |  |  |  |  |
| Connection Data |  |  |  | c= line |  |  |  |  |  |
| nettype |  | "IN" |  |  |  |  |  |  |  |
| Addrtype |  | "IP4" or "IP6" depending on IP address |  | This depends on the connection address |  |  |  |  |  |
| connection-address |  | IP address of the SS |  |  |  |  |  |  |  |
| media attribute |  |  |  | a= line attribute = fmtp |  |  |  |  |  |
| fmtp |  |  |  |  |  |  |  |  |  |
| format |  | "MCPTT" |  |  |  |  |  |  |  |
| format specific parameters |  |  |  |  |  |  |  | SDP_OFFER |  |
| mc_queueing |  | Present |  | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 |  |  |  |
| mc_priority |  | "3" |  | "3" is the value of the <user-priority> element for user A in the MCPTT Group Configuration (Table 5.5.7.1-1) |  | TS 24.380 [10] cl. 12.1.2.3 and cl. 143.3 |  |  |  |
| mc_granted |  | not present |  |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |  |  |
| mc_implicit_request |  | not present |  |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |  |  |
| format specific parameters |  |  |  |  |  |  |  | SDP_ANSWER |  |
| mc_queueing |  | present if included in the offer |  | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 |  |  |  |
| mc_priority |  | if a value is provided in the offer: "3" or the value provided in the offer, whichever is the lower value; otherwise not present |  | "3" is the value of the <user-priority> element for user A in the MCPTT Group Configuration (Table 5.5.7.1-1) NOTE: <num-levels-priority-hierarchy> has a value of 10 for on-network i.e. it is greater than 3 |  | TS 24.380 [10] cl. 12.1.2.3 and cl. 14.3.3 |  |  |  |
| mc_granted |  | not present |  |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |  |  |
|  |  | present |  | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 |  | IMPLICIT_FLOOR_GRANTED |  |
| mc_implicit_request |  | not present |  |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |  |  |
|  |  | present |  | Parameter has no value |  | TS 24.380 [10] cl. 12.1.2.3 |  | IMPLICIT_GRANT_REQUESTED |  |
| mc_ssrc |  | not present |  |  |  | TS 24.380 [10] cl. 12.1.2.3 |  |  |  |
|  |  | same value as in the offer if provided in the offer and there is no collision with the value used by the SS; otherwise value assigned by the SS |  |  |  | TS 24.380 [10] cl. 12.1.2.3 |  | IMPLICIT_GRANT_REQUESTED |  |
| media attribute |  |  |  | a=line attribute=”candidate” |  | RFC 5245 [115] |  | PRE_ESTABLISHED_SESSION |  |
| candidate |  |  |  | candidate for Media Control messages |  |  |  |  |  |
| foundation |  | 4321 |  | arbitrarily selected; different than for RTP/RTCP |  |  |  |  |  |
| component-id |  | 1 |  | according to RFC 5245 [115] clause 4.1.1.1 |  |  |  |  |  |
| transport |  | "UDP" |  |  |  |  |  |  |  |
| priority |  | 2130706431 |  | RFC 5245 [115] clause 4.2: 224 * 126 + 28 * 65535 + 256 - component id |  |  |  |  |  |
| connection-address |  | IP address of the SS (same IP address as in the c=line for media control) |  | default candidate |  |  |  |  |  |
| port |  | same port number as in the m= line for application |  |  |  |  |  |  |  |
| cand-type |  | "host" |  |  |  |  |  |  |  |

##### -	MCVideo

Table 5.5.3.1.2-2: SDP Message from the SS for MCVideo

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the SS except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_SS |
| username | px_MCVideo_ID_User_B | Username of client sending message |  |  |
| sess-id | "12345678" | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | "12345678" |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address | This depends on the unicast address of the UE |  |  |
| unicast-address | IP address of the SS |  |  |  |
| Session Name | " " | s= line |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS:" | bwtype:bandwidth |  |  |
| bandwidth | "38" | kilobits per second; Maximum AMR-WB at 23.85 kbps but limit to 12.65 kbps plus overhead | TS 26.114 [64] Table K.6 |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media description[1] |  | Media description for audio |  |  |
| media description |  | m= line media = audio | RFC 4867 [59] |  |
| media | "audio" |  |  |  |
| port | port number assigned by the SS (even integer) | The transport port to which the media stream is sent | RFC 6335 [63] clause 6 |  |
| proto | "RTP/AVP" |  |  |  |
| fmt | "99" | RTP/AVP payload type for AMR-WB is dynamic |  |  |
| media title | "speech" | i= line |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address | This depends on the connection address |  |  |
| connection-address | IP address of the SS |  |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "99" |  |  |  |
| encoding name | "AMR-WB" |  |  |  |
| clock rate | 16000 |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "1" if present | Channel number |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "99" |  |  |  |
| format specific parameters |  | Parameters of WB-AMR codec |  |  |
| mode-change-capability | "2" | To be able to interoperate fully with gateways to circuit switched networks | RFC 4867 [59] clause 8.2 |  |
| max-red | "0" | No redundancy will be used | RFC 4867 [59] clause 8.2 |  |
| media attribute |  | a= line attribute =ptime |  |  |
| ptime | "20" | packet time |  |  |
| media attribute |  | a= line attribute =maxptime |  |  |
| maxptime | "240" | maximum packet time |  |  |
| media attribute |  | a= line attribute = key-mgmt |  | PRIVATE-CALL |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 | Use condition MCVIDEO | RFC 4567 [44] |  |
| Media description[2] |  | Media description for video |  |  |
| media description |  | m= line media = video SDP media-level section for a media-transmission control entity |  |  |
| media | "video" |  |  |  |
| port | port number of the audio stream incremented by 2 (resulting in even integer) | The port for the media-transmission control entity |  |  |
| proto | "udp" | User Datagram Protocol. With UDP, computer applications can send messages to other hosts on an Internet Protocol (IP) network. Time-sensitive applications often use UDP because dropping packets is preferable to waiting for packets delayed due to retransmission, which may not be an option in a real-time system. |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data |  | c= line Included if the media plane control channel uses a different IP address than other media described in the SDP |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the SS |  |  |  |
| media attribute |  | a= line attribute = rtpmap |  |  |
| rtpmap | "rtpmap" |  |  |  |
| payload type | "" |  |  |  |
| encoding name | "H.264" |  |  |  |
| clock rate |  |  | RFC 4867 [59] clause 8.3 |  |
| encoding parameter | "" if present | Channel number |  |  |
| media attribute |  | a= line attribute = key-mgmt |  | PRIVATE-CALL |
| key-mgmt |  |  | TS 24.281 [86] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2 | Use condition MCVIDEO | RFC 4567 [44] |  |
| Media description[3] |  | Media description for media control |  |  |
| media description |  | m= line media = application SDP media-level section for a media-floor control entity |  |  |
| media | "application" |  |  |  |
| port | port number assigned by the SS being different than the port number of the audio and video channels (RTP) and their associated control channels (RTCP)" | The port for the media-floor control entity |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCVideo" |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address | This depends on the connection address |  |  |
| connection-address | IP address of the SS |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCVideo" |  |  |  |
| format specific parameters |  |  |  | SDP_OFFER |
| mc_queueing | Present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_priority | "5" | Any integer value in the range of 1..255 | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_implicit_request | Present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_reception_priority" | not present |  | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| format specific parameters |  |  |  | SDP_ANSWER |
| mc_queueing | Present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_priority | "5" | Any integer value in the range of 1..255 | 3GPP TS 24.581 [88] clause 12, clause 14 |  |
| mc_granted | not present |  |  |  |
|  | present | Parameter has no value | 3GPP TS 24.581 [88] clause 12, clause 14 | IMPLICIT_FLOOR_GRANTED |
| mc_implicit_request | Present | Parameter has no value |  |  |
|  |  |  | 3GPP TS 24.581 [88] clause 12, clause 14 | IMPLICIT_GRANT_REQUESTED |
| mc_reception_priority" | not present or "10" | No present if not present in the SDP offer. If present in the SDP offer, then the arbitrarily chosen value of "10 should be used | 3GPP TS 24.581 [88] clause 12, clause 14 |  |

##### -	MCData

Table 5.5.3.1.2-3: SDP Message from the SS for MCData

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin |  | o= line |  |  |
| username | px_MCDATA_ID_User_B | Username of client |  |  |
| sess-id | "12345678" | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | "12345678" |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| unicast-address | IP address of the SS |  |  |  |
| Session Name | " " | s= line |  |  |
| Session Information | “message” | i=<session description> The "i=" field is intended to provide a free-form human-readable description of the session or the purpose of a media stream. It is not suitable for parsing by automata. |  |  |
| Connection Data | not required if included in all media | c= line Included if the media plane control channel uses a different IP address than other media described in the SDP |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the SS |  |  |  |
| Bandwidth |  | b= line |  |  |
| bwtype | "AS:" | bwtype:bandwidth |  |  |
| bandwidth | "38" | kilobits per second; Maximum AMR-WB at 23.85 kbps but limit to 12.65 kbps plus overhead | TS 26.114 [64] Table K.6 |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = message | RFC 4867 [59] TS 24.282 [31] |  |
| media | "message" |  |  |  |
| port | "49152" | The transport port to which the media stream is sent |  |  |
| proto | "TCP/MSRP " |  |  |  |
| fmt | “*” |  |  |  |
| media title | "message" | i= line |  |  |
| Connection Data |  | c= line Included if the media plane for audio uses a different IP address than other media described in the SDP |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address |  |  |  |
| connection-address | IP address of the SS |  |  |  |
| media attribute |  | a= line attribute = recvonly |  |  |
| recvonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = path |  |  |
| path | px_MSRP_URI_SS_ID | attribute containing its own MSRP URI. An example: msrp://mcdata.example.com:7654/abcde1; tcp | TS 24.282 [31] |  |
| media attribute |  | a= line attribute = accept-types |  |  |
| accept-types | "application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload" |  |  |  |
| media attribute |  | a= line attribute = setup |  |  |
| role | “actpass” |  |  |  |
| media attribute |  | a= line attribute = key-mgmt |  | MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A | Use condition MCDATA | RFC 4567 [44] |  |