3GPP TSG-RAN5 Meeting #92-e	R5-216777

# Online, , 8th Nov - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0258 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IMS5GS test case 7.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current wording of test case 7.24 has a design flaw in that SS would cancel an MO call initiation. This is not possible due to the nature of CANCEL – only the UA that sent the INVITE can cancel it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reworded the test case such that we test that the UE correctly responds to cancellation of an MT call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 7.24	MTSI MT Voice Call / Forking / UE receives CANCEL request for a forked MT voice call / 5GS

7.24.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions and having received INVITE for voice call and having proceeded the call initiation until before accepting the voice call }

ensure that {

when { UE receives a CANCEL request }

then { UE sends 200 OK for CANCEL and 487 Request Terminated for INVITE }

}

7.24.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.4.1]:

If the UE sends a CANCEL request to cancel an initial INVITE request, the UE shall when applicable include in the CANCEL request a Reason header field with a protocol value set to "RELEASE_CAUSE" and a "cause" header field parameter as specified in subclause 7.2A.18.11.2. The UE may also include the "text" header field parameter with reason-text as specified in subclause 7.2A.18.11.2.

7.24.3	Profile Requirements (Informative)

[GSMA NG.114 V1.0, clause 2.3.7]:

The IMS core network can support sending and the UE must support receiving a SIP CANCEL request including a Reason header field with values of:

- SIP; cause=200; text="Call completed elsewhere"

- SIP; cause=603; text="Declined"

- SIP; cause=600; text=” Busy Everywhere”

for forked calls as defined in 3GPP TS 24.229 [8].

7.24.4	Test description

7.24.4.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use preconditions.

Preamble:

-	UE is in state 1N-A and registered to IMS

7.24.4.2	Test procedure sequence

Table 7.24.4.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed | - | - |  |  |
| 8-16 | MT voice call setup is initiated and proceeds until before UE is made to accept the voice call (Steps 1-8 of Annex A.5.1) | - | - |  |  |
| 17 | SS sends a CANCEL request | <-- | CANCEL |  |  |
| 18 | UE sends 200 OK for CANCEL | --> | 200 OK | 1 | P |
| 19 | UE sends 487 Request Terminated response for INVITE Note: steps 18 and 19 can happen in any order | --> | 487 Request Terminated | 1 | P |
| 20 | SS sends ACK | <-- | ACK |  |  |

7.24.4.3	Specific message contents

Table 7.24.4.3-1: CANCEL (step 17, table 7.24.4.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.15

Table 7.24.4.3-2: 200 OK (step 18, table 7.24.4.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.3.1

Table 7.24.4.3-3: 487 Request Terminated (step 19, table 7.24.4.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.16

Table 7.24.4.3-4: ACK (step 20, table 7.24.4.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.7