3GPP TSG-RAN5 Meeting #93-e	R5-216802

# Electronic Meeting, 8th Nov 2021 – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5062 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 7.3.5.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To update test case 7.3.5.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following have been updated Title - The title has been updated Conformance requirements - Requirements has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### 7.3.5.6	PDCP handover / DAPS handover with key change / Status reporting / Intra-Frequency

7.3.5.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for DAPS handover with key change }

ensure that {

when { upper layers request uplink data switching during DAPS handover }

then { UE shall send a PDCP status report for the DAPS bearer }

}

7.3.5.6.2	Conformance requirements

[TS 36.323, clause 5.3.1]

When upper layers request a PDCP re-establishment or PDCP Data Recovery; or when PDCP status report is triggered by polling or periodic reporting; or when PDCP status report is triggered by WLAN Connection Status Reporting of temporary unavailability (suspended, see TS 36.331 [3]); or when upper layers request uplink data switching during DAPS handover, or when upper layers reconfigure the PDCP entity to release DAPS and daps-SourceRelease is configured in TS 36.331 [3], for radio bearers that are mapped on RLC AM, or when upper layers request uplink data switching during DAPS handover for radio bearers that are mapped on RLC UM, the UE shall:

-	if the radio bearer is configured by upper layers to send a PDCP status report in the uplink (statusReportRequired, see TS 36.331 [3]) or the status report is triggered by PDCP status report polling or PDCP periodic status reporting or the status report is triggered by WLAN Connection Status Reporting of temporary unavailability (suspended, see TS 36.331 [3]) when wlan-SuspendTriggersStatusReport is configured, see TS 36.331 [3], compile a status report as indicated below after processing the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers as specified in the clause 5.2.2.1, and submit it to lower layers as the first PDCP PDU for the transmission, by:

-	setting the FMS field to the PDCP SN of the first missing PDCP SDU;

-	if there is at least one out-of-sequence PDCP SDU stored, allocating a Bitmap field of length in bits equal to the number of PDCP SNs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 8188 bytes, whichever comes first;

-	setting as '0' in the corresponding position in the bitmap field for all PDCP SDUs that have not been received as indicated by lower layers, and optionally PDCP SDUs for which decompression have failed;

-	indicating in the bitmap field as '1' for all other PDCP SDUs.

7.3.5.6.3	Test description

7.3.5.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

7.3.5.6.3.2	Test procedure sequence

Table 7.3.5.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message including mobilityControlInfo containing rach-ConfigDedicated to order UE to perform non-contention based DAPS handover to Cell 2. DRB #1 is configured as DAPS bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does UE transmit an RRCConnectionReconfigurationComplete message in Cell 2? | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMS field = 0. | --> | PDCP status report | 1 | P |
| 4 | The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMS field = 0. | <-- | PDCP status report | - | - |

7.3.5.6.3.3	Specific message contents

Table 7.3.5.6.3.3-1: RRCConnectionReconfiguration (step 1, Table 7.3.5.6.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.5.6.3.3-2: MobilityControlInfo (Table 7.3.5.6.3.3-1)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Rach-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

Table 7.3.5.6.3.3-3: RadioResourceConfigCommon-DEFAULT (Table 7.3.5.6.3.3-2)

| Derivation Path: 36.508, Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |

Table 7.3.5.6.3.3-4: RadioResourceConfigDedicated (Table 7.3.5.6.3.3-1)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated::= SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList SEQUENCE { |  | 1 entries |  |  |
| drb-ToAddMod |  | DRB-ToAddMod-DEFAULT |  |  |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig |  | Not present |  |  |
| mac-MainConfig |  | Not Present |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.5.6.3.3-5: DRB-ToAddMod-DEFAULT (Table 7.3.5.6.3.3-4)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity |  | Not Present |  |  |
| drb-Identity |  | 1 |  |  |
| pdcp-Config |  | Not Present |  |  |
| rlc-Config |  | Not Present |  |  |
| logicalChannelIdentity |  | Not Present |  |  |
| logicalChannelConfig |  | Not Present |  |  |
| daps-HO-r16 |  | TRUE |  |  |
| } |  |  |  |  |

<End of modified section>