3GPP TSG-RAN5 Meeting #93-e	R5-217148

# Electronic Meeting, 8 – 19 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.905 | CR | 0245 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test point analysis for cat M2 A-MPR with subPRB allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | subPRB allocation is missing in cat M2 A-MPR test case. Test point analysis for NS_05 is therefore needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test point analysis for cat M2 A-MPR test case, NS_05 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | efeMTC work item cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

## 4.1	Test point selection in Additional Maximum Power Reduction (A-MPR) test cases

When deriving test points for these test cases the calculation of maximum output power backoff and determination of possible worst cases for out-of-band emissions are non-trivial and therefore require an analysis which is documented here.

### 4.1.1	A-MPR test case for single carrier

This section contains information on test point selection for test case 6.2.4 in [2], Additional Maximum Power Reduction (A-MPR).

Test points in this test were added in the past, and no selection information is therefore available prior the NS values in table 4.1.1-1.

Selection of test points should include some possible worst combinations based on the A-MPR characteristics specified for each NS value and these shall be selected so that they match with corresponding spectrum emission requirements test points. The number of test points should be realistic.

Table 4.1.1-1: NS value specific test points for A-MPR

| NS value | Justification | Comments |
| --- | --- | --- |
| NS_05 | See attachment “TpAnalysisAMPR(NS_05)_6.2.4_v2.zip” | Added at RAN5#73 |
| NS_24 | See attachment “TpAnalysisAMPR_6.2.4(NS_24 and NS_25).zip” | Added at RAN5#72 |
| NS_25 | See attachment “TpAnalysisAMPR_6.2.4(NS_24 and NS_25).zip” | Added at RAN5#72 |
| NS_27 | See attachment “TpAnalysisAMPR_6.2.4(NS_27)_v3.zip” | Added at RAN5#80 |
| NS_04 (Power Class 2) | See attachment “TpAnalysisAMPR_6.2.4_1(NS_04_PC2).zip” | Added at RAN5#74 |
| NS_04 (Power Class 3) | See attachment “TpAnalysisAMPR_6.2.4(NS_04_PC3).zip” | Added at RAN5#75 |
| NS_22 | See attachment “TpAnalysisAMPR_6.2.4(NS_22 and NS_23).zip” | Added at RAN5#75 |
| NS_23 | See attachment “TpAnalysisAMPR_6.2.4(NS_22 and NS_23).zip” | Added at RAN5#75 |

The analyses for UE category M1 and category M2 are performed per NS-value in table 4.1.1-2 for category M1, table 4.1.1-2a for category M1 with subPRB allocation and in table 4.1.1-3 for category M2. The general principles for selection of test points are:

-	For Additional spurious emissions, requirements are the same for all bandwidths. Since channel bandwidth is not judged to have any impact on the UE transmit signal for category M1, only one bandwidth needs to be tested. The lowest bandwidth with emissions requirement has lowest guard band and is therefore the most stringent. For category M2 the same applies, with the exception that channel bandwidth 5 MHz is always additionally selected, even if this is not the smallest bandwidth. Because maximum RB allocation of 24 resource blocks requires at least 5 MHz channel bandwidth.

-	For NS-values with Additional SEM requirement and without any allowed A-MPR, same test points as in SEM test case 6.6.2.1EA for category M1 and same test points as in SEM test case 6.6.2.1EC for category M2 can be used.

-	For NS-values with Additional Spurious requirement and without any allowed A-MPR, same test points as in spurious test case 6.6.3EA.1 for category M1 and same test points as in spurious test case 6.6.3EC.1 for category M2 can be used

-	For Additional SEM, test frequency is selected as in SEM test case.

-	For Additional spurious emissions, test frequency selection depends on if protected range is above or below the UE transmit signal in frequency. If protected range is above UE transmit frequency, then High frequency range is selected. If protected range is below UE transmit frequency, then Low frequency range is selected.

Table 4.1.1-2: NS value specific test points for A-MPR UE category M1

| NS value | Operating band | Applicable test case | RB allocation and modulation | Bandwidth | Test frequency | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| NS_03 | 2, 4 | 6.6.2.2EA | SEM (Note 1) | Lowest, 5MHz, 10MHz, Highest | Low, Mid, High |  |
| NS_04 | 41 | 6.6.2.2EA | TBD |  |  |  |
| NS_05 | 1 | 6.6.3EA.3 | Spur (Note 2) | 5 MHz | Low |  |
| NS_06 | 12, 13 | 6.6.2.2EA | SEM (Note 1) | Lowest, 5MHz, 10MHz, Highest | Low, Mid, High |  |
| NS_07 | 13 | 6.6.2.2EA | See attachment “TpAnalysisAMPR(NS_07)_6.2.4EA.zip” | 10MHz | Low, Mid, High | Testing only 10 MHz to align with legacy LTE test case |
|  |  | 6.6.3EA.3 |  | 10 MHz | Low | Emission requirements are only defined for 10 MHz channel bandwidth |
| NS_08 | 19 | 6.6.3EA.3 | Spur (Note 2) | 5 MHz | High |  |
| NS_09 | 21 | 6.6.3EA.3 | Spur (Note 2) | 5 MHz | High |  |
| NS_10 | 20 | N/A |  |  |  | Not tested due to missing requirements |
| NS_12 | 26 | 6.6.3EA.3 | TBD |  |  |  |
| NS_13 | 26 | 6.6.3EA.3 | Spur (Note 2) | 1.4 MHz | Low (Note 3) |  |
| NS_14 | 26 | 6.6.3EA.3 | Spur (Note 2) | 10 MHz | Low (Note 3) |  |
| NS_15 | 26 | 6.6.3EA.3 | See attachment “TpAnalysisAMPR(NS_15)_6.2.4EA.zip” | 1.4 MHz | High, High – 4 MHz |  |
| NS_16 | 27 | 6.6.3EA.3 | Spur (Note 2) | 1.4 MHz | Low |  |
| NS_17 | 28 | 6.6.3EA.3 | Spur (Note 2) | 5 MHz | Low (Note 3) |  |
| NS_18 | 28 | 6.6.3EA.3 | Spur (Note 2) | 5 MHz | Low |  |
| Note 1: No A-MPR allowed, same test points as SEM test case 6.6.2.1EA can be used for Additional SEM Note 2: No A-MPR allowed, same test points as Spurious test case 6.6.3EA.1 can be used for Additional Spurious Note 3: Protected range below UE transmit signal, and restricted carrier frequency. Use lowest allowed frequency |  |  |  |  |  |  |

Table 4.1.1-2a: NS value specific test points for A-MPR UE category M1, subPRB allocation

| NS value | Operating band | Applicable test case | RB allocation and modulation | Bandwidth | Test frequency | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| NS_07 | 13 | 6.6.2.2EA | See attachment “TpAnalysisAMPR(NS_07)_6.2.4EA_subPRB.zip” | 10MHz | Low, Mid, High | A-MPR requirement is only defined for 10 MHz channel bandwidth. |
|  |  | 6.6.3EA.3 | Spur (Note 1) | 10 MHz | Low | Emission requirement is only defined for 10 MHz channel bandwidth |
| Note 1: Protected range is below UE transmit signal, and restricted carrier frequency. Use lowest allowed frequency |  |  |  |  |  |  |

Table 4.1.1-3: NS value specific test points for A-MPR UE category M2

| NS value | Operating band | Applicable test case | RB allocation and modulation | Bandwidth | Test frequency | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| NS_03 | 2, 4 | 6.6.2.2EC | TBD |  |  |  |
| NS_04 | 41 | 6.6.2.2EC | TBD |  |  |  |
| NS_05 | 1 | 6.6.3EC.3 | See attachment “TpAnalysisAMPR(NS_05)_6.2.4EC.zip” | 5 MHz, 20 MHz | Low |  |
| NS_06 | 12, 13, 14, 85 | 6.6.2.2EC | SEM (Note 1) | Lowest, 5 MHz, 10 MHz, Highest | Low, Mid, High |  |
| NS_07 | 13 | 6.6.2.2EC | TBD |  |  |  |
|  |  | 6.6.3EC.3 | TBD |  |  |  |
| NS_08 | 19 | 6.6.3EC.3 | Spur (Note 2) | 5 MHz | High |  |
| NS_09 | 21 | 6.6.3EC.3 | Spur (Note 2) | 5 MHz | High |  |
| NS_10 | 20 | N/A |  |  |  | Not tested due to missing requirements |
| NS_12 | 26 | 6.6.3EC.3 | TBD |  |  |  |
| NS_13 | 26 | 6.6.3EC.3 | TBD |  |  |  |
| NS_14 | 26 | 6.6.3EC.3 | TBD |  |  |  |
| NS_15 | 26 | 6.6.3EC.3 | TBD |  |  |  |
| NS_16 | 27 | 6.6.3EC.3 | TBD |  |  |  |
| NS_17 | 28 | 6.6.3EC.3 | TBD |  |  |  |
| NS_18 | 28 | 6.6.3EC.3 | TBD |  |  |  |
| NS_35 | 71 | 6.6.2.2EC | SEM (Note 1) | Lowest, 5 MHz, 10 MHz, Highest | Low, Mid, High |  |
| Note 1: No A-MPR allowed, same test points as SEM test case 6.6.2.1EC can be used for Additional SEM Note 2: No A-MPR allowed, same test points as Spurious test case 6.6.3EC.1 can be used for Additional Spurious |  |  |  |  |  |  |

Table 4.1.1-3a: NS value specific test points for A-MPR UE category M2, subPRB allocation

| NS value | Operating band | Applicable test case | RB allocation and modulation | Bandwidth | Test frequency | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| NS_05 | 1 | 6.2.4EC | See attachment “TpAnalysisAMPR(NS_05)_6.2.4EC_subPRB.zip” | 5MHz, 10MHz, 15MHz, 20MHz | Low | RAN5#93-e |
|  |  | 6.6.3EC.3 | Spur (Note 1) | 5MHz, 10MHz, 15MHz, 20MHz | Low | RAN5#93-e |
| Note 1: Protected range is below UE transmit signal, and restricted carrier frequency. Use lowest allowed frequency |  |  |  |  |  |  |

<< End of changes>>

Add the following attached .zip file to TR 36.905:

TpAnalysisAMPR(NS_05)_6.2.4EC_subPRB.zip