3GPP TSG-RAN5 Meeting #93-e	R5-217868

# Electronic Meeting, 8th November 2021 – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5063 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 7.1.3.23 Enhanced Coverage / DL Fexible starting PRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add a new test case, 7.1.3.23 Enhanced Coverage / DL Fexible starting PRB, according to the Work Plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new test case, 7.1.3.23 Enhanced Coverage / DL Fexible starting PRB, according to the Work Plan |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Work Item will not progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.23 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-217450 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

Table 7.1.3.22.3.3-2: PhysicalConfigDedicated-DEFAULT (preamble: Table 4.5.3.3-1, step 8)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 ::= SEQUENCE{ |  |  |  |  |
| pucch-ConfigDedicated-v1530 SEQUENCE { |  |  |  |  |
| n1PUCCH-AN-SPT-r15 |  | 2 |  |  |
| codebooksizeDeterminationSTTI-r15 |  | Not present |  |  |
| } |  |  |  |  |
| shortProcessingTime-r15 |  | TRUE |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 7.1.3.23	Enhanced Coverage / DL Fexible starting PRB

7.1.3.23.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE on PDCCH receives DCI format 6-1A indicating Resource Allocation Type 2 with  resource allocation field consiststing of values of  and  }

then { UE decodes the received transport block of size correspondent to the read  and  and forwards it to higher layers  }

}

7.1.3.23.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.312, clause 7.1.6.3 & 7.1.6.5.

[TS 36.213, clause 7.1.6.3]

For a BL/CE UE configured with CEModeA, and configured with higher layer parameter ce-PDSCH-FlexibleStartPRB-AllocConfig-r15, and , the  and  is determined according to Table 7.1.6.3-2 where,

-	 is the smallest and the largest physical resource-block number, respectively, of the allocated narrowband as defined in Subclause 6.2.7 of [3]

-	 is the value of the downlink system bandwidth

-	P is the RBG size associated with the downlink system bandwidth,, according to Table 7.1.6.1-1

-

-	Physical resource-blocks with indices  or ,  correspond to physical resource-blocks outside the allocated narrowband relative to physical resource-block

-	 shall not exceed ()

Table 7.1.6.3-2:  and for  and CEModeA

| 0 |  | 2 |
| --- | --- | --- |
| 1 |  | 3 |
| 2 |  | 4 |
| 3 |  | 5 |
| 4 |  | 6 |
| 5 |  | 6 |
| 6 |  | 5 |
| 7 |  | 4 |
| 8 |  | 3 |
| 9 |  | 2 |

[TS 36.321, clause 7.1.6.5]

If the UE is a BL/CE UE not configured with higher layer parameter ce-PDSCH-FlexibleStartPRB-AllocConfig-r15, PRGs of size =3 partition a narrowband with RB indices 0-2 in the narrowband in one PRG and RB indices 3-5 in the narrowband in another PRG.

If the UE is a BL/CE UE configured with CEModeA and configured with higher layer parameter ce-PDSCH-FlexibleStartPRB-AllocConfig-r15,

-	if  in Table 7.1.6.3-2, then the set of two PRGs is starting from ;

-	if , then the set of two PRGs is ending ending at .

7.1.3.23.3	Test description

7.1.3.23.3.1	Pre-test conditions

System Simulator:

- Cell 1

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 1.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4-CE) according to [18].

7.1.3.23.3.2	Test procedure sequence

Table 7.1.3.23.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS indicates a new transmission on PDCCH DCI Format 6-1A with RA type 2 using Localized VRB and resource allocation field consiststing of values of and according to Table 7.1.6.3-2 in TS 36.213. | <-- | MAC PDU | - | - |
| 2 | At the reception of scheduling request, the SS transmits UL Grant for transmitting loop back MAC PDU. | <-- | (UL Grant) | - | - |
| 3 | CHECK: Does UE return the same MAC PDU as received in step 1? | --> | MAC PDU | 1 | P |

7.1.3.23.3.3	Specific message contents

Table 7.1.3.23.3.3-1: RadioResourceConfigCommon-DEFAULT (Table 7.1.3.23.3.3-1)

Derivation Path : 36.508 table 4.6.3-13 with condition FullConfig and CEmodeA

Table 7.1.3.23.3.3-2: PhysicalConfigDedicated-DEFAULT (preamble)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 with CEmodeA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| pdsch-ConfigDedicated-v1530 SEQUENCE { |  |  |  |
| ce-PDSCH-FlexibleStartPRB-AllocConfig-r15 | on |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.23.3.3-3: PDSCH-ConfigCommon-v1310-DEFAULT (Table 7.1.3.23.3.3-2)

Derivation Path: 36.508 Table 4.6.3-5A with CEmodeA