3GPP TSG-RAN WG5 #93-e		R5-217217
Electronic Meeting, 8th – 19th November 2021

Title:	Proposing update to Rel-16 test cases based on TEI-16 maintenance in RAN4

Source:	Qualcomm Europe Inc. (Spain)

Agenda item	5.3.1.17

Document for:	Endorsement

# 1.	Introduction

RAN4 has agreed and added a feature requirement for UE supporting short transient periods capability to TS 38.101-1 Rel-16 (introduced in RAN4 R4-2103152 CR 0641). The CR was agreed under WI TEI16.  This discussion paper provide some details of the changes RAN4 introduced and request RAN5 endorsement to handle the corresponding Ran5 spec updating and changes under WIC TEI16_Test.

# 2. 	Discussion

## 2.1	TEI16 maintenance in RAN4

In order to improve UE performance, short transient period capability has been discussed extensively in several early RAN4 meetings. In RAN4#93 meeting, a WF [1] was agreed. In the WF it was agreed that DFT-spread can be tested by excluding the transient period from the measurement interval.

In 3GPP TSG-RAN4 Meeting #93 Proposal for EVM measurement interval to include symbols with short transient period was agreed [2].  RAN4 has introduced new RF requirement 6.4.2.1a Error Vector Magnitude including symbols with transient period under Rel 16 general maintenance work item TEI16[3].

## 2.2 	Addition of Short Transient Period Capability in TS 38.101-1

The details of adding short transient period UE capability and new EVM measurement was captured in CR R4-2103152.  Related key content was copied below:

### 6.3.3	Transmit ON/OFF time mask

#### 6.3.3.1	General

The transmit power time mask defines the transient period(s) allowed

-	between transmit OFF power as defined in subclause 6.3.2 and transmit ON power symbols (transmit ON/OFF)

-	between continuous ON-power transmissions with power change or RB hopping is applied. When a UE signals the transient period capability, the transient period value (tp) can be 2, 4, or 7 usec.  If no capability is signalled, the default transient period value of 10 usec applies.

In case of RB hopping, transition period is shared symmetrically when the transient period is 10usec (tpstart would then be equal to -5s in following figures). If the UE signals a transient period (tp) of 2, 4 or 7usec, the transient period start position is given by tpstart in Table 6.3.3.1-1.

Table 6.3.3.1-1 tpstart  values

| tp (s) | tpstart (s) |
| --- | --- |
| 2 | [-0.5] |
| 4 | [-1] |
| 7 | [-2] |
| NOTE 1: Negative values mean that the transient period starts before the symbol boundary |  |

Unless otherwise stated the requirements in clause 6.5 apply also in transient periods.

In the following subclauses, following definitions apply:

-	A slot or long subslot transmission is a transmission with more than 2 symbols.

-	A short subslot transmission is a transmission with 1 or 2 symbols.

< end of changes >

<start of changes>

#### 6.3.3.6	SRS time mask

……

When power change between consecutive SRS transmissions is required, then Figure 6.3.3.6-3 and Figure 6.3.3.6-4 apply.

Figure 6.3.3.6-3: Consecutive SRS time mask for the case when power change is required and when 15 kHz and 30 kHz SCS is used in FR1

Figure 6.3.3.6-4: Consecutive SRS time mask for the case when power change is required and when 60 kHz SCS is used in FR1, when the transient period is 10 µs

#### 6.3.3.7	PUSCH-PUCCH and PUSCH-SRS time masks

The PUCCH/PUSCH/SRS time mask defines the observation period between sounding reference symbol (SRS) and an adjacent PUSCH/PUCCH symbol and subsequent UL transmissions. The time masks apply for all types of frame structures and their allowed PUCCH/PUSCH/SRS transmissions unless otherwise stated.

Figure 6.3.3.7-1: PUCCH/PUSCH/SRS time mask when there is a transmission before or after or both before and after SRS, when sounded on the same antenna (Ant 'x')

Figure 6.3.3.7-2: PUCCH/PUSCH/SRS time mask when there is a transmission before or after or both before and after SRS, when sounded on a different antenna (Ant 'x' and Ant 'y' are different antenna ports)

Figure 6.3.3.7-3: Consecutive long subslot transmission and long subslot transmission time mask

#### 6.3.3.8	Transmit power time mask for consecutive slot or long subslot transmission and short subslot transmission boundaries

The transmit power time mask for consecutive slot or long subslot transmission and short slot transmission boundaries defines the transient periods allowed between such transmissions.

Figure 6.3.3.8-1: Consecutive slot or long subslot transmission and short subslot transmission time mask

#### 6.3.3.9	Transmit power time mask for consecutive short subslot  transmissions boundaries

The transmit power time mask for consecutive short subslot transmission boundaries defines the transient periods allowed between short subslot transmissions.

The transient period shall be equally shared as shown on Figure 6.3.3.9-2.

Figure 6.3.3.9-1: Void

Figure 6.3.3.9-2: Consecutive short subslot transmissions time mask

Figure 6.3.3.9-3: Consecutive short subslot (1 symbol gap) time mask for the case when transient period is required on both sides of the symbol and when 60 kHz SCS is used in FR1, when the transient period is 10 µs.

<end of changes>

< start of changes >

#### 6.4.2.1a	Error Vector Magnitude including symbols with transient period

In 6.4.2.1, EVM has been defined by excluding the symbols which have a transient period. In this section, measurement interval is defined for the symbols with a transient period to include these symbols in the RMS average EVM computation when the UE reports a transient period capability other than the default. Before calculating the EVM, the measured waveform is corrected for sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM. The symbols with transient period should not be used for equalization. Only CP-OFDM waveform is used for conformance testing.”

In the case of PUSCH or PUCCH transmissions when the mean power, modulation or RB allocation across slot or subslot boundaries is expected to change the EVM result over the symbols where the transient occurs is calculated according to Table 6.4.2.1a-1.

Table 6.4.2.1a-1: EVM definition for reported transient period

| Reported transient capability (us) | EVM definition | tpstart (µs) | SCS4 |
| --- | --- | --- | --- |
| 2 | EVMafter=max⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | [-0.5] | 15kHz or 30kHz5 |
| 4 | EVMafter=max⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | [-1] | 15kHz |
| 7 | EVMafter=min⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | [-2] | 15kHz |
| NOTE 1: EVMl , EVMh, EVMl_tp,and EVMh_tp are defined in Annex F NOTE 2: EVMafter is the EVM for a symbol right after a transition; EVMbefore is the EVM for a symbol right before a transition NOTE 3: tpstart denotes the start position of the EVM exclusion window as shown in Annex F.4 NOTE 4: SCS denotes the SCS that can be used in the conformance test NOTE 5: 30kHz shall be used in the conformance test unless the UE signals in supportedSubCarrierSpacingUL in FeatureSetPerCC that it only supports 15kHz in the corresponding band |  |  |  |

The RMS average of the basic EVM measurements over [108] subframes for the symbols where the transient occurs for the different modulation schemes shall not exceed the values  specified in Table 6.4.2.1a-2 for the parameters defined in Table 6.4.2.1a-3.  This requirement can be verified with 64 QAM and 256 QAM modulation.

Table 6.4.2.1a-2: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | [10] |
| 256 QAM | % | [8] |

Table 6.4.2.1a-3: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | Table 6.3.1-1 |
| UE Output Power for 256 QAM | dBm | Table 6.3.1-1 + 10 dB |
| Operating conditions |  | Normal conditions |

## 2.3 	Impacted test case in TS 38.521-1

From the changes introduced by RAN4 CRs,  Transmit ON/OFF time mask test case in TS 38.521-1 shall be updated  per the newly added UE capability.   A new test case of EVM measurement shall be added.

The test cases and corresponding target completion date are proposed as below:

| Clause | Test case title | Target completion date | Tdoc Number | Progress |
| --- | --- | --- | --- | --- |
| 6.3.3.1 | General | RAN5#94 |  | 0% |
| 6.3.3.6 | SRS time mask | RAN5#94 |  | 0% |
| 6.4.2.1a | Error Vector Magnitude including symbols with transient period | RAN5#94 |  | 0% |
| Annex E.5.9 | Derivation of the results | RAN5#94 |  | 0% |

## 2.4 	Proposal

In order to align RAN5 specification to RAN4 core RF requirement, this new test requirement shall be added to TS 38.521-1. Considering the changes was introduced under WI TEI16 at RAN4 group, the following is proposed:

Proposal 1: Seeking RAN5 endorsement for handling proposed updates/addition under Rel-16 maintenance – i.e. WIC - ‘TEI16_Test’

# 3. 	Conclusion

Based on the discussion, it’s suggested that RAN5 to discuss and endorse below proposal:

Proposal 1: 	Seeking RAN5 endorsement for handling proposed updates/addition under Rel-16 maintenance – i.e. WIC - ‘TEI16_Test’

# 4. 	References

- 3GPP R4-1912921, “WF on Transient period capability”

- 3GPP R4-1915290 “Proposal for EVM measurement interval to include symbols with transient period”

- 3GPP R4-2103152, “CR on introduction of shorter Transient Period Capability”