3GPP TSG-RAN5 Meeting #94-e			R5-222032

Electronic Meeting, 21st February - 4th March 2022

Title:	Conformance Test Case Development for 3GPP Rel-16 Standalone Non-Public Network (SNPN) and other Verticals UE

Source:	TSG RAN WG5

To:	GCF CAG, PTCRB PVG, GCF SG, PTCRB

Cc:

Contact Person:

#### Name:	Yogesh Tugnawat

Tel. Number:

E-mail Address:	yogesht [dot] qti [dot] qualcomm [dot] com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN5 is currently working on defining signalling conformance test cases for Rel-16 UE supporting SNPN. RAN5 is following the normal process of defining the Rel-16 specific test cases for SNPN feature based on relevant core requirements. However, 3GPP core specifications allow implementation of a UE that only operates in a SNPN  (SNPN-only UE), such a UE also has to meet applicable Rel-15 5GS core requirements. This means Rel-16 specific test cases for SNPN and applicable Rel-15 5GS test cases must be executed for formal conformance testing of a SNPN-only UE.

RAN5 has been discussing the above for protocol conformance testing, as the same principle might apply to several other Rel-16 and Rel-17 (and beyond) features that address new “Verticals”, for ex – IIoT only, NR-U only or a RedCap UE. Below approach was considered for SNPN-only UE test case scoping-

Scoping -1 --> Since SNPN is only applicable for Standalone test cases, all option-2\standalone\5GC core Rel-15 test cases were chosen.

Scoping 2 --> Due to the lack of conformance requirement, below features and corresponding test cases will be marked as "Not Applicable", following procedures are not defined in Rel-16 for a UE operating in SNPN, for ex-

- PLMN Selection

- IRAT to LTE

- CAG

- Autonomous Search

- IMS

- Emergency

- HO, Roaming.. etc

Scoping 3-->Each test case to be analyzed to determine what is the minimum test environment change that will make it eligible for execution. For example - NR Cell Change to SNPN; Sys Info Combination; SIB1, SIB10 changes etc

Scoping 4 and Proposal--> For fully scoped test cases, update the prose of such tests in a separate clause in TS38.523-1/2 for TTCN Implementation addressing testability and applicability.

RAN5 has reached an agreement in principle to define applicable Rel-15 5GS test cases in a separate clause in TS 38.523-1 for “Verticals” with sub clauses to be added for the “Verticals’ features mentioned above. Test cases in these sub clauses will be based on the Rel-15 5GS baseline test cases but adapted/modified to cater for the “Verticals’ features. These test cases along with “Verticals’ feature specific test cases must be executed for formal conformance testing of a “Vertical” UE

2. Actions:

To GCF CAG and PTCRB PVG.

ACTION: 	RAN5 would like to inform GCF and PTCRB on the above-mentioned proactive approach being considered for defining signalling conformance test cases for SNPN-only UEs and welcome any feedback on this topic.

To GCF SG and PTCRB.

ACTION: 	RAN5 kindly asks GCF and PTCRB to keep RAN5 updated about the industry need and signalling conformance test cases required for the certification of “Vertical” UEs.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#95-e 	 9th – 20th May 2022		Electronic Meeting

TSG-RAN5 Meeting#96	 	 22nd – 26th August 2022	Toulouse, FR