3GPP TSG-RAN5 Meeting #95-e	R5-222257

# Online, , 9th May – 20th May 2022

Source:	Qualcomm Incorporated

Title:	Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE

# Document for:	Agreement

Agenda Item:	6.3.11.6

## 1.	Introduction

Standalone Non-Public networks (SNPN) were introduced in Rel-16 and work is currently commencing in RAN5 for conformance specifications, which are tracking core specifications. There may be a possibility that a UE can be SNPN ONLY UE, i.e. designed to access Standalone Network only [4]. For such UE, we may have to provide coverage for applicable legacy NR (Rel-15) test cases. As per agreed w/f [5], initial scoping was captured in an xls attached with discussion document [6], Chapter 6 [1] test cases were scoped initially. After further discussion [7], below way forward was considered –

- Scoping of “SNPN Only UE” applicable R-15 test to continue for rest of the areas in SIG (RRC, NAS etc).

- General agreement on Proposal 2 i.e create a separate chapter\cl in TS38.523-1, 38.523-2 for “verticals” and add applicable test cases to that clause. However, need to address applicability to ensure legacy R-15 tests are “not” applied.

- As LS [8] was sent to GCF\PTCRB to alert them on RAN5 progress as well seek updates, if any.

## 2.	Discussion

After securing agreement on a separate clause and defining applicable test cases in that clause, the only open item in this discussion is towards applying such test cases to properly define applicability of the new test cases as well as ensure that all Rel-15 mandatory test cases are “not applied” in test case selection expression. For example, an SNPN Only UE shall select all test cases in the new clause in TS38.523-1 along with applicable R-16 specific SNPN test cases but NOT R-15 mandatory test cases. Also, the goal is to not alter existing applicability of R-15 test cases to ensure spec clarity.

Above problem can be solved as follows –

2.1 Define a new basic capability PICS in TS38.508-2  - “UEs supporting 5G Core without PLMN access” instead of existing  “UEs supporting 5G Core”. This new PICS indicate the UE does not support accessing 5G core using PLMN identity thus indicating UE is a private network operation UE only.

2.2 Supplement above PICS with SNPN to create a unique condition, like below in TS38.523-2. Adding SNPN to above PICS indicate the private network only UE support Standalone Non Public Networks, in other words, they can access the 5G Core using SNPN –

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 14 | Verticals |  |  |  |
| 14.1 | Legacy test cases for Rel-16 SNPN-Only UE |  |  |  |
| 14.1.1 | Idle and RRC Inactive Mode Operation for SNPN-Only |  |  |  |
| 14.1.1.1 | Cell Selection/Qrxlevmin & Cell Reselection (Intra NR) | Rel-16 | Cxyz | UEs supporting 5G Core without PLMN access and SNPN |

2.3 Change the existing R-16 SNPN specific tests as below, thus still allowing coverage for UE’s supporting both PLMN and SNPN as well SNPN-Only access –

| 6.5 | SNPN and CAG Selection |  |  |  |
| --- | --- | --- | --- | --- |
| 6.5.1 | SNPN Only Selection |  |  |  |
| 6.5.1.1 | SNPN Selection in Manual Mode | Rel-16 | Cxyz | SNPN and UEs supporting 5G Core OR UEs supporting 5G Core without PLMN access |
| 6.5.1.2 | SNPN Selection in Automatic Mode | Rel-16 | Cxyz | SNPN and UEs supporting 5G Core OR UEs supporting 5G Core without PLMN access |
| 6.5.1.3 | SNPN / User Reselection in Automatic Mode | Rel-16 | Cxyz | SNPN and user initiated SNPN reselection in automatic mode on NR and UEs supporting 5G Core OR UEs supporting 5G Core without PLMN access |

2.4 An SNPN-only UE would then indicate PICS for “UE supporting 5G Core” as Not-supported, thus any R-15 applicable tests will not be applied.

## 3.	Proposal/Way Forward

Introduce the new PICS – “UEs supporting 5G Core without PLMN access” in TS 38.508-2 as defined in #2 above and apply as per #2.1-2.4 mentioned in discussion.

## 4.	References

[1]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	 TS 36.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; EUTRA; User Equipment (UE) conformance specification; Part 1: Protocol

[3]	TS 38.508-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception User Equipment (UE) conformance specification;

[4]	TS 38.300: NR; NR and NG-RAN Overall description; Stage-2; Rel-16 (cl 16.6.1)

[5]	R5-213408: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#91e

[6]	R5-214736: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#92e

[7]	R5-220837: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#94e

[8] 	R5-222032: LS to GCF, PTCRB - Conformance Test Case Development for 3GPP Rel-16 Standalone Non-Public Network (SNPN) and other Verticals UE, RAN5#94e