3GPP TSG-RAN5 Meeting #95-e	R5-222693

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5406 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to test applicability for 4G test caes without UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test applicability used in many without UL CA test cases are not aligned in the description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update test applicability to align the same wording in different CA test cases without UL CA support; - Editorial update to 3GPP style. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test applicability in CA test cases without UL CA supported will keep misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.2, 7.3A.3.2, 7.3A.4.2, 7.4A.2.2, 7.4A.2_H.2, 7.4A.3.2, 7.4A.3_H.2, 7.4A.4.2, 7.4A.4_H.2, 7.5A.2.2, 7.5A.3.2, 7.5A.4.2, 7.6.1A.2.2, 7.6.1A.3.2, 7.6.1A.4.2, 7.6.2A.2.2, 7.6.2A.3.2, 7.6.2A.4.2, 7.6.3A.2.2, 7.6.3A.3.2, 7.6.3A.4.2, 7.7A.2.2, 7.7A.3.2, 7.7A.4.2, 7.8.1A.2.2, 7.8.1A.3.2, 7.8.1A.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

### 7.3A.2	Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA)

7.3A.2.1	Test purpose

Same as in clause 7.3A.1.1.

7.3A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

7.3A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

<Unchanged Sections Skipped>

### 7.3A.3	Reference sensitivity level for CA (inter-band DL CA without UL CA)

Editor’s note:	Band 70 CA configurations are missing test point analysis

7.3A.3.1	Test purpose

Same as in clause 7.3A.1.1.

7.3A.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support inter-band DL CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 2DL CA with FDD-TDD inter-band CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support DL LAA.

This test case applies only for CA configurations that do not support UL CA.

7.3A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

<Unchanged Sections Skipped>

### 7.3A.4	Reference sensitivity level for CA (intra-band non-contiguous DL CA without UL CA)

7.3A.4.1	Test purpose

Same as in clause 7.3A.1.1.

7.3A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

7.3A.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

<Unchanged Sections Skipped>

### 7.4A.2	Maximum input level for CA (intra-band contiguous DL CA without UL CA)

Editor's note: Any restriction of the applicability of test points due to the supported UE category is under investigation

7.4A.2.1	Test purpose

Same test purpose as in clause 7.4A.1.1.

7.4A.2.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

7.4A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

<Unchanged Sections Skipped>

### 7.4A.2_H	Maximum input level for CA (intra-band contiguous DL CA without UL CA) for 256QAM in DL

7.4A.2_H.1	Test purpose

Same test purpose as in clause 7.4A.2.1.

7.4A.2_H.2	Test applicability

This test applies to all types of E-UTRA UE release 12 and forward that support intra-band contiguous DL CA and 256QAM in DL. This test case applies only for CA configurations that do not support UL CA.

7.4A.2_H.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.1_H.3.

<Unchanged Sections Skipped>

### 7.4A.3	Maximum input level for CA (inter-band DL CA without UL CA)

Editor's note:

- Any restriction of the applicability of test points due to the supported UE category is under investigation

#### 7.4A.3.1	Test purpose

Same test purpose as in clause 7.4A.1.1

#### 7.4A.3.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support inter-band DL CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 2DL CA with FDD-TDD inter-band CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support DL LAA.

This test case applies only for CA configurations that do not support UL CA.

#### 7.4A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

<Unchanged Sections Skipped>

### 7.4A.3_H	Maximum input level for CA (inter-band DL CA without UL CA) for 256QAM in DL

#### 7.4A.3_H.1	Test purpose

Same test purpose as in clause 7.4A.3.

#### 7.4A.3_H.2	Test applicability

This test applies to all types of E-UTRA UE release 12 and forward that support inter-band DL CA and 256QAM in DL. This test case applies only for CA configurations that do not support UL CA.

#### 7.4A.3_H.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.1_H.3.

<Unchanged Sections Skipped>

### 7.4A.4	Maximum input level for CA (intra band non-contiguous DL CA without UL CA)

#### 7.4A.4.1	Test purpose

Same as in clause 7.4A.1.1.

#### 7.4A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

#### 7.4A.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

<Unchanged Sections Skipped>

### 7.4A.4_H	Maximum input level for CA (intra band non-contiguous DL CA without UL CA) for 256QAM in DL

#### 7.4A.4_H.1	Test purpose

Same as in clause 7.4A.1.1.

#### 7.4A.4_H.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward that support intra-band non-contiguous DL CA and 256QAM in DL. This test case applies only for CA configurations that do not support UL CA.

#### 7.4A.4_H.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.1_H.3

<Unchanged Sections Skipped>

### 7.5A.2	Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA without UL CA)

#### 7.5A.2.1	Test purpose

The test purpose is the same as in clause 7.5A.1.1.

#### 7.5A.2.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

#### 7.5A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.5A.0.

<Unchanged Sections Skipped>

### 7.5A.3	Adjacent Channel Selectivity (ACS) for CA (inter-band DL CA without UL CA)

#### 7.5A.3.1	Test purpose

The test purpose is the same as in clause 7.5A.1.1.

#### 7.5A.3.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support inter-band DL CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 2DL CA with FDD-TDD inter-band CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support DL LAA.

This test case applies only for CA configurations that do not support UL CA.

#### 7.5A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.5A.0.

<Unchanged Sections Skipped>

### 7.5A.4	Adjacent Channel Selectivity (ACS) for CA (intra-band non-contiguous DL CA without UL CA)

#### 7.5A.4.1	Test purpose

Same as in clause 7.5A.1.1.

#### 7.5A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

#### 7.5A.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.5A.0.

<Unchanged Sections Skipped>

#### 7.6.1A.2	In-band blocking for CA (intra-band contiguous DL CA without UL CA)

##### 7.6.1A.2.1	Test Purpose

Same test purpose as in clause 7.6.1A.1.

##### 7.6.1A.2.2	Test Applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

##### 7.6.1A.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.1A.0.

<Unchanged Sections Skipped>

#### 7.6.1A.3	In-band blocking for CA (inter-band DL CA without UL CA)

##### 7.6.1A.3.1	Test Purpose

In-band blocking is defined for an unwanted interfering signal falling into the range from 15MHz below (60 MHz for band 46) to 15MHz above (60 MHz for band 46) the UE receive band, at which the relative throughput shall meet or exceed the requirement for the specified measurement channels.

##### 7.6.1A.3.2	Test Applicability

This test applies to all types of E-UTRA UE release 10 and forward that support inter-band DL CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 2DL CA with FDD-TDD inter-band CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support DL LAA.

This test case applies only for CA configurations that do not support UL CA.

##### 7.6.1A.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.1A.0.

<Unchanged Sections Skipped>

#### 7.6.1A.4	In-band blocking for CA (intra-band non-contiguous DL CA without UL CA)

##### 7.6.1A.4.1	Test Purpose

Same as in clause 7.6.1A.1.1.

##### 7.6.1A.4.2	Test Applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

##### 7.6.1A.4.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.1A.0.

<Unchanged Sections Skipped>

#### 7.6.2A.2	Out-of-band blocking for CA (intra-band contiguous DL CA without UL CA)

##### 7.6.2A.2.1	Test Purpose

Same test purpose as 7.6.2A.1.

##### 7.6.2A.2.2	Test Applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

##### 7.6.2A.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.2A.0.

<Unchanged Sections Skipped>

#### 7.6.2A.3	Out-of-band blocking for CA (inter-band DL CA without UL CA)

Editor's notes: This clause is incomplete for Band 46. The following aspects are either missing or not yet determined.

- Test system uncertainties are missing in annex F.

##### 7.6.2A.3.1	Test Purpose

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above (60 MHz for band 46) the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz below or above (60 MHz for band 46) the UE receive band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5.1 and sub-clause 7.6.1 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

##### 7.6.2A.3.2	Test Applicability

This test applies to all types of E-UTRA UE release 10 and forward that support inter-band DL CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 2DL CA with FDD-TDD inter-band CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support DL LAA.

This test case applies only for CA configurations that do not support UL CA.

##### 7.6.2A.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.2A.0.

<Unchanged Sections Skipped>

#### 7.6.2A.4	Out-of-band blocking for CA (intra-band non-contiguous DL CA without UL CA)

##### 7.6.2A.4.1	Test Purpose

Same as in clause 7.6.2A.1.1.

##### 7.6.2A.4.2	Test Applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

##### 7.6.2A.4.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.2A.0.

<Unchanged Sections Skipped>

#### 7.6.3A.2	Narrow band blocking for CA (intra-band contiguous DL CA without UL CA)

##### 7.6.3A.2.1	Test Purpose

Same test purpose as 7.6.3A.1.

##### 7.6.3A.2.2	Test Applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA. This test case applies only for CA vonfigurations that do not support UL CA.

##### 7.6.3A.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.3A.0.

<Unchanged Sections Skipped>

#### 7.6.3A.3	Narrow band blocking for CA (inter-band DL CA without UL CA)

##### 7.6.3A.3.1	Test Purpose

Same test purpose as 7.6.3A.1.

##### 7.6.3A.3.2	Test Applicability

This test applies to all types of E-UTRA UE release 10 and forward that support inter-band DL CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 2DL CA with FDD-TDD inter-band CA.

This test case applies only for CA configurations that do not support UL CA.

##### 7.6.3A.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.3A.0.

<Unchanged Sections Skipped>

#### 7.6.3A.4	Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA)

##### 7.6.3A.4.1	Test Purpose

Same as in clause 7.6.3A.1.1.

##### 7.6.3A.4.2	Test Applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

##### 7.6.3A.4.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.3A.0.

<Unchanged Sections Skipped>

### 7.7A.2	Spurious response for CA (intra-band contiguous DL CA without UL CA)

#### 7.7A.2.1	Test Purpose

Same test purpose as in clause 7.7A.1.

#### 7.7A.2.2	Test Applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

#### 7.7A.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7A.0.

<Unchanged Sections Skipped>

### 7.7A.3	Spurious response for CA (inter-band DL CA without UL CA)

#### 7.7A.3.1	Test Purpose

Same test purpose as in clause 7.7A.1.

#### 7.7A.3.2	Test Applicability

This test applies to all types of E-UTRA UE release 10 and forward that support inter-band DL CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 2DL CA with FDD-TDD inter-band CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support DL LAA.

This test case applies only for CA configurations that do not support UL CA.

#### 7.7A.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7A.0.

<Unchanged Sections Skipped>

### 7.7A.4	Spurious response for CA (intra-band non-contiguous DL CA without UL CA)

#### 7.7A.4.1	Test Purpose

Same test purpose as in clause 7.7A.1.

#### 7.7A.4.2	Test Applicability

This test applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

#### 7.7A.4.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7A.0.

<Unchanged Sections Skipped>

#### 7.8.1A.2	Wideband intermodulation for CA (intra-band contiguous DL CA without UL CA)

##### 7.8.1A.2.1	Test purpose

Same as 7.8.1A.1

##### 7.8.1A.2.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that supports intra-band contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

##### 7.8.1A.2.3	Minimum conformance requirements

Same as 7.8.1A.1

<Unchanged Sections Skipped>

#### 7.8.1A.3	Wideband intermodulation for CA (inter-band DL CA without UL CA)

##### 7.8.1A.3.1	Test purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

##### 7.8.1A.3.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that supports inter-band contiguous DL CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 2DL CA with FDD-TDD inter-band CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support DL LAA.

This test case applies only for CA configurations that do not support UL CA.

##### 7.8.1A.3.3	Minimum conformance requirements

Same as 7.8.1A.1

<Unchanged Sections Skipped>

#### 7.8.1A.4	Wideband intermodulation for CA (intra band non-contiguous DL CA without UL CA)

##### 7.8.1A.4.1	Test purpose

Same as 7.8.1A.1

##### 7.8.1A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA. This test case applies only for CA configurations that do not support UL CA.

##### 7.8.1A.4.3	Minimum conformance requirements

Same as 7.8.1A.1

## {End of changes}