3GPP TSG-RAN WG5 Meeting #95-e	R5-222397

# Electronic Meeting, , 9th May  – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0259 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.6 - SIMPLE-FILTER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In filter(1) for Per_CLIENT condition the comment for client id refers to MCPTT only - For "PER_CLIENT" condition detailed encryption requirements are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Comment for client id corrected - For "PER_CLIENT" condition Editors Note added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.6	SIMPLE-FILTER

Table 5.5.3.6-1: SIMPLE-FILTER

| Derivation Path: RFC 4661 [48] clause 7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| filter-set |  |  |  |  |
| ns-bindings |  | TS 24.379 [9] clause 9.3.2.2 requires two separate ns-binding elements |  |  |
| ns-binding urn [1] |  |  |  |  |
| prefix | "pidf" |  |  |  |
| urn | "urn:ietf:params:xml:ns:pidf" |  |  |  |
| ns-binding urn [2] |  |  |  | MCPTT |
| prefix | "mcpttPI10" |  |  |  |
| urn | "urn:3gpp:ns:mcpttPresInfo:1.0" |  |  |  |
| ns-binding urn [2] |  |  |  | MCVIDEO |
| prefix | "mcvideoPI10" |  |  |  |
| urn | "urn:3gpp:ns:mcvideoPresInfo:1.0" |  |  |  |
| ns-binding urn [2] |  |  |  | MCDATA |
| prefix | "mcdataPI10" |  |  |  |
| urn | "urn:3gpp:ns:mcdataPresInfo:1.0" |  |  |  |
| filter[1] |  |  |  |  |
| id attribute | Any value | The value of the 'id' attribute has to be unique within the <filter-set> element |  |  |
| uri attribute | Not present | According to TS 24.379 [9] clause 9.3.2.2 |  |  |
| domain attribute | Not present | According to TS 24.379 [9] clause 9.3.2.2 |  |  |
| remove attribute | Not present | 'false' per default |  |  |
| enabled attribute | Not present | 'true' per default |  |  |
| what |  |  |  | PER-CLIENT |
| include | "//presence/tuple[@id=" & client id (NOTE 1) & "]" Editor's Note: FFS whether and how this element should be encrypted | contains the value, according to IETF RFC 4661 [48], set to concatenation of the '//presence/tuple[@id="' string, the MCX client ID, and the '"]' string |  |  |
| what |  |  | RFC 4661 [48] | PER-GROUP |
| include | "//pidf:presence/pidf:additionalData/@pidf:groupCallOngoing" |  | TS 24.379 [9] clause 9.3.2.2 |  |
| trigger | Not present |  |  |  |
| NOTE 1: UUID URN as provided by the client at initial registration |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PER-CLIENT | Per-client restrictions of presence event package notification information according to TS 24.379 [9] clause 9.3.2.2 |
| PER-GROUP | Per-group restrictions of presence event package notification information according to TS 24.379 [9] clause 9.3.2.2 |

Table 5.5.3.6-2: Void

Table 5.5.3.6-3: Void