3GPP TSG-RAN WG5 Meeting #96-e                                         R5-224844

Electronic Meeting, August 15-26, 2022

Source: 	Huawei, HiSilicon

Title: 	WF on RedCap RF testing

Agenda Item:

Document for:	Approval

Introduction

The Rel-17 NR_redcap_plus_ARCH-UEConTest WI was approved at RAN#95 [1]. This document discusses how to handle the RF testing of the RedCap UEs.

Discussion

Tx test cases

For RedCap UE, RAN4 doesn’t specify extra requirements. Existing PC3 requirements are reused for RedCap UEs.

- 6.2.1I	Maximum output power for RedCap UE

For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply.

In RAN5, the similar approach could be considered: reusing the existing PC3 test cases for RedCap UE. The benefits of this approach are:

-	Avoiding repeatedly adding the same contents within the specification.

-	The readability and maintainability of the specification could be improved.

-	The work load of specifying RedCap test could be largely reduced.

Comparing with legacy UE, the RedCap UE has limited capability as specified in clause 4.2.21.1 in TS 38.306. The limitations that might have impact on RF testing are:

-	The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

-	1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported. UE features and corresponding capabilities related to more than 2 UE Rx branches and more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 2 UE Tx branches and more than 2 UL MIMO layers are not supported by RedCap UEs;

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise.

Having this in mind, the feasibility of reusing the existing test cases for RedCap UE is analysed below using 6.2.1 MOP as an example.

- 6.2.1	UE maximum output power

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

Observation 1: Test purpose could be reused without any change.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1 and 3 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 UE not supporting txDiversity-r16 release 15 and forward.

In Rel-17, the RedCap UE could only support Power Class 3, which is already included in the highlighted test applicability statement. Therefore test applicability could be reused for RedCap UE without any change.

Observation 2: Test applicability could be reused without any change.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ± 2 |
| n2 |  |  |  |  | 23 | ± 23 |
| n3 |  |  |  |  | 23 | ± 23 |

<….>

As stated above, there is no extra requirements specified for RedCap UE. Therefore the existing minimum conformance requirements could be reused by RedCap UE without any change.

Observation 3: Minimum conformance requirments could be reused without any change.

6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For band n28, the Highest test channel bandwidth is replaced by 20MHz due to MPR is always larger than 0dB for 30MHz bandwidth. |  |  |  |

The test configuration needs to consider the RedCap UE’s limitation of supporting up to 20MHz in FR1. In last RAN5 meeting, the tables of test channel bandwidth in subclause 4.3.1 of TS 38.508-1 were updated to add one separated column for RedCap UEs as copied below. With this update, the test channel bandwidth could be correctly selected for RedCap UEs without changes in the test configuration table.

< Abstract from TS 38.508-1 >

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap UE Mid Test Channel bandwidth [MHz] |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 20 | 15 |
| n5 | 15 | 15 |
| n7 | 25 | 15 |
| n8 | 15 | 15 |
| n12 | 10 | 10 |
| <…> |  |  |

Observation 4: Test configuration table could be reused without any change. The limitation of up to 20MHz channel bandwidth is reflected in TS 38.508-1 subclause 4.3.1.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

The initial setup procedures include the default parameter settings and the generic procedure, which would require some RedCap specific changes for the UE to access the network. Currently the plan is to update the messages and generic procedures in a way transparent to test cases. This is not only for RF but also for SIG testing. Considering there are hundreds of legacy SIG test cases could apply to RedCap UEs, it would be of great value to make sure the test cases could be directly used by RedCap UEs without updating every single test case.

Observation 5: Initial setup procedures could be reused without any change. The RedCap specific changes to parameter settings and generic procedures would be reflected in TS 38.508-1 in a test case transparent way.

6.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

4.	For UEs supporting Power Class 2 or Power Class 1, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2.1.4.3-2.

The test procedure is quite general for all type of NR UEs. Nothing needs to be changed for RedCap UEs.

Observation 6: Test procedure could be reused without any change.

6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.1.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1 UE |

Observation 7: Message contents could be reused without any change.

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±2±TT |
| n2 |  |  |  |  | 23 | ±23±TT |
| n3 |  |  |  |  | 23 | ±23±TT |
| n5 |  |  |  |  | 23 | ±2±TT |
| n7 |  |  |  |  | 23 | ±23±TT |

Observation 8: Test requirements could be reused without any change.

Above analysis shows the feasibility of reusing existing test cases for RedCap UEs, which leads to following proposal.

Proposal 1: For RedCap Tx testing, reusing existing test cases without introducing separate sections.

Rx test cases

In Rel-15 and Rel-16, RAN4 has specified the Rx requirements for TDD/FDD UEs of 2Rx and 4Rx. For RedCap UEs, additional 1Rx REFSENS requirements are specified. In addition, HD-FDD 1Rx/2Rx REFSENS requirements are also specified. These new REFSENS requirements are introduced by a separate subclause 7.3I.

- 7.3I.2	Reference sensitivity power level

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2,5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3 |

Similarly with the Tx cases, legacy REFSENS test case 7.3.2 could apply to RedCap UE without any change except for the new 1Rx and HD-FDD requirements. It could be an option to merge all the RedCap specific requirements to the legacy test case 7.3.2, so that the test of RedCap Rx is simplified. Another option is to follow the RAN4’s structure, to specify a separate REFSENS for Redcap test case.

Proposal 2: Specify a separate REFSENS for RedCap test case 7.3I.2 to capture 1Rx and HD-FDD requirements.

For other Rx test cases, the requirements are depending on the REFSENS level. RAN4 has introduced a general section 7.1I to illustrate that for RedCap UE the REFSENS requirements in 7.3I shall apply where needed.

- 7.1I	General

For a Redcap UE the requirements in Section 7 shall be verified with the channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I.

RAN5 could follow the same approach. With the description in 7.1I, the Rx test cases other than REFSENS could be reused for RedCap UE without any further change. RAN5 specific configurations could be added to 7.1I if required.

Proposal 3: For RedCap Rx test cases other than REFSENS, reusing existing test cases without introducing separate sections.

Conclusion

In this contribution we discussed how to handle the RedCap RF testing. Following observations and proposals are made for RAN5’s consideration.

For Tx test cases:

Observation 1: Test purpose could be reused without any change.

Observation 2: Test applicability could be reused without any change.

Observation 3: Minimum conformance requirments could be reused without any change.

Observation 4: Test configuration table could be reused without any change. The limitation of up to 20MHz channel bandwidth is reflected in TS 38.508-1 subclause 4.3.1.

Observation 5: Initial setup procedures could be reused without any change. The RedCap specific changes to parameter settings and generic procedures would be reflected in TS 38.508-1 in a test case transparent way.

Observation 6: Test procedure could be reused without any change.

Observation 7: Message contents could be reused without any change.

Observation 8: Test requirements could be reused without any change.

Proposal 1: For RedCap Tx testing, reusing existing test cases without introducing separate sections.

For Rx test cases:

Proposal 2: Specify a separate REFSENS for RedCap test case 7.3I.2 to capture 1Rx and HD-FDD requirements.

Proposal 3: For RedCap Rx test cases other than REFSENS, reusing existing test cases without introducing separate sections.

- References

- RP-220830, “New WID on UE Conformance – Support of reduced capability NR devices”, China Unicom, Hisilicon, Ericsson, Huawei, Qualcomm, RAN #95-e, March 2022.