3GPP TSG RAN Meeting #96-e                                                  R5-225032

Electronic Meeting, 15th August – 26th August 2022

Agenda item:	5.3.19.7

Source:   Apple Portugal

Title:       Discussion on NR-DC RF tests

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

LTE-NR & NR-NR Dual Connectivity and NR CA enhancements work item has been in progress in RAN5. The current work plan includes within its scope updates to NR-DC requirements for RF testcases. The RF test cases in TS 38.521-1 were a later addition to the work item as per discussions and decisions in last RAN5 and plenary meetings. This discussion paper focuses on aspects of testing of the NR-DC RF test cases and proposes some alignments and streamline of test rules.

# 2	Discussion

## 2.1     General scope of NR-DC requirements added in core and test specifications.

It would help to understand the scope of new NR-DC test requirements added in TS 38101-1 which has subsequently been adopted in TS 38.521-1.

An analysis of the Ch6 tests in TS 38.521-1 (based on core requirements in TS 38.101-1) indicates that apart from the MOP tests all the remaining clauses do not have any specific exception requirements and point back to the NR-CA requirements for coverage.

For Ch7 (Receiver) NR-DC tests in TS 38.521-1/TS 38.101-1, all the NR-DC clauses leverage NR-CA test/requirement content. An example from Clause 7.3B is provided below.

Table 2.1-1 summarizes the complete picture regarding which tests have some exception requirements or NR-DC specific test needs and which purely leverage NR-CA requirements.

Table 2.1-1 – NR-DC requirements matrix

| 38.521-1 Clause | Test Case Title | Whether any testing unique to NR-DC |
| --- | --- | --- |
| 6.2B.1 | UE maximum output power for NR-DC | Yes. Combined power from all antenna connectors in NR-DC mode. |
| 6.2B.2 | UE maximum output power reduction for NR-DC | Yes. Similar test points as NR-CA but some unique requirements defined depending on NR-DC config |
| 6.2B.3 | UE additional maximum output power reduction for NR-DC | Yes. Depends on NR-DC config. Exception Requirements |
| 6.2B.4 | Configured output power for NR-DC | Yes. 38.101-1 defines specific/exception requirements for NR-DC |
| 6.3B.1 | Minimum output power for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.3B.2 | Transmit OFF power for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.3B.3 | Transmit ON/OFF time mask for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.4B.1 | Frequency error for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.4B.2.1 | Transmit modulation quality for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.4B.2.2 | Carrier leakage for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.4B.2.3 | In-band emissions for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.5B.1 | Output RF spectrum emissions for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.5B.2 | Out of band emission for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.5B.3 | Spurious emission for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.5B.4 | Transmit intermodulation for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 6.5B | Output RF spectrum emissions for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 7.3B | Reference sensitivity for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 7.4B | Maximum input level for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 7.5B | Adjacent channel selectivity for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 7.6B | Blocking characteristics for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 7.7B | Spurious response for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 7.8B | Intermodulation characteristics for NR-DC | No exception requirements. Pointer to NR CA sub-clause |
| 7.9B | Spurious emissions for NR-DC | No exception requirements. Pointer to NR CA sub-clause |

Observation 1: Apart from NR-DC FR1 MOP test which have some specific requirements defined to be tested and have dependency on the NR-DC Configuration under test, all other NR-DC clauses are coupled to NR-CA clauses

It should be noted that the NR-DC configurations defined in TS 38.101-1 also derive the bandwidth combination sets from the corresponding NR-CA configuration. Therefore it is not likely that any new NR-DC configuration is likely to be picked to be tested.

Observation 2: No new/unique NR-DC configurations are expected to be tested as they map to the bandwidth combination sets already defined for NR-CA combos.

## 2.2 Work Plan Approach to define NR-DC tests.

After an email discussion during RAN5#95e, RAN5 WG decided to adopt the option to leverage as much of the NR-CA test case content as possible due to the close coupling between these two sub-clauses. This further bolsters the observation made in Section 2.1 of this document. The agreement was captured as “Option 1.5” by the rapporteur.

“Option 1.5” is stated below:

## 2.3    RAN5 test requirements for NR-DC/NR-CA RF

From an architecture standpoint, there should not be any variation expected in performance between NR-DC and NR-CA since all that changes is the way the component carriers are aggregated; at MAC layer in case of NR-CA versus PDCP in case of NR-DC. RF Tx/Rx performance should therefore be agnostic to whether the configuration is NR-DC or NR-CA since the per-component carrier measurement (which is the basic principle behind both NR-CA and NR-DC) would ensure the requirements are verified similarly in both configs.

Observation 3: There should not be any variation expected in performance between NR-DC and NR-CA for equivalent combos/configurations involving the same component carriers/bands since all that changes is the way the component carriers are aggregated; at MAC layer in case of NR-CA versus PDCP in case of NR-DC

Table 2.3-1 NR-DC vs NR-CA [3]

Proposal 1: Introduce a test leverage statement in TS 38.521-1 common clause that, unless there are exception requirements defined: if a DUT is tested on an FR1 NR-CA configuration, the DUT need not be re-tested for the FR1 NR-DC configuration involving the same bands within the component carriers/carrier groups (CGs).

# 3	Summary

Below are the observations and proposal in this discussion paper

Observation 1: Apart from NR-DC FR1 MOP test which have some specific requirements defined to be tested and have dependency on the NR-DC Configuration under test, all other NR-DC clauses are coupled to NR-CA clauses

Observation 2: No new/unique NR-DC configurations are expected to be tested as they map to the bandwidth combination sets already defined for NR-CA combos.

Observation 3: There should not be any variation expected in performance between NR-DC and NR-CA for equivalent combos/configurations involving the same component carriers/bands since all that changes is the way the component carriers are aggregated; at MAC layer in case of NR-CA versus PDCP in case of NR-DC

Proposal 1: Introduce a test leverage statement in TS 38.521-1 common clause that, unless there are exception requirements defined: if a DUT is tested on an FR1 NR-CA configuration, the DUT need not be re-tested for the FR1 NR-DC configuration involving the same bands within the component carriers/carrier groups (CGs).

# 4	References

TS 38.521-1, FR1 RF Test Specification

R5-223262, Work Plan for WP LTE-NR & NR-NR Dual Connectivity and NR CA enhancements , Nokia, 3GPP RAN#95e, 9th May-22nd May 2022.

- [3]	Diagram reference: https://www.grandmetric.com/wp-content/uploads/2020/05/CA-vs-DC.png