3GPP TSG-RAN5 Meeting #96-e	R5-225062

# Electronic Meeting, 15th – 26th Aug. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2548 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing FR2 signal test frequencies for intra-band contiguous 3CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_IioT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the industry needs, test frequencies for FR2 intra-band contigous CA (3cc) need to be introduced into 6.2.3.4 for signalling test cases. |  |  |  |  |  |  |  |  |
| f |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding intra-band contiguous 3CA test frequencies in 6.2.3.4 for signalling testing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3CA test frequencies is absent for signalling testing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2.3.4	Test frequencies for NR CA configurations for signalling testing

The default channel bandwidths for NR CA signalling test are specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For NR CA Inter-band case (2 bands) the NR CA configurations are specified in clause 4.3.1.1.2 for NR CA within FR1, in clause 4.3.1.2.2 for NR CA within FR2 and in clause 4.3.1.3.1 for NR CA between FR1and FR2. NR test frequencies are specified in clause 6.2.3.1 for the NR band used as PCell (NRf1, NRf2, NRf3, NRf4) and for the NR band used as Scell (NRf5, NRf6, NRf7).

For NR CA Intra-band Contiguous case (2 CCs) the NR CA configurations and the test frequencies are specified in Table 6.2.3.4-1 for FR1 and in Table 6.2.3.4-2 for FR2. For NR CA Intra-band Contiguous case (3 CCs) the NR CA configurations and the test frequencies are specified in Table 6.2.3.4-2a for FR2. For NR CA Intra-band Non-Contiguous (2 CCs) case the NR CA configurations and test frequencies are specified in Table 6.2.3.4-3 for FR1 and in Table 6.2.3.4-4 for FR2.

For NR CA Intra-Band Contiguous case (2 CCs and 3CCs) and NR CA Intra-Band Non-Contiguous case (2 CCs) the mapping of frequency ranges to NR test frequencies NRf1, NRf2, NRf3, and NRf4 and PCell (CC1) and SCell (CC2, CC3) are as follows:

-	for Intra-band configurations with only one test frequency: Low Range (NRf1=CC1 and NRf2=CC2); and

-	for Intra-band configurations with up to two frequencies: Low Range (NRf1=CC1 and NRf2=CC2), High Range (NRf3=CC1 and NRf4=CC2); and

-	for Intra-band configurations with up to three frequencies: Low Range (NRf1=CC1, NRf2=CC2, NRf3=CC3), High Range (NRf4=CC1, NRf5=CC2 and NRf6=CC3).

Table 6.2.3.4-1: Test frequencies for NR CA Intra-band Contiguous configurations with FR1

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41C | CC1 | 60 | 162+162 | Downlink | Low | Same values as for Low range in Table 4.3.1.1.3.41.1-1 for CBW combination 60+60 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 162 | & Uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48B | CC1 | 10 | 24 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.3.48.1-2 for CBW combination 10+10 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 24 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66B | CC1 | 10 | 52 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.3.66.1-1 for CBW combination 10+15 and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 79 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77C | CC1 | 100 | 273 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.3.77.1-1 for CBW combination 100+100 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 273 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78C | CC1 | 100 | 273 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.3.78.1-1 for CBW combination 100+100 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 273 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-2: Test frequencies for NR CA Intra-band Contiguous configurations with FR2

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n257G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.1.6-4 for CBW combination 100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n258G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.2.6-2 for CBW combination 100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.4.6-2 for CBW combination 100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.5.6-2 for CBW combination 100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-2a: Test frequencies for NR CA Intra-band Contiguous configurations with FR2 (3CC)

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n257H | CC1, CC2,CC3 | 100+100+100 | 66+66+66 | Downlink & Uplink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.1.7-4 for CBW combination 100+100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n258H | CC1, CC2,CC3 | 100+100+100 | 66+66+66 | Downlink & Uplink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.2.7-2 for CBW combination 100+100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260H | CC1, CC2,CC3 | 100+100+100 | 66+66+66 | Downlink & Uplink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.4.7-1 for CBW combination 100+100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261H | CC1, CC2,CC3 | 100+100+100 | 66+66+66 | Downlink & Uplink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.5.7-2 for CBW combination 100+100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-3: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR1

| NR CA configuration | SB | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A) | SB1 | 10+10 | 24+24 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.48-2 for CBW combination 10+10 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A) | SB1 | 10+10 | 52+52 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.66-1 for CBW combination 10+10 and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n71(2A) | SB1, | 10+10 | 52+52 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.71-1 for CBW combination 10+10 and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78(2A) | SB1 | 50+50 | 133+ | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.78-2 for CBW combination 50+50 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 |  | 133 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

End of Changes>