3GPP TSG-RAN5 Meeting #96-e	R5-225647
Electronic Meeting, 15th August – 26th August 2022

Title: 	Discussion on Rel-16 Power Boost measurement uncertainties and test tolerances

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.15.8

Document for:	Discussion and Endorsement

# 1.	Introduction

As part of the Rel-16 FR2 RF enhancements, RAN4 has defined a mechanism to enable UL power boosting in exchange for suspended In-Band Emissions requirements ([1], [2], [3], [4]).

[5] provides an analysis on how to introduce such core requirements into test specifications ([6]).

[7], [8], [9] provides CRs to 38.521-2 ([6]) and 38.905 ([10]) proposing concrete changes in test specifications to enable testing on Rel-16 FR2 RF enhancements for Power Boost.

This document makes a brief analysis on how measurement uncertainties and test tolerances could be defined for the new test proposed in [7] (6.2.4_1 Configured transmitted power with Power Boost).

# 2.	Discussion

According to [7], measurements performed in new test case 6.2.4_1 Configured transmitted power with Power Boost will be very similar to the ones performed for EIRP in test case 6.2.1.1 maximum output power- EIRP and TRP. The main difference is that the minimum EIRP power value measured now will be ΔPIBE=1.0 dB higher than in test 6.2.1.1.

This implies that:

- Noise impact will be reduced when compared to noise impact of test case 6.2.1.1 in those test systems implementations not requiring attenuation adjustment to keep amplifier compression.

- Noise impact will be kept equal to the one defined for 6.2.1.1 in those test systems implementations requiring attenuation adjustment to keep amplifier compression.

Worst case will be the second one.

Proposal 1: Define measurement uncertainties for new test case 6.2.4_1 Configured transmitted power with Power Boost similar to the ones defined for 6.2.1.1 maximum output power- EIRP and TRP.

On the other hand, test tolerances formula for new test case 6.2.4_1 Configured transmitted power with Power Boost can be leveraged from the one defined for 6.2.1.1 maximum output power- EIRP and TRP, i.e. TT = 0.60 x (MTSU - 0.1).

Proposal 2: Define test tolerances for new test case 6.2.4_1 Configured transmitted power with Power Boost as TT = 0.60 x (MTSU – influence of noise).

# 3. 	Conclusion

This document makes a brief analysis on how measurement uncertainties and test tolerances could be defined for the new test proposed in [7] (6.2.4_1 Configured transmitted power with Power Boost) and makes the following proposals:

Proposal 1: Define measurement uncertainties for new test case 6.2.4_1 Configured transmitted power with Power Boost similar to the ones defined for 6.2.1.1 maximum output power- EIRP and TRP.

Proposal 2: Define test tolerances for new test case 6.2.4_1 Configured transmitted power with Power Boost as TT = 0.60 x (MTSU – influence of noise).

All above proposals are implemented in [7], [11] and [12].

Similar approach has been followed for corresponding EN-DC test case in [13].

# 4.	References

[1] 3GPP TS 38.101-2 v16.12.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[2] R4-2011905, “CR to 38.101-2: FR2 UE EIRP increase with IBE relaxation”, Qualcomm, Nokia, AT&T, T-Mobile, Verizon, RAN4#96-e

[3] R4-1912975, “WF on UL Power Boost and IBE Relaxation”, Qualcomm, Skyworks, RAN4#92-bis, Chongqing, China

[4] R4-2006782, “FR2 UE EIRP increase with IBE relaxation”, Qualcomm, Nokia, AT&T, T-Mobile, Verizon, RAN4#95-e.

[5] R5-224046, “Discussion on Rel-16 Power Boost test definition”, Keysight Technologies UK, RAN5#96-e

[6] 3GPP TS 38.521-2 v16.12.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

[7] R5-224071, “CR to 38.521-2 on New test case addition: 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[8] R5-224072, “CR to 38.521-2: In-band emissions minimum conformance requirements update”, Keysight Technologies UK, RAN5#96-e.

[9] R5-224073, “CR to 38.905: Test point analysis for new test 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[10] 3GPP TS 38.905 v17.5.0 Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases

[11] R5-224076, “CR to 38.521-2 on Measurement uncertainties and test tolerances for New test case addition: 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[12] R5-224075, “CR to 38.903 on Measurement uncertainties for New test case addition: 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[13] R5-224077, “CR to 38.521-3 on Measurement uncertainties and test tolerances for New test case addition: 6.2B.4.1.4_1 Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC)”, Keysight Technologies UK, RAN5#96-e.