3GPP TSG-RAN5 Meeting #98	R5-230207r1

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2686 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Propagation Delay Compensation tables for UE Rx-Tx measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Content of PDC tables is FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 4.6.3-76B: MeasObjectRxTxDiff has been updated. Table 4.6.3-79E: MeasResultRxTxTimeDiff has been updated. Table 4.6.3-84AC: NR-DL-PRS-PDC-Info has been updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE conformance tests for PDC cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.522 CR 0252 TS/TR 38.533 CR 2155, 2156, 2157 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Corrections in table MeasObjectRxTxDiff and NR-DL-PRS-PDC-Info. Updated other specs affected. |  |  |  |  |  |  |  |  |

## 4.6	Default NG-RAN RRC message and information elements contents

### 4.6.3	Radio resource control information elements

### –	MeasObjectRxTxDiff

----- Start of changes -----

Table 4.6.3-76B: MeasObjectRxTxDiff

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectRxTxDiff-r17 ::= SEQUENCE { |  |  |  |
| dl-Ref-r17 CHOICE { |  |  |  |
| prs-Ref-r17 | NULL |  | PRS_REF |
| csi-RS-Ref-r17 | NULL |  | CSI-RS_REF |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PRS_REF | Test cases using PRS reference signal to measure Rx-Tx time difference |
| CSI-RS_REF | Test cases using CSI-RS for tracking to measure Rx-Tx time difference |

----- End of changes ------

–	MeasResultRxTxTimeDiff

----- Start of changes -----

Table 4.6.3-79E: MeasResultRxTxTimeDiff

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultRxTxTimeDiff-r17 ::= SEQUENCE { |  |  |  |
| rxTxTimeDiff-ue-r17 SEQUENCE { |  |  |  |
| result-k5-r17 | INTEGER (0..61565) | Integer value for result-k5 measurements |  |
| } |  |  |  |
| } |  |  |  |

----- End of changes ------

–	NR-DL-PRS-PDC-Info

----- Start of changes -----

Table 4.6.3-84AC: NR-DL-PRS-PDC-Info

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-PDC-Info-r17 ::= SEQUENCE { |  |  |  |
| NR-DL-PRS-PDC-ResourceSet-r17 SEQUENCE { |  |  |  |
| periodicityAndOffset-r17 CHOICE { |  |  |  |
| scs15-r17 CHOICE { |  | entry 1 | SCS15 |
| n160-r17 | 10 |  |  |
| } |  |  |  |
| scs30-r17 CHOICE { |  | entry 2 | SCS30 |
| n160-r17 | 10 |  |  |
| } |  |  |  |
| scs120-r17 CHOICE { |  | entry 3 | SCS120 |
| n160-r17 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| numSymbols-r17 | n4 |  |  |
| dl-PRS-ResourceBandwidth-r17 | 8 |  | SCS15 |
|  | 22 |  | SCS30 |
|  | 11 |  | SCS120 |
| dl-PRS-StartPRB-r17 | 0 |  |  |
| resourceList-r17 SEQUENCE { |  |  |  |
| NR-DL-PRS-Resource-r17[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r17 | 0 |  |  |
| dl-PRS-SequenceID-r17 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r17 CHOICE { |  |  |  |
| n4-r17 | 0 |  | SCS15 OR SCS30 |
| n2-r17 | 0 |  | SCS120 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r17 | 4 |  |  |
| dl-PRS-ResourceSymbolOffset-r17 | 0 |  |  |
| dl-PRS-QCL-Info-r17 CHOICE { |  |  |  |
| ssb-r17 SEQUENCE { |  |  |  |
| ssb-Index-r17 | 0 | SSB#0 |  |
| rs-Type-r17 | typeD |  | SCS120 |
|  | Not present |  | SCS15 or SCS30 |
| } |  |  |  |
| dl-PRS-r17 SEQUENCE { |  |  |  |
| qcl-DL-PRS-ResourceID-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| RepFactorAndTimeGap-r17 SEQUENCE { |  |  |  |
| repetitionFactor-r17 | n2 |  |  |
| timeGap-r17 | s1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

----- End of changes ------