3GPP TSG-RAN5 Meeting #98	R5-230936r1

# Athens, Greece, 27th February 2023 - 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5181 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New MUSIM TC 9.3.1.20- Service Request / MUSIM / Rejection of paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-2-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of new MUSIM TC 9.3.1.20 in accordance with work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of New MUSIM TC 9.3.1.20- Service Request / MUSIM / Rejection of paging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.20(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1394 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact 4.5.XX to be resolved based on clause number from TS 36.508 CR 1410. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Editorial update. |  |  |  |  |  |  |  |  |

<Start of Change>

9.3.1.20	Service Request / MUSIM / Rejection of paging

9.3.1.20.1	Test Purpose (TP)

(1)

with { UE in EMM-IDLE state and has transmitted an EXTENDED SERVICE REQUEST message and set the Request type to "Rejection of paging" in the UE request type IE and Service type to "packet services via S1" }

ensure that {

when { the UE receives SERVICE ACCEPT message }

then { the UE enters the state EMM-REGISTERED }

}

9.3.1.20.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.6.1.4.1 and 5.6.2.2.1.1. Unless otherwise stated these are Rel-17 requirements.

[TS 24.301, clause 5.6.1.4.1]

…

For cases o, p and q in clause 5.6.1.1, when the MUSIM UE in the EXTENDED SERVICE REQUEST message sets the Request type to "NAS signalling connection release" or to "Rejection of paging" in the UE request type IE, the UE shall treat the receipt of SERVICE ACCEPT message as the successful completion of the procedure and the UE shall reset the service request attempt counter, stop timer T3417 and enter the state EMM-REGISTERED.

If the SERVICE REQUEST message or an EXTENDED SERVICE REQUEST message for packet services was used, the UE shall locally deactivate the EPS bearer contexts that do not have a user plane radio bearer established upon successful completion of the service request procedure, except for the case when the MUSIM UE in the EXTENDED SERVICE REQUEST message sets the Request type to "NAS signalling connection release" or to "Rejection of paging" in the UE request type IE.

…

[TS 24.301, clause 5.6.2.2.1.1]

…

Upon reception of a paging indication, if the network supports the reject paging request and the MUSIM UE decides not to accept the paging, the UE may initiate a service request procedure to reject the paging as specified in clause 5.6.1.1.

…

9.3.1.20.3	Test description

9.3.1.20.3.1	Pre-test conditions

System Simulator:

-	Cell A (home PLMN1) and Cell G (home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

UE:

-	The UE is a MUSIM UE;

-	Each USIM of the UE is configured to initiate EPS attach;

-    The UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Tables 4.5.XX.1-1 and 4.5.XX.1-2.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.5.XX.3-1 of  TS 36.508 [18].

9.3.1.20.3.2	Test procedure sequence

Table 9.3.1.20.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | UE is configured to reject Paging (Note 1) | - | - | - | - |
| 1 | Set the cell type of cell A to the " Serving cell" Set the cell type of cell G to the "Non-Suitable cell". | - | - | - | - |
| - | The following messages are sent and shall be received on Cell A. | - | - | - | - |
| 2 | The SS sends a paging message. | <-- | Paging | - | - |
| 3 | The UE transmits an EXTENDED SERVICE REQUEST message and set the Request type to "Rejection of paging" in the UE request type IE and Service type to "packet services via S1" on Cell A. | --> | EXTENDED SERVICE REQUEST | - | - |
| 4 | The SS transmits a SERVICE ACCEPT message on Cell A | <-- | SERVICE ACCEPT | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.4 indicate that the UE is in EMM-REGISTERED state on Cell A? | - | - | 1 | P |
| Note 1: This step is triggered by MMI or AT command |  |  |  |  |  |

9.3.1.20.3.3	Specific message contents

Table 9.3.1.20.3.3-1: EXTENDED SERVICE REQUEST (step 3, Table 9.3.1.20.3.2)

| Derivation Path: 36.508 Table 4.7.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  | ‘1000’B | packet services via S1 |  |
| UE request type |  |  |  |  |
| UE request type IEI |  | 29 |  |  |
| Length of UE request type contents |  | ‘00000001’B |  |  |
| Request type |  | ‘00000010’B | Rejection of paging |  |

<End of Change>