3GPP TSG-RAN5 Meeting #98	R5-231033r2

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5189 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of IoT NTN TC 8.5.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Lack of coverage of T318 stopped in UE side as specified in TS 36.331 clause 5.3.18. - Timing is SS side needs to be clarified. - SIB31 needs to be resumed before re-establishment as mentioned in TS 36.331 clause 5.2.2.4. - Steps in the last need to be extend for a successful re-establishment. - UE postion needs to be pre-configured in the preamble. - T318 is same as the default value in TS 36.508 Table 4.6.3-14. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Steps 27A-27E was added to cover T318 stop requirement. - Timers in steps were clarified and Notes were added. - Step 28A was added tto resume SIB31 broadcasting. - Steps 30-31 were added to complete the re-establishment procedure. - UE postion was pre-configured in the preamble. - Table 8.5.6.1.3.3-1 was set to void. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: updated the description in the preamble for UE position. Step 26 description was updated. r2: add a new TP2 to verify the coverage of T318 stopped and UIL sync. Restored. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

#### 8.5.6.1	eMTC / NTN / Ephemeris information update / T317 Expiry / T318 Expiry

8.5.6.1.1	Test Purpose (TP)

(1)

with { UE has acquired SystemInformationBlockType31 and starts timer T317 with the duration ul-SyncValidityDuration }

ensure that {

when { T317 expires in RRC_CONNECTED state }

then { UE informs lower layers that the UL synchronisation is lost and start T318 }

}

(2)

with { UE has starts T318 in RRC_CONNECTED state }

ensure that {

when { SystemInformationBlockType31 acquire successfully }

then { UE stops T318 and informs lower layers that the UL synchronization is restored }

}

(3)

with { UE has starts T318 in RRC_CONNECTED state }

ensure that {

when { T318 expires }

then { UE considers radio link failure and initiates the connection re-establishment procedure }

}

8.5.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.2.1.3, 5.2.2.4, 5.2.2.39, 5.3.18 and 5.3.11.3 Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.2.1.3]

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

NOTE 3:	When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

[TS 36.331, clause 5.2.2.4]

The UE shall:

…

1>	if the UE is NTN capable:

2>	if schedulingInfoList indicates that SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is present:

3>	immediately before establishing, resuming or re-establishing an RRC connection; or

3>	if in RRC_CONNECTED and T317 is not running:

4>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

[TS 36.331, clause 5.2.2.39]

Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall:

1>	start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime.

[TS 36.331, clause 5.3.18]

The UE shall:

1>	if in RRC_CONNECTED:

2>	inform lower layers that the UL synchronisation is lost;

2>	start timer T318;

2>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) as specified in 5.2.2;

2>	upon successful acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT):

3>	stop timer T318;

3>	inform lower layers that the UL synchronisation is restored;

NOTE:	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) may be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE.

[TS 36.331, clause 5.3.11.3]

The UE shall:

…

- upon T318 expiry; or

…

2>	consider radio link failure to be detected for the MCG i.e. RLF;

…

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

8.5.6.1.3	Test description

8.5.6.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 as specified in TS 36.508 [18] clause 4.4.1.1 is configured.

-	System information combination 32 as defined in TS 36.508 [18] clause 4.4.3.1.1 is used.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.xx.

Preamble:

-	The UE is in in state Switched OFF (State 1) according to TS 36.508 [18] clause 4.5.1.

8.5.6.1.3.2	Test procedure sequence

Table 8.5.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | UE acquires SystemInformationBlockType31 and start T317 = 60 seconds. | - | - | - | - |
| 3-20 | Steps 1 to 18 of the registration and test mode activated procedure described in TS 36.508 [18] subclause 4.5.2A.3 with TEST LOOP MODE A are performed on Cell 1. Note: After receiving the message in step3, SS start a timer T1 with value = T317. | - | - | - | - |
| 20A-20B | Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 4.5.4.3 are performed on Cell 1. | - | - | - | - |
| 21 | SS stops broadcasting SystemInformationBlockType31. | - | - | - | - |
| 22 | Wait until expiry of T1. Note: UE starts timer T318. SS start a timer T2 with value = T318. | - | - | - | - |
| 23-24 | Void | - | - | - | - |
| 25 | The SS transmits a MAC PDU containing an RLC SDU. | <-- | MAC PDU (RLC SN=0) | - | - |
| 26 | UL grants assigned at the next available USS, allowing the UE to return the RLC SDU as received in step 25. | <-- | (UL Grant (known C-RNTI)) | - | - |
| 27 | Check: Does the UE transmit a MAC PDU corresponding to grant in step 26? | --> | MAC PDU | 1 | F |
| 27A | SS resumes broadcasting SystemInformationBlockType31. Note: Actually timer T318 shall be stopped and T317 shall be started in UE side. SS start a timer T3 with value = T317. | - | - | - | - |
| 27B | Check: Does the UE transmit a MAC PDU corresponding to grant in step 27C? | --> | MAC PDU | 2 | P |
| 27C | SS stops broadcasting SystemInformationBlockType31. | - | - | - | - |
| 27D | Wait until expiry of timer T3 in SS side. Note: UE starts timer T318. SS starts a timer T4 with value = T318 | - | - | - | - |
| 28 | Wait until expiry of T4. | - | - | - | - |
| 28A | SS resumes broadcasting SystemInformationBlockType31. | - | - | - | - |
| 29 | Check: Does the UE transmit an RRCConnectionReestablishmentRequest message on SRB0? | --> | RRCConnectionReestablishmentRequest | 2 | P |
| 30 | The SS transmits a RRCConnectionReestablishment message on SRB0 | <-- | RRCConnectionReestablishment | - | - |
| 31 | The UE transmits RRCConnectionReestablishmentComplete message on SRB1. | --> | RRCConnectionReestablishmentComplete | - | - |

8.5.6.1.3.3	Specific message contents

Table 8.5.6.1.3.3-1: Void

Table 8.5.6.1.3.3-2: SystemInformationBlockType31 (steps 2, 27A and 28A, table 8.5.6.1.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ul-SyncValidityDuration-r17 | s60 | Value of T317 |  |
| } |  |  |  |
| } |  |  |  |

[End of change]