3GPP TSG-RAN5 Meeting #98	R5-230958

# Athens, GR, 27 Feb 2023 - 03 Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5182 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MUSIM test case 9.2.1.1.33 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of MUSIM test case 9.2.1.1.33 in accordance with work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new MUSIM test case 9.2.1.1.33: Attach / Success / MUSIM / IMSI offset |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.1.33(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1394 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

9.2.1.1.33	Attach / Success / MUSIM / IMSI offset

9.2.1.1.33.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message containing the Requested IMSI offset IE }

ensure that {

when { the UE receives the Negotiated IMSI offset IE in the ATTACH ACCEPT message }

then { the UE receives a paging message in a valid new paging occasion which calculated based on the Negotiated IMSI offset }

}

(2)

with { UE has transmited ATTACH REQUEST message containing the Requested IMSI offset IE }

ensure that {

when { the UE receives the ATTACH ACCEPT message without Negotiated IMSI offset IE }

then { the UE receives a paging message in a valid old paging occasion which calculated without any IMSI offset }

}

9.2.1.1.33.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.5.1.2.4 and in TS 23.401, clause 4.3.33.5.

[TS 24.301, clause 5.5.1.2.4]

…

If the MUSIM UE has included a Requested IMSI offset IE in the ATTACH REQUEST message and if the MME supports paging timing collision control, the MME shall include the Negotiated IMSI offset IE in the ATTACH ACCEPT message, and the MME shall set the IMSI offset value to:

-	A value that is different than what the UE has provided, if the MME has a different value; or

-	A value that is same as what the UE has provided, if the MME does not have a different value;

and the MME shall calculate an alternative IMSI value using the IMSI offset value and store it in the UE context as specified in 3GPP TS 23.401 [10]. The alternative IMSI value is used for deriving the paging occasion as specified in 3GPP TS 36.304 [21].

…

If the ATTACH ACCEPT message contains Negotiated IMSI offset IE, the MUSIM UE shall forward the IMSI offset value to lower layers. If the ATTACH ACCEPT message does not contain Negotiated IMSI offset IE, the MUSIM UE shall indicate to lower layers to erase any IMSI offset value, if available.

…

[TS 23.401, clause 4.3.33.5]

To avoid possible paging occasion collision and to enhance the likelihood that paging is received successfully for different USIMs, a Multi-USIM UE may provide, for at least one USIM, a Requested IMSI Offset value that is used for the determination of paging occasions. Upon reception of a Requested IMSI Offset value from UE in Attach Request or Tracking Area Update Request, a supporting MME provides an Accepted IMSI Offset value to the UE in the Attach Accept or Tracking Area Update Accept message to acknowledge it supports the feature and provide the accepted value. The Accepted IMSI Offset Value may be different from the Requested IMSI Offset provided by the UE. The Alternative IMSI value, determined as below, is stored in the UE context in the MME. If the UE does not provide any Requested IMSI Offset value in Attach Request or Tracking Area Request, the MME removes any stored Alternative IMSI value in the UE context. The UE and the network use the Accepted IMSI Offset to determine the paging occasion. The UE and MME use the Accepted IMSI Offset value to calculate the Alternative IMSI value that is determined based on UE's IMSI as follows:

Alternative IMSI value = [MCC] [MNC] [(MSIN value + Accepted IMSI Offset) mod (MSIN address space)]

where: the MCC, MNC and MSIN value are the fields of the UE's IMSI as defined in TS 23.003 [9].

The MME uses the Alternative IMSI value to compute the UE Identity Index Value. The MME sends the UE Identity Index Value to RAN in the Paging message (see TS 36.413 [36]) for RAN to derive the paging occasions according to TS 36.304 [34].

The UE uses the Alternative IMSI value for the determination of paging occasions as specified in TS 36.304 [34].

NOTE 1:	It is recommended to avoid excessive signalling load from UE due to this procedure.

NOTE 2:	The MME does not remove Alternative IMSI value if the Tracking Area Update Request is for periodic Tracking Area Update.

9.2.1.1.33.3	Test description

9.2.1.1.33.3.1	Pre-test conditions

System Simulator:

-	Cell A(home PLMN1) and Cell G(home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3 in TS 36.508 [18] and belong to the different frequency as specified in TS 36.523-3 [20].

UE:

-	The UE is a MUSIM UE;

-	Each USIM of the UE is configured to initiate EPS attach;

-	The UE is equipped with two USIMs with configuration as defined in TS 36.508 [18] Tables 4.5.XX.1-1 and 4.5.XX.1-2.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.33.3.2	Test procedure sequence

Table 9.2.1.1.33.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Configure the UE to perform ATTACH including a Requested IMSI offset IE. (Note1) | - | - | - | - |
| 2 | The SS configures: - Cell A as the "Serving cell". - Cell G as a "Non-Suitable Off cell". | - | - | - | - |
| 3 | The UE is switched on. | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 4 | The UE transmits an RRCConnectionRequest message not including S-TMSI and with establishmentCause set to ‘mo-Signalling’. | - | - | - | - |
| 5 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionSetupComplete message with ATTACH REQUEST message including a Requested IMSI offset IE? (Note1) | --> | ATTACH REQUEST | - | - |
| 7-15 | Steps 5-13 of the generic procedure of UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 16 | The SS transmits an ATTACH ACCEPT with the Negotiated IMSI offset IE. | <-- | ATTACH ACCEPT |  |  |
| 17-20b1 | Steps 15-18b1 of the generic procedure registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 21 | The SS transmits a Paging message in a PO which calculated based on the Negotiated IMSI offset. | - | - | - | - |
| 22-31 | Check: Does the UE respond to Paging according to procedure TS 36.508 subclause 6.4.2.2 steps 2 to 11? | - | - | 1 | P |
| 32 | The SS configures: - Cell G as a "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 33 | The UE transmits an RRCConnectionRequest message not including S-TMSI and with establishmentCause set to ‘mo-Signalling’. | - | - | - | - |
| 34 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 35 | Check: Does the UE transmit an RRCConnectionSetupComplete message with ATTACH REQUEST message including a Requested IMSI offset IE? (Note1) | --> | ATTACH REQUEST | - | - |
| 36-44 | Steps 5-13 of the generic procedure registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 45 | The SS transmits an ATTACH ACCEPT without the Negotiated IMSI offset IE. | <-- | ATTACH ACCEPT | - | - |
| 46-49b1 | Steps 15-18b1 of the generic procedure registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 49 | The SS transmits a Paging message in a PO which calculated without the Negotiated IMSI offset. | - | - | - | - |
| 50-59 | Check: Does the UE respond to Paging according to procedure TS 36.508 subclause 6.4.2.2 steps 2 to 11? | - | - | 2 | P |
| Note 1: The ATTACH REQUEST message including a Requested IMSI offset IE may be performed by MMI or AT command. |  |  |  |  |  |

9.2.1.1.33.3.3	Specific message contents

Table 9.2.1.1.33.3.3-1: Message ATTACH REQUEST (step 6 and 35, Table 9.2.1.1.33.3.2-1)

| Derivation path: TS 36.508 table 4.5.XX.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested IMSI offset | Any allowed value |  |  |

Table 9.2.1.1.33.3.3-2: Message ATTACH ACCEPT (step 16, Table 9.2.1.1.33.3.2-1)

| Derivation path: TS 36.508 table 4.5.XX.4-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated IMSI offset | Same as what the Requested IMSI offset IE |  |  |

<End of Changed Section>