3GPP TSG-RAN WG5 Meeting #98	R5-231682

# Athens, Greece, 27th Feb – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1553 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update 7.3B.2.3 for DC_71_n2A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reference sensitivity testing for DC_71A_n2A is not available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added reference sensitivity testing for DC_71A_n2A is not available. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis is in R5- 231613 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-230904 – Updated TP analysis CR number. Updated TP# 1 based on offlline discussion |  |  |  |  |  |  |  |  |

<< Start of changes >>

#### 7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

<< Unchanged sections skipped>>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  | SCG -NR |  |  |  |  |  |  |
|  | Band | Range | NRB |  |  | Band | Range |  |  | NRB |  |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc |  | UL MOD | DL MOD |  |  | UL/DL Ch BW | DLalloc / UL alloc |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 | Mid |  |  |  | n3 | Mid |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 |  | N/A | CP-OFDM QPSK |  |  | 40 MHz | All RBs / 0 |  |
| 24,10 | 1 | UL 1950 / DL 2140 |  |  |  | n3 | UL 1760 / DL 1855 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 36 | 1 | Low |  |  |  | n3 | High |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 |  | N/A | CP-OFDM QPSK |  |  | 40 MHz | All RBs / 0 |  |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1965 / DL 2155 |  |  |  | n8 | UL 887.5 / DL 932.5 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950/ DL 2140 |  |  |  | n77 | UL/DL 3900 |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 1 | UL 1950/ DL 2140 |  |  |  | n77 | UL/DL 3870 |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 1 | UL 1950/ DL 2140 |  |  |  | n77 | UL/DL 4090.005 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | N/A | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 1 | UL 1950/ DL 2140 |  |  |  | n77 | UL/DL 3709.005 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | N/A | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1950 / DL 2140 |  |  |  | n78 | UL/DL 3709.995 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_2A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 2 | High |  |  |  | n41 | Low |  |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / REFSENS_LTE |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 2 | UL 1855 / DL 1935 |  |  |  | n66 |  | DL 2175 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 2 | UL 1883.3 /DL1963.3 |  |  |  | n66 |  | DL 2150 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | DL 1980 MHZ |  |  |  | n71 | UL 665.5 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 23 | 2 | DL1980 MHZ |  |  |  | n71 | UL 673 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 35 | 2 | UL 1881 MHZ |  |  |  | n71 | Low |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 20 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1860 |  |  |  | n77 | UL/DL 3720 |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 2 | UL 1860 |  |  |  | n77 | UL/DL 3690.005 |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 20 MHz | All RBs / 0 |  |  |
| 3 8 | 2 | UL 1860 |  |  |  | n77 | UL/DL 3850.005 |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 10 MHz | All RBs / 0 |  |  |
| 45 | 2 | DL Mid |  |  |  | n77 | UL/DL 3920.01 |  |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 4a 9 | 2 | DL Mid |  |  |  | n77 | UL/DL 3999.99 |  |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 2 | UL 1855/DL 1935 |  |  |  | n77 | UL/DL 3790.005 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 64 | 2 | UL 1900/DL 1980 |  |  |  | n77 | UL/DL 3720 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 74 | 2 | UL 1885/DL 1965 |  |  |  | n77 | UL/DL 3810 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1870 |  |  |  | n78 | UL/DL 3740.01 |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 2 | UL 1885 |  |  |  | n78 | UL/DL 3739.995 |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 2 | UL 1855/ DL 1935 |  |  |  | n78 | UL/DL 3790.005 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 2 | UL 1885/ DL 1965 |  |  |  | n78 | UL/DL 3690 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 | Mid |  |  |  | n1 | Mid |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |
| 26 | 3 | High |  |  |  | n1 | Low |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |
| 34 | 3 | UL 1760 / DL 1855 |  |  |  | n1 | UL 1950 / DL 2140 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1771 / DL 1866 |  |  |  | n5 |  | UL 838 / DL 883 |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 | 3 | UL 1721 / DL 1816 |  |  |  | n5 |  | UL 838 / DL 883 |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1730 / DL 1825 |  |  |  | n7 |  | UL 2535 / DL 2655 |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1755 / DL 1850 |  |  |  | n8 |  | UL 900 / DL 945 |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 | 3 | UL 1747.5 / DL 1842.5 |  |  |  | n8 |  | UL 897.5 / DL 942.5 |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | High |  |  |  | n41 | Low |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| 26 | 3 | High |  |  |  | n41 | Low |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |
| 34, 10 | 3 | UL 1740 / DL 1835 |  |  |  | n41 | UL/DL 2657.5 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for DC_3A_n77A and DC_3A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  | n78 | UL/DL 3495 |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| 23 | 3 | Mid |  |  |  | n78 | UL/DL 3525 |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 20 MHz | All RBs / 0 |  |
| 35 | 3 | Mid |  |  |  | n78 | UL/DL 3685.005 |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |
| 39 | 3 | Low |  |  |  | n78 | High |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs/0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |
| 44 | 3 | UL 1740 / DL 1835 |  |  |  | n78 | UL/DL 3574.995 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 54 | 3 | UL 1765 / DL 1860 |  |  |  | n78 | UL/DL 3435 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 5 | UL 838/ DL 883 |  |  |  | n66 | UL 1721/ DL 2121 |  |  |  |  |  |  |
|  | QPSK | N/A | 5 MHz | All RBs / REFSENS_ENDC_4 |  | N/A | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 837.5 |  |  |  | n77 | UL/DL 3350.01 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 5 | UL 829 |  |  |  | n77 | UL/DL 4145.01 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 38 | 5 | UL Mid (836.5 MHz) 840 |  |  |  | n77 | UL/DL Mid |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 44, 13 | 5 | UL 844/DL 889 |  |  |  | n77 | UL/DL 3421.005 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 54, 13 | 5 | UL 826.5/DL 871.5 |  |  |  | n77 | UL/DL 4177.5 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 840 |  |  |  | n78 | UL/DL 3360 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 5 | UL 840 |  |  |  | n78 | UL/DL 3560.01 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 5 | UL 844/DL 889 |  |  |  | n78 | UL/DL 3421.005 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  |  | n3 | DL 1825 |  |  |  |  |  |
|  | QPSK | N/A | 10 MHz | N/A |  | N/A | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2667 |  |  |  | n5 | DL 879 |  |  |  |  |  |
|  | QPSK | N/A | 10 MHz | N/A |  | N/A | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 | High |  |  |  | n78 | Low |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 932.5 |  |  |  | n1 | DL 2155 |  |  |  |  |  |
|  | QPSK | N/A | 5 MHz | N/A |  | N/A | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 945 |  |  |  | n3 | DL 1850 |  |  |  |  |  |
|  | QPSK | N/A | 5 MHz | N/A |  | N/A | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 | 8 | DL 942.5 |  |  |  | n3 | DL 1842.5 |  |  |  |  |  |
|  | QPSK | N/A | 5 MHz | N/A |  | N/A | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | UL 890.5 / DL 935.5 |  |  |  | n20 | UL 849.5 / DL 808.5 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 | 8 | UL 892.5 / DL 937.5 |  |  |  | n20 | UL 847.5 / DL 806.5 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for DC_8A_n77A and DC_8A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | Mid |  |  |  | n77 | UL/DL 3590.01 |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| 2,8 | 8 | Mid |  |  |  | n77 | UL/DL 3520.005 |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| 34 | 8 | UL 897.5 |  |  |  | n77 | UL/DL 3634.995 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for DC_8A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | UL 900 |  |  |  | n79 | UL/DL 4500 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 2,8 | 8 | UL 900 |  |  |  | n79 | UL/DL 4299.99 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 8 | UL 897.5 |  |  |  | n79 | UL/DL 4532.505 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 | Mid |  |  |  | n79 | UL/DL 4457.7 |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 2,9 | 11 | Mid |  |  |  | n79 | UL/DL 4512.69 |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_12A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | UL 710 |  |  |  | n66 | UL 1730 /DL 2130 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 40 MHz | All RBs / 0 |  |  |
| 28 | 12 | UL 710 |  |  |  | n66 | UL 1770 /DL 2170 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 | Mid |  |  |  | n77 | UL/DL 3909.99 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 13 | Mid |  |  |  | n77 | UL/DL Mid |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 35 | 13 | Mid |  |  |  | n77 | UL/DL 3909.99 |  |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 49 | 13 | Mid |  |  |  | n77 | UL/DL 3834 |  |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 13 | UL 784.5/DL 753.5 |  |  |  | n77 | UL/DL 3891.495 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 | DL 884 |  |  |  | n79 | Low |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |
| 2,9 | 19 | DL 884 |  |  |  | n79 | UL/DL 4445.025 |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 840 / DL 799 |  |  |  | n3 | UL 1775 / DL 1870 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 | 20 | UL 847 / DL 806 |  |  |  | n3 | UL 1735 / DL 1830 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 851 / DL 810 |  |  |  | n7 | UL 2512 / DL 2632 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 849.5 / DL 808.5 |  |  |  | n8 | UL 892.5 / DL 937.5 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 | Mid |  |  |  | n78 | Mid |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| 23 | 20 | Mid |  |  |  | n78 | UL/DL 3387.99 |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| 34 | 20 | UL 850 / DL 809 |  |  |  | n78 | UL/DL 3358.995 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 | DL 1506 |  |  |  | n79 | UL/DL 4518 |  |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 21 | DL 1506 |  |  |  | n79 | UL/DL 4800 |  |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 21 | UL 1457.5/ DL 1505.5 |  |  |  | n79 | UL/DL 4420.5 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  | n41 | UL/DL 2524.5 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 50 MHz | All RBs / 0 |  |
| 2,8 | 26 | High |  |  |  | n41 | UL/DL 2572 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | QPSK |  |  | 50 MHz | All RBs / 0 |  |
| 34 | 26 | UL 839 |  |  |  | n41 | UL/DL 2562 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 45 | 26 | DL 886.6 |  |  |  | n41 | UL/DL 2660 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz | All RBs / REFSENS_LTE |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  | n77/n78 | UL/DL 3366 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| 2,8 | 26 | High |  |  |  | n77/n78 | UL/DL 3446.01 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| 34 | 26 | UL 836.5 |  |  |  | n77/n78 | UL/DL 3391.005 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 | High |  |  |  | n79 | UL/DL 4432.5 |  |  |  |  |  |
|  | N/A | QPSK | 15 MHz | All RBs / 0 |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  | 60 MHz | All RBs / REFSENS_ENDC_2 |  |
| 22,9 | 26 | High |  |  |  | n79 | UL/DL 4470.51 |  |  |  |  |  |
|  | N/A | QPSK | 15 MHz | All RBs / 0 |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  | 60 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 | High |  |  |  | n5 | Low |  |  |  |  |  |
|  | QPSK | QPSK | 20MHz | All RBs / REFSENS_ENDC_3 |  | N/A | CP-OFDM QPSK |  |  | 20 MHz | All RBs / 0 |  |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 28 | Low |  |  |  | n77/n78 | UL/DL 3540 |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 28 | Low |  |  |  | n77/n78 | Low |  |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 28 | UL 705.5 / DL 760.5 |  |  |  | n77/n78 | UL/DL 3582.495 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 45 | 28 | Low |  |  |  | n77 | UL/DL 3815.01 |  |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_30A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 30 | Low |  |  |  | n66 | High |  |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / REFSENS_ENDC_3 |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  | n78 | UL/DL 3525 |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |
| 2,9 | 40 | Low |  |  |  | n78 | Mid |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 | Low |  |  |  | n77/n78 | UL/DL 3750 |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 26 | 41 | High |  |  |  | n77/n78 | Low |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 36 | 41 | High |  |  |  | n77/n78 | Low |  |  |  |  |  |
|  | QPSK | QPSK | 20MHz | All RBs / REFSENS_ENDC_3 |  | N/A | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 48 | UL/DL 3555 |  |  |  | n66 | UL 1760 MHZ |  |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 23 | 48 | UL/DL 3565 |  |  |  | n66 | UL 1770 |  |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 38 | 48 | UL/DL 3600 |  |  |  | n66 | UL1760 |  |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 44 | 48 | UL/DL 3630 |  |  |  | n66 | UL 1715 / DL 2115 |  |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 | 66 | UL1775/ DL2175 |  |  |  | n2 | UL 1855 / DL 1935 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24, 10 | 66 | UL 1750/ DL 2150 |  |  |  | n2 | UL 1883.3 / DL 1963.3 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 | UL 1721/ DL 2121 |  |  |  | n5 | UL 838/ DL 883 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 | UL 1750/ DL 2150 |  |  |  | n71 | UL 675/ DL 629 |  |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 66 | UL 1720 |  |  | n77 |  |  |  | UL/DL 3440.01 |  |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK | 100 MHz |  | All RBs / 0 |  |
| 23 | 66 | UL 1720 |  |  | n77 |  |  |  | UL/DL 3410 |  |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK | 20 MHz |  | All RBs / 0 |  |
| 38 | 66 | UL Mid |  |  | n77 |  |  |  | UL/DL Mid |  |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK | 100 MHz |  | All RBs / 0 |  |
| 44 | 66 | UL 1775/ DL 2175 |  |  | n77 |  |  |  | UL/DL 3949.995 |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |
| 54 | 66 | UL 1760 / DL 2160 |  |  | n77 |  |  |  | UL/DL 3720 |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 66 | UL 1755 |  |  | n78 |  |  |  | UL/DL 3510 |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK | 100 MHz |  | All RBs / 0 |  |
| 23 | 66 | UL 1755 |  |  | n78 |  |  |  | UL/DL 3480 |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK | 20 MHz |  | All RBs / 0 |  |
| 34 | 66 | UL 1730/ DL2130 |  |  | n78 |  |  |  | UL/DL 3660 |  |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for DC_71A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 71 | UL 665.5 |  |  | n2 |  |  |  | DL 1987.5 |  |  |  |  |
|  | QPSK | QPSK | 5MHz | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  |
| 23 | 71 | UL 673 |  |  | n2 |  |  |  | DL1980 |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 10 MHz |  | All RBs / REFSENS_ENDC_1 |  |
| 35 | 71 | Low |  |  | n2 |  |  |  | UL 1881 |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 20 MHz |  | All RBs / 0 |  |
| 39 | 71 | High |  |  | n2 |  |  |  | UL 1881 |  |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 20 MHz |  | All RBs / 0 |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections skipped>>

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n772 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n772, 13 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n782 | 15 |  | -71.9+TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
| 2 | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.9 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | --95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| 3 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  |  |  | -74.1 +TT15 | -74.1 +TT15 | -74.0 +TT15 |  |  | -73.9 +TT15 | -74.0 +TT15 |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  |  |  | -74.4 +TT15 | -74.2 +TT15 | -74.2 +TT15 |  |  | -74.0 +TT15 | -74.1 +TT15 | --74.1 +TT15 | -74.0 +TT15 | -74.0 +TT15 | -74.0 +TT15 |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  |  |  |  | -74.8 +TT15 | -74.5 +TT15 | -74.4 +TT15 |  |  | -74.2 +TT15 | -74.2 +TT15 | -74.2 +TT15 | -74.1 +TT15 | -74.1 +TT15 | -74.1 +TT15 |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -96.9 +TT15 | -95.4 +TT15 | -94.6 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.2 +TT15 | -95.5 +TT15 | -94.8 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.6 +TT15 | -95.8 +TT15 | -95.0 +TT15 |  |  |  |  |  |  |  |  |
| 5 | n78 | 15 |  | -84.2 +TT | -84.4 +TT |  |  |  | -84.2 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -84.3 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.3 +TT | -84.5 +TT | -84.4 +TT | -84.3 +TT | -84.0 +TT | -84.2 +TT |
|  |  | 60 |  | -84.5 +TT | -84.6 +TT | -84.5 +TT |  |  | -84.5 +TT | -84.6 +TT | -84.5 +TT | -84.4 +TT | -84.1 +TT | -84.3 +TT |
| 8 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -84.0 +TT | -83.9 +TT |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -84.3 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 8 | n41 | 15 |  | -81.8 +TT | -81.7 +TT | -81.7 +TT |  |  | -81.6 +TT | -81.5 +TT |  |  |  |  |
|  |  | 30 |  | -82.1 +TT | -81.8 +TT | -81.9 +TT |  |  | -81.7 +TT | -81.6 +TT | -81.4 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
|  |  | 60 |  | -82.5 +TT | -82.1 +TT | -81.1 +TT |  |  | -81.9 +TT | -81.7 +TT | -81.6 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
| 8 | n794,5 | 15 |  |  |  |  |  |  | -82.8 +TT | -82.4 +TT |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  | -82.9 +TT | -82.5 +TT | -82.3 +TT | -81.7 +TT |  | -81.2 +TT |
|  |  | 60 |  |  |  |  |  |  | -83.1 +TT | -82.6 +TT | -82.4 +TT | -81.8 +TT |  | -81.3 +TT |
| 12 | n66 | 15 | -89.5 +TT | -88.8 +TT | -88.3 +TT | -87.8 +TT |  |  | -87.7 +TT |  |  |  |  |  |
|  |  | 30 |  | -89.1 +TT | -88.4 +TT | -88.0 +TT |  |  | -87.8 +TT |  |  |  |  |  |
|  |  | 60 |  | -89.5 +TT | -88.7 +TT | -88.2 +TT |  |  | -87.9 +TT |  |  |  |  |  |
| 12 | n78 | 15 |  | -85.4 +TT | -85.1 +TT | -84.9 +TT |  |  | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  | -85.7 +TT | -85.2 +TT | -85.1 +TT |  |  | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  | -86.1 +TT | -85.5 +TT | -85.3 +TT |  |  | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| 13 | n774, 5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 18， 19 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 28 | n51 | 15 | -72.2 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n774,5 n784,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 20 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n41 | 15 |  | -84.5 +TT | -84.6 +TT | -84.4 +TT |  |  | -83.6 +TT | -83.3 +TT | 3.9 +TT | 3.1 +TT | 2.7 +TT |  |
|  |  | 30 |  | -84.8 +TT | -84.7 +TT | -84.6 +TT |  |  | -83.7 +TT | -83.4 +TT | -83.0 +TT | -82.5 +TT | -82.4 +TT |  |
|  |  | 60 |  | -85.2 +TT | -85.0 +TT | -84.8 +TT |  |  | -83.9 +TT | -83.5 +TT | -83.2 +TT | -82.5 +TT | -82.4 +TT |  |
| 26 | n776,7 n786,7 | 15 |  | -85.0 +TT | -84.9 +TT | -84.7 +TT |  |  | -83.6 +TT | -84.6 +TT |  |  |  |  |
|  |  | 30 |  | -85.3 +TT | -85.0+TT | -84.9 +TT |  |  | -83.7 +TT | -84.7 +TT | -84.7 +TT | -84.6 +TT | -84.6 +TT | -84.6 +TT |
|  |  | 60 |  | -85.7 +TT | --85.3 +TT | --85.1 +TT |  |  | -83.9 +TT | -84.8 +TT | -84.8 +TT | -84.7 +TT | -84.7 +TT | -84.7 +TT |
| 26 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -85.6 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -85.7 +TT | -84.5 +TT |
| n28 | 18,9,10 | N/A | -89.1 | -88.7 | -88.3 | -88.0 |  |  |  |  |  |  |  |  |
| 71 | n211 | 15 | -93.4 +TT | -93.8 +TT | -92.3 +TT | -91.2 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.1 +TT | -92.4 +TT | -91.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.5 +TT | -92.7 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |
| 71 | n212 | 15 | -96.3 +TT | -93.8 +TT | -92.3 +TT | -91.2 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.1 +TT | -92.4 +TT | -91.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.5 +TT | -92.7 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |
| n71 | 211 | N/A | -92.7 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 212 | N/A | -95.6 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 7 | N/A | -82.7 | -82.6 | -82.4 | -82.3 |  |  |  |  |  |  |  |  |
| 66 | n772, 13 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 66 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2+TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 | N/A | -70.7 | -70.6 | -70.8 | -70.8 |  |  |  |  |  |  |  |  |
| n66 | 48,3 | N/A | -96.1 | -93.6 | -92.3 | -91.6 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 15: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 16: Void NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n2 | 71 | N/A | -69.7 | -69.9 | -70.1 | -71.2 |  |  |  |  |  |  |  |
| n41 | 264 | N/A | -72.5 | -69.5 | -69.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | N/A | -91.2 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.910 | -9210 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n77 | 13 | N/A |  | -65.3 | -65.3 |  |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 282 | N/A | -69.8 | -69.8 | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -90.6 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.310 | -92.010 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>