3GPP TSG-RAN5 Meeting #99	R5-232517

# Incheon, Korea, 22nd May – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | n/a | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NTN AMPR tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | AMPR test is missing in 38.521-5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 6.2.3 to specify test purpose and minimum conformance requirement of AMPR tests for NR NTN UE. Test point analysis and test configuration are TBD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR NTN AMPR tests are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Change 1 ***

### 6.2.3	UE additional maximum output power reduction

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support NR satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated

with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this

specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR

satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of

additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

FFS

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-2.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.7 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [8] absent is default for all NTN satellite bands.]

Table 6.2.3.3.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-3: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.2.3.1.

6.2.3.3.2	A-MPR for NS_24

Table 6.2.3.3.2-1: A-MPR regions for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rbend*12*SCS MHz | LCRB*12*SCS MHz | Rbend*12*SCS MHz |  | LCRB*12*SCS MHz | Rbend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5MHz | Fc=1992.5 |  | >3.24 |  |  |  |  |  |  |
| 5MHz | Fc=1997.5 |  | >3.24 |  |  |  |  |  |  |
| 5MHz | Fc=2002.5 |  | >2.16 | >3.78 |  | ≤1.98 | ≤3.6 | ≤1.98 |  |
| 10MHz | Fc=1985 | >5.4 |  |  |  |  |  |  |  |
| 10MHz | Fc=1995 |  | >4.5 | >7.56 |  | ≤4.32 | ≤7.38 | ≤4.32 |  |
| 10MHz | Fc=2000 | >6.84 |  | <2.88 |  |  | ≥3.06 ≤6.66 | >1.44 |  |
| 15MHz | Fc=1987.5 |  | >7.02 | >11.52 |  | ≤6.84 | ≤11.34 | ≤6.84 |  |
| 15MHz | Fc=1997.5 | >9.36 |  | <3.6 |  |  | ≥3.78 ≤9.18 | >1.44 |  |
| 20MHz | Fc=1990 | >13.5 |  | <4.5 |  |  | ≥4.68 ≤13.32 | >2.16 |  |
| 20MHz | Fc=1995 | >12.6 |  | <5.4 |  |  | ≥5.58 ≤12.42 | >1.44 |  |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.3.2-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.2-2: A-MPR for NS_24

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
|  | 256 QAM | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.5 |  |

*** End of Change 1 ***