3GPP TSG-RAN5 Meeting #99	R5-233222
Athens, Greece, 22nd May – 26th May 2023

Agenda item:	5.4.17

Source:	Ericsson

Title:	Testability issue in FR2 Relative Power Control test case

Document for:	Endorsement

# Introduction

The FR2 Relative Power Control test case is incomplete due to pending testability analysis. RAN5 have extensively discussed the testability issues including 2 rounds of LS exchange with RAN4 [1-4] without resolving the issue.

# Discussion

The understanding of RAN5 that the test case is not meaningful was well captured in [5]. At the same meeting as [5] was endorsed RAN5 sent an LS to RAN4 suggesting a change to the core requirements that would make the test case testable [3]. RAN4 decided not to update the requirements but made the following suggestion to RAN5 in [4]:

“A test method that only verifies the requirements as they stand today could be employed for UEs: If the start and target power lever are not within the same power range, apply the worse tolerance in the test verdict.”

It is our opinion that the suggestion to apply the worse tolerance of the two power regions will not help the testability since the essential part of the test case is to verify the 1 dB TPC step requirement which does not apply in the lower power region.This means that the observation 3 in [5] still apply and it is not possible to proceed with the testing.

“Observation 3: In order to enable testing of relative power tolerance, the minimum conformance requirement needs to be updated by RAN4”

Proposal 1: The suggestion by RAN4 in their latest LS does not help testability and does not make the test case more meaningful. Therefore it is proposed that RAN5 does not recommend testing

Proposal 2: To send a final LS to RAN4 on this topic to inform that RAN5 does not recommend testing of Relative Power Tolerance

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1: The suggestion by RAN4 in their latest LS does not help testability and does not make the test case more meaningful. Therefore it is proposed that RAN5 does not recommend testing

Proposal 2: To send a final LS to RAN4 on this topic to inform that RAN5 does not recommend testing of Relative Power Tolerance

# References

- R5-214106	“LS on FR2 relative power tolerance ", 3GPP TSG-RAN5 Meeting #91-e, RAN5 (Keysight)

- R5-217785	“Reply to LS (in R5-214106) on FR2 UE relative power control tolerance requirements” , 3GPP TSG-RAN5 Meeting #93-e, RAN4 (Mediatek)

- R5-218231	“Reply to LS (in R4-2120025) on FR2 UE relative power control tolerance requirements ", 3GPP TSG-RAN5 Meeting #93-e, RAN5 (R&S)

- R5-223920	“Reply LS on FR2 UE relative power control tolerance requirements ", 3GPP TSG-RAN5 Meeting #96-e, RAN4 (Huawei)

- R5-218231	“On the testability and MU of relative power tolerance", 3GPP TSG-RAN5 Meeting #93-e, R&S