3GPP TSG-RAN WG5 Meeting #99	R5-233398

Incheon, South Korea, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0176 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new positioning test case for UE positioning assistance information procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE positioning assistance information procedures were introduced in Rel-17 positioning WI. A new test case need to be introduced in TS 37.271-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new test case 9.4.6 UE positioning assistance information procedures was added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to Release-17 NR positioning methods signal test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0161 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.4.6	UE Positioning Assistance Information

9.4.6.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { UE receives RRCReconfiguration message requesting the association information of UL SRS resources for positioning with Tx TEGs }

then { the UE sends a UEPositioningAssistanceInfo message including TxTEG information }

}

9.4.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.305 clause 7.4.1.2 and TS 38.331 clause 5.7.14. Unless otherwise stated these are Rel-17 requirements.

[TS 38.305, clause 7.4.1.2]

The UE Positioning Assistance Information procedure is used by UE to report the UE Positioning Assistance Information for UL-TDOA. The UE reports the association between UL-SRS resources for positioning and the UE Tx TEG ID.

Figure 7.4.1.2-1: RRC procedure for UE TxTEG

Precondition:	The serving gNB of a UE has received a NRPPa message from an LMF requesting the TxTEG of the UE for NR UL-TDOA positioning.

1.	The serving gNB may send a RRC Reconfiguration message to the UE, requesting the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports UE Tx TEG reporting. Based on the request from the LMF, the RRC Reconfiguration message from the serving gNB to the UE indicates the UE should provide either a single report or a periodic report of UE TxTEG association to the serving gNB.

2.	When the UE receives the request via RRC Reconfiguration message, the UE sends a UE Positioning Assistance Info message to the serving gNB to report the UE TxTEG information, including all the changes of the UE TxTEG during the report period if the UE is required to report UE Tx TEG periodically. The UE will report all the UE TxTEG at the time when the RRC Reconfiguration message is received if the UE is only required to report the one-shot UE TxTEG information.

[TS 38.331, clause 5.7.14]

Figure 5.7.14.1-1: UE Positioning Assistance Information procedure

The UE Positioning Assistance Information procedure is used by UE to report the UE Positioning Assistance Information. The UE reports the association between UL-SRS resources for positioning and the UE Tx TEG ID as defined in TS 38.305 [73].

[TS 38.331, clause 5.7.14.2]

A UE capable of providing the association between UL SRS Resource for positioning and UE Tx TEG ID in RRC_CONNECTED may initiate the procedure upon being configured to provide this association information.

Upon initiation of the procedure, the UE shall:

1>	initiate transmission of the UEPositioningAssistanceInfo message in accordance with 5.7.14.3 to provide the association.

[TS 38.331, clause 5.7.14.3]

The UE shall set the contents of the UEPositioningAssistanceInfo message as follows:

1>	if ue-TxTEG-RequestUL-TDOA-Config in RRCReconfiguration message is configured with periodicReporting:

2>	for all the association changes store ue-TxTEG-Association corresponding to each ue-TxTEG-ID with nr-TimeStamp;

2>	include the results in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message on expiry of each configured period;

2>	optionally include one ue-TxTEG-TimingErrorMarginValue for each UEPositioningAssistanceInfo message;

1>	else if ue-TxTEG-RequestUL-TDOA-Config in RRCReconfiguration message is configured with oneShot:

2>	identify the ue-TxTEG-Association corresponding to each ue-TxTEG-ID with nr-TimeStamp;

2>	include the results in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message only one time;

2>	optionally include one ue-TxTEG-TimingErrorMarginValue for each UEPositioningAssistanceInfo message.

The UE shall submit the UEPositioningAssistanceInfo message to lower layers for transmission.

9.4.6.3	Test description

9.4.6.3.1	Pre-test conditions

System Simulator:

-	Sub-test 29: NR Cell 1

UE:

-

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

Related PICS/PIXIT Statements:

-

9.4.6.3.2	Test procedure sequence

Table 9.4.6.3.2-1: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| B | NG-RAN NR |  |

Table 9.4.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a LPP message of type Request Capabilities. | <-- | DLInformationTransfer (LPP REQUEST CAPABILITIES) | - | - |
| 2 | The UE sends a LPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (LPP PROVIDE CAPABILITIES) | - | - |
| 3 | IF the UE LPP message at step 2 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |
| 4 | The SS sends an RRCReconfiguration message requesting the association information of UL SRS resources for positioning with Tx TEGs | <-- | RRCReconfiguration |  |  |
| 5 | Check: Does the UE send a UEPositioningAssistanceInfo message including TxTEG information? | --> | UEPositioningAssistanceInfo | 1 | P |

9.4.6.3.3	Specific message contents

Table 9.4.6.3.3-1: DLInformationTransfer (step 1 and step 3, Table 9.4.6.3.2-2)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.4.6.3.3-2 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.6.3.3-2: DL NAS TRANSPORT (DLInformationTransfer, Table 9.4.6.3.3-1)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1: Set according to Table 8.4-2 | LPP Request Capabilities. |  |
|  |  | Step 3 Set according to Table 9.4.6.3.3-3 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.4.6.3.3-3: LPP Acknowledgement (DL NAS TRANSPORT, Table 9.4.6.3.3-2)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID |  | Not present |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |  |
| ackRequested |  | FALSE |  |  |
| ackIndicator |  | (0..255) | Contains the same value of the sequenceNumber field in step 1, Table 9.4.6.3.2-2 |  |
| } |  |  |  |  |
| lpp-MessageBody |  | Not present. |  |  |
| } |  |  |  |  |

Table 9.4.6.3.3-4: RRCReconfiguration (Step 4, Table 9.4.6.3.2-2)

| Derivation Path: TS 38.508-1 [30], table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-TxTEG-RequestUL-TDOA-Config-r17 CHOICE SEQUENCE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| UE-TxTEG-RequestUL-TDOA-Config-r17 CHOICE { |  |  |  |  |
| oneShot-r17 |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.6.3.3-5: ULInformationTransfer (step 2, Table 9.4.6.3.2-2)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.4.6.3.3-6 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.6.3.3-6: UL NAS TRANSPORT (ULInformationTransfer, Table 9.4.6.3.3-5)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 2: Set according to Table 9.4.6.3.3-7 | LPP Provide Capabilities |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 9.4.6.3.3-2) |  |

Table 9.4.6.3.3-7: LPP Provide Capabilities (UL NAS TRANSPORT, Table 9.4.6.3.3-6)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in the LPP Request Capabilities message in step 1, Table 9.4.6.3.2-2 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | Present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| nr-UL-ProvideCapabilities-r16 SEQUENCE { |  | Dependent on UE capabilities |  |  |
| nr-UL-SRS-Capability-r16 SEQUENCE { |  |  |  |  |
| srs-CapabilityBandList-r16 |  | Dependent on UE capabilities |  |  |
| srs-PosResourceConfigCA-BandList-r16 |  | Dependent on UE capabilities |  |  |
| maxNumberSRS-PosPathLossEstimateAllServingCells-r16 |  | Dependent on UE capabilities d |  |  |
| maxNumberSRS-PosSpatialRelationsAllServingCells-r16 |  | Dependent on UE capabilities |  |  |
| } |  |  |  |  |
| nr-UE-TEG-Capability-r17 SEQUENCE { |  |  |  |  |
| nr-UE-TEG-ID-CapabilityBandList-r17 SEQUENCE (SIZE (1..nrMaxBands-r16)) OF NR-UE-TEG-ID-CapabilityPerBand-r17 { |  | n entries | The size of NR-UE-TEG-ID-CapabilityPerBand-r17 is depending on UE capabilities. |  |
| NR-UE-TEG-ID-CapabilityPerBand-r17[n] SEQUENCE { |  | entry n |  |  |
| freqBandIndicatorNR-r17 |  | Present. Any value acceptable. |  |  |
| nr-UE-RxTEG-ID-MaxSupport-r17 |  | Not present. |  |  |
| nr-UE-TxTEG-ID-MaxSupport-r17 |  | Present. The specific value depending on UE capabilities. |  |  |
| nr-UE-RxTxTEG-ID-MaxSupport-r17 |  | Not present. |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 |  | Not present. |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 |  | Not present. |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.6.3.3-8: UEPositioningAssistanceInfo (step 5, Table 9.4.6.3.2-2)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEPositioningAssistanceInfo-r17 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| uePositioningAssistanceInfo-r17 SEQUENCE { |  |  |  |  |
| ue-TxTEG-AssociationList-r17 SEQUENCE (SIZE (1..maxNrOfTxTEGReport-r17)) OF UE-TxTEG-Association-r17 { |  | n entries | The size of UE-TxTEG-Association-r17 is depending on UE capabilities. |  |
| UE-TxTEG-Association-r17[n] SEQUENCE { |  |  |  |  |
| ue-TxTEG-ID-r17 |  | Present. Depending on UE capabilities. |  |  |
| nr-TimeStamp-r17 SEQUENCE { |  |  |  |  |
| nr-SFN-r17 |  | (0..1023) |  |  |
| nr-Slot-r17 |  | Present. Any value acceptable |  |  |
| } |  |  |  |  |
| associatedSRS-PosResourceIdList-r17 |  | Present. Any value acceptable |  |  |
| servCellId-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension { } |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-TxTEG-TimingErrorMarginValue-r17 |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |