3GPP TSG-RAN5 Meeting #99	R5-233469

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3624 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new NR Multi-SIM test case 8.1.5.10.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add new NR Multi-SIM test case 8.1.5.10.2 for UE Assistance Information |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 8.1.5.10.2 for NR Multi-SIM devices is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ..38.523-2. CR .0338.. |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The MUSIM test function for TS 38.509 was defined in the R5-233380. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-232037r3. |  |  |  |  |  |  |  |  |

8.1.5.10.2	UE Assistance Information / MUSIM

8.1.5.10.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE configured to provide MUSIM assistance information for leaving RRC_CONNECTED and timer T346g is not running }

then { The UE transmits a UEAssistanceInformation message }

}

(2)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE configured to provide MUSIM assistance information for leaving RRC_CONNECTED and timer T346g is running }

then { the UE Dose NOT transmit a UEAssistanceInformation message }

}

(3)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE configured to provide MUSIM assistance information without leaving RRC_CONNECTED and the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList }

then { The UE transmits a UEAssistanceInformation message }

}

8.1.5.10.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.7.4.1, 5.7.4.2, 5.7.4.3.

[TS 38.331, clause 5.3.5.9]

The UE shall:

…

1>	if the received otherConfig includes the musim-GapAssistanceConfig:

2>	if musim-GapAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for gap preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for gap preference and stop timer T346h, if running;

1>	if the received otherConfig includes the musim-LeaveAssistanceConfig:

2>	if musim-LeaveAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED and stop timer T346g, if running.

[TS 38.331, clause 5.7.4.1]

Figure 5.7.4.1-1: UE Assistance Information

The purpose of this procedure is for the UE to inform the network of:

…

-	its preference to transition out of RRC_CONNECTED state for MUSIM operation, or;

-	its preference on the MUSIM gaps, or;

[TS 38.331, clause 5.7.4.2]

…

A UE capable of providing MUSIM assistance information for gap preference may initiate the procedure if it was configured to do so, upon determining it needs the gaps, or upon change of the gap preference information.

A UE capable of providing MUSIM assistance information for leave indication may initiate the procedure if it was configured to do so upon determining that it needs to leave RRC_CONNECTED state.

…

1>	if configured to provide MUSIM assistance information for leaving RRC_CONNECTED:

2>	if the UE needs to leave RRC_CONNECTED state and the timer T346g is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information for leaving RRC_CONNECTED;

3>	start the timer T346g with the timer value set to the musim-LeaveWithoutResponseTimer;

1>	if configured to provide MUSIM assistance information for gap preference:

2>	if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or

2>	if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList;

3>	start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer.

NOTE 3:	The UE does not need to initiate transmission of the UAI message if the difference between the current musim-GapPreferenceList and the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList is only due to removal of an ended aperiodic gap.

[TS 38.331, clause 5.7.4.3]

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide MUSIM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for MUSIM periodic gap(s):

3>	include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured;

4>	set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with;

2>	if the UE has a preference for MUSIM aperiodic gap:

3>	include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured;

4>	include musim-GapLength in the musim-GapInfo IE and set it to the values of the length of the gap the UE prefers to be configured with;

4>	optionally include musim-Starting-SFN-AndSubframe in the musim-GapInfo IE and set it to the starting SFN/subframe of the gap the UE prefers to be configured with;

2>	if the UE has no longer preference for the periodic/aperiodic gaps:

3>	do not include musim-GapPreferenceList in the musim-Assistance IE;

2>	if UE has a preference to leave RRC_CONNECTED state:

3>	set musim-PreferredRRC-State to the preferred RRC state.

8.1.5.10.2.3	Test description

8.1.5.10.2.3.1	Pre-test conditions

System Simulator:

-   NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequency.

-  NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell''.

-  NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001,MNC=02) is set to '' Serving cell''.

UE:

- 	The UE is a MUSIM UE;

-    The UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.9.36.2.2-1 of TS 38.508-1 [4].

8.1.5.10.2.3.2	Test procedure sequence

Table 8.1.5.10.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are to be observed on NR Cell 1 unless explicitly stated otherwise |  |  |  |  |
| 1-8 | The UE performs the generic procedure by executing steps 1 to 8 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 | - | - | - | - |
| 9 | The SS transmits an RRCReconfiguration message to configure musim-LeaveAssistanceConfig-r17 with musim-LeaveWithoutResponseTimer-r17 IE | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
|  | EXCEPTION: Steps 11a1 and 11a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR is TBD) ELSE Step 11b1 is executed(Note 1). |  |  |  |  |
| 11a1 | The SS transmits an SET MUSIM UAI REQUEST message indicating musim-PreferredRRC-State-r17 = ‘idle | <-- | SET MUSIM UAI REQUEST | - | - |
| 11a2 | The UE transmits an SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 11b1 | Make the UE send UE Assistance information |  |  |  |  |
| 12 | Check: Does the UE transmits an UEAssistanceInformation message to indicate IE musim-PreferredRRC-State-r17 = ‘idle’ within musim-LeaveAssistanceConfig-r17 IE? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 13 | SS starts timer T346g = 100ms. | - | - | - | - |
|  | EXCEPTION: Steps 14a1 and 14a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR is TBD) ELSE Step 14b1 is executed(Note 1). |  |  |  |  |
| 14a1 | The SS transmits an SET MUSIM UAI REQUEST message indicating musim-PreferredRRC-State-r17 = ‘inactive’ | <-- | SET MUSIM UAI REQUEST | - | - |
| 14a2 | The UE transmits an SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 14b1 | Make the UE send UE Assistance information |  |  |  |  |
| 15 | Check: Does the UE transmit an UEAssistanceInformation message to indicate musim-PreferredRRC-State-r17 = ‘inactive’ within musim-LeaveAssistanceConfig-r17 IE? | --> | NR RRC: UEAssistanceInformation | 2 | F |
| 16 | SS waits for timer T346g expiry. | - | - | - | - |
| 17-24 | The UE performs the generic procedure by executing steps 1 to 8 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 | - | - | - | - |
| 25 | The SS transmits an RRCReconfiguration message to configure musim-GapProhibitTimer-r17 with musim-GapAssistanceConfig-r17 IE. | <-- | NR RRC: RRCReconfiguration | - | - |
| 26 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
|  | EXCEPTION: Steps 27a1 and 27a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR is TBD) ELSE Step 27b1 is executed(Note 1). |  |  |  |  |
| 27a1 | The SS transmits an SET MUSIM UAI REQUEST message indicating musim-GapLength-r17 = ‘ms6’. | <-- | SET MUSIM UAI REQUEST | - | - |
| 27a2 | The UE transmits an SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 27b1 | Make the UE send UE Assistance information |  |  |  |  |
| 28 | The UE transmit an UEAssistanceInformation message to indicate musim-GapLength-r17 = ‘ms6’ within MUSIM-GapPreferenceList-r17. | --> | NR RRC: UEAssistanceInformation | - | - |
|  | EXCEPTION: Steps 29a1 and 29a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR is TBD) ELSE Step 29b1 is executed(Note 1). |  |  |  |  |
| 29a1 | The SS transmits an SET MUSIM UAI REQUEST message indicating musim-GapLength-r17 = ‘ms10’ after 1s. | <-- | SET MUSIM UAI REQUEST | - | - |
| 29a2 | The UE transmits an SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 29b1 | Make the UE send UE Assistance information |  |  |  |  |
| 30 | Check: Does the UE transmit an UEAssistanceInformation message to indicate musim-GapLength-r17 = ‘ms10’ within MUSIM-GapPreferenceList-r17 IE? | --> | NR RRC: UEAssistanceInformation | 3 | P |
| Note1: The test function is optional, it can be replaced by alternative solutions (For Ex- AT command or MMI) due to UE implementation specific behavior. |  |  |  |  |  |

8.1.5.10.2.3.3	Specific message contents

Table 8.1.5.10.2.3.3-1: RRCReconfiguration (step 9, Table 8.1.5.10.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| musim-LeaveAssistanceConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-LeaveWithoutResponseTimer-r17 | ms40 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.2.3.3-2: RRCReconfiguration (step 25, Table 8.1.5.10.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| musim-GapAssistanceConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-GapProhibitTimer-r17 | s1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.2.3.3-3: UEAssistanceInformation (Step 12, Step 28, Step 30, Table 8.1.5.10.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| musim-Assistance-r17 SEQUENCE { |  |  |  |  |
| musim-PreferredRRC-State-r17 |  | idle |  | Step 12 |
| musim-GapPreferenceList-r17 SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17{ |  |  |  |  |
| MUSIM-GapInfo-r17 [1] SEQUENCE { |  |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 |  | Not Present |  |  |
| musim-GapLength-r17 |  | ms6 |  | Step 28 |
|  |  | ms10 |  | Step 30 |
| musim-GapRepetitionAndOffset-r17 CHOICE { |  |  |  |  |
| ms80-r17 |  | 0 |  | Step 28Step 30 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |