3GPP TSG-RAN5 Meeting #99 	R5-233676

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2809 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to frequency range for ssb-PositionsInBurst and SSB-ToMeasure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Frequency ranges in Table 4.6.3-168, Table 4.6.3-187, Table 7.3.1-4, Table 7.3.1-31 do not align with that for case C of clause 4.1 in TS 38.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Frequency ranges were corrected from 2.3 GHz (or 2.4 GHz) to 1.88 GHz in the following tables. - Table 4.6.3-168 - Table 4.6.3-187 - Table 7.3.1-4 - Table 7.3.1-31 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation of frequency range for case C is inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

4.6.3	Radio resource control information elements

<<Unchanged tables skipped>>

ServingCellConfigCommon

Table 4.6.3-168: ServingCellConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| downlinkConfigCommon | DownlinkConfigCommon |  |  |
|  | DownlinkConfigCommon with condition SCell_add |  | SCell_add |
| uplinkConfigCommon | UplinkConfigCommon |  |  |
|  | Not present |  | No_UL |
| supplementaryUplinkConfig | Not present |  |  |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 0100 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHZ) |
|  | 1000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHz) |
| mediumBitmap | 01000000 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
|  | 10000000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 0100000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#1 |
|  | 1000000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#0 |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| dmrs-TypeA-Position | pos2 |  |  |
| lte-CRS-ToMatchAround | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| ssbSubcarrierSpacing | SubcarrierSpacing | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. Value SS block SCS. |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| channelAccessMode-r16 | Not present |  |  |
| channelAccessMode-r16 CHOICE { |  |  | SharedSpectrum |
| dynamic | NULL |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | Not present |  |  |
| ssb-PositionQCL-r16 | Not present |  |  |
|  | SSB-PositionQCL-Relation-r16 |  | SharedSpectrum |
| highSpeedConfig-r16 | Not present |  |  |
|  | HighSpeedConfig-r16 |  | R16 HST OR R17 HST FR1 |
| highSpeedConfig-v1700 | Not present |  |  |
|  | HighSpeedConfig-v1700 |  | R17 HST FR1 |
| channelAccessMode2-r17 | Not present |  |  |
| discoveryBurstWindowLength-r17 | Not present |  |  |
| ssb-PositionQCL-r17 | Not present |  |  |
| highSpeedConfigFR2-r17 | Not present |  |  |
| uplinkConfigCommon-v1700 | Not present |  |  |
| ntn-Config-r17 | Not present |  |  |
| featurePriorities-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=1.88GHz | Frequency range <= 1.88GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| No_UL | No uplink CA |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |
| SharedSpectrum | Operation with shared spectrum channel access in FR1 |
| R16 HST | For R16 HST test |
| R17 HST FR1 | For R17 HST FR1 test |

<<Unchanged tables skipped>>

–	SSB-ToMeasure

Table 4.6.3-187: SSB-ToMeasure

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-ToMeasure ::= CHOICE { |  |  |  |
| shortBitmap | 1100 |  | FR1 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHZ) |
| mediumBitmap | 11000000 |  | FR1 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 11000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  | FR2 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=1.88GHz | Frequency range <= 1.88GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |

<<Unchanged sections skipped>>

7.3.1	Radio resource control information elements for RRM

<<Unchanged tables skipped>>

–	ServingCellConfigCommon

Table 7.3.1-4: ServingCellConfigCommon

| Derivation Path: Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 1100 | 2 SS Blocks in low FR1 frequencies | 2SSB AND ((SCS15 AND 1.88GHz<FREQ<=3GHz) OR FREQ<=1.88GHZ) |
| mediumBitmap | 11000000 | 2 SS Blocks in high FR1 frequencies | 2SSB AND ((SCS30 AND 1.88GHz<FREQ<=3GHz) OR FREQ>3GHZ) |
| longBitmap | 1100000000000000000000000000000000000000000000000000000000000000 | 2 SS Blocks in FR2 | 2SSB AND FR2 |
| } |  |  |  |
| ssbSubcarrierSpacing | kHz15 |  |  |
|  | kHz30 |  | SCS30 |
|  | kHz120 |  | FR2 |
| } | kHz240 |  | FR2 AND SCS240 |

| Condition | Explanation |
| --- | --- |
| FREQ<=1.88GHz | Frequency range <= 1.88GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| 2SSB | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 contain 2 SSBs within a burst |

<<Unchanged tables skipped>>

–	SSB-ToMeasure

Table 7.3.1-31: SSB-ToMeasure

| Derivation Path: Table 4.6.3-187 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-ToMeasure ::= CHOICE { |  |  |  |
| shortBitmap | 1000 |  | ((1.88GHz<FREQ<=3GHz AND SCS15) OR FREQ<=1.88GHZ) AND NOT SECOND_SSB |
| mediumBitmap | 10000000 |  | (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) AND NOT SECOND_SSB |
| longBitmap | 10000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  | FR2 AND NOT SECOND_SSB |
|  | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  | SSB.7 FR2 OR SSB.8 FR2 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=1.88GHz | Frequency range <= 1.88GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| SECOND_SSB | SSB configuration used in test case contain two SSBs in a burst |

<<End of change>>