3GPP TSG-RAN5 Meeting #99	R5-232599r1

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0308 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to MCData PIDF for functional alias |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There was no functional alias IEs in the MCData PIDF definitions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added functional alias IEs in the MCData PIDF definitions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCData functional alias testing may not work properly and may produce false results |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: corrected a typo in Table 5.5.3.5.1-3 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

#### 5.5.3.5	PIDF

##### 5.5.3.5.1	PIDF from the UE

##### -	MCPTT

Table 5.5.3.5.1-1: PIDF for MCPTT from the UE

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  | MCPTT extension | TS 24.379 [9] clause 9.3.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCPTT_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | not present |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCPTT extension | TS 24.379 [9] Table 9A.3.1.2-1 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | not present |  |  |  |
| expires attribute | not present |  |  |  |
| contact | not present |  |  |  |
| note | not present |  |  |  |
| timestamp | not present |  |  |  |
| note | not present |  |  |  |
| p-id | any allowed value if present |  | TS 24.379 [9] clause 9.3.1 | AFFILIATION |
| p-id-fa | Any allowed value | a globally unique value set to an identifier of a SIP PUBLISH request | TS 24.379 [9] clause 9A.2.1.2 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_STATUS_CHANGE | PIDF sent by the UE in request for functional alias status change |
| For further conditions see table 5.5.1-1 |  |

##### -	MCVideo

Table 5.5.3.5.1-2: PIDF for MCVideo from the UE

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  |  | TS 24.281 [86] clause 8.3.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCVideo_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | not present |  |  |  |
| expires | not present |  |  |  |
| p-id | any allowed value if present |  |  | AFFILIATION |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

##### -	MCData

Table 5.5.3.5.1-3: PIDF for MCData from the UE

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCData_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  |  | TS 24.282 [87] clause 8.4.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCDATA_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | not present |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCData extension | TS 24.282 [87] Table 22.3.1.2-1 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCData_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | not present |  |  |  |
| expires attribute | not present |  |  |  |
| p-id | any allowed value or same value as sent in SIP PUBLISH | set to an identifier of a SIP PUBLISH request |  | AFFILIATION |
| p-id-fa | Any allowed value | a globally unique value set to an identifier of a SIP PUBLISH request | TS 24.282 [87] clause 22.2.1.2 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

##### 5.5.3.5.2	PIDF from the SS

##### -	MCPTT

Table 5.5.3.5.2-1: PIDF for MCPTT from the SS

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  | MCPTT extension | TS 24.379 [9] clause 9.3.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCPTT_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | "affiliating" |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCPTT extension | TS 24.379 [9] Table 9A.3.1.2-1 | FUNCTIONAL_ALIAS_ACTIVATED |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | "activated" |  |  |  |
| expires attribute | not present |  |  |  |
| contact | not present |  |  |  |
| note | not present |  |  |  |
| timestamp | not present |  |  |  |
| note | not present |  |  |  |
| p-id | not present |  |  | AFFILIATION |
| p-id-fa | same value as received in the SIP PUBLISH message |  | TS 24.379 [9] clause 9A.2.2.2.5 | NOTIFY_FOR_PUBLISH |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_ACTIVATED | PIDF sent by the SS in notification for functional alias getting activated |
| NOTIFY_FOR_PUBLISH | PIDF sent by the SS in notification associated with a previous SIP PUBLISH message sent by the UE |
| For further conditions see table 5.5.1-1 |  |

##### -	MCVideo

Table 5.5.3.5.2-2: PIDF for MCVideo from the SS

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  |  | TS 24.281 [86] clause 8.3.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCVideo_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | “affiliating” |  |  |  |
| expires | not present |  |  |  |
| p-id | not present |  |  | AFFILIATION |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

##### -	MCData

Table 5.5.3.5.2-3: PIDF for MCData from the SS

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCDATA_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  |  | TS 24.282 [87] clause 8.4.1 | AFFILIATION |
| group | px_MCDATA_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | “affiliating” |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCData extension | TS 24.282 [87] Table 22.3.1.2-1 | FUNCTIONAL_ALIAS_ACTIVATED |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCData_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | "activated" |  |  |  |
| expires attribute | not present |  |  |  |
| p-id | not present |  |  | AFFILIATION |
| p-id-fa | same value as received in the SIP PUBLISH message |  | TS 24.282 [87] clause 22.2.2.2.5 | NOTIFY_FOR_PUBLISH |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_ACTIVATED | PIDF sent by the SS in notification for functional alias getting activated |
| NOTIFY_FOR_PUBLISH | PIDF sent by the SS in notification associated with a previous SIP PUBLISH message sent by the UE |
| For further conditions see table 5.5.1-1 |  |

<END OF CHANGES>