3GPP TSG-RAN5 Meeting #99 	R5-232997r1

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2280 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of BW condition to 6.5D.2.3 A-SEM for UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some channel bandwidths are missing in the test requirements of Additional SEM for UL MIMO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 6.5D.2.3.5-3 (NS_04): - BW 30 MHz and BW 70 MHz were added to conditions. Table 6.5D.2.3.5-4 (NS_03 and NS_03U) - BW 35 MHz and BW 45 MHz were added to conditions. - Table style was corrected. - Referred table for TT was updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some channel bandwidths will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5D.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: In Table 6.5D.2.3.5-4, reffered table for TT was updated. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.5D.2.3	Additional spectrum emission mask for UL MIMO

Editor’s note:

-Test coverage for the NS_XXs other than NS_03, NS_03U, NS_04 and NS_35 is FFS

- Supporting of ULFPTx is only completed for NS_04 in A-MPR

6.5D.2.3.1	Test purpose

To verify that the sum of power of any UE emission from both transmit antennas shall not exceed specified lever for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5D.2.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3UE release 15 and forward that supports 2-layer codebook based UL MIMO.

6.5D.2.3.3	Minimum conformance requirements

The sum of power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3-1 for NS_35, Table 6.5.2.3.3.2-1 for NS_04, and Table 6.5.2.3.3.3 for NS_03 and NS_03U for the specified channel bandwidth from both transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5D.2 and 6.5.2.3

6.5D.2.3.4	Test description

6.5D.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 6.5D.2.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.2.3.4.1-1 to 6.5D.2.3.4.1-3: Void

Table 6.5D.2.3.4.1-2: Test Configuration Table for NS_04 for band n41

Same as defined in table 6.2D.3.4.1-1

Table 6.5D.2.3.4.1-3: Test Configuration table for NS_03 and NS_03U

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_03 and NS_03U |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation |  |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A for A-MPR test cases | CP- OFDM | QPSK |  | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | QPSK |  | Outer_Full |
| 4 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | 16 QAM |  | Outer_Full |
| 7 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |
| 10 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Tables 6.5D.2.3.4.1-1 as appropriate for NS_35.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.3.4.3.

6.5D.2.3.4.2	Test procedure

Same test procedure as defined in clause 6.5D.2.2.4.2 with the following exceptions:

-	Instead of Table 6.2D.2.5-1, test requirements in clause 6.2D.3.5 are applied in step 3;

-	Instead of Table 6.5D.2.2.5-1, test requirements in clause 6.5D.2.2.5 are applied in step 4;

-	Instead of Table 6.5D.2.2.4.1-2 and Table 6.5D.2.2.4.1-3, test configurations for ULFPTx in clause 6.2D.3 are applied in step 5;

6.5D.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and exceptions listed in clause 6.2D.3.4.3.

6.5D.2.3.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth, derived in step 3 and step 5 shall fulfill the requirements as specified in clause 6.2D.3.5 for NS_35, NS_04, NS_03 and NS_03U as appropriate, and the sum of power of any UE emission measured from both antennas in step 4 and step 5 shall fulfill requirements in current subclause.

Table 6.5D.2.3.5-1: Additional requirements for “NS_35”

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| 0-0.1 | -15 + TT | -18 + TT | -20 + TT | -21 + TT | 30 kHz |
| 0.1-6 | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 100 kHz |
| 6-10 | -251 + TT | -13 + TT | -13 + TT | -13 + TT | 100 kHz |
| 10-15 |  | -251 + TT | -13 + TT | -13 + TT | 100 kHz |
| 15-20 |  |  | -251 + TT | -13 + TT | 100 kHz |
| 20-25 |  |  |  | -25 + TT | 1 MHz |
| NOTE 1: The measurement bandwidth shall be 1 MHz; NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.3.5-2 |  |  |  |  |  |

Table 6.5D.2.3.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

Table 6.5D.2.3.5-3: Additional requirements for “NS_04”

|  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Measurement bandwidth |
| ± 0 - 1 | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  | -10 + TT |  |  |  |  |  | 1 MHz |
| ± 1 - 5 | -10 + TT |  |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: X is defined in Table 6.5D.2.3.5-3a for CP-OFDM NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.3.5-2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5D.2.3.5-3a: n41 maximum transmission bandwidths (MHz) for CP-OFDM

| SCS (kHz) | Channel bandwidths (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 |
| 15 | 9.36 | 14.22 | 19.08 | 38.88 | 48.6 | N/A | N/A | N/A | N/A |
| 30 | 8.64 | 13.68 | 18.36 | 38.16 | 47.88 | 58.32 | 78.12 | 88.02 | 98.28 |
| 60 | 7.92 | 12.96 | 17.28 | 36.72 | 46.8 | 56.88 | 77.04 | 87.12 | 97.20 |

Table 6.5D.2.3.5-4: Additional requirements for “NS_03 and NS_03U”

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 |  |
| 0-1 | -13 + TT | -13 + TT | 1 % of channel BW |
| 1-6 | -13 + TT | -13 + TT | 1 MHz |
| 6-10 | -25 + TT |  |  |
| 1-BWChannel |  |  |  |
| BWChannel-(BWChannel+5) |  | -25 + TT |  |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.3.5-2. |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<<End of change>>