3GPP TSG-RAN5 Meeting #101	R5-237463r2

# Chicago , United States, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0567 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 11.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | "eCall.invalidMSD" uses in step 15 is not specified in any core specification (3GPP and non-3GPP) and unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note in Table 11.6.3.3-3 to state the intention of value “eCall.invalidMSD”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear in which core specification “eCall.invalidMSD” is specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Add a note in Table 11.6.3.3-3 to state the intention of value “eCall.invalidMSD”. 2nd revision: - Update the coversheet and the NOTE according to TF160 comments. |  |  |  |  |  |  |  |  |

<Start of Changes>

11.6	eCall over IMS / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for unsupported MSD / UE indicates unsuccessful in SIP INFO / 5GS

11.6.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is being made to initiate an automatic eCall over IMS }

then { UE performs IMS emergency registration and sends correctly composed INVITE with MSD }

}

(2)

with { UE having sent an  INVITE }

ensure that {

when { UE receives 200 OK containing acknowledgement of reception of MSD }

then { UE sends ACK }

}

(3)

with { automatic eCall over IMS being established }

ensure that {

when { UE receives SIP INFO requesting MSD update with unsupported datatype }

then { UE sends 200 OK acknowledging SIP INFO }

}

(4)

with { UE having sent 200 OK for SIP INFO }

ensure that {

when { UE is unable to provide the MSD information }

then { UE sends a correctly composed SIP INFO with “success” attribute set to false in ack element }

}

11.6.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-16 requirements.

[TS 24.229 clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

2)	if the UE is not able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

-	a "ref" attribute set to the Content-ID of the "application/EmergencyCallData.Control+xml" MIME body part in the INFO request received by the UE; and

-	an "actionResult" child element containing:

A)	an "action" attribute set to "send-data";

B)	a "success" attribute set to "false"; and

C)	a "reason" attribute set to an appropriate value as defined in RFC 8147 [244]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body.

NOTE:	Further content for the INFO request is as defined in RFC 8147 [244].

11.6.3	Test description

11.6.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application with eCall subscription on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.6.3.2	Test procedure sequence

Table 11.6.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start an automatic eCall | - | - | - | - |
| 2-9 | Steps 1-8 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 10-11 below the events specified in Table 11.6.3.2-2 take place | - | - | - | - |
| 10-11 | Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 12 and 13, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens | --> | INVITE | 1 | P |
| 13 | Step 2 of Annex A.23 happens | <-- | 200 OK | - | - |
| 14 | Step 3 of Annex A.23 happens | --> | ACK | 2 | P |
| 15 | Step 4 of Annex A.23 happens | <-- | SIP INFO | - | - |
| 16 | Step 5 of Annex A.23 happens Check: Does the UE send 200 OK acknowledging reception of SIP INFO? | --> | 200 OK | 3 | P |
| 17 | Step 6 of Annex A.23 happens Check: Does UE send a correctly composed SIP INFO with ack element containing “success” attribute set to “false”? | --> | SIP INFO | 4 | P |
| 18 | Step 7 of Annex A.23 happens | <-- | 200 OK | - | - |
| 19 | Step 1 of Annex A.8 happens | <-- | BYE | - | - |
| 20 | Step 2 of Annex A.8 happens | --> | 200 OK | - | - |

Table 11.6.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 takes place. | - | - | - | - |
| 2 | Step 1 of Annex A.3 happens Check: Does UE send a correctly composed initial REGISTER request for IMS emergency registration? | --> | REGISTER | - | - |
| 3 | Step 2 of Annex A.3 happens | <-- | 401 Unauthorized | - | - |
| 4 | Step 3 of Annex A.3 happens Check: Does UE send another REGISTER with AKAv1-MD5 credentials? | --> | REGISTER | - | - |
| 5 | Step 4 of Annex A.3 happens | <-- | 200 OK | - | - |

11.6.3.3	Specific message contents

Table 11.6.3.3-1: INVITE (step 12, table 11.6.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.6.3.3-2: 200 OK for INVITE (step 13, table 11.6.3.2-1)

Derivation path: Step 2 in Annex A.23 with Conditions A12 and A13

Table 11.6.3.3-3: SIP INFO (step 15, table 11.6.3.2-1)

| Derivation path: Step 4 in Annex A.23 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |  |
| Message-body |  | --boundaryXXX Content-Type: application/EmergencyCallData.Control+xml Content-ID: <test-info@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <request action="send-data" datatype="eCall.invalidMSD"/>(NOTE 1) </EmergencyCallData.Control> --boundaryXXX |  |  |  |
| NOTE 1: "eCall.invalidMSD" is not specified in any core specification (3GPP and non-3GPP). It has been created as an example of unsupported datatype, as a way to test the 3GPP core requirement “the UE is not able to provide an updated MSD”, under TP3. |  |  |  |  |  |

Table 11.6.3.3-4: SIP INFO (step 17, table 11.6.3.2-1)

Derivation path: Step 6 in Annex A.23 with Conditions A2 and A4

<End of Changes>