3GPP TSG-RAN5 Meeting #102	R5-240537

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3058 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Harmonization of SCell_add condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Throughout subclauses 4.6.x, one can find different spelling for the condition, whose purpose is to add an SCell: “Scell_add”, “Scell_Add”, SCell_add”, “SCell_Add”… |  |  |  |  |  |  |  |  |
| Summary of change: |  | The terminology is aligned throughout subclauses 4.6.x to use “SCell_add”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification quality will remain deprecated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.0, 4.6.1, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.6	Default NG-RAN RRC message and information elements contents

### 4.6.0	General

#### 4.6.0.1	Global conditions

Groups of RRC and 5GMM conditions including always one and only one condition set to true per table, unless explicitly stated.

Table 4.6.0.1-1: Signalling, RF/RRM/Performance

| Condition | Explanation |
| --- | --- |
| SIG | Used for signalling test cases |
| RF | Used for RF/Performance test cases |
| RRM | Used for RRM test cases |

Table 4.6.0.1-2: NR operating bands

| Condition | Explanation |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

Table 4.6.0.1-3: FDD/TDD

| Condition | Explanation |
| --- | --- |
| FDD | Frequency Division Duplex |
| TDD | Time Division Duplex |

Table 4.6.0.1-4: Subcarrier spacing

| Condition | Explanation |
| --- | --- |
| SCS15 | 15kHz |
| SCS30 | 30kHz |
| SCS60 | 60kHz |
| SCS120 | 120kHz |
| SCS240 | 240kHz |

Table 4.6.0.1-5: SST

| Condition | Explanation |
| --- | --- |
| SST_eMBB | Slice suitable for the handling of 5G enhanced Mobile Broadband. |
| SST_URLLC | Slice suitable for the handling of ultra- reliable low latency communications. |
| SST_MIoT | Slice suitable for the handling of massive IoT. |
| SST_V2X | Slice suitable for the handling of V2X services. |
| NOTE: For the conditions in this table, one or more conditions are set to true. |  |

Table 4.6.0.1-6: Channel bandwidth

| Condition | Explanation |
| --- | --- |
| BW5 | 5mhz |
| BW10 | 10mhz |
| BW15 | 15mhz |
| BW20 | 20mhz |
| BW25 | 25mhz |
| BW30 | 30mhz |
| BW40 | 40mhz |
| BW50 | 50mhz |
| BW60 | 60mhz |
| BW80 | 80mhz |
| BW100 | 100mhz |

#### 4.6.0.2	ASN.1 extension groups

The tables in the subclauses of clause 4.6 specify the default message contents of RRC messages based on the ASN.1 message definitions of TS 38.331 [6] clause 6.

The ASN.1 of TS 38.331 [6] uses extension groups (also called extension addition groups), with an extension group being defined as a set of non-critical extension fields grouped together using double brackets [[ ]] (also called version brackets).

In general, the default message contents do not need to contain all extension groups. The following rules apply for each table in the subclauses of clause 4.6:

1.	An extension group is contained in the default message contents when at least one of its fields needs to be specified a value. In this case all fields of the extension group shall be specified in the default message contents.

2.	A specific message content (e.g., TS 38.523-1 [12]) of a test case may add extension group(s) in which case all fields of the extension group(s) shall be added.

3.	When an extension group is not specified in a message content then all its fields are considered as being 'Not present' in downlink and 'Not checked' in uplink.

4.	It is up to test implementation (e.g., TS 38.523-3 [23]) whether the fields of extension groups not being specified by the test specifications are explicitly set according to rule 3 or whether they are handled implicitly (e.g., by compilers or other tools).

### 4.6.1	Contents of RRC messages

#### –	CounterCheck

Table 4.6.1-1: CounterCheck

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CounterCheck ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| counterCheck SEQUENCE { |  |  |  |  |
| drb-CountMSB-InfoList SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info |  | 1 entry |  |  |
| DRB-CountMSB-Info[1] SEQUENCE { |  |  | entry 1 |  |
| drb-Identity |  | DRB-Identity |  |  |
| countMSB-Uplink |  | 0 |  |  |
| countMSB-Downlink |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	CounterCheckResponse

Table 4.6.1-2: CounterCheckResponse

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CounterCheckResponse ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| counterCheckResponse SEQUENCE { |  |  |  |  |
| drb-CountInfoList SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo { |  | 1 entry |  |  |
| DRB-CountInfo[1] SEQUENCE { |  |  | entry 1 |  |
| drb-Identity |  | DRB-Identity |  |  |
| count-Uplink |  | Not checked |  |  |
| count-Downlink |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	DedicatedSIBRequest

Table 4.6.1-2A: DedicatedSIBRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DedicatedSIBRequest-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dedicatedSIBRequest-r16 SEQUENCE { |  |  |  |  |
| onDemandSIB-RequestList-r16 SEQUENCE { |  |  |  |  |
| requestedSIB-List-r16 |  | Not checked |  |  |
| requestedPosSIB-List-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	DLDedicatedMessageSegment

Table 4.6.1-2B: DLDedicatedMessageSegment

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |  |
| segmentNumber-r16 |  | 0 |  | firstSegment |
|  |  | 1 |  | lastSegment |
| rrc-MessageSegmentContainer-r16 |  | Set according to specific message content | OCTET STRING including segmented RRCReconfiguration or RRCResume message |  |
| rrc-MessageSegmentType-r16 |  | notLastSegment |  | firstSegment |
|  |  | lastSegment |  | lastSegment |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| firstSegment | The first segment of the RRCReconfiguration or RRCResume message |
| lastSegment | The last segment of the RRCReconfiguration or RRCResume message |

#### –	DLInformationTransfer

Table 4.6.1-3: DLInformationTransfer

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message |  | DedicatedNAS-Message |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  | TSC |
| referenceTimeInfo-r16 |  | ReferenceTimeInfo |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| TSC | For test cases requiring TSC (Time Sensitive Communication) functions enabled. |

#### –	DLInformationTransferMRDC

Table 4.6.1-3A: DLInformationTransferMRDC

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransferMRDC-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	FailureInformation

Table 4.6.1-4: FailureInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FailureInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| failureInformation SEQUENCE { |  |  |  |  |
| failureInfoRLC-Bearer SEQUENCE { |  |  |  |  |
| cellGroupId |  | Not checked |  |  |
| logicalChannelIdentity |  | Not checked |  |  |
| failureType |  | Not checked |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | DAPS_HOF |
| failureInfoDAPS-r16 SEQUENCE { |  |  |  |  |
| failureType-r16 |  | daps-failure | Indicate handover failure type is DAPS handover failure |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| DAPS_HOF | DAPS handover failure |

#### –	IABOtherInformation

Table 4.6.1-4A: IABOtherInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| IABOtherInformation-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	LocationMeasurementIndication

Table 4.6.1-5: LocationMeasurementIndication

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LocationMeasurementIndication ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| locationMeasurementIndication SEQUENCE { |  |  |  |  |
| measurementIndication CHOICE { |  |  |  |  |
| setup |  | LocationMeasurementInfo |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	LoggedMeasurementConfiguration

Table 4.6.1-5AA: LoggedMeasurementConfiguration (Thres1)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |  |  |
| plmn-Identity-r16 |  | plmn-Identity in USIM | MCC/MNC=MCC/MNC in USIM |  |  |
| traceId-r16 |  | ‘0EF’H | OCTET STRING (SIZE (3)) |  |  |
| } |  |  |  |  |  |
| traceRecordingSessionRef-r16 |  | ‘1A’H | OCTET STRING (SIZE (2)) |  |  |
| tce-Id-r16 |  | ‘5’H | OCTET STRING (SIZE (1)) |  |  |
| absoluteTimeInfo-r16 |  | Set to value corresponding to the absolute time when the message is sent | BIT STRING (SIZE (48)) |  |  |
| areaConfiguration-r16 |  | Not present |  |  |  |
| plmn-IdentityList-r16 |  | Not present |  |  |  |
| bt-NameList-r16 |  | Not present |  |  |  |
| wlan-NameList-r16 |  | Not present |  |  |  |
| sensor-NameList-r16 |  | Not present |  |  |  |
| loggingDuration-r16 |  | min120 |  |  |  |
| reportType CHOICE { |  |  |  |  |  |
| periodical SEQUENCE { |  |  |  | PERIODICAL |  |
| loggingInterval-r16 |  | ms2560 | 2.56s |  |  |
| } |  |  |  |  |  |
| eventTriggered SEQUENCE { |  |  |  | EVENT |  |
| eventType-r16 CHOICE { |  |  |  |  |  |
| outOfCoverage |  | NULL |  | OUT-OF-COVERAG |  |
| eventL1 SEQUENCE { |  |  |  | EVENTL1 |  |
| l1-Threshold CHOICE { |  |  |  |  |  |
| rsrp |  | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |  |
| } |  |  |  |  |  |
| hysteresis |  | 0 |  |  |  |
| timeToTrigger |  | ms100 | 0.1s |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| loggingInterval-r16 |  | ms2560 | 2.56s |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | LOG-MEAS-R17 |
| sigLoggedMeasType-r17 |  | Not present |  |  |
|  |  | true |  | SigLogMDT |
| earlyMeasIndication-r17 |  | Not present |  |  |
|  |  | true |  | EarlyMeas |
| areaConfiguration-v1700 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODICAL | Configuration of LoggedPeriodicalReport |
| EVENT | Configuration of LoggedEventTrigger |
| OUT-OF-COVERAGE | Configuration of outOfCoverage |
| EVENTL1 | Configuration of eventL1 |
| LOG-MEAS-R17 | Configuration of LoggedMeasurement in R17 |
| SigLogMDT | Signalling based logged measurements |
| EarlyMeas | Log measurements on early measurement |

#### –	MBSBroadcastConfiguration

Table 4.6.1-5ABA: MBSBroadcastConfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSBroadcastConfiguration-r17 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbsBroadcastConfiguration-r17 SEQUENCE { |  |  |  |
| mbs-SessionInfoList-r17 | MBS-SessionInfoList |  |  |
| mbs-NeighbourCellList-r17 | Not present |  |  |
| drx-ConfigPTM-List-r17 | Not present |  |  |
| drx-ConfigPTM-List-r17 SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17 { | 1 entry |  | DRX_MBS_Broadcast |
| DRX-ConfigPTM-r17[1] | DRX-ConfigPTM | entry 1 |  |
| } |  |  |  |
| pdsch-ConfigMTCH-r17 | Not present |  |  |
| mtch-SSB-MappingWindowList-r17 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX_MBS_Broadcast | DRX is used for MBS Broadcast test |

#### –	MBSInterestIndication

Table 4.6.1-5ABB: MBSInterestIndication

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSInterestIndication-r17 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbsInterestIndication-r17 SEQUENCE { |  |  |  |
| mbs-FreqList-r17[n] SEQUENCE (SIZE (1..maxFreqMBS-r17)) OF ARFCN-ValueNR | The number of entries is set according to specific message content. |  |  |
| mbs-Priority-r17 | Not checked |  |  |
| mbs-ServiceList-r17 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MCGFailureInformation

Table 4.6.1-5AB: MCGFailureInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MCGFailureInformation-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	MeasurementReport

Table 4.6.1-5A: MeasurementReport

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults |  | MeasResults |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	MIB

Table 4.6.1-6: MIB

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MIB ::= SEQUENCE { |  |  |  |  |
| systemFrameNumber |  | A valid value as defined in TS 38.331 [6] |  |  |
| subCarrierSpacingCommon |  | scs15or60 | For signalling test cases see clause 6.2.3, otherwise see clause 4.3.1. | SCS15or60 |
|  |  | scs30or120 | For signalling test cases see clause 6.2.3, otherwise see clause 4.3.1. | SCS30or120 |
| ssb-subcarrierOffset |  | Set to the integer value of the 4 LSB of kSSB defined for the frequency of the cell | For signalling test cases see clause 6.2.3, otherwise see clause 4.3.1. |  |
|  |  | 15 | Note 1 | NCD-SSB |
| dmrs-TypeA-Position |  | pos2 |  |  |
| pdcch-ConfigSIB1 |  | PDCCH-ConfigSIB1 |  |  |
|  |  | PDCCH-ConfigSIB1 with condition NCD-SSB |  | NCD-SSB |
| cellBarred |  | notBarred |  |  |
| intraFreqReselection |  | allowed |  |  |
| spare |  | 0 |  |  |
| } |  |  |  |  |
| Note 1: For FR1, kSSB =31(1 MSB of kSSB is 1 and 4 LSB of kSSB is 15). 1 MSB of kSSB is in the PBCH payload specified in TS 38.211 clause 7.4.3.1. For FR2, kSSB=15. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS15or60 | SCS is 15kHz or 60kHz |
| SCS30or120 | SCS is 30kHz or 120kHz. |
| NCD-SSB | MIB for NCD-SSB |

#### –	MobilityFromNRCommand

Table 4.6.1-8: MobilityFromNRCommand

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromNRCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| mobilityFromNRCommand SEQUENCE { |  |  |  |  |
| targetRAT-Type |  | eutra |  |  |
|  |  | utra-fdd-v1610 |  | HO-TO-UTRA_FDD |
| targetRAT-MessageContainer |  | OCTET STRING including the RRCConnectionReconfiguration message according TS 36.508 [2], table 4.6.1-8 with condition HO-TO-EUTRA |  |  |
|  |  | OCTET STRING including the Handover TO UTRAN message specified in TS 36.508 [2] Table 4.7B.1-1 with condition UTRA FDD PS RB |  | HO-TO-UTRA_FDD |
| nas-SecurityParamFromNR |  | The 4 LSB of the downlink NAS COUNT |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| HO-TO-UTRA_FDD | For Handover from NR to UTRA FDD |

#### –	Paging

Table 4.6.1-9: Paging

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |  |
| pagingRecordList SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord { |  | 1 entry |  |  |
| PagingRecord[1] SEQUENCE { |  |  | entry 1 |  |
| ue-Identity CHOICE { |  |  |  |  |
| ng-5G-S-TMSI |  | NG-5G-S-TMSI |  |  |
| fullI-RNTI |  | I-RNTI-Value |  | NR_RRC_RESUME |
| } |  |  |  |  |
| accessType |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | TMGI |
| pagingRecordList-v1700 |  | Not present |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { |  | 1 entry |  |  |
| TMGI-r17[1] |  | TMGI-r17 | entry 1 |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR_RRC_RESUME | To page a UE in RRC_INACTIVE state to request RRC connection resumption |
| TMGI | TMGI is used as paging identity |

#### –	RRCReestablishment

Table 4.6.1-10: RRCReestablishment

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RemoteUE |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RemoteUE-Config-r17 |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| L2RemoteUE | For L2 U2N Remote UE test cases. |

#### –	RRCReestablishmentComplete

Table 4.6.1-11: RRCReestablishmentComplete

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCReestablishmentRequest

Table 4.6.1-12: RRCReestablishmentRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |  |
| c-RNTI |  | RNTI-Value |  |  |
| physCellId |  | PhysCellId | The physical cell identity of the PCell the UE was connected to prior to the failure |  |
| shortMAC-I |  | ShortMAC-I |  |  |
| } |  |  |  |  |
| reestablishmentCause |  | Not checked |  |  |
| spare |  | Present but contents not checked |  |  |
| } |  |  |  |  |

#### –	RRCReconfiguration

Table 4.6.1-13: RRCReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | RadioBearerConfig with conditions SRB2 and DRB1 |  | NR, NR-DC |
|  |  | RadioBearerConfig with conditions SRB2 and REEST |  | REEST |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  |  | CellGroupConfig with condition NR-DC_SCG | OCTET STRING (CONTAINING CellGroupConfig) | NR-DC_SCG |
|  |  | CellGroupConfig with conditions EN-DC and PSCell_change | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_HO |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_MEAS |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_SCell_add |
|  |  | Not present |  |  |
| measConfig |  | Not present |  |  |
|  |  | MeasConfig | Measurements configuration | NR_MEAS, EN-DC_MEAS, IRAT_MEAS |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR, SCell_add, NR_MEAS, NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC, REEST, L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB1 | OCTET STRING (CONTAINING CellGroupConfig) | NR, NR-DC |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | SCell_add |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | NR_MEAS |
|  |  | CellGroupConfig with condition REEST | OCTET STRING (CONTAINING CellGroupConfig) | REEST |
|  |  | CellGroupConfig with condition RELAY | OCTET STRING (CONTAINING CellGroupConfig) | RELAY |
| fullConfig |  | Not present |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message { |  | 1 entry |  | NR |
| DedicatedNAS-Message[1] |  | DedicatedNAS-Message | entry 1 A sequence of OCTET STRING (s) containing one or more DedicatedNAS-Message(s) |  |
| } |  |  |  |  |
| masterKeyUpdate |  | Not present |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  | MasterKeyChange |
| keySetChangeIndicator |  | true |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| nas-Container |  | Not present |  |  |
|  |  | OCTET STRING including the 10 Octets value generated according to TS 24.501 [28] clause 9.11.2.9 |  | Inter_Sys_HO |
| } |  |  |  |  |
| dedicatedSIB1-Delivery |  | Not present |  |  |
| dedicatedSystemInformationDelivery |  | Not present |  |  |
| otherConfig |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC, NE-DC,L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1540 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig |  | Not present |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  | NR-DC, NE-DC |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration with condition NR-DC_SCG | OCTET STRING (CONTAINING RRCReconfiguration) | NR-DC |
| eutra-SCG |  | RRCConnectionReconfiguration according TS 36.508 [2], table 4.6.1-8 with condition NE-DC | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | NE-DC |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioBearerConfig2 |  | Not present |  | NE-DC |
|  |  | RadioBearerConfig with condition DRBn and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) | NR-DC |
| sk-Counter |  | SK-Counter |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC,L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1610 |  | Not present |  |  |
| bap-Config-r16 |  | Not present |  |  |
| iab-IP-AddressConfigurationList-r16 |  | Not present |  |  |
| conditionalReconfiguration-r16 |  | Not present |  |  |
|  |  | ConditionalReconfiguration |  | CHO, CPA, CPC |
| daps-SourceRelease-r16 |  | Not present |  |  |
|  |  | true |  | DAPS_HO_ReleaseSource |
| t316-r16 |  | Not present |  |  |
| needForGapsConfigNR-r16 |  | Not present |  |  |
| onDemandSIB-Request-r16 |  | Not present |  |  |
| dedicatedPosSysInfoDelivery-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  | SIDELINK |
| setup |  | SL-ConfigDedicatedNR-r16 |  |  |
| } |  |  |  |  |
| sl-ConfigDedicatedEUTRA-Info-r16 |  | Not present |  |  |
| smtc-r16 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1700 |  | Not present |  |  |
| otherConfig-v1700 SEQUENCE{ |  |  |  | SHRT304, SHRT310, SHRT312, SHRRLF |
| ul-GapFR2-PreferenceConfig-r17 |  | Not present |  |  |
| musim-GapAssistanceConfig-r17 |  | Not present |  |  |
| musim-LeaveAssistanceConfig-r1 |  | Not present |  |  |
| successHO-Config-r17 |  | Not present |  |  |
| successHO-Config-r17 CHOICE { |  |  |  | SHRT304, SHRT310, SHRT312, SHRRLF |
| setup SEQUENCE { |  |  |  |  |
| thresholdPercentageT304-r17 |  | Not present |  |  |
|  |  | p40 |  | SHRT304 |
| thresholdPercentageT310-r17 |  | Not present |  |  |
|  |  | p40 |  | SHRT310 |
| thresholdPercentageT312-r17 |  | Not present |  |  |
|  |  | p20 |  | SHRT312 |
| sourceDAPS-FailureReporting-r17 |  | Not present |  |  |
|  |  | true |  | SHRRLF |
| } |  |  |  |  |
| } |  |  |  |  |
| maxBW-PreferenceConfigFR2-2-r17 |  | Not present |  |  |
| maxMIMO-LayerPreferenceConfigFR2-2-r17 |  | Not present |  |  |
| minSchedulingOffsetPreferenceConfigExt-r17 |  | Not present |  |  |
| rlm-RelaxationReportingConfig-r17 |  | Not present |  |  |
| bfd-RelaxationReportingConfig-r17 |  | Not present |  |  |
| scg-DeactivationPreferenceConfig-r17 |  | Not present |  |  |
| rrm-MeasRelaxationReportingConfig-r17 |  | Not present |  |  |
| propDelayDiffReportConfig-r17 |  | Not present |  |  |
| } |  |  |  |  |
| sl-L2RelayUE-Config-r17 |  | Not present |  |  |
| sl-L2RelayUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RelayUE-Config with condition SL-SRB0 and SL-SRB1 |  | L2RelayUE |
| } |  |  |  |  |
| sl-L2RemoteUE-Config-r17 |  | Not present |  |  |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  | L2RemoteUE |
| setup |  | SL-L2RemoteUE-Config with condition SL-SRB2 and SL-DRB1 |  |  |
| } |  |  |  |  |
| dedicatedPagingDelivery-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigNR-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigEUTRA-r17 |  | Not present |  |  |
| musim-GapConfig-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 CHOICE { |  |  |  | NR-FR2- Uplink-Gaps |
| setup |  | UL-GapFR2-Config-r17 |  |  |
| } |  |  |  |  |
| scg-State-r17 |  | Not present |  |  |
|  |  | deactivated |  | DeactivatedSCG |
| appLayerMeasConfig-r17 |  | Not present |  |  |
| ue-TxTEG-RequestUL-TDOA-Config-r17 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| EN-DC_MEAS | An EN-DC measurement is configured |
| IRAT_MEAS | An IRAT measurement is configured in the NR PCell |
| NR_MEAS | A NR measurement is configured |
| NR | NR connected to 5GC |
| NR-DC | NR-NR Dual Connectivity is configured |
| NR-DC_SCG | Add SCG side configuration (NR-DC) |
| EN-DC_HO | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| EN-DC_SCell_add | Add SCell (EN-DC) |
| MasterKeyChange | MasterKeyUpdate when performing ReconfigurationWithSync and indicating a change of the AS security algorithms associated to the master key |
| Inter_Sys_HO | Used during inter-system handover to NR procedure |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure |
| SIDELINK | For NR sidelink dedicated configuration |
| DAPS_HO_ReleaseSource | The source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released. |
| CHO | Conditional handover |
| CPA | Conditional PSCell addition |
| CPC | Conditional PSCell change |
| NE-DC | NR E-UTRA Dual Connectivity |
| L2RelayUE | For L2 U2N Relay UE test cases. |
| L2RemoteUE | For L2 U2N Remote UE test cases. |
| RELAY | Add Uu Relay RLC channel configuration |
| NR-FR2-Upink-Gaps | Configure Uplink Gaps for NR FR2 |
| SHRT304 | Report the successful handover information to the network for the elapsed time of the timer T304 over the threshold |
| SHRT310 | Report the successful handover information to the network for the elapsed time of the timer T310 over the threshold |
| SHRT312 | Report the successful handover information to the network the ror elapsed time of the timer T312 over the threshold |
| SHRRLF | Report the successful handover information to the network for radio link failure |
| DeactivatedSCG | Indicate that SCG is in deactivated state |

<SECTIONS SKIPPED>

### 4.6.3	Radio resource control information elements

#### –	AdditionalSpectrumEmission

Table 4.6.3-1: AdditionalSpectrumEmission

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 |  |  |

#### –	Alpha

Table 4.6.3-2: Alpha

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Alpha | alpha0 |  |  |

#### –	AMF-Identifier

Table 4.6.3-3: AMF-Identifier

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AMF-Identifier | FFS |  |  |

#### –	ARFCN-ValueEUTRA

Table 4.6.3-4: ARFCN-ValueEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ARFCN-ValueEUTRA | The ARFCN value for the E-UTRA carrier frequency. | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. |  |

#### –	ARFCN-ValueNR

Table 4.6.3-5: ARFCN-ValueNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ARFCN-ValueNR | absoluteFrequencySSB as defined for the frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | DL_SSB |
|  | absoluteFrequencyPointA as defined for the DL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | DL_PointA |
|  | absoluteFrequencyPointA as defined for the UL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | UL_PointA |
|  | sl-absoluteFrequencySSB as defined for the SL NRf1 carrier | For signalling test cases see subclause 6.2.3.7. Otherwise, see subclause 4.3.1.8. | SL_SSB |
|  | sl-AbsoluteFrequencyPointA as defined for the SL NRf1 carrier | For signalling test cases see subclause 6.2.3.7. Otherwise, see subclause 4.3.1.8. | SL_PointA |

| Condition | Explanation |
| --- | --- |
| DL_SSB | IE absoluteFrequencySSB for downlink |
| SL_SSB | IE sl-absoluteFrequencySSB for sidelink |
| DL_PointA | IE absoluteFrequencyPointA for downlink |
| UL_PointA | IE absoluteFrequencyPointA for uplink |
| SL_PointA | IE sl-absoluteFrequencyPointA for sidelink |

#### –	ARFCN-ValueUTRA-FDD

Table 4.6.3-5A: ARFCN-ValueUTRA-FDD

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ARFCN-ValueUTRA-FDD-r16 | FFS |  |  |

#### –	AvailabilityCombinationsPerCell

Table 4.6.3-5B: AvailabilityCombinationsPerCell

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AvailabilityCombinationsPerCell-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	AvailabilityIndicator

Table 4.6.3-5C: AvailabilityIndicator

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AvailabilityIndicator-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BAP-Routing-ID

Table 4.6.3-5D: BAP-Routing-ID

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BAP-Routing-ID-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BeamFailureRecoveryConfig

Table 4.6.3-6: BeamFailureRecoveryConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | Set according to the PRACH-rootSequenceIndex given in Table 4.4.2-2 |  |  |
| rach-ConfigBFR | RACH-ConfigGeneric |  |  |
| rsrp-ThresholdSSB | 57 | Actual value = 100 dBm |  |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR { | 1 entry |  |  |
| PRACH-ResourceDedicatedBFR[1] CHOICE { |  | entry 1 |  |
| ssb SEQUENCE { |  |  | SSB |
| ssb | SSB-Index for BFR |  |  |
| ra-PreambleIndex | 56 |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-rs[1] SEQUENCE { |  |  | CSI-RS |
| csi-rs | NZP-CSI-RS-ResourceId for BFR |  |  |
| ra-OccasionList | Not present | The RA occasion associated with the SSB that is QCLed with this CSI-RS is used |  |
| ra-PreambleIndex | Not present | The preamble index associated with the SSB that is QCLed with CSI-RS is used |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | one |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| recoverySearchSpaceId | searchSpaceId for BFR search space | The CORESET associated with the BFR search space can not be associated with another search space according to 38.331 [6] |  |
| ra-Prioritization | RA-Prioritization |  |  |
| beamFailureRecoveryTimer | ms200 |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | SSB is used as reference signal for BFR |
| CSI-RS | CSI-RS is used as reference signal for BFR |

#### –	BeamFailureRecoveryRSConfig

Table 4.6.3-6AA: BeamFailureRecoveryRSConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryRSConfig-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

Table 4.6.3-6AB: Void

#### –	BetaOffsets

Table 4.6.3-6A: BetaOffsets

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BetaOffsets ::= SEQUENCE { |  |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |

#### –	BetaOffsetsCrossPri

Table 4.6.3-6BA: BetaOffsetsCrossPri

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BetaOffsetsCrossPri-r17 | FFS |  |  |

#### –	BH-RLC-ChannelConfig

Table 4.6.3-6B: BH-RLC-ChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BH-RLC-ChannelConfig-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BH-LogicalChannelIdentity

Table 4.6.3-6C: BH-LogicalChannelIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BH-LogicalChannelIdentity-r16 ::= CHOICE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BH-LogicalChannelIdentity-Ext

Table 4.6.3-6D: BH-LogicalChannelIdentity-Ext

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BH-LogicalChannelIdentity-Ext-r16 | FFS |  |  |

#### –	BH-RLC-ChannelID

Table 4.6.3-6E: BH-RLC-ChannelID

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BH-RLC-ChannelID-r16 | FFS |  |  |

#### –	BSR-Config

Table 4.6.3-7: BSR-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BSR-Config ::= SEQUENCE { |  |  |  |
| periodicBSR-Timer | sf1 |  |  |
| retxBSR-Timer | sf80 |  |  |
| logicalChannelSR-DelayTimer | Not present |  |  |
| } |  |  |  |

#### –	BWP

Table 4.6.3-8: BWP

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set to value of locationAndBandwidth in Table 4.3.1.0D-1 for the bandwidth and subcarrier spacing under test |  | FR1 |
|  | Set to value of locationAndBandwidth in Table 4.3.1.0D-2 for the bandwidth and subcarrier spacing under test |  | FR2 |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| cyclicPrefix | Not present |  |  |
| } |  |  |  |

#### –	BWP-Downlink

Table 4.6.3-9: BWP-Downlink

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id with condition BWP-Id1 |  |  |
| bwp-Common | BWP-DownlinkCommon with condition BWP-Id1 |  |  |
| bwp-Dedicated | BWP-DownlinkDedicated |  |  |
| } |  |  |  |

#### –	BWP-DownlinkCommon

Table 4.6.3-10: BWP-DownlinkCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon |  |  |
|  | PDCCH-ConfigCommon with condition SCell_add |  | SCell_add |
|  | PDCCH-ConfigCommon with condition InitialBWP_SIB |  | InitialBWP_SIB |
|  | PDCCH-ConfigCommon with condition BWP-Id1 |  | BWP-Id1 |
|  | PDCCH-ConfigCommon with condition PEI |  | PEI |
| } |  |  |  |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| InitialBWP_SIB | Configured via DownlinkConfigCommonSIB |
| BWP-Id1 | Additional BWP 1 |
| PEI | Paging Early Indication is configured in the cell. |
| SCell_add | Add SCell |

#### –	BWP-DownlinkDedicated

Table 4.6.3-11: BWP-DownlinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setu | PDSCH-Config |  |  |
| } |  |  |  |
| sps-Config | Not present |  |  |
| radioLinkMonitoringConfig | Not present |  | SCell_add |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| setup | RadioLinkMonitoringConfig |  |  |
| } |  |  |  |
| sps-ConfigToAddModList-r16 | Not present |  |  |
| sps-ConfigToReleaseList-r16 | Not present |  |  |
| sps-ConfigDeactivationStateList-r16 | Not present |  |  |
| beamFailureRecoverySCellConfig-r16 | Not present |  |  |
| sl-PDCCH-Config-r16 | Not present |  |  |
| sl-PDCCH-Config-r16 CHOICE { |  |  | SIDELINK |
| setup | PDCCH-Config with condition SLSS |  |  |
| } |  |  |  |
| sl-V2X-PDCCH-Config-r16 | Not present |  |  |
| preConfGapStatus-r17 | Not present |  |  |
| beamFailureRecoverySpCellConfig-r17 | Not present |  |  |
| harq-FeedbackEnablingforSPSactive-r17 | Not present |  |  |
| cfr-ConfigMulticast-r17 | Not present |  |  |
| cfr-ConfigMulticast-r17 CHOICE { |  |  | MBS_Multicast |
| setup | CFR-ConfigMulticast |  |  |
| } |  |  |  |
| dl-PPW-PreConfigToAddModList-r17 | Not present |  |  |
| dl-PPW-PreConfigToReleaseList-r17 | Not present |  |  |
| nonCellDefiningSSB-r17 | Not present |  |  |
| servingCellMO-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SIDELINK | Used for sidelink communication |
| SCell_add | Add SCell |
| MBS_Multicast | Used for MBS Multicast reception |

#### –	BWP-Id

Table 4.6.3-12: BWP-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Id | 0 | Initial BWP |  |
|  | 1 |  | BWP-Id1 |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Additional BWP 1 |

#### –	BWP-Uplink

Table 4.6.3-13: BWP-Uplink

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id with condition BWP-Id1 |  |  |
| bwp-Common | BWP-UplinkCommon |  |  |
| bwp-Dedicated | BWP-UplinkDedicated |  |  |
| } |  |  |  |

#### –	BWP-UplinkCommon

Table 4.6.3-14: BWP-UplinkCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon |  |  |
| } |  |  |  |
| rach-ConfigCommon | Not present |  | SUL_SUL AND (RF OR RRM), RF AND SCell_add |
| pusch-ConfigCommon CHOICE { |  |  |  |
| setup | PUSCH-ConfigCommon |  |  |
| } |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon |  |  |
| } |  |  |  |
| pucch-ConfigCommon | Not present |  | RF AND SCell_add |
| enableRA-PrioritizationForSlicing-r17 | Not present |  |  |
|  | true |  | Slice_RACH |
| additionalRACH-ConfigList-r17 | Not present |  |  |
| rsrp-ThresholdMsg3-r17 | Not present |  |  |
| numberOfMsg3-RepetitionsList-r17 | Not present |  |  |
| mcs-Msg3-Repetitions-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL_SUL | On the SUL carrier when supplementary carrier is configured |
| SCell_add | Add SCell |
| Slice_RACH | Slice specific RACH configuration |

#### –	BWP-UplinkDedicated

Table 4.6.3-15: BWP-UplinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| pucch-Config | Not present |  | SUL_NUL, RF AND SCell_add |
| pusch-Config | Not present |  | RESUME |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| configuredGrantConfig | Not present |  |  |
|  | ConfiguredGrantConfig |  | CG_Config |
| srs-Config | Not present |  | Short_DCI |
|  | SRS-Config |  |  |
| beamFailureRecoveryConfig | Not present |  |  |
| sl-PUCCH-Config-r16 | Not present |  |  |
| sl-PUCCH-Config-r16 CHOICE { |  |  | SIDELINK |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| cp-ExtensionC2-r16 | Not present |  |  |
| cp-ExtensionC3-r16 | Not present |  |  |
| useInterlacePUCCH-PUSCH-r16 | Not present |  |  |
| pucch-ConfigurationList-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 CHOICE { |  |  | SharedSpectrum |
| setup | LBT-FailureRecoveryConfig |  |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r16 | Not present |  |  |
| configuredGrantConfigToReleaseList-r16 | Not present |  |  |
| configuredGrantConfigType2DeactivationStateList-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SUL_NUL | On the NUL carrier when supplementary carrier is configured |
| RESUME | Used in RRCResume Message |
| SIDELINK | Used for sidelink communication |
| SCell_add | Add SCell |
| SharedSpectrum | Operation with shared spectrum channel access |
| CG_Config | Configured Grant Configuration |

#### –	CandidateBeamRS

Table 4.6.3-15A: CandidateBeamRS

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CandidateBeamRS-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CellAccessRelatedInfo

Table 4.6.3-16: CellAccessRelatedInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo ::= SEQUENCE { |  |  |  |
| plmn-IdentityInfoList | PLMN-IdentityInfoList |  |  |
| cellReservedForOtherUse | Not present |  |  |
|  | true |  | SNPN, CAG |
| cellReservedForFutureUse-r16 | Not present |  |  |
| npn-IdentityInfoList-r16 | Not present |  |  |
|  | NPN-IdentityInfoList-r16 |  | SNPN, CAG |
| snpn-AccessInfoList-r17 | Not present |  |  |
| snpn-AccessInfoList-r17 SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17 { | 1 entry |  | eNPN |
| SNPN-AccessInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| extCH-Supported-r17 | true |  |  |
| extCH-WithoutConfigAllowed-r17 | true |  |  |
| onboardingEnabled-r17 | true |  |  |
| imsEmergencySupportForSNPN-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SNPN | Standalone NPN cell |
| CAG | PNI-NPN |
| eNPN | Rel-17 enhanced NPN cell |

#### –	CellAccessRelatedInfo-EUTRA-5GC

Table 4.6.3-17: CellAccessRelatedInfo-EUTRA-5GC

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo-EUTRA-5GC ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CellAccessRelatedInfo-EUTRA-EPC

Table 4.6.3-18: CellAccessRelatedInfo-EUTRA-EPC

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo-EUTRA-EPC ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CellGroupConfig

Table 4.6.3-19: CellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId |  |  |
|  | CellGroupId condition NR-DC_SCG |  | NR-DC_SCG |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | EN-DC |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRB2 | entry 1 |  |
|  | RLC-BearerConfig with conditions AM and DRB2 and Re-establish_RLC |  | PSCell_change |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | SRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB1 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB2 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB2 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | DRBn, NR-DC_SCG |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRBn | entry 1 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | PCell_change |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRB1 and Re-establish_RLC | entry 3 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1+n entries | n is the number of DRBs established before RRC resume | RESUME |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 and RESUME | entry 1 |  |
| RLC-BearerConfig[k+1, k=1..n] | RLC-BearerConfig with condition DRBk and RESUME | entry [k+1, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2+n entries | n is the number of DRBs established before RRC re-establishement | REEST |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[k+2, k=1..n] | RLC-BearerConfig with condition DRBk and Re-establish_RLC | entry [k+2, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | MRBm AND (AM_PTP OR UM_bi_PTP OR UM_PTP OR UM_PTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AM_PTP |
|  | RLC-BearerConfig with conditions UM and PTP |  | UM_bi_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UM_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTM |  | UM_PTM |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | MRBm_DRBn AND (AM_PTP OR UM_bi_PTP OR UM_PTP OR UM_PTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP and MRBm | entry 1 | AM_PTP |
|  | RLC-BearerConfig with conditions UM and PTP and MRBm |  | UM_bi_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP and MRBm |  | UM_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm |  | UM_PTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRBn | entry 2 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | MRBm AND (AMPTP_UMPTM OR UMPTP_UMPTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP and MRBm | entry 1 | AMPTP_UMPTM |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP and MRBm |  | UMPTP_UMPTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 | AMPTP_UMPTM, UMPTP_UMPTM |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | MRBm_DRBn AND (AMPTP_UMPTM OR UMPTP_UMPTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP and MRBm | entry 1 | AMPTP_UMPTM |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP and MRBm |  | UMPTP_UMPTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 | AMPTP_UMPTM, UMPTP_UMPTM |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBn | entry 3 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| rlc-BearerToReleaseList | Not present |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig |  |  |
|  | MAC-CellGroupConfig with condition MBS_Multicast |  | AMPTP_UMPTM, UMPTP_UMPTM, UM_PTM |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add |
| physicalCellGroupConfig | PhysicalCellGroupConfig |  |  |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add, MRBm, MRBm_DRBn |
| spCellConfig | Not present |  | SRB2_DRB1, SRB2_DRB2, SCell_add, AM_PTP, UM_bi_PTP, UM_PTP |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
|  | ServCellIndex |  | EN-DC, EN-DC AND MEAS |
|  | ServCellIndex with condition NR-DC_SCG |  | NR-DC_SCG |
| reconfigurationWithSync | Not present |  |  |
| reconfigurationWithSync SEQUENCE { |  |  | EN-DC, PCell_change, PSCell_change, NR-DC_SCG |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| newUE-Identity | RNTI-Value |  |  |
|  | RNTI-Value with condition NR-DC_SCG |  | NR-DC_SCG |
| t304 | ms1000 |  |  |
| rach-ConfigDedicated | Not present |  |  |
| rach-ConfigDedicated CHOICE { |  |  | CFRA |
| uplink | RACH-ConfigDedicated |  |  |
| supplementaryUplink | RACH-ConfigDedicated |  | SUL AND SIG |
| } |  |  |  |
| } |  |  |  |
| rlf-TimersAndConstants CHOICE { |  |  |  |
| setup | RLF-TimersAndConstants |  |  |
| } |  |  |  |
| rlf-TimersAndConstants | Not present |  | MEAS, RESUME |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig |  | EN-DC, SRB1, PCell_change, PSCell_change, NR-DC_SCG, REEST |
|  | Not present |  |  |
|  | ServingCellConfig with condition MEAS |  | MEAS |
|  | ServingCellConfig with condition RESUME |  | RESUME |
|  | ServingCellConfig with condition MBS_Multicast |  | AMPTP_UMPTM, UMPTP_UMPTM, UM_PTM |
| lowMobilityEvaluationConnected-r17 | Not present |  |  |
| goodServingCellEvaluationRLM-r17 | Not present |  |  |
| goodServingCellEvaluationBFD-r17 | Not present |  |  |
| deactivatedSCG-Config-r17 | Not present |  |  |
| deactivatedSCG-Config-r17 CHOICE { |  |  | RLM_BFD_PSCell |
| setup SEQUENCE { |  |  |  |
| bfd-and-RLM-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  | SCell_add |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition No_UL and SCell_add |  |  |
|  | ServingCellConfigCommon with condition SCell_add |  | RF AND UL_CA |
| sCellConfigDedicated | ServingCellConfig with condition No_UL and SCell_add |  |  |
|  | ServingCellConfig with condition SCell_add |  | RF AND UL_CA |
| } |  |  |  |
| } |  |  |  |
| sCellToReleaseList | Not present |  |  |
| f1c-TransferPathNRDC-r17 | Not present |  |  |
| uplinkTxSwitching-2T-Mode-r17 | Not present |  |  |
| uplinkTxSwitching-DualUL-TxState-r17 | Not present |  |  |
| uu-RelayRLC-ChannelToAddModList-r17 | Not present |  |  |
| uu-RelayRLC-ChannelToAddModList-r17 SEQUENCE (SIZE (1.. maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelConfig-r17{ |  |  | RELAY |
| Uu-RelayRLC-ChannelConfig-r17[1] | Uu-RelayRLC-ChannelConfig |  |  |
| } |  |  |  |
| uu-RelayRLC-ChannelToReleaseList-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList1-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList2-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList3-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList4-r17 | Not present |  |  |
| rlc-BearerToReleaseListExt-r17 | Not present |  |  |
| iab-ResourceConfigToAddModList-r17 | Not present |  |  |
| iab-ResourceConfigToReleaseList-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| CFRA | This condition applies when CFRA is configured |
| SUL | Supplementary Uplink |
| SRB1 | Establishment of SRB1 |
| SRB2_DRB1 | Establishment of SRB2 and DRB1 |
| SRB2_DRB2 | Establishment of SRB2 and DRB2 |
| DRBn | Establishment of DRBn |
| PCell_change | Intra-NR PCell change NR) |
| PSCell_change | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| MEAS | A NR or EN-DC measurement is configured |
| NR-DC_SCG | Add SCG (NR-DC) |
| RESUME | Used in RRCResume Message |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure. |
| UL_CA | Uplink CA |
| MRBm | Establishment of MRBm |
| MRBm_DRBn | Establishment of MRBm and DRBn |
| AM_PTP | Multicast MRB with RLC-AM entity configuration for receiving PTP transmission |
| UM_bi_PTP | Multicast MRB with bidirectional RLC-UM configuration for receiving PTP transmission |
| UM_PTP | Multicast MRB with DL only RLC-UM configuration for receiving PTP transmission |
| UM_PTM | Multicast MRB with DL only RLC-UM entity for receiving PTM transmission |
| AMPTP_UMPTM | Multicast MRB with two RLC entities, one RLC-AM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |
| UMPTP_UMPTM | Multicast MRB with two RLC-UM entities, one DL only RLC-UM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |
| RELAY | This condition applies when Relay UE is configured |
| RLM_BFD_PSCell | Perform RLM and BFD on the PSCell when the SCG is deactivated |

#### –	CellGroupId

Table 4.6.3-20: CellGroupId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupId | 0 |  |  |
|  | 1 |  | EN-DC, NR-DC_SCG |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| NR-DC_SCG | Add SCG (NR-DC) |

#### –	CellIdentity

Table 4.6.3-21: CellIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellIdentity | Set to NR Cell Identifier defined in table 4.4.2-2 | BIT STRING (SIZE (36)) |  |

#### –	CellReselectionPriority

Table 4.6.3-22: CellReselectionPriority

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellReselectionPriority | 4 |  |  |
|  | 5 |  | Priority_5 |

| Condition | Explanation |
| --- | --- |
| Priority_5 | Priority 5 |

#### –	CellReselectionSubPriority

Table 4.6.3-23: CellReselectionSubPriority

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellReselectionSubPriority | FFS |  |  |

#### –	CFR-ConfigMulticast

Table 4.6.3-23AA: CFR-ConfigMulticast

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CFR-ConfigMulticast-r17 ::= SEQUENCE { |  |  |  |
| locationAndBandwidthMulticast-r17 | Not present |  |  |
| pdcch-ConfigMulticast-r17 | PDCCH-Config with condition MSS |  |  |
| pdsch-ConfigMulticast-r17 | PDSCH-Config with condition MBS_Multicast |  |  |
| sps-ConfigMulticastToAddModList-r17 | Not present |  |  |
| sps-ConfigMulticastToReleaseList-r17 | Not present |  |  |
| } |  |  |  |

#### –	CGI-InfoEUTRA

Table 4.6.3-23A: CGI-InfoEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CGI-InfoEUTRA | FFS |  |  |

#### –	CGI-InfoEUTRALogging

Table 4.6.3-23B: CGI-InfoEUTRALogging

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CGI-InfoEUTRALogging ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CGI-InfoNR

Table 4.6.3-24: CGI-InfNRo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CGI-InfoNR | FFS |  |  |

#### –	CGI-Info-Logging

Table 4.6.3-24A: CGI-Info-Logging

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CGI-Info-Logging-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CLI-RSSI-Range

Table 4.6.3-24B: CLI-RSSI-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CLI-RSSI-Range-r16 | FFS |  |  |

#### –	CodebookConfig

Table 4.6.3-25: CodebookConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| two SEQUENCE { |  |  | 2TX |
| twoTX-CodebookSubsetRestriction | 111111 |  |  |
| } |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| two-one-TypeI-SinglePanel-Restriction | 11111111 |  | FR2 |
| four-one-TypeI-SinglePanel-Restriction | 11111111 11111111 |  | FR1 |
| } |  |  |  |
| typeI-SinglePanel-codebookSubsetRestriction-i2 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 11111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX | 2 Tx CSI-RS configuration. |

#### –	CommonLocationInfo

Table 4.6.3-25A: CommonLocationInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CommonLocationInfo-r16 ::= SEQUENCE { |  |  |  |
| gnss-TOD-msec-r16 | Not checked | OCTET STRING |  |
| locationTimestamp-r16 | Not checked | OCTET STRING |  |
| locationCoordinate-r16 | Not checked | OCTET STRING |  |
| locationError-r16 | Not checked | OCTET STRING |  |
| locationSource-r16 | Not checked | OCTET STRING |  |
| velocityEstimate-r16 | Not checked | OCTET STRING |  |
| } |  |  |  |

#### –	CondReconfigId

Table 4.6.3-25B: CondReconfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigId-r16 | 1 |  |  |

#### –	CondReconfigToAddModList

Table 4.6.3-25C: CondReconfigToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16[1] ::= SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | CondReconfigId-r16 |  |  |
| condExecutionCond-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 | identify a measurement configuration |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO | Table 4.8.1-1A |  |
|  | RRCReconfiguration with condition NR-DC_SCG |  | CPA |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CPA | Conditional PSCell addition |

#### –	ConditionalReconfiguration

Table 4.6.3-25D: ConditionalReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| attemptCondReconfig-r16 | true |  |  |
| condReconfigToRemoveList-r16 | Not present |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList |  |  |
| } |  |  |  |

#### –	ConfiguredGrantConfig

Table 4.6.3-26: ConfiguredGrantConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| frequencyHopping | Not present |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| uci-OnPUSCH | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| rbg-Size | Not present |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 0 |  |  |
| transformPrecoder | Not present |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| repK-RV | Not present |  |  |
| periodicity | Sym40x14 | 40ms | SCS15 |
|  | Sym80x14 | 40ms | SCS30 |
|  | Sym160x14 | 40ms | SCS60 |
|  | Sym320x14 | 40ms | SCS120 |
| configuredGrantTimer | Not present |  |  |
| rrc-ConfiguredUplinkGrant | Not present |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  | CG_Config_Type1 |
| timeDomainOffset | 9 |  |  |
| timeDomainAllocation | 0 |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
| antennaPort | 2 |  |  |
| dmrs-SeqInitialization | Not present |  |  |
| precodingAndNumberOfLayers | 1 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 1 |  |  |
| frequencyHoppingOffset | Not present |  |  |
| pathlossReferenceIndex | 0 |  |  |
| pathlossReferenceIndex2-r17 | Not present |  |  |
| srs-ResourceIndicator2-r17 | Not present |  |  |
| precodingAndNumberOfLayers2-r17 | Not present |  |  |
| timeDomainAllocation-v1710 | Not present |  |  |
| timeDomainOffset-r17 | Not present |  |  |
| cg-SDT-Configuration-r17 | Not present |  |  |
| cg-SDT-Configuration-r17 SEQUENCE { |  |  | pc_cg_SDT_r17 |
| cg-SDT-RetransmissionTimer | Not present |  |  |
| sdt-SSB-Subset-r17 | Not present | If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs configured by SIB1. |  |
| sdt-SSB-PerCG-PUSCH-r17 | one |  |  |
| sdt-P0-PUSCH-r17 | 0 |  |  |
| sdt-Alpha-r17 | alpha08 |  |  |
| sdt-DMRS-Ports-r17 CHOICE { |  |  |  |
| dmrsType1-r17 | ’100000000000’B | Port0 |  |
| } |  |  |  |
| sdt-NrofDMRS-Sequences-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CG_Config_Type1 | Configured Grant Configuration Type1 |

#### –	ConfiguredGrantConfigIndex

Table 4.6.3-26A: ConfiguredGrantConfigIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfigIndex-r16 | FFS |  |  |

#### –	ConfiguredGrantConfigIndexMAC

Table 4.6.3-26B: ConfiguredGrantConfigIndexMAC

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfigIndexMAC-r16 | FFS |  |  |

#### –	ConnEstFailureControl

Table 4.6.3-27: ConnEstFailureControl

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConnEstFailureControl ::= SEQUENCE { |  |  |  |
| connEstFailCount | n1 |  |  |
| connEstFailOffsetValidity | s30 |  |  |
| connEstFailOffset | 1 |  |  |
| } |  |  |  |

#### –	ControlResourceSet

Table 4.6.3-28: ControlResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP |  |
| duration | 2 | SearchSpace duration of 2 symbols |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| tci-StatesPDCCH-ToReleaseList | Not present |  |  |
| tci-PresentInDCI | Not present |  |  |
| pdcch-DMRS-ScramblingID | Not present |  |  |
| } |  |  |  |

#### –	ControlResourceSetId

Table 4.6.3-29: ControlResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSetId | 1 |  |  |
|  | 0 |  | Common0 |

| Condition | Explanation |
| --- | --- |
| Common0 | CommonCORESET#0 |

#### –	ControlResourceSetZero

Table 4.6.3-30: ControlResourceSetZero

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSetZero | Set to CORESET#0 Index as defined for the frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. |  |
|  | 0 |  | NCD-SSB |

| Condition | Explanation |
| --- | --- |
| NCD-SSB | MIB for NCD-SSB |

#### –	CrossCarrierSchedulingConfig

Table 4.6.3-31: CrossCarrierSchedulingConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CrossCarrierSchedulingConfig ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CSI-AperiodicTriggerStateList

Table 4.6.3-32: CSI-AperiodicTriggerStateList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState { | 1 entry |  |  |
| CSI-AperiodicTriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 8 |  | FR1 |
|  | 16 |  | FR2 |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | 8 |  | FR1 |
|  | 16 |  | FR2 |
| nzp-CSI-RS-ResourcesforInterference | 8 |  | FR1 |
|  | 16 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-FrequencyOccupation

Table 4.6.3-33: CSI-FrequencyOccupation

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| startingRB | 0 |  |  |
| nrofRBs | 160 |  | FR1 AND BW60 |
|  | 216 |  | FR1 AND BW80 |
|  | 108 |  | FR1 AND BW40 |
|  | 52 |  | FR1 AND BW10 |
|  | 276 |  | FR1 AND BW100 |
|  | 64 |  | FR2 AND BW100 |
|  | 52 |  | TRS |
|  | 40 |  | TRS AND ((FR1 AND BW15 AND SCS30) OR (FR1 AND BW30AND SCS60)) |
|  | 32 |  | TRS AND ((FR1 AND BW25 AND SCS60) OR (FR2 AND BW50 AND SCS120)) |
|  | 28 |  | TRS AND FR1 AND BW5 AND SCS15 |
|  | 24 |  | TRS AND ((FR1 AND BW5 AND SCS30) OR (FR1 AND BW10 AND SCS30) OR (FR1 AND BW10 AND SCS60) OR (FR1 AND BW15 AND SCS60) OR (FR1 AND BW20 AND SCS60)) |
|  | 64 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal |

#### –	CSI-IM-Resource

Table 4.6.3-34: CSI-IM-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| pattern1 SEQUENCE { |  |  |  |
| subcarrierLocation-p1 | s4 |  |  |
| symbolLocation-p1 | 3 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| periodicityAndOffset | Not present |  |  |
| } |  |  |  |

#### –	CSI-IM-ResourceId

Table 4.6.3-35: CSI-IM-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | 7 |  | FR1 |
|  | 31 |  | FR2 |

#### –	CSI-IM-ResourceSet

Table 4.6.3-36: CSI-IM-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId |  |  |
| csi-IM-Resources SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-IM-ResourceSetId

Table 4.6.3-37: CSI-IM-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

#### –	CSI-MeasConfig

Table 4.6.3-38: CSI-MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource | entry 1 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToReleaseList | Not present |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToReleaseList | Not present |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceSetToReleaseList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-SSB-ResourceSet[1] | CSI-SSB-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetToReleaseList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig | entry 1 |  |
| } |  |  |  |
| csi-ResourceConfigToReleaseList | Not present |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | entry 1 |  |
| } |  |  |  |
| csi-ReportConfigToReleaseList | Not present |  |  |
| reportTriggerSize | 0 |  |  |
| aperiodicTriggerStateList CHOICE { |  |  |  |
| setup | CSI-AperiodicTriggerStateList |  |  |
| } |  |  |  |
| semiPersistentOnPUSCH-TriggerStateList | Not present |  |  |
| reportTriggerSizeDCI-0-2-r16 | Not present |  |  |
| } |  |  |  |

#### –	CSI-ReportConfig

Table 4.6.3-39: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| nzp-CSI-RS-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER { | 1 entry |  |  |
| INTEGER[1] | 14 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL, |  | FR1 |
| cri-RI-LI-PMI-CQI | NULL |  | FR2 |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand | Not present |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table1 | 64QAM |  |
|  | table2 |  | 256QAM |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 256QAM | Test cases using 256QAM for PDSCH |

#### –	CSI-ReportConfigId

Table 4.6.3-40: CCSI-ReportConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigID | 0 |  |  |

#### –	CSI-ResourceConfig

Table 4.6.3-41: CSI-ResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 0 | entry 1 |  |
| NZP-CSI-RS-ResourceSetId[2] | 1 | entry 2 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

#### –	CSI-ResourceConfigId

Table 4.6.3-42: CSI-ResourceConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | 0 |  |  |

#### –	CSI-ResourcePeriodicityAndOffset

Table 4.6.3-43: CSI-ResourcePeriodicityAndOffset

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots80 | 10 |  | FR1 |
| slots320 | 40 |  | FR2 |
| } |  |  |  |

#### –	CSI-RS-ResourceConfigMobility

Table 4.6.3-44: CSI-RS-ResourceConfigMobility

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceConfigMobility ::= SEQUENCE { |  |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| csi-RS-CellList-Mobility | FFS |  |  |
| } |  |  |  |

#### –	CSI-RS-ResourceMapping

Table 4.6.3-45: CSI-RS-ResourceMapping

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 1000 |  | (FR1 AND TRS) OR (FR2 AND TRS) |
| row4 | 010 |  | FR2 |
| other | 011110 |  | FR1 |
| } |  |  |  |
| nrofPorts | p8 |  | FR1 |
|  | p4 |  | FR2 |
|  | p1 |  | (FR1 AND TRS) OR (FR2 AND TRS) |
| firstOFDMSymbolInTimeDomain | 3 |  | FR1 |
|  | 13 |  | FR2 |
|  | 4 |  | (FR1 AND TRS) OR (FR2 AND TRS) |
| firstOFDMSymbolInTimeDomain2 | Not present |  |  |
| cdm-Type | fd-CDM2 |  |  |
|  | noCDM |  | TRS |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| three | NULL |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal is configured. |

#### –	CSI-SemiPersistentOnPUSCH-TriggerStateList

Table 4.6.3-46: CSI-SemiPersistentOnPUSCH-TriggerStateList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SemiPersistentOnPUSCH-TriggerStateList ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CSI-SSB-ResourceSet

Table 4.6.3-47: CSI-SSB-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SSB-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-SSB-ResourceSetId | CSI-SSB-ResourceSetId |  |  |
| csi-SSB-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index { | 1 entry |  |  |
| SSB-Index[1] | SSB-Index | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-SSB-ResourceSetId

Table 4.6.3-48: CSI-SSB-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SSB-ResourceSetId | 0 |  |  |

Table 4.6.3-48A:Void

#### –	DedicatedNAS-Message

Table 4.6.3-49: DedicatedNAS-Message

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DedicatedNAS-Message | Set according to specific message content |  |  |

#### –	DL-PPW-PreConfig

Table 4.6.3-49A: DL-PPW-PreConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DL-PPW-PreConfig-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	DMRS-BundlingPUCCH-Config

Table 4.6.3-49B: DMRS-BundlingPUCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-BundlingPUCCH-Config-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	DMRS-BundlingPUSCH-Config

Table 4.6.3-49C: DMRS-BundlingPUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-BundlingPUSCH-Config-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	DMRS-DownlinkConfig

Table 4.6.3-50: DMRS-DownlinkConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-Type | Not present | DMRS type 1 |  |
| dmrs-AdditionalPosition | pos1 |  | FR1 |
|  | pos0 |  | FR2 |
| maxLength | Not present | len1 |  |
| scramblingID0 | Not present |  |  |
| scramblingID1 | Not present |  |  |
| phaseTrackingRS | Not present |  | FR1 |
| phaseTrackingRS CHOICE { |  |  | FR2 |
| setup | PTRS-DownlinkConfig |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	DMRS-UplinkConfig

Table 4.6.3-51: DMRS-UplinkConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-Type | Not present | DMRS type 1 |  |
| dmrs-AdditionalPosition | pos1 |  | FR1 |
|  | pos0 |  | FR2 |
| phaseTrackingRS | Not present |  |  |
| phaseTrackingRS CHOICE { |  |  | PTRS_UL_CONFIG |
| setup | PTRS-UplinkConfig |  |  |
| } |  |  |  |
| maxLength | Not present | len1 |  |
| transformPrecodingDisabled SEQUENCE { |  |  |  |
| scramblingID0 | Not present |  |  |
| scramblingID1 | Not present |  |  |
| } |  |  |  |
| transformPrecodingEnabled | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |

#### –	DownlinkConfigCommon

Table 4.6.3-52: DownlinkConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon |  |  |
|  | BWP-DownlinkCommon with condition SCell_add |  | SCell_add |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_add | Add SCell |

#### –	DownlinkConfigCommonSIB

Table 4.6.3-53: DownlinkConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL-SIB |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon with condition InitialBWP_SIB |  |  |
|  | BWP-DownlinkCommon with condition PEI |  | PEI |
| bcch-Config SEQUENCE { |  |  |  |
| modificationPeriodCoeff | n4 |  |  |
| } |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |
| defaultPagingCycle | rf128 |  |  |
| nAndPagingFrameOffset CHOICE { |  |  |  |
| halfT | 0 |  |  |
|  | 1 |  | PEI |
| } |  |  |  |
| ns | one |  |  |
|  | two |  | PEI |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16 | Not present |  |  |
| } |  |  |  |
| pei-Config-r17 | Not present |  |  |
| pei-Config-r17 SEQUENCE { |  |  | PEI |
| po-NumPerPEI-r17 | po2 |  |  |
| payloadSizeDCI-2-7-r17 | 22 |  |  |
| pei-FrameOffset-r17 | 3 |  |  |
| subgroupConfig-r17 SEQUENCE { |  |  |  |
| subgroupsNumPerPO-r17 | 2 |  |  |
| subgroupsNumForUEID-r17 | 2 |  |  |
| } |  |  |  |
| lastUsedCellOnly-r17 | Not present |  |  |
| } |  |  |  |
| initialDownlinkBWP-RedCap-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PEI | Paging Early Indication is configured in the cell. |

#### –	DownlinkPreemption

Table 4.6.3-54: DownlinkPreemption

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkPreemption ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	DRB-Identity

Table 4.6.3-55: DRB-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-Identity | n |  | DRBn |

| Condition | Explanation |
| --- | --- |
| DRBn | DRB-Identity n |

#### –	DRX-Config

Table 4.6.3-56: DRX-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms6 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms1280 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | sl16 |  | FR1 |
|  | sl64 |  | FR2 |
| drx-RetransmissionTimerUL | sl16 |  | FR1 |
|  | sl64 |  | FR2 |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10240 | 0 |  |  |
| } |  |  |  |
| shortDRX | not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |

#### –	DRX-ConfigSecondaryGroup

Table 4.6.3-56A: DRX-ConfigSecondaryGroup

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-ConfigSecondaryGroup-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	DRX-ConfigSL

Table 4.6.3-56B: DRX-ConfigSL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RX-ConfigSL-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	EphemerisInfo

Table 4.6.3-56C: EphemerisInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  | GSO |
| positionX-r17 | -16976014 |  |  |
| positionY-r17 | 27636499 |  |  |
| positionZ-r17 | 0 |  |  |
| velocityVX-r17 | 0 |  |  |
| velocityVY-r17 | 0 |  |  |
| velocityVZ-r17 | 0 |  |  |
| } |  |  |  |
| orbital-r17 SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| meanAnomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

#### –	FeatureCombination

Table 4.6.3-56D: FeatureCombination

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombination-r17 ::= SEQUENCE { |  |  |  |
| redCap-r17 | Not present |  |  |
| smallData-r17 | Not present |  |  |
| nsag-r17 | Not present |  |  |
| nsag-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF NSAG-ID-r17{ | n entries | n is the number of NSAG values associated with the preambles indicated in FeatureCombinationPreambles | Slice_RACH |
| NSAG-ID-r17[k, k=1..n] | Set to the corresponding NSAG value used in the test case | entry [k, k=1..n] |  |
| } |  |  |  |
| msg3-Repetitions-r17 | Not present |  |  |
|  | true |  | MSG3_REP |
| spare4 | Not present |  |  |
| spare3 | Not present |  |  |
| spare2 | Not present |  |  |
| spare1 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Slice_RACH | Slice specific RACH configuration |
| MSG3_REP | Msg.3 repetition configuration |

#### –	FeatureCombinationPreambles

Table 4.6.3-56E: FeatureCombinationPreambles

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 | FeatureCombination-r17 |  |  |
| startPreambleForThisPartition-r17 | 0 |  |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 8 |  | FR1 |
|  | 4 |  | FR2 |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | Not present |  |  |
| msgA-RSRP-Threshold-r17 | RSRP-Range |  |  |
| rsrp-ThresholdSSB-r17 | RSRP-Range |  |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

#### –	FilterCoefficient

Table 4.6.3-57: FilterCoefficient

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FilterCoefficient | fc4 |  |  |

#### –	FreqBandIndicatorNR

Table 4.6.3-58: FreqBandIndicatorNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandIndicatorNR | Operating band of the frequency as specified in Table 4.4.2-1 |  |  |
|  | Secondary band under test |  | CA-InterBand, NR-DC-SecondaryBand |

| Condition | Explanation |
| --- | --- |
| CA-InterBand | Used in CA interBand test cases |
| NR-DC-SecondaryBand | Used in NR-DC test cases |

#### –	FreqPriorityListDedicatedSlicing

Table 4.6.3-58AA: FreqPriorityListDedicatedSlicing

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqPriorityDedicatedSlicing-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	FreqPriorityListSlicing

Table 4.6.3-58A: FreqPriorityListSlicing

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqPriorityListSlicing-r17 ::= SEQUENCE (SIZE (1..maxFreqPlus1)) OF FreqPrioritySlicing-r17 { | 2 entries |  |  |
| FreqPrioritySlicing-r17[1] SEQUENCE { |  | entry 1 |  |
| dl-ImplicitCarrierFreq-r17 | 1 |  |  |
| sliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17 { | 1 entry |  |  |
| SliceInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| nsag-IdentityInfo-r17 | NSAG-IdentityInfo |  |  |
| nsag-CellReselectionPriority-r17 | CellReselectionPriority |  |  |
| nsag-CellReselectionSubPriority-r17 | Not present |  |  |
| sliceCellListNR-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqPrioritySlicing-r17[2] SEQUENCE { |  | entry 2 |  |
| dl-ImplicitCarrierFreq-r17 | 2 |  |  |
| sliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17 { | 1 entry |  |  |
| SliceInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| nsag-IdentityInfo-r17 | NSAG-IdentityInfo |  |  |
| nsag-CellReselectionPriority-r17 | CellReselectionPriority with condition Priority_5 |  |  |
| nsag-CellReselectionSubPriority-r17 | Not present |  |  |
| sliceCellListNR-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	FrequencyInfoDL

Table 4.6.3-59: FrequencyInfoDL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoDL::= SEQUENCE { |  |  |  |
| absoluteFrequencySSB | ARFCN-ValueNR with condition DL_SSB |  |  |
| frequencyBandList | MultiFrequencyBandListNR |  |  |
| absoluteFrequencyPointA | ARFCN-ValueNR with condition DL_PointA |  |  |
| scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { | 1 entry |  |  |
| SCS-SpecificCarrier[1] | SCS-SpecificCarrier with condition DL_PointA | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	FrequencyInfoDL-SIB

Table 4.6.3-60: FrequencyInfoDL-SIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoDL-SIB ::= SEQUENCE { |  |  |  |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  |  |
| offsetToPointA | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. |  |  |
| scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { | 1 entry |  |  |
| SCS-SpecificCarrier[1] | SCS-SpecificCarrier with condition DL_PointA | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	FrequencyInfoUL

Table 4.6.3-61: FrequencyInfoUL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL::= SEQUENCE { |  |  |  |
| frequencyBandList | MultiFrequencyBandListNR |  | FDD |
|  | Not present |  | TDD |
| absoluteFrequencyPointA | ARFCN-ValueNR with condition UL_PointA |  | FDD |
|  | Not present |  | TDD |
| scs-SpecificCarriers SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { | 1 entry |  |  |
| SCS-SpecificCarrier1 | SCS-SpecificCarrier with condition UL_PointA | entry 1 |  |
| } |  |  |  |
| additionalSpectrumEmission | AdditionalSpectrumEmission |  |  |
| p-Max | P-Max |  |  |
| frequencyShift7p5khz | Not present |  |  |
|  | true |  | DSS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DSS | Dynamic Spectrum Sharing |

#### –	FrequencyInfoUL-SIB

Table 4.6.3-62: FrequencyInfoUL-SIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB ::= SEQUENCE { |  |  |  |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  | FDD |
|  | Not present |  | TDD |
| absoluteFrequencyPointA | ARFCN-ValueNR with condition UL_PointA |  | FDD |
|  | Not present |  | TDD |
| scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { | 1 entry |  |  |
| SCS-SpecificCarrier[1] | SCS-SpecificCarrier with condition UL_PointA | entry 1 |  |
| } |  |  |  |
| p-Max | P-Max |  |  |
| frequencyShift7p5khz | Not present |  |  |
|  | true |  | DSS |
| } |  |  |  |

#### –	GapPriority

Table 4.6.3-62AA: GapPriority

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GapPriority-r17 | FFS |  |  |

#### –	HighSpeedConfig

Table 4.6.3-62A: HighSpeedConfig-r16

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HighSpeedConfig-r16 ::= SEQUENCE { |  |  |  |
| highSpeedMeasFlag-r16 | Not present |  |  |
| highSpeedDemodFlag-r16 | Not present |  |  |
| } |  |  |  |

Table 4.6.3-62B: HighSpeedConfig-v1700

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HighSpeedConfig-v1700 ::= SEQUENCE { |  |  |  |
| highSpeedMeasCA-Scell-r17 | Not present |  |  |
| highSpeedMeasInterFreq-r17 | Not present |  |  |
| highSpeedDemodCA-Scell-r17 | Not present |  |  |
| } |  |  |  |

Table 4.6.3-62C: HighSpeedConfigFR2

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HighSpeedConfigFR2-r17 ::= SEQUENCE { |  |  |  |
| highSpeedMeasFlagFR2-r17 | set2 |  |  |
| highSpeedDeploymentTypeFR2-r17 | bidirectional |  |  |
| highSpeedLargeOneStepUL-TimingFR2-r17 | true |  |  |
| } |  |  |  |

#### –	Hysteresis

Table 4.6.3-63: Hysteresis

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Hysteresis | 4 |  |  |

#### –	HysteresisLocation

Table 4.6.3-63AA: HysteresisLocation

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HysteresisLocation-r17 | FFS |  |  |

#### –	InvalidSymbolPattern

Table 4.6.3-63A: InvalidSymbolPattern

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| InvalidSymbolPattern-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	I-RNTI-Value

Table 4.6.3-64: I-RNTI-Value

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| I-RNTI-Value | SS arbitrarily selects a value between ’00 0000 0001’H and ‘FF FFFF FFFF’H. | BIT STRING (SIZE(40)) |  |

#### –	LBT-FailureRecoveryConfig

Table 4.6.3-64A: LBT-FailureRecoveryConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LBT-FailureRecoveryConfig-r16 ::= SEQUENCE { |  |  |  |
| lbt-FailureInstanceMaxCount-r16 | n4 |  |  |
| lbt-FailureDetectionTimer-r16 | ms80 |  |  |
| } |  |  |  |

#### –	LocationInfo

Table 4.6.3-64B: LocationInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LocationInfo-r16 ::= SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | CommonLocationInfo-r16 |  | MDT_LocationInfo |
| bt-LocationInfo-r16 | LogMeasResultListBT-r16 |  | MDT_BT |
| wlan-LocationInfo-r16 | LogMeasResultListWLAN-r16 |  | MDT_WLAN |
| sensor-LocationInfo-r16 | Sensor-LocationInfo-r16 |  | MDT_Sensor |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MDT_LocationInfo | Used for CommonLocationInfo included in MDT (Minimized Driving Test) |
| MDT_BT | Used for Bluetooth measurement in MDT (Minimized Driving Test) services |
| MDT_WLAN | Used for WLAN measurement in MDT (Minimized Driving Test) services |
| MDT_Sensor | Used for Sensor measurement in MDT (Minimized Driving Test) services |

#### –	LocationMeasurementInfo

Table 4.6.3-65: LocationMeasurementInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LocationMeasurementInfo ::= CHOICE { |  |  |  |
| eutra-RSTD SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info { | 1 entry |  |  |
| EUTRA-RSTD-Info[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueEUTRA |  |  |
| measPRS-Offset | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.6.3-65A: Void

#### –	LogicalChannelConfig

Table 4.6.3-66: LogicalChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LogicalChannelConfig ::= SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| priority | 1 |  |  |
|  | 3 |  | SRB2, SRB4 |
| prioritisedBitRate | infinity |  |  |
| bucketSizeDuration | ms50 |  |  |
| allowedServingCells | Not present |  |  |
| allowedSCS-List | Not present |  |  |
| maxPUSCH-Duration | Not present |  |  |
| configuredGrantType1Allowed | Not present |  |  |
| logicalChannelGroup | 1 |  | HI |
|  | 2 |  | LO |
|  | 0 |  | SRB1, SRB2, SRB3, SRB4 |
| schedulingRequestID | SchedulingRequestId |  |  |
| logicalChannelSR-Mask | false |  |  |
| logicalChannelSR-DelayTimerApplied | false |  |  |
| bitRateQueryProhibitTimer | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HI | Used for DRBs with high logical channel priority |
| LO | Used for DRBs with low logical channel priority |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| SRB4 | Establishment of SRB4 |

#### –	LogicalChannelIdentity

Table 4.6.3-67: LogicalChannelIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LogicalChannelIdentity | 1 |  | SRB1 |
| LogicalChannelIdentity | 2 |  | SRB2 |
| LogicalChannelIdentity | 3 |  | SRB3 |
| LogicalChannelIdentity | 4 |  | SRB4 |
| LogicalChannelIdentity | n+3 |  | DRBn |
| LogicalChannelIdentity | 2m+18 |  | MRBm AND PTP |
| LogicalChannelIdentity | 2m+19 |  | MRBm AND PTM |

| Condition | Explanation |
| --- | --- |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| SRB4 | Establishment of SRB4 |
| DRBn | Establishment of DRBn; n=1..16 |
| MRBm | Establishment of MRBm; m = 1.. (16-n) |
| PTM | RLC entity is used for receving PTM transmission |
| PTP | RLC entity is used for receving PTP transmission |

#### –	LTE-NeighCellsCRS-AssistInfoList

Table 4.6.3-67A: LTE-NeighCellsCRS-AssistInfoList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTE-NeighCellsCRS-AssistInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofCRS-IM-InterfCell-r17)) OF LTE-NeighCellsCRS-AssistInfo-r17 { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MAC-CellGroupConfig

Table 4.6.3-68: MAC-CellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config | Not present |  |  |
| drx-Config CHOICE { |  |  | DRX |
| setup | DRX-Config |  |  |
| } |  |  |  |
| schedulingRequestConfig | SchedulingRequestConfig |  |  |
| bsr-Config | BSR-Config |  |  |
| tag-Config | TAG-Config |  |  |
| phr-Config CHOICE { |  |  |  |
| setup | PHR-Config |  |  |
| } |  |  |  |
| skipUplinkTxDynamic | false |  |  |
| intraCG-Prioritization-r17 | Not present |  |  |
| drx-ConfigSL-r17 | Not present |  |  |
| drx-ConfigExt-v1700 | Not present |  |  |
| schedulingRequestID-BFR-r17 | Not present |  |  |
| schedulingRequestID-BFR2-r17 | Not present |  |  |
| schedulingRequestConfig-v1700 | Not present |  |  |
| tar-Config-r17 | Not present |  |  |
| g-RNTI-ConfigToAddModList-r17 | Not present |  |  |
| g-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17 { | 1 entry |  | MBS_Multicast |
| MBS-RNTI-SpecificConfig-r17[1] SEQUENCE { |  | entry 1 |  |
| mbs-RNTI-SpecificConfigId-r17 | 0 |  |  |
| groupCommon-RNTI-r17 CHOICE { |  |  |  |
| g-RNTI | RNTI-Value |  |  |
| } |  |  |  |
| drx-ConfigPTM-r17 | Not present |  |  |
| drx-ConfigPTM-r17 CHOICE { |  |  | DRX_MBS_Multicast |
| setup | DRX-ConfigPTM |  |  |
| } |  |  |  |
| harq-FeedbackEnablerMulticast-r17 | Not present |  |  |
|  | dci-enabler |  | DCI |
|  | enabled |  | RRC_Enable_HARQFeedback |
| harq-FeedbackOptionMulticast-r17 | Not present |  |  |
|  | ack-nack |  | ACK_NACK |
|  | nack-only |  | NACK_ONLY |
| pdsch-AggregationFactor-r17 | Not present |  |  |
| } |  |  |  |
| g-RNTI-ConfigToReleaseList-r17 | Not present |  |  |
| g-CS-RNTI-ConfigToAddModList-r17 | Not present |  |  |
| g-CS-RNTI-ConfigToReleaseList-r17 | Not present |  |  |
| allowCSI-SRS-Tx-MulticastDRX-Active-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX | This condition applies when DRX is configured |
| MBS_Multicast | Used for MBS Multicast reception |
| DRX_MBS_Multicast | DRX is used for MBS Multicast reception test |
| DCI | HARQ feedback for MBS multicast is indicated by DCI |
| RRC_Enable_HARQFeedback | HARQ feedback for MBS multicast is enabled by RRC |
| ACK_NACK | ACK/NACK based HARQ-ACK feedback |
| NACK_ONLY | NACK-only based HARQ-ACK feedback |

#### –	MeasConfig

Table 4.6.3-69: MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | MeasObjectToAddModList |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | ReportConfigToAddModList |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | MeasIdToAddModList |  |  |
| s-MeasureConfig | Not present |  |  |
| quantityConfig | QuantityConfig |  |  |
| measGapConfig | Not present |  |  |
| measGapSharingConfig | Not present |  |  |
| } |  |  |  |

#### –	MeasGapConfig

Table 4.6.3-70: MeasGapConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 | Not present |  |  |
| gapFR2 CHOICE { |  |  | GAP_FR2 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 159 |  |  |
| mgl | ms3dot5 |  |  |
| mgrp | ms160 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapFR1 | Not present |  |  |
| gapFR1 CHOICE { |  |  | GAP_FR1 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  |  |
|  | 9 |  | SIG AND INTER-FREQ_ODD |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| refServCellIndicator | Not present |  |  |
| refFR2ServCellAsyncCA-r16 | Not present |  |  |
| mgl-r16 | Not present |  |  |
|  | ms10 |  | PRS |
| } |  |  |  |
| gapUE | Not present |  | GAP_FR1 OR GAP_FR2 |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  |  |
|  | 9 |  | SIG AND INTER-FREQ_ODD |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| refServCellIndicator | Not present |  |  |
| refFR2ServCellAsyncCA-r16 | Not present |  |  |
| mgl-r16 | Not present |  |  |
|  | ms10 |  | PRS |
| } |  |  |  |
| } |  |  |  |
| gapToAddModList-r17 | Not present |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 1 entry |  | GAP_ADD |
| GapConfig-r17[1] SEQUENCE { |  |  | entry 1 |
| measGapId-r17 | MeasGapId |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 |  |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
|  | true |  | Pre-config |
| ncsgInd-r17 | Not present |  |  |
|  | true |  | NCSG |
| gapAssociationPRS-r17 | Not present |  |  |
|  | true |  | PRS |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| gapToReleaseList-r17 | Not present |  |  |
| gapToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17 { | 1 entry |  | GAP_RELEASE |
| MeasGapId-r17[1] | 1 |  | entry 1 |
| } |  |  |  |
| posMeasGapPreConfigToAddModList-r17 | Not present |  |  |
| posMeasGapPreConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF PosGapConfig-r17 { | 1 entry |  | PRS_GAP_ADD |
| PosGapConfig-r17[1] SEQUENCE { |  |  | entry 1 |
| measPosPreConfigGapId-r17 | 1 |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 |  |  |
| mgta-r17 | ms0 |  |  |
| gapType-r17 | perUE |  |  |
| } |  |  |  |
| posMeasGapPreConfigToReleaseList-r17 | Not present |  |  |
| posMeasGapPreConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF MeasPosPreConfigGapId-r17 { | 1 entry |  | PRS_GAP_RELEASE |
| MeasPosPreConfigGapId-r17[1] | 1 |  | entry 1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GAP_FR1 | Configuration for FR1 per-FR gaps |
| GAP_FR2 | Configuration for FR2 per-FR gaps |
| INTER-FREQ_ODD | When the SFNoffset of inter frequency neighbour cell is odd number. SFNoffset is defined in TS 38.523-3 [23]Table 7.1.5.2-1 |
| GAP_ADD | Addition or modification of measurement gaps |
| GAP_RELEASE | Release of measurement gaps |
| PRS | PRS measurement |
| PRS_GAP_ADD | Addition or modification of preconfigured measurement gap for positioning |
| PRS_GAP_RELEASE | Addition or modification of preconfigured measurement gap for positioning |

#### –	MeasGapId

Table 4.6.3-70A: MeasGapId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapId-r17 | 1 |  |  |

#### –	MeasGapSharingConfig

Table 4.6.3-71: MeasGapSharingConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapSharingConfig ::= SEQUENCE { |  |  |  |
| gapSharingFR2 | Not present |  |  |
| } |  |  |  |

#### –	MeasId

Table 4.6.3-72: MeasId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasId | 1 |  |  |

#### –	MeasIdleConfig

Table 4.6.3-72A: MeasIdleConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigSIB-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not present |  |  |
| measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16 { | The same number of entries as NR cells configured for measurements | Serving cell not included | EMR_NR_SIB4, EMR_NR_SIB11 |
| MeasIdleCarrierNR-r16[n] SEQUENCE { |  | entry n |  |
| carrierFreq-r16 | ARFCN-ValueEUTRA |  |  |
| ssbSubcarrierSpacing-r16 | SubcarrierSpacing | Table 4.6.3-188 |  |
| frequencyBandList | Not present |  |  |
| measCellListNR-r16 | Not present |  |  |
| reportQuantities-r16 | both |  |  |
| qualityThreshold-r16 | Not present |  |  |
| ssb-MeasConfig-r16 | Not present |  | EMR_NR_SIB4 |
| ssb-MeasConfig-r16 SEQUENCE { |  |  | EMR_NR_SIB11 |
| nrofSS-BlocksToAverage-r16 | 2 |  |  |
| absThreshSS-BlocksConsolidation-r16 SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| smtc-r16 | SSB-MTC | Table 4.6.3-185 |  |
| ssb-ToMeasure-r16 | SSB-ToMeasure | Table 4.6.3-187 |  |
| deriveSSB-IndexFromCell-r16 | false |  |  |
|  | true |  | FR1_TDD, FR2_TDD |
| ss-RSSI-Measurement-r16 | Not present |  |  |
| } |  |  |  |
| beamMeasConfigIdle-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleCarrierListEUTRA-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16 { | The same number of entries as E-UTRA cells configured for measurements | Serving cell not included | EMR_EUTRA_SIB11 |
| MeasIdleCarrierEUTRA-r16[n] SEQUENCE { |  | entry n |  |
| carrierFreqEUTRA-r16 | ARFCN-ValueEUTRA |  |  |
| allowedMeasBandwidth-r16 | EUTRA-AllowedMeasBandwidth | Table 4.6.5-1. |  |
| measCellListEUTRA-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF EUTRA-PhysCellIdRange { | 1 entry |  |  |
| EUTRA-PhysCellIdRange | EUTRA-PhysCellIdRange | entry 1 |  |
| } |  |  |  |
| reportQuatitiesEUTRA-r16 | both |  |  |
| qualityThresholdEUTRA-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EMR_NR_SIB4 | FFS |
| EMR_NR_SIB11 | FFS |

#### –	MeasIdToAddModList

Table 4.6.3-73: MeasIdToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | MeasId |  |  |
| measObjectId | MeasObjectId |  |  |
| reportConfigId | ReportConfigId |  |  |
| } |  |  |  |

#### –	MeasObjectCLI

Table 4.6.3-73A: MeasObjectCLI

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCLI-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasObjectEUTRA

Table 4.6.3-74: MeasObjectEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA ::= SEQUENCE { |  |  |  |
| carrierFreq | Downlink EARFCN for Freq |  |  |
| allowedmeasBandwidth | Set according to TS 36.508 [2] Table 4.4.3.4-1 for E-UTRA cell | row ‘measurement Bandwidth’ |  |
| cellsToRemoveListEUTRAN | Not present |  |  |
| cellsToAddModListEUTRAN | Not present |  |  |
| excludedCellsToRemoveListEUTRAN | Not present |  |  |
| excludedCellsToAddModListEUTRAN | Not present |  |  |
| eutra-PresenceAntennaPort1 | false |  |  |
|  | true | at least two cell-specific antenna ports are used in all neighbouring cells | All neighCells with port1 |
| eutra-Q-OffsetRange | Not present |  |  |
| widebandRSRQ-Meas | false |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |

#### –	MeasObjectId

Table 4.6.3-75: MeasObjectId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectId | 1 |  |  |

#### –	MeasObjectNR

Table 4.6.3-76: MeasObjectNR(Thres)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR with condition DL_SSB |  |  |
| ssbSubcarrierSpacing | SubcarrierSpacing |  |  |
| smtc1 | SSB-MTC |  |  |
| smtc2 | Not present |  |  |
| refFreqCSI-RS | Not present |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure CHOICE { |  |  |  |
| setup | SSB-ToMeasure |  |  |
| } |  |  |  |
| deriveSSB-IndexFromCell | false |  | FDD |
|  | true |  | TDD |
| ssb-PositionQCL-Common-r16 | Not present |  |  |
|  | SSB-PositionQCL-Relation |  | SharedSpectrum |
| ssb-PositionQCL-CellsToAddModList-r16 | Not present |  |  |
| ssb-PositionQCL-CellsToRemoveList-r16 | Not present |  |  |
| ss-RSSI-Measurement | Not present |  |  |
| } |  |  |  |
| csi-rs-ResourceConfigMobility | Not present |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | Thres | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| absThreshCSI-RS-Consolidation | Not present |  |  |
| nrofSS-BlocksToAverage | 2 |  |  |
| nrofCSI-RS-ResourcesToAverage | Not present |  |  |
| quantityConfigIndex | 1 |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB0 |  |  |
| rsrqOffsetSSB | dB0 |  |  |
| sinrOffsetSSB | dB0 |  |  |
| rsrpOffsetCSI-RS | dB0 |  |  |
| rsrqOffsetCSI-RS | dB0 |  |  |
| sinrOffsetCSI-RS | dB0 |  |  |
| } |  |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList | Not present |  |  |
| excludedCellsToRemoveList | Not present |  |  |
| excludedCellsToAddModList | Not present |  |  |
| allowedCellsToRemoveList | Not present |  |  |
| allowedCellsToAddModList | Not present |  |  |
| freqBandIndicatorNR | FreqBandIndicatorNR |  |  |
| rmtc-Config-r16 | Not present |  |  |
|  | RMTC-Config |  | SharedSpectrum |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SharedSpectrum | Operation with shared spectrum channel access |

#### –	MeasObjectNR-SL

Table 4.6.3-76A: MeasObjectNR-SL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-SL-r16 ::= SEQUENCE { |  |  |  |
| tx-PoolMeasToRemoveList-r16 | Not present |  |  |
| tx-PoolMeasToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-PoolToMeasureNR-r16)) OF SL-ResourcePoolID-r16{ | 1 entry |  |  |
| SL-ResourcePoolID-r16[1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasObjectRxTxDiff

Table 4.6.3-76B: MeasObjectRxTxDiff

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectRxTxDiff-r17 ::= SEQUENCE { |  |  |  |
| dl-Ref-r17 CHOICE { |  |  |  |
| prs-Ref-r17 | NULL |  | PRS_REF |
| csi-RS-Ref-r17 | NULL |  | CSI-RS_REF |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PRS_REF | Test cases using PRS reference signal to measure Rx-Tx time difference |
| CSI-RS_REF | Test cases using CSI-RS for tracking to measure Rx-Tx time difference |

#### –	MeasObjectToAddModList

Table 4.6.3-77: MeasObjectToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | MeasObjectId |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  | InterRAT |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| InterRAT | Configuration with at least one NR PCell and one or more E-UTRA neighbour cell(s) |

#### –	MeasObjectUTRA-FDD

Table 4.6.3-77A: MeasObjectUTRA-FDD

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA-FDD-r16 ::= SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN-ValueUTRA-FDD-r16 |  |  |
| utra-FDD-Q-OffsetRange-r16 | UTRA-FDD-Q-OffsetRange-r16 |  |  |
| cellsToRemoveList-r16 | Not present |  |  |
| cellsToAddModList-r16 | Not present |  |  |
| } |  |  |  |

#### –	MeasResultCellListSFTD-NR

Table 4.6.3-78: MeasResultCellListSFTD-NR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultCellListSFTD-NR ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultCellListSFTD-EUTRA

Table 4.6.3-78A: MeasResultCellListSFTD-EUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultCellListSFTD-EUTRA ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultForRSSI

Table 4.6.3-78AA: MeasResultForRSSI

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultForRSSI ::= SEQUENCE { |  |  |  |
| rssi-Result-r16 | FFS |  |  |
| channelOccupancy-r16 | FFS |  |  |
| } |  |  |  |

#### –	MeasResults

Table 4.6.3-79: MeasResults

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | MeasId |  |  |
| measResultServingMOList | Not checked |  | SFTD_NEIGHBOUR or SFTD_PSCELL |
| measResultServingMOList SEQUENCE (SIZE (1.. maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | Not checked |  |  |
| rsrq | Not checked |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| measResultBestNeighCell | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells | Not present |  | A1, A2, SFTD_NEIGHBOUR or SFTD_PSCELL, SFTD_EUTRA |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR | Set according to specific message content |  | A3, A4, A5, A6 |
| measResultListEUTRA | Set according to specific message content |  | B1, B2 |
| measResultListUTRA-FDD-r16 | Set according to specific message content |  | B1_UTRA, B2_UTRA |
| } |  |  |  |
| measResultServFreqListEUTRA-SCG | Not present |  |  |
| measResultServFreqListNR-SCG | Not checked |  |  |
| measResultSFTD-EUTRA | Not present |  |  |
| measResultSFTD-EUTRA SEQUENCE { |  |  | SFTD_EUTRA |
| eutra-PhysCellId | PhysCellId of E-UTRA Cell 1 |  |  |
| sfn-OffsetResult | (0..1023) |  |  |
| frameBoundaryOffsetResult | (-30720..30719) |  |  |
| rsrp-Result | Not checked |  |  |
| } |  |  |  |
| measResultSFTD-NR | Not present |  |  |
| measResultSFTD-NR SEQUENCE { |  |  | SFTD_PSCELL |
| physCellId | PhysCellId of PSCell |  |  |
| sfn-OffsetResult | Not checked |  |  |
| frameBoundaryOffsetResult | Not checked |  |  |
| rsrp-Result | Not checked |  |  |
| } |  |  |  |
| measResultCellListSFTD-NR | Not present |  |  |
| measResultCellListSFTD-NR SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultCellSFTD-NR { | 1 entry |  | SFTD_NEIGHBOUR |
| MeasResultCellSFTD-NR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | PhysCellId |  |  |
| sfn-OffsetResult | Not checked |  |  |
| frameBoundaryOffsetResult | Not checked |  |  |
| rsrp-Result | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForRSSI-r16 | Not present |  |  |
|  | MeasResultForRSSI |  | SharedSpectrum |
| locationInfo-r16 | Not present |  |  |
| ul-PDCP-DelayValueResultList-r16 | Not present |  |  |
| measResultsSL-r16 | Not present |  |  |
| measResultCLI-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | If event trigger Id in corresponding Measurement Configuration was Event A1 |
| A2 | If event trigger Id in corresponding Measurement Configuration was Event A2 |
| A3 | If event trigger Id in corresponding Measurement Configuration was Event A3 |
| A4 | If event trigger Id in corresponding Measurement Configuration was Event A4 |
| A5 | If event trigger Id in corresponding Measurement Configuration was Event A5 |
| A6 | If event trigger Id in corresponding Measurement Configuration was Event A6 |
| B1 | If event trigger Id in corresponding Measurement Configuration was Event B1 |
| B2 | If event trigger Id in corresponding Measurement Configuration was Event B2 |
| B1_UTRA | If event trigger Id in corresponding Measurement Configuration was Event B1-UTRA-FDD |
| B2_UTRA | If event trigger Id in corresponding Measurement Configuration was Event B2-UTRA-FDD |
| SFTD_NEIGHBOUR | Measurement reporting triggered by SFTD measurement on NR neighbour |
| SFTD_PSCELL | Measurement reporting triggered by SFTD measurement on NR PSCell |
| SharedSpectrum | Operation with shared spectrum channel access |
| SFTD-EUTRA | Measurement reporting triggered by SFTD measurement on E-UTRA PSCell |

#### –	MeasResult2EUTRA

Table 4.6.3-79A: MeasResult2EUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResult2EUTRA ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResult2NR

Table 4.6.3-79B: MeasResult2NR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResult2NR ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultIdleEUTRA

Table 4.6.3-79C: MeasResultIdleEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultIdleEUTRA-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultIdleNR

Table 4.6.3-79D: MeasResultIdleNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultIdleNR-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultRxTxTimeDiff

Table 4.6.3-79E: MeasResultRxTxTimeDiff

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultRxTxTimeDiff-r17 ::= SEQUENCE { |  |  |  |
| rxTxTimeDiff-ue-r17 SEQUENCE { |  |  |  |
| result-k5-r17 | INTEGER (0..61565) | Integer value for result-k5 measurements |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasResultSCG-Failure

Table 4.6.3-80: MeasResultSCG-Failure

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultSCG-Failure ::= SEQUENCE { |  | measResultPerMOList for each MeasOjectNR for which a measId is configured (by the NR RRCConfiguration message) and measurement results are available include an entry |  |
| measResultPerMOList SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR { | n entries of MeasResult2NR | MOList [1] n denotes the number of non-serving frequencies being measured |  |
| MeasResult2NR[1] SEQUENCE { |  | entry 1 |  |
| ssbFrequency | ARFCN-ValueNR with condition DL_SSB | the ARFCN if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb |  |
| refFreqCSI-RS | INTEGER (0..3279165) | the ARFCN if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs |  |
| measResultServingCell SEQUENCE { |  | if a serving cell is associated with the MeasObjectNR |  |
| physCellId | INTEGER (0..1007) | the physCellId configured for this serving cell |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| resultsCSI-RS-Cell SEQUENCE { |  |  |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |
| resultsSSB-Indexes SEQUENCE (SIZE (1..maxNrofSSBs)) OF ResultsPerSSB-Index { | n entires of ResultsPerSSB-Index | ResultsPerSSB-IndexList |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index | SSB-Index | an SS-Block within an SS-Burst |  |
| ssb-Results SEQUENCE { |  | MeasQuantityResults |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
|  |  | ResultsPerSSB-Index entry [x] if any |  |
| } |  |  |  |
| resultsCSI-RS-Indexes SEQUENCE (SIZE (1..maxNrofCSI-RS)) OF ResultsPerCSI-RS-Index { | n entires of ResultsPerCSI-RS-Index | ResultsPerCSI-RS-IndexList |  |
| ResultsPerCSI-RS-Index[1] SEQUENCE { |  | entry 1 |  |
| csi-RS-Index | INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1) | CSI-RS resource index associated to the measurement information to be reported |  |
| csi-RS-Results SEQUENCE { |  | MeasQuantityResults |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
|  |  | ResultsPerCSI-RS-Index entry [x] if any |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR { | n entires of MeasResultNR | include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | INTEGER (0..1007) | the physCellId configured for the measured cell |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| resultsCSI-RS-Cell SEQUENCE { |  |  |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
|  |  | MeasResultNR entry [x] if any |  |
| } |  |  |  |
|  |  | MeasResult2NR entry [x] if any |  |
| } |  |  |  |
|  |  | MOList [x] if any |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasResultsSL

Table 4.6.3-80A: MeasResultsSL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultsSL-r16 ::= SEQUENCE { |  |  |  |
| measResultsListSL-r16 CHOICE { |  |  |  |
| measResultNR-SL-r16 SEQUENCE { |  |  |  |
| measResultListCBR-NR-r16 SEQUENCE (SIZE (1.. maxNrofSL-PoolToMeasureNR-r16)) OF MeasResultCBR-NR-r16 { | 1 entry |  |  |
| MeasResultCBR-NR-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-poolReportIdentity-r16 | 1 |  |  |
| sl-CBR-ResultsNR-r16 | (0..100) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasRSSI-ReportConfig

Table 4.6.3-80AA: MeasRSSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasRSSI-ReportConfig-r16 ::= SEQUENCE { |  |  |  |
| channelOccupancyThreshold-r16 | FFS |  |  |
| } |  |  |  |

#### –	MeasTriggerQuantityEUTRA

Table 4.6.3-80B: MeasTriggerQuantityEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasTriggerQuantityEUTRA-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MobilityStateParameters

Table 4.6.3-81: MobilityStateParameters

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityStateParameters ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MRB-Identity

Table 4.6.3-81AA: MRB-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MRB-Identity-r17 | m |  | MRBm |

| Condition | Explanation |
| --- | --- |
| MRBm | Establishment of MRBm |

#### –	MsgA-ConfigCommon

Table 4.6.3-81A: MsgA-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-ConfigCommonL-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA |  |  |
| msgA-PUSCH-Config-r16 | MsgA-PUSCH-Config |  |  |
| } |  |  |  |

#### –	MsgA-PUSCH-Config

Table 4.6.3-81B: MsgA-PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| msgA-PUSCH-ResourceGroupA-r16 SEQUENCE { |  |  |  |
| msgA-MCS-r16 | 1 |  |  |
| nrofSlotsMsgA-PUSCH-r16 | 1 |  |  |
| nrofMsgA-PO-PerSlot-r16 | one |  |  |
| msgA-PUSCH-TimeDomainOffset-r16 | 4 |  |  |
| msgA-PUSCH-TimeDomainAllocation-r16 | Not present |  |  |
| startSymbolAndLengthMsgA-PO-r16 | 27 | S=0, L=14 |  |
| mappingTypeMsgA-PUSCH-r16 | typeA |  |  |
| guardPeriodMsgA-PUSCH-r16 | 0 |  |  |
| guardBandMsgA-PUSCH-r16 | 0 |  |  |
| frequencyStartMsgA-PUSCH-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 1 |  |  |
| nrofMsgA-PO-FDM-r16 | one |  |  |
| msgA-IntraSlotFrequencyHopping-r16 | Not present |  |  |
| msgA-HoppingBits-r16 | Not present |  |  |
| msgA-DMRS-Config-r16 SEQUENCE { |  |  |  |
| msgA-DMRS-AdditionalPosition-r16 | pos0 |  |  |
| msgA-MaxLength-r16 | len2 |  |  |
| msgA-PUSCH-DMRS-CDM-Group-r16 | 0 |  |  |
| msgA-PUSCH-NrofPorts-r16 | 0 |  |  |
| msgA-ScramblingID0-r16 | Not present |  |  |
| msgA-ScramblingID1-r16 | Not present |  |  |
| } |  |  |  |
| nrofDMRS-Sequences-r16 | 1 |  |  |
| msgA-Alpha-r16 | alpha0 |  |  |
| interlaceIndexFirstPO-MsgA-PUSCH-r16 | 1 |  |  |
| nrofInterlacesPerMsgA-PO-r16 | 1 |  |  |
| } |  |  |  |
| msgA-PUSCH-ResourceGroupB-r16 | Not present |  |  |
| msgA-TransformPrecoder-r16 | enabled |  |  |
| msgA-DataScramblingIndex-r16 | Not present |  |  |
| msgA-DeltaPreamble-r16 | 1 |  |  |
| } |  |  |  |

#### –	MultiFrequencyBandListNR

Table 4.6.3-82: MultiFrequencyBandListNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MultiFrequencyBandListNR ::= SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR { | 1 entry |  |  |
| FreqBandIndicatorNR[1] | FreqBandIndicatorNR | entry 1 |  |
| } |  |  |  |

#### –	MultiFrequencyBandListNR-SIB

Table 4.6.3-82A: MultiFrequencyBandListNR-SIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MultiFrequencyBandListNR-SIB ::= SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo { | 1 entry |  |  |
| NR-MultiBandInfo[1] SEQUENCE { |  | entry 1 |  |
| freqBandIndicatorNR | FreqBandIndicatorNR |  |  |
| nr-NS-PmaxList | NR-NS-PmaxList |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MUSIM-GapConfig

Table 4.6.3-82BA: MUSIM-GapConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MUSIM-GapId

Table 4.6.3-82BB: MUSIM-GapId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapId-r17 | FFS |  |  |

#### –	MUSIM-GapInfo

Table 4.6.3-82BC: MUSIM-GapInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapInfo-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NeedForGapsConfigNR

Table 4.6.3-82B: NeedForGapsConfigNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForGapsConfigNR-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NeedForGapsInfoNR

Table 4.6.3-82C: NeedForGapsInfoNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForGapsInfoNR-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NeedForGapNCSG-ConfigEUTRA

Table 4.6.3-82D: NeedForGapNCSG-ConfigEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForGapNCSG-ConfigEUTRA-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NeedForGapNCSG-ConfigNR

Table 4.6.3-82E: NeedForGapNCSG-ConfigNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForGapNCSG-ConfigNR-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NeedForGapNCSG-InfoEUTRA

Table 4.6.3-82F: NeedForGapNCSG-InfoEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForGapNCSG-InfoEUTRA-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NeedForGapNCSG-InfoNR

Table 4.6.3-82G: NeedForGapNCSG-InfoNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForGapNCSG-InfoNR-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NextHopChainingCount

Table 4.6.3-83: NextHopChainingCount

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NextHopChainingCount | 0 |  |  |

#### –	NG-5G-S-TMSI

Table 4.6.3-84: NG-5G-S-TMSI

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NG-5G-S-TMSI | Set to the value of the NG-5G-S-TMSI of the UE | BIT STRING (SIZE(40)) |  |

#### –	NonCellDefiningSSB

Table 4.6.3-84AAA: NonCellDefiningSSB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NonCellDefiningSSB-r17 ::= SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | Set to value of absoluteFrequencySSB for NCD-SSB in Table 6.2.3.1-8/9 |  |  |
| ssb-Periodicity-r17 | Not present | The periodicity of this NCD-SSB is same as SSB periodicity of the CD-SSB |  |
| ssb-TimeOffset-r17 | Not present | The time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero |  |
| } |  |  |  |

#### –	NPN-Identity

Table 4.6.3-84AA: NPN-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPN-Identity-r16 ::= CHOICE { |  |  |  |
| pni-npn-r16 SEQUENCE { |  |  | CAG |
| plmn-Identity-r16 | PLMN-Identity |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| cag-Identity-r16[1] | 1 | cag-Identity is coded as a 32 bit BITSTRING |  |
| manualCAGselectionAllowed-r16 | true |  |  |
| } |  |  |  |
| snpn-r16 SEQUENCE { |  |  | SNPN |
| plmn-Identity-r16 | PLMN-Identity |  |  |
| nid-List-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16{ | 1 entry |  |  |
| NID-r16[1] | See table 4.4.2-4 | NID is coded as a BITSTRING based on TS 38.508-1 Table 4.4.2-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CAG | PNI-NPN |
| SNPN | Standalone NPN cell |

#### –	NPN-IdentityInfoList

Table 4.6.3-84AB: NPN-IdentityInfoList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPN-IdentityInfoList-r16 ::= SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  |  |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] | NPN-Identity with condition CAG |  | CAG |
|  | NPN-Identity with condition SNPN |  | SNPN |
| } |  |  |  |
| trackingAreaCode-r16 | TrackingAreaCode |  |  |
| ranac-r16 | RAN-AreaCode |  |  |
| cellIdentity-r16 | CellIdentity |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| iab-Support-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CAG | PNI-NPN |
| SNPN | Standalone NPN cell |

#### –	NR-DL-PRS-PDC-Info

Table 4.6.3-84AC: NR-DL-PRS-PDC-Info

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-PDC-Info-r17 ::= SEQUENCE { |  |  |  |
| NR-DL-PRS-PDC-ResourceSet-r17 SEQUENCE { |  |  |  |
| periodicityAndOffset-r17 CHOICE { |  |  |  |
| scs15-r17 CHOICE { |  | entry 1 | SCS15 |
| n160-r17 | 10 |  |  |
| } |  |  |  |
| scs30-r17 CHOICE { |  | entry 2 | SCS30 |
| n160-r17 | 10 |  |  |
| } |  |  |  |
| scs120-r17 CHOICE { |  | entry 3 | SCS120 |
| n160-r17 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| numSymbols-r17 | n4 |  |  |
| dl-PRS-ResourceBandwidth-r17 | 8 |  | SCS15 |
|  | 22 |  | SCS30 |
|  | 11 |  | SCS120 |
| dl-PRS-StartPRB-r17 | 0 |  |  |
| resourceList-r17 SEQUENCE (SIZE (1..maxNrofPRS-ResourcesPerSet-r17)) OF NR-DL-PRS-Resource-r17 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r17[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r17 | 0 |  |  |
| dl-PRS-SequenceID-r17 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r17 CHOICE { |  |  |  |
| n4-r17 | 0 |  | SCS15 OR SCS30 |
| n2-r17 | 0 |  | SCS120 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r17 | 4 |  |  |
| dl-PRS-ResourceSymbolOffset-r17 | 0 |  |  |
| dl-PRS-QCL-Info-r17 CHOICE { |  |  |  |
| ssb-r17 SEQUENCE { |  |  | SCS120 |
| ssb-Index-r17 | 0 | SSB#0 |  |
| rs-Type-r17 | typeD |  |  |
| } |  |  |  |
| dl-PRS-r17 SEQUENCE { |  |  | SCS15 OR SCS30 |
| qcl-DL-PRS-ResourceID-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| repFactorAndTimeGap-r17 SEQUENCE { |  |  |  |
| repetitionFactor-r17 | n2 |  |  |
| timeGap-r17 | s1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	NR-NS-PmaxList

Table 4.6.3-84A: NR-NS-PmaxList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-NS-PmaxList ::= SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-NS-PmaxValue { | 1 entry |  |  |
| NR-NS-PmaxValue[1] SEQUENCE { |  | entry 1 |  |
| additionalPmax | Not present |  |  |
| additionalSpectrumEmission | AdditionalSpectrumEmission |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	NSAG-ID

Table 4.6.3-84BA: NSAG-ID

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NSAG-ID-r17 | Set according to specific message content |  |  |

#### –	NSAG-IdentityInfo

Table 4.6.3-84B: NSAG-IdentityInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NSAG-IdentityInfo-r17 ::= SEQUENCE { |  |  |  |
| nsag-ID-r17 | NSAG-ID |  |  |
| trackingAreaCode-r17 | Not present |  |  |
| } |  |  |  |

#### –	NTN-Config

Table 4.6.3-84C: NTN-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  | GSO |
| epochTime-r17 SEQUENCE { |  |  | NGSO |
| sfn-r17 | FFS |  |  |
| subFrameNR-r17 | FFS |  |  |
| } |  |  |  |
| ntn-UlSyncValidityDuration-r17 | s240 |  |  |
| cellSpecificKoffset-r17 | 550 |  |  |
| kmac-r17 | Not present |  |  |
| ta-Info-r17 | FFS |  |  |
| ntn-PolarizationDL-r17 | linear |  |  |
| ntn-PolarizationUL-r17 | linear |  |  |
| ephemerisInfo-r17 | EphemerisInfo with condition GSO |  | GSO |
|  | EphemerisInfo with condition NGSO |  | NGSO |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

#### –	NZP-CSI-RS-Resource

Table 4.6.3-85: NZP-CSI-RS-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId |  |  |
| resourceMapping | CSI-RS-ResourceMapping |  |  |
| powerControlOffset | -3 |  |  |
| powerControlOffsetSS | Not present |  |  |
| scramblingID | ScramblingId |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId |  |  |
| } |  |  |  |

#### –	NZP-CSI-RS-ResourceId

Table 4.6.3-86: NZP-CSI-RS-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | 0 |  |  |

#### –	NZP-CSI-RS-ResourceSet

Table 4.6.3-87: NZP-CSI-RS-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId | entry 1 |  |
| } |  |  |  |
| repetition | off |  |  |
|  | Not present |  | TRS |
| aperiodicTriggeringOffset | Not present |  |  |
| trs-Info | Not present |  |  |
|  | true |  | TRS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal |

#### –	NZP-CSI-RS-ResourceSetId

Table 4.6.3-88: NZP-CSI-RS-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | 0 |  |  |

#### –	P-Max

NOTE:	Exceptions in clause 5.4.1 shall be appied for RF test cases.

Table 4.6.3-89: P-Max

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | NR OR (EN-DC AND pc_dynamicPowerSharing) |
|  | 20 |  | EN-DC AND (NOT pc_dynamicPowerSharing) |

| Condition | Explanation |
| --- | --- |
| NR | NR connected to 5GC |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |

#### –	PathlossReferenceRS

Table 4.6.3-89AA: PathlossReferenceRS

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PathlossReferenceRS-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PathlossReferenceRS-Id

Table 4.6.3-89AB: PathlossReferenceRS-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PathlossReferenceRS-Id-r17 | FFS |  |  |

#### –	PCI-ARFCN-EUTRA

Table 4.6.3-89A: PCI-ARFCN-EUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-ARFCN-EUTRA-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PCI-ARFCN-NR

Table 4.6.3-89B: PCI-ARFCN-NR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-ARFCN-NR-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PCI-List

Table 4.6.3-90: PCI-List

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-List ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PCI-Range

Table 4.6.3-91: PCI-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-Range ::= SEQUENCE { |  |  |  |
| start | PhysCellId |  |  |
| range | FFS |  |  |
| } |  |  |  |

#### –	PCI-RangeElement

Table 4.6.3-92: PCI-RangeElement

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-RangeElement ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PCI-RangeIndex

Table 4.6.3-93: PCI-RangeIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-RangeIndex | 0 |  |  |

#### –	PCI-RangeIndexList

Table 4.6.3-94: PCI-RangeIndexList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-RangeIndexList ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PDCCH-Config

Table 4.6.3-95: PDCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  | EN-DC, MSS |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | entry 1 |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace with condition USS | entry 1 |  |
|  | SearchSpace with condition MSS | entry 1 | MSS |
|  | SearchSpace with condition SLSS | entry 1 | SLSS |
|  | SearchSpace with condition DCI_2_6 | entry 1 | DCI_2_6 |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| controlResourceSetToAddModListSizeExt-v1610 | Not present |  |  |
| controlResourceSetToReleaseListSizeExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-r16 { | 1 entry |  | DCI_2_6 |
| SearchSpaceExt-r16[1] SEQUENCE{ |  | entry 1 |  |
| controlResourceSetId-r16 | ControlResourceSetId |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common-r16 SEQUENCE { |  |  |  |
| dci-Format2-4-r16 | Not present |  |  |
| dci-Format2-5-r16 | Not present |  |  |
| dci-Format2-6-r16 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r16 | Not present |  |  |
| freqMonitorLocations-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkCancellation-r16 | Not present |  |  |
| monitoringCapabilityConfig-r16 | Not present |  |  |
| searchSpaceSwitchConfig-r16 | Not present |  |  |
| searchSpacesToAddModListExt-v1700 | Not present |  |  |
| searchSpacesToAddModListExt-v1700 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700 { | 1 entry |  | MSS |
| SearchSpaceExt-v1700[1] SEQUENCE { |  | entry 1 |  |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 SEQUENCE { |  |  |  |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 | Not present |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format2-7-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| monitoringCapabilityConfig-v1710 | Not present |  |  |
| searchSpaceSwitchConfig-r17 | Not present |  |  |
| pdcch-SkippingDurationList-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MSS | SearchSpace for MBS Multicast receptioin |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |

#### –	PDCCH-ConfigCommon

Table 4.6.3-96: PDCCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero | Initial BWP |  |
|  | Not present |  | SCell_add InitialBWP_SIB, BWP-Id1 |
| commonControlResourceSet | ControlResourceSet |  | EN-DC |
|  | Not present |  |  |
| searchSpaceZero | SearchSpaceZero | Initial BWP |  |
|  | Not present |  | SCell_add InitialBWP_SIB, BWP-Id1 |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
|  | 3 entries |  | PEI |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| SearchSpace[2] | SearchSpace with condition SISS | entry 2 |  |
| SearchSpace[3] | SearchSpace with condition PEI | entry 3 | PEI |
| } |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  | EN-DC |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| commonSearchSpaceList | Not present |  | SCell_add |
| searchSpaceSIB1 | 0 |  |  |
|  | Not present |  | EN-DC, SCell_add |
| searchSpaceOtherSystemInformation | SearchSpaceId with condition SISS |  |  |
|  | Not present |  | EN-DC, SCell_add |
| pagingSearchSpace | 0 |  |  |
|  | Not present |  | EN-DC, SCell_add |
| ra-SearchSpace | SearchSpaceId with condition CSS |  |  |
|  | Not present |  | SCell_add |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| commonSearchSpaceListExt-r16 | Not present |  |  |
| sdt-SearchSpace-r17 | Not present |  |  |
| sdt-SearchSpace-r17 CHOICE { |  |  | SDT |
| existingSearchSpace | SearchSpaceId with condition CSS |  |  |
| } |  |  |  |
| searchSpaceMCCH-r17 | Not present |  |  |
| searchSpaceMCCH-r17 | SearchSpaceId with condition CSS |  | MBS_Broadcast |
| searchSpaceMTCH-r17 | Not present |  |  |
| commonSearchSpaceListExt2-r17 | Not present |  |  |
| commonSearchSpaceListExt2-r17 SEQUENCE(SIZE (1..4)) OF SearchSpaceExt-v1700 { | 2 entries |  | MBS_Broadcast |
|  | 3 entries |  | PEI |
| SearchSpaceExt-v1700[1] SEQUENCE { |  | entry 1 | MBS_Broadcast, PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 | Not present |  | PEI |
| searchSpaceType-r17 SEQUENCE { |  |  | MBS_Broadcast |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 | Not present |  |  |
| dci-Format2-7-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| SearchSpaceExt-v1700[2] SEQUENCE { |  | entry 2 | MBS_Broadcast, PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 | Not present |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| SearchSpaceExt-v1700[3] SEQUENCE { |  | entry 3 | PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 SEQUENCE { |  |  |  |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 | Not present |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 | Not present |  |  |
| dci-Format2-7-r17 SEQUENCE { |  |  |  |
| nrofCandidates-PEI-r17 SEQUENCE { |  |  |  |
| aggregationLevel4-r17 | n2 |  |  |
| aggregationLevel8-r17 | Not present |  |  |
| aggregationLevel16-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| firstPDCCH-MonitoringOccasionOfPO-v1710 | Not present |  |  |
| pei-ConfigBWP-r17 | Not present |  |  |
| pei-ConfigBWP-r17 SEQUENCE { |  |  | PEI |
| pei-SearchSpace-r17 | SearchSpaceId with condition PEI |  |  |
| firstPDCCH-MonitoringOccasionOfPEI-O-r17 CHOICE { |  |  |  |
| sCS15KHZoneT[1] | 0 |  |  |
| sCS15KHZoneT[2] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| followUnifiedTCIstate-v1720 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| SCell_add | Add SCell |
| InitialBWP_SIB | Configured via DownlinkConfigCommonSIB |
| BWP-Id1 | Additional BWP 1 |
| SDT | For SDT test cases |
| PEI | Paging Early Indication is configured in the cell. |
| MBS_Broadcast | NR Cell provide MBS Broadcast service |

#### –	PDCCH-ConfigSIB1

Table 4.6.3-97: PDCCH-ConfigSIB1

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigSIB1 ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero |  |  |
|  | ControlResourceSetZero with condition NCD-SSB |  | NCD-SSB |
| searchSpaceZero | SearchSpaceZero |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NCD-SSB | MIB for NCD-SSB |

#### –	PDCCH-ServingCellConfig

Table 4.6.3-98: PDCCH-ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| slotFormatIndicator | Not present |  |  |
| } |  |  |  |

#### –	PDCP-Config

Table 4.6.3-99: PDCP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
|  | len12bits |  | pc_supportOfRedCap_r17 |
|  | Not present |  | MRBm |
| pdcp-SN-Size-DL | len18bits |  |  |
|  | len12bits |  | pc_supportOfRedCap_r17 |
| headerCompression CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| integrityProtection | Not present |  |  |
| statusReportRequired | true | AM is default |  |
|  | Not present |  | UM, UM_MRB |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| drb | Not present |  | SRB, Split_SRB |
| moreThanOneRLC | Not present |  |  |
| moreThanOneRLC SEQUENCE { |  |  | Split, Split_SRB, NR_split |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | CellGroupId |  |  |
|  | 0 | MCG path | Split_SRB, NR_split |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
|  | Not present |  | Split_SRB |
| pdcp-Duplication | false |  |  |
|  | Not present | one UL path | Split_SRB |
| } |  |  |  |
| t-Reordering | Not present |  |  |
|  | 80ms |  | UM, UM_MRB |
| survivalTimeStateSupport-r17 | Not present |  |  |
| uplinkDataCompression-r17 | Not present |  |  |
| uplinkDataCompression-r17 CHOICE { |  |  | UDC |
| setup CHOICE { |  |  |  |
| newSetup CHOICE { |  |  |  |
| bufferSize-r17 | kbyte2 |  |  |
| dictionary-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| discardTimerExt2-r17 | Not present |  |  |
| initialRX-DELIV-r17 | Not present |  |  |
|  | 0 |  | MRB_Initialization |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Split | More than one RLC |
| SRB | SRB |
| UM | RLC UM DRB |
| Split_SRB | SRB with more than one RLC |
| NR_split | MCG and split for NR-DC. |
| UDC | RLC AM DRB with uplinkDataCompression |
| MRB_Initialization | Multicast MRB setup or PDCP re-establishment for multicast MRB |
| UM_MRB | UM multicast MRB |
| MRBm | Establishment of MRBm |

#### –	PDSCH-Config

Table 4.6.3-100: PDSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeB | Not present |  |  |
| tci-StatesToAddModList | Not present |  | MBS_Multicast |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1 entry |  |  |
| TCI-State[1] | TCI-State | entry 1 |  |
| } |  |  |  |
| tci-StatesToReleaseList | Not present |  |  |
| vrb-ToPRB-Interleaver | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
|  | resourceAllocationType0 |  | Used_for_Type0 |
| pdsch-TimeDomainAllocationList | Not present |  |  |
| pdsch-AggregationFactor | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| rateMatchPatternGroup1 | Not present |  |  |
| rateMatchPatternGroup2 | Not present |  |  |
| rbg-Size | config1 |  |  |
| mcs-Table | Not present | qam64 per default |  |
| maxNrofCodeWordsScheduledByDCI | Not present |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | wideband |  |  |
| } |  |  |  |
| } |  |  |  |
| zp-CSI-RS-ResourceToAddModList | Not present |  |  |
| zp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| p-ZP-CSI-RS-ResourceSet | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Used_for_Type0 | Used for RF performance test cases |
| MBS_Multicast | Used for MBS Multicast reception |

#### –	PDSCH-ConfigCommon

Table 4.6.3-101: PDSCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |

#### –	PDSCH-ServingCellConfig

Table 4.6.3-102: PDSCH-ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| xOverhead | Not present |  |  |
| nrofHARQ-ProcessesForPDSCH | n4 |  | RF AND FDD |
|  | n16 |  | RRM AND FDD |
|  | Not present | Default value: 8 HARQ processes |  |
| pucch-Cell | Not present |  |  |
| } |  |  |  |

#### –	PDSCH-TimeDomainResourceAllocationList

Table 4.6.3-103: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  | FR1 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocationList SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 1 entry |  | FR2 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| } |  |  |  |

#### –	PHR-Config

Table 4.6.3-104: PHR-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PHR-Config ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | sf10 |  |  |
| phr-ProhibitTimer | sf0 |  |  |
| phr-Tx-PowerFactorChange | dB1 |  |  |
| multiplePHR | false |  |  |
|  | true |  | MR-DC OR NR-CA |
| dummy | false |  |  |
| Phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MR-DC | EN-DC, NGEN-DC, NE-DC or NR-DC. |
| NR-CA | UL CA for NR |

#### –	PhysCellId

Table 4.6.3-105: PhysCellId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysCellId | Set according to table 4.4.2-2 for the NR Cell. |  |  |

#### –	PhysicalCellGroupConfig

Table 4.6.3-106: PhysicalCellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| harq-ACK-SpatialBundlingPUCCH | Not present |  |  |
| harq-ACK-SpatialBundlingPUSCH | Not present |  |  |
| p-NR-FR1 | P-Max |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  |  |
| tpc-SRS-RNTI | Not present |  |  |
| tpc-PUCCH-RNTI | Not present |  |  |
| tpc-PUSCH-RNTI | Not present |  |  |
| sp-CSI-RNTI | Not present |  |  |
| cs-RNTI | Not present |  |  |
| dcp-Config-r16 | Not present |  |  |
| dcp-Config-r16 CHOICE { |  |  | DCP |
| setup SEQUENCE { |  |  |  |
| ps-RNTI-r16 | RNTI-Value |  |  |
| ps-Offset-r16 | 120 |  |  |
| sizeDCI-2-6-r16 | 6 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| ps-WakeUp-r16 | Not present |  |  |
| ps-TransmitPeriodicL1-RSRP-r16 | Not present |  |  |
| ps-TransmitOtherPeriodicCSI-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup-r16 | Not present |  |  |
| harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup-r16 | Not present |  |  |
| pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 | Not present |  |  |
| p-NR-FR2-r16 | Not present |  |  |
| p-UE-FR2-r16 | Not present |  |  |
| nrdc-PCmode-FR1-r16 | Not present |  |  |
| nrdc-PCmode-FR2-r16 | Not present |  |  |
| pdsch-HARQ-ACK-Codebook-r16 | Not present |  |  |
| nfi-TotalDAI-Included-r16 | Not present |  |  |
| ul-TotalDAI-Included-r16 | Not present |  |  |
| pdsch-HARQ-ACK-OneShotFeedback-r16 | Not present |  |  |
| pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 | Not present |  |  |
| pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 | Not present |  |  |
| downlinkAssignmentIndexDCI-0-2-r16 | Not present |  |  |
| downlinkAssignmentIndexDCI-1-2-r16 | Not present |  |  |
| pdsch-HARQ-ACK-CodebookList-r16 | Not present |  |  |
| ackNackFeedbackMode-r16 | Not present |  |  |
| pdcch-BlindDetectionCA-CombIndicator-r16 | Not present |  |  |
| pdcch-BlindDetection2-r16 | Not present |  |  |
| pdcch-BlindDetection3-r16 | Not present |  |  |
| bdFactorR-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DCP | This condition applies when DCP is configured |

#### –	PLMN-Identity

Table 4.6.3-107: PLMN-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PLMN-Identity ::= SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-MNC-Digit | See table 4.4.2-3 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit | See table 4.4.2-3 |  |  |
| } |  |  |  |

#### –	PLMN-IdentityInfoList

Table 4.6.3-108: PLMN-IdentityInfoList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PLMN-IdentityInfoList ::= SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { | 1 entry |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { | 1 entry |  |  |
| PLMN-Identity[1] | PLMN-Identity | entry 1 |  |
| } |  |  |  |
| trackingAreaCode | TrackingAreaCode |  |  |
| ranac | RAN-AreaCode |  |  |
| cellIdentity | CellIdentity |  |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PLMN-IdentityList2

Table 4.6.3-108A: PLMN-IdentityList2

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PLMN-IdentityList2-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PRB-Id

Table 4.6.3-109: PRB-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRB-Id | 0 |  |  |
|  | Set to value of the L_RBs - nrofPRBs where L_RBs is found in Table 4.3.1.0D-1 or Table 4.3.1.0D-2 for the bandwidth and SCS configured and nrofPRBs is defined for the corresponding PUCCH-Resource (1 otherwise). |  | secondHopPRB |

| Condition | Explanation |
| --- | --- |
| secondHopPRB | The IE secondHopPRB in PUCCH-Resource is now set. |

#### –	PTRS-DownlinkConfig

Table 4.6.3-110: PTRS-DownlinkConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PTRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| frequencyDensity | Not present |  |  |
| timeDensity | Not present |  |  |
| epre-Ratio | 0 |  |  |
| resourceElementOffset | Not present |  |  |
| } |  |  |  |

#### –	PTRS-UplinkConfig

Table 4.6.3-111: PTRS-UplinkConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PTRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecoderDisabled SEQUENCE { |  |  |  |
| frequencyDensity | Not present |  |  |
| timeDensity | Not present |  |  |
| maxNrofPorts | n1 |  |  |
| resourceElementOffset | Not present |  |  |
| ptrs-Power | p00 |  |  |
| } |  |  |  |
| transformPrecoderEnabled SEQUENCE { |  |  |  |
| sampleDensity SEQUENCE (SIZE (5)) OF INTEGER { | 5 entries |  |  |
| INTEGER[1] | 1 | entry 1 |  |
| INTEGER[2] | 8 | entry 2 |  |
| INTEGER[3] | 32 | entry 3 |  |
| INTEGER[4] | 32 | entry 4 |  |
| INTEGER[5] | 108 | entry 5 |  |
| } |  |  |  |
| timeDensityTransformPrecoding | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PUCCH-Config

Table 4.6.3-112: PUCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| resourceSetToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet { | 2 entries |  |  |
| PUCCH-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceSetId | 0 |  |  |
| resourceList SEQUENCE (SIZE (0..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId { | 8 entries |  |  |
| PUCCH-ResourceId[1] | 0 | entry 1 |  |
| PUCCH-ResourceId[2] | 1 | entry 2 |  |
| PUCCH-ResourceId[3] | 2 | entry 3 |  |
| PUCCH-ResourceId[4] | 3 | entry 4 |  |
| PUCCH-ResourceId[5] | 4 | entry 5 |  |
| PUCCH-ResourceId[6] | 5 | entry 6 |  |
| PUCCH-ResourceId[7] | 6 | entry 7 |  |
| PUCCH-ResourceId[8] | 7 | entry 8 |  |
| } |  |  |  |
| maxPayloadSize | Not present |  |  |
| } |  |  |  |
| PUCCH-ResourceSet[2] SEQUENCE { |  | entry 2 |  |
| pucch-ResourceSetId | 1 |  |  |
| resourceList SEQUENCE (SIZE (8..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId { | 8 entries |  |  |
| PUCCH-ResourceId[1] | 8 | entry 1 |  |
| PUCCH-ResourceId[2] | 9 | entry 2 |  |
| PUCCH-ResourceId[3] | 10 | entry 3 |  |
| PUCCH-ResourceId[4] | 11 | entry 4 |  |
| PUCCH-ResourceId[5] | 12 | entry 5 |  |
| PUCCH-ResourceId[6] | 13 | entry 6 |  |
| PUCCH-ResourceId[7] | 14 | entry 7 |  |
| PUCCH-ResourceId[8] | 15 | entry 8 |  |
| } |  |  |  |
| maxPayloadSize | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceSetToReleaseList | Not present |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource { | 16 entries |  |  |
| PUCCH-Resource[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceId | 0 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[2] SEQUENCE { |  | entry 2 |  |
| pucch-ResourceId | 1 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[3] SEQUENCE { |  | entry 3 |  |
| pucch-ResourceId | 2 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[4] SEQUENCE { |  | entry 4 |  |
| pucch-ResourceId | 3 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[5] SEQUENCE { |  | entry 5 |  |
| pucch-ResourceId | 4 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[6] SEQUENCE { |  | entry 6 |  |
| pucch-ResourceId | 5 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[7] SEQUENCE { |  | entry 7 |  |
| pucch-ResourceId | 6 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[8] SEQUENCE { |  | entry 8 |  |
| pucch-ResourceId | 7 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format1 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 14 |  |  |
| startingSymbolIndex | 0 |  |  |
| timeDomainOCC | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[9] SEQUENCE { |  | entry 9 |  |
| pucch-ResourceId | 8 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[10] SEQUENCE { |  | entry 10 |  |
| pucch-ResourceId | 9 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[11] SEQUENCE { |  | entry 11 |  |
| pucch-ResourceId | 10 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[12] SEQUENCE { |  | entry 12 |  |
| pucch-ResourceId | 11 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[13] SEQUENCE { |  | entry 13 |  |
| pucch-ResourceId | 12 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRB | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[14] SEQUENCE { |  | entry 14 |  |
| pucch-ResourceId | 13 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBsinitial | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[15] SEQUENCE { |  | entry 15 |  |
| pucch-ResourceId | 14 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRB | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[16] SEQUENCE { |  | entry 16 |  |
| pucch-ResourceId | 15 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
|  | Not present |  | RF OR RRM |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
|  | Not present |  | RF OR RRM |
| format CHOICE { |  |  |  |
| format3 SEQUENCE { |  |  |  |
| nrofPRBs | 1 |  |  |
| nrofSymbols | 14 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceToReleaseList | Not present |  |  |
| format1CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | enabled |  |  |
| additionalDMRS | Not Present |  |  |
| maxCodeRate | Not Present |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| format2 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | Not Present |  |  |
| additionalDMRS | Not Present |  |  |
| maxCodeRate | zeroDot25 |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | True |  |  |
| } |  |  |  |
| } |  |  |  |
| format3 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | enabled |  |  |
| additionalDMRS | True |  |  |
| maxCodeRate | zeroDot25 |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | true |  |  |
| } |  |  |  |
| } |  |  |  |
| format4 | Not present |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] | SchedulingRequestResourceConfig | entry 1 |  |
| } |  |  |  |
| schedulingRequestResourceToReleaseList | Not present |  |  |
| multi-CSI-PUCCH-ResourceList | Not present |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 8 entries |  |  |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| INTEGER[3] | 4 | entry 3 |  |
| INTEGER[4] | 5 | entry 4 |  |
| INTEGER[5] | 6 | entry 5 |  |
| INTEGER[6] | 7 | entry 6 |  |
| INTEGER[7] | 8 | entry 7 |  |
| INTEGER[8] | 9 | entry 8 |  |
| } |  |  |  |
| dl-DataToUL-ACK | Not present |  | Short_DCI |
| spatialRelationInfoToAddModList | Not present |  |  |
| spatialRelationInfoToReleaseList | Not present |  |  |
| pucch-PowerControl | PUCCH-PowerControl |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |

#### –	PUCCH-ConfigCommon

Table 4.6.3-113: PUCCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | 0 |  |  |
| pucch-GroupHopping | enable |  |  |
| hoppingId | Not present |  |  |
| p0-nominal | -90 |  |  |
| } |  |  |  |

#### –	PUCCH-ConfigurationList

Table 4.6.3-113A: PUCCH-ConfigurationList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigurationList-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PUCCH-PathlossReferenceRS-Id

Table 4.6.3-114: PUCCH-PathlossReferenceRS-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-PathlossReferenceRS-Id | 0 |  |  |

#### –	PUCCH-PowerControl

Table 4.6.3-115: PUCCH-PowerControl

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-PowerControl ::= SEQUENCE { |  |  |  |
| deltaF-PUCCH-f0 | 0 |  |  |
| deltaF-PUCCH-f1 | 0 |  |  |
| deltaF-PUCCH-f2 | 0 |  |  |
| deltaF-PUCCH-f3 | 0 |  |  |
| deltaF-PUCCH-f4 | 0 |  |  |
| p0-Set | Not present |  |  |
| pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS { | 1 entry |  |  |
| PUCCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| pucch-PathlossReferenceRS-Id | PUCCH-PathlossReferenceRS-Id |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| twoPUCCH-PC-AdjustmentStates | Not present |  |  |
| } |  |  |  |

#### –	PUCCH-SpatialRelationInfo

Table 4.6.3-116: PUCCH-SpatialRelationInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-SpatialRelationInfo ::= SEQUENCE { |  |  |  |
| pucch-SpatialRelationInfoId | 1 |  |  |
| servingCellId | ServCellIndex |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| pucch-PathlossReferenceRS-Id | PUCCH-PathlossReferenceRS-Id |  |  |
| p0-PUCCH-Id | 1 |  |  |
| closedLoopIndex | i0 |  |  |
| } |  |  |  |

#### –	PUCCH-SpatialRelationInfo-Id

Table 4.6.3-116A: PUCCH-SpatialRelationInfo-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-SpatialRelationInfoId ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PUCCH-TPC-CommandConfig

Table 4.6.3-117: PUCCH-TPC-CommandConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-TPC-CommandConfig ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PUSCH-Config

Table 4.6.3-118: PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPUSCH | Not present |  |  |
| txConfig | Not Present |  | Short_DCI |
|  | codebook |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB | Not present |  |  |
| pusch-PowerControl | PUSCH-PowerControl |  |  |
| frequencyHopping | Not present |  |  |
| frequencyHoppingOffsetLists | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| pusch-TimeDomainAllocationList | Not present |  |  |
| pusch-AggregationFactor | Not present |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| transformPrecoder | enabled |  | TRANSFORM_PRECODER_ENABLED |
|  | Not present | TRANSFORM_PRECODER_DISABLED |  |
| codebookSubset | Not present |  | Short_DCI |
|  | nonCoherent |  |  |
| maxRank | Not present |  | Short_DCI |
|  | 1 |  |  |
|  | 2 |  | 2TX_UL_MIMO |
| rbg-Size | Not present |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| betaOffsets CHOICE { |  |  |  |
| semiStatic | BetaOffsets |  |  |
| } |  |  |  |
| scaling | f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| tp-pi2BPSK | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (DFT-s-OFDM UL waveform is configured) |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |

#### –	PUSCH-ConfigCommon

Table 4.6.3-119: PUSCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| groupHoppingEnabledTransformPrecoding | Not present |  |  |
| pusch-TimeDomainAllocationList | PUSCH-TimeDomainResourceAllocationList |  |  |
| msg3-DeltaPreamble | 1 |  |  |
| p0-NominalWithGrant | -90 |  |  |
| } |  |  |  |

#### –	PUSCH-PowerControl

Table 4.6.3-120: PUSCH-PowerControl

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | Not present |  |  |
| msg3-Alpha | alpha08 |  |  |
| p0-NominalWithoutGrant | -90 |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  | entry 1 |  |
| p0-PUSCH-AlphaSetId | 0 |  |  |
| p0 | 0 |  |  |
| alpha | alpha08 |  |  |
| } |  |  |  |
| } |  |  |  |
| pathlossReferenceRSToAddModList SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS { | 1 entry |  |  |
| PUSCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| pusch-PathlossReferenceRS-Id | 0 |  |  |
| referenceSignal CHOICE{ |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pathlossReferenceRSToReleaseList | Not present |  |  |
| twoPUSCH-PC-AdjustmentStates | Not present |  |  |
| deltaMCS | Not present |  |  |
| sri-PUSCH-MappingToAddModList SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl { | 1 entry |  |  |
| SRI-PUSCH-PowerControl[1] SEQUENCE { |  | entry 1 |  |
| sri-PUSCH-PowerControlId | 0 |  |  |
| sri-PUSCH-PathlossReferenceRS-Id | 0 |  |  |
| sri-P0-PUSCH-AlphaSetId | 0 |  |  |
| sri-PUSCH-ClosedLoopIndex | i0 |  |  |
| } |  |  |  |
| } |  |  |  |
| sri-PUSCH-MappingToReleaseList | Not present |  |  |
| } |  |  |  |

#### –	PUSCH-ServingCellConfig

Table 4.6.3-121: PUSCH-ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| rateMatching | Not present |  |  |
| xOverhead | Not present |  |  |
| } |  |  |  |

#### –	PUSCH-TimeDomainResourceAllocationList

Table 4.6.3-122: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | 4 |  |  |
|  | 2 |  | (RF OR RRM) AND (FR1 AND (SCS15 OR SCS30)) OR (FR2 AND SCS60 AND (DL OR RRM)) |
|  | 3 |  | ((RF AND DL) OR RRM) AND FR2 AND SCS120 |
|  | 6 |  | ((RF AND DL) OR RRM) AND FR1 AND SCS60 |
|  | 4 |  | RF AND FR2 AND SCS60 AND UL |
|  | 8 |  | RF_FR2_120kHz_UL |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | Not present |  |  |
|  | 2 | K2+ Δ=4 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1 AND SCS15 |
|  | 6 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1 AND SCS30 |
|  | 3 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR2 |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Values are chosen so that first slot of a TDD-UL-DL slot configuration period can be used for the Random Access Response and the last slot (of the same or another period) for the corresponding Msg3. |  |  |  |

| Condition | Explanation |
| --- | --- |
| DL | RF Rx measurements. |
| UL | RF UL measurements. |

Table 4.6.3-122A: PUSCH-TimeDomainResourceAllocationList-r16

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList-r16 ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations-r16)) OF PUSCH-TimeDomainResourceAllocation-r16 | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation-r16[1] SEQUENCE { |  | entry 1 |  |
| k2-r16 | 4 |  |  |
| puschAllocationList-r16 SEQUENCE (SIZE(1..maxNrofMultiplePUSCHs-r16)) OF PUSCH-Allocation-r16 { | 1 entry |  |  |
| PUSCH-Allocation-r16[1] SEQUENCE { |  | entry 1 |  |
| mappingType-r16 | typeA |  |  |
| startSymbolAndLength-r16 | 27 | start symbol(S)=0, Length(L)=14 |  |
| startSymbol-r16 | Not present |  |  |
| length-r16 | Not present |  |  |
| numberOfRepetitions-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation-r16[2] SEQUENCE { |  | entry 2 |  |
| k2-r16 | 4 |  |  |
| puschAllocationList-r16 SEQUENCE (SIZE(1..maxNrofMultiplePUSCHs-r16)) OF PUSCH-Allocation-r16 { | 1 entry |  |  |
| PUSCH-Allocation-r16[1] SEQUENCE { |  | entry 1 |  |
| mappingType-r16 | typeA |  |  |
| startSymbolAndLength-r16 | 27 | start symbol(S)=0, Length(L)=14 |  |
| startSymbol-r16 | Not present |  |  |
| length-r16 | Not present |  |  |
| numberOfRepetitions-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PUSCH-TPC-CommandConfig

Table 4.6.3-123: PUSCH-TPC-CommandConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TPC-CommandConfig ::= SEQUENCE { |  |  |  |
| tpc-Index | Not present |  |  |
| tpc-IndexSUL | Not present |  |  |
| targetCell | Not present |  |  |
| } |  |  |  |

#### –	Q-OffsetRange

Table 4.6.3-124: Q-OffsetRange

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Q-OffsetRange | dB0 |  |  |

#### –	Q-QualMin

Table 4.6.3-125: Q-QualMin

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Q-QualMin | FFS |  |  |

#### –	Q-RxLevMin

Table 4.6.3-126: Q-RxLevMin

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Q-RxLevMin | FFS |  |  |

#### –	QuantityConfig

Table 4.6.3-127: QuantityConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig ::= SEQUENCE { |  |  |  |
| quantityConfigNR-List SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR { | 2 entries |  |  |
| QuantityConfigNR[1] SEQUENCE { |  | entry 1 |  |
| quantityConfigCell SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigRS-Index SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| QuantityConfigNR[2] SEQUENCE { |  | entry 2 |  |
| quantityConfigCell SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigRS-Index SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigEUTRA | Not present |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  | INTER-RAT |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| quantityConfigUTRA-FDD-r16 | Not present |  |  |
| quantityConfigUTRA-FDD-r16 SEQUENCE { |  |  | INTER-RAT_UTRA |
| filterCoefficientRSCP-r16 | FilterCoefficient |  |  |
| filterCoefficientEcNO-r16 | FilterCoefficient |  |  |
| } |  |  |  |
| quantityConfigCLI-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-RAT | Configuration for EUTRA inter-RAT measurements |
| INTER-RAT_UTRA | Configuration for UTRA inter-RAT measurements |

#### –	RACH-ConfigCommon

Table 4.6.3-128: RACH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | Not present |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| one | n8 |  | FR1 |
|  | n4 |  | FR2 |
| } |  |  |  |
| groupBconfigured | Not present |  |  |
| ra-ContentionResolutionTimer | sf64 |  |  |
| rsrp-ThresholdSSB | RSRP-Range |  |  |
| rsrp-ThresholdSSB-SUL | Not present |  |  |
|  | RSRP-Range |  | SUL |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| restrictedSetConfig | unrestrictedSet |  |  |
| msg3-transformPrecoder | Not present | transform precoding is disabled for Msg3 PUSCH transmission and any PUSCH transmission scheduled with DCI format 0_0 |  |
| ra-PrioritizationForSlicing-r17 | Not present |  |  |
|  | RA-PrioritizationForSlicing-r17 |  | Slice_RACH |
| featureCombinationPreambles-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17{} | Not present |  |  |
|  | FeatureCombinationPreambles-r17 |  | Slice_RACH |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | Supplementary uplink |
| Slice_RACH | Slice specific RACH configuration |

#### –	RACH-ConfigCommonTwoStepRA

Table 4.6.3-128A: RACH-ConfigCommonTwoStepRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 | RACH-ConfigGenericTwoStepRA |  |  |
| msgA-TotalNumberOfRA-Preambles-r16 | Not present |  |  |
| msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16 CHOICE { | Not present |  |  |
| one | n8 |  | FR1 |
|  | n4 |  | FR2 |
| } |  |  |  |
| msgA-CB-PreamblesPerSSB-PerSharedRO-r16 | Not present |  |  |
| msgA-SSB-SharedRO-MaskIndex-r16 | Not present |  |  |
| groupB-ConfiguredTwoStepRA-r16 | Not present |  |  |
| msgA-PRACH-RootSequenceIndex-r16 CHOICE {} | Not present |  |  |
| msgA-TransMax-r16 | N8 |  |  |
| msgA-RSRP-Threshold-r16 | RSRP-Range |  |  |
| msgA-RSRP-ThresholdSSB-r16 | RSRP-Range |  |  |
| msgA-SubcarrierSpacing-r16 | Not present |  |  |
| msgA-RestrictedSetConfig-r16 | Not present |  |  |
| ra-PrioritizationForAccessIdentityTwoStep-r16 SEQUENCE { |  |  |  |
| ra-Prioritization-r16 | RA-Prioritization |  |  |
| ra-PrioritizationForAI-r16 | '10'B |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer-r16 | sf32 |  |  |
| ra-PrioritizationForSlicingTwoStep-r17 | Not present |  |  |
|  | RA-PrioritizationForSlicing |  | Slice_RACH |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17{} | Not present |  |  |
|  | FeatureCombinationPreambles |  | Slice_RACH |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Slice_RACH | Slice specific RACH configuration |

#### –	RACH-ConfigDedicated

Table 4.6.3-129: RACH-ConfigDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated ::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| ssb-perRACH-Occasion | one |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { | 1 entry |  |  |
| CFRA-SSB-Resource[1] SEQUENCE { |  | entry 1 |  |
| ssb | SSB-Index |  |  |
| ra-PreambleIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ra-Prioritization | Not present |  |  |
| ra-PrioritizationTwoStep-r16 | Not present |  |  |
| cfra-TwoStep-r16 | Not present |  |  |
| cfra-TwoStep-r16 SEQUENCE { |  |  | 2-step RA |
| occasionsTwoStepRA-r16 SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 | RACH-ConfigGenericTwoStepRA |  |  |
| ssb-PerRACH-OccasionTwoStepRA-r16 | oneEighth |  |  |
| } |  |  |  |
| msgA-CFRA-PUSCH-r16 SEQUENCE { |  |  |  |
| msgA-MCS-r16 | 1 |  |  |
| nrofSlotsMsgA-PUSCH-r16 | 1 |  |  |
| nrofMsgA-PO-PerSlot-r16 | one |  |  |
| msgA-PUSCH-TimeDomainOffset-r16 | 4 |  |  |
| msgA-PUSCH-TimeDomainAllocation-r16 | Not present |  |  |
| startSymbolAndLengthMsgA-PO-r16 | 32 |  |  |
| mappingTypeMsgA-PUSCH-r16 | typeA |  |  |
| guardPeriodMsgA-PUSCH-r16 | 0 |  |  |
| guardBandMsgA-PUSCH-r16 | 0 |  |  |
| frequencyStartMsgA-PUSCH-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 1 |  |  |
| nrofMsgA-PO-FDM-r16 | one |  |  |
| msgA-IntraSlotFrequencyHopping-r16 | Not present |  |  |
| msgA-HoppingBits-r16 | Not present |  |  |
| msgA-DMRS-Config-r16 SEQUENCE { |  |  |  |
| msgA-DMRS-AdditionalPosition-r16 | pos0 |  |  |
| msgA-MaxLength-r16 | len2 |  |  |
| msgA-PUSCH-DMRS-CDM-Group-r16 | 0 |  |  |
| msgA-PUSCH-NrofPorts-r16 | 0 |  |  |
| msgA-ScramblingID0-r16 | Not present |  |  |
| msgA-ScramblingID1-r16 | Not present |  |  |
| } |  |  |  |
| nrofDMRS-Sequences-r16 | 1 |  |  |
| msgA-Alpha-r16 | alpha0 |  |  |
| interlaceIndexFirstPO-MsgA-PUSCH-r16 | 1 |  |  |
| nrofInterlacesPerMsgA-PO-r16 | 1 |  |  |
| } |  |  |  |
| msgA-TransMax-r16 | n10 |  |  |
| resourcesTwoStep-r16 SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { | 1 entry |  |  |
| CFRA-SSB-Resource SEQUENCE { |  | entry 1 |  |
| ssb[1] | 0 |  |  |
| ra-PreambleIndex[1] | 52 | Randomly selected |  |
| } |  |  |  |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2-step RA | 2-step RA type Random Access |

#### –	RACH-ConfigGeneric

Table 4.6.3-130: RACH-ConfigGeneric

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 160 |  | FR1 |
|  | 157 |  | FR1 AND HD_FDD |
|  | 149 |  | FR2 |
| msg1-FDM | four |  | FR1 |
|  | one |  | FR2 |
| msg1-FrequencyStart | 0 |  |  |
| zeroCorrelationZoneConfig | 15 |  |  |
| preambleReceivedTargetPower | -118 |  |  |
| preambleTransMax | n7 |  |  |
| powerRampingStep | dB4 |  |  |
| ra-ResponseWindow | sl20 |  |  |
|  | sl10 |  | FR1 AND SCS15 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HD_FDD | SIG AND pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e HD_FDD UE are performing signaling test on FDD band) |

#### –	RACH-ConfigGenericTwoStepRA

Table 4.6.3-130A: RACH-ConfigGenericTwoStepRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGenericTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| msgA-PRACH-ConfigurationIndex-r16 | 120 |  |  |
| msgA-RO-FDM-r16 | four |  |  |
| msgA-RO-FrequencyStart-r16 | 0 |  |  |
| msgA-ZeroCorrelationZoneConfig-r16 | 15 |  |  |
| msgA-PreamblePowerRampingStep-r16 | dB0 |  |  |
| msgA-PreambleReceivedTargetPower-r16 | -118 |  |  |
| msgB-ResponseWindow-r16 | sl2 |  |  |
| preambleTransMax-r16 | n10 |  |  |
| } |  |  |  |

#### –	RA-Prioritization

Table 4.6.3-131: RA-Prioritization

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment | Condition |
| RA-Prioritization::= SEQUENCE { | 0 |  |  |  |
| powerRampingStepHighPriority | dB0 |  |  |  |
|  | FFS |  |  | Slice_RACH |
| scalingFactorBI | zero | zero corresponds to 0 |  |  |
|  | FFS |  |  | Slice_RACH |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Slice_RACH | Slice specific RACH configuration |

#### –	RA-PrioritizationForSlicing

Table 4.6.3-131A: RA-PrioritizationForSlicing

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RA-PrioritizationForSlicing-r17::= SEQUENCE { |  |  |  |
| ra-PrioritizationSliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF RA-PrioritizationSliceInfo-r17 { | 1 entry |  |  |
| RA-PrioritizationSliceInfo-r17[1] SEQUENCE{ |  | entry 1 |  |
| nsagIDList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF NSAG-ID-r17 { | n entries | n is the number of NSAG values associated with the configured set of RA resources |  |
| NSAG-ID-r17[k, k=1..n] | Set to the corresponding NSAG value used in the test case | entry [k, k=1..n] |  |
| } |  |  |  |
| ra-Prioritization-r17 | RA-Prioritization |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RadioBearerConfig

Table 4.6.3-132: RadioBearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB1 |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB2, RESUME |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | RESUME |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB3 |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| srb-Identity | SRB-Identity with condition SRB3 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  | SRB_NR_PDCP REEST |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | SRB_NR_PDCP AND Re-establish_PDCP |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| SRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | SRB_NR_PDCP AND Re-establish_PDCP, REEST |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb3-ToRelease | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | EN-DC_DRB |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | EN-DC_DRB AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | EN-DC_DRB AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | MCG_NR_PDCP |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 12 | EPS Bearer Id of default MCG DRB |  |
| } |  |  |  |
| drb-Identity | 8 | DRB Id of default MCG DRB |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRB1 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB1 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | DRB1 AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | DRB1 AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| daps-Config-r16 | Not present |  |  |
|  | true |  | DRB1 AND DAPS_PDCP |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRB2 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | DRB2 AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | DRB2 AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| daps-Config-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is the number of DRBs established before RRC resume or RRC re-establishment | RESUME, REEST |
| DRB-ToAddMod[k, k=1..n] SEQUENCE { |  | entry [k, k=1..n] |  |
| cnAssociation | Not present |  |  |
| drb-Identity | DRB-Identity with condition DRBk |  |  |
| reestablishPDCP | true |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | Not present |  |  |
| daps-Config-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRBn |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
|  | SDAP-Config with conditionNo-defaultDRB |  | SCG_DRB |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBn |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
|  | PDCP-Config with condition Split |  | Split |
| } |  |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| securityConfig | Not present |  | SRB1 |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig | SecurityAlgorithmConfig |  |  |
| keyToUse | master |  |  |
|  | secondary |  | SRB3, EN-DC_DRB, SecondaryKeys |
| } |  |  |  |
| mrb-ToAddModList-r17 | Not present |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  | MRBm |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm |  |  |
| mrb-IdentityNew-r17 | Not present |  |  |
| reestablishPDCP-r17 | Not present |  |  |
| recoverPDCP-r17 | Not present |  |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm |  | UM_bi_PTP, UM_PTP, UM_PTM, UMPTP_UMPTM |
|  | PDCP-Config with condition MRB_Initialization and MRBm |  |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToReleaseList-r17 | Not present |  |  |
| srb4-ToAddMod-r17 | Not present |  |  |
| srb4-ToAddMod-r17 SEQUENCE { |  |  | SRB4 |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present |  |  |
| srb-Identity-v1700 | SRB-Identity-v1700 |  |  |
| } |  |  |  |
| srb4-ToRelease-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCG_NR_PDCP | EN-DC MCG DRB configured or reconfigured with NR PDCP |
| SRB_NR_PDCP | EN-DC and NR SRB1 and SRB2 configured with NR PDCP |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| SRB4 | Establishment of SRB4 |
| DRB1 | Establishment of DRB1 |
| DRB2 | Establishment of DRB2 |
| DRBn | Establishment of DRBn |
| EN-DC_DRB | EN-DC DRB configured on SCG |
| Re-establish_PDCP | Re-establishment of PDCP |
| Recover_PDCP | Recovery of PDCP |
| RESUME | Used in RRCResume Message |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure. |
| SecondaryKeys | NR-DC SCG or MCG DRB configured or reconfigured with secondary security keys |
| Split | Split PDCP: more than one RLC |
| DAPS_PDCP | Used when the bearer is configured as DAPS bearer |
| SCG_DRB | Used when the bearer is configured as an SCG bearer in either NR-DC or NE-DC |
| MRBm | Establishment of MRBm |
| UM_bi_PTP | Multicast MRB with bidirectional RLC-UM configuration for receiving PTP transmission |
| UM_PTP | Multicast MRB with DL only RLC-UM configuration for receiving PTP transmission |
| UM_PTM | Multicast MRB with DL only RLC-UM entity for receiving PTM transmission |
| UMPTP_UMPTM | Multicast MRB with two RLC-UM entities, one DL only RLC-UM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |

#### –	RadioLinkMonitoringConfig

Table 4.6.3-133: RadioLinkMonitoringConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS { | 1 entry |  |  |
| RadioLinkMonitoringRS[1] SEQUENCE { |  | entry 1 |  |
| radioLinkMonitoringRS-Id | RadioLinkMonitoringRS-Id |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToReleaseList | Not present |  |  |
| beamFailureInstanceMaxCount | Not present |  |  |
| beamFailureDetectionTimer | Not present |  |  |
| } |  |  |  |

#### –	RadioLinkMonitoringRS-Id

Table 4.6.3-134: RadioLinkMonitoringRS-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringRS-Id | 0 |  |  |

#### –	RAN-AreaCode

Table 4.6.3-135: RAN-AreaCode

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RAN-AreaCode | 1 |  |  |

#### –	RateMatchPattern

Table 4.6.3-136: RateMatchPattern

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPattern ::= SEQUENCE { |  |  |  |
| rateMatchPatternId | RateMatchPatternId |  |  |
| patternType CHOICE { |  |  |  |
| controlResourceSet | ControlResourceSetId |  |  |
| } |  |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| dummy | semiStatic | Dummy IE value |  |
| } |  |  |  |

#### –	RateMatchPatternId

Table 4.6.3-137: RateMatchPatternId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternId | 0 |  |  |

#### –	RateMatchPatternLTE-CRS

Table 4.6.3-138: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ReferenceLocation

Table 4.6.3-138AA: ReferenceLocation

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReferenceLocation-r17 | FFS |  |  |

#### –	ReferenceTimeInfo

Table 4.6.3-138A: ReferenceTimeInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReferenceTimeInfo-r16 ::= SEQUENCE { |  |  |  |
| time-r16 ::= SEQUENCE { |  |  |  |
| refDays-r16 | obtained from the local clock |  |  |
| refSeconds-r16 | obtained from the local clock |  |  |
| refMilliSeconds-r16 | obtained from the local clock |  |  |
| refTenNanoSeconds-r16 | obtained from the local clock |  |  |
| } |  |  |  |
| uncertainty-r16 | not present |  |  |
| timeInfoType-r16 | localClock |  |  |
| referenceSFN-r16 | SFN of PCell |  |  |
| } |  |  |  |

#### –	RejectWaitTime

Table 4.6.3-139: RejectWaitTime

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RejectWaitTime | 1 |  |  |

#### –	RepetitionSchemeConfig

Table 4.6.3-139A: RepetitionSchemeConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RepetitionSchemeConfig-r16 ::= CHOICE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ReportConfigId

Table 4.6.3-140: ReportConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigId | 1 |  |  |

#### –	ReportConfigInterRAT

Table 4.6.3-141: ReportConfigInterRAT (InterRAT-Thres, NR-Thres)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL OR PERIODICAL_UTRA |
| reportInterval | ReportInterval |  |  |
| reportAmount | infinity |  |  |
| reportQuantity SEQUENCE { |  | UE shall ignore the value(s) provided in reportQuantity if reportQuantityUTRA-FDD-r16 is configured. |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityUTRA-FDD-r16 | Not present |  |  |
| reportQuantityUTRA-FDD-r16 SEQUENCE { |  |  | PERIODICAL_UTRA |
| cpich-RSCP | true |  |  |
| cpich-EcN0 | false |  |  |
| } |  |  |  |
| } |  |  |  |
| eventTriggered SEQUENCE { |  |  | EVENT_B1 OR EVENT_B2 OR EVENT_B1_UTRA OR EVENT_B2_UTRA |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  | EVENT_B1 |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| rsrp | InterRAT-Thres | INTEGER (0..97) |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| Hysteresis | 0 (0 dB) | The actual value is field value * 0.5 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| eventB2 SEQUENCE { |  |  | EVENT_B2 |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | NR-Thres | INTEGER(0..127) |  |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | InterRAT-Thres | INTEGER (0..97) |  |
| } |  |  |  |
| reportOnLeave | FALSE |  |  |
| Hysteresis | 3 (1.5dB) | The actual value is field value * 0.5 dB |  |
| timeToTrigger | ms1024 |  |  |
| } |  |  |  |
| eventB1-UTRA-FDD-r16 SEQUENCE { |  |  | EVENT_B1_UTRA |
| b1-ThresholdUTRA-FDD-r16 CHOICE { |  |  |  |
| utra-FDD-EcN0-r16 | InterRAT-Thres |  |  |
| } |  |  |  |
| reportOnLeave-r16 | false |  |  |
| hysteresis-r16 | 0 |  |  |
| timeToTrigger-r16 | ms0 |  |  |
| } |  |  |  |
| eventB2-UTRA-FDD-r16 SEQUENCE { |  |  | EVENT_B2_UTRA |
| b2-Threshold1-r16 CHOICE { |  |  |  |
| rsrp | NR-Thres | INTEGER(0..127) |  |
| } |  |  |  |
| b2-Threshold2UTRA-FDD-r16 CHOICE { |  |  |  |
| utra-FDD-EcN0-r16 | InterRAT-Thres |  |  |
| } |  |  |  |
| reportOnLeave-r16 | false |  |  |
| hysteresis-r16 | 0 |  |  |
| timeToTrigger-r16 | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ms120 |  |  |
| reportAmount | r2 |  |  |
| reportQuantity SEQUENCE { |  | UE shall ignore the value(s) provided in reportQuantity if reportQuantityUTRA-FDD-r16 is configured. |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityUTRA-FDD-r16 | Not present |  |  |
| reportQuantityUTRA-FDD-r16 SEQUENCE { |  |  | EVENT_B1_UTRA OR EVENT_B2_UTRA |
| cpich-RSCP | false |  |  |
| cpich-EcN0 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| reportCGI SEQUENCE { |  |  | CGI |
| cellForWhichToReportCGI | EUTRA-PhysCellId |  |  |
| } |  |  |  |
| reportSFTD SEQUENCE { |  |  | SFTD-EUTRA |
| reportSFTD-Meas | true |  |  |
| reportRSRP | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EVENT_B1 | Configuration of Event B1 on E-UTRA carrier |
| EVENT_B2 | Configuration of Event B2 on E-UTRA carrier |
| EVENT_B1_UTRA | Configuration of Event B1 on UTRA carrier |
| EVENT_B2_UTRA | Configuration of Event B2 on UTRA carrier |
| CGI | Configuration of CGI measurement |
| PERIODICAL | Configuration of periodical reporting on E-UTRA carrier |
| PERIODICAL_UTRA | Configuration of periodical reporting on UTRA carrier |
| SFTD-EUTRA | Configuration of SFTD measurement on E-UTRA PSCell |

#### –	ReportConfigNR

Table 4.6.3-142: ReportConfigNR(Thres1, Thres2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| rsType | ssb |  |  |
| reportInterval | ReportInterval |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
|  | true |  | pc_ss_SINR_Meas |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityRS-Indexes | Not present |  |  |
| maxNrofRS-IndexesToReport | Not present |  |  |
| includeBeamMeasurements | false |  |  |
| useAllowedCellList | false |  |  |
| measRSSI-ReportConfig-r16 | Not present |  |  |
| includeCommonLocationInfo-r16 | Not present |  |  |
| includeBT-Meas-r16 | Not present |  |  |
| includeBT-Meas-r16 CHOICE { |  |  | MDT_BT |
| setup | BT-NameList |  |  |
| } |  |  |  |
| includeWLAN-Meas-r16 | Not present |  |  |
| includeWLAN-Meas-r16 CHOICE { |  |  | MDT_WLAN |
| setup | WLAN-NameList |  |  |
| } |  |  |  |
| includeSensor-Meas-r16 | Not present |  |  |
| includeSensor-Meas-r16 CHOICE { |  |  | MDT_SENSOR |
| setup | Sensor-NameList |  |  |
| } |  |  |  |
| ul-DelayValueConfig-r16 | Not present |  |  |
| ul-DelayValueConfig-r16 CHOICE { |  |  | MDT_DELAY |
| setup | UL-DelayValueConfig |  |  |
| } |  |  |  |
| reportAddNeighMeas-r16 | Not present |  |  |
| } |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA1 SEQUENCE { |  |  | EVENT_A1 |
| a1-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| eventA2 SEQUENCE { |  |  | EVENT_A2 |
| a2-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| eventA3 SEQUENCE { |  |  | EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useAllowedCellList | false |  |  |
| } |  |  |  |
| eventA4 SEQUENCE { |  |  | EVENT_A4 |
| a4-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useAllowedCellList | false |  |  |
| } |  |  |  |
| eventA5 SEQUENCE { |  |  | EVENT_A5 |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useAllowedCellList | false |  |  |
| } |  |  |  |
| eventA6 SEQUENCE { |  |  | EVENT_A6 |
| a6-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useAllowedCellList | false |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ReportInterval |  |  |
| reportAmount | r2 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
|  | true |  | pc_ss_SINR_Meas |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityRS-Indexes | Not present |  |  |
| maxNrofRS-IndexesToReport | Not present |  |  |
| includeBeamMeasurements | false |  |  |
| reportAddNeighMeas | Not present |  |  |
| measRSSI-ReportConfig-r16 | Not present |  |  |
|  | MeasRSSI-ReportConfig |  | SharedSpectrum |
| useT312-r16 | Not present |  |  |
| includeCommonLocationInfo-r16 | Not present |  | MDT |
| includeBT-Meas-r16 | Not present |  |  |
| includeBT-Meas-r16 CHOICE { |  |  | MDT_BT |
| setup | BT-NameList |  |  |
| } |  |  |  |
| includeWLAN-Meas-r16 | Not present |  |  |
| includeWLAN-Meas-r16 CHOICE { |  |  | MDT_WLAN |
| setup | WLAN-NameList |  |  |
| } |  |  |  |
| includeSensor-Meas-r16 | Not present |  |  |
| includeSensor-Meas-r16 CHOICE { |  |  | MDT_SENSOR |
| setup | Sensor-NameList |  |  |
| } |  |  |  |
| } |  |  |  |
| reportCGI SEQUENCE { |  |  | CGI OR AUTO_GAPS |
| cellForWhichToReportCGI | PhysCellId |  |  |
| useAutonomousGaps-r16 | Not present |  |  |
|  | setup |  | AUTO_GAPS |
| } |  |  |  |
| reportSFTD SEQUENCE { |  |  | SFTD_NEIGHBOUR or SFTD_PSCELL |
| reportSFTD-Meas | false |  | SFTD_NEIGHBOUR |
|  | true |  | SFTD_PSCELL |
| reportRSRP | false |  |  |
| reportSFTD-NeighMeas | true |  | SFTD_NEIGHBOUR |
|  | Not present |  | SFTD_PSCELL |
| drx-SFTD-NeighMeas | Not present |  |  |
| cellsForWhichToReportSFTD | Not present |  |  |
| } |  |  |  |
| condTriggerConfig SEQUENCE { |  |  | CHO, CPC |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  | CHO AND EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| condEventA5 SEQUENCE { |  |  | CHO AND EVENT_A5 |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType-r16 | ssb |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EVENT_A1 | Configuration of Event A1 |
| EVENT_A2 | Configuration of Event A2 |
| EVENT_A3 | Configuration of Event A3 |
| EVENT_A4 | Configuration of Event A4 |
| EVENT_A5 | Configuration of Event A5 |
| EVENT_A6 | Configuration of Event A6 |
| PERIODICAL | Configuration of periodical reporting |
| CGI | Configuration of CGI measurement |
| AUTO_GAPS | Configuration of Autonomous Gaps |
| CHO | Configuration of conditional handover |
| CPC | Conditional PSCell change |
| MDT | Configuration of MDT |
| MDT_BT | Configuration of MDT including Bluetooth measurements |
| MDT_WLAN | Configuration of MDT including WLAN measurements |
| MDT_SENSOR | Configuration of MDT including Sensor measurements |
| MDT_DELAY | Configuration of UL PDCP Packet Delay per DRB |
| SFTD_NEIGHBOUR | Configurations of SFTD measurement on NR neighbour |
| SFTD_PSCELL | Configurations of SFTD measurement on NR PSCell |
| SharedSpectrum | Operation with shared spectrum channel access |

#### –	ReportConfigNR-SL

Table 4.6.3-142A: ReportConfigNR-SL (Thres)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR-SL-r16 ::= SEQUENCE { |  |  |  |
| reportType-r16 CHOICE { |  |  |  |
| periodical-r16 SEQUENCE { |  |  | PERIODICAL |
| reportInterval-r16 | ReportInterval |  |  |
| reportAmount-r16 | r2 |  |  |
| reportQuantity-r16 SEQUENCE { |  |  |  |
| cbr-r16 | true |  |  |
| } |  |  |  |
| eventTriggered-r16 SEQUENCE { |  |  | EVENT_C1, EVENT_C2 |
| eventId-r16 CHOICE { |  |  |  |
| eventC1 SEQUENCE { |  |  | EVENT_C1 |
| c1-Threshold-r16 | 100*Thres | The actual CBR threshold is 100*Thres % |  |
| hysteresis-r16 | Hysteresis |  |  |
| timeToTrigger-r16 | TimeToTrigger |  |  |
| } |  |  |  |
| eventC2 SEQUENCE { |  |  | EVENT_C2 |
| c2-Threshold-r16 | 100*Thres | The actual CBR threshold is 100*Thres % |  |
| hysteresis-r16 | Hysteresis |  |  |
| timeToTrigger-r16 | TimeToTrigger |  |  |
| } |  |  |  |
| } |  |  |  |
| reportInterval-r16 | ReportInterval |  |  |
| reportAmount-r16 | r2 |  |  |
| reportQuantity-r16 SEQUENCE { |  |  |  |
| cbr-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODICAL | Configuration of periodical reporting |
| EVENT_C1 | Configuration of Event C1 |
| EVENT_C2 | Configuration of Event C2 |

#### –	ReportConfigToAddModList

Table 4.6.3-143: ReportConfigToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigToAddModList ::= SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | ReportConfigId |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR |  |  |
| reportConfigInterRAT | ReportConfigInterRAT |  | InterRAT |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| InterRAT | Configuration with at least one NR PCell and one or more E-UTRA neighbour cell(s) |

#### –	ReportInterval

Table 4.6.3-144: ReportInterval

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportInterval | ms480 |  |  |

#### –	ReselectionThreshold

Table 4.6.3-145: ReselectionThreshold

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReselectionThreshold | FFS |  |  |

#### –	ReselectionThresholdQ

Table 4.6.3-146: ReselectionThresholdQ

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReselectionThresholdQ | FFS |  |  |

#### –	ResumeCause

Table 4.6.3-147: ResumeCause

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ResumeCause | mt-Access |  |  |

#### –	RLC-BearerConfig

Table 4.6.3-148: RLC-BearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRBn |  | DRBn |
|  | LogicalChannelIdentity with condition SRB1 |  | SRB1 |
|  | LogicalChannelIdentity with condition SRB2 |  | SRB2 |
|  | LogicalChannelIdentity with condition SRB3 |  | SRB3 |
|  | LogicalChannelIdentity with condition SRB4 |  | SRB4 |
|  | LogicalChannelIdentity with condition MRBm and PTP |  | MRBm AND PTP |
|  | LogicalChannelIdentity with condition MRBm and PTM |  | MRBm AND PTM |
| servedRadioBearer CHOICE { |  |  |  |
| srb-Identity | SRB-Identity with condition SRB1 |  | SRB1 |
|  | SRB-Identity with condition SRB2 |  | SRB2 |
|  | SRB-Identity with condition SRB3 |  | SRB3 |
| drb-Identity | DRB-Identity with condition DRBn |  | DRBn |
| } |  |  |  |
| servedRadioBearer | Not present |  | RESUME, MRBm, SRB4 |
| reestablishRLC | Not present |  |  |
|  | true |  | Re-establish_RLC, RESUME |
| rlc-Config | RLC-Config using condition AM |  | AM |
|  | RLC-Config using condition UM. |  | UM |
|  | RLC-Config using condition UM_DLonly |  | UM_DLonly |
|  | Not present | Use default parameters as per TS 38.331 [6] clause 9.2.1 | SRB1, SRB2, SRB3, RESUME |
|  | RLC-Config with condition SRB4 |  | SRB4 |
| mac-LogicalChannelConfig | LogicalChannelConfig using condition HI |  | AM |
|  | LogicalChannelConfig using condition LO |  | UM, UM_DLonly |
|  | LogicalChannelConfig using condition SRBn | n= 1, 2, 3, 4 for SRB1, SRB2, SRB3, SRB4 resp. | SRB1, SRB2, SRB3, SRB4, RESUME |
| rlc-Config-v1700 | Not present |  |  |
| logicalChannelIdentityExt-r17 | Not present |  |  |
| multicastRLC-BearerConfig-r17 | Not present |  |  |
| multicastRLC-BearerConfig-r17 SEQUENCE { |  |  | MRBm |
| servedMBS-RadioBearer-r17 | MRB-Identity with condition MRBm |  |  |
| isPTM-Entity-r17 | true |  | PTM |
|  | Not present |  | PTP |
| } |  |  |  |
| servedRadioBearerSRB4-r17 | Not present |  |  |
|  | SRB-Identity-v1700 |  | SRB4 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| AM | RLC AM DRB |
| UM | RLC UM DRB |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| SRB4 | Establishment of SRB4 |
| DRBn | Establishment of DRBn |
| Re-establish_RLC | Re-establishment of RLC |
| RESUME | Used in RRCResume Message |
| MRBm | Establishment of MRBm |
| PTM | RLC entity is used for receving PTM transmission |
| PTP | RLC entity is used for receving PTP transmission |
| UM_DLonly | DL only RLC UM |

#### –	RLC-Config

Table 4.6.3-149: RLC-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  | AM, SRB4 |
| ul-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size18 |  |  |
|  | size12 |  | pc_supportOfRedCap_r17, SRB4 |
| t-PollRetransmit | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
|  | ms45 |  | SRB4 AND (FR1 OR FR2) |
| pollPDU | p32768 |  |  |
|  | infinity |  | SRB4 |
| pollByte | kB750 |  |  |
|  | infinity |  | SRB4 |
| maxRetxThreshold | t8 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size18 |  |  |
|  | size12 |  | pc_supportOfRedCap_r17, SRB4 |
| t-Reassembly | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
|  | ms35 |  | SRB4 AND (FR1 OR FR2) |
| t-StatusProhibit | ms30 |  |  |
|  | ms0 |  | SRB4 |
| } |  |  |  |
| } |  |  |  |
| um-Bi-Directional SEQUENCE { |  |  | UM |
| ul-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size12 |  | pc_um_WIthLongSN |
|  | size6 |  | NOT pc_um_WIthLongSN AND pc_um_WithShortSN |
| } |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size12 |  | pc_um_WIthLongSN |
|  | size6 |  | NOT pc_um_WIthLongSN AND pc_um_WithShortSN |
| t-Reassembly | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| um-Uni-Directional-DL SEQUENCE { |  |  | UM_DLonly |
| dl-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size12 |  | pc_um_WIthLongSN |
|  | size6 |  | NOT pc_um_WIthLongSN AND pc_um_WithShortSN |
| t-Reassembly | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| AM | RLC AM |
| UM | RLC UM |
| UM_DLonly | DL only RLC UM |
| SRB4 | Establishment of SRB4 |

#### –	RLF-TimersAndConstants

Table 4.6.3-150: RLF-TimersAndConstants

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms1000 |  |  |
| n310 | n1 |  |  |
| n311 | n1 |  |  |
| t311 | ms1000 |  |  |
| } |  |  |  |

#### –	RMTC-Config

Table 4.6.3-150A: Void

#### –	RNTI-Value

Table 4.6.3-151: RNTI-Value

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RNTI-Value | SS arbitrarily selects a value between ‘0001’H and ‘FFEF’H |  |  |
|  | SS arbitrarily selects a value between ‘0001’H and ‘FFEF’H different from the MCG RNTI-Value. |  | NR-DC_SCG |

| Condition | Explanation |
| --- | --- |
| NR-DC_SCG | Add SCG (NR-DC) |

#### –	RSRP-Range

Table 4.6.3-152: RSRP-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RSRP-Range | 0 | For measurements, 0 means L3 SS-RSRP<-156dBm according to Table 10.1.6.1-1 in TS 38.133 [13]. For thresholds, 0 means -156dBm. |  |

#### –	RSRQ-Range

Table 4.6.3-153: RSRQ-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RSRQ-Range | 0 | For measurements, 0 means SS-RSRQ<-43dB according to Table 10.1.11.1-1 in TS 38.133 [14]. For thresholds, 0 means -43.5dB. |  |

#### –	RSSI-Range

Table 4.6.3-153A: RSSI-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RSSI-Range-r16 | FFS |  |  |

#### –	RxTxTimeDiff

Table 4.6.3-153B: RxTxTimeDiff

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RxTxTimeDiff-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SCellActivationRS-Config

Table 4.6.3-153C: SCellActivationRS-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellActivationRS-Config-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SCellActivationRS-ConfigId

Table 4.6.3-153D: SCellActivationRS-ConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellActivationRS-ConfigId-r17 | FFS |  |  |

#### –	SCellIndex

Table 4.6.3-154: SCellIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellIndex | 1 |  |  |
|  | 2 |  | EN-DC |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |

#### –	SchedulingRequestConfig

Table 4.6.3-155: SchedulingRequestConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestToAddModList SEQUENCE (SIZE(1..maxNrofSR-ConfigPerCellGroup)) OF SSchedulingRequestToAddMod { | 1 entry |  |  |
| SchedulingRequestToAddMod[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestId | SchedulingRequestId |  |  |
| sr-ProhibitTimer | Not present |  |  |
| sr-TransMax | n16 |  |  |
| } |  |  |  |
| } |  |  |  |
| schedulingRequestToReleaseList | Not present |  |  |
| } |  |  |  |

#### –	SchedulingRequestId

Table 4.6.3-156: SchedulingRequestId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestId | 0 |  |  |

#### –	SchedulingRequestResourceConfig

Table 4.6.3-157: SchedulingRequestResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceId | SchedulingRequestResourceId |  |  |
| schedulingRequestID | SchedulingRequestId |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 9 | With SCS = kHz15 results in repetition every 10 ms | SCS15 |
| sl20 | 9 | With SCS = kHz30 results in repetition every 10 ms | SCS30 |
| sl40 | 19 | With SCS = kHz60 results in repetition every 10 ms | FR1 AND SCS60 |
|  | 9 | With SCS = kHz60 results in repetition every 10 ms | FR2 AND SCS60 |
| sl80 | 9 | With SCS = kHz120 results in repetition every 10 ms | SCS120 |
| } |  |  |  |
| resource | 0 | ID of the PUCCH resource as configured by PUCCH-Config (Table 4.6.3-112) |  |
|  | 6 | Same as the PUCCH resource for HARQ feedback indicated in Table 4.3.6.1.2.1-1 and Table 4.3.6.1.2.1-2 | SIG AND (NOT pc_twoPUCCH_AnyOthersInSlot) |
| } |  |  |  |

#### –	SchedulingRequestResourceId

Table 4.6.3-158: SchedulingRequestResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceId | 1 |  |  |

#### –	ScramblingId

Table 4.6.3-159: ScramblingId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ScramblingId | 0 |  |  |

#### –	SCS-SpecificCarrier

Table 4.6.3-160: SCS-SpecificCarrier

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCS-SpecificCarrier ::= SEQUENCE { |  |  |  |
| offsetToCarrier | offsetToCarrier as defined for the DL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | DL_PointA |
|  | offsetToCarrier as defined for the UL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | UL_PointA |
|  | offsetToCarrier as defined for the SL NRf1 frequency | For signalling test cases see subclause 6.2.3.7. Otherwise, see subclause 4.3.1.8. | SL_PointA |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| carrierBandwidth | carrierBandwidth as defined for the frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DL_PointA | IE absoluteFrequencyPointA for downlink |
| UL_PointA | IE absoluteFrequencyPointA for uplink |
| SL_PointA | IE absoluteFrequencyPointA for sidelink |

#### –	SDAP-Config

Table 4.6.3-161: SDAP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDAP-Config ::= SEQUENCE { |  |  |  |
| pdu-Session | The same value as the PDU session ID IE of the contained message |  |  |
| sdap-HeaderDL | absent |  |  |
| sdap-HeaderUL | present |  |  |
| defaultDRB | true |  |  |
|  | false |  | No-defaultDRB |
| mappedQoS-FlowsToAdd SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI { | n entries |  |  |
| QFI[n] | The list of QFIs of the Authorized QoS flow descriptions IE of the contained 5GSM message | entry n |  |
| } |  |  |  |
| mappedQoS-FlowsToRelease | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| No-defaultDRB | The defaultDRB value is false. |

#### –	SearchSpace

Table 4.6.3-162: SearchSpace

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition CSS |  | CSS |
|  | SearchSpaceId with condition USS |  | USS |
|  | SearchSpaceId with condition SISS |  | SISS |
|  | SearchSpaceId with condition MSS |  | MSS |
|  | SearchSpaceId with condition SLSS |  | SLSS |
|  | SearchSpaceId with condition DCI_2_6 |  | DCI_2_6 |
|  | SearchSpaceId with condition PEI |  | PEI |
| controlResourceSetId | ControlResourceSetId |  |  |
|  | ControlResourceSetId with condition Common0 |  | CSS, SISS, PEI |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| sl5 | 0 |  | (CSS OR USS) AND HD_FDD |
| sl10 | 5 |  | SISS |
| } |  |  |  |
| duration | Not present | 1 slot per default |  |
|  | 2 |  | SISS |
|  | 4 |  | (CSS OR USS) AND HD_FDD |
| monitoringSymbolsWithinSlot | 10000000000000 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n4 |  | FR1 |
|  | n3 |  | FR2 |
| aggregationLevel4 | n2 |  |  |
| aggregationLevel8 | n1 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType | Not present |  | MSS |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS, DCI_2_6, PEI |
| dci-Format0-0-AndFormat1-0 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format2-0 | Not present |  |  |
| dci-Format2-1 | Not present |  |  |
| dci-Format2-2 | Not present |  |  |
| dci-Format2-3 | Not present |  |  |
| } |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS, SLSS |
| dci-Formats | formats0-0-And-1-0 |  | Short_DCI |
| dci-Formats | formats0-1-And-1-1 |  |  |
| dci-Formats-MT-r16 | Not present |  |  |
| dci-FormatsSL-r16 | formats3-0 |  | SLSS |
| dci-FormatsExt-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |
| USS | UE-Specific SearchSpace |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SISS | SearchSpace for SI |
| MSS | SearchSpace for MBS Multicast reception |
| PEI | Paging Early Indication is configured in the cell. |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |
| HD_FDD | SIG AND pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e HD_FDD UE are performing signalling test on FDD band) |

#### –	SearchSpaceId

Table 4.6.3-163: SearchSpaceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceID | 1 |  | CSS |
|  | 2 |  | USS |
|  | 3 |  | SISS |
|  | 4 |  | PEI |
|  | 5 |  | MSS |
|  | 6 |  | SLSS |
|  | 7 |  | DCI_2_6 |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |
| USS | UE-Specific SearchSpace |
| SISS | SearchSpace for SI |
| PEI | Paging Early Indication is configured in the cell. |
| MSS | SearchSpace for MBS Multicast reception. |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |

#### –	SearchSpaceZero

Table 4.6.3-164: SearchSpaceZero

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceZero | 0 | Index addressing SearchSpace#0 parameter set in Tables 13.11 .. 13.15 of TS 38.213 [22] |  |

#### –	SecurityAlgorithmConfig

Table 4.6.3-165: SecurityAlgorithmConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityAlgorithmConfig ::= SEQUENCE { |  |  |  |
| cipheringAlgorithm | nea0 |  | RF OR RRM |
|  | Set according to PIXIT px_NR_CipheringAlgorithm | see TS 38.523-3 [23] | SIG |
| integrityProtAlgorithm | nia2 |  |  |
|  | Set according to PIXIT px_NR_IntegrityProtAlgorithm | see TS 38.523-3 [23] | SIG |
| } |  |  |  |

#### –	SemiStaticChannelAccessConfig

Table 4.6.3-165A: SemiStaticChannelAccessConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SemiStaticChannelAccessConfig-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SemiStaticChannelAccessConfigUE

Table 4.6.3-165BA: SemiStaticChannelAccessConfigUE

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SemiStaticChannelAccessConfigUE-r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	Sensor-LocationInfo

Table 4.6.3-165B: Sensor-LocationInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sensor-LocationInfo-r16 ::= SEQUENCE { |  |  |  |
| sensor-MeasurementInformation-r16 | Not checked | OCTET STRING |  |
| sensor-MotionInformation-r16 | Not checked | OCTET STRING |  |
| } |  |  |  |

#### –	ServingCellAndBWP-Id

Table 4.6.3-165C: ServingCellAndBWP-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellAndBWP-Id -r17 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ServCellIndex

Table 4.6.3-166: ServCellIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServCellIndex | 0 |  |  |
|  | 1 |  | EN-DC, NR-DC_SCG |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| NR-DC_SCG | Add SCG (NR-DC) |

#### –	ServingCellConfig

Table 4.6.3-167: ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
|  | BWP-DownlinkDedicated with condition SCell_add |  | SCell_add |
|  | BWP-DownlinkDedicated with condition MBS_Multicast |  | MBS_Multicast |
|  | Not present |  | MEAS, RESUME |
| downlinkBWP-ToReleaseList | Not present |  |  |
| downlinkBWP-ToAddModList | Not present |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  | DLBWP_add |
| BWP-Downlink[1] | BWP-Downlink |  |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS |
| bwp-InactivityTimer | Not present |  |  |
| defaultDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS, RESUME |
| uplinkConfig | Not present |  | MEAS, No_UL |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition SUL_NUL |  | PUSCH_PUCCH_ON_SUL |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  | ULBWP_add |
| BWP-Uplink[1] | BWP-Uplink |  |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig | Not present |  | RESUME |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| } |  |  |  |
| supplementaryUplink | Not present |  |  |
| supplementaryUplink SEQUENCE { |  |  | PUSCH_PUCCH_ON_SUL |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDCCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  | MEAS, RESUME |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig | Not present |  | MEAS, RESUME |
| csi-MeasConfig | Not present |  |  |
| sCellDeactivationTimer | Not present |  |  |
| crossCarrierSchedulingConfig | Not present |  |  |
| tag-Id | 0 |  |  |
| dummy1 | Not present |  |  |
| pathlossReferenceLinking | Not present |  |  |
| servingCellMO | Not present |  |  |
|  | MeasObjectId |  | MEAS |
| nr-dl-PRS-PDC-Info-r17 | Not present |  |  |
| semiStaticChannelAccessConfigUE-r17 | Not present |  |  |
| mimoParam-r17 | Not present |  |  |
| channelAccessMode2-r17 | Not present |  |  |
| timeDomainHARQ-BundlingType1-r17 | Not present |  |  |
| nrofHARQ-BundlingGroups-r17 | Not present |  |  |
| fdmed-ReceptionMulticast-r17 | Not present |  |  |
| moreThanOneNackOnlyMode-r17 | Not present |  |  |
| tci-ActivatedConfig-r17 | Not present |  |  |
| tci-ActivatedConfig-r17 SEQUENCE { |  |  | SCG_Activate |
| pdcch-TCI-r17 SEQUENCE (SIZE(1..5)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId | entry 1 |  |
| } |  |  |  |
| pdsch-TCI-r17 | 1 | BIT STRING (SIZE (1..maxNrofTCI-States)) |  |
| } |  |  |  |
| directionalCollisionHandling-DC-r17 | Not present |  |  |
| lte-NeighCellsCRS-AssistInfoList-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PUSCH_PUCCH_ON_SUL | For the purpose of SUL test under condition that supplementary uplink is configured with both PUSCH and PUCCH on SUL carrier. |
| MEAS | A NR or EN-DC measurement is configured. |
| No_UL | No uplink CA |
| RESUME | Used in RRCResume Message |
| SCell_add | Add SCell |
| MBS_Multicast | Used for MBS Multicast reception |
| DLBWP_add | A dedicated DL BWP is configured |
| ULBWP_add | A dedicated UL BWP is configured |
| SCG_Activate | When SCG is being activated |

#### –	ServingCellConfigCommon

Table 4.6.3-168: ServingCellConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| downlinkConfigCommon | DownlinkConfigCommon |  |  |
|  | DownlinkConfigCommon with condition SCell_add |  | SCell_add |
| uplinkConfigCommon | UplinkConfigCommon |  |  |
|  | Not present |  | No_UL |
| supplementaryUplinkConfig | Not present |  |  |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 0100 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHZ) |
|  | 1000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHz) |
| mediumBitmap | 01000000 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
|  | 10000000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 0100000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#1 |
|  | 1000000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#0 |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| dmrs-TypeA-Position | pos2 |  |  |
| lte-CRS-ToMatchAround | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| ssbSubcarrierSpacing | SubcarrierSpacing | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. Value SS block SCS. |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| channelAccessMode-r16 | Not present |  |  |
| channelAccessMode-r16 CHOICE { |  |  | SharedSpectrum |
| dynamic | NULL |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | Not present |  |  |
| ssb-PositionQCL-r16 | Not present |  |  |
|  | SSB-PositionQCL-Relation-r16 |  | SharedSpectrum |
| highSpeedConfig-r16 | Not present |  |  |
|  | HighSpeedConfig-r16 |  | R16 HST OR R17 HST FR1 |
| highSpeedConfig-v1700 | Not present |  |  |
|  | HighSpeedConfig-v1700 |  | R17 HST FR1 |
| channelAccessMode2-r17 | Not present |  |  |
| discoveryBurstWindowLength-r17 | Not present |  |  |
| ssb-PositionQCL-r17 | Not present |  |  |
| highSpeedConfigFR2-r17 | Not present |  |  |
| uplinkConfigCommon-v1700 | Not present |  |  |
| ntn-Config-r17 | Not present |  |  |
|  | NTN-Config |  | GSO, NGSO |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=1.88GHz | Frequency range <= 1.88GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| No_UL | No uplink CA |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |
| SharedSpectrum | Operation with shared spectrum channel access in FR1 |
| R16 HST | For R16 HST test |
| R17 HST FR1 | For R17 HST FR1 test |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |
| SCell_add | Add SCell |