3GPP TSG WG RAN5 Meeting #104																R5-244035

Maastricht, Netherlands

19th – 23rd August 2024

3GPP TSG WG RAN5 Meeting #103																		R5-244022

Fukuoka, Japan

20th – 24th May 2024

# Report from the RAN WG5#103 Meeting

# Fukuoka, Japan

20th – 24th May 2024

# v 1.0

# Chair:					Jacob John, Motorola Mobility

Meeting Secretary:		Ingbert Sigovich, ETSI/MCC Project manager

Vice Chair (SIG sub WG):	Xiaozhong Chen, CATT

RF session Chair (RF sub WG):	Pradeep Gowda, Qualcomm Inc

RF session Secretary:		Amy Tao, Bureau Veritas

Contents:

Contents:	2

1	Opening of the meeting	22

1.1	Welcoming brief by the host	22

2	Reports	23

2.1	Live Reports	23

2.2	General Reports for information	26

3	Incoming Liaison Statements	26

4	RAN5 General Issues	28

4.1	New Work Item proposals - for intro only	28

4.2	General Discussion Papers	38

4.2.1	5GS	38

4.2.2	NR Applicability specifications (Software Implementation)	38

4.2.3	All other topics	39

4.3	RAN5 PRDs/Templates	42

4.4	Meeting schedule for 2024-25	42

4.5	Tdocs for mid-week joint session	42

4.5.1	Agenda for mid-week session	42

4.5.2	RF group docs for WG review/verdict - original A.I. retained	42

4.5.3	Sig group docs for WG review/verdict - original A.I. retained	43

4.5.4	Other open issues from joint sessions - original A.I. retained	43

4.5.5	Other	45

5	RF Functional Area	45

5.1	Review action points (fm A.I. 2.1)	45

5.2	Review incoming LS (fm A.I. 3) & new subject discussion papers	45

5.3	Open Work Items	47

5.3.1	REL-16 NR CA and DC; and NR and LTE DC Configurations (UID-830083) NR_CADC_NR_LTE_DC_R16-UEConTest	47

5.3.1.1	TS 38.508-1	47

5.3.1.1.1	Test frequencies (Clause 4.3.1)	47

5.3.1.1.2	Test environment for RF (Clauses 5)	48

5.3.1.1.3	Test environment for RRM (Clause 7)	48

5.3.1.1.4	Other clauses, Annexes	48

5.3.1.2	TS 38.508-2	48

5.3.1.3	TS 38.521-1	49

5.3.1.3.1	Tx Requirements (Clause 6)	49

5.3.1.3.2	Rx Requirements (Clause 7)	50

5.3.1.3.3	Clauses 1-5, Annexes	52

5.3.1.4	TS 38.521-2	52

5.3.1.4.1	Tx Requirements (Clause 6)	52

5.3.1.4.2	Rx Requirements (Clause 7)	52

5.3.1.4.3	Clauses 1-5, Annexes	52

5.3.1.5	TS 38.521-3	52

5.3.1.5.1	Tx Requirements (Clause 6)	52

5.3.1.5.2	Rx Requirements (Clause 7)	54

5.3.1.5.3	Clauses 1-5, Annexes	55

5.3.1.6	TS 38.521-4	56

5.3.1.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	56

5.3.1.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	56

5.3.1.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	56

5.3.1.6.4	Clauses 1-4, Annexes	56

5.3.1.7	TS 38.522	56

5.3.1.8	TS 38.533	56

5.3.1.9	TR 38.903 (NR MU & TT analyses)	56

5.3.1.10	TR 38.905 (NR Test Points Radio Transmission and Reception)	56

5.3.1.11	Discussion Papers, Work Plan, TC lists	58

5.3.2	5G V2X with NR sidelink (UID-880069) 5G_V2X_NRSL_eV2XARC-UEConTest	58

5.3.2.1	TS 38.508-1	58

5.3.2.1.1	Test frequencies (Clause 4.3.1)	58

5.3.2.1.2	Test environment for RF (Clauses 5)	58

5.3.2.1.3	Test environment for RRM (Clause 7)	58

5.3.2.1.4	Other clauses, Annexes	58

5.3.2.2	TS 38.508-2	58

5.3.2.3	TS 38.509	58

5.3.2.4	TS 38.521-1	58

5.3.2.4.1	Tx Requirements (Clause 6)	58

5.3.2.4.2	Rx Requirements (Clause 7)	59

5.3.2.4.3	Clauses 1-5, Annexes	59

5.3.2.5	TS 38.521-3	60

5.3.2.5.1	Tx Requirements (Clause 6)	60

5.3.2.5.2	Rx Requirements (Clause 7)	60

5.3.2.5.3	Clauses 1-5, Annexes	60

5.3.2.6	TS 38.521-4	60

5.3.2.6.1	V2X Requirements (Clause 11)	60

5.3.2.6.2	Clauses 1-4, Annexes	60

5.3.2.7	TS 38.522	60

5.3.2.8	TS 38.533	60

5.3.2.9	TS 36.509	60

5.3.2.10	TR 38.903 (NR MU & TT analyses)	60

5.3.2.11	TR 38.905 (NR Test Points Radio Transmission and Reception)	60

5.3.2.12	Discussion Papers, Work Plan, TC lists	61

5.3.3	Rel-17 NR CA and DC; and NR and LTE DC Configurations (UID-900056) NR_CADC_NR_LTE_DC_R17-UEConTest	61

5.3.3.1	TS 38.508-1	61

5.3.3.1.1	Test frequencies (Clause 4.3.1)	61

5.3.3.1.2	Test environment for RF (Clauses 5)	62

5.3.3.1.3	Test environment for RRM (Clause 7)	62

5.3.3.1.4	Other clauses, Annexes	62

5.3.3.2	TS 38.508-2	62

5.3.3.3	TS 38.521-1	63

5.3.3.3.1	Tx Requirements (Clause 6)	63

5.3.3.3.2	Rx Requirements (Clause 7)	65

5.3.3.3.3	Clauses 1-5, Annexes	66

5.3.3.4	TS 38.521-2	67

5.3.3.4.1	Tx Requirements (Clause 6)	67

5.3.3.4.2	Rx Requirements (Clause 7)	67

5.3.3.4.3	Clauses 1-5, Annexes	67

5.3.3.5	TS 38.521-3	67

5.3.3.5.1	Tx Requirements (Clause 6)	67

5.3.3.5.2	Rx Requirements (Clause 7)	67

5.3.3.5.3	Clauses 1-5, Annexes	68

5.3.3.6	TS 38.521-4	68

5.3.3.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	68

5.3.3.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	68

5.3.3.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	68

5.3.3.6.4	Clauses 1-4, Annexes	68

5.3.3.7	TS 38.522	68

5.3.3.8	TS 38.533	68

5.3.3.9	TR 38.903 (NR MU & TT analyses)	68

5.3.3.10	TR 38.905 (NR Test Points Radio Transmission and Reception)	68

5.3.3.11	Discussion Papers, Work Plan, TC lists	69

5.3.4	NR-based access to unlicensed spectrum (UID-911003) NR_unlic-UEConTest	69

5.3.4.1	TS 38.508-1	69

5.3.4.1.1	Test frequencies (Clause 4.3.1)	69

5.3.4.1.2	Test environment for RF (Clauses 5)	69

5.3.4.1.3	Test environment for RRM (Clause 7)	69

5.3.4.1.4	Other clauses, Annexes	69

5.3.4.2	TS 38.508-2	69

5.3.4.3	TS 38.509	69

5.3.4.4	TS 38.521-1	69

5.3.4.4.1	Tx Requirements (Clause 6)	69

5.3.4.4.2	Rx Requirements (Clause 7)	71

5.3.4.4.3	Clauses 1-5, Annexes	72

5.3.4.5	TS 38.521-3	73

5.3.4.5.1	Tx Requirements (Clause 6)	73

5.3.4.5.2	Rx Requirements (Clause 7)	73

5.3.4.5.3	Clauses 1-5, Annexes	73

5.3.4.6	TS 38.521-4	73

5.3.4.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	73

5.3.4.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	73

5.3.4.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	73

5.3.4.6.4	Clauses 1-4, Annexes	73

5.3.4.7	TS 38.522	73

5.3.4.8	TS 38.533	74

5.3.4.9	TR 38.903 (NR MU & TT analyses)	81

5.3.4.10	TR 38.905 (NR Test Points Radio Transmission and Reception)	90

5.3.4.11	Discussion Papers, Work Plan, TC lists	90

5.3.5	Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) (UID-930051) ENDC_PC2_R17_xLTE_yNR-UEConTest	90

5.3.5.1	TS 38.508-2	90

5.3.5.2	TS 38.521-3	90

5.3.5.2.1	Tx Requirements (Clause 6)	90

5.3.5.2.2	Rx Requirements (Clause 7)	90

5.3.5.2.3	Clauses 1-5, Annexes	90

5.3.5.3	TS 38.522	90

5.3.5.4	TR 38.905 (NR Test Points Radio Transmission and Reception)	90

5.3.5.5	Discussion Papers, Work Plan, TC lists	91

5.3.6.	Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 (UID-950062) NR_FR2_FWA_Bn257_Bn258-UEConTest	91

5.3.6.1	TS 38.508-1	91

5.3.6.2	TS 38.508-2	91

5.3.6.3	TS 38.521-2	91

5.3.6.3.1	Tx Requirements (Clause 6)	91

5.3.6.3.2	Rx Requirements (Clause 7)	91

5.3.6.3.3	Clauses 1-5, Annexes	92

5.3.6.4	TS 38.521-4	92

5.3.6.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	92

5.3.6.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	92

5.3.6.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	92

5.3.6.4.4	Clauses 1-4, Annexes	92

5.3.6.5	TS 38.522	92

5.3.6.6	TS 38.533	92

5.3.6.7	TR 38.903 (NR MU & TT analyses)	92

5.3.6.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	93

5.3.6.9	Discussion Papers, Work Plan, TC lists	93

5.3.7	NR small data transmissions in INACTIVE state (UID-960072) NR_SmallData_INACTIVE-UEConTest	93

5.3.7.1	TS 38.508-1	93

5.3.7.2	TS 38.508-2	93

5.3.7.3	TS 38.522	93

5.3.7.4	TS 38.533	93

5.3.7.5	TR 38.903 (NR MU & TT analyses)	95

5.3.7.6	Discussion Papers, Work Plan, TC lists	95

5.3.8	Solutions for NR to support non-terrestrial networks (NTN) (UID-960074) NR_NTN_solutions_plus_CT-UEConTest	95

5.3.8.1	TS 38.508-1	95

5.3.8.2	TS 38.508-2	95

5.3.8.3	TS 38.509	95

5.3.8.4	TS 38.521-5	95

5.3.8.5	TS 38.522	102

5.3.8.6	TS 38.533	102

5.3.8.7	TR 38.903 (NR MU & TT analyses)	103

5.3.8.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	103

5.3.8.9	Discussion Papers, Work Plan, TC lists	104

5.3.9	Further enhancement on NR demodulation performance (UID-960075) NR_demod_enh2-UEConTest	104

5.3.9.1	TS 38.508-1	104

5.3.9.2	TS 38.508-2	105

5.3.9.3	TS 38.521-4	105

5.3.9.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	105

5.3.9.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	106

5.3.9.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	106

5.3.9.3.4	Clauses 1-4, Annexes	106

5.3.9.4	TS 38.522	106

5.3.9.5	TR 38.903 (NR MU & TT analyses)	107

5.3.9.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	107

5.3.9.7	Discussion Papers, Work Plan, TC lists	107

5.3.10	Further enhancements on MIMO for NR (UID-960079) NR_feMIMO-UEConTest	107

5.3.10.1	TS 38.508-1	107

5.3.10.2	TS 38.508-2	107

5.3.10.3	TS 38.521-4	107

5.3.10.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	107

5.3.10.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	108

5.3.10.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	108

5.3.10.3.4	Clauses 1-4, Annexes	108

5.3.10.4	TS 38.522	108

5.3.10.5	TS 38.533	109

5.3.10.6	TR 38.903 (NR MU & TT analyses)	111

5.3.10.7	Discussion Papers, Work Plan, TC lists	111

5.3.11	NR support for high speed train scenario in frequency range 2 (FR2) (UID-960080) NR_HST_FR2-UEConTest	111

5.3.11.1	TS 38.508-1	111

5.3.11.2	TS 38.508-2	111

5.3.11.3	TS 38.521-2	111

5.3.11.3.1	Tx Requirements (Clause 6)	111

5.3.11.3.2	Rx Requirements (Clause 7)	112

5.3.11.3.3	Clauses 1-5, Annexes	112

5.3.11.4	TS 38.521-4	112

5.3.11.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	112

5.3.11.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	112

5.3.11.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	112

5.3.11.4.4	Clauses 1-4, Annexes	112

5.3.11.5	TS 38.522	112

5.3.11.6	TS 38.533	113

5.3.11.7	TR 38.903 (NR MU & TT analyses)	114

5.3.11.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	114

5.3.11.9	Discussion Papers, Work Plan, TC lists	114

5.3.12	NR Sidelink Relay (UID-960083) NR_SL_relay-UEConTest	114

5.3.12.1	TS 38.508-1	114

5.3.12.2	TS 38.508-2	114

5.3.12.3	TS 38.521-1	114

5.3.12.3.1	Tx Requirements (Clause 6)	114

5.3.12.3.2	Rx Requirements (Clause 7)	114

5.3.12.3.3	Clauses 1-5, Annexes	114

5.3.12.4	TS 38.522	114

5.3.12.5	TS 38.533	114

5.3.12.6	TR 38.903 (NR MU & TT analyses)	114

5.3.12.7	TR 38.905 (NR Test Points Radio Transmission and Reception)	114

5.3.12.8	Discussion Papers, Work Plan, TC lists	114

5.3.13	NR Sidelink enhancement (UID-960084) NR_SL_enh-UEConTest	114

5.3.13.1	TS 38.508-1	114

5.3.13.2	TS 38.508-2	114

5.3.13.3	TS 38.521-1	114

5.3.13.3.1	Tx Requirements (Clause 6)	114

5.3.13.3.2	Rx Requirements (Clause 7)	115

5.3.13.3.3	Clauses 1-5, Annexes	115

5.3.13.4	TS 38.522	115

5.3.13.5	TS 38.533	115

5.3.13.6	TR 38.903 (NR MU & TT analyses)	115

5.3.13.7	TR 38.905 (NR Test Points Radio Transmission and Reception)	115

5.3.13.8	Discussion Papers, Work Plan, TC lists	116

5.3.14	NR RRM enhancement (UID-960089) NR_RRM_enh-UEConTest	116

5.3.14.1	TS 38.508-1	116

5.3.14.2	TS 38.508-2	116

5.3.14.3	TS 38.522	116

5.3.14.4	TS 38.533	117

5.3.14.5	TR 38.903 (NR MU & TT analyses)	131

5.3.14.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	133

5.3.14.7	Discussion Papers, Work Plan, TC lists	133

5.3.15.	NR Positioning Enhancements  (UID-970075) NR_pos_enh-UEConTest	133

5.3.15.1	TS 38.508-1	133

5.3.15.1.1	Test frequencies (Clause 4.3.1)	133

5.3.15.1.2	Test environment for RF (Clauses 5)	133

5.3.15.1.3	Test environment for RRM (Clause 7)	133

5.3.15.1.4	Other clauses, Annexes	133

5.3.15.2	TS 37.571-1	133

5.3.15.3	TS 37.571-3	148

5.3.15.4	TS 37.571-5	149

5.3.15.5	TR 38.903 (NR MU & TT analyses)	149

5.3.15.6	Discussion Papers, Work Plan, TC lists	151

5.3.16	NR and MR-DC measurement gap enhancements (UID-981035) NR_MG_enh-UEConTest	151

5.3.16.1	TS 38.508-1	151

5.3.16.2	TS 38.508-2	151

5.3.16.3	TS 38.522	151

5.3.16.4	TS 38.533	152

5.3.16.5	TR 38.903 (NR MU & TT analyses)	154

5.3.16.6	Discussion Papers, Work Plan, TC lists	157

5.3.17	Further Multi-RAT Dual-Connectivity enhancement (UID-991033) LTE_NR_DC_enh2-UEConTest	157

5.3.17.1	TS 38.508-1	157

5.3.17.2	TS 38.508-2	157

5.3.17.3	TS 38.522	157

5.3.17.4	TS 38.533	157

5.3.17.5	TS 36.508	157

5.3.17.6	TR 38.903 (NR MU & TT analyses)	157

5.3.17.7	Discussion Papers, Work Plan, TC lists	157

5.3.18	Additional NR bands for UL-MIMO in Rel-18 (UID-1000050) NR_bands_UL_MIMO_R18-UEConTest	157

5.3.18.1	TS 38.508-1	157

5.3.18.2	TS 38.508-2	157

5.3.18.3	TS 38.521-1	157

5.3.18.3.1	Tx Requirements (Clause 6)	157

5.3.18.3.2	Rx Requirements (Clause 7)	158

5.3.18.3.3	Clauses 1-5, Annexes	158

5.3.18.4	TS 38.522	158

5.3.18.5	TR 38.905 (NR Test Points Radio Transmission and Reception )	158

5.3.18.6	Discussion Papers, Work Plan, TC lists	158

5.3.19	High power UE (power class 2) for NR FR1 FDD single band (UID-1000051) HPUE_NR_FR1_FDD_R18-UEConTest	158

5.3.19.1	TS 38.508-1	158

5.3.19.2	TS 38.508-2	158

5.3.19.3	TS 38.521-1	158

5.3.19.3.1	Tx Requirements (Clause 6)	158

5.3.19.3.2	Rx Requirements (Clause 7)	159

5.3.19.3.3	Clauses 1-5, Annexes	159

5.3.19.4	TS 38.522	159

5.3.19.5	TR 38.905 (NR Test Points Radio Transmission and Reception )	159

5.3.19.6	Discussion Papers, Work Plan, TC lists	160

5.3.20	High power UE (power class 1.5) for NR FR1 TDD single band (UID-1000053) HPUE_NR_FR1_TDD_R18-UEConTest	160

5.3.20.1	TS 38.508-1	160

5.3.20.2	TS 38.508-2	160

5.3.20.3	TS 38.521-1	160

5.3.20.3.1	Tx Requirements (Clause 6)	160

5.3.20.3.2	Rx Requirements (Clause 7)	160

5.3.20.3.3	Clauses 1-5, Annexes	160

5.3.20.4	TS 38.522	160

5.3.20.5	TR 38.905 (NR Test Points Radio Transmission and Reception )	160

5.3.20.6	Discussion Papers, Work Plan, TC lists	161

5.3.21	Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations (UID-1000054) HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest	161

5.3.21.1	TS 38.508-1	161

5.3.21.2	TS 38.508-2	161

5.3.21.3	TS 38.521-1	162

5.3.21.3.1	Tx Requirements (Clause 6)	162

5.3.21.3.2	Rx Requirements (Clause 7)	162

5.3.21.3.3	Clauses 1-5, Annexes	162

5.3.21.4	TS 38.521-3	162

5.3.21.4.1	Tx Requirements (Clause 6)	162

5.3.21.4.2	Rx Requirements (Clause 7)	162

5.3.21.4.3	Clauses 1-5, Annexes	163

5.3.21.5	TS 38.522	163

5.3.21.6	TR 38.905 (NR Test Points Radio Transmission and Reception )	163

5.3.21.7	Discussion Papers, Work Plan, TC lists	165

5.3.22	Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL (UID-1000056) NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-UEConTest	165

5.3.22.1	TS 38.508-1	165

5.3.22.2	TS 38.508-2	165

5.3.22.3	TS 38.521-1	166

5.3.22.3.1	Tx Requirements (Clause 6)	166

5.3.22.3.2	Rx Requirements (Clause 7)	166

5.3.22.3.3	Clauses 1-5, Annexes	166

5.3.22.4	TS 38.522	166

5.3.22.5	TR 38.905 (NR Test Points Radio Transmission and Reception )	166

5.3.22.6	Discussion Papers, Work Plan, TC lists	166

5.3.23	Rel-18 NR CA and DC; and NR and LTE DC Configurations (UID-1000057) NR_CADC_NR_LTE_DC_R18-UEConTest	166

5.3.23.1	TS 38.508-1	166

5.3.23.1.1	Test frequencies (Clause 4.3.1)	166

5.3.23.1.2	Test environment for RF (Clauses 5)	166

5.3.23.1.3	Test environment for RRM (Clause 7)	166

5.3.23.1.4	Other clauses, Annexes	166

5.3.23.2	TS 38.508-2	166

5.3.23.3	TS 38.521-1	167

5.3.23.3.1	Tx Requirements (Clause 6)	167

5.3.23.3.2	Rx Requirements (Clause 7)	167

5.3.23.3.3	Clauses 1-5, Annexes	167

5.3.23.4	TS 38.521-2	167

5.3.23.4.1	Tx Requirements (Clause 6)	167

5.3.23.4.2	Rx Requirements (Clause 7)	167

5.3.23.4.3	Clauses 1-5, Annexes	167

5.3.23.5	TS 38.521-3	167

5.3.23.5.1	Tx Requirements (Clause 6)	167

5.3.23.5.2	Rx Requirements (Clause 7)	167

5.3.23.5.3	Clauses 1-5, Annexes	167

5.3.23.6	TS 38.521-4	167

5.3.23.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	167

5.3.23.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	167

5.3.23.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	167

5.3.23.6.4	Clauses 1-4, Annexes	167

5.3.23.7	TS 38.522	167

5.3.23.8	TS 38.533	168

5.3.23.9	TR 38.903 (NR MU & TT analyses)	168

5.3.23.10	TR 38.905 (NR Test Points Radio Transmission and Reception)	168

5.3.23.11	Discussion Papers, Work Plan, TC lists	168

5.3.24	New Rel-18 NR licensed bands and extension of existing NR bands (UID-1000058) NR_lic_bands_BW_R18-UEConTest	168

5.3.24.1	TS 38.508-1	168

5.3.24.1.1	Test frequencies (Clause 4.3.1)	168

5.3.24.1.2	Test environment for RF (Clauses 5)	168

5.3.24.1.3	Test environment for RRM (Clause 7)	168

5.3.24.1.4	Other clauses, Annexes	168

5.3.24.2	TS 38.508-2	168

5.3.24.3	TS 38.521-1	169

5.3.24.3.1	Tx Requirements (Clause 6)	169

5.3.24.3.2	Rx Requirements (Clause 7)	169

5.3.24.3.3	Clauses 1-5, Annexes	170

5.3.24.4	TS 38.521-2	171

5.3.24.4.1	Tx Requirements (Clause 6)	171

5.3.24.4.2	Rx Requirements (Clause 7)	171

5.3.24.4.3	Clauses 1-5, Annexes	171

5.3.24.5	TS 38.521-3	171

5.3.24.5.1	Tx Requirements (Clause 6)	171

5.3.24.5.2	Rx Requirements (Clause 7)	171

5.3.24.5.3	Clauses 1-5, Annexes	171

5.3.24.6	TS 38.521-4	171

5.3.24.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	171

5.3.24.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	171

5.3.24.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	171

5.3.24.6.4	Clauses 1-4, Annexes	171

5.3.24.7	TS 38.521-5	171

5.3.24.8	TS 38.533	173

5.3.24.9	TS 36.521-1	174

5.3.24.10	TR 38.903 (NR MU & TT analyses)	174

5.3.24.11	TR 38.905 (NR Test Points Radio Transmission and Reception)	174

5.3.24.12	Discussion Papers, Work Plan, TC lists	174

5.3.25	High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 (UID-1000059) LTE_NR_HPUE_FWVM_R18-UEConTest	174

5.3.25.1	TS 38.508-2	174

5.3.25.2	TS 38.521-1	174

5.3.25.2.1	Tx Requirements (Clause 6)	174

5.3.25.2.2	Rx Requirements (Clause 7)	176

5.3.25.2.3	Clauses 1-5, Annexes	176

5.3.25.3	TS 38.522	176

5.3.25.4	TS 36.521-1	176

5.3.25.5	TS 36.521-2	176

5.3.25.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	176

5.3.25.7	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )	176

5.3.25.8	Discussion Papers, Work Plan, TC lists	176

5.3.26	Rel-17 LTE CA Configurations (UID-1010051) LTE_CA_R17-UEConTest	176

5.3.26.1	TS 36.508	176

5.3.26.2	TS 36.521-1	176

5.3.26.3	TS 36.521-3	176

5.3.26.4	TS 36.521-2	176

5.3.26.5	TS 37.571-1	177

5.3.26.6	TS 37.571-3	177

5.3.26.7	TS 37.571-5	177

5.3.26.8	TR 36.903 (E-UTRAN RRM TT analyses)	177

5.3.26.9	TR 36.904 (E-UTRAN Radio Reception TT analyses)	177

5.3.26.10	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )	177

5.3.26.11	Discussion Papers, Work Plan, TC lists	178

5.3.27	Air-to-ground network for NR (UID-1020087) NR_ATG-UEConTest	178

5.3.27.1	TS 38.508-1	178

5.3.27.1.1	Test frequencies (Clause 4.3.1)	178

5.3.27.1.2	Test environment for RF (Clauses 5)	178

5.3.27.1.3	Test environment for RRM (Clause 7)	178

5.3.27.1.4	Other clauses, Annexes	178

5.3.27.2	TS 38.508-2	178

5.3.27.3	TS 38.521-1	178

5.3.27.3.1	Tx Requirements (Clause 6)	178

5.3.27.3.2	Rx Requirements (Clause 7)	178

5.3.27.3.3	Clauses 1-5, Annexes	179

5.3.27.4	TS 38.521-4	180

5.3.27.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	180

5.3.27.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	180

5.3.27.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	180

5.3.27.4.4	Clauses 1-4, Annexes	180

5.3.27.5	TS 38.522	180

5.3.27.6	TS 38.533	180

5.3.27.7	TR 38.903 (NR MU & TT analyses)	180

5.3.27.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	180

5.3.27.9	Discussion Papers, Work Plan, TC lists	180

5.3.28	4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC (UID-1020088) 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest	180

5.3.28.1	TS 38.508-1	180

5.3.28.2	TS 38.508-2	180

5.3.28.3	TS 38.521-1	181

5.3.28.3.1	Tx Requirements (Clause 6)	181

5.3.28.3.2	Rx Requirements (Clause 7)	183

5.3.28.3.3	Clauses 1-5, Annexes	183

5.3.28.4	TS 38.521-3	183

5.3.28.4.1	Tx Requirements (Clause 6)	183

5.3.28.4.2	Rx Requirements (Clause 7)	184

5.3.28.4.3	Clauses 1-5, Annexes	184

5.3.28.5	TS 38.522	184

5.3.28.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	184

5.3.28.7	Discussion Papers, Work Plan, TC lists	185

5.3.29	NR support for dedicated spectrum less than 5MHz for FR1 (UID-1030060) NR_FR1_lessthan_5MHz_BW-UEConTest	185

5.3.29.1	TS 38.508-1	185

5.3.29.1.1	Test frequencies (Clause 4.3.1)	185

5.3.29.1.2	Test environment for RF (Clauses 5)	186

5.3.29.1.3	Test environment for RRM (Clause 7)	186

5.3.29.1.4	Other clauses, Annexes	186

5.3.29.2	TS 38.508-2	186

5.3.29.3	TS 38.521-1	186

5.3.29.3.1	Tx Requirements (Clause 6)	186

5.3.29.3.2	Rx Requirements (Clause 7)	186

5.3.29.3.3	Clauses 1-5, Annexes	187

5.3.29.4	TS 38.521-4	188

5.3.29.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	188

5.3.29.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	188

5.3.29.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	188

5.3.29.4.4	Clauses 1-4, Annexes	188

5.3.29.5	TS 38.522	188

5.3.29.6	TS 38.533	188

5.3.29.7	TR 38.903 (NR MU & TT analyses)	188

5.3.29.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	188

5.3.29.9	Discussion Papers, Work Plan, TC lists	188

5.3.30	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR (UID-1030061) NR_DualTxRx_MUSIM-UEConTest	188

5.3.30.1	TS 38.508-1	188

5.3.30.2	TS 38.508-2	188

5.3.30.3	TS 38.509	188

5.3.30.4	TS 38.522	188

5.3.30.5	TS 38.533	188

5.3.30.6	TR 38.903 (NR MU & TT analyses)	188

5.3.30.7	Discussion Papers, Work Plan, TC lists	188

5.3.31	Enhanced support of reduced capability NR devices plus CT1 aspects (UID-1030062) NR_redcap_enh_plus_CT1-UEConTest	188

5.3.31.1	TS 38.508-1	188

5.3.31.2	TS 38.508-2	188

5.3.31.3	TS 38.521-1	188

5.3.31.3.1	Tx Requirements (Clause 6)	188

5.3.31.3.2	Rx Requirements (Clause 7)	188

5.3.31.3.3	Clauses 1-5, Annexes	188

5.3.31.4	TS 38.521-4	189

5.3.31.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	189

5.3.31.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	189

5.3.31.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	189

5.3.31.4.4	Clauses 1-4, Annexes	189

5.3.31.5	TS 38.522	189

5.3.31.6	TS 38.533	189

5.3.31.7	TR 38.903 (NR MU & TT analyses)	189

5.3.31.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	189

5.3.31.9	Discussion Papers, Work Plan, TC lists	189

5.3.32	Introduction of 900 MHz LTE band in the US (UID-1030064) LTE_900MHz_US-UEConTest	189

5.3.32.1	TS 36.508	189

5.3.32.2	TS 36.521-1	189

5.3.32.3	TS 36.521-3	190

5.3.32.4	TS 36.521-2	190

5.3.32.5	TS 38.521-1	190

5.3.32.6	Discussion Papers, Work Plan, TC lists	190

5.3.33	NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 (UID-1030065) NR_RF_FR2_req_Ph3-UEConTest	190

5.3.33.1	TS 38.508-1	190

5.3.33.2	TS 38.508-2	190

5.3.33.3	TS 38.509	190

5.3.33.4	TS 38.521-2	191

5.3.33.4.1	Tx Requirements (Clause 6)	191

5.3.33.4.2	Rx Requirements (Clause 7)	191

5.3.33.4.3	Clauses 1-5, Annexes	191

5.3.33.5	TS 38.521-3	191

5.3.33.5.1	Tx Requirements (Clause 6)	191

5.3.33.5.2	Rx Requirements (Clause 7)	192

5.3.33.5.3	Clauses 1-5, Annexes	192

5.3.33.6	TS 38.522	192

5.3.33.7	TR 38.903 (NR MU & TT analyses)	192

5.3.33.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	192

5.3.33.9	Discussion Papers, Work Plan, TC lists	192

5.3.34	Further RF requirements enhancement for NR and EN-DC in frequency range 1 (UID-1030066) NR_ENDC_RF_FR1_enh2-UEConTest	194

5.3.34.1	TS 38.508-1	194

5.3.34.2	TS 38.508-2	194

5.3.34.3	TS 38.521-1	194

5.3.34.3.1	Tx Requirements (Clause 6)	194

5.3.34.3.2	Rx Requirements (Clause 7)	194

5.3.34.3.3	Clauses 1-5, Annexes	194

5.3.34.4	TS 38.521-3	194

5.3.34.4.1	Tx Requirements (Clause 6)	194

5.3.34.4.2	Rx Requirements (Clause 7)	194

5.3.34.4.3	Clauses 1-5, Annexes	194

5.3.34.5	TS 38.521-4	194

5.3.34.5.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	194

5.3.34.5.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	194

5.3.34.5.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	194

5.3.34.5.4	Clauses 1-4, Annexes	194

5.3.34.6	TS 38.522	194

5.3.34.7	TS 38.533	194

5.3.34.8	TR 38.903 (NR MU & TT analyses)	194

5.3.34.9	TR 38.905 (NR Test Points Radio Transmission and Reception)	194

5.3.34.10	Discussion Papers, Work Plan, TC lists	194

5.3.35	Network energy savings for NR (UID-1030067) Netw_Energy_NR-UEConTest	194

5.3.35.1	TS 38.508-1	194

5.3.35.2	TS 38.508-2	194

5.3.35.3	TS 38.521-4	194

5.3.35.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	195

5.3.35.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	195

5.3.35.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	195

5.3.35.3.4	Clauses 1-4, Annexes	195

5.3.35.4	TS 38.522	195

5.3.35.5	TS 38.533	195

5.3.35.6	TR 38.903 (NR MU & TT analyses)	195

5.3.35.7	Discussion Papers, Work Plan, TC lists	195

5.3.36	Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 (UID-1030068) LTE_NR_Simult_RxTx_R18-UEConTest	195

5.3.36.1	TS 38.521-1	195

5.3.36.1.1	Tx Requirements (Clause 6)	195

5.3.36.1.2	Rx Requirements (Clause 7)	195

5.3.36.1.3	Clauses 1-5, Annexes	197

5.3.36.2	TS 38.521-3	197

5.3.36.2.1	Tx Requirements (Clause 6)	197

5.3.36.2.2	Rx Requirements (Clause 7)	197

5.3.36.2.3	Clauses 1-5, Annexes	197

5.3.36.3	TR 38.905 (NR Test Points Radio Transmission and Reception)	197

5.3.36.4	Discussion Papers, Work Plan, TC lists	198

5.3.37	Further NR coverage enhancements (UID-1030069) NR_cov_enh2-UEConTest	198

5.3.37.1	TS 38.508-1	198

5.3.37.2	TS 38.508-2	198

5.3.37.3	TS 38.521-1	198

5.3.37.3.1	Tx Requirements (Clause 6)	198

5.3.37.3.2	Rx Requirements (Clause 7)	198

5.3.37.3.3	Clauses 1-5, Annexes	198

5.3.37.4	TS 38.521-3	198

5.3.37.4.1	Tx Requirements (Clause 6)	198

5.3.37.4.2	Rx Requirements (Clause 7)	198

5.3.37.4.3	Clauses 1-5, Annexes	198

5.3.37.5	TS 38.522	198

5.3.37.6	TR 38.905 (NR Test Points Radio Transmission and Reception)	198

5.3.37.7	Discussion Papers, Work Plan, TC lists	198

5.3.38	Multi-carrier enhancements for NR (UID-1030070) NR_MC_enh-UEConTest	198

5.3.38.1	TS 38.508-1	198

5.3.38.2	TS 38.508-2	198

5.3.38.3	TS 38.521-1	198

5.3.38.3.1	Tx Requirements (Clause 6)	198

5.3.38.3.2	Rx Requirements (Clause 7)	200

5.3.38.3.3	Clauses 1-5, Annexes	200

5.3.38.4	TS 38.522	201

5.3.38.5	TS 38.533	201

5.3.38.6	TR 38.903 (NR MU & TT analyses)	203

5.3.38.7	TR 38.905 (NR Test Points Radio Transmission and Reception)	203

5.3.38.8	Discussion Papers, Work Plan, TC lists	205

5.3.39	Enhanced NR support for high speed train scenario in frequency range 2 (FR2) (UID-1030071) NR_HST_FR2_enh-UEConTest	205

5.3.39.1	TS 38.508-1	205

5.3.39.2	TS 38.508-2	205

5.3.39.3	TS 38.521-2	205

5.3.39.3.1	Tx Requirements (Clause 6)	205

5.3.39.3.2	Rx Requirements (Clause 7)	205

5.3.39.3.3	Clauses 1-5, Annexes	205

5.3.39.4	TS 38.521-4	205

5.3.39.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	205

5.3.39.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	205

5.3.39.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	205

5.3.39.4.4	Clauses 1-4, Annexes	205

5.3.39.5	TS 38.522	205

5.3.39.6	TS 38.533	205

5.3.39.7	TR 38.903 (NR MU & TT analyses)	205

5.3.39.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	205

5.3.39.9	Discussion Papers, Work Plan, TC lists	205

5.3.40	NR MIMO evolution for downlink and uplink  (FR2) (UID-1030072) NR_MIMO_evo_DL_UL-UEConTest	205

5.3.40.1	TS 38.508-1	205

5.3.40.1.1	Test frequencies (Clause 4.3.1)	205

5.3.40.1.2	Test environment for RF (Clauses 5)	205

5.3.40.1.3	Test environment for RRM (Clause 7)	205

5.3.40.1.4	Other clauses, Annexes	205

5.3.40.2	TS 38.508-2	205

5.3.40.3	TS 38.521-2	206

5.3.40.3.1	Tx Requirements (Clause 6)	206

5.3.40.3.2	Rx Requirements (Clause 7)	206

5.3.40.3.3	Clauses 1-5, Annexes	206

5.3.40.4	TS 38.521-4	206

5.3.40.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	206

5.3.40.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	206

5.3.40.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	206

5.3.40.4.4	Clauses 1-4, Annexes	206

5.3.40.5	TS 38.522	206

5.3.40.6	TS 38.533	206

5.3.40.7	TR 38.903 (NR MU & TT analyses)	206

5.3.40.8	TR 38.905 (NR Test Points Radio Transmission and Reception)	206

5.3.40.9	Discussion Papers, Work Plan, TC lists	206

5.3.41	IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects (UID-1030073) IoT_NTN_enh_plus_CT1-UEConTest	206

5.3.41.1	TS 36.508	206

5.3.41.2	TS 36.509	206

5.3.41.3	TS 36.521-3	206

5.3.41.4	TS 36.521-4	207

5.3.41.5	TS 36.521-2	208

5.3.41.6	TR 36.903 (E-UTRAN RRM TT analyses)	208

5.3.41.7	Discussion Papers, Work Plan, TC lists	208

5.4	Routine Maintenance for 5G NR only TEIx_Test	208

5.4.1	TS 38.508-1	208

5.4.1.1	Test frequencies (Clause 4.3.1)	208

5.4.1.2	Test environment for RF (Clauses 5)	210

5.4.1.3	Test environment for RRM (Clause 7)	211

5.4.1.4	Other clauses, Annexes	212

5.4.2	TS 38.508-2	213

5.4.3	TS 38.509	215

5.4.4	TS 38.521-1	215

5.4.4.1	Tx Requirements (Clause 6)	215

5.4.4.2	Rx Requirements (Clause 7)	219

5.4.4.3	Clauses 1-5, Annexes	222

5.4.5	TS 38.521-2	224

5.4.5.1	Tx Requirements (Clause 6)	224

5.4.5.2	Rx Requirements (Clause 7)	226

5.4.5.3	Clauses 1-5, Annexes	227

5.4.6	TS 38.521-3	228

5.4.6.1	Tx Requirements (Clause 6)	228

5.4.6.2	Rx Requirements (Clause 7)	231

5.4.6.3	Clauses 1-5, Annexes	232

5.4.7	TS 38.521-4	232

5.4.7.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)	232

5.4.7.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)	233

5.4.7.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)	233

5.4.7.4	Clauses 1-4, Annexes	234

5.4.8	TS 38.521-5	235

5.4.9	TS 38.522	235

5.4.10	TS 38.533	240

5.4.10.1	EN-DC with all NR cells in FR1 (Clause 4)	240

5.4.10.2	NE-DC with all NR cells in FR1 (Clause 4A)	243

5.4.10.3	EN-DC with at least 1 NR Cell in FR2 (Clause5)	243

5.4.10.4	NR Standalone in FR1 (Clause 6)	243

5.4.10.5	NR standalone with at least one NR cell in FR2 (Clause7)	245

5.4.10.6	E-UTRA – NR Inter-RAT with E-UTRA serving cell (Clause 8)	246

5.4.10.7	Clauses 1-3, Annexes	246

5.4.10.8	NR standalone tests with all NR cells in FR1 for RedCap (Clause 16)	247

5.4.10.9	NR standalone tests with all NR cells in FR2 for RedCap (Clause 17)	254

5.4.10.10	E-UTRA - NR inter-RAT with E-UTRA serving cell (Clause 18)	262

5.4.11	TS 38.551	264

5.4.12	TS 38.561	266

5.4.13	TS 36.508	268

5.4.14	TS 36.521-3	268

5.4.15	TS 37.571-1	268

5.4.16	TS 37.571-3	271

5.4.17	TS 37.571-5	271

5.4.18	TR 38.903 (NR MU & TT analyses)	271

5.4.19	TR 38.905 (NR Test Points Radio Transmission and Reception )	278

5.4.20	Discussion Papers, Work Plan, TC lists	279

5.5	Routine Maintenance for LTE only TEIx_Test	283

5.5.1	LTE RF	283

5.5.1.1	TS 36.508	283

5.5.1.2	TS 36.509	284

5.5.1.3	TS 36.521-1	284

5.5.1.3.1	Tx Requirements (Clause 6)	284

5.5.1.3.2	Rx Requirements (Clause 7)	285

5.5.1.3.3	Clauses 1-5, 8-10, Annexes	286

5.5.1.4	TS 36.521-2	286

5.5.1.5	TS 36.521-3	287

5.5.1.6	TS 36.521-4	292

5.5.1.7	RRM Test & Radio Reception Test Tolerances	295

5.5.1.7.1	TR 36.903 (E-UTRAN RRM TT analyses)	295

5.5.1.7.2	TR 36.904 (E-UTRAN Radio Reception TT analyses)	296

5.5.1.7.3	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )	296

5.5.1.8	TS 34.121-1	296

5.5.1.9	TS 34.121-2	296

5.5.1.10	TS 34.122	296

5.5.1.11	TS 34.108	296

5.5.1.12	TR 34.902 (UTRAN RRM Test Tolerance analyses)	296

5.5.1.13	Discussion Papers, Work Plan, TC lists	296

5.6	Other Routine Maintenance TEIx_Test	297

5.6.1	TS 34.108	298

5.6.2	TS 34.121-1 All sections other than annexes	298

5.6.3	TS 34.121-1 Annexes only	298

5.6.4	TS 34.121-2	298

5.6.5	TS 34.122	298

5.6.6	TS 34.171	298

5.6.7	TS 34.172	298

5.6.8	TS 34.114	298

5.6.9	TS 37.571-1	298

5.6.10	TS 37.571-3	298

5.6.11	TS 37.571-5	298

5.6.12	TS 51.010-1 (RF/Performance)	298

5.6.13	TS 51.010-2 (RF/Performance)	298

5.6.14	TS 51.010-7 (RF/Performance)	298

5.6.15	TS 37.544	298

5.6.16	TR 37.901	299

5.6.17	TR 37.901-5	299

5.6.18	TR 38.918	299

5.6.19	Discussion Papers, Work Plan, TC lists	299

5.7	Outgoing liaison statements for provisional approval	299

5.8	AOB	299

6	Signalling Protocol Functional Area	299

6.1	Review action points (fm A.I. 2.1)	299

6.2	Review incoming LS (fm A.I. 3) & new subject discussion papers	299

6.3	Open Work Items	302

6.3.1	REL-16 NR CA and DC; and NR and LTE DC Configurations (UID-830083)  NR_CADC_NR_LTE_DC_R16-UEConTest	302

6.3.1.1	TS 38.508-1	302

6.3.1.2	TS 38.508-2	302

6.3.1.3	TS 38.523-1	302

6.3.1.4	TS 38.523-2	302

6.3.1.5	TS 38.523-3	302

6.3.1.6	Discussion Papers, Work Plan, TC lists	302

6.3.2	5G V2X with NR sidelink (UID-880069)  5G_V2X_NRSL_eV2XARC-UEConTest	302

6.3.2.1	TS 38.508-1	302

6.3.2.2	TS 38.508-2	302

6.3.2.3	TS 38.509	302

6.3.2.4	TS 38.523-1	302

6.3.2.5	TS 38.523-2	304

6.3.2.6	TS 38.523-3	305

6.3.2.7	TS 36.509	305

6.3.2.8	TS 37.571-4	305

6.3.2.9	Discussion Papers, Work Plan, TC lists	305

6.3.3	Rel-17 NR CA and DC; and NR and LTE DC Configurations (UID-900056) NR_CADC_NR_LTE_DC_R17-UEConTest	305

6.3.3.1	TS 38.508-1	305

6.3.3.2	TS 38.508-2	305

6.3.3.3	TS 38.523-1	305

6.3.3.4	TS 38.523-2	305

6.3.3.5	TS 38.523-3	305

6.3.3.6	Discussion Papers, Work Plan, TC lists	305

6.3.4	NR-based access to unlicensed spectrum (UID-911003) NR_unlic-UEConTest	305

6.3.4.1	TS 38.508-1	305

6.3.4.2	TS 38.508-2	306

6.3.4.3	TS 38.509	306

6.3.4.4	TS 38.523-1	306

6.3.4.5	TS 38.523-2	306

6.3.4.6	TS 38.523-3	307

6.3.4.7	Discussion Papers, Work Plan, TC lists	307

6.3.5	NR small data transmissions in INACTIVE state (UID-960072) NR_SmallData_INACTIVE-UEConTest	307

6.3.5.1	TS 38.508-1	307

6.3.5.2	TS 38.508-2	307

6.3.5.3	TS 38.523-1	307

6.3.5.4	TS 38.523-2	309

6.3.5.5	TS 38.523-3	309

6.3.5.6	Discussion Papers, Work Plan, TC lists	309

6.3.6	Solutions for NR to support non-terrestrial networks (NTN) (UID-960074) NR_NTN_solutions_plus_CT-UEConTest	309

6.3.6.1	TS 38.508-1	309

6.3.6.2	TS 38.508-2	312

6.3.6.3	TS 38.509	312

6.3.6.4	TS 38.523-1	312

6.3.6.5	TS 38.523-2	317

6.3.6.6	TS 38.523-3	318

6.3.6.7	Discussion Papers, Work Plan, TC lists	318

6.3.7	Enhancement of Private Network Support for NG-RAN including CT aspects (UID-960076) NG_RAN_PRN_enh_plus_CT-UEConTest	319

6.3.7.1	TS 38.508-1	319

6.3.7.2	TS 38.508-2	319

6.3.7.3	TS 38.523-1	319

6.3.7.4	TS 38.523-2	321

6.3.7.5	TS 38.523-3	322

6.3.7.6	Discussion Papers, Work Plan, TC lists	322

6.3.8	Further enhancements on MIMO for NR (UID-960079) NR_feMIMO-UEConTest	322

6.3.8.1	TS 38.508-1	322

6.3.8.2	TS 38.508-2	322

6.3.8.3	TS 38.523-1	323

6.3.8.4	TS 38.523-2	323

6.3.8.5	TS 38.523-3	324

6.3.8.6	Discussion Papers, Work Plan, TC lists	324

6.3.9	NR Sidelink Relay (UID-960083) NR_SL_relay-UEConTest	324

6.3.9.1	TS 38.508-1	324

6.3.9.2	TS 38.508-2	328

6.3.9.3	TS 38.523-1	328

6.3.9.4	TS 38.523-2	328

6.3.9.5	TS 38.523-3	328

6.3.9.6	Discussion Papers, Work Plan, TC lists	328

6.3.10	NR Sidelink enhancement (UID-960084) NR_SL_enh-UEConTest	328

6.3.10.1	TS 38.508-1	328

6.3.10.2	TS 38.508-2	328

6.3.10.3	TS 38.523-1	328

6.3.10.4	TS 38.523-2	328

6.3.10.5	TS 38.523-3	328

6.3.10.6	Discussion Papers, Work Plan, TC lists	328

6.3.11	NR QoE management and optimizations for diverse services (UID-970072) NR_QoE-UEConTest	328

6.3.11.1	TS 38.508-1	328

6.3.11.2	TS 38.508-2	328

6.3.11.3	TS 38.523-1	328

6.3.11.4	TS 38.523-2	328

6.3.11.5	TS 38.523-3	328

6.3.11.6	Discussion Papers, Work Plan, TC lists	329

6.3.12	NR Positioning Enhancements (UID-970075) NR_pos_enh-UEConTest	329

6.3.12.1	TS 38.508-1	329

6.3.12.2	TS 37.571-2	329

6.3.12.3	TS 37.571-3	329

6.3.12.4	TS 37.571-4	329

6.3.12.5	TS 37.571-5	329

6.3.12.6	Discussion Papers, Work Plan, TC lists	329

6.3.13	Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData) (UID – 970077) MCProtoc16_enh2MCPTT_eMCData2-ConTest	329

6.3.13.1	TS 36.579-1	329

6.3.13.2	TS 36.579-2	329

6.3.13.3	TS 36.579-3	329

6.3.13.4	TS 36.579-4	329

6.3.13.5	TS 36.579-5	329

6.3.13.6	TS 36.579-6	329

6.3.13.7	TS 36.579-7	330

6.3.13.8	TS 36.579-8 (pCRs only)	332

6.3.13.9	TS 36.579-9 (pCRs only)	332

6.3.13.10	Discussion Papers, Work Plan, TC lists	332

6.3.14	Further Multi-RAT Dual-Connectivity enhancement (UID-991033) LTE_NR_DC_enh2-UEConTest	333

6.3.14.1	TS 38.508-1	333

6.3.14.2	TS 38.508-2	334

6.3.14.3	TS 38.523-1	334

6.3.14.4	TS 38.523-2	336

6.3.14.5	TS 38.523-3	336

6.3.14.6	TS 36.508	336

6.3.14.7	Discussion Papers, Work Plan, TC lists	336

6.3.15	Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking (UID-991034) ID_UAS-UEConTest	336

6.3.15.1	TS 38.508-1	336

6.3.15.2	TS 38.508-2	337

6.3.15.3	TS 38.523-1	337

6.3.15.4	TS 38.523-2	338

6.3.15.5	TS 38.523-3	338

6.3.15.6	TS 36.508	338

6.3.15.7	TS 36.523-1	338

6.3.15.8	TS 36.523-2	338

6.3.15.9	TS 36.523-3	338

6.3.15.10	Discussion Papers, Work Plan, TC lists	339

6.3.16	IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS (UID-1000052) ING_5GS-UEConTest	339

6.3.16.1	TS 38.508-2	339

6.3.16.2	TS 38.523-1	339

6.3.16.3	TS 38.523-2	339

6.3.16.4	TS 38.523-3	340

6.3.16.5	Discussion Papers, Work Plan, TC lists	340

6.3.17	IMS Data Channel (UID-1000055) IMSProtoc17_dataCH-UEContest	340

6.3.17.1	TS 36.508	340

6.3.17.2	TS 36.509	340

6.3.17.3	TS 38.508-1	340

6.3.17.4	TS 38.509	340

6.3.17.5	TS 34.229-1	340

6.3.17.6	TS 34.229-2	340

6.3.17.7	TS 34.229-3	340

6.3.17.8	TS 34.229-5	340

6.3.17.9	Discussion Papers, Work Plan, TC lists	341

6.3.18	Rel-18 NR CA and DC; and NR and LTE DC Configurations (UID-1000057) NR_CADC_NR_LTE_DC_R18-UEConTest	341

6.3.18.1	TS 38.508-1	341

6.3.18.2	TS 38.508-2	341

6.3.18.3	TS 38.523-1	341

6.3.18.4	TS 38.523-2	341

6.3.18.5	TS 38.523-3	341

6.3.18.6	Discussion Papers, Work Plan, TC lists	341

6.3.19	New Rel-18 NR licensed bands and extension of existing NR bands (UID-1000058) NR_lic_bands_BW_R18-UEConTest	341

6.3.19.1	TS 38.523-3	341

6.3.19.2	Discussion Papers, Work Plan, TC lists	341

6.3.19.3	TS 38.508-1	341

6.3.20	Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 (UID-1010050) ATSSS_Ph2-UEConTest	342

6.3.20.1	TS 38.508-1	342

6.3.20.2	TS 38.508-2	342

6.3.20.3	TS 38.523-1	342

6.3.20.4	TS 38.523-2	345

6.3.20.5	TS 38.523-3	346

6.3.20.6	Discussion Papers, Work Plan, TC lists	346

6.3.21	Rel-17 LTE CA Configurations (UID-1010051) LTE_CA_R17-UEConTest	346

6.3.21.1	TS 36.508	346

6.3.21.2	TS 36.523-1	346

6.3.21.3	TS 36.523-2	346

6.3.21.4	TS 36.523-3	346

6.3.21.5	Discussion Papers, Work Plan, TC lists	346

6.3.22	Air-to-ground network for NR (UID-1020087) NR_ATG-UEConTest	346

6.3.22.1	TS 38.508-1	346

6.3.22.2	TS 38.508-2	346

6.3.22.3	TS 38.523-1	346

6.3.22.4	TS 38.523-2	346

6.3.22.5	TS 38.523-3	346

6.3.22.6	Discussion Papers, Work Plan, TC lists	346

6.3.23	NR support for dedicated spectrum less than 5MHz for FR1 (UID-1030060) NR_FR1_lessthan_5MHz_BW-UEConTest	346

6.3.23.1	TS 38.508-1	346

6.3.23.2	TS 38.508-2	346

6.3.23.3	TS 38.523-1	346

6.3.23.4	TS 38.523-2	346

6.3.23.5	TS 38.523-3	346

6.3.23.6	Discussion Papers, Work Plan, TC lists	346

6.3.24	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR (UID-1030061) NR_DualTxRx_MUSIM-UEConTest	346

6.3.24.1	TS 38.508-1	346

6.3.24.2	TS 38.508-2	346

6.3.24.3	TS 38.509	347

6.3.24.4	TS 38.523-1	347

6.3.24.5	TS 38.523-2	347

6.3.24.6	TS 38.523-3	347

6.3.24.7	Discussion Papers, Work Plan, TC lists	347

6.3.25	Enhanced support of reduced capability NR devices plus CT1 aspects (UID-1030062) NR_redcap_enh_plus_CT1-UEConTest	347

6.3.25.1	TS 38.508-1	347

6.3.25.2	TS 38.508-2	347

6.3.25.3	TS 38.523-1	347

6.3.25.4	TS 38.523-2	347

6.3.25.5	TS 38.523-3	347

6.3.25.6	TS 34.229-2	347

6.3.25.7	Discussion Papers, Work Plan, TC lists	347

6.3.26	XR (eXtended Reality) enhancements for NR plus CT1 aspects (UID-1030063) NR_XR_enh_plus_CT1-UEConTest	347

6.3.26.1	TS 38.508-1	347

6.3.26.2	TS 38.508-2	347

6.3.26.3	TS 38.509	347

6.3.26.4	TS 38.523-1	347

6.3.26.5	TS 38.523-2	347

6.3.26.6	TS 38.523-3	347

6.3.26.7	Discussion Papers, Work Plan, TC lists	347

6.3.27	Introduction of 900 MHz LTE band in the US (UID-1030064) LTE_900MHz_US-UEConTest	347

6.3.27.1	TS 36.508	347

6.3.27.2	TS 36.523-1	348

6.3.27.3	TS 36.523-2	348

6.3.27.4	TS 36.523-3	349

6.3.27.5	Discussion Papers, Work Plan, TC lists	349

6.3.28	Further RF requirements enhancement for NR and EN-DC in frequency range 1 (UID-1030066) NR_ENDC_RF_FR1_enh2-UEConTest	349

6.3.28.1	TS 38.508-1	349

6.3.28.2	TS 38.508-2	349

6.3.28.3	TS 38.523-1	349

6.3.28.4	Discussion Papers, Work Plan, TC lists	349

6.3.29	Network energy savings for NR (UID-1030067) Netw_Energy_NR-UEConTest	349

6.3.29.1	TS 38.508-1	349

6.3.29.2	TS 38.508-2	349

6.3.29.3	TS 38.523-1	349

6.3.29.4	TS 38.523-2	349

6.3.29.5	TS 38.523-3	349

6.3.29.6	Discussion Papers, Work Plan, TC lists	349

6.3.30	Further NR coverage enhancements (UID-1030069) NR_cov_enh2-UEConTest	349

6.3.30.1	TS 38.508-1	349

6.3.30.2	TS 38.508-2	349

6.3.30.3	TS 38.523-1	349

6.3.30.4	TS 38.523-2	349

6.3.30.5	TS 38.523-3	349

6.3.30.6	Discussion Papers, Work Plan, TC lists	349

6.3.31	Multi-carrier enhancements for NR (UID-1030070) NR_MC_enh-UEConTest	349

6.3.31.1	TS 38.508-1	349

6.3.31.2	TS 38.508-2	349

6.3.31.3	TS 38.523-1	350

6.3.31.4	TS 38.523-2	350

6.3.31.5	TS 38.523-3	350

6.3.31.6	Discussion Papers, Work Plan, TC lists	350

6.3.32	Enhanced NR support for high speed train scenario in frequency range 2 (FR2) (UID-1030071) NR_HST_FR2_enh-UEConTest	350

6.3.32.1	TS 38.508-1	350

6.3.32.2	TS 38.508-2	350

6.3.32.3	TS 38.523-1	350

6.3.32.4	TS 38.523-2	350

6.3.32.5	TS 38.523-3	350

6.3.32.6	Discussion Papers, Work Plan, TC lists	350

6.3.33	NR MIMO evolution for downlink and uplink  (FR2) (UID-1030072) NR_MIMO_evo_DL_UL-UEConTest	350

6.3.33.1	TS 38.508-1	350

6.3.33.2	TS 38.508-2	350

6.3.33.3	TS 38.523-1	350

6.3.33.4	TS 38.523-2	350

6.3.33.5	TS 38.523-3	350

6.3.33.6	Discussion Papers, Work Plan, TC lists	350

6.3.34	IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects (UID-1030073) IoT_NTN_enh_plus_CT1-UEConTest	350

6.3.34.1	TS 36.508	350

6.3.34.2	TS 36.509	350

6.3.34.3	TS 36.523-1	350

6.3.34.4	TS 36.523-2	350

6.3.34.5	TS 36.523-3	351

6.3.34.6	Discussion Papers, Work Plan, TC lists	351

6.4	Routine Maintenance for TS 38 Series TEIx_Test	351

6.4.1	TS 38.508-1	351

6.4.1.1	Generic Procedures and Test Procedures (Clauses 4.5, 4.5A & 4.9)	351

6.4.1.2	Default NG-RAN RRC messages and IEs (Clause 4.6)	352

6.4.1.3	Default 5GC NAS messages and IEs (Clause 4.7)	359

6.4.1.4	Test environment for SIG (Clause 6)	359

6.4.1.5	Other clauses, Annexes	359

6.4.2	TS 38.508-2	360

6.4.3	TS 38.509	362

6.4.4	TS 38.523-1	362

6.4.4.1	Clauses 1 - 5	362

6.4.4.2	Idle Mode (Clause 6)	362

6.4.4.3	Layer 2	366

6.4.4.3.1	NR Layer 2	366

6.4.4.3.1.1	Common Test Case Specific Values for Layer 2 (Clause 7.1.0)	366

6.4.4.3.1.2	MAC	366

6.4.4.3.1.3	RLC	368

6.4.4.3.1.4	PDCP	368

6.4.4.3.1.5	SDAP	369

6.4.4.4	RRC	369

6.4.4.4.1	NR RRC	369

6.4.4.4.1.1	RRC Connection Management Procedures (clause 8.1.1)	369

6.4.4.4.1.2	RRC Reconfiguration (clause 8.1.2)	370

6.4.4.4.1.3	RRC Measurement Configuration Control and Reporting (clause 8.1.3)	371

6.4.4.4.1.4	RRC Handover (clause 8.1.4)	371

6.4.4.4.1.5	RRC Others (clause 8.1.5)	371

6.4.4.4.1.6	RRC SON and MDT support for NR (clause 8.1.6)	373

6.4.4.4.1.7	RRC Non-public networks (clause 8.1.7)	376

6.4.4.4.2	MR-DC RRC	376

6.4.4.4.2.1	RRC UE Capability / Others (clause 8.2.1)	376

6.4.4.4.2.2	RRC Radio Bearer (clause 8.2.2)	376

6.4.4.4.2.3	RRC Measurement / Handovers (clause 8.2.3)	377

6.4.4.4.2.4	RRC Carrier Aggregation (clause 8.2.4)	379

6.4.4.4.2.5	RRC Reconfiguration / Radio Link Failure (clause 8.2.5)	379

6.4.4.4.2.6	RRC Others (clause 8.2.6)	379

6.4.4.4.2.7	RRC Resume (clause 8.2.7)	379

6.4.4.5	5GS Mobility Management	379

6.4.4.5.1	MM Primary authentication and key agreement (clause 9.1.1)	379

6.4.4.5.2	MM Security mode control, Identification & Generic UE configuration update (clauses 9.1.2, 9.1.3 & 9.1.4)	379

6.4.4.5.3	MM Registration & De-registration (clauses 9.1.5 & 9.1.6)	379

6.4.4.5.4	MM Service Request (clause 9.1.7)	380

6.4.4.5.5	MM SMS Over NAS (clause 9.1.8)	380

6.4.4.5.6	RACS (clause 9.1.9)	380

6.4.4.5.7	MM Network slice-specific authentication and authorization (clause 9.1.10)	380

6.4.4.5.8	MM SNPN (clause 9.1.11)	380

6.4.4.5.9	MM NSAC/NSSRG (clauses 9.1.12 & 9.1.13)	380

6.4.4.5.10	MM Paging Early Indication (clause 9.1.14)	381

6.4.4.6	5GS Non-3GPP Access Mobility Management (clause 9.2)	381

6.4.4.7	5GS Inter-system Mobility (clause 9.3)	381

6.4.4.8	5GS Session Management	381

6.4.4.8.1	SM PDU session authentication and authorization (clause 10.1.1)	381

6.4.4.8.2	SM Network-requested PDU session modification & release (clauses 10.1.2 & 10.1.3)	381

6.4.4.8.3	SM UE-requested PDU session establishment, modification & release (clauses 10.1.4, 10.1.5 & 10.1.6)	381

6.4.4.8.4	SM NSAC (clauses 10.1.8)	381

6.4.4.9	EN-DC Session Management (clause 10.2)	382

6.4.4.10	5GS Non-3GPP Access & ATSS Session Management (clauses 10.3 & 10.4)	382

6.4.4.11	5GS Multilayer and Services	382

6.4.4.11.1	EPS Fallback (clause 11.1)	382

6.4.4.11.2	5G-SRVCC (clause 11.2)	383

6.4.4.11.3	Unified Access Control (UAC) (clause 11.3)	383

6.4.4.11.4	Emergency Services (clause 11.4)	384

6.4.4.11.5	eCall over IMS (clause 11.5)	384

6.4.4.11.6	3GPP PS Data Off (clause 11.6)	385

6.4.4.11.7	eDRX (clause 11.7)	385

6.4.4.11.8	Inter-system mobility between untrusted Non-3GPP and 3GPP system (clause 11.8)	385

6.4.4.11.9	MBS (clause 14)	385

6.4.5	TS 38.523-2	386

6.4.6	TS 38.523-3	390

6.4.7	Discussion Papers, Work Plan, TC lists	391

6.5	Routine Maintenance for TS 36 Series TEIx_Test	395

6.5.1	Routine Maintenance for TS 36.508	395

6.5.2	Routine Maintenance for TS 36.509	396

6.5.3	Routine Maintenance for TS 36.523-1	396

6.5.3.1	Idle Mode	396

6.5.3.2	Layer 2	397

6.5.3.2.1	MAC	397

6.5.3.2.2	RLC	397

6.5.3.2.3	PDCP	397

6.5.3.3	RRC	397

6.5.3.3.1	RRC Part 1 (clauses 8.1 and 8.5)	397

6.5.3.3.2	RRC Part 2 (clause 8.2),	398

6.5.3.3.3	RRC Part 3 (clause 8.3)	398

6.5.3.3.4	Inter-RAT (clauses 8.4 & 8.4A)	398

6.5.3.3.5	RRC LTE MDT (clause 8.6)	398

6.5.3.3.6	RRC ANR for UTRAN (clause 8.7)	398

6.5.3.4	EPS Mobility Management	398

6.5.3.5	EPS Session Management	399

6.5.3.6	General Tests	399

6.5.3.7	Interoperability Radio Bearers	400

6.5.3.8	Multilayer Procedures	400

6.5.3.9	PWS - ETWS, CMAS	400

6.5.3.10	Non-3GPP	400

6.5.3.11	Others (TS 36.523-1 clauses not covered by other AIs under AI 6.5.3, e.g. eMBMS, Home (e)NB, MBMS in LTE, D2D, SC-PTM, NB-IoT, CIoT...)	400

6.5.4	Routine Maintenance for TS 36.523-2	403

6.5.5	Routine Maintenance for TS 36.523-3	405

6.5.6	Discussion Papers, Work Plan, TC lists	405

6.6	Other Maintenance TEIx_Test	406

6.6.1	Routine Maintenance for TS 34.108	406

6.6.2	Routine Maintenance for TS 34.109	406

6.6.3	Routine Maintenance for TS 34.123	406

6.6.3.1	TS 34.123-1	406

6.6.3.2	TS 34.123-2	406

6.6.3.3	TS 34.123-3	406

6.6.4	Discussion Papers, Work Plan, TC lists	407

6.6.6	Routine Maintenance for TS 34.229	407

6.6.6.1	TS 34.229-1	407

6.6.6.2	TS 34.229-2	409

6.6.6.3	TS 34.229-3	409

6.6.6.4	TS 34.229-4	409

6.6.6.5	TS 34.229-5	409

6.6.6.6	Discussion Papers, Work Plan, TC lists	413

6.6.7	Routine Maintenance for TS 37.571	413

6.6.7.1	TS 37.571-2	413

6.6.7.2	TS 37.571-3	414

6.6.7.3	TS 37.571-4	414

6.6.7.4	TS 37.571-5	414

6.6.7.5	Discussion Papers, Work Plan, TC lists	414

6.6.8	Routine Maintenance for TS 51.010	414

6.6.8.1	TS 51.010-1 (Signalling)	414

6.6.8.2	TS 51.010-2 (Signalling)	414

6.6.8.3	TS 51.010-5 (Signalling)	414

6.6.8.4	TS 51.010-7 (Signalling)	414

6.6.8.5	Discussion Papers, Work Plan, TC list & CR summary	414

6.6.9	Routine Maintenance for TS 36.579	414

6.6.9.1	TS 36.579-1	414

6.6.9.2	TS 36.579-2	417

6.6.9.3	TS 36.579-3	418

6.6.9.4	TS 36.579-4	418

6.6.9.5	TS 36.579-5	418

6.6.9.6	TS 36.579-6	419

6.6.9.7	TS 36.579-7	419

6.6.9.8	Other Specs	420

6.6.9.9	Discussion Papers, Work Plan, TC lists	420

6.7	Outgoing liaison statements for provisional approval	420

6.8	AOB	420

7	Closing Joint Session	420

7.1	Agenda for closing session	420

7.2	Pointer CRs	420

7.3	Open Issues	425

7.3.1	RF group docs still requiring WG verdict/confirmation - original A.I. retained	425

7.3.2	Sig group docs still requiring WG verdict/confirmation - original A.I. retained	425

7.3.3	Other open issues from joint sessions - original A.I. retained	425

7.3.4	Other	425

7.4	iWD/PRD Updates	425

7.4.1	iWD-003: Record of RAN5 owned test cases not ready for RAN5 agreement or verifiable on one UE only	425

7.4.2	PRD17: Guidance to using Work Item Codes with RAN5 test cases	425

7.4.3	PRD20: Status updates E-UTRA CA	425

7.4.4	PRD21: Status Updates and Completion Declaration Statements (CDS) for NR bands, NR band CBW extensions, 5G NR CADC configurations for all Power Classes	426

7.4.5	Other PRD updates	430

7.5	Work Items/ Study Items	430

7.5.1	Final version of Work Item Proposals	430

7.5.2	Active Work Items/ Study Item: work plans (wp), status reports (sr), Work Item Descriptions (wid)	434

7.5.3	Work Plan updates of recently closed work items	449

7.6	Docs still needing agreement/endorsement/approval (e.g. Outgoing LS, Reports, New Specs, Info for certification bodies etc.)	450

7.7	Confirmation of Future RAN5 Matters	451

7.8	AOB	452

Annex A: Contribution documents and status	453

A1: List of TDocs	453

Annex B: List of change requests	501

Annex C: Lists of liaisons	546

C1: Incoming liaison statements	546

C2: Outgoing liaison statements	546

Annex D: List of agreed/approved new and revised Work Items	547

Annex E: List of draft Technical Specifications and Reports	548

Annex F: List of action items	549

Joint:		549

Action Points at RAN5#103	549

SIG:		549

Action Points at RAN5#103	549

Action Points at RAN5#102	550

Action Points at RAN5#101	550

RF:		551

Action Points at RAN5#103	551

Action Points at RAN5#102	551

Action Points at RAN5#101	551

Action Points at RAN5#99	552

Action Points at RAN5#97	552

Annex G: Void	553

Annex H: List of participants	554

Annex I: List of future meetings	561

## 1	Opening of the meeting

### 1.1	Welcoming brief by the host

R5-242100	Agenda - opening session

Type: agenda		For: Information
					Source: WG Chairman

Decision: 		The document was revised to R5-243354.

R5-242103	RAN5#103 Session Programme

Type: agenda		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-243354	Agenda - opening session

Type: agenda		For: Information
					Source: WG Chairman

(Replaces R5-242100)

Abstract:

correction 5.3.28.5.3 -> 5.3.28.4.3

Discussion:

The RAN5 Chair Mr. Jacob John welcomed all delegates to the RAN5#103 meeting in Fukuoka, Japan.

NTT Docomo welcomed the delegates to the venue and explained the meeting practicalities.

Then the RAN5 Chair reminded:

Reminder for IPR declaration

I draw your attention to your obligations under the 3GPP Partner Organizations’ IPR policies. Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

Delegates are asked to take note that they are thereby invited:

•	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

•	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms http://webapp.etsi.org/Ipr/).

Antitrust Guidance

“I also draw your attention to the fact that 3GPP activities are subject to all applicable antitrust and competition laws and that compliance with said laws is therefore required of any participant of this TSG/WG meeting including the Chairman and Vice Chairman. In case of question I recommend that you contact your legal counsel.

The leadership shall conduct the present meeting with impartiality and in the interests of 3GPP.

Furthermore, I would like to remind you that timely submission of work items in advance of TSG/WG meetings is important to allow for full and fair consideration of such matters.”

http://www.3gpp.org/about-3gpp/legal-matters/21-3gpp-calendar/1616-statement-of-antitrust-compliance

Then the new delgates from Samsung, Ericsson, KDDI, Rakuten Mobile introduced themselves.

Decision: 		The document was approved.

## 2	Reports

### 2.1	Live Reports

R5-242104	RAN5 Leadership Team

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-242105	RAN5#102 WG Minutes

Type: report		For: Approval
					Source: ETSI Secretariat

Decision: 		The document was approved.

R5-242106	RAN5#102 WG Action Points

Type: report		For: Information
					Source: ETSI Secretariat

Decision: 		The document was noted.

R5-242107	Latest RAN Plenary notes

Type: report		For: Information
					Source: WG Chairman

Discussion:

- RAN5 Chair: pls, take care to submit integer final documents which can be opened.

- Pls. submit the revised WIDs to RAN5 first, before sending them to the RAN plenary. They have to be aligned with the status reports.

Decision: 		The document was noted.

R5-242108	Latest RAN Plenary draft Report

Type: report		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-242109	Post Plenary Active Work Item update

Type: other		For: Information
					Source: ETSI Secretariat

Decision: 		The document was noted.

R5-242308	MCC TF160 Status Report

Type: report		For: Approval
					Source: MCC TF160

Abstract:

Test models & system interface design
Progress at TTCN Workshop #65 (18th Apr’24):

Test Models – 5G:

Rel-17:

NR NTN: endorsed Test Model & ASP updates to support GSO scenario.

NR SDT: endorsed Test Model updates to support RA-SDT and CG-SDT.

NR QoE: endorsed Test Model & ASP updates to support SRB4.

Prose CRs to TS 38.523-3 submitted at RAN5#103 for the above.

Test Models – NB-IoT:

Rel-17 NTN: Endorsed Test Model & ASP updates to support NGSO scenario.

Prose CR to TS 36.523-3 submitted at RAN5#103 for the above.

TTCN development 
Progress for period: Feb to May’24:

Completed:

5G Rel-16:

NR MIMO enhancements

5G Rel-17:

RedCap

EN-DC user-plane integrity protection (UPIP)

NB-IoT:

Rel-17 non-terrestrial networks (NTN) GSO

LTE:

Rel-11 IMS emergency call in ATTACH-NEEDED state

Rel-16 redirection with MPS indication

Progressed:

5G Rel-15:

NE-DC

5G Rel-16:

V2X with NR sidelink

5G Rel-17:

Non-public networks (NPN) enhancements

Mission Critical:

Rel-16 MCVideo / MCData over LTE

Started:

5G Rel-16:

NR unlicensed (NR-U)

5G Rel-17:

NR SDT in INACTIVE state

NR coverage enhancements

MR-DC further enhancements

LTE:

Rel-14 network policy

Rel-17 UAS connectivity, identification, tracking

TTCN funding 2024

Status:

2024 workload is estimated at 101 person-months (pm), see previous slides.

PCG#51/OP#50 approved the 3GPP funding of 58 pm for 2024 TTCN tasks.

CTIA/PTCRB and GCF have agreed to continue TF160 financial support in 2024.

Current commitments for 2024 are as follows:

3GPP 	58

GCF	 12

CTIA/PTCRB	 6

Qualcomm	 2

Total: 	78

3GPP companies / 3GPP MRPs committed to provide 19 pm as voluntary contributions for 2024 TTCN development.

Total resources of 97 pm  estimated funding gap of 4 pm.

TTCN deliveries and baseline
2024 schedule:

One TTCN-2 full delivery (FDD) and five TTCN-3 full deliveries.

Type definitions baseline upgrade to 3GPP Release 18 required in 2024:

Rel-18 Stage3 freeze declared by 3GPP in March 2024.

Rel-18 ASN.1 freeze planned by 3GPP in June 2024.

TTCN workplan: 2025 early candidates
WIs with no 2024 budget, candidate for 2025 budgeting:

LTE Band 106 (3 MHz BW)

Air-to-ground network for NR

NR MIMO evolution for DL & UL

NR FR1 less than 5MHz BW

Enhanced NR support for high-speed train scenario in FR2

MCX Client – Rel-17 enhancements

NR MT SDT

In-Device Co-existence (IDC) enhancements for NR and MR-DC

Decision: 		The document was revised to R5-243450.

### 2.2	General Reports for information

R5-242110	RAN5 SR to RP#103

Type: report		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-242111	TF160 SR to RP#103

Type: report		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-242617	GCF 3GPP TCL after GCF CAG#78

Type: report		For: Information
					Source: Ericsson

Abstract:

At the GCF CAG#78 meeting, April 2024, the GCF 3GPP Test Case List (TCL) was produced in [1]. The document includes the list of GCF selected 3GPP test cases and priorities as the outcome of GCF CAG#78.

The purpose of this document is to inform RAN5 about the changes introduced in GCF work items related to 3GPP test cases.

Summary of changes to GCF selected 3GPP test cases:

•	19 new Test Cases have been selected by GCF.

•	71 Test Cases have been removed by GCF.

•	52 Test Cases have changed GCF priority.

•	No change of GCF Work Item assignments for current selected Test Cases.

•	21 Test Cases have changed description in the GCF Test Case database.

Attachment:

[1]	CAG-24-139	GCF 3GPP TCL after CAG_78 CC v3.93.0 vs v3.92.0

Decision: 		The document was noted.

## 3	Incoming Liaison Statements

R5-242119	LS on Head Phantoms for XR devices OTA testing

Type: LS in		For: Information
					Original outgoing LS: R4-2406085, to CTIA OTA, cc TSG RAN
					Source: TSG WG RAN4

Abstract:

3GPP RAN Plenary approved the new Rel-19 OTA WI [1] in the most recent RAN#103 meeting held in March 2024. The workplan for WI is presented in [2]. One of the objectives of this WI is to define FR1 OTA test methodology for non-RedCap headworn eXtended Reality (XR) devices, i.e.,

•	Define test methodology for FR1 non-RedCap headworn XR devices

o	Define TRP and TRS test methodology and configuration

	The performance metric of XR (1Tx and2Tx) is aligned with the definition up to Rel-18 handheld UE

	Testing time reduction solutions can be considered (further measurement grid optimization is precluded)

o	Study and specify the proper head phantom if needed, consider the coordination with CTIA on this aspect

o	Develop preliminary Measurement Uncertainty (MU)  (RAN5)

3GPP truly appreciates previous well-coordinated efforts with CTIA on OTA phantoms, specifically the allowance to reproduce figures and OTA Phantom MU assessed values from the respective CTIA test plans. Given the need to test headworn XR devices on an OTA phantom, 3GPP would like to collaborate with CTIA on the development of suitable XR phantoms and corresponding OTA test methods. It is our understanding that preliminary findings on the AR/VR OTA capable phantoms was presented in CTIA.

3GPP RAN4 would like to encourage CTIA to expedite the work (e.g., development, manufacturing) on applicable XR phantoms and corresponding OTA test methods. The proposed WI time scale anticipates the conclusion of the core part by September 2025 and the performance part by March 2026 for which commercially available XR phantoms are required as 3GPP OTA performance requirements are derived based on actual measurement campaigns. As such, 3GPP RAN4 requests feedback whether the work on applicable XR phantoms can be expedited with such phantoms being specified and commercially available by April 2025 the latest.

3GPP RAN4 would highly appreciate if CTIA can consider above information for the work on OTA head phantom suitable for XR and provide feedback on the timelines.

2	Actions

To CTIA:

ACTION: 	3GPP RAN4 respectfully asks CTIA to share feedback on the above information and encourages CTIA to further collaborate with 3GPP RAN4 on XR phantom development and corresponding test methods including positioning guidelines and measurement uncertainty.

Discussion:

presented by Keysight USA.

Decision: 		The document was noted.

R5-242244	LS RAN4 on 5G NR FR1 OTA Technical Specifications

Type: LS in		For: Information
					Original outgoing LS: CAG-24-217r3, to TSG WG RAN5, cc TSG WG RAN5, TSG RAN, MSG TFES, GCF SG, CTIA OTA WG, GSMA TSG-AP
					Source: GCF CAG

Abstract:

GCF has approved a work item for 5G NR FR1 OTA antenna conformance testing based on 3GPP TS.38.561 and 3GPP TS38.161.

However, GCF did find the test scope within TS38.161 and TS38.561 very limited. As so far only two bands have been defined in comparison

Discussion:

presented by Apple USA.

The RAN5 Vice Chair RF wondered whther CTIA is covering these new NR FR1 bands as well.

This was confirmed by the RAN5 Chair.

Decision: 		The document was noted.

R5-243449	LS regarding the development of NB-IoT test cases for Release 15 and Release 16

Type: LS in		For: Action
					Original outgoing LS: TSGIoT66a_003v3, to -, cc -
					Source: GSMA TSGIoT

Discussion:

RAN5 Chair: Rel-15 and 16 is not NTN, which is Rel-17.

Not sure why GSMA is involved.

Deferred.

Presented on Wed by Novamint in the joint. GSMA representatives were present. The LS should have gone to GCF FTAG, not CAG. No duplication of RAN5 work is intended.

IF there is a need for testing in Rel-15 and -16, we have to discover what is the need?

RAN5 Chair: for Rel-15 and -16 there was no commercial interest so far in RAN5.

RAN5 is happy to fill the test gap, following the 3GPP process, whether to start a new WI, or to do it under maintenance.

A response LS will be drafted.

Decision: 		The document was noted.

## 4	RAN5 General Issues

### 4.1	New Work Item proposals - for intro only

R5-242124	New WID on UE Conformance - Further NR mobility enhancements

Type: WID new		For: Endorsement
					Source: MediaTek Inc., Apple, China Telecom

Abstract:

Re-submission of R5-237307 as it was for information only.

Decision: 		The document was revised to R5-243451.

R5-242125	New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation

Type: WID new		For: Endorsement
					Source: MediaTek Inc., China Telecom

Abstract:

Re-submission of R5-241460 as it was not submitted for RANP #103.

Decision: 		The document was revised to R5-243452.

R5-242126	New WID on UE Conformance - Introduction of the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) for IoT NTN

Type: WID new		For: Endorsement
					Source: MediaTek Inc., Inmarsat

Abstract:

To address various IoT demands and enable IoT operation in an NTN, the ongoing Rel-18 WI on IoT NTN finally adds the missing RAN4 piece by specifying RAN4 core and performance requirements in addition to the related RAN1/2/3 work carried out in Rel-17, and these Rel-18 requirements will be applicable to the Rel-17 specifications for NB-IoT and eMTC operation over NTN in a release dependence manner. The WI specifies two new FDD bands NB-IoT/eMTC NTN operation by leveraging the studies and requirements of NR NTN bands n255 and n256:

o	S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

o	L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

Recently some demands for new IoT NTN bands are emerging, for example, a new E-UTRA NTN FDD band, with UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz. This FDD band 253 is Release-independent from Rel-17 for the IoT NTN UE.

The Rel-18 WI IoT_NTN_extLband-Core has been 100% completed at RAN#102 meeting (Dec. 2023) and the IoT_NTN_extLband-Perf will be completed by Jun. 2024. According to the requirements of  IoT NTN bands, it is justified now to start the work on the corresponding UE conformance test specifications for Extended-L band for IoT NTN operation in 3GPP RAN WG5.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to IoT_NTN_extLband, by analysing the test case impact, applicability and test environment, and updating the relevant conformance specifications. This work item will cover Signalling, RF, Demod and RRM conformance test specifications for FDD IoT NTN band 253.

Decision: 		The document was revised to R5-243453.

R5-242239	New WID on UE Conformance Test Aspects - Expanded and improved NR positioning

Type: WID new		For: Endorsement
					Source: CATT, CAICT

Abstract:

The objective of the parent work item is to specify solutions for support of sidelink ranging/positioning, integrity for RAT-dependent positioning methods, LPHAP (Low Power High Accuracy Positioning), bandwidth aggregation for positioning measurements, DL and UL carrier phase positioning, and to introduce support of positioning for RedCap UEs.

The specification work in the parent work item except the performance parts is finished. The overall completion level for performance part is 35% at RP#103 (March-2024). There is a need to introduce an associated RAN5 work item to enable UE conformance testing for expanded and improved NR positioning.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to the WID on Expanded and Improved NR Positioning completed by RAN1, RAN2 and RAN4. This work item will cover signalling and RRM conformance test specifications aspects for NR positioning enhancements including the test applicabilities, test environment, initial conditions, test procedures, message exceptions, test setup, connection diagram(s), etc.

Discussion:

r1

+Ericsson

RAN5 Chair: if all specs will only complete in 2026 , TTCN will only come in 2027.

Decision: 		The document was revised to R5-243454.

R5-242446	New WID on UE Conformance – NR demodulation performance evolution

Type: WID new		For: Endorsement
					Source: China Telecom, Qualcomm, Samsung

Abstract:

In Rel-18, RAN4 has introduced performance requirements for more advanced UE receiver for MU-MIMO scenario to further improve the performance with advanced receiver to cancel inter-user interference. As an outcome, in the Core part, the network assistant signalling the new DCI and RRC based network assistant signalling is introduced and the corresponding basic UE feature design is captured. And the UE demodulation requirements for R-ML receiver for MU-MIMO is introduced in TS38.101-4 in the Perf part.

In Rel-17, RAN4 concluded the feasibility of defining absolute physical layer throughput requirements with link adaptation under RAN5 Rel-16 SI FS_UE_5GNR_App_Data_Perf. Operators think it’s beneficial to specify absolute physical layer throughput requirements with link adaptation in Rel-18 taking the study outcome of Rel-17. As a result, the absolute physical layer throughput requirements with link adaptation is introduced. 2T2R and 2T4R is covered in the requirement definition and HARQ re-transmission is enabled in the test.

After RAN#103, according to the latest SR RP-240348, the completion of the core part has achieved 100%, and the performance part is targeted at June 2024. As a result, it is proper time to start the work on the corresponding UE conformance specifications for Rel-18 NR_demod_enh3 requirements in 3GPP RAN WG5.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to enable UE conformance testing for the Rel-18 NR demodulation performance evolution WI which includes analysing the test case impact, applicability, test environment, and updating the relevant conformance specifications.

The conformance testing aspects for this WI would consist of Demod area.

Discussion:

Qulcomm confirmed Keysight Spain's question that the MAC layer was added.

The RAN5 Chair RF menationed 37.901-5

Decision: 		The document was revised to R5-243455.

R5-242540	New WID on UE Conformance - Support for UAV (Uncrewed Aerial Vehicles) and Further Architecture Enhancement for UAV and UAM

Type: WID new		For: Information
					Source: Qualcomm Incorporated, Nokia, China Unicom

Abstract:

only for information

Discussion:

r1

Decision: 		The document was revised to R5-243456.

R5-242541	New WID on UE Conformance - Mobile Terminated-Small Data Transmission (MT-SDT) for NR

Type: WID new		For: Endorsement
					Source: Qualcomm Incorporated, China Telecom, ZTE

Discussion:

r1

Decision: 		The document was revised to R5-243457.

R5-242605	New WID on UE Conformance - Protocol enhancements for Mission Critical Services (MCPTT, MCVideo, MCData) for Rel-17

Type: WID new		For: Information
					Source: Ericsson, FirstNet, NIST, AT&T, Element, Samsung

Abstract:

The demand and need for Mission Critical Services (MCS) on 3GPP-compliant networks is growing. And, the need to enhance MCS functionality is also growing. 3GPP has answered this demand with enhancements to core specifications for MCS. MCS includes Mission Critical Push To Talk (MCPTT), Mission Critical Video (MCVideo), and Mission Critical Data (MCData). RAN5 has completed the UE conformance tests for MCS for Release 15 and Release 16 (scheduled to be completed in September 2024). RAN5 needs to address developing test specifications that relate to Release 17 MCPTT, MCVideo and MCData requirements for both LTE and 5G.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to provide conformance test specifications for the Rel-17 core requirements for MCPTT, MCVideo and MCData. The RAT for this Work Item is both LTE and 5G NR. A new series of specifications (3GPP TS 37.579-x), under 3GPP TS 37-series (Multiple radio access technology aspects series), is proposed to contain mission Critical Services test cases for both LTE and 5G NR RATs in the same specification.

All contents of the existing LTE MCS test specifications (TS 36.579-x series) will be transferred to the corresponding proposed TS 37.579-x series specifications, which will be the initial content of the proposed TS 37.579-x series specifications. Subsequently, the TS 36.579-x series will be turned into pointer specifications to corresponding TS 37.579-x series specifications. Rel-17 MCX test cases for both LTE and NR RATs will be added directly to proposed TS 37.579-x series.

It is possible that the core specifications may have included the functionality in releases prior to Rel-17 for a new test case, but it was deemed not necessary to test at the time under previous RAN5 work items. Associated generic test procedures, default message definitions and other related material for the new test cases will be included in the proposed TS 37.579-1. Applicability and other related material for the new test cases will be included in TS 37.579-4. TS 37.579-5 will have inclusions for the new test cases as needed.

In general, in SA1, SA2, SA3, SA4 and SA6, functional architecture and information flows to support mission critical communication services are defined which will be reflected in this RAN5 work item. In CT1, CT3, CT4 and CT6, MCPTT, MCVideo, and MCData protocol aspects are defined which, whenever relevant, will be reflected in this RAN5 work item. In CT1, IMS profiles to support MCPTT, MCVideo, and MCData are defined which will be reflected in this RAN5 work item.

The relevant core specifications for this Release 17 work item are:

TS 24.379 – MCPTT Call Control

TS 24.380 – MCPTT Media Plane

TS 24.481 – MCS Group Management

TS 24.482 – MCS Identity Management

TS 24.483 – MCS Management Object (MO)

TS 24.484 – MCS Configuration Management

TS 24.281 – MCVideo signalling protocol

TS 24.581 – MCVideo media plane control

TS 24.282 – MCData signalling protocol

TS 24.582 – MCData media plane control

TR 24.980 - Minimum requirements for support of MCPTT over the Gm

TS 33.180 - Security of the mission critical service

Decision: 		The document was revised to R5-243458.

R5-243458	New WID on UE Conformance - Protocol enhancements for Mission Critical Services (MCPTT, MCVideo, MCData) for Rel-17

Type: WID new		For: Information
					Source: Ericsson, FirstNet, NIST, AT&T, Element, Samsung

(Replaces R5-242605)

Discussion:

only noted.

Will be submitted at the next meeting for endorsement.

Decision: 		The document was noted.

R5-242691	New WID on UE Conformance – Enhanced support of Non-Public Networks Phase 2

Type: WID new		For: Endorsement
					Source: China Telecom, Qualcomm, ZTE

Abstract:

SA2 has been studying enhancements of support for non-public networks phase 2 under study item "Study on enhanced support of Non-Public Networks Phase 2" (FS_eNPN_Ph2). SA2 study has agreed conclusions for all key issues. The conclusions of the SA2 study are captured in 3GPP TR 23.700-08 and provide a good overview of what is to be continued into normative phase and of what impacts to other working groups are expected in normative phase. SA plenary approved work item "Enhanced support of Non-Public Networks Phase 2" (eNPN_Ph2) for SA2 normative work.

Impacts on protocols and interfaces under CT WGs' responsibilities have been identified and related work in CT WGs is 100% complete. The CT core spec changes will require corresponding RAN5 specification update to complete conformance requirements.

RAN plenary has also approved work item "Non-Public Networks Phase 2: NG-RAN aspect " for RAN normative work, which causes RAN2 core specification changes for Non-Public Networks to support equivalent SNPNs and non-3GPP access for SNPN services. The overall completion level for the core part of eNPN_Ph2 WI on the RAN side is 100% completed, requiring corresponding RAN5 specification update to complete conformance requirements.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to WID on Rel-18 Enhanced support of Non-Public Networks Phase 2 for both NG-RAN and CT1 aspects, analyse the test case impact, applicability, test environment, and update the relevant conformance specifications for Rel-18.

Decision: 		The document was revised to R5-243459.

R5-242841	New WID on UE Conformance - Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC)

Type: WID new		For: Endorsement
					Source: Apple, ROHDE & SCHWARZ, Vivo

Abstract:

RAN5 has completed definition of Rel-17 conformance test cases for NR FR1 TRP TRS. The scope covered FR1 OTA testing on NR FR1 bands n41 and n78 in browsing mode with Hand Phantom. In Rel-18, enhancements for TRP TRS requirements as well as corresponding verification methodologies are introduced:

(1)	Enhancements of TRP TRS test methodology

The baseline test method (anechoic-chamber based system) of FR1 TRP TRS was introduced in Rel-17 for FR1 TRP TRS. However, the specified test method is only applicable to UE with NR 1Tx capability, the advanced UE features with more Tx chains such as TxD and UL MIMO are not testable. Enhancement of test methodologies and metric to support the verification of the above 2 Tx feature is required to ensure the radiated performance of the devices.

The RedCap UE was introduced and finalized in Rel-17 with maximum channel bandwidth 20MHz and minimum 1Rx, however the OTA test configuration and corresponding new phantoms are not defined. Enhancement of TRP TRS test method to support RedCap UE performance verification is included in the Rel-18 scope.

In addition, the reverberation-chamber based system was discussed but deprioritized in Rel-17, which is also valuable to perform TRP TRS measurement as an alternative method to save measurement time and re-use large scale of legacy OTA systems.

(2)	TRP TRS requirements for NR 1Tx (Standalone)

In Rel-17, the requirement is only defined for selected bands (n41 and n78) under SA mode with very specific configuration, i.e., wide handset UE (width ≥72mm) operated with 1Tx at PC2 under browsing mode scenario (hand phantom only), while other cases are all deprioritized, e.g., other NR bands, talk mode scenarios (head and hand phantom), and other power classes. 1Tx Requirements for n41 and n78 in talk mode (using head and hand phantoms) and for n1, n28 in browsing and talk mode are defined in Rel-18. The corresponding conformance test cases will be added as part of this work item.

(3)	Metric and test method for NR 2Tx configuration (NR Standalone)

In Rel-16, many requests from operators and vendors on bands support UL-MIMO were seen. With increased interests from industry on UL coverage performance, a significant number of WIs in Rel-17 were focused on high power solutions and multi-antenna Tx features, which include, e.g., UL MIMO, PC2, TxD, CA high power limit, and PC1.5. All of these enhancements are anticipated to enhance the performance of NR networks overall; thus, developing a metric and test method for measuring radiated requirements for UEs supporting coherent and non-coherent UL-MIMO is in the scope of Rel-18. The developed test methods in the Core Part of the work item will be added to RAN5 conformance specifications as part of this work item.

(4)	Updates for NR 1Tx (EN-DC)

Updates to the framework for EN-DC TRP TRS testing is included. No new conformance requirements are introduced as there are no new core requirements for TRP TRS in EN-DC mode.

Until RAN#103, the Core part of the RAN4 Rel-18 WI “Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)” is 100% complete and Performance part is 75% completed. This necessitates the introduction of the RAN5 Conformance work item to enable the industry to pursue certification testing associated with new test methods and requirements enhancements for NR FR1 TRP TRS that is the scope of Rel-18.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this Work Item is to enable UE conformance testing for the corresponding Rel-18 NR FR1 TRP TRS Enhancement Work Items listed under clause 2.2. RAN5 conformance test procedures will be created to verify test requirements as per the scope being covered in the core and performance parts of the Rel-18 Work item namely:

(1)	Enhancements of TRP TRS test methodology

	Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline):

-	UE with NR 2Tx configuration

	Case 1: TxD (i.e., TxD capability supported)

	Case 2: single layer UL-MIMO (i.e., codebook-based capability supported)

	Define test case applicability for case 1 and case 2

	Addition of such NR 2Tx updates is pending completion of core requirements

-	RedCap devices (focused on wearable device only)

	Include definition of Forearm phantom

-	UE with DL/UL carrier aggregation configuration

	Independent measurements of each CC can be the baseline approach.

	Limited to Inter-band CA with up to 2DL/1UL cases

	Addition of such CA related updates is pending completion of core requirements

-	Specify reverb-chamber based test methodology to support SA and EN-DC TRP TRS testing: Align the RAN5 test specifications with any outcome of the harmonization between anechoic chamber and Reverb-chamber based method which can be applied independently for the conditions between 1Tx and 2Tx, among browsing mode, talk mode, and wrist-worn mode.

•	Define conformance tests containing test requirements for NR FR1 SISO SA TRP and TRS (OTA) for the following:

-	NR FR1 Bands n41, n78 for PC2 and PC2 UEs in Talk Mode (using Head and Hand Phantoms)

-	NR FR1 Bands n28 and n1 for PC3 UEs in Talk Mode (using Head and Hand phantoms) and Browsing Mode (using Hand Phantoms only). Addition of Band n79 test requirements is pending completion of core requirements.

-	Define test cases and test requirements for the above listed bands and phantom configurations based on the conclusion of core performance requirement definitions.

-	4Rx test requirements are in scope for n41, n78, n1 and 2Rx test requirements for n1.

-	Specifying test requirements of SA with 1 CC is the priority

•	DUT feature focus

o	Rel-18 focus is on smartphone. Test coverage for additional DUT form factors to be determined in future Release(s)

o	The scope covers Wide Grip Hand for UE with Width >72mm and ≤92mm. Additional device widths to be determined in future Release(s)

o	Power Class: Both PC2 and PC3 with 1Tx

•	Updates to r NR FR1 SISO EN-DC TRP and TRS (OTA) sections:

-	The framework created in Rel-17 for EN-DC TRP TRS testing is retained and updates introduced in the core specifications is incorporated in the conformance test specifications

-	No new test requirements for EN-DC are introduced within the scope of this work item due to no core requirements created.

•	Measurement Uncertainty (MU) and Test tolerances (TT)

•	 RAN5 has primary responsibility for MU assessment for FR1 TRP/TRS.

•	A preliminary/placeholder MU table has been added in Annex B of TR 38.870 to cover updates to the MU for browsing mode testing and new values for Talk Mode based FR1 TRP TRS (OTA) testing.

•	This work item scope includes further optimizing and finalizing MU values and then determine TT for the defined test requirements based on recommended Test Tolerances from RAN4.

Discussion:

r1

Decision: 		The document was revised to R5-243460.

R5-243079	New WID on UE Conformance - In-Device Co-existence (IDC) enhancements for NR and MR-DC

Type: WID new		For: Endorsement
					Source: CMCC, Xiaomi

Abstract:

Rel-18 RAN2 work item NR_IDC_Enh was carried out to enable well interference mitigation between 3GPP and other RAT under the following assumptions:

As a general assumption, IDC is assumed to work as follows:

1.	The UE detects internal issue or the possibility of internal issue caused by coexistence related to usage of certain radio resources, that the UE cannot resolve by itself.

2.	The UE provides information to the gNB to assist that the gNB may restrict radio resource usage to avoid the UE internal issue (or potential issue) caused by coexistence.

The IDC solution prior to Rel-18 in NR has the following limitations: It does not support well interference mitigation between 3GPP and other RAT, as e.g. the affected frequencies cannot be adequately indicated via the NR FDM solution. Introducing a TDM solution would make it possible to handle scenarios for which alternative non-interfered frequencies are not available. The TDM solution may be used to avoid the interference caused by simultaneous uplink transmission on the UL frequencies to non-3GPP RAT.

For Rel-18 RAN2 work item NR_IDC_Enh, the overall completion level has reached 100% after RP#102. Therefore it is justified to introduce the conformance testing for In-Device Co-existence (IDC) enhancements for NR and MR-DC UE requirements into RAN5 specifications.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this RAN5 work item is to define the corresponding UE conformance requirements, applicability, test environment, test points, test procedures and update the relevant conformance specifications for the R18 In-Device Co-existence (IDC) enhancements for NR and MR-DC UE requirements introduced by RAN2.

Discussion:

r1

Decision: 		The document was revised to R5-243461.

R5-243222	UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs

Type: WID new		For: Endorsement
					Source: Apple, CAICT, Keysight Technologies

Decision: 		The document was revised to R5-243462.

R5-243284	New WID on UE Conformance  4Rx for NR bands for FWA UEs (up to 2.6GHz) and handheld UEs (1GHz to 2.6GHz)

Type: WID new		For: Endorsement
					Source: ZTE, China Telecom

Abstract:

3GPP TSG RAN WG4 has created the WID for Rel-18 4Rx for NR bands for FWA (Fixed Wireless Access) UEs (<2.6GHz) and handheld UEs (1GHz to 2.6GHz). The WID is to provide higher throughput and better coverage with the introduction of 4Rx UE requirements based on the operator and market demand. Following requirements are introduced under this WI:

•	Specify the 4Rx REFSENS requirement for NR FR1 bands

o	Support of 4Rx for the bands <2.6GHz added in this WI is optional

o	Note 1: for operating frequency less than 1GHz, only FWA is considered.

The WID was completed with 100% completion level in last RAN plenary. It’s time for 3GPP TSG RAN WG5 to start working on the corresponding conformance testing part to meet the industry interest.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to develop RF test cases for Rel-18 4Rx for NR bands for FWA (Fixed Wireless Access) UEs (<2.6GHz) and handheld UEs (1GHz to 2.6GHz). For the enhancements for 4Rx FWA (<2.6GHz) and handheld UEs (<2.6GHz), example bands considered in core WI are n5, n13, n25, n26 and n85.

Band	Additional information	status

Note: any specific technical challenge should be triggered and highlighted here

n5	4 Rx operation is targeted for FWA form factor	Completed

n25		Completed

n85	4 Rx operation is targeted for FWA form factor	Completed

n26	4 Rx operation is targeted for FWA form factor	Completed

n13	4 Rx operation is targeted for FWA form factor	Completed

In the conformance test WI, example bands would be selected from the above list based on the industry interest to complete the new RF test cases.

Decision: 		The document was revised to R5-243463.

R5-243355	New WID on UE Conformance - NR NTN (Non-Terrestrial Networks) enhancements

Type: WID new		For: Endorsement
					Source: Qualcomm Inc, China Unicom, Mediatek

Abstract:

In Release 17, a work item was carried out to define solutions enabling NR and NG-RAN to support Non-Terrestrial Networks (NTN):

•	Transparent payload based GSO and NGSO network scenarios addressing 3GPP power class 3 UE in both Earth fixed and/or moving cell configurations.

As part of Release 18, the parent work item proposed enhancements for NG-RAN based Non-Terrestrial Networks in order to:

•	Support new scenarios to cover deployments in frequency bands above 10 GHz for VSAT UE

•	Offer optimized performance especially when addressing handset terminals (including smartphones with more realistic assumptions on antenna gains instead of 0 dBi antenna gain with the specific realistic antenna gain assumption to be determined at the working group level) w.r.t. coverage considering the NTN characteristics such as large propagation delay and satellite movement.

•	Provide mobility and service continuity enhancements considering the NTN characteristics such as large propagation delay and satellite movement.

•	Address requirements, if needed based on the FS_NR_NTN_netw_verif_UE_loc study outcome, which mandate the network operator to cross check the UE location reported by the UE, which needs to be carried out in order to fulfil the regulatory requirements (e.g., Lawful intercept, emergency call, Public Warning System, …) regarding a network verified UE location i.e., to be able to check the UE reported location information (e.g. estimate UE location at the network side) and specify if needed mechanisms to fulfil the regulatory requirements.

There is an industry need to enable testing of the requirements specified in the core work item. The core WI has progressed sufficiently to initiate the RAN5 conformance WI.

4	Objective

The objective of this work item is to enable UE conformance testing for the Rel-18 NTN enhancements WI which includes analysing the requirements, creation of corresponding test cases by defining the test environment, special conformance testing function, test procedure, message contents, MU/TT analysis, associated PICS, applicability and updating the relevant conformance specifications.

The conformance testing aspects for this WI would consist of below areas:

	Protocol test cases for NR NTN enhancements

	Functionality introduced by the CT1 WI on satellite access phase 2

	Procedures for determining and negotiating out-of-coverage period due to discontinuous coverage and for power saving during coverage gaps:

o	Negotiation of “discontinuous coverage support” capability between the UE and the AMF

o	Negotiation of “out-of-coverage period” between the UE and the AMF

o	Indication of losing coverage by the UE

o	AMF requesting the UE to perform Registration update upon return to coverage

	Handling signalling overload due to loss of coverage and return to coverage of many UEs at the same time.

	UE RF test cases for NR NTN enhancements

	UE DEMOD test cases for NR NTN enhancements

	RRM test cases for NR NTN enhancements

Decision: 		The document was revised to R5-243464.

R5-243356	New WID on UE Conformance - Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception

Type: WID new		For: Endorsement
					Source: Qualcomm Inc

Abstract:

The existing Rel-15 NR FR2 minimum UE requirements are defined with an assumption that UE is equipped with a single antenna panel and capable to perform DL reception using a single RX beam/chain reception. Furthermore, the UE performance requirements are limited to DL MIMO rank 1 and 2. In FR2, 4-layer MIMO reception requires beam reception from at least two directions. Although this is supported by the MIMO features since Rel-15, no performance requirements have yet been specified. This is important for high-rate MIMO in FR2, as well as for FR2 HST scenarios.

During Rel-16 and Rel-17, the support of NR FR2 CA with IBM (Independent Beam Management) with simultaneous DL reception on different component carriers from the co-located and non-co-located TRPs was defined. The IBM concept implies a UE is capable of DL simultaneous reception on different UE panels/chains using separate beams on different component carriers and requires improved UE baseband and RF capabilities (multiple baseband chains and support of multiple antenna panels).

Several enhancements to enable efficient and robust DL multi-TRP/panel operation were introduced in the Rel-16 NR eMIMO WI. For instance, DL transmission schemes with simultaneous and non-simultaneous multi-beam reception from multiple TRPs/panels were introduced. The simultaneous reception may require support of simultaneous multi-panel operation with several independent RX beams/chains at the UE side. As part of this item, a new FR2 UE capability for simultaneous multi-beam reception was introduced (simultaneousReceptionDiffTypeD-r16). However, no RF, RRM or performance requirements were defined in Rel-16 and Rel-17 for FR2 UEs with simultaneousReceptionDiffTypeD-r16 capability.

Enhanced NR FR2 UEs with multi-beam simultaneous reception and multiple RX chains can provide a meaningful performance improvement in FR2 improving both demodulation performance (4-layer DL MIMO), RRM performance and improve RF spherical coverage.

The parent work item aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier to achieve improved RF, RRM and UE demodulation performance.

Different implementation scenarios could be considered at the UE. Single-TCI reception on different beams has been supported by the RAN1 specifications since Rel-15 via the Type I codebook, which could be achieved at the UE with either a single panel or multiple panels. Alternatively, dual TCI operation can be combined with the Rel-17 mTRP framework even if the base station is actually deployed as a single TRP.

The parent WI therefore provides the requirements for both single and dual TCI assumptions to specify requirements for reception of 4-layer downlink MIMO with simultaneous reception at the UE from two different directions.

4	Objective

The objective of this work item is to enable UE conformance testing for the Rel-18 FR2 Multi-Rx WI which includes analysing the requirements, creation of corresponding test cases by defining the test environment, special conformance testing function, test procedure, message contents, MU/TT analysis, associated PICS, applicability and updating the relevant conformance specifications.

The conformance testing aspects for this WI would consist of RF, Demodulation and RRM area

Decision: 		The document was revised to R5-243465.

### 4.2	General Discussion Papers

#### 4.2.1	5GS

#### 4.2.2	NR Applicability specifications (Software Implementation)

R5-242560	Progress and Way Forward on SW Implementation on Machine Readable PICS and Applicability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

Recap: Industry Value and Progress

Update - Machine-Readable PICS File from TS 38.508-2

Formatting issues in current specs

Procedural Proposals and overall W/F

Further Proposals on PICS processing

Allow DUT Declaration i.e allow user to enter PICS value

Data Processing of DUT Declaration (for ex - in a separate tab)

Meaningful association of extracted data to be used for outside automation

Proposals and Updates:

RAN5 to endorse creation of “machine readable PICS” during every spec release of TS 38.508-2

Draft version already shared with the applicability reflector for the latest version

Further enhancement of Machine-Readable PICS

[In progress, Can be delivered Pre RAN5#104] Allow DUT Declaration i.e allow user to enter PICS value

[In Progress]Data Processing of above(ex -  in a separate tab of excel) will be required

3. Providing all above as an output will be most valuable to ecosystem as Step-1, this will allow to

Fill DUT PICS compliance in standard format

Help further automation to derive test case applicability

Discussion:

r1

RAN5 Chair: check if the excel sheet can be included in the spec zipfile

MCC to check with the specs manager if this is viable.

TF160: maybe putting the excelfile into forge is better; also in order to decouple the spec publishing and the excelsheet availability

Decision: 		The document was revised to R5-243567.

R5-243567	Progress and Way Forward on SW Implementation on Machine Readable PICS and Applicability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

(Replaces R5-242560)

Decision: 		The document was revised to R5-243970.

R5-243970	Progress and Way Forward on SW Implementation on Machine Readable PICS and Applicability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

(Replaces R5-243567)

Decision: 		The document was noted.

#### 4.2.3	All other topics

R5-242567	Challenges and Proposals for Testing eSIM-only Devices

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

eSIM (embedded SIM) is an industry-standard digital SIM. It eliminates the need for a physical SIM card in a device, enabling remote provisioning of a mobile operator’s service and this simplifies switching between operators. eUICC (Embedded Universal Integrated Circuit Card), is the software component located inside the eSIM chip.

The GSMA Terminal Steering Group has developed TS.48 [1] Generic eUICC Test Profile for Device Testing, ensuring compliance with 3GPP specifications for devices with non-removable SIMs. 3GPP RAN5 is aligned with this Test Profile. Devices equipped with eSIM-only have unique challenges in certification. These challenges are crucial for ensuring the successful adoption and integration of eSIM technology.

Challenges for Certification of eSIM-only Device

1.	Enabling Generic Test Profile

Before a device test begins, the Generic Test Profile must be present on the Device Under Test (DUT). The tester or test equipment should activate (enable) the Generic Test Profile before executing the test case. As required by the test case, the profile contents shall be modified.

GSMA PRD specifications SGP.22[2] (for Consumer Devices), SGP.32[3] (for Consumer IOT), SGP.02[4] (for M2M) provide details on enabling and disabling test profiles. The Generic Test Profile remains active until the profile is disabled. Even after a power cycle of the DUT, the Generic Test Profile remains active.

Ensuring that Generic Test Profile available is the basic requirement for all test areas with in RAN5. Only Signalling test have further requirements to modify the contents of this Generic Test Profile and challenges listed below are signalling specific with no impact to performance test areas.

2.	Test Profile Modification & Restoration

Only signalling test specifications have further requirement to modify Elementary Files inside the Generic Test Profile, TS 38.508-1 cl4.8.3 defines common parameters while cl 6.4 defines Signalling Test Case specific USIM Configurations, which are required to execute by modifying impacted Elementaty Files (EF) in the USIM. The procedures used to modify USIM are well established by test equipment vendors while the same for eSIM-only devices is not harmonized.

To avoid carrying over updated data on the Generic Test Profile from previously run test cases it is recommended to restore to the default generic test profile data back to its original TS.48 data before starting a test case.

As. As per TS 48[1], One of the following methods below shall be used to modify the Generic Test Profile. Each method for Profile modification shall provide a way to return to original (default) values.

2.1 RFM/RAM (Remote File/Application Management)

The DUT shall support an OTA interface as defined in GSMA PRDs SGP.02 [4] and SGP.22 [2] (ES6 interface). This interface may be used to modify Generic Test Profile content by using standardized RFM/RAM commands as defined in ETSI 102 225 [5], ETSI 102 226 [6] and Global Platform Amd. B [7].  The RFM commands for modifying the profile (EFs) may be sent as payload in SMS -PP Download over the Air.

2.2 AT Commands

AT commands may be sent over a USB interface to update the Generic Test Profile. AT+CSIM command defined in 3GPP 27.007 [8] cl 8.17 and AT+CRSM commands defined in cl 8.18 can be used for selecting EFs and updating the files with given data, other AT commands used to modify EFs are AT+CGLA,AT+CRLA, AT+CCHO, AT+CCHC..

The security restrictions may have to be disabled (for generic test profile) for some devices to allow access to all data in eSIM/eUICC for updating the EF using AT CMD. Generic SIM Access +CSIM and Restricted SIM Access +CRSM commands can be used to access all data in eSIM/eUICC.

The test profile modification using AT commands might add additional execution time to the test procedure, especially if AT commands are used for restoring the Generic Test Profile data to original default values.

2.3 Test Applet

A test applet can be loaded separately or it can be loaded with the Generic Test Profile (as a separate Profile Element) for handling the EF updates. Also, test applet can be used to restore the generic test profile data to its original default values before starting each TC. This method of restoring the Generic Test Profile to original default values is more efficient compared to other methods described in this section.

2.4 Proprietary Approach

eUICC within a device can be connected to a card reader using a proprietary method implemented in the device.

e.g. Devices supporting CCID interface over USB can be put in CCID mode to detect the device as a card reader. An external card reader tool can be used to update the generic test profile contents over the CCID interface.

3.	Spec Language

The USIM related procedures that are defined in TS 38.508-1[9] & TS 38.523-1[10] needs to be considered for the devices which support eSIM only.  For example, USIM removal is not applicable for eSIM. Following are some of the USIM related procedures referred in TS 38.508-1 and TS 38.523-1, which may be need editorial or technical update to avoid confusion and facilitate eSIM procedures.

•	Default parameters for the test USIM and ISIM [TS 38.508-1, clause 4.8.3.2]

•	USIM removal is a feasible alternative to switch off UE [TS38.508-1, clause 4.9.6]

•	UE is fitted with a USIM with Access Technology data files for each PLMN [TS38.523-1 clause 6.1]

•	The USIM is removed [TS38.523-1, multiple clauses].

•	The UE is equipped with a USIM configuration [TS38.523-1, clause 6.1.1.2.3]

•	If pc_USIM_Removal is selected, use same USIM [TS38.523-1 clause 6.3.2.5, clause 9.1.6, 9.1.10. 11.3.2, 11.5.6.]

•	UE is switched off with a valid USIM inserted [TS38.523-1 clause 9.1.5.1.3a].

•	The USIM is removed from the UE [TS 38.523-1 clause 9.2.6].

•	UE supports remove USIM without power down [TS38.523-1 clauses 9.1.6, 9.2.6]

•	The UICC containing the USIM is removed [TS38.523-1 multiple clauses].

•	Signaling test case specific USIM configurations [ TS38.508-1, clause 6.4]

Proposals

1.	The Generic Test Profile must be present on the DUT as a precondition for Performance and Protocol testing defined by 3GPP RAN5 specification. UE Manufacturer is responsible to enable this for test devices.

Signalling Specific

2.	For test case specific profile modification, AT command method is preferred out the four methods discussed above. The security restrictions need to be disabled to allow access all data in eSIM/eUICC for updating the EF using AT CMD.

2.1	A new PICS can be introduced for UE supporting eSIM profile update through AT Command to decide if the test system can use this approach for updating eSIM profile. This will ensure that UE has also removed any security restrictions on that device for AT CMD to modify required EF’s.

3.	For restoring the Generic Test Profile to its original values efficiently, test applet method is preferred out the four methods in #2. However, even AT CMDs can be used.

4.	The security restrictions need to be disabled to allow access all data in eSIM/eUICC for updating the EF using AT CMD.

5.	The specification language should be updated to include eSIM-related procedures for better readability and to avoid confusion. An Action Item can be taken up in Signalling to review impacted clauses and align with eSIM-only devices. An example of update might be like below-

o	Ex -If USIM removal is part of a test flow verification, same can be replaced for eSIM as “proprietary commands can be used to power down and power up the eSIM”.

Discussion:

moved to SIG.

r1

Anritsu:where is the profile, to be loaded?

Qualcomm: we have to address the GSM members.

TF160 manager: we have to define a pics.

China Telecom: test cases based on USIM cannot be verified in RAN5.

Endorsing in principle porposal 1 and 2.

Implementation will be evaluated based on CRs.

AP raised to fix spec language issues.

Decision: 		The document was revised to R5-243466.

R5-243466	Challenges and Proposals for Testing eSIM-only Devices

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

(Replaces R5-242567)

Decision: 		The document was noted.

### 4.3	RAN5 PRDs/Templates

R5-242112	RAN5#103 LS Template

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-242309	RAN5 PRD12 version 6.10

Type: other		For: Approval
					Source: MCC TF160

Decision: 		The document was approved.

### 4.4	Meeting schedule for 2024-25

R5-242113	Meeting schedule for 2024-25

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

### 4.5	Tdocs for mid-week joint session

#### 4.5.1	Agenda for mid-week session

R5-242101	Agenda - midweek session

Type: agenda		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

#### 4.5.2	RF group docs for WG review/verdict - original A.I. retained

R5-242708	Definition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A applicability to protocol testing

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3168  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Abstract:

AI 5.3.3.1.1

Discussion:

endorsed by RF.

Decision: 		The document was agreed.

#### 4.5.3	Sig group docs for WG review/verdict - original A.I. retained

R5-242324	Deletion of PICS pc_pdsch_MappingTypeA

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0615  Cat: F (Rel-18)

					Source: MCC TF160

Abstract:

AI 6.4.2

Discussion:

agreed in SIG.

Agreed again in the joint on Wed.

Decision: 		The document was agreed.

#### 4.5.4	Other open issues from joint sessions - original A.I. retained

R5-243357	New WID on UE Conformance - New bands and bandwidth allocation for LTE based 5G terrestrial broadcast

Type: WID new		For: Endorsement
					Source: Qualcomm Inc

Abstract:

In Rel-14 and Rel-16, RAN1/2/4 introduced a set of features under the umbrella of “LTE based 5G terrestrial broadcast” that, together with the corresponding enhancements in the core network, enable the deployment of a downlink-only dedicated broadcast network using 3GPP technologies.

As part of the parent RAN4 Rel18 WI, a portion of UHF spectrum allocated to broadcast systems (~470 - ~694/698MHz, depending on the region) was identified as spectrum for deployment of LTE based 5G broadcast. The parent work item introduced the necessary requirements for LTE based 5G terrestrial broadcast to operate in the portion of UHF spectrum allocated to broadcast systems in different regions of the globe. For instance:

•	For ITU region 1, a channelization of 8MHz is used.

•	For ITU region 2, a channelization of 6MHz is used.

o	In some countries (e.g. North America), frequencies above 614MHz are used by FDD (B71).

•	For ITU region 3, different channelizations (6/7/8MHz) are used in different countries, for instance China and India are using 8MHz.

o	In China broadcast systems are using the spectrum ranges 470  – 702 MHz..

o	In India broadcast systems are using the spectrum ranges 470 – 610 MHz and 614 – 698 MHz.

There is an industry need to enable testing of the requirements specified in the core work item. The core WI has been completed which paves the way for initiating the RAN5 conformance WI.

4	Objective

The objective of this work item is to enable UE conformance testing for the Rel-18 LTE based 5G Broadcast WI which includes analysing the requirements, creation of corresponding test cases by defining the test environment, special conformance testing function, test procedure, message contents, MU/TT analysis, associated PICS, applicability and updating the relevant conformance specifications.

The conformance testing aspects for this WI would consist of below areas:

	Protocol test cases for LTE based 5G Broadcast

	UE RF test cases for LTE based 5G Broadcast

	UE DEMOD test cases for LTE based 5G Broadcast

Discussion:

r1

Decision: 		The document was revised to R5-243568.

R5-243358	Discussion on introducing the WID for LTE based 5G terrestrial broadcast

Type: discussion		For: Approval
					Source: Qualcomm Inc

Abstract:

There is an ongoing WI in RAN4 for LTE based 5G terrestrial broadcast. The core WI has a rel17 WI and a Rel18 WI. This paper discusses the potential options for introducing a corresponding RAN5 conformance WI.

2.	Discussion

Objective of Rel17 part1 WID (RP-211144) New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 1 is as pasted below.

The key changes are introduction of a rel17 IE to signal to the UE MBMS dedicated cell info for bandwidth 15/25 RB. Corresponding UE capability CR was introduced to allow UE to advertise in its UE capability info regarding support for PMCH BW of 40/35/30 PRBs

As seen in the above snippet, the objective of the part1 WID was to introduce the necessary 36.331 and 36.306 signaling to enable the introduction of new band/bandwidth and corresponding RF requirements for LTE based 5G broadcast.

Rel18 part2 WID (New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2) (UID 920071) was introduced to add the new band (n107, n108)/bandwidth (6/7/8 MHz) along with RF requirements.

There are 2 options for introducing the corresponding RAN5 conformance WI.

Option1: RAN5 to introduce 2 separate WI: Rel17 part1 followed by a Rel18 part2

Option2: RAN5 to introduce a single Rel18 WI which to address the objectives of both the Rel17 and Rel18 core WI.

Option1 will allow a 1:1 correspondence with the core WI. But without the new bands/BW that are being introduced in Rel18 part2 WI, the Rel17 aspects cannot be tested in RAN5.

Hence, our preference is to go with option2.

Proposal 1: Introduce a single Rel18 WI for LTE based 5G broadcast that addresses the objectives of both the Rel17 and Rel18 core WI

Additionally, some of the physical layer enhancements to support LTE based 5G broadcast such as new sub carrier spacing of 2.5 kHz, 0.37 kHz and corresponding 36.331 signalling was introduced in Rel14 and Rel16 WI.

The Rel16 RAN4 WI (Perf. part: LTE-based 5G terrestrial broadcast) adds PMCH demodulation requirements for 10 MHz channel bandwidth. But the bandwidth introduced for the new bands n107 and n108 only have 6/7/8 MHz bandwidth.

So the 10MHz PMCH demod requirements defined in the Rel16 WI cannot be directly applied to the n107/n108 bands. Clarifications are needed from Ran4 on how to test the PMCH demodulation requirements for n107/n108 bands.

Our suggestion is to include all these Rel14/Rel16 changes and discussion as part of the same Rel18 work plan since the contents of all these WI are inter-dependent.

Proposal 2: Include the relevant SIG/RF/Demod related changes introduced in Rel14/Rel16 core WI in the same Rel18 RAN5 WI.

Discussion:

Noted, proposals are accepted.

Decision: 		The document was noted.

#### 4.5.5	Other

## 5	RF Functional Area

### 5.1	Review action points (fm A.I. 2.1)

### 5.2	Review incoming LS (fm A.I. 3) & new subject discussion papers

R5-242118	Reply LS on TT work for Rel-18 NR FR1 TRP TRS

Type: LS in		For: Information
					Original outgoing LS: R4-2406079, to -, cc -
					Source: TSG WG RAN4

Abstract:

3GPP RAN4 would like to thank RAN5 for their LS in R4-2404119/R5-241956 on TT work for Rel-18 NR FR1 TRP TRS.

Follow the similar approach of Rel-17, RAN4 will specify Rel-18 TRP TRS requirements and recommended TT based on the agreed scope in WID [1].

RAN4 has discussed the TT value, the agreements are listed as following:

	TT value is recommended as a ratio of MU, where TT=0.62*MU; according to currently available analysis of MU of AC system in RAN4, these TT values should be:

o	For browsing mode: TRP TT is 1.1dB for above 3GHz bands and 1dB for below 3GHz bands, TRS TT is 1.3 dB for above 3GHz bands and 1.2 dB for below 3GHz bands;

o	For talk mode: TRP TT is 1.2dB for above 3GHz bands and 1.1dB for below 3GHz bands, TRS TT is 1.5 dB for above 3GHz bands and 1.4 dB for below 3GHz bands;

	The above recommended TT values for NR FR1 TRP and TRS are decided based on the assessed preliminary MU budget and outcome of Rel-18 FR1 TRP TRS lab alignment activity, thereby incorporating the actual bounds of the lab alignment measurements and associated uncertainties [2]

	Considering the deep relation between requirements and TT for FR1 TRP TRS, RAN4 suggests not to further modify the TT value if MU values are further modified in RAN5

With regards to the core requirements definition process, RAN4 agree a detailed working procedure [3] (and some further update agreements in [4] [5]) to guide Rel-18 FR1 TRP TRS requirements definition work, some of the key aspects are summarized as following:

	RAN4 performed a successful FR1 TRP TRS Lab alignment activity with Anechoic Chamber method [2], all the participated 8 labs (ISO 17025 accredited labs) are well aligned based on lab alignment activity of reference LADs (Lab Alignment Device) with AC test system:

o	Rel-18 AC lab alignment is performed under talk mode (Head and Hand phantom)

o	In total, there are 4 LADs, two for band n28 lab alignment, two for band n78 lab alignment

o	The pass/fail limit (max offset between individual results and averaged value) for Rel-18 AC lab alignment activity is defined as 0.75*MU, i.e., 1.47dB for n78 TRP, and 1.41dB for n28 TRP, 1.73dB for n78 TRS and 1.69 dB for n28 TRS;

	RAN4 adopted a “partial anonymous” approach in Rel-18 to collect UE model information those for performance measurement campaign, which is to check the representative of 3GPP data pool, some key aspects of this approach:

o	The disclosure of exact device model is done only between each test lab to the Neutral Party, by using the same template [4]

o	The disclosed UE information includes model name, model Number, vendor Name, power class, supported bands, certifications, year of production

o	Instead of disclosing dedicated UE model information list, the Neutral Party will do the post-processing of all the UE information, and finally publish JUST the Statistics outcome of RAN4 UE model information pool, e.g.,

	Total Models

	Total number of device vendors

	Percentage of devices per power class/supported band

	Percentage of devices per year of production

	Percentage of devices for each certification

o	RAN4 also defined the thresholds for above aspects to ensure the statistical relevance of the devices pool

	RAN4 performed a performance test campaign to collect measurement results from aligned test labs

o	The devices for measurement campaign are from operators, vendors and 3rd party test labs under a well overall-balance of device sources

o	RAN4 is targeting to finalize the TRP and TRS requirements for band n28/n1/n41/n78 under both browsing mode and talk mode

	Note that band n41/n78 browsing mode requirements had been defined in Rel-17

	The final TRP TRS requirements are defined based on per-band Data driven approach with CDF analysis of all the collected measurement results from Rel-18 performance measurement campaign.

2	Actions

To RAN5:

ACTION: 	3GPP RAN4 respectfully asks RAN5 to take the above feedback into account when developing NR FR1 TRP TRS conformance test specification.

Discussion:

moved to RF

Decision: 		The document was noted.

### 5.3	Open Work Items

#### 5.3.1	REL-16 NR CA and DC; and NR and LTE DC Configurations (UID-830083) NR_CADC_NR_LTE_DC_R16-UEConTest

##### 5.3.1.1	TS 38.508-1

###### 5.3.1.1.1	Test frequencies (Clause 4.3.1)

R5-242240	Addition of test frequencies for CA_n28A-n40A

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3112  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-242450	 Update for additional ENDC band configurations

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3136  Cat: F (Rel-18)

					Source: Verizon Spain

Discussion:

agreed again in the joint on Wed.

Decision: 		The document was agreed.

R5-242515	Test frequency parameters corrections for CA n48B

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3138  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242590	Addition of test frequencies for new EN-DC comb within FR2

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3149  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-243031	Update inter-band NR CA configuration for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3196  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was revised to R5-243600.

R5-243600	Update inter-band NR CA configuration for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3196  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-243031)

Decision: 		The document was agreed.

###### 5.3.1.1.2	Test environment for RF (Clauses 5)

###### 5.3.1.1.3	Test environment for RRM (Clause 7)

###### 5.3.1.1.4	Other clauses, Annexes

##### 5.3.1.2	TS 38.508-2

R5-242242	Addition of UE capability for CA_n28A-n40A

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0610  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

r1

Decision: 		The document was revised to R5-243812.

R5-243812	Addition of UE capability for CA_n28A-n40A

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0610  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-242242)

Decision: 		The document was agreed.

R5-242255	Editorial correction of UE capability for R16 EN-DC combs within FR1

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0613  Cat: F (Rel-18)

					Source: KDDI Corporation

Abstract:

Editorial correction

Decision: 		The document was agreed.

R5-242451	Update for additional ENDC band configurations

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0622  Cat: F (Rel-18)

					Source: Verizon

Decision: 		The document was agreed.

R5-242591	Addition of UE capability for new EN-DC comb within FR2

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0627  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was withdrawn.

R5-243189	Addition of CA_n1A-n8A-n78A,CA_n3A-n8A-n78A into NR inter-band CA within FR1 three bands

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0645  Cat: F (Rel-18)

					Source: China Unicom

Discussion:

()

r1

Decision: 		The document was revised to R5-243607.

R5-243607	Addition of CA_n1A-n8A-n78A,CA_n3A-n8A-n78A into NR inter-band CA within FR1 three bands

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0645  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-243189)

Decision: 		The document was agreed.

##### 5.3.1.3	TS 38.521-1

###### 5.3.1.3.1	Tx Requirements (Clause 6)

R5-242247	Addition of delta TIBc for CA_n28A-n40A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2736  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-243027	Addition of delta TIB into 6.2A.4.0.2.3 for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2802  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was agreed.

R5-243259	Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL Rel16

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2845  Cat: F (Rel-18)

					Source: Apple, Keysight

Discussion:

r1

Decision: 		The document was revised to R5-243816.

R5-243816	Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL Rel16

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2845  rev 1 Cat: F (Rel-18)

					Source: Apple, Keysight

(Replaces R5-243259)

Decision: 		The document was agreed.

R5-243260	Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-16

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2846  Cat: F (Rel-18)

					Source: Apple, Keysight

Discussion:

r2

Decision: 		The document was revised to R5-243817.

R5-243817	Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-16

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2846  rev 1 Cat: F (Rel-18)

					Source: Apple, Keysight

(Replaces R5-243260)

Decision: 		The document was agreed.

###### 5.3.1.3.2	Rx Requirements (Clause 7)

R5-242249	Addition of reference sensitivity for CA_n28A-n40A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2738  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

r2

Decision: 		The document was revised to R5-243821.

R5-243821	Addition of reference sensitivity for CA_n28A-n40A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2738  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-242249)

Decision: 		The document was agreed.

R5-242538	Addition of CA_n48C reference sensitivity requirement

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2773  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-243028	Addition of delta RIB into 7.3A.0.3.2.3 for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2803  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was revised to R5-243632.

R5-243632	Addition of delta RIB into 7.3A.0.3.2.3 for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2803  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-243028)

Decision: 		The document was agreed.

R5-243029	Addition of 3DL CA minimun requirements of reference sensitivity exceptions into 7.3A.0.5 for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2804  Cat: F (Rel-18)

					Source: China Unicom

Abstract:

To align witn core spec

Decision: 		The document was agreed.

R5-243030	Addition of 3DL CA reference sensitivity TC into 7.3A.2 for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2805  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was revised to R5-243617.

R5-243617	Addition of 3DL CA reference sensitivity TC into 7.3A.2 for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2805  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-243030)

Decision: 		The document was agreed.

R5-243045	Update to REFSENSE TC for CA_n79C

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2809  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

###### 5.3.1.3.3	Clauses 1-5, Annexes

R5-242572	General updates of TS 38.521-1 clause 5 for R16 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2775  Cat: F (Rel-18)

					Source: China Unicom, KDDI

Decision: 		The document was agreed.

##### 5.3.1.4	TS 38.521-2

###### 5.3.1.4.1	Tx Requirements (Clause 6)

###### 5.3.1.4.2	Rx Requirements (Clause 7)

###### 5.3.1.4.3	Clauses 1-5, Annexes

##### 5.3.1.5	TS 38.521-3

###### 5.3.1.5.1	Tx Requirements (Clause 6)

R5-242599	Core requirements alignment for UE co-existence of Inter-band within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1761  Cat: F (Rel-18)

					Source: CAICT

Abstract:

Core spec alignment

Discussion:

AI? spec?

Decision: 		The document was revised to R5-243665.

R5-243665	Core requirements alignment for UE co-existence of Inter-band within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1761  rev 1 Cat: F (Rel-18)

					Source: CAICT

(Replaces R5-242599)

Decision: 		The document was agreed.

R5-242719	Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1764  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

This CR was already submitted as R5-241785 but it could not be approved at RAN #103, because digital rights restrictions did not allow to open the Tdoc.

Discussion:

will resubmit at the next meeting.

Decision: 		The document was withdrawn.

R5-242721	Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1765  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

This CR was already submitted as R5-241786 but it could not be approved at RAN #103, because digital rights restrictions did not allow to open the Tdoc.

Discussion:

will resubmit at the next meeting.

Decision: 		The document was withdrawn.

R5-242722	Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1766  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

This CR was already submitted as R5-241787 but it could not be approved at RAN #103, because digital rights restrictions did not allow to open the Tdoc.

Discussion:

will resubmit at the next meeting.

Decision: 		The document was withdrawn.

R5-242723	Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1767  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

This CR was already submitted as R5-241788 but it could not be approved at RAN #103, because digital rights restrictions did not allow to open the Tdoc.

Discussion:

will resubmit at the next meeting.

Decision: 		The document was withdrawn.

R5-243223	Addition of delta TIBc for R16 inter-band EN-DC within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1789  Cat: F (Rel-18)

					Source: KT Corp.

Discussion:

date

r1

Decision: 		The document was revised to R5-243666.

R5-243666	Addition of delta TIBc for R16 inter-band EN-DC within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1789  rev 1 Cat: F (Rel-18)

					Source: KT Corp.

(Replaces R5-243223)

Decision: 		The document was agreed.

###### 5.3.1.5.2	Rx Requirements (Clause 7)

R5-243225	Addition of delta RIBc for R16 inter-band EN-DC within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1790  Cat: F (Rel-18)

					Source: KT Corp.

Discussion:

date

r1

Decision: 		The document was revised to R5-243667.

R5-243667	Addition of delta RIBc for R16 inter-band EN-DC within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1790  rev 1 Cat: F (Rel-18)

					Source: KT Corp.

(Replaces R5-243225)

Decision: 		The document was agreed.

R5-243229	Addition of minimum requirements of reference sensitivity exceptions for R16 inter-band EN-DC within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1791  Cat: F (Rel-18)

					Source: KT Corp.

Discussion:

Tdoc, date!

r1

Decision: 		The document was revised to R5-243668.

R5-243668	Addition of minimum requirements of reference sensitivity exceptions for R16 inter-band EN-DC within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1791  rev 1 Cat: F (Rel-18)

					Source: KT Corp.

(Replaces R5-243229)

Decision: 		The document was agreed.

###### 5.3.1.5.3	Clauses 1-5, Annexes

R5-242452	 Update for additional ENDC band configurations

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1760  Cat: F (Rel-18)

					Source: Verizon Spain

Decision: 		The document was agreed.

R5-243231	Update of R16 Configuration for inter-band EN-DC

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1792  Cat: F (Rel-18)

					Source: KT Corp.

Discussion:

Tdoc, date

r1

Decision: 		The document was revised to R5-243669.

R5-243669	Update of R16 Configuration for inter-band EN-DC

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1792  rev 1 Cat: F (Rel-18)

					Source: KT Corp.

(Replaces R5-243231)

Decision: 		The document was agreed.

##### 5.3.1.6	TS 38.521-4

###### 5.3.1.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.1.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.1.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.1.6.4	Clauses 1-4, Annexes

##### 5.3.1.7	TS 38.522

R5-243041	Update to R16 NR CADC configuration test cases applicability

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0426  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was revised to R5-243714.

R5-243714	Update to R16 NR CADC configuration test cases applicability

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0426  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243041)

Discussion:

"Revised from: R5-243041r1.

withdrawn since TC CRs are withdrawn"

no R16 CADC applicabilities needed to be added in TS38.522 in this meeting.

Decision: 		The document was withdrawn.

##### 5.3.1.8	TS 38.533

##### 5.3.1.9	TR 38.903 (NR MU & TT analyses)

##### 5.3.1.10	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-242245	Addition of reference sensitivity TP analysis for CA_n28A-n40A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0882  Cat: F (Rel-18)

					Source: KDDI Corporation

Abstract:

Test Case CR t-doc

R5-242249

Discussion:

r3

Decision: 		The document was revised to R5-243820.

R5-243820	Addition of reference sensitivity TP analysis for CA_n28A-n40A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0882  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-242245)

Decision: 		The document was agreed.

R5-242536	Addition of TP analysis for CA_n48C

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0890  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-243032	Update reference sensitivity TC for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0901  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was revised to R5-243616.

R5-243616	Update reference sensitivity TC for CA_n3A-n8A-n78A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0901  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-243032)

Decision: 		The document was agreed.

R5-243044	Update REFSENSE TP analysis for CA_n79C

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0902  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

##### 5.3.1.11	Discussion Papers, Work Plan, TC lists

#### 5.3.2	5G V2X with NR sidelink (UID-880069) 5G_V2X_NRSL_eV2XARC-UEConTest

##### 5.3.2.1	TS 38.508-1

###### 5.3.2.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.2.1.2	Test environment for RF (Clauses 5)

###### 5.3.2.1.3	Test environment for RRM (Clause 7)

###### 5.3.2.1.4	Other clauses, Annexes

##### 5.3.2.2	TS 38.508-2

##### 5.3.2.3	TS 38.509

##### 5.3.2.4	TS 38.521-1

###### 5.3.2.4.1	Tx Requirements (Clause 6)

R5-243134	Addition of NR V2X MOP for inter-band concurrent operation

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2821  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP analysis in R5-243135

Decision: 		The document was agreed.

R5-243136	Update of NR V2X MPR test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2822  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243137	Update of NR V2X AMPR test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2823  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was revised to R5-243629.

R5-243629	Update of NR V2X AMPR test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2823  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243137)

Decision: 		The document was agreed.

R5-243138	Addition of NR V2X NR configured transmit power test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2824  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP analysis in R5-243139

Decision: 		The document was agreed.

R5-243140	Update to NR V2X minimum output power test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2825  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243141	Update to NR V2X transmit off power test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2826  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243142	Addition of NR V2X ON/OFF time mask test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2827  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP analysis in R5-243143

Decision: 		The document was agreed.

###### 5.3.2.4.2	Rx Requirements (Clause 7)

###### 5.3.2.4.3	Clauses 1-5, Annexes

R5-243228	Update to Annex F for V2X test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2841  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

Tdoc#, CR#

r1

Decision: 		The document was revised to R5-243630.

R5-243630	Update to Annex F for V2X test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2841  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243228)

Decision: 		The document was agreed.

##### 5.3.2.5	TS 38.521-3

###### 5.3.2.5.1	Tx Requirements (Clause 6)

###### 5.3.2.5.2	Rx Requirements (Clause 7)

###### 5.3.2.5.3	Clauses 1-5, Annexes

##### 5.3.2.6	TS 38.521-4

###### 5.3.2.6.1	V2X Requirements (Clause 11)

###### 5.3.2.6.2	Clauses 1-4, Annexes

##### 5.3.2.7	TS 38.522

R5-243144	Update to test applicability of V2X test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0433  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243226	Update to applicability of V2X test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0435  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

##### 5.3.2.8	TS 38.533

##### 5.3.2.9	TS 36.509

##### 5.3.2.10	TR 38.903 (NR MU & TT analyses)

##### 5.3.2.11	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-243135	Addition of TP analysis for NR V2X MOP

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0906  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-243134

Decision: 		The document was agreed.

R5-243139	Addition of TP analysis for NR V2X Configured transmit power

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0907  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-243138

Decision: 		The document was agreed.

R5-243143	Addition of TP analysis for NR V2X ON OFF time mask

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0908  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-243142

Decision: 		The document was agreed.

##### 5.3.2.12	Discussion Papers, Work Plan, TC lists

#### 5.3.3	Rel-17 NR CA and DC; and NR and LTE DC Configurations (UID-900056) NR_CADC_NR_LTE_DC_R17-UEConTest

##### 5.3.3.1	TS 38.508-1

###### 5.3.3.1.1	Test frequencies (Clause 4.3.1)

R5-242241	Addition of test frequencies for CA_n3A-n28A-n41A-n77A

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3113  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-242460	Update for additional NRCA band configurations

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3137  Cat: F (Rel-18)

					Source: Verizon, Ericsson

Decision: 		The document was agreed.

R5-242707	Definition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A applicability to protocol testing

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2780  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was withdrawn.

R5-242715	Correction of test bandwidth for CA_n2(2A) protocol testing

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3170  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T, Anritsu

Discussion:

reissued as R5-243654 because of title change.

Decision: 		The document was withdrawn.

R5-243654	Correction of test bandwidth for CA_n2(2A)

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3214  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T, Anritsu

Abstract:

reissued from R5-242715 because of title change.

Decision: 		The document was agreed.

###### 5.3.3.1.2	Test environment for RF (Clauses 5)

###### 5.3.3.1.3	Test environment for RRM (Clause 7)

###### 5.3.3.1.4	Other clauses, Annexes

##### 5.3.3.2	TS 38.508-2

R5-242243	Addition of UE capability for CA_n3A-n28A-n41A-n77A

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0611  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-242461	Update for additional NRCA band configurations

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0623  Cat: F (Rel-18)

					Source: Verizon Spain

Discussion:

r2

Decision: 		The document was revised to R5-243608.

R5-243608	Update for additional NRCA band configurations

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0623  rev 1 Cat: F (Rel-18)

					Source: Verizon Spain

(Replaces R5-242461)

Decision: 		The document was agreed.

R5-242709	Addition of capability declaration for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0628  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Discussion:

r1

Decision: 		The document was revised to R5-243811.

R5-243811	Addition of capability declaration for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0628  rev 1 Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

(Replaces R5-242709)

Decision: 		The document was agreed.

##### 5.3.3.3	TS 38.521-1

###### 5.3.3.3.1	Tx Requirements (Clause 6)

R5-242248	Addition of delta TIBc for CA_n3A-n28A-n41A-n77A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2737  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

r2

Decision: 		The document was revised to R5-243822.

R5-243822	Addition of delta TIBc for CA_n3A-n28A-n41A-n77A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2737  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-242248)

Decision: 		The document was agreed.

R5-242705	Addition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A Tx test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2778  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was revised to R5-243655.

R5-243655	Addition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A Tx test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2778  rev 1 Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

(Replaces R5-242705)

Decision: 		The document was agreed.

R5-243257	Correction of spurious emission minimum conformance requirements for UE co-ex for intra-band contiguous CA

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2843  Cat: F (Rel-18)

					Source: Apple, Keysight

Discussion:

r1

Decision: 		The document was revised to R5-243633.

R5-243633	Correction of spurious emission minimum conformance requirements for UE co-ex for intra-band contiguous CA

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2843  rev 1 Cat: F (Rel-18)

					Source: Apple, Keysight

(Replaces R5-243257)

Decision: 		The document was agreed.

R5-243261	Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-17

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2847  Cat: F (Rel-18)

					Source: Apple, Keysight

Discussion:

r1

Decision: 		The document was revised to R5-243634.

R5-243634	Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-17

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2847  rev 1 Cat: F (Rel-18)

					Source: Apple, Keysight

(Replaces R5-243261)

Decision: 		The document was agreed.

###### 5.3.3.3.2	Rx Requirements (Clause 7)

R5-242250	Addition of delta RIBc and reference sensitivity for CA_n3A-n28A-n41A-n77A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2739  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-242375	Correction to reference sensitivity test requirements for CA_n25A-n77A, CA_n25A-n78A and CA_n66A-n78A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2750  Cat: F (Rel-18)

					Source: Nokia

Discussion:

r1

Decision: 		The document was revised to R5-243635.

R5-243635	Correction to reference sensitivity test requirements for CA_n25A-n77A, CA_n25A-n78A and CA_n66A-n78A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2750  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242375)

Decision: 		The document was agreed.

R5-242463	Update for additional NRCA band configurations

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2757  Cat: F (Rel-18)

					Source: Verizon

Decision: 		The document was agreed.

R5-242706	Addition of reference sensitivity tests for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2779  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Discussion:

r1

Decision: 		The document was revised to R5-243656.

R5-243656	Addition of reference sensitivity tests for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2779  rev 1 Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

(Replaces R5-242706)

Decision: 		The document was agreed.

R5-243022	Correction to LCRB of CA_n5-n48-n66 in 7.3A.0.5

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2800  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-24xxxxx

Discussion:

r1

Decision: 		The document was revised to R5-243834.

R5-243834	Correction to LCRB of CA_n5-n48-n66 in 7.3A.0.5

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2800  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-243022)

Decision: 		The document was agreed.

###### 5.3.3.3.3	Clauses 1-5, Annexes

R5-242573	General updates of TS 38.521-1 clause 5 for R17 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2776  Cat: F (Rel-18)

					Source: China Unicom, Verizon, KDDI, WE Certification Oy, AT&T

Decision: 		The document was revised to R5-243636.

R5-243636	General updates of TS 38.521-1 clause 5 for R17 CA configurations

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2776  rev 1 Cat: F (Rel-18)

					Source: China Unicom, Verizon, KDDI, WE Certification Oy, AT&T

(Replaces R5-242573)

Decision: 		The document was agreed.

R5-243009	Correction to note for inter-band CA including n48

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2795  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

##### 5.3.3.4	TS 38.521-2

###### 5.3.3.4.1	Tx Requirements (Clause 6)

###### 5.3.3.4.2	Rx Requirements (Clause 7)

###### 5.3.3.4.3	Clauses 1-5, Annexes

##### 5.3.3.5	TS 38.521-3

###### 5.3.3.5.1	Tx Requirements (Clause 6)

###### 5.3.3.5.2	Rx Requirements (Clause 7)

R5-242716	Correction of Reference Sensitivity test for DC_30A-n77A

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1763  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T, Anritsu

Discussion:

r1

Decision: 		The document was revised to R5-243670.

R5-243670	Correction of Reference Sensitivity test for DC_30A-n77A

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1763  rev 1 Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T, Anritsu

(Replaces R5-242716)

Decision: 		The document was agreed.

###### 5.3.3.5.3	Clauses 1-5, Annexes

##### 5.3.3.6	TS 38.521-4

###### 5.3.3.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.3.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.3.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.3.6.4	Clauses 1-4, Annexes

##### 5.3.3.7	TS 38.522

R5-243042	Update to R17 NR CADC configuration test cases applicability

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0427  Cat: F (Rel-18)

					Source: CMCC

Discussion:

"Author confirmed no overlap/conflicts

no TC CR at this meeting"

Decision: 		The document was withdrawn.

##### 5.3.3.8	TS 38.533

##### 5.3.3.9	TR 38.903 (NR MU & TT analyses)

##### 5.3.3.10	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-242246	Addition of reference sensitivity TP analysis for CA_n3A-n28A-n41A-n77A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0883  Cat: F (Rel-18)

					Source: KDDI Corporation

Abstract:

Test Case CR t-doc

R5-242250

Decision: 		The document was agreed.

R5-242710	Test point analysis for spurious emissions CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0892  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was agreed.

##### 5.3.3.11	Discussion Papers, Work Plan, TC lists

R5-242532	Discussion regarding handling of missing test frequencies for UL CA combinations.

Type: discussion		For: Agreement
					Source: Ericsson

Discussion:

"prop1 seems agreeable to the group , pending questions on how to implement prop1 . E/// to discuss further offline with TF160 /MCC to evaluate prop1a and come back with way fwd in a future meeting. Prop2 is working assumption being followed to update TS38.508-1

Revised to: R5-243605."

Decision: 		The document was revised to R5-243605.

R5-243605	Discussion regarding handling of missing test frequencies for UL CA combinations.

Type: discussion		For: Agreement
					Source: Ericsson

(Replaces R5-242532)

Discussion:

"Revised from: R5-242532.

add tdoc number to the first page.

Update prop2 wording to remove 'UE vendor input'"

Decision: 		The document was noted.

#### 5.3.4	NR-based access to unlicensed spectrum (UID-911003) NR_unlic-UEConTest

##### 5.3.4.1	TS 38.508-1

###### 5.3.4.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.4.1.2	Test environment for RF (Clauses 5)

###### 5.3.4.1.3	Test environment for RRM (Clause 7)

###### 5.3.4.1.4	Other clauses, Annexes

##### 5.3.4.2	TS 38.508-2

##### 5.3.4.3	TS 38.509

##### 5.3.4.4	TS 38.521-1

###### 5.3.4.4.1	Tx Requirements (Clause 6)

R5-242360	Correction to Table 6.F-1

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2744  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-242488	Addition of test case 6.3F.4.4, Aggregate power tolerance for shared spectrum channel access

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2762  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243885.

R5-243885	Addition of test case 6.3F.4.4, Aggregate power tolerance for shared spectrum channel access

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2762  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242488)

Decision: 		The document was agreed.

R5-242490	Updates to editors note in NR-U Tx test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2763  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243646.

R5-243646	Updates to editors note in NR-U Tx test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2763  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242490)

Decision: 		The document was agreed.

R5-242528	Apply interlaced RB allocation in test configuration table in test case 6.4F.2.2

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2771  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

TP analysis in R5-242525

Discussion:

cl. aff.

Decision: 		The document was revised to R5-243623.

R5-243623	Apply interlaced RB allocation in test configuration table in test case 6.4F.2.2

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2771  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242528)

Decision: 		The document was agreed.

R5-242602	Correction to Table 6.F-1

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2777  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R5-243647.

R5-243647	Correction to Table 6.F-1

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2777  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242602)

Decision: 		The document was agreed.

R5-243285	Corrections on 6.2F.3 for UE A-MPR requirements for shared spectrum access

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2849  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

###### 5.3.4.4.2	Rx Requirements (Clause 7)

R5-242485	Addition of test case 7.4F, Maximum input level for shared spectrum channel access

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2761  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243886.

R5-243886	Addition of test case 7.4F, Maximum input level for shared spectrum channel access

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2761  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242485)

Decision: 		The document was agreed.

###### 5.3.4.4.3	Clauses 1-5, Annexes

R5-242491	Addition of MU and TT for new NR-U test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2764  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r2

Decision: 		The document was revised to R5-243884.

R5-243884	Addition of MU and TT for new NR-U test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2764  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242491)

Decision: 		The document was agreed.

R5-242529	Addition of MU and TT for NR-U test case

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2772  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R5-243648.

R5-243648	Addition of MU and TT for NR-U test case

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2772  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242529)

Decision: 		The document was agreed.

##### 5.3.4.5	TS 38.521-3

###### 5.3.4.5.1	Tx Requirements (Clause 6)

###### 5.3.4.5.2	Rx Requirements (Clause 7)

###### 5.3.4.5.3	Clauses 1-5, Annexes

##### 5.3.4.6	TS 38.521-4

###### 5.3.4.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-242492	Addition of test case 6.2A.4.1.1, 4Rx FR1 CQI reporting under AWGN for Scell on band with shared spectrum access for CA (2DLCA)

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0829  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242493	Core spec alignment of general clause for CSI reporting requirements

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0830  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Core spec alignment

Decision: 		The document was revised to R5-243694.

R5-243694	Core spec alignment of general clause for CSI reporting requirements

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0830  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242493)

Decision: 		The document was agreed.

###### 5.3.4.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.4.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.4.6.4	Clauses 1-4, Annexes

##### 5.3.4.7	TS 38.522

R5-242494	Addition of applicability for new NR-U test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0409  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-243395	Update to NR-U test applicability

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0438  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

##### 5.3.4.8	TS 38.533

R5-242606	Completion of NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case including test tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3098  Cat: F (Rel-18)

					Source: Nokia

Discussion:

A few parameters were missed to be modified after completing the TT analysis.

r2

Decision: 		The document was revised to R5-243751.

R5-243751	Completion of NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case including test tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3098  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242606)

Decision: 		The document was agreed.

R5-242607	Completion of NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case including test tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3099  Cat: F (Rel-18)

					Source: Nokia

Abstract:

RAN4

Discussion:

A few parameters were missed to be modified after completing the TT analysis.

r1

Decision: 		The document was revised to R5-243749.

R5-243749	Completion of NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case including test tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3099  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242607)

Decision: 		The document was agreed.

R5-242642	Correction of NR_U SA Intra-frequency measurements test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3113  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

3GU allocation: NR_unlic-UEConTest / AI allocation: NR_FR2_FWA_Bn257_Bn258-UEConTest

Decision: 		The document was withdrawn.

R5-242643	Correction of NR_U SA Inter_RAT measurements test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3114  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

3GU allocation: NR_unlic-UEConTest / AI allocation: NR_FR2_FWA_Bn257_Bn258-UEConTest

Decision: 		The document was agreed.

R5-242644	Correction of NR_U SA L1-RSRP measurements test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3115  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

3GU allocation: NR_unlic-UEConTest / AI allocation: NR_FR2_FWA_Bn257_Bn258-UEConTest

r1

Decision: 		The document was revised to R5-243748.

R5-243748	Correction of NR_U SA L1-RSRP measurements test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3115  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242644)

Discussion:

"Revised from: R5-242644r1.

Document was sent FOR E-MAIL AGREEMENT"

Email agreed

Decision: 		The document was agreed.

R5-242988	Addition of SA FR1 NR-U RSSI minimum conformance requirements

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3175  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-242989	Addition of SA FR1 NR-U RSSI measurement accuracy test case 11.6.5.3

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3176  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-242990	Addition of SA FR1 NR-U SSB based L1-RSRP measurement test case 11.6.4.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3177  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-242991	Addition of SA FR1 NR-U channel occupancy inter-frequency test case 11.6.6.3

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3178  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-243097	Addition of SA FR1 NR-U channel occupancy intra-frequency test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3189  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on RAN4 CR R4-2407387

Decision: 		The document was agreed.

R5-243329	Addition of EN-DC FR1 NR-U Random Access and Handover test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3213  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Decision: 		The document was agreed.

R5-243364	Update to NR-U Cases 10.4.1.1 and 10.4.1.4 including TTs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3225  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependent to R4-2407559

Decision: 		The document was agreed.

R5-243365	Update to NR-U Timing Accuracy Test 10.2.1.1 including TTs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3226  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243366	Update to NR-U Timing Advance Test 10.2.2.1 including TTs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3227  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243367	Update to NR-U L1-RSRP measurement reporting tests 10.4.3.x including TTs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3228  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243368	Update to SCell activation and deactivation tests 10.3.3.x including TTs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3229  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243369	Addition of NR-U EN-DC PSCell addition and release delay test case including TTs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3230  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243370	Addition of NR-U SA FR1 UE transmit timing and timing advance test cases including TTs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3231  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243371	Addition of NR-U SA FR1 Scell activation and deactivation test cases including TTs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3232  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243372	Update to Annex E and F for NR-U RRM tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3233  Cat: F (Rel-18)

					Source: Qualcomm France

Discussion:

r1

Decision: 		The document was revised to R5-243733.

R5-243733	Update to Annex E and F for NR-U RRM tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3233  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243372)

Discussion:

"Revised from: R5-243372r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-243994.

R5-243994	Update to Annex E and F for NR-U RRM tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3233  rev 2 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243733)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-243373	Removal of interruption during SCell operation NR-U tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3234  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependency: R4-2407562 (agreed)

Discussion:

"RAN4 dependency: R4-2407562

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243374	Update to NSA measurement accuracy NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3235  Cat: F (Rel-18)

					Source: Qualcomm France

Discussion:

r1

Decision: 		The document was revised to R5-243734.

R5-243734	Update to NSA measurement accuracy NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3235  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243374)

Discussion:

"Revised from: R5-243374r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-243995.

R5-243995	Update to NSA measurement accuracy NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3235  rev 2 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243734)

Discussion:

Email agreed

Decision: 		The document was agreed.

R5-243375	Update to SA measurement accuracy NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3236  Cat: F (Rel-18)

					Source: Qualcomm France

Discussion:

r1

Decision: 		The document was revised to R5-243735.

R5-243735	Update to SA measurement accuracy NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3236  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243375)

Discussion:

"Revised from: R5-243375r1.

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243376	Update to NSA event-triggered measurement NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3237  Cat: F (Rel-18)

					Source: Qualcomm France

Discussion:

r2

Decision: 		The document was revised to R5-243892.

R5-243892	Update to NSA event-triggered measurement NR-U tests including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3237  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243376)

Decision: 		The document was agreed.

R5-243377	Addition of testing methodology for Shared Spectrum

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3238  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243378	Clean up of pending items in section 10 NR-U tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3239  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243379	Update to Annex H for NR-U

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3240  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243380	Clean up of pending items in section 11 NR-U tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3241  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243381	Editorial update to Annex D for NR-U

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3242  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

Editorial

Decision: 		The document was agreed.

R5-243382	Update to OCNG modeling for NR-U

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3243  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

##### 5.3.4.9	TR 38.903 (NR MU & TT analyses)

R5-242608	Addition of Test Tolerances analysis for NR-U redirection test cases

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0732  Cat: F (Rel-18)

					Source: Nokia

Abstract:

RRM TT

Discussion:

r3

Decision: 		The document was revised to R5-243752.

R5-243752	Addition of Test Tolerances analysis for NR-U redirection test cases

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0732  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242608)

Decision: 		The document was agreed.

R5-243320	Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases 10.4.2.3, 10.4.2.4, 10.4.2.5, 10.4.2.6

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0746  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r2

Decision: 		The document was revised to R5-243893.

R5-243893	Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases 10.4.2.3, 10.4.2.4, 10.4.2.5, 10.4.2.6

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0746  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243320)

Decision: 		The document was agreed.

R5-243321	Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases with DRX 10.4.2.7, 10.4.2.8, 10.4.2.9, 10.4.2.10

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0747  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r2

Decision: 		The document was revised to R5-243894.

R5-243894	Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases with DRX 10.4.2.7, 10.4.2.8, 10.4.2.9, 10.4.2.10

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0747  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243321)

Decision: 		The document was agreed.

R5-243322	Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0748  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r1

Decision: 		The document was revised to R5-243736.

R5-243736	Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0748  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243322)

Discussion:

"Revised from: R5-243322r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-243996.

R5-243996	Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0748  rev 2 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243736)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-243323	Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0749  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r1

Decision: 		The document was revised to R5-243737.

R5-243737	Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0749  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243323)

Discussion:

"Revised from: R5-243323r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-243997.

R5-243997	Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0749  rev 2 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243737)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-243324	Test Tolerance analysis for NR-U RSRQ intra-frequency accuracy test cases 10.5.2.1 and 11.6.2.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0750  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r1

Decision: 		The document was revised to R5-243738.

R5-243738	Test Tolerance analysis for NR-U RSRQ intra-frequency accuracy test cases 10.5.2.1 and 11.6.2.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0750  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243324)

Discussion:

"Revised from: R5-243324r1.

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243325	Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0751  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r1

Decision: 		The document was revised to R5-243739.

R5-243739	Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0751  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243325)

Discussion:

"Revised from: R5-243325r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-243998.

R5-243998	Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0751  rev 2 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243739)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-243326	Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0752  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r1

Decision: 		The document was revised to R5-243740.

R5-243740	Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0752  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243326)

Discussion:

r1

Decision: 		The document was revised to R5-243999.

R5-243999	Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0752  rev 2 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243740)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-243327	Test Tolerance analysis for NR-U SINR inter-frequency accuracy test cases 10.5.3.2 and 11.6.3.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0753  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r1

Decision: 		The document was revised to R5-243741.

R5-243741	Test Tolerance analysis for NR-U SINR inter-frequency accuracy test cases 10.5.3.2 and 11.6.3.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0753  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243327)

Discussion:

"Revised from: R5-243327r1.

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243328	Test Tolerance analysis for NR-U L1-RSRP accuracy test case 10.5.4.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0754  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r1

Decision: 		The document was revised to R5-243742.

R5-243742	Test Tolerance analysis for NR-U L1-RSRP accuracy test case 10.5.4.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0754  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243328)

Discussion:

"Revised from: R5-243328r1.

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243399	Addition of TTs for NR-U intra-frequency measurement reporting test

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0755  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependent to R4-2407559

Discussion:

r1

Decision: 		The document was revised to R5-243895.

R5-243895	Addition of TTs for NR-U intra-frequency measurement reporting test

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0755  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243399)

Decision: 		The document was agreed.

R5-243400	Addition of TTs for NR-U Timing Accuracy Test

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0756  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243401	Addition of TTs for NR-U Timing Advance Test

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0757  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243402	Addition of TTs for NR-U L1-RSRP measurement reporting tests

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0758  Cat: F (Rel-18)

					Source: Qualcomm France

Discussion:

r1

Decision: 		The document was revised to R5-243743.

R5-243743	Addition of TTs for NR-U L1-RSRP measurement reporting tests

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0758  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243402)

Discussion:

"Revised from: R5-243402r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244000.

R5-244000	Addition of TTs for NR-U L1-RSRP measurement reporting tests

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0758  rev 2 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243743)

Discussion:

email agreed

wrong doc was uploaded

Decision: 		The document was revised to R5-244021.

R5-244021	Addition of TTs for NR-U L1-RSRP measurement reporting tests

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0758  rev 3 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-244000)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-243403	Addition of TTs for SCell activation and deactivation tests

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0759  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243404	Addition of TTs for PSCell addition and release test

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0760  Cat: F (Rel-18)

					Source: Qualcomm France

Discussion:

r1

Decision: 		The document was revised to R5-243753.

R5-243753	Addition of TTs for PSCell addition and release test

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0760  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243404)

Decision: 		The document was agreed.

##### 5.3.4.10	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-242429	Update of test case 6.5F.3.3 and 6.5F.2.4.2

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0884  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

updates of Table 4.1.1.6-1 for 6.5F.3.3 and 6.5F.2.4.2

Decision: 		The document was agreed.

R5-242495	Addition of new NR-U test cases

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0886  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242525	TP analysis for NR-U test case 6.4F.2.2, carrier leakage

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0889  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Implemented in R5-242528

Decision: 		The document was agreed.

##### 5.3.4.11	Discussion Papers, Work Plan, TC lists

#### 5.3.5	Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) (UID-930051) ENDC_PC2_R17_xLTE_yNR-UEConTest

##### 5.3.5.1	TS 38.508-2

##### 5.3.5.2	TS 38.521-3

###### 5.3.5.2.1	Tx Requirements (Clause 6)

###### 5.3.5.2.2	Rx Requirements (Clause 7)

###### 5.3.5.2.3	Clauses 1-5, Annexes

##### 5.3.5.3	TS 38.522

##### 5.3.5.4	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-242435	Update reference sensitivity test cases for ENDC band configurations with PC2 UL

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0885  Cat: F (Rel-18)

					Source: Verizon

Abstract:

associated with 38.521-3 CR 1741

Decision: 		The document was agreed.

##### 5.3.5.5	Discussion Papers, Work Plan, TC lists

##### 5.3.6.	Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 (UID-950062) NR_FR2_FWA_Bn257_Bn258-UEConTest

##### 5.3.6.1	TS 38.508-1

##### 5.3.6.2	TS 38.508-2

##### 5.3.6.3	TS 38.521-2

###### 5.3.6.3.1	Tx Requirements (Clause 6)

R5-242266	PC5 FR2 MU - Tx test cases update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1043  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r2

Decision: 		The document was revised to R5-243721.

R5-243721	PC5 FR2 MU - Tx test cases update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1043  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242266)

Decision: 		The document was agreed.

###### 5.3.6.3.2	Rx Requirements (Clause 7)

R5-242267	PC5 FR2 MU - Rx test cases update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1044  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

###### 5.3.6.3.3	Clauses 1-5, Annexes

R5-242268	PC5 FR2 MU - Annex F update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1045  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243722.

R5-243722	PC5 FR2 MU - Annex F update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1045  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242268)

Decision: 		The document was agreed.

##### 5.3.6.4	TS 38.521-4

###### 5.3.6.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.6.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.6.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.6.4.4	Clauses 1-4, Annexes

##### 5.3.6.5	TS 38.522

##### 5.3.6.6	TS 38.533

##### 5.3.6.7	TR 38.903 (NR MU & TT analyses)

R5-242265	PC5 FR2 MU definition in 38.903

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0728  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243723.

R5-243723	PC5 FR2 MU definition in 38.903

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0728  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242265)

Decision: 		The document was agreed.

##### 5.3.6.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.6.9	Discussion Papers, Work Plan, TC lists

R5-242264	Discussion on MU and TT analysis for PC5

Type: discussion		For: Endorsement
					38.521-2 v..
					Source: Keysight Technologies UK Ltd

Abstract:

CRs R5-242265, R5-242266, R5-242267, R5-242268, R5-242269

Discussion:

"Implemented in R5-242265, R5-242266, R5-242267, R5-242268, R5-242269

5/15

Moderator (AT&T): Offline discussions are occurring.

5/21:

Moderator (AT&T): This document can be noted. Proposals 1, 5, 6, 7, and 8 can be endorsed.

Proposals 1, 5, 6, 7, and 8 endorsed"

Decision: 		The document was noted.

#### 5.3.7	NR small data transmissions in INACTIVE state (UID-960072) NR_SmallData_INACTIVE-UEConTest

##### 5.3.7.1	TS 38.508-1

##### 5.3.7.2	TS 38.508-2

##### 5.3.7.3	TS 38.522

##### 5.3.7.4	TS 38.533

R5-242863	Correction to SDT RRM test case 6.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3127  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2409135 -> R4-2410226(Agreed)

Discussion:

"RRM TT, RAN4 dependent CR R4-2409135

5/21 Moderator (E///): Needs to go for email agreement due to RAN4 discussion

Document was sent FOR E-MAIL AGREEMENT

"

r2

is uploaded to align with R4-2410226

Decision: 		The document was revised to R5-244001.

R5-244001	Correction to SDT RRM test case 6.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3127  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242863)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242864	Correction to SDT RRM test case 7.2.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3128  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency

RRM TT R4-2408543

Discussion:

"RRM TT, RAN4 dependency R4-2408543, R4-2409135

Author confirmed no overlap/conflicts

5/21 Moderator (E///): Needs to go for email agreement due to RAN4 discussion

Document was sent FOR E-MAIL AGREEMENT

"

Changes are not agreed in RAN4.

email withdrawn

Decision: 		The document was withdrawn.

##### 5.3.7.5	TR 38.903 (NR MU & TT analyses)

##### 5.3.7.6	Discussion Papers, Work Plan, TC lists

#### 5.3.8	Solutions for NR to support non-terrestrial networks (NTN) (UID-960074) NR_NTN_solutions_plus_CT-UEConTest

##### 5.3.8.1	TS 38.508-1

R5-243286	Corrections on 4.3.1 for NTN FR1 test channel bandwidth

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3207  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

##### 5.3.8.2	TS 38.508-2

R5-243287	Update on A.4.1 and A.4.3.1 for FR1-NTN implementation types and RF baseline capabilities

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0646  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

agreed in the joint on Wed

Decision: 		The document was agreed.

R5-243398	Correction to NTN PICS

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0647  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

##### 5.3.8.3	TS 38.509

##### 5.3.8.4	TS 38.521-5

R5-242500	Introduction of new NR NTN test case - 6.5.1 Occupied bandwidth

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0038  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242501	Introduction of new NR NTN test case - 6.4.2.1 Error Vector Magnitude

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0039  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242502	Introduction of new NR NTN test case - 6.4.2.1a Error Vector Magnitude including symbols with transient period

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0040  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243896.

R5-243896	Introduction of new NR NTN test case - 6.4.2.1a Error Vector Magnitude including symbols with transient period

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0040  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242502)

Decision: 		The document was agreed.

R5-242503	Introduction of new NR NTN test case - 6.4.2.2 Carrier leakage

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0041  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243673.

R5-243673	Introduction of new NR NTN test case - 6.4.2.2 Carrier leakage

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0041  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242503)

Decision: 		The document was agreed.

R5-242504	Introduction of new NR NTN test case - 6.4.2.3 In-band emissions

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0042  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243674.

R5-243674	Introduction of new NR NTN test case - 6.4.2.3 In-band emissions

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0042  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242504)

Decision: 		The document was agreed.

R5-242505	Introduction of new NR NTN test case - 6.4.2.4 EVM equalizer spectrum flatness

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0043  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243675.

R5-243675	Introduction of new NR NTN test case - 6.4.2.4 EVM equalizer spectrum flatness

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0043  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242505)

Decision: 		The document was agreed.

R5-242506	Introduction of new NR NTN test case - 6.4.2.5 EVM equalizer spectrum flatness for Pi2 BPSK

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0044  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242507	Update of NR NTN test case - 6.2.1 UE maximum output power

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0045  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

resolve overlap with R5-243420.

r1

Decision: 		The document was revised to R5-243676.

R5-243676	Update of NR NTN test case - 6.2.1 UE maximum output power

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0045  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242507)

Decision: 		The document was agreed.

R5-242508	Update of NR NTN test case - 6.2.2 UE maximum output power reduction

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0046  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242509	Update of NR NTN test case - 6.2.3 UE additional maximum output power reduction

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0047  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

resolve overlap with R5-243421.

r1

Decision: 		The document was revised to R5-243677.

R5-243677	Update of NR NTN test case - 6.2.3 UE additional maximum output power reduction

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0047  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242509)

Decision: 		The document was agreed.

R5-242510	Update of NR NTN test case - 6.2.4 Configured transmitted power

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0048  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242511	Update MU and TT for NR NTN test cases

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0049  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243678.

R5-243678	Update MU and TT for NR NTN test cases

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0049  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242511)

Decision: 		The document was agreed.

R5-242946	Updates to additional Spurious emissions TC 6.5.3.3

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0050  Cat: F (Rel-18)

					Source: Qualcomm Tech. Netherlands B.V

Abstract:

Dependent on R5-242509 (Update of NR NTN test case - 6.2.3 UE additional maximum output power reduction). The test case 6.5.3.3 updated in this CR points to configuration table in test case 6.2.3.  Upon completion of test case 6.2.3 the editors notes in t

Decision: 		The document was withdrawn.

R5-242947	Updates to additional spurious emissions TC 6.5.3.3

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0051  Cat: F (Rel-18)

					Source: Qualcomm Tech. Netherlands B.V

Abstract:

Dependent on R5-242509 (Update of NR NTN test case - 6.2.3 UE additional maximum output power reduction).

Decision: 		The document was agreed.

R5-242951	Adding new NTN test case 6.3.4 Power control

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0052  Cat: F (Rel-18)

					Source: Qualcomm Tech. Netherlands B.V

Discussion:

r1

Decision: 		The document was revised to R5-243679.

R5-243679	Adding new NTN test case 6.3.4 Power control

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0052  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Tech. Netherlands B.V

(Replaces R5-242951)

Decision: 		The document was agreed.

R5-243288	Corrections on 5.1 for NTN FR1 frequency range designation

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0053  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-243289	Corrections on 6.2.3 for NTN UE additional maximum output power reduction

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0054  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-243290	Corrections on 6.5.3.3 for NTN UE additional spurious emissions

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0055  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-243291	Updates on Annex for satellite access conformance test

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0056  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

r1

Decision: 		The document was revised to R5-243680.

R5-243680	Updates on Annex for satellite access conformance test

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0056  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-243291)

Decision: 		The document was agreed.

R5-243415	Annex F updates for NTN RF test on Tx on-off time mask

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0057  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

r1

Decision: 		The document was revised to R5-243681.

R5-243681	Annex F updates for NTN RF test on Tx on-off time mask

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0057  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-243415)

Decision: 		The document was agreed.

R5-243416	Annex F updates for NTN RF test on Spectrum emission mask

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0058  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Decision: 		The document was revised to R5-243814.

R5-243814	Annex F updates for NTN RF test on Spectrum emission mask

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0058  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-243416)

Decision: 		The document was agreed.

R5-243417	Annex F updates for NTN RF test on ACLR

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0059  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Decision: 		The document was agreed.

R5-243441	Update to nrn-ntn pdsch demod test case to update min test time

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0065  Cat: F (Rel-18)

					Source: Qualcomm Inc

Abstract:

associated discussion paper R5-243431

Discussion:

r1

Decision: 		The document was revised to R5-243899.

R5-243899	Update to nrn-ntn pdsch demod test case to update min test time

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0065  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Inc

(Replaces R5-243441)

Decision: 		The document was agreed.

##### 5.3.8.5	TS 38.522

R5-242512	Update applicability for NR NTN test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0410  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-243292	Corrections on 4.0 for NTN tested bands selection criteria

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0436  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-243394	Addition of applicability table for NR NTN RRM tests

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0437  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243440	Applicability spec update for NR-NTN test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0444  Cat: F (Rel-18)

					Source: Qualcomm Inc

Decision: 		The document was agreed.

##### 5.3.8.6	TS 38.533

R5-243359	Addition of NTN Timing Advance test 14.3.2.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3220  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243360	Addition of NTN distance-based CHO tests 14.2.1.5 and 14.2.1.6

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3221  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243361	Addition of NTN inter-frequency Measurement Initiation Cell Reselection tests 14.1.7 and 14.1.8

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3222  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243362	Addition of NTN L1-RSRP Measurement Accuracy tests 14.6.4.1 and 14.6.4.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3223  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243363	Update to NTN general sections

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3224  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

##### 5.3.8.7	TR 38.903 (NR MU & TT analyses)

R5-243418	MU TT updates for NTN RF tests

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0761  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

reserved in error

Decision: 		The document was withdrawn.

##### 5.3.8.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-242513	Update MU and TT for NR NTN test cases

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0888  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

Ericsson: title shall be: Update Test Point analysis for NR NTN test cases

reissued as R5-243448.

Decision: 		The document was withdrawn.

R5-243448	Update Test Point analysis for NR NTN test cases

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0915  Cat: F (Rel-18)

					Source: Keysight Technologies

Abstract:

reissued from R5-242513 because of title change

Discussion:

check!!

Decision: 		The document was agreed.

##### 5.3.8.9	Discussion Papers, Work Plan, TC lists

R5-242514	On phase continuity requirements for DMRS bundling for NR NTN

Type: discussion		For: Endorsement
					38.521-5 v..
					Source: Keysight Technologies UK Ltd

Discussion:

"No associated CR

noted proposal to send LS is agreed

request LS tdoc# . "

Decision: 		The document was noted.

R5-243431	Simulation results to derive min test time for nr-ntn pdsch demod test cases

Type: discussion		For: Agreement
					38.521-5 v..
					Source: Qualcomm Inc

Decision: 		The document was noted.

#### 5.3.9	Further enhancement on NR demodulation performance (UID-960075) NR_demod_enh2-UEConTest

##### 5.3.9.1	TS 38.508-1

R5-243438	Addition of connection diagram for CSI test cases for inter-cell interference scenarios

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3211  Cat: F (Rel-18)

					Source: Qualcomm Inc

Discussion:

cl. aff.

r1

Decision: 		The document was revised to R5-243900.

R5-243900	Addition of connection diagram for CSI test cases for inter-cell interference scenarios

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3211  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Inc

(Replaces R5-243438)

Decision: 		The document was agreed.

##### 5.3.9.2	TS 38.508-2

##### 5.3.9.3	TS 38.521-4

###### 5.3.9.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-242440	Completion of FDD CQI reporting test cases with inter-cell interference

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0826  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-243691.

R5-243691	Completion of FDD CQI reporting test cases with inter-cell interference

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0826  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-242440)

Decision: 		The document was agreed.

R5-242441	Completion of TDD CQI reporting test cases with inter-cell interference

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0827  Cat: F (Rel-18)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-243692.

R5-243692	Completion of TDD CQI reporting test cases with inter-cell interference

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0827  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-242441)

Decision: 		The document was agreed.

R5-243434	Update to inter-cell CRS interference test case

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0845  Cat: F (Rel-18)

					Source: Qualcomm Inc

Decision: 		The document was agreed.

###### 5.3.9.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.9.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.9.3.4	Clauses 1-4, Annexes

R5-242442	Addition of MU and TT for CQI reporting test cases with inter-cell interference

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0828  Cat: F (Rel-18)

					Source: China Telecom

Decision: 		The document was revised to R5-243693.

R5-243693	Addition of MU and TT for CQI reporting test cases with inter-cell interference

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0828  rev 1 Cat: F (Rel-18)

					Source: China Telecom

(Replaces R5-242442)

Decision: 		The document was agreed.

R5-243435	Corrections to the FRC Table A.3.2.2.2-29

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0846  Cat: F (Rel-18)

					Source: Qualcomm Inc

Discussion:

"RAN4 dependency R4-2407405

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

R4-2407405 was revised to R4-2409799, , but no change to the technical content.

Email agreed

Decision: 		The document was agreed.

##### 5.3.9.4	TS 38.522

R5-242443	Correction of test applicability rule for CQI reporting test cases with inter-cell interference

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0408  Cat: F (Rel-18)

					Source: China Telecom

Decision: 		The document was agreed.

R5-243437	Applicability spec update for inter-cell CRS interference test case

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0442  Cat: F (Rel-18)

					Source: Qualcomm Inc

Decision: 		The document was agreed.

##### 5.3.9.5	TR 38.903 (NR MU & TT analyses)

R5-242439	TT analysis for FR1 CQI reporting test cases with inter-cell interference

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0730  Cat: F (Rel-18)

					Source: China Telecom, Qualcomm

Decision: 		The document was agreed.

R5-243433	Addition of MU/TT analysis for inter-cell CRS interference

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0762  Cat: F (Rel-18)

					Source: Qualcomm Inc

Decision: 		The document was agreed.

##### 5.3.9.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.9.7	Discussion Papers, Work Plan, TC lists

#### 5.3.10	Further enhancements on MIMO for NR (UID-960079) NR_feMIMO-UEConTest

##### 5.3.10.1	TS 38.508-1

##### 5.3.10.2	TS 38.508-2

##### 5.3.10.3	TS 38.521-4

###### 5.3.10.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-243150	Update to HST scheme A and B test cases

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0840  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

###### 5.3.10.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.10.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.10.3.4	Clauses 1-4, Annexes

R5-242668	Additional annex Channel Profile of FR2 HST-DPS

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0831  Cat: F (Rel-18)

					Source: Samsung

Decision: 		The document was agreed.

R5-243151	Adding RMC for feMIMO test cases

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0841  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Samsung

Decision: 		The document was agreed.

##### 5.3.10.4	TS 38.522

R5-242610	Editorial Correction of FR2 feMIMO test case

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0413  Cat: F (Rel-18)

					Source: Nokia

Discussion:

spec! 18.2.1!

r1

Decision: 		The document was revised to R5-243715.

R5-243715	Editorial Correction of FR2 feMIMO test case

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0413  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242610)

Decision: 		The document was agreed.

R5-242669	Additional applicability for Inter-cell SSB based L1-RSRP measurements

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0415  Cat: F (Rel-18)

					Source: Samsung

Discussion:

r1

Decision: 		The document was revised to R5-243716.

R5-243716	Additional applicability for Inter-cell SSB based L1-RSRP measurements

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0415  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-242669)

Decision: 		The document was agreed.

R5-242725	Correction condition C111 for 38.521-1 TC6.4.2.5 Rel-16

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0418  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

had wrong WIC in 3GU.

Offline comments from Huawei

Decision: 		The document was withdrawn.

R5-242846	Correction to Test Bands Selection Condition for Performance test case 5.3.2.2.5

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0421  Cat: F (Rel-18)

					Source: TTA

Decision: 		The document was agreed.

R5-243152	Update to applicability of FeMIMO test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0434  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.10.5	TS 38.533

R5-242609	Editorial correction to 7.6.3.6 test case

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3100  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

R5-242670	Addition test case for Inter-cell SSB based L1-RSRP Reporting

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3118  Cat: F (Rel-18)

					Source: Samsung

Discussion:

r2

Decision: 		The document was revised to R5-243880.

R5-243880	Addition test case for Inter-cell SSB based L1-RSRP Reporting

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3118  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-242670)

Decision: 		The document was agreed.

R5-242671	Adding TRP specific Minimum conformance requirements

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3119  Cat: F (Rel-18)

					Source: Samsung

Decision: 		The document was revised to R5-243705.

R5-243705	Adding TRP specific Minimum conformance requirements

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3119  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-242671)

Decision: 		The document was agreed.

R5-243153	Update to Unified TCI state test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3190  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Samsung

Decision: 		The document was revised to R5-243703.

R5-243703	Update to Unified TCI state test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3190  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Samsung

(Replaces R5-243153)

Decision: 		The document was agreed.

R5-243154	Correcting propagation AWGN in test cases without Noc

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3191  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

' ' '

r1

Decision: 		The document was revised to R5-243704.

R5-243704	Correcting propagation AWGN in test cases without Noc

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3191  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243154)

Decision: 		The document was agreed.

##### 5.3.10.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.10.7	Discussion Papers, Work Plan, TC lists

#### 5.3.11	NR support for high speed train scenario in frequency range 2 (FR2) (UID-960080) NR_HST_FR2-UEConTest

##### 5.3.11.1	TS 38.508-1

##### 5.3.11.2	TS 38.508-2

##### 5.3.11.3	TS 38.521-2

###### 5.3.11.3.1	Tx Requirements (Clause 6)

R5-242672	Additional test case for Enhanced Beam correspondence for PC6

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1046  Cat: F (Rel-18)

					Source: Samsung

Discussion:

r1

Decision: 		The document was revised to R5-243652.

R5-243652	Additional test case for Enhanced Beam correspondence for PC6

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1046  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-242672)

Decision: 		The document was agreed.

###### 5.3.11.3.2	Rx Requirements (Clause 7)

###### 5.3.11.3.3	Clauses 1-5, Annexes

##### 5.3.11.4	TS 38.521-4

###### 5.3.11.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.11.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

R5-242673	Additional demodulation test case for FR2 high speed train

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0832  Cat: F (Rel-18)

					Source: Samsung

Discussion:

RAN5!

r1

Decision: 		The document was revised to R5-243881.

R5-243881	Additional demodulation test case for FR2 high speed train

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0832  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-242673)

Decision: 		The document was agreed.

###### 5.3.11.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.11.4.4	Clauses 1-4, Annexes

##### 5.3.11.5	TS 38.522

R5-242674	Additional applicability for HST FR2 test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0416  Cat: F (Rel-18)

					Source: Samsung

Discussion:

r1

Decision: 		The document was revised to R5-243717.

R5-243717	Additional applicability for HST FR2 test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0416  rev 1 Cat: F (Rel-18)

					Source: Samsung

(Replaces R5-242674)

Decision: 		The document was agreed.

##### 5.3.11.6	TS 38.533

R5-242612	Addition of cell configuration mapping for HST FR2 test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3102  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 5.3.11.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.11.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.11.9	Discussion Papers, Work Plan, TC lists

#### 5.3.12	NR Sidelink Relay (UID-960083) NR_SL_relay-UEConTest

##### 5.3.12.1	TS 38.508-1

##### 5.3.12.2	TS 38.508-2

##### 5.3.12.3	TS 38.521-1

###### 5.3.12.3.1	Tx Requirements (Clause 6)

###### 5.3.12.3.2	Rx Requirements (Clause 7)

###### 5.3.12.3.3	Clauses 1-5, Annexes

##### 5.3.12.4	TS 38.522

##### 5.3.12.5	TS 38.533

##### 5.3.12.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.12.7	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.12.8	Discussion Papers, Work Plan, TC lists

#### 5.3.13	NR Sidelink enhancement (UID-960084) NR_SL_enh-UEConTest

##### 5.3.13.1	TS 38.508-1

##### 5.3.13.2	TS 38.508-2

##### 5.3.13.3	TS 38.521-1

###### 5.3.13.3.1	Tx Requirements (Clause 6)

R5-243145	Addition of NR V2X MOP minimum requirement for sidelink enhancement

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2828  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243146	Update to NR V2X MPR test cases to add PC2 and intra-band concurrent operation

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2829  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP analysis in R5-243147

Decision: 		The document was agreed.

R5-243148	Update to NR V2X AMPR test cases to add PC2 and intra-band concurrent operation

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2830  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP analysis in R5-243149

Decision: 		The document was agreed.

###### 5.3.13.3.2	Rx Requirements (Clause 7)

###### 5.3.13.3.3	Clauses 1-5, Annexes

##### 5.3.13.4	TS 38.522

##### 5.3.13.5	TS 38.533

##### 5.3.13.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.13.7	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-243147	Addition of TP analysis for NR V2X MPR for PC2 and intra-band concurrent operation

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0909  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-243146

Discussion:

V2X WIC??

r2

Decision: 		The document was revised to R5-243621.

R5-243621	Addition of TP analysis for NR V2X MPR for PC2 and intra-band concurrent operation

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0909  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243147)

Decision: 		The document was agreed.

R5-243149	Addition of TP analysis for NR V2X AMPR for PC2 and intra-band concurrent operation

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0910  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-243148

Discussion:

V2X WIC??

r1

Decision: 		The document was revised to R5-243622.

R5-243622	Addition of TP analysis for NR V2X AMPR for PC2 and intra-band concurrent operation

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0910  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243149)

Decision: 		The document was agreed.

##### 5.3.13.8	Discussion Papers, Work Plan, TC lists

#### 5.3.14	NR RRM enhancement (UID-960089) NR_RRM_enh-UEConTest

##### 5.3.14.1	TS 38.508-1

##### 5.3.14.2	TS 38.508-2

##### 5.3.14.3	TS 38.522

R5-242645	Applicabilty correction of RRM enhancements test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0414  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243718.

R5-243718	Applicabilty correction of RRM enhancements test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0414  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242645)

Decision: 		The document was agreed.

R5-242694	Addition of test applicability for Rel-16 RRM EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0417  Cat: F (Rel-18)

					Source: Sporton

Discussion:

r1

Decision: 		The document was revised to R5-243719.

R5-243719	Addition of test applicability for Rel-16 RRM EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0417  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-242694)

Decision: 		The document was agreed.

##### 5.3.14.4	TS 38.533

R5-242632	Correction of interruptions at NR SRS carrier based switching test case 4.5.2.8 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3103  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243754.

R5-243754	Correction of interruptions at NR SRS carrier based switching test case 4.5.2.8 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3103  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242632)

Decision: 		The document was agreed.

R5-242633	Correction of interruptions at E-UTRA SRS carrier based switching test case 4.5.2.9 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3104  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243755.

R5-243755	Correction of interruptions at E-UTRA SRS carrier based switching test case 4.5.2.9 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3104  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242633)

Decision: 		The document was agreed.

R5-242634	Correction of FR2 interruptions at SRS carrier based switching test case 5.5.2.7 and 5.5.2.8

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3105  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243706.

R5-243706	Correction of FR2 interruptions at SRS carrier based switching test case 5.5.2.7 and 5.5.2.8

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3105  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242634)

Decision: 		The document was agreed.

R5-242635	Correction of SA FR1 interruptions at NR SRS carrier based switching test case 6.5.2.2 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3106  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243756.

R5-243756	Correction of SA FR1 interruptions at NR SRS carrier based switching test case 6.5.2.2 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3106  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242635)

Decision: 		The document was agreed.

R5-242636	Correction of NR SA event triggered reporting tests with additional mandatory gap pattern test case 6.6.2.9 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3107  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242637	Correction of NR SA event triggered reporting tests for FR1 without gap when DRX is used test case 6.6.2.10 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3108  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242638	Correction of NR SA event triggered reporting tests for FR1 without gap when DRX is not used test case 6.6.2.11 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3109  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242639	Addition of FR2 SA event triggered reporting test without gap under non-DRX test case 7.6.2.10 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3110  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243757.

R5-243757	Addition of FR2 SA event triggered reporting test without gap under non-DRX test case 7.6.2.10 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3110  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242639)

Discussion:

"Revised from: R5-242639r1.

Verdict chage from P.Agreed to Deferred as Author requested and also add RAN4 dependent CR R4-2409233

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-242640	Addition of FR2 SA event triggered reporting test without gap under DRX test case 7.6.2.11 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3111  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243758.

R5-243758	Addition of FR2 SA event triggered reporting test without gap under DRX test case 7.6.2.11 including Test Tolerance

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3111  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242640)

Discussion:

"Revised from: R5-242640r1.

Verdict chage from P.Agreed to Deferred as Author requested and also add RAN4 dependent CR R4-2409233

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-242641	Test Tolerance correction in Annex F for RRM enhancement test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3112  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

"5/21 Moderator (E///): good to go

Author confirmed no overlap

Verdict chage from P.Agreed to Deferred as Author requested and also add RAN4 dependent CR R4-2409233

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-242646	Cell configuration correction in Annex E for SA FR2 RRM enhancement test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3116  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242647	Minimum conformance requirements for FR2 SA event triggered reporting without gap

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3117  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242693	Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3120  Cat: F (Rel-18)

					Source: Sporton

Decision: 		The document was withdrawn.

R5-242695	Addition of new test case 5.6.7.1 EN-DC event triggered reporting test without gap under non-DRX

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3121  Cat: F (Rel-18)

					Source: Sporton

Discussion:

r1

Decision: 		The document was revised to R5-243707.

R5-243707	Addition of new test case 5.6.7.1 EN-DC event triggered reporting test without gap under non-DRX

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3121  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-242695)

Decision: 		The document was agreed.

R5-242696	Addition of new test case 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3122  Cat: F (Rel-18)

					Source: Sporton

Discussion:

r1

Decision: 		The document was revised to R5-243708.

R5-243708	Addition of new test case 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3122  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-242696)

Decision: 		The document was agreed.

R5-242697	Addition of new test case 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3123  Cat: F (Rel-18)

					Source: Sporton

Discussion:

r1

Decision: 		The document was revised to R5-243709.

R5-243709	Addition of new test case 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3123  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-242697)

Decision: 		The document was agreed.

R5-242698	Addition of new test case 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3124  Cat: F (Rel-18)

					Source: Sporton

Discussion:

r1

Decision: 		The document was revised to R5-243710.

R5-243710	Addition of new test case 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3124  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-242698)

Decision: 		The document was agreed.

R5-242699	Addition of MU and test tolerance in Annex F for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3125  Cat: F (Rel-18)

					Source: Sporton

Discussion:

"Author confirmed no overlap/conflicts

5/21 Moderator (E///): ongoing discussion

5/22 Moderator (E///): needs to go for email for further check

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244002.

R5-244002	Addition of MU and test tolerance in Annex F for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3125  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-242699)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242915	Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3166  Cat: F (Rel-18)

					Source: Sporton

Discussion:

file '&'

r2

Decision: 		The document was revised to R5-243732.

R5-243732	Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3166  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-242915)

Discussion:

"Revised from: R5-242915r2.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244003.

R5-244003	Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3166  rev 2 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-243732)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-243235	Addition of Clause 4.5.6.4.0 minimum conformance requirements for simultaneous RRC-based active BWP switch on multiple CCs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3192  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-243236	Addition of Clause 4.6.8.0 minimum conformance requirements for CSI-RS based intra-frequency measurement

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3193  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-243237	Addition of Clause 4.6.9.0 minimum conformance requirements for CSI-RS based inter-frequency measurement

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3194  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-243238	Addition of Clause 5.5.3.0 minimum conformance requirements for SCell activation and deactivation delay

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3195  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-243239	Addition of Clause 5.5.6.5.0 minimum conformance requirements for simultaneous RRC-based active BWP switch on multiple CC

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3196  Cat: F (Rel-18)

					Source: Apple

Decision: 		The document was agreed.

R5-243240	Addition of message contents to test case 4.7.9.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3197  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243815.

R5-243815	Addition of message contents to test case 4.7.9.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3197  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243240)

Decision: 		The document was agreed.

R5-243241	Addition of message contents to test case 4.7.9.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3198  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243842.

R5-243842	Addition of message contents to test case 4.7.9.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3198  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243241)

Decision: 		The document was agreed.

R5-243242	Addition of message contents to test case 4.7.10.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3199  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243843.

R5-243843	Addition of message contents to test case 4.7.10.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3199  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243242)

Decision: 		The document was agreed.

R5-243243	Addition of message contents to test case 4.7.10.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3200  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243844.

R5-243844	Addition of message contents to test case 4.7.10.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3200  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243243)

Decision: 		The document was agreed.

R5-243244	Addition of message contents to test case 5.7.8.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3201  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243845.

R5-243845	Addition of message contents to test case 5.7.8.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3201  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243244)

Decision: 		The document was agreed.

R5-243245	Addition of message contents to test case 5.7.8.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3202  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243846.

R5-243846	Addition of message contents to test case 5.7.8.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3202  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243245)

Decision: 		The document was agreed.

R5-243246	Addition of message contents to test case 5.7.9.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3203  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243847.

R5-243847	Addition of message contents to test case 5.7.9.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3203  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243246)

Decision: 		The document was agreed.

R5-243247	Addition of message contents to test case 5.7.9.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3204  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243848.

R5-243848	Addition of message contents to test case 5.7.9.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3204  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243247)

Decision: 		The document was agreed.

R5-243248	Addition of message contents to test case 6.7.11.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3205  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243849.

R5-243849	Addition of message contents to test case 6.7.11.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3205  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243248)

Decision: 		The document was agreed.

R5-243249	Addition of message contents to test case 6.7.11.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3206  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243850.

R5-243850	Addition of message contents to test case 6.7.11.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3206  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243249)

Decision: 		The document was agreed.

R5-243250	Addition of message contents to test case 6.7.12.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3207  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243851.

R5-243851	Addition of message contents to test case 6.7.12.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3207  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243250)

Decision: 		The document was agreed.

R5-243251	Addition of message contents to test case 6.7.12.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3208  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243852.

R5-243852	Addition of message contents to test case 6.7.12.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3208  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243251)

Decision: 		The document was agreed.

R5-243252	Addition of message contents to test case 7.7.8.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3209  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243853.

R5-243853	Addition of message contents to test case 7.7.8.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3209  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243252)

Decision: 		The document was agreed.

R5-243253	Addition of message contents to test case 7.7.8.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3210  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243854.

R5-243854	Addition of message contents to test case 7.7.8.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3210  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243253)

Decision: 		The document was agreed.

R5-243254	Addition of message contents to test case 7.7.9.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3211  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243855.

R5-243855	Addition of message contents to test case 7.7.9.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3211  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243254)

Decision: 		The document was agreed.

R5-243255	Addition of message contents to test case 7.7.9.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3212  Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

Discussion:

r1

Decision: 		The document was revised to R5-243856.

R5-243856	Addition of message contents to test case 7.7.9.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3212  rev 1 Cat: F (Rel-18)

					Source: Apple, Huawei, Hisilicon

(Replaces R5-243255)

Decision: 		The document was agreed.

##### 5.3.14.5	TR 38.903 (NR MU & TT analyses)

R5-242629	Test Tolerance analysis for SRS carrier based switching test cases

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0733  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242630	Test Tolerance analysis for event triggered reporting test without gap under non-DRX test case 7.6.2.10

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0734  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243759.

R5-243759	Test Tolerance analysis for event triggered reporting test without gap under non-DRX test case 7.6.2.10

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0734  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242630)

Discussion:

"Revised from: R5-242630r1.

Verdict chage from P.Agreed to Deferred as Author requested and also add RAN4 dependent CR R4-2409233

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-242631	Test Tolerance analysis for event triggered reporting test without gap under DRX test case 7.6.2.11

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0735  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

"5/21 Moderator (E///): good to go

Verdict chage from P.Agreed to Deferred as Author requested and also add RAN4 dependent CR R4-2409233

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-242700	Addition of test tolerance analysis for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0736  Cat: F (Rel-18)

					Source: Sporton

Discussion:

"5/21 Moderator (E///): ongoing discussion

5/22 Moderator (E///): needs to go for email for further check

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244004.

R5-244004	Addition of test tolerance analysis for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0736  rev 1 Cat: F (Rel-18)

					Source: Sporton

(Replaces R5-242700)

Discussion:

email agreed

Decision: 		The document was agreed.

##### 5.3.14.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.14.7	Discussion Papers, Work Plan, TC lists

##### 5.3.15.	NR Positioning Enhancements  (UID-970075) NR_pos_enh-UEConTest

##### 5.3.15.1	TS 38.508-1

###### 5.3.15.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.15.1.2	Test environment for RF (Clauses 5)

###### 5.3.15.1.3	Test environment for RRM (Clause 7)

###### 5.3.15.1.4	Other clauses, Annexes

##### 5.3.15.2	TS 37.571-1

R5-242123	Addition of Test Case 14.2.6

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0472  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243803.

R5-243803	Addition of Test Case 14.2.6

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0472  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242123)

Decision: 		The document was agreed.

R5-242193	Update TC 14.2.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0475  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243760.

R5-243760	Update TC 14.2.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0475  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242193)

Decision: 		The document was agreed.

R5-242194	Update TC 14.2.9 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0476  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243761.

R5-243761	Update TC 14.2.9 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0476  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242194)

Decision: 		The document was agreed.

R5-242195	Update TC 14.2.10 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0477  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243762.

R5-243762	Update TC 14.2.10 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0477  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242195)

Decision: 		The document was agreed.

R5-242198	Update TC 14.3.7 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0480  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243763.

R5-243763	Update TC 14.3.7 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0480  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242198)

Decision: 		The document was agreed.

R5-242199	Update TC 14.3.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0481  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243764.

R5-243764	Update TC 14.3.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0481  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242199)

Decision: 		The document was agreed.

R5-242204	Update TC 15.2.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0486  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243765.

R5-243765	Update TC 15.2.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0486  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242204)

Decision: 		The document was agreed.

R5-242205	Update TC 15.2.9 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0487  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243766.

R5-243766	Update TC 15.2.9 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0487  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242205)

Decision: 		The document was agreed.

R5-242206	Update TC 15.2.10 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0488  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243767.

R5-243767	Update TC 15.2.10 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0488  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242206)

Decision: 		The document was agreed.

R5-242209	Update TC 15.3.5 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0491  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242210	Update TC 15.3.6 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0492  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242213	Update TC 16.2.7 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0495  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242214	Update TC 16.2.8 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0496  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242216	Update TC 16.3.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0498  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242217	Update NR PRS-based measurement requirements in annex based on the updated TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0499  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243768.

R5-243768	Update NR PRS-based measurement requirements in annex based on the updated TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0499  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242217)

Decision: 		The document was agreed.

R5-242471	Addition of new UE Rx-Tx time difference reporting delay test case 15.4.1 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0500  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242472	Addition of new UE Rx-Tx time difference reporting delay test case 15.4.2 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0501  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242473	Addition of new UE Rx-Tx time difference reporting delay test case 15.4.3 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0502  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242474	Addition of new UE Rx-Tx time difference reporting delay test case 15.4.4 in RRC_INACTIVE state

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0503  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242475	Addition of new NR PRS-RSRPP measurement period test case 17.4.1 in RRC_INACTIVE

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0504  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242476	Addition of new NR PRS-RSRPP measurement period test case 17.4.2 in RRC_INACTIVE

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0505  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242477	Addition of new NR PRS-RSRPP measurement period test case 17.4.3 in RRC_INACTIVE

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0506  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242478	Addition of new NR PRS-RSRPP measurement period test case 17.4.4 in RRC_INACTIVE

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0507  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242479	Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.1 in RRC_INACTIVE

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0508  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242480	Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.2 in RRC_INACTIVE

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0509  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242481	Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.3 in RRC_INACTIVE

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0510  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242482	Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.4 in RRC_INACTIVE

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0511  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242613	Addition of Test Case: UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0512  Cat: F (Rel-17)

					Source: Nokia

Discussion:

Add the missing message contents tables which are different from test case 15.5.3.

r1

Decision: 		The document was revised to R5-243804.

R5-243804	Addition of Test Case: UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0512  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-242613)

Decision: 		The document was agreed.

R5-242614	Addition of UE Rx-Tx time difference measurements in RRC_INACTIVE test case

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0513  Cat: F (Rel-17)

					Source: Nokia

Discussion:

Correct the PRS-related configuration for two different subtests.

r1

Decision: 		The document was revised to R5-243805.

R5-243805	Addition of UE Rx-Tx time difference measurements in RRC_INACTIVE test case

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0513  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-242614)

Decision: 		The document was agreed.

R5-242959	Addition of Test Case 14.2.5

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0514  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243857.

R5-243857	Addition of Test Case 14.2.5

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0514  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242959)

Decision: 		The document was agreed.

R5-242960	Addition of Test Case 14.2.7

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0515  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243858.

R5-243858	Addition of Test Case 14.2.7

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0515  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242960)

Decision: 		The document was agreed.

R5-242961	Addition of Test Case 14.3.5

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0516  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243859.

R5-243859	Addition of Test Case 14.3.5

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0516  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242961)

Decision: 		The document was agreed.

R5-242962	Addition of Test Case 14.3.6

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0517  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243860.

R5-243860	Addition of Test Case 14.3.6

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0517  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242962)

Decision: 		The document was agreed.

R5-242963	Addition of Test Case 14.4.1

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0518  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243861.

R5-243861	Addition of Test Case 14.4.1

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0518  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242963)

Decision: 		The document was agreed.

R5-242964	Addition of Test Case 14.4.2

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0519  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243862.

R5-243862	Addition of Test Case 14.4.2

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0519  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242964)

Decision: 		The document was agreed.

R5-242965	Addition of Test Case 14.5.1

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0520  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243863.

R5-243863	Addition of Test Case 14.5.1

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0520  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242965)

Decision: 		The document was agreed.

R5-242966	Addition of Test Case 14.5.2

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0521  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243864.

R5-243864	Addition of Test Case 14.5.2

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0521  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242966)

Decision: 		The document was agreed.

R5-242967	Addition of Test Case 15.2.5

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0522  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243865.

R5-243865	Addition of Test Case 15.2.5

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0522  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242967)

Decision: 		The document was agreed.

R5-242968	Addition of Test Case 15.2.6

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0523  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243866.

R5-243866	Addition of Test Case 15.2.6

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0523  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242968)

Decision: 		The document was agreed.

R5-242969	Addition of Test Case 15.2.7

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0524  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243867.

R5-243867	Addition of Test Case 15.2.7

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0524  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242969)

Decision: 		The document was agreed.

R5-242970	Addition of Test Case 15.3.3

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0525  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243868.

R5-243868	Addition of Test Case 15.3.3

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0525  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242970)

Decision: 		The document was agreed.

R5-242971	Addition of Test Case 15.3.4

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0526  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243869.

R5-243869	Addition of Test Case 15.3.4

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0526  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242971)

Decision: 		The document was agreed.

R5-242972	Addition of Test Case 16.2.5

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0527  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243870.

R5-243870	Addition of Test Case 16.2.5

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0527  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242972)

Decision: 		The document was agreed.

R5-242973	Addition of Test Case 16.2.6

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0528  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243871.

R5-243871	Addition of Test Case 16.2.6

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0528  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242973)

Decision: 		The document was agreed.

R5-242974	Addition of Test Case 16.3.3

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0529  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243872.

R5-243872	Addition of Test Case 16.3.3

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0529  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242974)

Decision: 		The document was agreed.

R5-242975	Addition of Test Case 16.4.1

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0530  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243873.

R5-243873	Addition of Test Case 16.4.1

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0530  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242975)

Decision: 		The document was agreed.

R5-242976	Addition of Test Case 16.4.2

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0531  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243874.

R5-243874	Addition of Test Case 16.4.2

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0531  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242976)

Decision: 		The document was agreed.

R5-242977	Addition of Test Case 16.5.1

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0532  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243875.

R5-243875	Addition of Test Case 16.5.1

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0532  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242977)

Decision: 		The document was agreed.

R5-242978	Addition of Test Case 16.5.2

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0533  Cat: F (Rel-17)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243876.

R5-243876	Addition of Test Case 16.5.2

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0533  rev 1 Cat: F (Rel-17)

					Source: Rohde & Schwarz

(Replaces R5-242978)

Decision: 		The document was agreed.

##### 5.3.15.3	TS 37.571-3

R5-242483	Addition of test applicabilities for Rel-17 PRS-RSRPP and UE Rx-Tx time difference test cases

Type: CR		For: Agreement
					37.571-3 v17.4.0	  CR-0169  Cat: F (Rel-17)

					Source: CATT, Rohde&Schwarz

Discussion:

Tdoc#

r1

Decision: 		The document was revised to R5-243802.

R5-243802	Addition of test applicabilities for Rel-17 PRS-RSRPP and UE Rx-Tx time difference test cases

Type: CR		For: Agreement
					37.571-3 v17.4.0	  CR-0169  rev 1 Cat: F (Rel-17)

					Source: CATT, Rohde&Schwarz

(Replaces R5-242483)

Decision: 		The document was agreed.

R5-242995	Addition of Test Applicability UE Rx-Tx time difference measurement accuracy in RRC_INACTIVE state in FR2 SA

Type: CR		For: Agreement
					37.571-3 v17.4.0	  CR-0170  Cat: F (Rel-17)

					Source: Nokia

Discussion:

r1

- clauses affected

- 	Reused the conditions defined by CATT in the dependent CR and defined RA-SDT PICS in the additional information table of A.4.4-1

Decision: 		The document was revised to R5-243806.

R5-243806	Addition of Test Applicability UE Rx-Tx time difference measurement accuracy in RRC_INACTIVE state in FR2 SA

Type: CR		For: Agreement
					37.571-3 v17.4.0	  CR-0170  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-242995)

Decision: 		The document was agreed.

##### 5.3.15.4	TS 37.571-5

##### 5.3.15.5	TR 38.903 (NR MU & TT analyses)

R5-242218	Update TC 14.2.3 and TC 14.2.10 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0707  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was revised to R5-243769.

R5-243769	Update TC 14.2.3 and TC 14.2.10 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0707  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242218)

Decision: 		The document was agreed.

R5-242220	Update TC 14.2.8 and TC 14.2.9 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0709  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243770.

R5-243770	Update TC 14.2.8 and TC 14.2.9 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0709  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242220)

Decision: 		The document was agreed.

R5-242221	Update TC 14.3.3 and TC 14.3.8 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0710  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243771.

R5-243771	Update TC 14.3.3 and TC 14.3.8 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0710  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242221)

Decision: 		The document was agreed.

R5-242223	Update TC 14.3.7 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0712  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243772.

R5-243772	Update TC 14.3.7 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0712  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242223)

Decision: 		The document was agreed.

R5-242228	Update TC 15.2.8, 15.2.9 and 15.2.10 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0717  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243773.

R5-243773	Update TC 15.2.8, 15.2.9 and 15.2.10 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0717  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242228)

Decision: 		The document was agreed.

R5-242231	Update TC 15.3.5 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0720  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242232	Update TC 15.3.6 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0721  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242233	Update TC 16.2.3, 16.2.7 and 16.2.8 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0722  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242236	Update TC 16.3.4 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0725  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

##### 5.3.15.6	Discussion Papers, Work Plan, TC lists

#### 5.3.16	NR and MR-DC measurement gap enhancements (UID-981035) NR_MG_enh-UEConTest

##### 5.3.16.1	TS 38.508-1

##### 5.3.16.2	TS 38.508-2

##### 5.3.16.3	TS 38.522

R5-242578	Correction to TC titles in applicability table of RRM event triggered reporting TCs with measurement gaps

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0412  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

dependent CR R4-2407175

Discussion:

r1

Decision: 		The document was revised to R5-243923.

R5-243923	Correction to TC titles in applicability table of RRM event triggered reporting TCs with measurement gaps

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0412  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-242578)

Discussion:

"Revised from: R5-242578r1.

RAN4 dependent CR R4-2407175

Document was sent FOR E-MAIL AGREEMENT

"

the RAN4 Cat F CR R4-2410235 (R4-2407175r1) was agreed in RAN4 RRM session.

Email agreed

Decision: 		The document was agreed.

##### 5.3.16.4	TS 38.533

R5-242127	Addition of TT for MG enhancements TC 6.6.18.3

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3092  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

"RAN4 dependent CR R4-2407693

Author confirmed no overlap/conflicts

5/21 Moderator (E///): discussion ongoing

5/22 Moderator (E///): needs to go for email for further check

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244005.

R5-244005	Addition of TT for MG enhancements TC 6.6.18.3

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3092  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-242127)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242128	Addition of TT for MG enhancements TC 6.6.18.4

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3093  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

"RAN4 dependent CR R4-2407693

Author confirmed no overlap/conflicts

5/21 Moderator (E///): discussion ongoing

5/22 Moderator (E///): needs to go for email for further check

Document was sent FOR E-MAIL AGREEMENT

"

Decision: 		The document was revised to R5-243920.

R5-243920	Addition of TT for MG enhancements TC 6.6.18.4

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3093  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-242128)

Discussion:

"Revised from: R5-242128r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244006.

R5-244006	Addition of TT for MG enhancements TC 6.6.18.4

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3093  rev 2 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-243920)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242579	Correction to TC titles for event triggered reporting with measurement gap

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3096  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-242580	Correction to 7.6.15.3 FR2 SA event triggered reporting tests with concurrent gaps including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3097  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2407175

Associated CR to 38.903 R5-242581

Discussion:

r1

Decision: 		The document was revised to R5-243744.

R5-243744	Correction to 7.6.15.3 FR2 SA event triggered reporting tests with concurrent gaps including TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3097  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-242580)

Discussion:

"Revised from: R5-242580r1.

Document was sent FOR E-MAIL AGREEMENT

"

the RAN4 Cat F CR R4-2410235 (R4-2407175r1) was agreed in RAN4 RRM session.

email agreed

Decision: 		The document was agreed.

##### 5.3.16.5	TR 38.903 (NR MU & TT analyses)

R5-242129	Addition of TT for MG enhancements TC 6.6.18.3

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0705  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

"5/21 Moderator (E///): ongoing discussion

5/22 Moderator (E///): needs to go for email for further check

Document was sent FOR E-MAIL AGREEMENT

"

Decision: 		The document was revised to R5-243921.

R5-243921	Addition of TT for MG enhancements TC 6.6.18.3

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0705  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-242129)

Discussion:

"Revised from: R5-242129r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244007.

R5-244007	Addition of TT for MG enhancements TC 6.6.18.3

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0705  rev 2 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-243921)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242130	Addition of TT for MG enhancements TC 6.6.18.4

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0706  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

"5/21 Moderator (E///): ongoing discussion

5/22 Moderator (E///): needs to go for email for further check

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-243922.

R5-243922	Addition of TT for MG enhancements TC 6.6.18.4

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0706  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-242130)

Discussion:

"Revised from: R5-242130r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244008.

R5-244008	Addition of TT for MG enhancements TC 6.6.18.4

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0706  rev 2 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-243922)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242581	Introduction of Test Tolerance analysis for 7.6.15.3 FR2 SA event triggered reporting tests for concurrent gaps

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0731  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2407175

Associated CR to 38.533 R5-242580

Discussion:

r1

Decision: 		The document was revised to R5-243745.

R5-243745	Introduction of Test Tolerance analysis for 7.6.15.3 FR2 SA event triggered reporting tests for concurrent gaps

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0731  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-242581)

Discussion:

"Revised from: R5-242581r1.

Document was sent FOR E-MAIL AGREEMENT

"

the RAN4 Cat F CR R4-2410235 (R4-2407175r1) was agreed in RAN4 RRM session.

email agreed

Decision: 		The document was agreed.

##### 5.3.16.6	Discussion Papers, Work Plan, TC lists

#### 5.3.17	Further Multi-RAT Dual-Connectivity enhancement (UID-991033) LTE_NR_DC_enh2-UEConTest

##### 5.3.17.1	TS 38.508-1

##### 5.3.17.2	TS 38.508-2

##### 5.3.17.3	TS 38.522

##### 5.3.17.4	TS 38.533

##### 5.3.17.5	TS 36.508

##### 5.3.17.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.17.7	Discussion Papers, Work Plan, TC lists

#### 5.3.18	Additional NR bands for UL-MIMO in Rel-18 (UID-1000050) NR_bands_UL_MIMO_R18-UEConTest

##### 5.3.18.1	TS 38.508-1

##### 5.3.18.2	TS 38.508-2

R5-242289	Editorial correction of n66 with UL MIMO capabilities

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0614  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial

Decision: 		The document was agreed.

##### 5.3.18.3	TS 38.521-1

###### 5.3.18.3.1	Tx Requirements (Clause 6)

R5-242287	Addition of n66 UL MIMO MOP

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2742  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Decision: 		The document was agreed.

R5-242288	Addition of n66 UL MIMO MPR

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2743  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Decision: 		The document was agreed.

R5-243123	Addition of PC2 for n8 UL MIMO into 6.2D.1 MOP TC

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2816  Cat: F (Rel-18)

					Source: China Unicom

Decision: 		The document was agreed.

###### 5.3.18.3.2	Rx Requirements (Clause 7)

###### 5.3.18.3.3	Clauses 1-5, Annexes

R5-242286	Update of Operating bands for UL MIMO band n66

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2741  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

CoreSpecAlig

Decision: 		The document was withdrawn.

##### 5.3.18.4	TS 38.522

##### 5.3.18.5	TR 38.905 (NR Test Points Radio Transmission and Reception )

##### 5.3.18.6	Discussion Papers, Work Plan, TC lists

#### 5.3.19	High power UE (power class 2) for NR FR1 FDD single band (UID-1000051) HPUE_NR_FR1_FDD_R18-UEConTest

##### 5.3.19.1	TS 38.508-1

##### 5.3.19.2	TS 38.508-2

##### 5.3.19.3	TS 38.521-1

###### 5.3.19.3.1	Tx Requirements (Clause 6)

R5-243205	Updating AMPR testing for NS_43 for band n8

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2833  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-243206R5-243206R5-243206R5-243206R5-243206R5-243206R5-243206R5-243206R5-243206R5-243206R5-243206R5-243206R5-243206

Decision: 		The document was agreed.

R5-243207	Updating PC2 AMPR testing for NS_100 for band n8

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2834  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was revised to R5-243631.

R5-243631	Updating PC2 AMPR testing for NS_100 for band n8

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2834  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243207)

Decision: 		The document was agreed.

###### 5.3.19.3.2	Rx Requirements (Clause 7)

###### 5.3.19.3.3	Clauses 1-5, Annexes

##### 5.3.19.4	TS 38.522

##### 5.3.19.5	TR 38.905 (NR Test Points Radio Transmission and Reception )

R5-243206	Updating TP analysis for AMPR testing for NS_43

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0912  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-243205

Decision: 		The document was revised to R5-243613.

R5-243613	Updating TP analysis for AMPR testing for NS_43

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0912  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243206)

Decision: 		The document was agreed.

##### 5.3.19.6	Discussion Papers, Work Plan, TC lists

#### 5.3.20	High power UE (power class 1.5) for NR FR1 TDD single band (UID-1000053) HPUE_NR_FR1_TDD_R18-UEConTest

##### 5.3.20.1	TS 38.508-1

##### 5.3.20.2	TS 38.508-2

##### 5.3.20.3	TS 38.521-1

###### 5.3.20.3.1	Tx Requirements (Clause 6)

R5-243065	Addition of PC1.5 n39 A-MPR

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2815  Cat: F (Rel-18)

					Source: CMCC

Abstract:

TPanalysis for PC1.5 n39 A-MPR is captured in R5-243066 (CR#0904)

Decision: 		The document was agreed.

R5-243442	Update for PC1.5 n39 A-SE

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2859  Cat: F (Rel-18)

					Source: CMCC

Abstract:

TPanalysis for PC1.5 n39 A-SE is captured in R5-243066 (CR#0904)

Decision: 		The document was agreed.

###### 5.3.20.3.2	Rx Requirements (Clause 7)

###### 5.3.20.3.3	Clauses 1-5, Annexes

##### 5.3.20.4	TS 38.522

##### 5.3.20.5	TR 38.905 (NR Test Points Radio Transmission and Reception )

R5-243066	TP analysis for PC1.5 n39 A-MPR and A-SE

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0904  Cat: F (Rel-18)

					Source: CMCC

Abstract:

TPanalysis for PC1.5 n39 A-MPR (R5-243065, CR#2815) and PC1.5 n39 A-SE (R5-243442, CR#2859)

Discussion:

Rel-17?

r2

Decision: 		The document was revised to R5-243614.

R5-243614	TP analysis for PC1.5 n39 A-MPR and A-SE

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0904  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243066)

Decision: 		The document was agreed.

##### 5.3.20.6	Discussion Papers, Work Plan, TC lists

#### 5.3.21	Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations (UID-1000054) HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest

##### 5.3.21.1	TS 38.508-1

##### 5.3.21.2	TS 38.508-2

R5-242131	Correction to HPUE PICS mnemonic

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0607  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

merged into revision of R5-242251

Decision: 		The document was withdrawn.

R5-242251	Addition and correction of RF baseline implementation capabilities for PC2 EN-DC combs within FR1

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0612  Cat: F (Rel-18)

					Source: KDDI Corporation, MediaTek Inc

Discussion:

r1

merged contents of R5-242131.

added coversheet infos.

Decision: 		The document was revised to R5-243609.

R5-243609	Addition and correction of RF baseline implementation capabilities for PC2 EN-DC combs within FR1

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0612  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation, MediaTek Inc

(Replaces R5-242251)

Decision: 		The document was agreed.

##### 5.3.21.3	TS 38.521-1

###### 5.3.21.3.1	Tx Requirements (Clause 6)

###### 5.3.21.3.2	Rx Requirements (Clause 7)

R5-242952	Addition of REFSENS for PC2 CA n78A-n79A with 1UL

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2794  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was agreed.

###### 5.3.21.3.3	Clauses 1-5, Annexes

R5-242944	Update to FR1 CA Configurations for n78A-n79A with 1UL

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2791  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was agreed.

##### 5.3.21.4	TS 38.521-3

###### 5.3.21.4.1	Tx Requirements (Clause 6)

R5-242252	Addition of UE maximum output power for new PC2 EN-DC combs within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1756  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

###### 5.3.21.4.2	Rx Requirements (Clause 7)

R5-242253	Addition of reference sensitivity for new PC2 EN-DC combs within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1757  Cat: F (Rel-18)

					Source: KDDI Corporation

Discussion:

r1

Decision: 		The document was revised to R5-243615.

R5-243615	Addition of reference sensitivity for new PC2 EN-DC combs within FR1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1757  rev 1 Cat: F (Rel-18)

					Source: KDDI Corporation

(Replaces R5-242253)

Decision: 		The document was agreed.

R5-242254	Addition and correction of reference sensitivity exceptions for PC2 EN-DC combos

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1758  Cat: F (Rel-18)

					Source: KDDI Corporation

Decision: 		The document was agreed.

R5-242798	Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC2

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1773  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Discussion:

r1

Decision: 		The document was revised to R5-243828.

R5-243828	Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC2

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1773  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

(Replaces R5-242798)

Decision: 		The document was agreed.

###### 5.3.21.4.3	Clauses 1-5, Annexes

R5-242796	Update to FR1 EN-DC Configurations for n78 and n79

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1771  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was agreed.

##### 5.3.21.5	TS 38.522

##### 5.3.21.6	TR 38.905 (NR Test Points Radio Transmission and Reception )

R5-242497	Addition of reference sensitivity TP analysis for new PC2 EN-DC combs within FR1

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0887  Cat: F (Rel-18)

					Source: KDDI Corporation

Abstract:

Test Case CR t-doc

R5-242253-4

Decision: 		The document was agreed.

R5-242860	Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1776  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Discussion:

3GU allocation: 38.521-3 / AI allocation: 38.905

Decision: 		The document was withdrawn.

R5-242916	Addition of REFSENS TP analysis for 2CC ENDC for n78A and n79A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0893  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was agreed.

R5-242917	Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0894  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

Test Case CR t-doc

R5-242797

R5-242798

Discussion:

r1

Decision: 		The document was revised to R5-243827.

R5-243827	Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0894  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

(Replaces R5-242917)

Decision: 		The document was agreed.

R5-242943	Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC2

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0900  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

Test Case CR t-doc

R5-242952

Discussion:

r1

Decision: 		The document was revised to R5-243826.

R5-243826	Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC2

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0900  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

(Replaces R5-242943)

Decision: 		The document was agreed.

##### 5.3.21.7	Discussion Papers, Work Plan, TC lists

#### 5.3.22	Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL (UID-1000056) NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-UEConTest

##### 5.3.22.1	TS 38.508-1

##### 5.3.22.2	TS 38.508-2

R5-242434	Fix the release information for a NR CA PC2 band configuration

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0620  Cat: F (Rel-18)

					Source: Verizon

Decision: 		The document was agreed.

##### 5.3.22.3	TS 38.521-1

###### 5.3.22.3.1	Tx Requirements (Clause 6)

###### 5.3.22.3.2	Rx Requirements (Clause 7)

###### 5.3.22.3.3	Clauses 1-5, Annexes

##### 5.3.22.4	TS 38.522

##### 5.3.22.5	TR 38.905 (NR Test Points Radio Transmission and Reception )

##### 5.3.22.6	Discussion Papers, Work Plan, TC lists

#### 5.3.23	Rel-18 NR CA and DC; and NR and LTE DC Configurations (UID-1000057) NR_CADC_NR_LTE_DC_R18-UEConTest

##### 5.3.23.1	TS 38.508-1

###### 5.3.23.1.1	Test frequencies (Clause 4.3.1)

R5-242714	Definition of DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A applicability to protocol testing

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3169  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was agreed.

###### 5.3.23.1.2	Test environment for RF (Clauses 5)

###### 5.3.23.1.3	Test environment for RRM (Clause 7)

###### 5.3.23.1.4	Other clauses, Annexes

##### 5.3.23.2	TS 38.508-2

R5-242713	Addition of capability declaration for DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0629  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was revised to R5-243657.

R5-243657	Addition of capability declaration for DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0629  rev 1 Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

(Replaces R5-242713)

Decision: 		The document was agreed.

##### 5.3.23.3	TS 38.521-1

###### 5.3.23.3.1	Tx Requirements (Clause 6)

###### 5.3.23.3.2	Rx Requirements (Clause 7)

###### 5.3.23.3.3	Clauses 1-5, Annexes

##### 5.3.23.4	TS 38.521-2

###### 5.3.23.4.1	Tx Requirements (Clause 6)

###### 5.3.23.4.2	Rx Requirements (Clause 7)

###### 5.3.23.4.3	Clauses 1-5, Annexes

##### 5.3.23.5	TS 38.521-3

###### 5.3.23.5.1	Tx Requirements (Clause 6)

###### 5.3.23.5.2	Rx Requirements (Clause 7)

###### 5.3.23.5.3	Clauses 1-5, Annexes

R5-242711	Addition of DC_66A_n66 BCS1, DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A to Ch5

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1762  Cat: F (Rel-18)

					Source: WE Certification Oy, AT&T

Decision: 		The document was agreed.

##### 5.3.23.6	TS 38.521-4

###### 5.3.23.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.23.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.23.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.23.6.4	Clauses 1-4, Annexes

##### 5.3.23.7	TS 38.522

R5-243043	Update to R18 NR CADC configuration test cases applicability

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0428  Cat: F (Rel-18)

					Source: CMCC

Discussion:

3GU allocation: NR_CADC_NR_LTE_DC_R18-UEConTest / AI allocation: HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest

Decision: 		The document was withdrawn.

##### 5.3.23.8	TS 38.533

##### 5.3.23.9	TR 38.903 (NR MU & TT analyses)

##### 5.3.23.10	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.23.11	Discussion Papers, Work Plan, TC lists

R5-243305	Discussion on spec quality related issues

Type: discussion		For: Approval
					Source: ZTE Corporation

Discussion:

"prop1 needs more specific examples from ran5 spec and background for change

prop2 needs to be checked if any specific action is needed looking at the whole spec and working with spec/wi rapp to see if any changes are needed

prop3 needs to be checkd with mcc for editorial correcfions and with spec rapp for technical corrections"

Decision: 		The document was noted.

#### 5.3.24	New Rel-18 NR licensed bands and extension of existing NR bands (UID-1000058) NR_lic_bands_BW_R18-UEConTest

##### 5.3.24.1	TS 38.508-1

###### 5.3.24.1.1	Test frequencies (Clause 4.3.1)

R5-242368	Introduction of test channel bandwidths and test frequencies for band n106

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3120  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

###### 5.3.24.1.2	Test environment for RF (Clauses 5)

###### 5.3.24.1.3	Test environment for RRM (Clause 7)

###### 5.3.24.1.4	Other clauses, Annexes

##### 5.3.24.2	TS 38.508-2

R5-242370	Introduction of band n106 for physical layer baseline implementation capabilities

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0618  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 5.3.24.3	TS 38.521-1

###### 5.3.24.3.1	Tx Requirements (Clause 6)

R5-242373	Adding transmitter requirements for band n106

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2748  Cat: F (Rel-18)

					Source: Nokia

Discussion:

r1

Decision: 		The document was revised to R5-243640.

R5-243640	Adding transmitter requirements for band n106

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2748  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242373)

Decision: 		The document was agreed.

###### 5.3.24.3.2	Rx Requirements (Clause 7)

R5-242374	Adding receiver requirements for band n106

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2749  Cat: F (Rel-18)

					Source: Nokia

Abstract:

3 MHz CBW for reference sensitivity is introduced in WIC “NR_FR1_lessthan_5MHz_BW-UEConTest” in R5-242397 (CR 2756).

Discussion:

r1

Decision: 		The document was revised to R5-243641.

R5-243641	Adding receiver requirements for band n106

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2749  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242374)

Decision: 		The document was agreed.

R5-242381	Adding reference sensitivity requirements for UEs that support optional symmetric UL/DL in band n71

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2752  Cat: F (Rel-18)

					Source: Nokia

Discussion:

r1

Decision: 		The document was revised to R5-243642.

R5-243642	Adding reference sensitivity requirements for UEs that support optional symmetric UL/DL in band n71

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2752  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242381)

Decision: 		The document was agreed.

###### 5.3.24.3.3	Clauses 1-5, Annexes

R5-242372	Introduction of new band n106 into clause 5

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2747  Cat: F (Rel-18)

					Source: Nokia

Abstract:

3 MHz CBW for each NR band and Synchronization raster for n106 is introduced in WIC “NR_FR1_lessthan_5MHz_BW-UEConTest” in R5-242395 (CR 2754).

Discussion:

r2

Decision: 		The document was revised to R5-243643.

R5-243643	Introduction of new band n106 into clause 5

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2747  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242372)

Decision: 		The document was agreed.

##### 5.3.24.4	TS 38.521-2

###### 5.3.24.4.1	Tx Requirements (Clause 6)

###### 5.3.24.4.2	Rx Requirements (Clause 7)

###### 5.3.24.4.3	Clauses 1-5, Annexes

##### 5.3.24.5	TS 38.521-3

###### 5.3.24.5.1	Tx Requirements (Clause 6)

###### 5.3.24.5.2	Rx Requirements (Clause 7)

###### 5.3.24.5.3	Clauses 1-5, Annexes

##### 5.3.24.6	TS 38.521-4

###### 5.3.24.6.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.24.6.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.24.6.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.24.6.4	Clauses 1-4, Annexes

##### 5.3.24.7	TS 38.521-5

R5-243419	Common clause updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0060  Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

Abstract:

Core spec alignment

Decision: 		The document was revised to R5-243682.

R5-243682	Common clause updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0060  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

(Replaces R5-243419)

Discussion:

agreed, but clashed with R5-243962.

Decision: 		The document was revised to R5-243987.

R5-243987	Common clause updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0060  rev 2 Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

(Replaces R5-243682)

Decision: 		The document was agreed.

R5-243420	MOP requirement updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0061  Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

Abstract:

Core spec alignment

Discussion:

r1

Decision: 		The document was revised to R5-243683.

R5-243683	MOP requirement updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0061  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

(Replaces R5-243420)

Decision: 		The document was agreed.

R5-243421	A-MPR requirements updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0062  Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

Abstract:

Core spec alignment

Discussion:

r1

Decision: 		The document was revised to R5-243684.

R5-243684	A-MPR requirements updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0062  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

(Replaces R5-243421)

Decision: 		The document was agreed.

R5-243422	REFSENS requirements updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0063  Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

Abstract:

Core spec alignment

Decision: 		The document was agreed.

R5-243423	IBB requirements updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0064  Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

Abstract:

Core spec alignment

Discussion:

r1

Decision: 		The document was revised to R5-243685.

R5-243685	IBB requirements updates related to NTN band n254

Type: CR		For: Agreement
					38.521-5 v18.1.0	  CR-0064  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V., Keysight

(Replaces R5-243423)

Decision: 		The document was agreed.

##### 5.3.24.8	TS 38.533

R5-242371	Introduction of band n106 to NR band group for FR1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3094  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 5.3.24.9	TS 36.521-1

##### 5.3.24.10	TR 38.903 (NR MU & TT analyses)

##### 5.3.24.11	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.24.12	Discussion Papers, Work Plan, TC lists

#### 5.3.25	High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 (UID-1000059) LTE_NR_HPUE_FWVM_R18-UEConTest

##### 5.3.25.1	TS 38.508-2

R5-242380	Introduction of bands n7 and n78 PC1 for physical layer baseline implementation capabilities

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0619  Cat: F (Rel-18)

					Source: Nokia

Abstract:

Band n41 is added to Table A.4.3.1-4b in R5-242498 (CR 0624). Item numbering to be sorted out by MCC.

Decision: 		The document was agreed.

R5-242498	Addition of n41 PC1 RF baseline ICS

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0624  Cat: F (Rel-18)

					Source: CMCC, Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to R5-243610.

R5-243610	Addition of n41 PC1 RF baseline ICS

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0624  rev 1 Cat: F (Rel-18)

					Source: CMCC, Nokia, Nokia Shanghai Bell

(Replaces R5-242498)

Decision: 		The document was agreed.

##### 5.3.25.2	TS 38.521-1

###### 5.3.25.2.1	Tx Requirements (Clause 6)

R5-242379	Introduction of PC1 transmitter requirements for bands n7, n41 and n78

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2751  Cat: F (Rel-18)

					Source: Nokia

Abstract:

Band n41 MOP in R5-242466 (CR 2758), Band n41 MPR in R5-242467 (CR 2759), Band n41 UTRA ACLR in R5-242468 (CR 2760).

Discussion:

r1

Modified Note in Table 6.2.3.3.1-1. Note numbering to be sorted out by MCC.

r4

Decision: 		The document was revised to R5-243877.

R5-243877	Introduction of PC1 transmitter requirements for bands n7, n41 and n78

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2751  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242379)

Decision: 		The document was agreed.

R5-242466	Addition of n41 PC1 MOP TC

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2758  Cat: F (Rel-18)

					Source: CMCC, Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

R5-242467	Addition of n41 PC1 MPR TC

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2759  Cat: F (Rel-18)

					Source: CMCC, Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

R5-242468	Update n41 PC1 UTRA ACLR TC

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2760  Cat: F (Rel-18)

					Source: CMCC, Nokia, Nokia Shanghai Bell

Decision: 		The document was agreed.

###### 5.3.25.2.2	Rx Requirements (Clause 7)

###### 5.3.25.2.3	Clauses 1-5, Annexes

##### 5.3.25.3	TS 38.522

##### 5.3.25.4	TS 36.521-1

##### 5.3.25.5	TS 36.521-2

##### 5.3.25.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.25.7	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )

##### 5.3.25.8	Discussion Papers, Work Plan, TC lists

#### 5.3.26	Rel-17 LTE CA Configurations (UID-1010051) LTE_CA_R17-UEConTest

##### 5.3.26.1	TS 36.508

##### 5.3.26.2	TS 36.521-1

R5-242701	Addition of Inter-band DL CA and CA_30A-48A band for A-MPR & Spurious emission test cases

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5509  Cat: F (Rel-18)

					Source: Sporton, Bureau Veritas ADT

Decision: 		The document was agreed.

R5-242782	Update of Receiver test cases for multiple CA combos

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5512  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Sporton International

Abstract:

Updated test requirement table for 4DL CA and 5DL CA cases for CA combo with intra-band contiguous CA. i.e. CA_2A-2A-29A-66A,  CA_2A-29A-66A-66A, CA_2A-2A-29A-30A-66A and CA_2A-2A-29A-66A-66A

Decision: 		The document was agreed.

##### 5.3.26.3	TS 36.521-3

##### 5.3.26.4	TS 36.521-2

R5-243283	Update Annex B for PRD20 E-UTRA CA list v170

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1033  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

18.2.0!

r1

Decision: 		The document was revised to R5-243810.

R5-243810	Update Annex B for PRD20 E-UTRA CA list v170

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1033  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-243283)

Decision: 		The document was agreed.

##### 5.3.26.5	TS 37.571-1

##### 5.3.26.6	TS 37.571-3

##### 5.3.26.7	TS 37.571-5

##### 5.3.26.8	TR 36.903 (E-UTRAN RRM TT analyses)

##### 5.3.26.9	TR 36.904 (E-UTRAN Radio Reception TT analyses)

##### 5.3.26.10	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )

R5-242702	Addition of test point analysis for Inter-band DL CA and UL CA_30A-48A for A-MPR & Spurious emission

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5510  Cat: F (Rel-18)

					Source: Sporton

Decision: 		The document was withdrawn.

R5-242712	Addition of test point analysis for Inter-band DL CA and UL CA_30A-48A for A-MPR & Spurious emission

Type: CR		For: Agreement
					36.905 v18.4.0	  CR-0273  Cat: F (Rel-18)

					Source: Sporton

Decision: 		The document was agreed.

##### 5.3.26.11	Discussion Papers, Work Plan, TC lists

#### 5.3.27	Air-to-ground network for NR (UID-1020087) NR_ATG-UEConTest

##### 5.3.27.1	TS 38.508-1

###### 5.3.27.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.27.1.2	Test environment for RF (Clauses 5)

###### 5.3.27.1.3	Test environment for RRM (Clause 7)

###### 5.3.27.1.4	Other clauses, Annexes

##### 5.3.27.2	TS 38.508-2

##### 5.3.27.3	TS 38.521-1

###### 5.3.27.3.1	Tx Requirements (Clause 6)

R5-243037	Update to ATG RF Tx TCs

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2807  Cat: F (Rel-18)

					Source: CMCC

Abstract:

Core spec alignment. RAN4 dependency. The dependent R4 Tdoc number is R4-2407601 (CR#2228).

Discussion:

"RAN4 dependency. The dependent R4 Tdoc number is R4-2407601 (CR#2228)

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

pending RAN4 Tdoc R4-2410743 (CR#2228) is agreed.

email agreed

Decision: 		The document was agreed.

###### 5.3.27.3.2	Rx Requirements (Clause 7)

R5-243038	Update to ATG RF Rx TCs

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2808  Cat: F (Rel-18)

					Source: CMCC, CAICT

Abstract:

Core spec alignment

Decision: 		The document was agreed.

###### 5.3.27.3.3	Clauses 1-5, Annexes

R5-243036	Update to ATG Definitions and Symbols

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2806  Cat: F (Rel-18)

					Source: CMCC

Abstract:

Core spec alignment. RAN4 dependency. The dependent R4 Tdoc number is R4-2407601(CR#2228).

Discussion:

r1

Decision: 		The document was revised to R5-243906.

R5-243906	Update to ATG Definitions and Symbols

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2806  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243036)

Discussion:

"Revised from: R5-243036r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Align with agreed RAN4 Tdoc R4-2410743 (CR#2228)

Decision: 		The document was revised to R5-244009.

R5-244009	Update to ATG Definitions and Symbols

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2806  rev 2 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243906)

Discussion:

email agreed

Decision: 		The document was agreed.

##### 5.3.27.4	TS 38.521-4

###### 5.3.27.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.27.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.27.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.27.4.4	Clauses 1-4, Annexes

##### 5.3.27.5	TS 38.522

##### 5.3.27.6	TS 38.533

##### 5.3.27.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.27.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.27.9	Discussion Papers, Work Plan, TC lists

#### 5.3.28	4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC (UID-1020088) 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest

##### 5.3.28.1	TS 38.508-1

R5-242919	Addition of TE and UE connection diagram for 3Tx NR CA and EN-DC

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3193  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.28.2	TS 38.508-2

R5-242920	Addition of physical layer baseline implementation capabilities for 3Tx NR CA and EN-DC

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0638  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

Tdoc#

r1

Decision: 		The document was revised to R5-243606.

R5-243606	Addition of physical layer baseline implementation capabilities for 3Tx NR CA and EN-DC

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0638  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242920)

Decision: 		The document was agreed.

##### 5.3.28.3	TS 38.521-1

###### 5.3.28.3.1	Tx Requirements (Clause 6)

R5-242844	Addition of test case 6.3H.3.1 Minimum output power for inter-band UL CA with UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2781  Cat: F (Rel-18)

					Source: TTA

Abstract:

Based on endorsed proposals in discussion paper R5-240377 which was handled in RAN5#102 meeting

Discussion:

r1

Added Test requirement parts.

Decision: 		The document was revised to R5-243626.

R5-243626	Addition of test case 6.3H.3.1 Minimum output power for inter-band UL CA with UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2781  rev 1 Cat: F (Rel-18)

					Source: TTA

(Replaces R5-242844)

Decision: 		The document was agreed.

R5-242845	Addition of test case 6.5H.3.1 Occupied bandwidth for inter-band UL CA with UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2782  Cat: F (Rel-18)

					Source: TTA

Abstract:

Based on endorsed proposals in discussion paper R5-240377 which was handled in RAN5#102 meeting

Discussion:

r1

Added Test requirement parts.

Decision: 		The document was revised to R5-243627.

R5-243627	Addition of test case 6.5H.3.1 Occupied bandwidth for inter-band UL CA with UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2782  rev 1 Cat: F (Rel-18)

					Source: TTA

(Replaces R5-242845)

Decision: 		The document was agreed.

R5-242921	Update of configured transmitted power requirements for 3Tx NR CA

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2784  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242922	Addition of new test case 6.2H.3.1 MOP for inter-band CA with MIMO

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2785  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242923	Addition of new test cases 6.2H.3.x for inter-band CA with MIMO

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2786  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242924	Addition of new test case 6.2L.3.1 MOP for inter-band CA with TxD

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2787  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242925	Addition of new test cases 6.2L.3.x for inter-band CA with TxD

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2788  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was revised to R5-243628.

R5-243628	Addition of new test cases 6.2L.3.x for inter-band CA with TxD

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2788  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242925)

Decision: 		The document was agreed.

###### 5.3.28.3.2	Rx Requirements (Clause 7)

###### 5.3.28.3.3	Clauses 1-5, Annexes

R5-242926	Addition of MU and TT analysis for 3Tx NR CA test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2789  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242927	Addition of general informationfor 3Tx NR CA

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2790  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-243878.

R5-243878	Addition of general informationfor 3Tx NR CA

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2790  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242927)

Decision: 		The document was agreed.

##### 5.3.28.4	TS 38.521-3

###### 5.3.28.4.1	Tx Requirements (Clause 6)

R5-242928	Addition of new test case 6.2H.1.3 MOP for inter-band EN-DC with MIMO

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1777  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242929	Addition of new test cases 6.2H.x.3 for inter-band EN-DC with MIMO

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1778  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242930	Addition of new test case 6.2L.1.3 MOP for inter-band EN-DC with TxD

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1779  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242931	Addition of new test cases 6.2L.x.3 for inter-band EN-DC with TxD

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1780  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

###### 5.3.28.4.2	Rx Requirements (Clause 7)

###### 5.3.28.4.3	Clauses 1-5, Annexes

R5-243220	Addition of MU and TT analysis for 3Tx EN-DC test cases

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1787  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243221	Addition of general information for 3Tx EN-DC

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1788  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.28.5	TS 38.522

R5-242932	Addition of applicabilities for 3Tx NR CA and EN-DC test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0424  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.28.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-242933	Addition of test point analysis for 6.2H.3.1 MOP for inter-band CA with MIMO

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0895  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242934	Addition of test point analysis for 6.2L.3.1 MOP for inter-band CA with TxD

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0896  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242935	Addition of test point analysis for 6.2H.1.3 MOP for inter-band EN-DC with MIMO

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0897  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242936	Addition of test point analysis for 6.2L.1.3 MOP for inter-band EN-DC with TxD

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0898  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.28.7	Discussion Papers, Work Plan, TC lists

#### 5.3.29	NR support for dedicated spectrum less than 5MHz for FR1 (UID-1030060) NR_FR1_lessthan_5MHz_BW-UEConTest

##### 5.3.29.1	TS 38.508-1

###### 5.3.29.1.1	Test frequencies (Clause 4.3.1)

R5-242394	Addition of 3 MHz CBW test frequencies for NR Bands n26, n28, n85 and n100

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3122  Cat: F (Rel-18)

					Source: Nokia

Abstract:

Band n85 Low test channel bandwidths and test frequencies for wider than 3 MHz are specified in R5-242362 (CR 3118)

Decision: 		The document was revised to R5-243601.

R5-243601	Addition of 3 MHz CBW test frequencies for NR Bands n26, n28, n85 and n100

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3122  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242394)

Decision: 		The document was agreed.

###### 5.3.29.1.2	Test environment for RF (Clauses 5)

###### 5.3.29.1.3	Test environment for RRM (Clause 7)

###### 5.3.29.1.4	Other clauses, Annexes

##### 5.3.29.2	TS 38.508-2

##### 5.3.29.3	TS 38.521-1

###### 5.3.29.3.1	Tx Requirements (Clause 6)

R5-242396	Addition of 3 MHz channel bandwidth for transmitter requirements

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2755  Cat: F (Rel-18)

					Source: Nokia

Discussion:

r1

Removed n85 from Table 6.2.2.5-1. n85 is included in R5-242365 (CR 2745). Modify Note in Table 6.2.3.3.1-1. Note numbering to be sorted out by MCC.

r3

Decision: 		The document was revised to R5-243637.

R5-243637	Addition of 3 MHz channel bandwidth for transmitter requirements

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2755  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242396)

Decision: 		The document was agreed.

###### 5.3.29.3.2	Rx Requirements (Clause 7)

R5-242397	Addition of 3 MHz channel bandwidth for receiver requirements

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2756  Cat: F (Rel-18)

					Source: Nokia

Abstract:

Band n85 reference sensitivity for wider than 3 MHz are specified in R5-242366 (CR 2746)

Discussion:

r1

Added reference to other overlapping CR in the cover sheet

r2

Decision: 		The document was revised to R5-243638.

R5-243638	Addition of 3 MHz channel bandwidth for receiver requirements

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2756  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242397)

Decision: 		The document was agreed.

###### 5.3.29.3.3	Clauses 1-5, Annexes

R5-242395	Introduction of dedicated spectrum less than 5MHz for FR1 to clause 5 and annex A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2754  Cat: F (Rel-18)

					Source: Nokia

Abstract:

1)	New Table 5.4.3.3-2 (SS raster) is introduced. SS raster is required also for Band n106. Band n106 is introduced in WIC “NR_lic_bands_BW_R18-UEConTest” in R5-242372 (CR 2747).

2)	3 MHz CBW is introduced for band n85 in Table 5.3.5-1. Band n85 is new an

Discussion:

r1

Decision: 		The document was revised to R5-243639.

R5-243639	Introduction of dedicated spectrum less than 5MHz for FR1 to clause 5 and annex A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2754  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242395)

Decision: 		The document was agreed.

##### 5.3.29.4	TS 38.521-4

###### 5.3.29.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.29.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.29.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.29.4.4	Clauses 1-4, Annexes

##### 5.3.29.5	TS 38.522

##### 5.3.29.6	TS 38.533

##### 5.3.29.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.29.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.29.9	Discussion Papers, Work Plan, TC lists

#### 5.3.30	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR (UID-1030061) NR_DualTxRx_MUSIM-UEConTest

##### 5.3.30.1	TS 38.508-1

##### 5.3.30.2	TS 38.508-2

##### 5.3.30.3	TS 38.509

##### 5.3.30.4	TS 38.522

##### 5.3.30.5	TS 38.533

##### 5.3.30.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.30.7	Discussion Papers, Work Plan, TC lists

#### 5.3.31	Enhanced support of reduced capability NR devices plus CT1 aspects (UID-1030062) NR_redcap_enh_plus_CT1-UEConTest

##### 5.3.31.1	TS 38.508-1

##### 5.3.31.2	TS 38.508-2

##### 5.3.31.3	TS 38.521-1

###### 5.3.31.3.1	Tx Requirements (Clause 6)

###### 5.3.31.3.2	Rx Requirements (Clause 7)

###### 5.3.31.3.3	Clauses 1-5, Annexes

##### 5.3.31.4	TS 38.521-4

###### 5.3.31.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.31.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.31.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.31.4.4	Clauses 1-4, Annexes

##### 5.3.31.5	TS 38.522

##### 5.3.31.6	TS 38.533

##### 5.3.31.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.31.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.31.9	Discussion Papers, Work Plan, TC lists

#### 5.3.32	Introduction of 900 MHz LTE band in the US (UID-1030064) LTE_900MHz_US-UEConTest

##### 5.3.32.1	TS 36.508

R5-242384	Addition of test frequencies for LTE Band 106

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1474  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 5.3.32.2	TS 36.521-1

R5-242386	Addition of band 106 into frequency bands and channel arrangements

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5504  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

R5-242387	Addition of band 106 transmitter requirements

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5505  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

R5-242388	Addition of band 106 receiver requirements

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5506  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 5.3.32.3	TS 36.521-3

R5-242390	Addition of band groups for band 106

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3024  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 5.3.32.4	TS 36.521-2

R5-242389	Addition of test case applicability statements for band 106

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1028  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 5.3.32.5	TS 38.521-1

R5-242391	Update to spurious emissions UE coexistence as part of introduction of LTE Band 106

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2753  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 5.3.32.6	Discussion Papers, Work Plan, TC lists

#### 5.3.33	NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 (UID-1030065) NR_RF_FR2_req_Ph3-UEConTest

##### 5.3.33.1	TS 38.508-1

##### 5.3.33.2	TS 38.508-2

##### 5.3.33.3	TS 38.509

R5-243264	Draft CR for introducing beamlock function in IDLE mode

Type: draftCR		For: Discussion
					38.509 v17.5.0
					Source: Huawei, HiSilicon

Abstract:

Discussion paper in R5-243159

Discussion:

"Discussion paper in R5-243159

next steps in discussion paper R5-243823 needs to be concluded before having RAN5 specification cr's"

Decision: 		The document was not pursued.

##### 5.3.33.4	TS 38.521-2

###### 5.3.33.4.1	Tx Requirements (Clause 6)

R5-242791	Introducing framework for Beam Correspondence during Initial Access in IDLE related Tx Power tests

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1047  Cat: F (Rel-18)

					Source: Apple AB Denmark

Discussion:

r2

Decision: 		The document was revised to R5-243841.

R5-243841	Introducing framework for Beam Correspondence during Initial Access in IDLE related Tx Power tests

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1047  rev 1 Cat: F (Rel-18)

					Source: Apple AB Denmark

(Replaces R5-242791)

Decision: 		The document was agreed.

###### 5.3.33.4.2	Rx Requirements (Clause 7)

###### 5.3.33.4.3	Clauses 1-5, Annexes

##### 5.3.33.5	TS 38.521-3

###### 5.3.33.5.1	Tx Requirements (Clause 6)

R5-242790	Clarification On Beam Correspondence requirement for EN-DC

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1770  Cat: F (Rel-18)

					Source: Apple AB Denmark

Decision: 		The document was agreed.

###### 5.3.33.5.2	Rx Requirements (Clause 7)

###### 5.3.33.5.3	Clauses 1-5, Annexes

##### 5.3.33.6	TS 38.522

##### 5.3.33.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.33.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-243160	Draft CR for introducing beamlock function in IDLE mode

Type: draftCR		For: Discussion
					38.905 v18.2.0
					Source: Huawei, HiSilicon

Abstract:

Associated discussion in R5-243159

Decision: 		The document was withdrawn.

##### 5.3.33.9	Discussion Papers, Work Plan, TC lists

R5-242792	On Beam Correspondence initial access in Idle and FR2 RRM RACH Test

Type: discussion		For: Endorsement
					Source: Apple AB Denmark

Decision: 		The document was revised to R5-243702.

R5-243702	On Beam Correspondence initial access in Idle and FR2 RRM RACH Test

Type: discussion		For: Endorsement
					Source: Apple AB Denmark

(Replaces R5-242792)

Discussion:

"Revised from: R5-242792.

update to indicate spec details TS38.521-2  clause and ts38.533 clause  in prop1 and table of comparisons

prop1 with change is endorsed"

Decision: 		The document was noted.

R5-243159	Discussion on beam correspondence for initial access

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Abstract:

Draft 38.509 CR in R5-243264

Discussion:

r1

Decision: 		The document was revised to R5-243823.

R5-243823	Discussion on beam correspondence for initial access

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

(Replaces R5-243159)

Decision: 		The document was noted.

#### 5.3.34	Further RF requirements enhancement for NR and EN-DC in frequency range 1 (UID-1030066) NR_ENDC_RF_FR1_enh2-UEConTest

##### 5.3.34.1	TS 38.508-1

##### 5.3.34.2	TS 38.508-2

##### 5.3.34.3	TS 38.521-1

###### 5.3.34.3.1	Tx Requirements (Clause 6)

###### 5.3.34.3.2	Rx Requirements (Clause 7)

###### 5.3.34.3.3	Clauses 1-5, Annexes

##### 5.3.34.4	TS 38.521-3

###### 5.3.34.4.1	Tx Requirements (Clause 6)

###### 5.3.34.4.2	Rx Requirements (Clause 7)

###### 5.3.34.4.3	Clauses 1-5, Annexes

##### 5.3.34.5	TS 38.521-4

###### 5.3.34.5.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.34.5.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.34.5.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.34.5.4	Clauses 1-4, Annexes

##### 5.3.34.6	TS 38.522

##### 5.3.34.7	TS 38.533

##### 5.3.34.8	TR 38.903 (NR MU & TT analyses)

##### 5.3.34.9	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.34.10	Discussion Papers, Work Plan, TC lists

#### 5.3.35	Network energy savings for NR (UID-1030067) Netw_Energy_NR-UEConTest

##### 5.3.35.1	TS 38.508-1

##### 5.3.35.2	TS 38.508-2

##### 5.3.35.3	TS 38.521-4

###### 5.3.35.3.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.35.3.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.35.3.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.35.3.4	Clauses 1-4, Annexes

##### 5.3.35.4	TS 38.522

##### 5.3.35.5	TS 38.533

##### 5.3.35.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.35.7	Discussion Papers, Work Plan, TC lists

#### 5.3.36	Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 (UID-1030068) LTE_NR_Simult_RxTx_R18-UEConTest

##### 5.3.36.1	TS 38.521-1

###### 5.3.36.1.1	Tx Requirements (Clause 6)

R5-243046	Update delta TIBc of CA_n39A-n41A for simultaneous  RxTx capability

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2810  Cat: F (Rel-18)

					Source: CMCC

Abstract:

Core spec alignment

Decision: 		The document was agreed.

###### 5.3.36.1.2	Rx Requirements (Clause 7)

R5-243047	Update delta RIBc of CA_n39A-n41A for simultaneous  RxTx capability

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2811  Cat: F (Rel-18)

					Source: CMCC

Abstract:

Core spec alignment

Decision: 		The document was agreed.

R5-243048	Update REFSENSE exceptions due to UL harmonic interference for CA_n39A-n41A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2812  Cat: F (Rel-18)

					Source: CMCC

Abstract:

Core spec alignment

Discussion:

r1

Overlapped with R5-242706 (WE).

Decision: 		The document was revised to R5-243837.

R5-243837	Update REFSENSE exceptions due to UL harmonic interference for CA_n39A-n41A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2812  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243048)

Decision: 		The document was agreed.

R5-243049	Update REFSENSE exceptions due to CBI for CA_n39A-n41A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2813  Cat: F (Rel-18)

					Source: CMCC

Abstract:

Core spec alignment

Decision: 		The document was agreed.

R5-243050	Update REFSENSE TC 7.3A.1_1 for CA_n39A-n41A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2814  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was revised to R5-243838.

R5-243838	Update REFSENSE TC 7.3A.1_1 for CA_n39A-n41A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2814  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243050)

Decision: 		The document was agreed.

###### 5.3.36.1.3	Clauses 1-5, Annexes

##### 5.3.36.2	TS 38.521-3

###### 5.3.36.2.1	Tx Requirements (Clause 6)

###### 5.3.36.2.2	Rx Requirements (Clause 7)

###### 5.3.36.2.3	Clauses 1-5, Annexes

##### 5.3.36.3	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-243051	Update REFSENSE TP analysis for CA_n39A-n41A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0903  Cat: F (Rel-18)

					Source: CMCC

Discussion:

r1

Decision: 		The document was revised to R5-243836.

R5-243836	Update REFSENSE TP analysis for CA_n39A-n41A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0903  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243051)

Decision: 		The document was agreed.

##### 5.3.36.4	Discussion Papers, Work Plan, TC lists

#### 5.3.37	Further NR coverage enhancements (UID-1030069) NR_cov_enh2-UEConTest

##### 5.3.37.1	TS 38.508-1

##### 5.3.37.2	TS 38.508-2

##### 5.3.37.3	TS 38.521-1

###### 5.3.37.3.1	Tx Requirements (Clause 6)

###### 5.3.37.3.2	Rx Requirements (Clause 7)

###### 5.3.37.3.3	Clauses 1-5, Annexes

##### 5.3.37.4	TS 38.521-3

###### 5.3.37.4.1	Tx Requirements (Clause 6)

###### 5.3.37.4.2	Rx Requirements (Clause 7)

###### 5.3.37.4.3	Clauses 1-5, Annexes

##### 5.3.37.5	TS 38.522

##### 5.3.37.6	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.37.7	Discussion Papers, Work Plan, TC lists

#### 5.3.38	Multi-carrier enhancements for NR (UID-1030070) NR_MC_enh-UEConTest

##### 5.3.38.1	TS 38.508-1

##### 5.3.38.2	TS 38.508-2

R5-242865	Addition of PICS for MCE RRM test cases

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0637  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.3.38.3	TS 38.521-1

###### 5.3.38.3.1	Tx Requirements (Clause 6)

R5-243209	Adding new test case Time mask for switching across three uplink bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2836  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-243214

Decision: 		The document was revised to R5-243658.

R5-243658	Adding new test case Time mask for switching across three uplink bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2836  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243209)

Decision: 		The document was agreed.

R5-243210	Adding new test case Time mask for switching across four uplink bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2837  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-243214

Decision: 		The document was revised to R5-243659.

R5-243659	Adding new test case Time mask for switching across four uplink bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2837  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243210)

Decision: 		The document was agreed.

R5-243211	Adding new SUL test case Time mask for switching across three uplink bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2838  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-243215

Discussion:

r1

Decision: 		The document was revised to R5-243620.

R5-243620	Adding new SUL test case Time mask for switching across three uplink bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2838  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243211)

Decision: 		The document was agreed.

R5-243212	Adding new SUL test case Time mask for switching across four uplink bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2839  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-243644.

R5-243644	Adding new SUL test case Time mask for switching across four uplink bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2839  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243212)

Decision: 		The document was agreed.

###### 5.3.38.3.2	Rx Requirements (Clause 7)

###### 5.3.38.3.3	Clauses 1-5, Annexes

R5-243208	Updating general clause for Uplink switching across up to four bands

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2835  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243213	Updating MU and TT for Time mask for switching across up to four bands testing

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2840  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was revised to R5-243645.

R5-243645	Updating MU and TT for Time mask for switching across up to four bands testing

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2840  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243213)

Decision: 		The document was agreed.

##### 5.3.38.4	TS 38.522

R5-242866	Addition of applicability for MCE RRM test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0422  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-243914.

R5-243914	Addition of applicability for MCE RRM test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0422  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242866)

Discussion:

"Revised from: R5-242866r1.

associated TC CRs R5-242867, 2868, 2869 which are pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

##### 5.3.38.5	TS 38.533

R5-242867	Addition of MCE RRM test case 6.5.7D.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3129  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency R4-2408548 (agreed)

Discussion:

"RAN4 dependency R4-2408548

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

R5-242868	Addition of MCE RRM test case 6.5.7D.2

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3130  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency R4-2408548 (agreed)

Discussion:

"RAN4 dependency R4-2408548

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

R5-242869	Addition of MCE RRM test case 6.5.7D.3

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3131  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency R4-2408548 (agreed)

Discussion:

"RAN4 dependency R4-2408548

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

R5-242870	Correction to Annex E for MCE RRM test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3132  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

"deferred pending TC CRs (which is pending ran4)

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

##### 5.3.38.6	TR 38.903 (NR MU & TT analyses)

##### 5.3.38.7	TR 38.905 (NR Test Points Radio Transmission and Reception)

R5-243214	Adding TP analysis for new FR1 test case 6.3A.3.6 and 6.3A.3.7

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0913  Cat: F (Rel-18)

					Source: Huawei, Hisilicon

Abstract:

TC in R5-243209,R5-243210

Discussion:

38.521-1!

r1

Decision: 		The document was revised to R5-243618.

R5-243618	Adding TP analysis for new FR1 test case 6.3A.3.6 and 6.3A.3.7

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0913  rev 1 Cat: F (Rel-18)

					Source: Huawei, Hisilicon

(Replaces R5-243214)

Decision: 		The document was agreed.

R5-243215	Adding TP analysis for new SUL test case 6.3C.3.6

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0914  Cat: F (Rel-18)

					Source: Huawei, Hisilicon

Abstract:

TC in R5-243211

Discussion:

38.521-1!

r1

Decision: 		The document was revised to R5-243619.

R5-243619	Adding TP analysis for new SUL test case 6.3C.3.6

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0914  rev 1 Cat: F (Rel-18)

					Source: Huawei, Hisilicon

(Replaces R5-243215)

Decision: 		The document was revised to R5-243796.

R5-243796	Adding TP analysis for new SUL test case 6.3C.3.6

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0914  rev 2 Cat: F (Rel-18)

					Source: Huawei, Hisilicon

(Replaces R5-243619)

Decision: 		The document was agreed.

##### 5.3.38.8	Discussion Papers, Work Plan, TC lists

#### 5.3.39	Enhanced NR support for high speed train scenario in frequency range 2 (FR2) (UID-1030071) NR_HST_FR2_enh-UEConTest

##### 5.3.39.1	TS 38.508-1

##### 5.3.39.2	TS 38.508-2

##### 5.3.39.3	TS 38.521-2

###### 5.3.39.3.1	Tx Requirements (Clause 6)

###### 5.3.39.3.2	Rx Requirements (Clause 7)

###### 5.3.39.3.3	Clauses 1-5, Annexes

##### 5.3.39.4	TS 38.521-4

###### 5.3.39.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.39.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.39.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.39.4.4	Clauses 1-4, Annexes

##### 5.3.39.5	TS 38.522

##### 5.3.39.6	TS 38.533

##### 5.3.39.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.39.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.39.9	Discussion Papers, Work Plan, TC lists

#### 5.3.40	NR MIMO evolution for downlink and uplink  (FR2) (UID-1030072) NR_MIMO_evo_DL_UL-UEConTest

##### 5.3.40.1	TS 38.508-1

###### 5.3.40.1.1	Test frequencies (Clause 4.3.1)

###### 5.3.40.1.2	Test environment for RF (Clauses 5)

###### 5.3.40.1.3	Test environment for RRM (Clause 7)

###### 5.3.40.1.4	Other clauses, Annexes

##### 5.3.40.2	TS 38.508-2

##### 5.3.40.3	TS 38.521-2

###### 5.3.40.3.1	Tx Requirements (Clause 6)

###### 5.3.40.3.2	Rx Requirements (Clause 7)

###### 5.3.40.3.3	Clauses 1-5, Annexes

##### 5.3.40.4	TS 38.521-4

###### 5.3.40.4.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

###### 5.3.40.4.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

###### 5.3.40.4.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

###### 5.3.40.4.4	Clauses 1-4, Annexes

##### 5.3.40.5	TS 38.522

##### 5.3.40.6	TS 38.533

##### 5.3.40.7	TR 38.903 (NR MU & TT analyses)

##### 5.3.40.8	TR 38.905 (NR Test Points Radio Transmission and Reception)

##### 5.3.40.9	Discussion Papers, Work Plan, TC lists

#### 5.3.41	IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects (UID-1030073) IoT_NTN_enh_plus_CT1-UEConTest

##### 5.3.41.1	TS 36.508

##### 5.3.41.2	TS 36.509

##### 5.3.41.3	TS 36.521-3

R5-243057	Addition of new RRM TC 13.5.1 for IoT NTN enh

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3038  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was revised to R5-243688.

R5-243688	Addition of new RRM TC 13.5.1 for IoT NTN enh

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3038  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243057)

Decision: 		The document was agreed.

R5-243058	Addition of new RRM TC 13.5.2 for IoT NTN enh

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3039  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was revised to R5-243689.

R5-243689	Addition of new RRM TC 13.5.2 for IoT NTN enh

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3039  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243058)

Decision: 		The document was agreed.

R5-243059	Addition of new RRM TC 13.5.3 for IoT NTN enh

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3040  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was revised to R5-243690.

R5-243690	Addition of new RRM TC 13.5.3 for IoT NTN enh

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3040  rev 1 Cat: F (Rel-18)

					Source: CMCC

(Replaces R5-243059)

Decision: 		The document was agreed.

R5-243060	Addition of cell configuration mapping for IoT NTN RRM TCs

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3041  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

##### 5.3.41.4	TS 36.521-4

R5-243430	Update to nb-ntn demod test case to update min test time

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0029  Cat: F (Rel-18)

					Source: Qualcomm Inc

Abstract:

associated discussion paper R5-243429

Discussion:

WIC -> WIC -> TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest

r1

Decision: 		The document was revised to R5-243926.

R5-243926	Update to nb-ntn demod test case to update min test time

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0029  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Inc

(Replaces R5-243430)

Decision: 		The document was withdrawn.

##### 5.3.41.5	TS 36.521-2

R5-243061	Update to R18 IoT NTN enh RRM TCs applicability

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1032  Cat: F (Rel-18)

					Source: CMCC

Abstract:

The related CRs are R5-243057(CR#3038), R5-243058(CR#3039) and R5-243059(CR#3040) which add new RRM TCs 13.5.1, 13.5.2 and 13.5.3 in TS 36.521-3.

Decision: 		The document was withdrawn.

##### 5.3.41.6	TR 36.903 (E-UTRAN RRM TT analyses)

##### 5.3.41.7	Discussion Papers, Work Plan, TC lists

### 5.4	Routine Maintenance for 5G NR only TEIx_Test

#### 5.4.1	TS 38.508-1

##### 5.4.1.1	Test frequencies (Clause 4.3.1)

R5-242362	Introduction of test channel bandwidths and test frequencies for band n85

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3118  Cat: F (Rel-18)

					Source: Nokia

Abstract:

Band n85 Low test channel bandwidths and test frequencies for 3 MHz are specified in R5-242394 (CR 3122)

Decision: 		The document was revised to R5-243602.

R5-243602	Introduction of test channel bandwidths and test frequencies for band n85

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3118  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242362)

Decision: 		The document was agreed.

R5-242534	Correction of clause structure of CA test frequencies

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3140  Cat: F (Rel-18)

					Source: Ericsson

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242598	Editorial correction for Editor Note and reference section number in 4.3.1

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3150  Cat: F (Rel-18)

					Source: CAICT

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242601	Addition of CBW 70MHz 90MHz and 100MHz for n40

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3151  Cat: F (Rel-18)

					Source: CAICT

Decision: 		The document was agreed.

R5-242788	Update of editors note for test frequencies

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3177  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Abstract:

Duplicate to Join for review if agreeable from RF group

Discussion:

-2?

r1

Decision: 		The document was revised to R5-243603.

R5-243603	Update of editors note for test frequencies

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3177  rev 1 Cat: F (Rel-18)

					Source: Bureau Veritas ADT

(Replaces R5-242788)

Decision: 		The document was agreed.

R5-242858	Correction to SUL n83 test frequencies in Table 4.3.1.1.1.83-1

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3191  Cat: F (Rel-18)

					Source: Qualcomm Tech. Netherlands B.V

Decision: 		The document was agreed.

R5-243008	Clarification of test frequency for n5 CBW 25 MHz

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3195  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-243293	Addition to 4.3.1 for FR2 Redcap UE test channel bandwidth

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3208  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was revised to R5-243750.

R5-243750	Addition to 4.3.1 for FR2 Redcap UE test channel bandwidth

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3208  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-243293)

Decision: 		The document was agreed.

R5-243294	Corrections on 4.3.1.4.1.2 for test frequency for DC_38A_n78A

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3209  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

##### 5.4.1.2	Test environment for RF (Clauses 5)

R5-243090	Addition of missing TRS configurations

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3199  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243604.

R5-243604	Addition of missing TRS configurations

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3199  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-243090)

Discussion:

"LATE DOCUMENT

Author confirmed withdrawn since not available"

Decision: 		The document was agreed.

R5-243184	Update of message contents for CQI reporting in CA

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3202  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was withdrawn.

##### 5.4.1.3	Test environment for RRM (Clause 7)

R5-242871	Correction to defalut configuration of R15 RACH RRM test cases

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3192  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency R4-2408533 (withdrawn)

Decision: 		The document was withdrawn.

R5-243007	Correction to K1 for RRM TDD SCS 15 kHz

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3194  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

offline comments from Huawei and Keysight Spain.

r2

Decision: 		The document was revised to R5-243833.

R5-243833	Correction to K1 for RRM TDD SCS 15 kHz

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3194  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-243007)

Decision: 		The document was agreed.

##### 5.4.1.4	Other clauses, Annexes

R5-242533	Clarification of Test frequency limitations for CA in Annex C

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3139  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243882.

R5-243882	Clarification of Test frequency limitations for CA in Annex C

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3139  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242533)

Decision: 		The document was agreed.

R5-242786	Update of Channel bandwidth in Default NG-RAN RRC message and information elements contents

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3176  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Abstract:

Duplicate to Join for review if agreeable from RF group

Discussion:

agreed in the joint on Wed

Decision: 		The document was agreed.

R5-243337	Correction on Rel-17 HST conditions in ServingCellConfigCommon

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3210  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

agreed in the joint on Wed

Decision: 		The document was agreed.

#### 5.4.2	TS 38.508-2

R5-242364	Introduction of band n85 for physical layer baseline implementation capabilities

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0617  Cat: F (Rel-18)

					Source: Nokia

Discussion:

Remove 4Rx related changes

r1

Decision: 		The document was revised to R5-243611.

R5-243611	Introduction of band n85 for physical layer baseline implementation capabilities

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0617  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242364)

Decision: 		The document was agreed.

R5-242449	 Update for additional ENDC band configuration

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0621  Cat: F (Rel-18)

					Source: Verizon Spain

Decision: 		The document was agreed.

R5-242784	Removal of defination and abbreviation for IXIT and PIXIT

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0633  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, CATT

Abstract:

Duplicate to Join for review if agreeable from RF group

Discussion:

agreed again in the joint on Wed.

Decision: 		The document was agreed.

R5-242787	Update of Rx antenna ports associated declaration

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0634  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Decision: 		The document was agreed.

R5-243068	Update NR band and CADC configs status in ICS Annex B

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0639  Cat: F (Rel-18)

					Source: CMCC

Decision: 		The document was agreed.

R5-243132	Addition of PICS for TxD for NR FR1 bands

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0642  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Bureau Veritas

Decision: 		The document was agreed.

R5-243413	ICS updates pertaining to RF phase continuity conformance tests

Type: CR		For: Agreement
					38.509 v17.5.0	  CR-0084  Cat: F (Rel-17)

					Source: Apple Benelux B.V.

Discussion:

3GU allocation: 38.509 / AI allocation: 38.508-2

reissued as R5-243446 because of wrong spec

ME x

Decision: 		The document was withdrawn.

R5-243446	ICS updates pertaining to RF phase continuity conformance tests

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0648  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Abstract:

reissued from R5-243413 because of wrong spec

Decision: 		The document was revised to R5-243612.

R5-243612	ICS updates pertaining to RF phase continuity conformance tests

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0648  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-243446)

Decision: 		The document was agreed.

#### 5.4.3	TS 38.509

#### 5.4.4	TS 38.521-1

##### 5.4.4.1	Tx Requirements (Clause 6)

R5-242365	Adding transmitter requirements for band n85

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2745  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

R5-242519	Absolute power control - test requirements correction

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2767  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242520	EVM in transient periods - test case definition update

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2768  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

"AP#97.27 (EVM on transient period EVM)

KS: Ongoing discussions to resolve overlap with R5-243013(Anritsu)

takes less time than implementation in 3013

splits fdd and tdd test implemenation

withdrawn as the group prefers implementation in R5-243013 which has slightly longer test itme but has same implemenation for fdd and tdd bands"

Decision: 		The document was withdrawn.

R5-242918	Correction to initial condition for SUL Tx switching TC 6.3C.3.2.4.1

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2783  Cat: F (Rel-18)

					Source: Qualcomm Tech. Netherlands B.V

Decision: 		The document was agreed.

R5-242945	Core spec alignment Missing DMRS configuration restriction for intra ULCA

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2792  Cat: F (Rel-18)

					Source: Qualcomm Tech. Netherlands B.V

Abstract:

Core spec alignment to RAN 4 apporved CR 	R4-2400569 (approved in Feb 2024 TSG-RAN4 Meeting #110)

Decision: 		The document was agreed.

R5-243010	Correction to test configuration and test requirements for NS_49 A-MPR

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2796  Cat: F (Rel-18)

					Source: Anritsu, China Unicom

Decision: 		The document was agreed.

R5-243011	Correction to test configuration and test requirements for NS_21 A-MPR

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2797  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-243012	Correction to notes for NS_21 A-SEM

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2798  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-243013	Update for EVM including symbols with transient period

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2799  Cat: F (Rel-18)

					Source: Anritsu, Keysight

Decision: 		The document was revised to R5-243839.

R5-243839	Update for EVM including symbols with transient period

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2799  rev 1 Cat: F (Rel-18)

					Source: Anritsu, Keysight

(Replaces R5-243013)

Decision: 		The document was agreed.

R5-243126	Adding clarification of trace mode in emission test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2817  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243127	Correction to general RB allocation in 6.1A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2818  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243128	Cleaning up test applicability for ULFPTx in test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2819  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243201	Updating AMPR requirements for PC3 UE supporting power boosting

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2831  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243202	Updating AMPR testing for CA_n41A-n79A

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2832  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TP in R5-243203

Decision: 		The document was agreed.

R5-243256	Correction of spurious emission minimum conformance requirements for UE co-ex to align with RAN4 specs

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2842  Cat: F (Rel-18)

					Source: Apple, Keysight

Decision: 		The document was agreed.

R5-243258	Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2844  Cat: F (Rel-18)

					Source: Apple, Keysight

Discussion:

r1

Decision: 		The document was revised to R5-243649.

R5-243649	Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2844  rev 1 Cat: F (Rel-18)

					Source: Apple, Keysight

(Replaces R5-243258)

Decision: 		The document was agreed.

R5-243295	Corrections on 6.2 and 6.2A for almost contiguous A-MPR requirements for HPUE

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2850  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

[Core spec alignment]

Discussion:

Wordfilename!

r1

new WIC -> TEI16_Test, LTE_NR_B41_Bn41_PC29dBm-UEConTest

r2

Decision: 		The document was revised to R5-243818.

R5-243818	Corrections on 6.2 and 6.2A for almost contiguous A-MPR requirements for HPUE

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2850  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-243295)

Decision: 		The document was agreed.

R5-243296	Corrections on 6.2.3 for UE additional maximum output power reduction requirements

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2851  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

[Editorial Correction]

Decision: 		The document was agreed.

R5-243297	Corrections on 6.5.2.2 for UE spectrum emission mask

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2852  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-243298	Corrections on 6.5.3.3 for UE additional spurious emissions

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2853  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-243299	Corrections on clause 6 for removal of power class capable UE denotation

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2854  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-243411	Updates to FR1 RF phase continuity test

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1056  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

3GU allocation: 38.521-2 / AI allocation: 38.521-1

reissued as R5-243447 because of wrong spec

Decision: 		The document was withdrawn.

R5-243447	Updates to FR1 RF phase continuity test

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2860  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Abstract:

reissued from R5-243411 because of wrong spec

Discussion:

r1

Decision: 		The document was revised to R5-243650.

R5-243650	Updates to FR1 RF phase continuity test

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2860  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-243447)

Decision: 		The document was agreed.

##### 5.4.4.2	Rx Requirements (Clause 7)

R5-242284	Update of Rx test cases to add 50MHz for NR band n7

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2740  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

CoreSpecAlig

Decision: 		The document was revised to R5-243625.

R5-243625	Update of Rx test cases to add 50MHz for NR band n7

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2740  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-242284)

Decision: 		The document was agreed.

R5-242366	Adding receiver requirements for band n85

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2746  Cat: F (Rel-18)

					Source: Nokia

Discussion:

Remove 4Rx related changes

r1

Decision: 		The document was revised to R5-243651.

R5-243651	Adding receiver requirements for band n85

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2746  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242366)

Decision: 		The document was agreed.

R5-242517	Introduction of new SUL test case 7.7C

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2766  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243897.

R5-243897	Introduction of new SUL test case 7.7C

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2766  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242517)

Decision: 		The document was agreed.

R5-242521	Clarification for Rx test requirements for PC1.5

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2769  Cat: F (Rel-18)

					Source: Keysight Technologies, Huawei

Decision: 		The document was agreed.

R5-242948	Addition of REFSENS for PC3 CA n78A-n79A with 1UL

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2793  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Discussion:

r1

Decision: 		The document was revised to R5-243824.

R5-243824	Addition of REFSENS for PC3 CA n78A-n79A with 1UL

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2793  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

(Replaces R5-242948)

Decision: 		The document was agreed.

R5-243279	Updates to Reference sensitivity for SUL

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2848  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R5-243300	Cleanups on 7.3.2 for reference sensitivity power level test

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2855  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-243319	Update of Refsens TC for RedCap UE

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2857  Cat: F (Rel-18)

					Source: China Unicom

Discussion:

in 3GU had wrong Rel, ver, & NR_redcap_plus_ARCH-UEConTest!

Decision: 		The document was agreed.

##### 5.4.4.3	Clauses 1-5, Annexes

R5-242516	Corrected MU and TT definition for FR1 UL MIMO test case 6.5D.2.3

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2765  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242522	MU and TT defintion for TxD EVM test case 6.4G.2.1

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2770  Cat: F (Rel-18)

					Source: Keysight Technologies, Rohde&Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243898.

R5-243898	MU and TT defintion for TxD EVM test case 6.4G.2.1

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2770  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies, Rohde&Schwarz

(Replaces R5-242522)

Decision: 		The document was agreed.

R5-242571	General updates of clause 5 for R17 new CBW configurations

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2774  Cat: F (Rel-18)

					Source: China Unicom, Nokia, CAICT

Decision: 		The document was agreed.

R5-243023	Correction to notes for support of SCS and CBW in CA configurations

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2801  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-24xxxxx

Discussion:

r2

Decision: 		The document was revised to R5-243912.

R5-243912	Correction to notes for support of SCS and CBW in CA configurations

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2801  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-243023)

Discussion:

"Revised from: R5-243023r2.

Depending on RAN4 CR R4-2410670 (agreed!)

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243130	Editorial correction of Annex F for R17 FR1 test cases

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2820  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

Editorial

Decision: 		The document was agreed.

R5-243301	Updates on 3.1 and 4.3 for Definitions and Specification suffix information

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2856  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

[Core spec alignment]

Discussion:

resolve the overlapping with R5-243036.

r2

new WID -> TEI17_Test, NR_RF_FR1_enh-UEConTest

Decision: 		The document was revised to R5-243819.

R5-243819	Updates on 3.1 and 4.3 for Definitions and Specification suffix information

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2856  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-243301)

Decision: 		The document was agreed.

R5-243336	Addition of suffix information for Carrier Aggregation (CA) with UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2858  Cat: F (Rel-18)

					Source: China Unicom

Discussion:

wrong Rel, ver!

r1

Decision: 		The document was revised to R5-243879.

R5-243879	Addition of suffix information for Carrier Aggregation (CA) with UL MIMO

Type: CR		For: Agreement
					38.521-1 v18.2.0	  CR-2858  rev 1 Cat: F (Rel-18)

					Source: China Unicom

(Replaces R5-243336)

Decision: 		The document was agreed.

#### 5.4.5	TS 38.521-2

##### 5.4.5.1	Tx Requirements (Clause 6)

R5-242260	Pending UL MIMO update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1041  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242263	FR2c MU - Tx test cases update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1042  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243726.

R5-243726	FR2c MU - Tx test cases update in 38.521-2

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1042  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242263)

Decision: 		The document was agreed.

R5-242999	Update for FR2c MU

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1048  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

AP#99.21; Depending on the outcome of MU discussion R5-242997, R5-242998

Discussion:

r2

Decision: 		The document was revised to R5-243831.

R5-243831	Update for FR2c MU

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1048  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-242999)

Decision: 		The document was agreed.

R5-243105	Update of MU and TT for CA and UL MIMO

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1052  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Discussion in R5-243103, update of TR 38.903 in CR R5-243104, update of EN-DC test cases in R5-243106, applicability update in CR R5-243107

Completing test cases 6.2D.2, 6.5D.2.1 and 6.5D.2.2 depends on Keysight's CR R5-242260.

Discussion:

r2

Decision: 		The document was revised to R5-243830.

R5-243830	Update of MU and TT for CA and UL MIMO

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1052  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-243105)

Decision: 		The document was agreed.

R5-243302	Corrections on 6.2.2 and 6.2A.2.0.2 for UE MPR requirements

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1053  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

[Core Spec alignment]

Decision: 		The document was revised to R5-243660.

R5-243660	Corrections on 6.2.2 and 6.2A.2.0.2 for UE MPR requirements

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1053  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-243302)

Decision: 		The document was agreed.

R5-243412	Updates to FR2 RF phase continuity test

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1057  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

invert WIC!

r1

Decision: 		The document was revised to R5-243661.

R5-243661	Updates to FR2 RF phase continuity test

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1057  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-243412)

Decision: 		The document was agreed.

##### 5.4.5.2	Rx Requirements (Clause 7)

R5-243102	Update of MU for n259

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1051  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Discussion in R5-243100, update of TR 38.903 in CR R5-243101

Discussion:

r1

Decision: 		The document was revised to R5-243727.

R5-243727	Update of MU for n259

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1051  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-243102)

Decision: 		The document was agreed.

R5-243303	Corrections on 7.1 for general description to receiver characteristics

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1054  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

[Core Spec alignment]

Discussion:

r1

Decision: 		The document was revised to R5-243662.

R5-243662	Corrections on 7.1 for general description to receiver characteristics

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1054  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-243303)

Decision: 		The document was agreed.

##### 5.4.5.3	Clauses 1-5, Annexes

R5-243014	Clarification of antenna array assumptions for in-band measurement

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1049  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was revised to R5-243663.

R5-243663	Clarification of antenna array assumptions for in-band measurement

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1049  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-243014)

Decision: 		The document was agreed.

R5-243091	Correction to Rx fast spherical coverage method

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1050  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was revised to R5-243664.

R5-243664	Correction to Rx fast spherical coverage method

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1050  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-243091)

Decision: 		The document was agreed.

R5-243309	Clarification of QoQZ Validation Procedure

Type: CR		For: Agreement
					38.521-2 v18.2.0	  CR-1055  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd, ETS-Lindgren

Decision: 		The document was agreed.

#### 5.4.6	TS 38.521-3

##### 5.4.6.1	Tx Requirements (Clause 6)

R5-242269	PC5 FR2 MU - Updates in 38.521-3

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1759  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243728.

R5-243728	PC5 FR2 MU - Updates in 38.521-3

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1759  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242269)

Decision: 		The document was agreed.

R5-242724	Correct typo for clause 6.5B.3.2.1.4 and 6.5B.3.4.1.4.1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1768  Cat: F (Rel-18)

					Source: SGS Wireless

Decision: 		The document was agreed.

R5-242785	Correction to IE name higherPowerLimit used in test cases

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1769  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Decision: 		The document was agreed.

R5-242856	Update of UE capability name for higher power limit

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1775  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

core spec alignment

Discussion:

"core spec alignment

Author requested withdrawn as fully covered in R5-242785(BV)"

Decision: 		The document was withdrawn.

R5-243000	Update for FR2c MU

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1781  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

AP#99.21; Depending on the outcome of MU discussion R5-242997

Discussion:

"AP#99.21; Depending on the outcome of MU discussion R5-242997

5/22:

Moderator (AT&T): According to the proponent, this CR should be withdrawn since Off Power cannot be completed at this meeting."

Decision: 		The document was withdrawn.

R5-243015	Correction to notes for FR1 EN-DC Spurious emission band UE co-existence

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1782  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-243092	Correction to Inter-band EN-DC spurious emission requirements

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1783  Cat: F (Rel-18)

					Source: Rohde & Schwarz, Oppo

Abstract:

dependent on R4-2408769

Discussion:

r1

Decision: 		The document was revised to R5-243840.

R5-243840	Correction to Inter-band EN-DC spurious emission requirements

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1783  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz, Oppo

(Replaces R5-243092)

Decision: 		The document was agreed.

R5-243106	Update of status of FR2 CA and UL MIMO test cases

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1784  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Discussion in R5-243103, update of TR 38.903 in CR R5-243104, SA test case update in CR R5-243105, applicability update in CR R5-243107

Decision: 		The document was agreed.

R5-243108	Addition of power tolerance CA test cases for FR2

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1785  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Applicability update in CR R5-243109, SA test case update in CR R5-243105

Decision: 		The document was agreed.

R5-243304	Corrections on 6.2B.1.3 for UE MOP for configuration DC_38A_n78A

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1793  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

r1

Decision: 		The document was revised to R5-243671.

R5-243671	Corrections on 6.2B.1.3 for UE MOP for configuration DC_38A_n78A

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1793  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-243304)

Decision: 		The document was agreed.

##### 5.4.6.2	Rx Requirements (Clause 7)

R5-242797	Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC3

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1772  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Discussion:

r1

Decision: 		The document was revised to R5-243825.

R5-243825	Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC3

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1772  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

(Replaces R5-242797)

Decision: 		The document was agreed.

R5-242855	Update of 7.3B.2 reference sensitivity for inter-band EN-DC

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1774  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

overlaps with R5-242254.

r1

Decision: 		The document was revised to R5-243672.

R5-243672	Update of 7.3B.2 reference sensitivity for inter-band EN-DC

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1774  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242855)

Decision: 		The document was agreed.

R5-243204	Updating description in test configuration table of test case 7.3B.2.3_1.1

Type: CR		For: Agreement
					38.521-3 v18.2.0	  CR-1786  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 5.4.6.3	Clauses 1-5, Annexes

#### 5.4.7	TS 38.521-4

##### 5.4.7.1	Conducted Demod Performance and CSI Reporting Requirements (Clauses 5&6)

R5-243017	Correction to PUCCH Resource ID in 2Rx FR1 RI reporting

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0834  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

r1

Decision: 		The document was revised to R5-243695.

R5-243695	Correction to PUCCH Resource ID in 2Rx FR1 RI reporting

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0834  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-243017)

Decision: 		The document was agreed.

R5-243024	Correction to test parameters in FR1 HST DPS

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0835  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2407179

Discussion:

r1

Decision: 		The document was revised to R5-243913.

R5-243913	Correction to test parameters in FR1 HST DPS

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0835  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-243024)

Discussion:

"Revised from: R5-243024r1.

Depending on RAN4 CR R4-2409932

Document was sent FOR E-MAIL AGREEMENT

"

R4-2409932 (R4-2407179r1) was agreed in RAN4 Demodulation session.

email agreed

Decision: 		The document was agreed.

R5-243089	Correction to PDSCH TDD Test Cases

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0836  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-243093	Correction to URLLC message exceptions

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0837  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-243111	Editorial correction of UE capability in PDSCH repetition test cases

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0839  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Editorial

Decision: 		The document was agreed.

R5-243338	Update for PMI TypeII codebook test cases

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0842  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

##### 5.4.7.2	Radiated Demod Performance and CSI Reporting Requirements (Clauses 7&8)

##### 5.4.7.3	Interworking Demod Performance and CSI Reporting Requirements (Clauses 9&10)

R5-243432	Addition of NSA SDR test case for DL 1024QAM

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0844  Cat: F (Rel-18)

					Source: Qualcomm Inc

Decision: 		The document was revised to R5-243696.

R5-243696	Addition of NSA SDR test case for DL 1024QAM

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0844  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Inc

(Replaces R5-243432)

Decision: 		The document was agreed.

##### 5.4.7.4	Clauses 1-4, Annexes

R5-243016	Correction to PDSCH Reference Channel for FDD HST

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0833  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was revised to R5-243697.

R5-243697	Correction to PDSCH Reference Channel for FDD HST

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0833  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-243016)

Decision: 		The document was agreed.

R5-243110	Update of MU and TT for FR1 CSI reporting

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0838  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

+TEI17!!!

Decision: 		The document was agreed.

R5-243339	Update mTT for missing DL RMC

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0843  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

also impacted WICS: TEI17_Test, NR_HST-UEConTest, NR_redcap_plus_ARCH-UEConTest!

Decision: 		The document was revised to R5-243698.

R5-243698	Update mTT for missing DL RMC

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0843  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-243339)

Decision: 		The document was agreed.

R5-243436	Corrections to Table G.1.5-1 for entries corresponding to FR1.15-1

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0847  Cat: F (Rel-18)

					Source: Qualcomm Inc

Decision: 		The document was revised to R5-243699.

R5-243699	Corrections to Table G.1.5-1 for entries corresponding to FR1.15-1

Type: CR		For: Agreement
					38.521-4 v18.2.0	  CR-0847  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Inc

(Replaces R5-243436)

Decision: 		The document was agreed.

#### 5.4.8	TS 38.521-5

#### 5.4.9	TS 38.522

R5-242523	Correction on applicability for PC1.5 in Rx tests

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0411  Cat: F (Rel-18)

					Source: Keysight Technologies, Huawei

Decision: 		The document was agreed.

R5-242726	Correction condition F011 for 38.533 TC16.4.1.1

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0419  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

had wrong WIC in 3GU!

r2

Decision: 		The document was revised to R5-243910.

R5-243910	Correction condition F011 for 38.533 TC16.4.1.1

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0419  rev 1 Cat: F (Rel-18)

					Source: SGS Wireless

(Replaces R5-242726)

Decision: 		The document was agreed.

R5-242789	Correction to applicability of 5G test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0420  Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Keysight

Discussion:

r1

Decision: 		The document was revised to R5-243720.

R5-243720	Correction to applicability of 5G test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0420  rev 1 Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Keysight

(Replaces R5-242789)

Discussion:

"Revised from: R5-242789r1.

Huawei requests to send the CR for email to check on the band applicability of 2Rx/4Rx demod requirements .

BV has more changes to be done which was missed in the CR

Document was sent FOR E-MAIL AGREEMENT

"

Huawei: agree to the second change to D029.

Regarding the first change, we don’t agree. The RAN4 applicability rule indicates that if the UE supports 4Rx on any band, 2Rx test cases are skipped. This is clearly indicated in the RAN4 way forward and RAN4 specification.

r2

Decision: 		The document was revised to R5-244010.

R5-244010	Correction to applicability of 5G test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0420  rev 2 Cat: F (Rel-18)

					Source: Bureau Veritas ADT, Keysight

(Replaces R5-243720)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242877	Correction to applicability for RedCap RRM test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0423  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-243915.

R5-243915	Correction to applicability for RedCap RRM test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0423  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242877)

Discussion:

"Revised from: R5-242877r1.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244011.

R5-244011	Correction to applicability for RedCap RRM test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0423  rev 2 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243915)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242984	Addition of missing applicability to RedCap SS-RSRP test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0425  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-243107	Update of applicability of FR2 CA and MIMO test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0429  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

SA test case update in CR R5-243105, NSA test case update in CR R5-243106

Discussion:

r1

Decision: 		The document was revised to R5-243729.

R5-243729	Update of applicability of FR2 CA and MIMO test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0429  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-243107)

Decision: 		The document was agreed.

R5-243109	Applicability for FR2 NSA power tolerance CA test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0430  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Addition of test cases in TS 38.521-3 in CR R5-243108, SA test case update in CR R5-243105

Decision: 		The document was agreed.

R5-243129	Cleaning up test applicability for ULFPTx

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0431  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243133	Updating applicability for Rx test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0432  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

Author requests to withdraw

Decision: 		The document was withdrawn.

R5-243396	Applicability correction to gap-based measurement tests for RedCap

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0439  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependency: R4-2407570 (agreed)

Discussion:

"RAN4 dependency: R4-2407570

TC CR pending verdict

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243397	Applicability correction to gap-based measurement tests

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0440  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependency: R4-2407449 (revised to R4-2410217 (agreed)

Discussion:

"RAN4 dependency: R4-2407449

TC CR pending verdict

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243414	Applicability updates for Phase continuity tests

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0441  Cat: F (Rel-18)

					Source: Apple Benelux B.V.

Discussion:

invert WIC!

r1

Decision: 		The document was revised to R5-243813.

R5-243813	Applicability updates for Phase continuity tests

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0441  rev 1 Cat: F (Rel-18)

					Source: Apple Benelux B.V.

(Replaces R5-243414)

Decision: 		The document was agreed.

R5-243439	Appllicability spec update to allow skipping 2DLCA and 3DCLA PDSCH Demodulation test cases

Type: CR		For: Agreement
					38.522 v18.2.0	  CR-0443  Cat: F (Rel-18)

					Source: Qualcomm Inc

Discussion:

3GU allocation: 38.522 / AI allocation: 38.521-4

Decision: 		The document was agreed.

#### 5.4.10	TS 38.533

##### 5.4.10.1	EN-DC with all NR cells in FR1 (Clause 4)

R5-242727	Correct Table 4.5.2.1.4.1-2 & Table 4.5.2.2.4.1-2 & Table 4.5.2.3.4.1-2 & Table 4.5.2.4.4.1-2 & Table 4.5.2.5.4.1-2 of test frequency

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3126  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

sub-AI??

Decision: 		The document was agreed.

R5-242983	Editorial correction of references to TS 38.101-1 in section 4

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3171  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-243018	Correction to message exception in non-contention based random access

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3182  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was withdrawn.

R5-243019	Correction to K1 in 4.6.4.3 and 6.6.4.3 for FDD and TDD SCS 30 kHz

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3183  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-243340	Correction to FR1 NSA Event Trigger TC 4.6.1.8

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3215  Cat: F (Rel-18)

					Source: Keysight Technologies

Abstract:

Depending on RAN4 CR R4-2407445

Discussion:

r2

Decision: 		The document was revised to R5-243909.

R5-243909	Correction to FR1 NSA Event Trigger TC 4.6.1.8

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3215  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-243340)

Discussion:

"Revised from: R5-243340r2.

RAN4 CR R4-2410247

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

RAN4 agreement in R4-2410422 with same technical content as in R4-2410247.

R4-2410247 was revised to R4-2410422 to fix some cover page issues.

Email agreed

Decision: 		The document was agreed.

R5-243389	Update to gap-based measurement reporting tests for HST

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3250  Cat: F (Rel-18)

					Source: Qualcomm France

Discussion:

r1

Decision: 		The document was revised to R5-243711.

R5-243711	Update to gap-based measurement reporting tests for HST

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3250  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243389)

Decision: 		The document was agreed.

R5-243390	Update to Rel.17 HST CA measurement reporting tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3251  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependency: R4-2407335

Discussion:

r1

Decision: 		The document was revised to R5-243901.

R5-243901	Update to Rel.17 HST CA measurement reporting tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3251  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243390)

Abstract:

Dependent on R4-2407335 (revised to R4-2410221, agreed)

Discussion:

"Revised from: R5-243390r1.

RAN4 dependency: R4-2407335

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243392	Correction to SUL test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3253  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependency: R4-2407337

Discussion:

r1

Decision: 		The document was revised to R5-243902.

R5-243902	Correction to SUL test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3253  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243392)

Abstract:

Dependent on R4-2407978 (R4-2407337 was withdrawn in favor of R4-2407978) (revised to R4-2410238, agreed)

Discussion:

"Revised from: R5-243392r1.

RAN4 dependency: R4-2407337

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

##### 5.4.10.2	NE-DC with all NR cells in FR1 (Clause 4A)

##### 5.4.10.3	EN-DC with at least 1 NR Cell in FR2 (Clause5)

R5-242982	Editorial correction of references to TS 38.101-1 in section 5

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3170  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

##### 5.4.10.4	NR Standalone in FR1 (Clause 6)

R5-242872	Correction to R15 L1-RSRP absolute accuracy RRM test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3133  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242873	Correction to R15 RACH RRM test cases and RMCs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3134  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency R4-2408533 (withdrawn)

Discussion:

r3

Decision: 		The document was revised to R5-243916.

R5-243916	Correction to R15 RACH RRM test cases and RMCs

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3134  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242873)

Decision: 		The document was agreed.

R5-242874	Correction to R15 reestablishment RRM test case 6.3.2.1.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3135  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 CR R4-2408537 (agreed)

Discussion:

r1

Decision: 		The document was revised to R5-243917.

R5-243917	Correction to R15 reestablishment RRM test case 6.3.2.1.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3135  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242874)

Discussion:

"Revised from: R5-242874r1.

RAN4 dependent CR R4-2408537

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

R5-242875	Correction to R15 SCell activation test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3136  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

Decision: 		The document was agreed.

R5-242876	Correction to R16 relaxed measurement RRM test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3137  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242980	Editorial correction of references to TS 38.101-1 in section 6.7

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3168  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-242981	Editorial correction of references to TS 38.101-1 in section 6.6

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3169  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-243020	Correction to DCI trigger in 6.6.4.3 for FDD and TDD SCS 15 kHz

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3184  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-243393	Correction to Handover tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3254  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

##### 5.4.10.5	NR standalone with at least one NR cell in FR2 (Clause7)

R5-242611	Incorrect clause number for test case 7.6.13.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3101  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

R5-242979	Editorial correction of references to TS 38.101-1 in section 7

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3167  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-242992	Correction to frequencies table for RRM FR2 SA test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3179  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-242993	Corrections to message contents for SA FR2 event triggered intra-frequency test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3180  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-242994	Corrections to message contents for SA FR2 timing test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3181  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

##### 5.4.10.6	E-UTRA – NR Inter-RAT with E-UTRA serving cell (Clause 8)

R5-243391	Correction to gap-based measurement tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3252  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

Dependent on R4-2407449 (revised to R4-2410217, agreed)

Discussion:

"RAN4 dependency: R4-2407449

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

##### 5.4.10.7	Clauses 1-3, Annexes

R5-242904	Correction to Annex E for RedCap RRM test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3164  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242905	Correction to Annex F for RedCap RRM test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3165  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243746.

R5-243746	Correction to Annex F for RedCap RRM test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3165  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242905)

Discussion:

"Revised from: R5-242905r2.

Document was sent FOR E-MAIL AGREEMENT

"

r2

Decision: 		The document was revised to R5-244012.

R5-244012	Correction to Annex F for RedCap RRM test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3165  rev 2 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243746)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-243344	Editorial correction in Annex E for FR1 RedCap TCs 16.6.2.3, 16.6.2.4

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3219  Cat: F (Rel-18)

					Source: Keysight Technologies

Abstract:

Editorial correction

Decision: 		The document was agreed.

##### 5.4.10.8	NR standalone tests with all NR cells in FR1 for RedCap (Clause 16)

R5-242433	Update test requirement for Redcap inter frequency test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3095  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

RAN4 dependency

RAN4#111 R4-2409798, revised to R4-2410377 (agreed)

Discussion:

r3

Decision: 		The document was revised to R5-243907.

R5-243907	Update test requirement for Redcap inter frequency test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3095  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-242433)

Abstract:

dependent on R4-2409798. But R4-2409798 is revised to R4-2410377. And R4-2410377 is agreed.

Discussion:

"Revised from: R5-242433r3.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244013.

R5-244013	Update test requirement for Redcap inter frequency test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3095  rev 2 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-243907)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242878	Correction to RedCap RRM test case 16.1.1.5 and 16.1.1.7

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3138  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242879	Correction to RedCap RRM test case 16.1.2.5 and 16.1.2.6

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3139  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint, Qualcomm

Abstract:

RAN4 dependencyR4-2408545 -> R4-2410348(Agreed)

Discussion:

r1

Decision: 		The document was revised to R5-243918.

R5-243918	Correction to RedCap RRM test case 16.1.2.5 and 16.1.2.6

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3139  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Starpoint, Qualcomm

(Replaces R5-242879)

Discussion:

"Revised from: R5-242879r1.

RAN4 dependency R4-2408545

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

R5-242880	Correction to RedCap RRM test case 16.2.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3140  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

"RRM TT

5/21 Moderator (E///): Needs to go for email agreement due to RAN4 discussion

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244014.

R5-244014	Correction to RedCap RRM test case 16.2.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3140  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242880)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242881	Correction to RedCap RRM test case 16.2.1.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3141  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2409135 -> R4-2410226(Agreed)

Discussion:

"RRM TT, RAN4 dependent CR R4-2409135

5/21 Moderator (E///): Needs to go for email agreement due to RAN4 discussion

Document was sent FOR E-MAIL AGREEMENT

"

r2

Decision: 		The document was revised to R5-244015.

R5-244015	Correction to RedCap RRM test case 16.2.1.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3141  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242881)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242986	Editorial correction to title of R17 RedCap test case 16.6.4.7

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3173  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Decision: 		The document was agreed.

R5-242987	Test procedure correction for R17 RedCap NCD-SSB test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3174  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

"KS request more time, requests for email

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

R5-243021	Correction to test parameters in 16.5.1.7

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3185  Cat: F (Rel-18)

					Source: Anritsu

Discussion:

overlap with R5-243335 Qualcomm

Decision: 		The document was withdrawn.

R5-243025	Correction to SMTC settings in 16.6.2.9 and 16.6.2.10

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3186  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on RAN4 CR R4-2407173

Decision: 		The document was agreed.

R5-243095	Removal of Editors note for applicability of RedCap SS-RSRP test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3187  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Related to R5-242984

Decision: 		The document was agreed.

R5-243096	Core spec alignment for R17 RedCap NCD-SSB test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3188  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Abstract:

Depends on Ran4 CR-2407390

Discussion:

Dependent R4-2407390 will also be withdrawn.

Decision: 		The document was withdrawn.

R5-243335	Correction to RRM RedCap TC 16.5.1.7

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3214  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int, Anritsu

Discussion:

r1

Decision: 		The document was revised to R5-243712.

R5-243712	Correction to RRM RedCap TC 16.5.1.7

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3214  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int, Anritsu

(Replaces R5-243335)

Decision: 		The document was agreed.

R5-243341	Corrections to NR FR1 RedCap E-UTRA cell re-selection TCs 16.1.2.1, 16.1.2.3

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3216  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-243342	Editorial correction in Redcap 16.6.2.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3217  Cat: F (Rel-18)

					Source: Keysight Technologies

Abstract:

Editorial correction

Decision: 		The document was agreed.

R5-243384	Correction to stationary relaxed measurement criterion tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3245  Cat: F (Rel-18)

					Source: Qualcomm France

Decision: 		The document was agreed.

R5-243385	Correction to stationary relaxed measurement criterion inter-RAT tests

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3246  Cat: F (Rel-18)

					Source: Qualcomm France

Discussion:

merged into R5-242879r1

"QC: Full overlap with R5-242879(HW). Discussion needed.

Content merged into R5-242879(HW)"

Decision: 		The document was withdrawn.

R5-243386	Correction to re-establishment test cases for RedCap

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3247  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependency: R4-2407568

Discussion:

r1

Decision: 		The document was revised to R5-243903.

R5-243903	Correction to re-establishment test cases for RedCap

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3247  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243386)

Abstract:

Dependent on R4-2407568 and R4-2408537 (revised to R4-2410387) (both agreed)

Discussion:

"Revised from: R5-243386r1.

RAN4 dependency: R4-2407568, R4-2408537

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243387	Correction to event-triggered measurement test cases for RedCap

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3248  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependency: R4-2407572

Discussion:

Full overlap with R5-243443.

merged into R5-243443r1.

"RAN4 dependency: R4-2407572

QC: Full overlap with R5-243443(HW). Both CRs are attempting the same change, and can be merged after RAN5 discussion and RAN4 conclusion.

Content merged into R5-243443(HW)"

Decision: 		The document was withdrawn.

R5-243443	Correction to RedCap inter-RAT measurement test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3255  Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Qualcomm

Abstract:

RAN4 dependency R4-2408543

Discussion:

late doc

r1

Decision: 		The document was revised to R5-243919.

R5-243919	Correction to RedCap inter-RAT measurement test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3255  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon, Qualcomm

(Replaces R5-243443)

Abstract:

R4-2408545 -> R4-2410348(Agreed)

Discussion:

"Revised from: R5-243443r1.

RAN4 dependency R4-2408545

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

##### 5.4.10.9	NR standalone tests with all NR cells in FR2 for RedCap (Clause 17)

R5-242882	Correction to RedCap RRM test case 17.2.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3142  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency

RRM TT R4-2408545

Discussion:

"RAN4 dependency R4-2408545, R4-2409135

RRM TT

5/21 Moderator (E///): Needs to go for email agreement due to RAN4 discussion

Document was sent FOR E-MAIL AGREEMENT

"

Changes are not agreed in RAN4

email withdrawn

Decision: 		The document was withdrawn.

R5-242883	Correction to RedCap RRM test case 17.3.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3143  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-242884	Correction to RedCap RRM test case 17.3.1.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3144  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-242885	Correction to RedCap RRM test case 17.3.2.3.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3145  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-242886	Correction to RedCap RRM test case 17.4.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3146  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243774.

R5-243774	Correction to RedCap RRM test case 17.4.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3146  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242886)

Decision: 		The document was agreed.

R5-242887	Correction to RedCap RRM test case 17.4.3.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3147  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-242888	Correction to RedCap RRM test case 17.5.1.5 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3148  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

RAN4 requirements discrepancy issue found during TT reviewing

Decision: 		The document was withdrawn.

R5-242889	Correction to RedCap RRM test case 17.5.1.6 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3149  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

RAN4 requirements discrepancy issue found during TT reviewing

Decision: 		The document was withdrawn.

R5-242890	Correction to RedCap RRM test case 17.5.1.7 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3150  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

RAN4 requirements discrepancy issue found during TT reviewing

Decision: 		The document was withdrawn.

R5-242891	Correction to RedCap RRM test case 17.5.1.8 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3151  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

18.2.0

r1

RAN4 requirements discrepancy issue found during TT reviewing

Decision: 		The document was revised to R5-243701.

R5-243701	Correction to RedCap RRM test case 17.5.1.8 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3151  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242891)

Decision: 		The document was withdrawn.

R5-242892	Correction to RedCap RRM test case 17.5.4.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3152  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency

RRM TT R4-2408545 -> R4-2410348(Agreed)

Discussion:

"RAN4 dependency R4-2408545, RRM TT

Author confirmed no overlap/conflicts

5/21 Moderator (E///): Good to go after RAN4 flag is cleared

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

R5-242893	Correction to RedCap RRM test case 17.5.4.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3153  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency

RRM TT R4-2408545 -> R4-2410348(Agreed)

Discussion:

"RAN4 dependency R4-2408545, RRM TT

Author confirmed no overlap/conflicts

5/21 Moderator (E///): Good to go after RAN4 flag is cleared

Document was sent FOR E-MAIL AGREEMENT

"

email agreed

Decision: 		The document was agreed.

R5-242894	Correction to RedCap RRM test case 17.5.5.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3154  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

+TEI17!

r1

Decision: 		The document was revised to R5-243775.

R5-243775	Correction to RedCap RRM test case 17.5.5.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3154  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242894)

Decision: 		The document was agreed.

R5-242895	Correction to RedCap RRM test case 17.5.5.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3155  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Decision: 		The document was agreed.

R5-242896	Correction to RedCap RRM test case 17.7.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3156  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243776.

R5-243776	Correction to RedCap RRM test case 17.7.1.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3156  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242896)

Decision: 		The document was agreed.

R5-242897	Correction to RedCap RRM test case 17.7.1.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3157  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243777.

R5-243777	Correction to RedCap RRM test case 17.7.1.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3157  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242897)

Decision: 		The document was agreed.

R5-242898	Correction to RedCap RRM test case 17.7.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3158  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243778.

R5-243778	Correction to RedCap RRM test case 17.7.2.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3158  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242898)

Decision: 		The document was agreed.

R5-242899	Correction to RedCap RRM test case 17.7.2.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3159  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243779.

R5-243779	Correction to RedCap RRM test case 17.7.2.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3159  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242899)

Decision: 		The document was agreed.

R5-242900	Correction to RedCap RRM test case 17.7.3.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3160  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243780.

R5-243780	Correction to RedCap RRM test case 17.7.3.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3160  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242900)

Decision: 		The document was agreed.

R5-242901	Correction to RedCap RRM test case 17.7.3.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3161  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243781.

R5-243781	Correction to RedCap RRM test case 17.7.3.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3161  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242901)

Decision: 		The document was agreed.

R5-242902	Correction to RedCap RRM test case 17.7.4.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3162  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243782.

R5-243782	Correction to RedCap RRM test case 17.7.4.1 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3162  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242902)

Decision: 		The document was agreed.

R5-242903	Addition of RedCap RRM test case 17.7.4.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3163  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

r1

Decision: 		The document was revised to R5-243783.

R5-243783	Addition of RedCap RRM test case 17.7.4.2 with TT

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3163  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242903)

Decision: 		The document was agreed.

##### 5.4.10.10	E-UTRA - NR inter-RAT with E-UTRA serving cell (Clause 18)

R5-242985	Corrections to R17 RedCap 18.3.1.x test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3172  Cat: F (Rel-18)

					Source: Rohde & Schwarz

Discussion:

overlap with - R5-243383

'o'

r1

Decision: 		The document was revised to R5-243713.

R5-243713	Corrections to R17 RedCap 18.3.1.x test cases

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3172  rev 1 Cat: F (Rel-18)

					Source: Rohde & Schwarz

(Replaces R5-242985)

Decision: 		The document was agreed.

R5-243343	Correction to FR1 RedCap E-UTRA - NR inter - RAT Handover TC 18.2.1.1

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3218  Cat: F (Rel-18)

					Source: Keysight Technologies

Abstract:

Depending on RAN4 CR R4-2407447

Decision: 		The document was agreed.

R5-243383	Correction to gap-based measurement tests for RedCap

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3244  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

RAN4 dependency: R4-2407570

Discussion:

r1

Decision: 		The document was revised to R5-243904.

R5-243904	Correction to gap-based measurement tests for RedCap

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3244  rev 1 Cat: F (Rel-18)

					Source: Qualcomm France

(Replaces R5-243383)

Abstract:

Dependent on R4-2407570 (agreed)

Discussion:

"Revised from: R5-243383r1.

RAN4 dependency: R4-2407570

Document was sent FOR E-MAIL AGREEMENT

"

Email agreed

Decision: 		The document was agreed.

R5-243388	Editorial correction to section 18 title

Type: CR		For: Agreement
					38.533 v18.2.1	  CR-3249  Cat: F (Rel-18)

					Source: Qualcomm France

Abstract:

Editorial

Decision: 		The document was agreed.

#### 5.4.11	TS 38.551

R5-243224	CR to TS 38.551 on Annex A.2.3

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0010  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial change

Discussion:

r1

redo the forgotten change on R5-241935 [RAN5#102] which voided cl. A.2.3.

Decision: 		The document was revised to R5-243800.

R5-243800	CR to TS 38.551 on Annex A.2.3

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0010  rev 1 Cat: F (Rel-17)

					Source: Apple (UK) Limited

(Replaces R5-243224)

Decision: 		The document was agreed.

R5-243227	CR to TS 38.551 on Annex C, editorial

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0011  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial change

Decision: 		The document was agreed.

R5-243230	CR to TS 38.551 on Annex G, editorial

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0012  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial change

Decision: 		The document was agreed.

R5-243232	CR to TS 38.551 on EUT positioning

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0013  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial change

Decision: 		The document was agreed.

R5-243233	CR to TS 38.551 on section 6.1.2

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0014  Cat: F (Rel-17)

					Source: Apple (UK) Limited

Abstract:

Editorial change

Decision: 		The document was agreed.

R5-243312	Clarification of Calibration and Ripple Test Procedures

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0015  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd, ETS-Lindgren

Decision: 		The document was agreed.

R5-243313	Clarification of Channel Model Coordinate System and Probe Placement

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0016  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd, OPPO

Decision: 		The document was agreed.

R5-243316	On Testing with Battery

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0017  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Discussion:

r1

w/d

Decision: 		The document was revised to R5-243801.

R5-243801	On Testing with Battery

Type: CR		For: Agreement
					38.551 v17.1.0	  CR-0017  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

(Replaces R5-243316)

Decision: 		The document was withdrawn.

#### 5.4.12	TS 38.561

R5-243311	Clarification of Calibration and Ripple Test Procedures

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0006  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd, ETS-Lindgren

Discussion:

r1

Decision: 		The document was revised to R5-243797.

R5-243797	Clarification of Calibration and Ripple Test Procedures

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0006  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd, ETS-Lindgren

(Replaces R5-243311)

Decision: 		The document was agreed.

R5-243314	Clarification of polynomial interpolation

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0007  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd, Rohde & Schwarz

Discussion:

offline discussions with Telecom Italia and R&S

r1

Decision: 		The document was revised to R5-243798.

R5-243798	Clarification of polynomial interpolation

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0007  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd, Rohde & Schwarz

(Replaces R5-243314)

Decision: 		The document was agreed.

R5-243318	On Testing with Battery

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0008  Cat: F (Rel-17)

					Source: Keysight Technologies UK Ltd

Discussion:

Author requests to withdraw

Decision: 		The document was withdrawn.

R5-243407	Updates across TS 38.561 V17.0.0

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0009  Cat: F (Rel-17)

					Source: Apple Benelux B.V.

Abstract:

Editorial and format, reference corrections. To incorporate feedback from editHelp!

Discussion:

r1

Decision: 		The document was revised to R5-243799.

R5-243799	Updates across TS 38.561 V17.0.0

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0009  rev 1 Cat: F (Rel-17)

					Source: Apple Benelux B.V.

(Replaces R5-243407)

Decision: 		The document was agreed.

R5-243408	Editorial updates within TRP tests

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0010  Cat: F (Rel-17)

					Source: Apple Benelux B.V.

Abstract:

Editor's Notes and Test requirements sub-clauses needs clean-up

Decision: 		The document was agreed.

R5-243409	Editorial updates within TRS tests

Type: CR		For: Agreement
					38.561 v17.1.1	  CR-0011  Cat: F (Rel-17)

					Source: Apple Benelux B.V.

Abstract:

Editor's Notes and Test requirements sub-clauses needs clean-up

Decision: 		The document was agreed.

#### 5.4.13	TS 36.508

#### 5.4.14	TS 36.521-3

#### 5.4.15	TS 37.571-1

R5-242191	Update TC 14.2.3 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0473  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was revised to R5-243784.

R5-243784	Update TC 14.2.3 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0473  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242191)

Decision: 		The document was agreed.

R5-242192	Update TC 14.2.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0474  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243785.

R5-243785	Update TC 14.2.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0474  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242192)

Decision: 		The document was agreed.

R5-242196	Update TC 14.3.3 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0478  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243786.

R5-243786	Update TC 14.3.3 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0478  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242196)

Decision: 		The document was agreed.

R5-242197	Update TC 14.3.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0479  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243787.

R5-243787	Update TC 14.3.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0479  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242197)

Decision: 		The document was agreed.

R5-242200	Update TC 15.2.1 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0482  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243788.

R5-243788	Update TC 15.2.1 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0482  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242200)

Decision: 		The document was agreed.

R5-242201	Update TC 15.2.2 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0483  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242202	Update TC 15.2.3 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0484  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243789.

R5-243789	Update TC 15.2.3 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0484  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242202)

Decision: 		The document was agreed.

R5-242203	Update TC 15.2.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0485  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243790.

R5-243790	Update TC 15.2.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0485  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242203)

Decision: 		The document was agreed.

R5-242207	Update TC 15.3.1 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0489  Cat: F (Rel-17)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243791.

R5-243791	Update TC 15.3.1 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0489  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242207)

Decision: 		The document was agreed.

R5-242208	Update TC 15.3.2 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0490  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242211	Update TC 16.2.3 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0493  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242212	Update TC 16.2.4 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0494  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

R5-242215	Update TC 16.3.2 with TT analysis results

Type: CR		For: Agreement
					37.571-1 v17.4.0	  CR-0497  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

#### 5.4.16	TS 37.571-3

#### 5.4.17	TS 37.571-5

#### 5.4.18	TR 38.903 (NR MU & TT analyses)

R5-242219	Update TC 14.2.4 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0708  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243792.

R5-243792	Update TC 14.2.4 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0708  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242219)

Decision: 		The document was agreed.

R5-242222	Update TC 14.3.4 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0711  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243793.

R5-243793	Update TC 14.3.4 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0711  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242222)

Decision: 		The document was agreed.

R5-242224	Update TC 15.2.1 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0713  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242225	Update TC 15.2.2 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0714  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242226	Update TC 15.2.3 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0715  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243794.

R5-243794	Update TC 15.2.3 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0715  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242226)

Decision: 		The document was agreed.

R5-242227	Update TC 15.2.4 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0716  Cat: F (Rel-18)

					Source: CATT

Discussion:

r1

Decision: 		The document was revised to R5-243795.

R5-243795	Update TC 15.2.4 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0716  rev 1 Cat: F (Rel-18)

					Source: CATT

(Replaces R5-242227)

Decision: 		The document was agreed.

R5-242229	Update TC 15.3.1 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0718  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242230	Update TC 15.3.2 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0719  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242234	Update TC 16.2.4 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0723  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242235	Update TC 16.3.2 TT analysis results

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0724  Cat: F (Rel-18)

					Source: CATT

Decision: 		The document was agreed.

R5-242259	Pending UL MIMO update in 38.903

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0726  Cat: F (Rel-18)

					Source: Keysight Technologies

Decision: 		The document was agreed.

R5-242262	FR2c MU definition in 38.903

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0727  Cat: F (Rel-18)

					Source: Keysight Technologies

Discussion:

r1

Decision: 		The document was revised to R5-243730.

R5-243730	FR2c MU definition in 38.903

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0727  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies

(Replaces R5-242262)

Decision: 		The document was agreed.

R5-242730	Update of the QoQZ MU

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0737  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-242731	Removal of the reference to efficiency calibration in QoQZ

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0738  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-242906	TT analysis for RedCap RRM test case 16.2.1.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0739  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

Discussion:

+TEI17!

r4

Decision: 		The document was revised to R5-243747.

R5-243747	TT analysis for RedCap RRM test case 16.2.1.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0739  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242906)

Discussion:

"Revised from: R5-242906r4.

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244016.

R5-244016	TT analysis for RedCap RRM test case 16.2.1.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0739  rev 2 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243747)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242907	TT analysis for RedCap RRM test case 16.2.1.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0740  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RRM TT

R4-2409135 -> R4-2410226(Agreed)

Discussion:

"RRM TT, RAN4 dependent CR R4-2409135

5/21 Moderator (E///): Needs to go for email agreement due to RAN4 discussion

Document was sent FOR E-MAIL AGREEMENT

"

r2

Decision: 		The document was revised to R5-244017.

R5-244017	TT analysis for RedCap RRM test case 16.2.1.2

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0740  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242907)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242908	TT analysis for RedCap RRM test case 17.2.1.1

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0741  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

RAN4 dependency

RRM TT R4-2408543

Discussion:

"RAN4 dependency R4-2408545, R4-2409135

RRM TT

5/21 Moderator (E///): Needs to go for email agreement due to RAN4 discussion

Document was sent FOR E-MAIL AGREEMENT

"

Changes are not agreed in RAN4

email withdrawn

Decision: 		The document was withdrawn.

R5-243001	Update for FR2c MU

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0742  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

AP#99.21; Depending on the outcome of MU discussion R5-242997, R5-242998

Discussion:

r2

Decision: 		The document was revised to R5-243832.

R5-243832	Update for FR2c MU

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0742  rev 1 Cat: F (Rel-18)

					Source: Anritsu

(Replaces R5-243001)

Decision: 		The document was agreed.

R5-243003	QoQZ for QZ size 40 cm

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0743  Cat: F (Rel-18)

					Source: Anritsu

Abstract:

Depending on the outcome of MU discussion R5-243002

Discussion:

"Depending on the outcome of MU discussion R5-243002

5/22:

Moderator (AT&T): Depends on outcome of discussion papers."

Decision: 		The document was withdrawn.

R5-243101	Documentation of MU for n259

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0744  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Discussion in R5-243100, test case update in CR R5-243102

Discussion:

r1

Decision: 		The document was revised to R5-243829.

R5-243829	Documentation of MU for n259

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0744  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-243101)

Decision: 		The document was agreed.

R5-243104	Documentation of MU for UL MIMO

Type: CR		For: Agreement
					38.903 v18.2.0	  CR-0745  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Discussion in R5-243103, SA test case update in R5-243105, NSA test case update in CR R5-243106

Decision: 		The document was agreed.

#### 5.4.19	TR 38.905 (NR Test Points Radio Transmission and Reception )

R5-242537	Addition of checklist for intra band CA in Annex B

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0891  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243883.

R5-243883	Addition of checklist for intra band CA in Annex B

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0891  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242537)

Decision: 		The document was agreed.

R5-242939	Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC3

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0899  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

Test Case CR t-doc

R5-242948

Discussion:

reissued as R5-243653 because of title change

Decision: 		The document was withdrawn.

R5-243653	Addition of REFSENS TP analysis for PC3

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0916  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Abstract:

reissued from R5-242939 because of title change

Decision: 		The document was agreed.

R5-243131	Adding test point information of 6.5H.1.1

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0905  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243203	Updating the TP analysis for AMPR testing for CA_n41A-n79A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0911  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

TC in R5-243202

Decision: 		The document was revised to R5-243624.

R5-243624	Updating the TP analysis for AMPR testing for CA_n41A-n79A

Type: CR		For: Agreement
					38.905 v18.2.0	  CR-0911  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243203)

Decision: 		The document was agreed.

#### 5.4.20	Discussion Papers, Work Plan, TC lists

R5-242261	Discussion on MU and TT analysis for FR2c

Type: discussion		For: Endorsement
					38.521-2 v..
					Source: Keysight Technologies UK Ltd

Abstract:

CRs R5-242262, R5-242263

Discussion:

R&S and Anristu reviewed.

r1

Decision: 		The document was revised to R5-243724.

R5-243724	Discussion on MU and TT analysis for FR2c

Type: discussion		For: Endorsement
					38.521-2 v..
					Source: Keysight Technologies UK Ltd

(Replaces R5-242261)

Discussion:

"Revised from: R5-242261r1.

noted prop1-3 endorsed, prop4 noted"

Decision: 		The document was noted.

R5-242518	Discussion on introducing new SUL test case 7.7C

Type: discussion		For: Endorsement
					38.521-1 v..
					Source: Keysight Technologies UK Ltd

Abstract:

CR R5-242517

Discussion:

"Implemented in R5-242517

noted and proposal is endorsed"

Decision: 		The document was noted.

R5-242600	Discussion on the Informative note for High test channel bandwidth

Type: discussion		For: Endorsement
					38.508-1 v..
					Source: CAICT

Discussion:

r2

Decision: 		The document was revised to R5-243891.

R5-243891	Discussion on the Informative note for High test channel bandwidth

Type: discussion		For: Endorsement
					38.508-1 v..
					Source: CAICT

(Replaces R5-242600)

Decision: 		The document was noted.

R5-242628	FR2 RRM test cases: Known Issue List

Type: discussion		For: Information
					Source: Ericsson

Abstract:

Document for tracking FR2 RRM known issues

Discussion:

r2

Decision: 		The document was revised to R5-243731.

R5-243731	FR2 RRM test cases: Known Issue List

Type: discussion		For: Information
					Source: Ericsson

(Replaces R5-242628)

Decision: 		The document was noted.

R5-242729	On the QoQZ MU for n262

Type: discussion		For: Endorsement
					38.903 v..
					Source: ROHDE & SCHWARZ

Abstract:

Associated CR in R5-242730

Discussion:

"Associated CR in R5-242730

5/21

Moderator (AT&T): Offline discussions are occurring

5/22:

Moderator (AT&T): Further discussion is required on MTSU for FR2d.

Noted , prop1 endorsed"

Decision: 		The document was noted.

R5-242997	MU discussion on FR2c

Type: discussion		For: Endorsement
					Source: Anritsu

Abstract:

AP#99.21; Related CR: R5-242999, R5-243000, R5-243001

Discussion:

r1

Decision: 		The document was revised to R5-243725.

R5-243725	MU discussion on FR2c

Type: discussion		For: Endorsement
					Source: Anritsu

(Replaces R5-242997)

Discussion:

"Revised from: R5-242997r1.

Noted, Proposal 4 endorsed"

Decision: 		The document was noted.

R5-242998	On testability issue of FR2c OBW

Type: discussion		For: Endorsement
					Source: Anritsu

Abstract:

AP#99.21; Related CR: R5-242999, R5-243001

Discussion:

"AP#99.21; Related CR: R5-242999, R5-243001

5/8

Moderator (AT&T): Offline discussions are occurring

5/22:

Moderator (AT&T): This document can be noted."

Decision: 		The document was noted.

R5-243002	QoQZ for QZ size 40 cm

Type: discussion		For: Endorsement
					Source: Anritsu

Abstract:

Related CR: R5-243003

Discussion:

"Related CR: R5-243003

5/20

Moderator (AT&T): Offline discussions are occurring

5/22:

Moderator (AT&T): This document can be noted.

Proposal in 2729 is agreeable

raise an AP associated with the discussion"

Decision: 		The document was noted.

R5-243100	On the MU for n259

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

Abstract:

Update of TR 38.903 in CR R5-243101, test case update in CR R5-243102

Discussion:

"AP#99.21

Update of TR 38.903 in CR R5-243101, test case update in CR R5-243102

5/8

Moderator (AT&T): Offline discussions are occurring.

5/22:

Moderator (AT&T): This document can be noted."

Decision: 		The document was noted.

R5-243103	On the MU and TT for CA and MIMO test cases

Type: discussion		For: Endorsement
					Source: ROHDE & SCHWARZ

Abstract:

Update of TR 38.903 in CR R5-243104, SA test case update in R5-243105, NSA test case update in CR R5-243106, applicability update in CR R5-243107

Discussion:

"Update of TR 38.903 in CR R5-243104, SA test case update in R5-243105, NSA test case update in CR R5-243106, applicability update in CR R5-243107

5/21:

Moderator (AT&T): No comments. This document can be noted. Proposals 1 and 2 can be endorsed.

Proposals 1 and 2 endorsed"

Decision: 		The document was noted.

R5-243315	On NTC MTSU and TT for 40cm QZ and FR2d

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd

Discussion:

"5/21

Moderator (AT&T): Offline discussions are occurring

5/22:

Moderator (AT&T): Further discussion is required on MTSU for FR2d.

Proposal in 2729 is agreeable to KS"

Decision: 		The document was noted.

R5-243345	Discussion on PRACH FR2 power measurement

Type: discussion		For: Endorsement
					Source: Keysight Technologies UK Ltd

Discussion:

prop1 endorsed. Prop2 needs further ran5 work to make the next steps of r15 prach test testable

Decision: 		The document was noted.

### 5.5	Routine Maintenance for LTE only TEIx_Test

#### 5.5.1	LTE RF

##### 5.5.1.1	TS 36.508

R5-243113	Correction of sr-PUCCH-ResourceIndex

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1477  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

Author requests to withdraw

Decision: 		The document was withdrawn.

##### 5.5.1.2	TS 36.509

##### 5.5.1.3	TS 36.521-1

###### 5.5.1.3.1	Tx Requirements (Clause 6)

R5-242282	Correction of Additional spurious emission test requirement for NS_45

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5502  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

CoreSpecAlig

Discussion:

Bureau Veritas: pls. check the latest TS36.101 v8.29.0, which not include any NS_45.

With our understanding, it can’t be adapted from Rel-8 UE.

Therefore, TEI8_Test is not the correct Release to be used for this changes.

Please check core specification which release start to adapt this change, then the RAN5 release shall be aligned with same 36.101 changes release.

r1

Decision: 		The document was revised to R5-243807.

R5-243807	Correction of Additional spurious emission test requirement for NS_45

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5502  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-242282)

Decision: 		The document was agreed.

R5-242663	Update Test Configuration for NS17 Band 28 3MHz

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5508  Cat: F (Rel-18)

					Source: Rakuten Mobile, Inc

Discussion:

TEI18->15!

Decision: 		The document was agreed.

R5-242732	Addition of MOP requirements for 2UL CA 3A-42A

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5511  Cat: F (Rel-18)

					Source: NTT DOCOMO INC.

Decision: 		The document was agreed.

###### 5.5.1.3.2	Rx Requirements (Clause 7)

R5-242283	Editorial Correction of test configuration table of Reference sensitivity level for 4DL CA

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5503  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-243004	Correction to PCC DL allocation of CA_XA-YA-ZA in 7.6.3A.5

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5513  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was withdrawn.

R5-243005	Correction to test requirement of CA_2A-2A-66A-66A in 7.3A.9

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5514  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-243006	Correction to test requirement of CA_2A-2A-12A-66A-66A in 7.3A.10

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5515  Cat: F (Rel-18)

					Source: Anritsu

Decision: 		The document was agreed.

R5-243112	Correction of narrow band blocking for CA

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5516  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ, Anritsu

Discussion:

r1

Decision: 		The document was revised to R5-243808.

R5-243808	Correction of narrow band blocking for CA

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5516  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ, Anritsu

(Replaces R5-243112)

Decision: 		The document was agreed.

###### 5.5.1.3.3	Clauses 1-5, 8-10, Annexes

R5-242421	Editorial correction to the RMC of 8.12.1.1.2

Type: CR		For: Agreement
					36.521-1 v18.4.0	  CR-5507  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Abstract:

Editorial

Decision: 		The document was agreed.

##### 5.5.1.4	TS 36.521-2

R5-242499	Update to R18 IoT NTN test cases applicability

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1029  Cat: F (Rel-18)

					Source: CMCC, Keysight, ROHDE & SCHWARZ, Qualcomm

Abstract:

This CR is related to R5-242422 (CR#0020). TC 8.3.1.1.1 applicability can be added only after R5-242422 is agreed by RAN5.

Discussion:

r2

Decision: 		The document was revised to R5-243835.

R5-243835	Update to R18 IoT NTN test cases applicability

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1029  rev 1 Cat: F (Rel-18)

					Source: CMCC, Keysight, ROHDE & SCHWARZ, Qualcomm

(Replaces R5-242499)

Decision: 		The document was agreed.

R5-242728	Correct Table A.4.5-1 item 60 due to it doesnt match reference

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1030  Cat: F (Rel-18)

					Source: SGS Wireless

Discussion:

r1

Decision: 		The document was revised to R5-243911.

R5-243911	Correct Table A.4.5-1 item 60 due to it doesnt match reference

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1030  rev 1 Cat: F (Rel-18)

					Source: SGS Wireless

(Replaces R5-242728)

Decision: 		The document was agreed.

R5-242783	Removal of definition and abbreviation for IXIT and PIXIT

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1031  Cat: F (Rel-18)

					Source: Bureau Veritas ADT

Decision: 		The document was agreed.

R5-243330	Update Table A.4.5-1 with additonal NB-NTN capabilities

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1034  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

Author confirmed withdrawn. contents in their jumbo CR R5-242499(CMCC)

Decision: 		The document was withdrawn.

R5-243331	Update applicability for NB-NTN Demod TC 8.3.1.1.1

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1035  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

Author confirmed withdrawn. contents in their jumbo CR R5-242499(CMCC)

Decision: 		The document was withdrawn.

R5-243332	Update applicability for NB-NTN RRM test cases

Type: CR		For: Agreement
					36.521-2 v18.4.0	  CR-1036  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

Author confirmed withdrawn. contents in their jumbo CR R5-242499(CMCC)

Decision: 		The document was withdrawn.

##### 5.5.1.5	TS 36.521-3

R5-242120	Editorial correction to TC 6.2.16

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3022  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242416	Editorial correction to the wrong citation of rel-15 RRM cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3025  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242417	Editorial correction to the wrong message content of NTN RA cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3026  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242419	Addition of Test configuration for satellite access

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3027  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt., CMCC

Decision: 		The document was agreed.

R5-242420	Addition of Antenna configuration for satellite access

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3028  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt., CMCC

Decision: 		The document was agreed.

R5-242423	Update NB-IoT NTN timing advance test cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3029  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

CoreSpecAlign

Discussion:

r1

Decision: 		The document was revised to R5-243908.

R5-243908	Update NB-IoT NTN timing advance test cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3029  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-242423)

Discussion:

"Revised from: R5-242423r1.

RAN4 CR R4-2407204, revised to R4-2410242.

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244018.

R5-244018	Update NB-IoT NTN timing advance test cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3029  rev 2 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-243908)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242424	Update TT for NB-IoT NTN timing accuracy test cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3030  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

IoT NTN TT

Decision: 		The document was agreed.

R5-242425	Update NB-IoT NTN test case TC 13.4.1.1

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3031  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

RAN4 dependency R4-2407200 (endorsed) and merged into R4-2410242 (agreed).

Discussion:

"RAN4 dependency R4-2407200

pending RAN4

Document was sent FOR E-MAIL AGREEMENT

"

r1

Decision: 		The document was revised to R5-244019.

R5-244019	Update NB-IoT NTN test case TC 13.4.1.1

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3031  rev 1 Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

(Replaces R5-242425)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242430	Update groups of bands for satellite access

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3032  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Abstract:

core spec alignment

Discussion:

r1

Decision: 		The document was revised to R5-243888.

R5-243888	Update groups of bands for satellite access

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3032  rev 1 Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

(Replaces R5-242430)

Decision: 		The document was withdrawn.

R5-242431	Correction to setup for SIB31 and SIB31-NB

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3033  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Abstract:

core spec alignment

Discussion:

r1

Decision: 		The document was revised to R5-243889.

R5-243889	Correction to setup for SIB31 and SIB31-NB

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3033  rev 1 Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

(Replaces R5-242431)

Decision: 		The document was withdrawn.

R5-242577	Update to NTN DCQR cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3034  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Discussion:

r1

Decision: 		The document was revised to R5-243887.

R5-243887	Update to NTN DCQR cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3034  rev 1 Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

(Replaces R5-242577)

Decision: 		The document was agreed.

R5-242909	Correction to L2U redirection RRM test case 6.3.12

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3035  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242910	Correction to LTE V2V and V2X RRM test cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3036  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Abstract:

Editorial

Discussion:

+LTE!

r1

Decision: 		The document was revised to R5-243809.

R5-243809	Correction to LTE V2V and V2X RRM test cases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3036  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242910)

Decision: 		The document was agreed.

R5-243053	Editorial corrections for IoT NTN RRM TCs

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3037  Cat: F (Rel-18)

					Source: CMCC, MediaTek

Abstract:

Editorial

Decision: 		The document was agreed.

R5-243334	Update applicability for NB-NTN Channel quality reporting accuracy testcases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3042  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r1

WIC -> TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest

Decision: 		The document was revised to R5-243687.

R5-243687	Update applicability for NB-NTN Channel quality reporting accuracy testcases

Type: CR		For: Agreement
					36.521-3 v18.4.0	  CR-3042  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243334)

Decision: 		The document was agreed.

##### 5.5.1.6	TS 36.521-4

R5-242285	Editorial correction to test description of Clause 6.5A and 6.5B

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0018  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242418	Addition of Editors note for 8.2.1.1.1

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0019  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt., Qualcomm Inc.

Decision: 		The document was agreed.

R5-242422	Correction and update to the RMC and minimum test time of NB-IoT NTN demod cases

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0020  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Discussion:

r3

Decision: 		The document was revised to R5-243924.

R5-243924	Correction and update to the RMC and minimum test time of NB-IoT NTN demod cases

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0020  rev 1 Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

(Replaces R5-242422)

Decision: 		The document was agreed.

R5-242586	Deletion of the Editors note in 8.3.1.1.1

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0021  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt., CMCC, Rohde&Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243925.

R5-243925	Deletion of the Editors note in 8.3.1.1.1

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0021  rev 1 Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt., CMCC, Rohde&Schwarz

(Replaces R5-242586)

Decision: 		The document was withdrawn.

R5-242588	Update to the simulation method of minimun test time

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0022  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Decision: 		The document was agreed.

R5-242589	Update the minimum test time for 8.3.1.1.1

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0023  Cat: F (Rel-18)

					Source: Mediatek India Technology Pvt.

Decision: 		The document was withdrawn.

R5-243114	Correction of MOP for NB1 and NB2 test case

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0024  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-243115	Update of frequency error test cases

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0025  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-243116	Update of test case 8.3.1.1.1

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0026  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-243278	Editorial corrections to GSO frequency error test cases

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0027  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Abstract:

Editorial corrections to GSO frequency error test cases

Discussion:

r1

Decision: 		The document was revised to R5-243686.

R5-243686	Editorial corrections to GSO frequency error test cases

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0027  rev 1 Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

(Replaces R5-243278)

Decision: 		The document was withdrawn.

R5-243333	Update applicability for NB-NTN Demod TC 8.3.1.1.1

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0028  Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

Discussion:

r2

WIC -> TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest

Decision: 		The document was revised to R5-243905.

R5-243905	Update applicability for NB-NTN Demod TC 8.3.1.1.1

Type: CR		For: Agreement
					36.521-4 v18.1.0	  CR-0028  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Int

(Replaces R5-243333)

Decision: 		The document was agreed.

##### 5.5.1.7	RRM Test & Radio Reception Test Tolerances

###### 5.5.1.7.1	TR 36.903 (E-UTRAN RRM TT analyses)

R5-242426	Update TT analysis for NB-IoT NTN timing accuracy test cases

Type: CR		For: Agreement
					36.903 v18.2.0	  CR-0462  Cat: F (Rel-18)

					Source: MediaTek Beijing Inc.

Abstract:

IoT NTN TT

Decision: 		The document was agreed.

###### 5.5.1.7.2	TR 36.904 (E-UTRAN Radio Reception TT analyses)

###### 5.5.1.7.3	TR 36.905 (E-UTRAN Test Points Radio Transmission and Reception )

##### 5.5.1.8	TS 34.121-1

##### 5.5.1.9	TS 34.121-2

##### 5.5.1.10	TS 34.122

##### 5.5.1.11	TS 34.108

##### 5.5.1.12	TR 34.902 (UTRAN RRM Test Tolerance analyses)

##### 5.5.1.13	Discussion Papers, Work Plan, TC lists

R5-242427	Discussion on minimum test time simulation methods for NB-IoT NTN

Type: discussion		For: Endorsement
					Source: Mediatek India Technology Pvt.

Abstract:

CRs R5-242422, R5-242588,  R5-242586

Discussion:

"Implemented in R5-242422, R5-242588,  R5-242586

noted proposal is endorsed"

Decision: 		The document was noted.

R5-242428	Discussion on NTN DCQR cases

Type: discussion		For: Endorsement
					Source: Mediatek India Technology Pvt.

Abstract:

CR R5-242577

Discussion:

"Implemented in R5-242577

QC requests more time to evaluate

noted prop1 endorsed"

Decision: 		The document was noted.

R5-242692	Discussion on minimum test time simulation results for 8.3.1.1.1

Type: discussion		For: Approval
					Source: Mediatek India Technology Pvt.

Abstract:

CRs R5-242422, R5-242586

Discussion:

r1

Decision: 		The document was revised to R5-243890.

R5-243890	Discussion on minimum test time simulation results for 8.3.1.1.1

Type: discussion		For: Approval
					Source: Mediatek India Technology Pvt.

(Replaces R5-242692)

Decision: 		The document was noted.

R5-243429	Simulation results to derive min test time for nb-ntn demod test cases

Type: discussion		For: Agreement
					36.521-4 v..
					Source: Qualcomm Inc

Discussion:

"TC CR R5-243429

Author requested to updated WIC to ""TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest""

deferred pending min test time finalization accounting for k offset

Agenda Allocation was changed: [5.3.41.4]->[5.5.1.13]."

Decision: 		The document was noted.

### 5.6	Other Routine Maintenance TEIx_Test

R5-243700	LS on phase continuity requirements for DMRS bundling for NR NTN

Type: LS out		For: Approval
					to TSG WG RAN4
					Source: TSG WG RAN5

Discussion:

RAN5 group reverted endorsed decision of sending it as last minute objections have been presented during draft LS review.

Decision: 		The document was withdrawn.

#### 5.6.1	TS 34.108

#### 5.6.2	TS 34.121-1 All sections other than annexes

#### 5.6.3	TS 34.121-1 Annexes only

#### 5.6.4	TS 34.121-2

#### 5.6.5	TS 34.122

#### 5.6.6	TS 34.171

#### 5.6.7	TS 34.172

#### 5.6.8	TS 34.114

#### 5.6.9	TS 37.571-1

#### 5.6.10	TS 37.571-3

#### 5.6.11	TS 37.571-5

#### 5.6.12	TS 51.010-1 (RF/Performance)

#### 5.6.13	TS 51.010-2 (RF/Performance)

#### 5.6.14	TS 51.010-7 (RF/Performance)

#### 5.6.15	TS 37.544

R5-243310	Clarification of Calibration and Ripple Test Procedures

Type: CR		For: Agreement
					37.544 v16.3.0	  CR-0033  Cat: F (Rel-16)

					Source: Keysight Technologies UK Ltd, ETS-Lindgren

Decision: 		The document was agreed.

R5-243317	On Testing with Battery

Type: CR		For: Agreement
					37.544 v16.3.0	  CR-0034  Cat: F (Rel-16)

					Source: Keysight Technologies UK Ltd

Discussion:

Author requests withdrawn

Decision: 		The document was withdrawn.

#### 5.6.16	TR 37.901

#### 5.6.17	TR 37.901-5

#### 5.6.18	TR 38.918

#### 5.6.19	Discussion Papers, Work Plan, TC lists

### 5.7	Outgoing liaison statements for provisional approval

### 5.8	AOB

## 6	Signalling Protocol Functional Area

### 6.1	Review action points (fm A.I. 2.1)

### 6.2	Review incoming LS (fm A.I. 3) & new subject discussion papers

R5-242116	LS on development of FRMCS conformance test cases by ETSI TTF

Type: LS in		For: Information
					Original outgoing LS: RT(24)093032, to TSG RAN WG5, cc TSG RAN
					Source: TC RT

Abstract:

The ETSI Technical Committee for Rail Telecommunications (TC RT) has initiated a Testing Task Force (TTF) dedicated to developing conformance test cases for the Future Railway Mobile Communication System (FRMCS). This TTF's mission is to develop conformance test cases (in prose) that target client side and server side for railway-specific and public safety Mission Critical Services (MCS) functionalities specified by 3GPP, following the test methodologies employed by the 3GPP RAN working group 5 (RAN5). Upon completion of dedicated Milestones as specified in TTF ToR, TC RT intention would be to forward those MCS client and server-side conformance test cases to RAN5, to be proposed for incorporation into the relevant 3GPP RAN5 Technical Specifications for MCS conformance.

Deliverable D1, Mission Critical Services over 5G, has been deemed beyond the scope of this TTF. Instead, the work on Mission Critical Services over 5G will proceed on a voluntary basis directly within the RAN5 group.

The ability to demonstrate that MCS implementations conform to the standard is a key tenet in support of telecom interoperability and, in a railway context, in support of railway interoperability. Thorough conformance testing will increase the level of confidence that equipment from various suppliers is conformed to the standards and will interoperate. This is particularly relevant in a European context due to very frequent cross-border operations.

2. Actions:

ETSI TC RT kindly asks RAN WG5 to take the above information into account.

Discussion:

moved to SIG

output from the ETSI TF can be submitted following the normal RAN5 precess for review & agreement.

Decision: 		The document was noted.

R5-242117	LS on development of Server Side Conformance for Mission Critical Services by ETSI TTF

Type: LS in		For: Information
					Original outgoing LS: TCCE(24)000026r2, to TSG RAN WG5, cc TSG RAN, TSG CT, TSG SA
					Source: TCCE

Abstract:

The ETSI Technical Committee (TC) TCCE (TETRA and Critical Communications Evolution) has initiated a Testing Task Force (TTF) dedicated to developing conformance test cases for Mission Critical Services (MCS). This TTF's objective is to develop conformance test cases (in prose) that target server-side MCS functionalities specified by 3GPP, following the test methodologies employed by the 3GPP RAN working group 5 (RAN5). There are ongoing internal discussions within TCCA (The Critical Communications Association) and UIC (International Union of Railways) to formally ask to include server-side conformance tests into the 3GPP RAN5 terms of reference and we assume that RAN5 is the best place to address these. Upon completion of dedicated Milestones as specified in TTF ToR T028 (attached), TCCE’s intention would be to forward those MCS server-side conformance test cases to RAN5, to be proposed for incorporation into the relevant 3GPP RAN5 Technical Specifications for MCS conformance.

The ability to demonstrate that MCS application server conform to the 3GPP standards will be of critical importance to all potential users, especially in the public safety and railways sectors. Thorough conformance testing will increase the level of confidence that MCS server equipment from various suppliers is conformed to the standards and will interoperate. This in turn will reduce implementation and rollout times and costs. Not providing timely test specifications, would ultimately delay the deployment of MCS. Further consequences are a delay in deploying MCS equipment paired with a loss of confidence in MCS as there is a risk that early implementations of MCS are not standard conform or interoperable as expected and serious rework may be needed to solve that.

Server-side conformance plays a critical role in ensuring interoperability within multi-vendor MCS systems, as well as across interconnected MCS systems from various vendors and migration across systems in cross-border operations.

There is a high risk that proprietary implementations have to be or will be rolled-out in case no conformance tests are available.

Discussion:

moved to SIG

concluded with discussion paper R5-242587.

Decision: 		The document was noted.

R5-242290	Proposal for new test cases related to P-CSCF Restoration procedures

Type: discussion		For: Endorsement
					Source: Orange, Qualcomm

Abstract:

With 3G/2G sunset in some countries CS Fall Back won’t be an option anymore when issue happens in IMS core network during call establishment. A set of standardized procedures for restoration of IMS connectivity after node or link failure is defined in TS

Decision: 		The document was withdrawn.

R5-243234	Proposal for new test cases related to P-CSCF Restoration procedures

Type: discussion		For: Endorsement
					Source: Orange, Qualcomm

Abstract:

With 3G/2G sunset, in some countries CS Fall Back won’t be an option anymore. Failures may occur during call establishment when issue happens in IMS core network. VoIMS service must be resilient.

A set of standardized procedures for restoration of IMS connectivity after node or link failure are defined in TS 23.380 [1] and TS 24.229 [2]. These procedures are also included in IR92 [3] and NG114 [4] in P-CSCF Discovery section.

2	Discussion

The discussion is about adding the following test cases in RAN5 specifications to cover IMS/P-CSCF restoration procedures bellow:

Proposal 2-1: Verify UE behaviour when P-CSCF addresses are included in PDU-Session Modification Command when UE is connected to 5GC

As described in section U.2.2.1C of TS 24.229 [2] if the UE receives one or more P-CSCF address(es) in the extended Protocol Configuration Options information element of a PDU SESSION MODIFICATION COMMAND message and the one or more P-CSCF addressee(s) do not include the address of the currently used P-CSCF, then the UE shall acquire a different P-CSCF address from the one or more P-CSCF addresse(s) in the PDU SESSION MODIFICATION COMMAND message. If more than one P-CSCF address with the same container identifier (i.e. "P-CSCF IPv6 Address" or "P-CSCF IPv4 Address") are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority

Proposal 2-2: Verify UE behavior when P-CSCF address are included in Modify EPS Bearer Context Request when UE is connected to EPS

As described in section L.2.2.1C of TS 24.229 [2] if the UE used method II for P-CSCF discovery and if the UE receives one or more P-CSCF address(es) in the Protocol Configuration Options information element of a Modify EPS Bearer Context Request message the one or more P-CSCF addresse(s) do not include the address of the currently used P-CSCF, then the UE shall acquire a different P-CSCF address from the one or more P-CSCF addresse(s) in the Modify EPS Bearer Context Request message. If more than one P-CSCF address with the same container identifier (i.e. “P-CSCF IPv6 Address” or “P-CSCF Ipv4 Address”) are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

Proposal 2-3: Verify UE behaviour when P-CSCSF address are included in IKEv2 INFORMATIONAL exchange when UE is connected to Non 3GPP Access

As described in section 5.6.5.2 of TS 23.380 [1] if the UE has sent its capability the indication of the support of P-CSCF restoration to the ePDG by including the P-CSCF_RESELECTION_SUPPORT Notify payload within an IKE_AUTH request message (as defined in TS 24.302 [5]) and if the UE receives an IKEv2 informational exchange message containing a list of P-CSCF addresses, the UE shall first send a response to the ePDG, and then the UE must acquire a different P-CSCF from the currently used P-CSCF, and initiate a new initial SIP registration

Proposal 2-4:

Verify UE behaviour for P-CSCF discovery after PDU Session Release command 5GSM Cause #39 Reactivation requested when UE is connected to 5GC

Verify UE behaviour for P-CSCF discovery after EPS Bearer Deactivation ESM Cause #39 Reactivation requested when UE is connected to MME

Verify UE behaviour for P-CSCF discovery after DELETE payload REACTIVATION_REQUESTED_CAUSE when UE is connected to Non 3GPP Access

As defined in TS 23.380 [1], upon receipt of a request to release an IMS PDN/PDU connection with the cause "reactivation requested" over the 3GPP or the WLAN access, the UE shall locally release the IMS PDN/PDU connection. As a result of the release of the IMS PDN/PDU connection, the UE shall re-establish the IMS PDN/PDU connection, and also perform a new P-CSCF discovery (as the IMS PDN connection was lost). Then the UE shall perform a new initial IMS registration after the new P-CSCF discovery.

Discussion:

proposals to add test cases are accepted, to go throug notmal approval process.

Decision: 		The document was noted.

### 6.3	Open Work Items

#### 6.3.1	REL-16 NR CA and DC; and NR and LTE DC Configurations (UID-830083)  NR_CADC_NR_LTE_DC_R16-UEConTest

##### 6.3.1.1	TS 38.508-1

##### 6.3.1.2	TS 38.508-2

##### 6.3.1.3	TS 38.523-1

##### 6.3.1.4	TS 38.523-2

##### 6.3.1.5	TS 38.523-3

##### 6.3.1.6	Discussion Papers, Work Plan, TC lists

#### 6.3.2	5G V2X with NR sidelink (UID-880069)  5G_V2X_NRSL_eV2XARC-UEConTest

##### 6.3.2.1	TS 38.508-1

##### 6.3.2.2	TS 38.508-2

##### 6.3.2.3	TS 38.509

##### 6.3.2.4	TS 38.523-1

R5-242314	Updates to 5G V2X test case 12.1.7.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4354  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242315	Updates to 5G V2X test case 12.2.9.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4355  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242582	Editorial correction for V2X TC 12.1.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4381  Cat: F (Rel-17)

					Source: ZTE Corporation

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242584	Correction for V2X TC 12.1.2.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4382  Cat: F (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-242585	Correction for V2X TC 12.1.3.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4383  Cat: F (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-242911	Addition of NR sidelink SIG test case 12.1.1.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4425  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r4

Decision: 		The document was revised to R5-243534.

R5-243534	Addition of NR sidelink SIG test case 12.1.1.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4425  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242911)

Decision: 		The document was agreed.

R5-242912	Correction to NR sidelink SIG test case 12.2.5.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4426  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242913	Correction to NR sidelink SIG test case 12.2.5.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4427  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r1

offline comments from TF160.

Deferred.

r2

Decision: 		The document was revised to R5-243535.

R5-243535	Correction to NR sidelink SIG test case 12.2.5.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4427  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242913)

Decision: 		The document was agreed.

##### 6.3.2.5	TS 38.523-2

R5-242914	Addition of applicability for NR sidelink SIG test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0474  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243444	Editorial correction to applicability of TC 12.1.2.1

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0482  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

late Tdoc

Decision: 		The document was agreed.

##### 6.3.2.6	TS 38.523-3

##### 6.3.2.7	TS 36.509

##### 6.3.2.8	TS 37.571-4

##### 6.3.2.9	Discussion Papers, Work Plan, TC lists

#### 6.3.3	Rel-17 NR CA and DC; and NR and LTE DC Configurations (UID-900056) NR_CADC_NR_LTE_DC_R17-UEConTest

##### 6.3.3.1	TS 38.508-1

##### 6.3.3.2	TS 38.508-2

##### 6.3.3.3	TS 38.523-1

##### 6.3.3.4	TS 38.523-2

##### 6.3.3.5	TS 38.523-3

##### 6.3.3.6	Discussion Papers, Work Plan, TC lists

#### 6.3.4	NR-based access to unlicensed spectrum (UID-911003) NR_unlic-UEConTest

##### 6.3.4.1	TS 38.508-1

R5-242771	Update of Logical Channel Config Table 4.6.3-66

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3174  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-243536.

R5-243536	Update of Logical Channel Config Table 4.6.3-66

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3174  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated

(Replaces R5-242771)

Decision: 		The document was agreed.

R5-242772	Update channel access configurations for shared spectrum

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3175  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-243481	Addition of DCI information for semi static channel access in shared spectrum

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3212  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

late doc

Decision: 		The document was agreed.

##### 6.3.4.2	TS 38.508-2

##### 6.3.4.3	TS 38.509

##### 6.3.4.4	TS 38.523-1

R5-243118	Correction to NR CA shared spectrum test case 7.1.1.10.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4436  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-243119	Correction to NR shared spectrum test case 8.1.5.6.6.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4437  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-243120	Correction to NR shared spectrum test case 8.2.5.7.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4438  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-243122	Correction to NR shared spectrum test case 8.2.5.7.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4440  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.4.5	TS 38.523-2

R5-243185	Applicability updates to NR shared spectrum test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0481  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.4.6	TS 38.523-3

##### 6.3.4.7	Discussion Papers, Work Plan, TC lists

#### 6.3.5	NR small data transmissions in INACTIVE state (UID-960072) NR_SmallData_INACTIVE-UEConTest

##### 6.3.5.1	TS 38.508-1

R5-243482	Common configuration change for SDT condition

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3213  Cat: F (Rel-18)

					Source: Nokia, Huawei

Discussion:

late doc

Decision: 		The document was agreed.

##### 6.3.5.2	TS 38.508-2

##### 6.3.5.3	TS 38.523-1

R5-242316	Corrections to SDT TC 7.1.1.13.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4356  Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

Decision: 		The document was agreed.

R5-242317	Corrections to SDT TC 7.1.1.13.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4357  Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

Decision: 		The document was agreed.

R5-242318	Corrections to SDT TC 7.1.1.13.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4358  Cat: F (Rel-17)

					Source: MCC TF160, Lenovo

Decision: 		The document was agreed.

R5-243084	Correction of SDT TC 7.1.1.13.5-cg-SDT-TATimer

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4433  Cat: F (Rel-17)

					Source: Huawei, HiSilicon, MCC TF160

Discussion:

r1

Decision: 		The document was revised to R5-243537.

R5-243537	Correction of SDT TC 7.1.1.13.5-cg-SDT-TATimer

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4433  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon, MCC TF160

(Replaces R5-243084)

Decision: 		The document was agreed.

R5-243085	Correction of SDT TC 8.1.5.13.1-cg-SDT

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4434  Cat: F (Rel-17)

					Source: Huawei, HiSilicon, MCC TF160

Discussion:

r1

Decision: 		The document was revised to R5-243538.

R5-243538	Correction of SDT TC 8.1.5.13.1-cg-SDT

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4434  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon, MCC TF160

(Replaces R5-243085)

Decision: 		The document was agreed.

R5-243170	Modification of testcase 8.1.5.13.3 for SDT-SRB2-Indication

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4448  Cat: F (Rel-17)

					Source: Nokia

Discussion:

r1

Decision: 		The document was revised to R5-243539.

R5-243539	Modification of testcase 8.1.5.13.3 for SDT-SRB2-Indication

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4448  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-243170)

Decision: 		The document was agreed.

R5-243483	Configuration fix for testcase 8.1.5.13.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4463  Cat: F (Rel-17)

					Source: Nokia

Discussion:

late doc

Decision: 		The document was agreed.

##### 6.3.5.4	TS 38.523-2

R5-243171	Modification of testcase 8.1.5.13.3 applicability clauses

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0479  Cat: F (Rel-17)

					Source: Nokia

Decision: 		The document was agreed.

##### 6.3.5.5	TS 38.523-3

R5-242319	NR SDT: Addition of NR/5GC Test Model aspects

Type: CR		For: Agreement
					38.523-3 v17.10.0	  CR-3433  Cat: F (Rel-17)

					Source: MCC TF160

Discussion:

r1

Decision: 		The document was revised to R5-243579.

R5-243579	NR SDT: Addition of NR/5GC Test Model aspects

Type: CR		For: Agreement
					38.523-3 v17.10.0	  CR-3433  rev 1 Cat: F (Rel-17)

					Source: MCC TF160

(Replaces R5-242319)

Decision: 		The document was agreed.

##### 6.3.5.6	Discussion Papers, Work Plan, TC lists

#### 6.3.6	Solutions for NR to support non-terrestrial networks (NTN) (UID-960074) NR_NTN_solutions_plus_CT-UEConTest

##### 6.3.6.1	TS 38.508-1

R5-242132	Update of NTN-config for NGSO

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3107  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

covered in R5-242546

Decision: 		The document was withdrawn.

R5-242535	Addition of signalling test frequencies for CA_n48C

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3141  Cat: F (Rel-18)

					Source: Ericsson, Verizon, MCC TF160

Abstract:

Related to Signaling test

Discussion:

r3

Decision: 		The document was revised to R5-243532.

R5-243532	Addition of signalling test frequencies for CA_n48C

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3141  rev 1 Cat: F (Rel-18)

					Source: Ericsson, Verizon, MCC TF160

(Replaces R5-242535)

Discussion:

reissued as R5-243533 because of title expansion

Decision: 		The document was withdrawn.

R5-243533	Addition of signalling test frequencies for CA_n48C and CA_n5B

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3215  Cat: F (Rel-18)

					Source: Ericsson, Verizon, MCC TF160

Abstract:

reissued from R5-243532 because of title expansion

Decision: 		The document was agreed.

R5-242543	Added Common test environment for NR-NTN test cases

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3142  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Discussion:

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

r2

Decision: 		The document was revised to R5-243554.

R5-243554	Added Common test environment for NR-NTN test cases

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3142  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

(Replaces R5-242543)

Decision: 		The document was agreed.

R5-242546	Updates to SIB19 and ephemeris information for NGSO and GSO condition

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3143  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland, MediaTek Inc.

Discussion:

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

r1

TF160: CR not based on latest spec

r2

Decision: 		The document was revised to R5-243597.

R5-243597	Updates to SIB19 and ephemeris information for NGSO and GSO condition

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3143  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland, MediaTek Inc.

(Replaces R5-242546)

Decision: 		The document was agreed.

R5-242760	Addition of system information combination for inter frequency NTN scenario

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3171  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

r1

The meeting authorizes MCC and the rapporteur to mirror Table 4.4.3.1.2-1 so that it fits on the page.

Decision: 		The document was revised to R5-243555.

R5-243555	Addition of system information combination for inter frequency NTN scenario

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3171  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated

(Replaces R5-242760)

Decision: 		The document was agreed.

##### 6.3.6.2	TS 38.508-2

R5-242133	Correction to the note of NR NTN PICS

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0608  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242759	Addition of NR NTN PICS

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0631  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was withdrawn.

R5-242765	Addition of NR NTN PICS

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0632  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.6.3	TS 38.509

##### 6.3.6.4	TS 38.523-1

R5-242134	Correction to NR NTN TC 8.1.3.1.26

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4310  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242135	Correction to NR NTN TC 9.4.1.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4311  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243556.

R5-243556	Correction to NR NTN TC 9.4.1.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4311  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242135)

Decision: 		The document was agreed.

R5-242136	Correction to NR NTN TC 9.4.1.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4312  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243557.

R5-243557	Correction to NR NTN TC 9.4.1.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4312  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242136)

Decision: 		The document was agreed.

R5-242542	Introduction of new TC NR-NTN - TC TAI(s) not allowed / forbidden TAI(s)

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4372  Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

Discussion:

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

r1

Decision: 		The document was revised to R5-243558.

R5-243558	Introduction of new TC NR-NTN - TC TAI(s) not allowed / forbidden TAI(s)

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4372  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

(Replaces R5-242542)

Decision: 		The document was agreed.

R5-242547	Introduction of new TC NR-NTN - ntn-UlSyncValidityDuration, T430 Expiry

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4373  Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

Discussion:

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

r1

Decision: 		The document was revised to R5-243559.

R5-243559	Introduction of new TC NR-NTN - ntn-UlSyncValidityDuration, T430 Expiry

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4373  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

(Replaces R5-242547)

Decision: 		The document was agreed.

R5-242548	Updates to NTN test cases with missing NGSO configuration

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4374  Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

Discussion:

38.508-1??

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

r1

spec still wrong on cover

Decision: 		The document was revised to R5-243560.

R5-243560	Updates to NTN test cases with missing NGSO configuration

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4374  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

(Replaces R5-242548)

Decision: 		The document was agreed.

R5-242549	Addition of new NR-NTN conditional handover condEventA4 test case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4375  Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

Discussion:

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

r1

Decision: 		The document was revised to R5-243598.

R5-243598	Addition of new NR-NTN conditional handover condEventA4 test case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4375  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

(Replaces R5-242549)

Decision: 		The document was agreed.

R5-242550	Addition of new NR NTN conditional handover CondEventD1 test case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4376  Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

Discussion:

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

r1

Decision: 		The document was revised to R5-243561.

R5-243561	Addition of new NR NTN conditional handover CondEventD1 test case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4376  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

(Replaces R5-242550)

Decision: 		The document was agreed.

R5-242551	Addition of new NR NTN conditional handover CondEventT1 test case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4377  Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

Discussion:

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

r1

Decision: 		The document was revised to R5-243599.

R5-243599	Addition of new NR NTN conditional handover CondEventT1 test case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4377  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

(Replaces R5-242551)

Decision: 		The document was agreed.

R5-242757	Correction to NR NTN TA report test case 7.1.1.10.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4402  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242767	Addition of NR NTN cell reselection test case 6.1.2.18a

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4405  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-243562.

R5-243562	Addition of NR NTN cell reselection test case 6.1.2.18a

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4405  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242767)

Decision: 		The document was agreed.

R5-242768	Addition of NR NTN cell reselection test case 6.1.2.18b

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4406  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was revised to R5-243563.

R5-243563	Addition of NR NTN cell reselection test case 6.1.2.18b

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4406  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242768)

Decision: 		The document was agreed.

R5-242769	Addition of NR NTN MAC test case 7.1.1.5.6

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4407  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242770	Addition of NR NTN RLC t-reassembly timer test case 7.1.2.3.9a

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4408  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-243564.

R5-243564	Addition of NR NTN RLC t-reassembly timer test case 7.1.2.3.9a

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4408  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242770)

Decision: 		The document was agreed.

##### 6.3.6.5	TS 38.523-2

R5-242137	Correction to the applicability condition of NR NTN

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0454  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243565.

R5-243565	Correction to the applicability condition of NR NTN

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0454  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242137)

Decision: 		The document was agreed.

R5-242552	Addition of applicability for new NR NTN test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0462  Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

Discussion:

3GU allocation: NR_NTN_solutions_plus_CT-UEConTest / AI allocation: NR_CADC_NR_LTE_DC_R16-UEConTest

Decision: 		The document was withdrawn.

R5-242766	Applicability updates to NR NTN test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0470  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was revised to R5-243950.

R5-243950	Applicability updates to NR NTN test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0470  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242766)

Decision: 		The document was agreed.

##### 6.3.6.6	TS 38.523-3

R5-242320	NR NTN: Addition of GSO Test Model

Type: CR		For: Agreement
					38.523-3 v17.10.0	  CR-3434  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

##### 6.3.6.7	Discussion Papers, Work Plan, TC lists

R5-242559	Discussion paper – Way forward for R-17 legacy testing for NR-NTN only devices

Type: discussion		For: Discussion
					38.523-1 v..
					Source: Qualcomm Incorporated

Abstract:

3GPP has implemented several NR features in standalone mode or namely ‘vertical-only’ mode. Which enables the industry to design and deploy application specific devices. TS 38.508-1[2] mentions such vertically capable UE, such as SNPN, RedCap and TS 36.508[3] mentions NB-NTN only UE.

For this vertically capable UE, the legacy test coverage was achieved with appropriate modification in the test environment and configurations. Following the same precedence, expectation is to do the same for R-17 NR-NTN devices.

Discussion:

However, according to RP-232732[2], the planned deployment for NR-NTN is 2026 or later. Considering the evolution of 3GPP specification, a later release (R-18, R-19) will be available to implement by the industry. Hence, it is not worthwhile to scope out legacy testing for NR-NTN only R-17 devices, that is required for commercialization, as there might be no commercialization for R-17.

Proposal:

Proposal 1: R-17 will not have legacy test cases applicable to NR-NTN only devices.

Proposal 2: Revisit the topic of legacy testing, depending on the market evolution, in a later release WI for NR-NTN evolution (R-18, R-19 etc).

Discussion:

RAN5 Chair: there will be only NR NTN capable UEs,  not NR NTN UEs. No Rel-17 deployment.

Noted.

The meeting concluded review and udate of legacy test cases will be considered anly when NTN UE will be available.

Decision: 		The document was noted.

#### 6.3.7	Enhancement of Private Network Support for NG-RAN including CT aspects (UID-960076) NG_RAN_PRN_enh_plus_CT-UEConTest

##### 6.3.7.1	TS 38.508-1

##### 6.3.7.2	TS 38.508-2

##### 6.3.7.3	TS 38.523-1

R5-242675	Update UE pre-test configurations for test case 6.5.3.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4388  Cat: F (Rel-17)

					Source: China Telecom

Decision: 		The document was agreed.

R5-242676	Update test case 6.5.3.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4389  Cat: F (Rel-17)

					Source: China Telecom

Decision: 		The document was agreed.

R5-242677	Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4390  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-243540.

R5-243540	Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4390  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-242677)

Decision: 		The document was agreed.

R5-242678	Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 Enpn

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4391  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-243541.

R5-243541	Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 Enpn

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4391  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-242678)

Decision: 		The document was agreed.

R5-242679	Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 Enpn

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4392  Cat: F (Rel-17)

					Source: China Telecom

Decision: 		The document was agreed.

R5-242680	Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 Enpn

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4393  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

r2

Decision: 		The document was revised to R5-243951.

R5-243951	Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 Enpn

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4393  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-242680)

Decision: 		The document was agreed.

R5-242681	Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4394  Cat: F (Rel-17)

					Source: China Telecom

Decision: 		The document was agreed.

R5-242682	Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4395  Cat: F (Rel-17)

					Source: China Telecom

Decision: 		The document was revised to R5-243542.

R5-243542	Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4395  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-242682)

Decision: 		The document was agreed.

R5-242683	Addition of new test case 9.1.11.7 for Rel-17 eNPN

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4396  Cat: F (Rel-17)

					Source: China Telecom

Decision: 		The document was agreed.

##### 6.3.7.4	TS 38.523-2

R5-242138	Addition of TC 9.1.11.5 into Table 4.3.1-1

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0455  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

covered in R5-242684.

Decision: 		The document was withdrawn.

R5-242684	Scoping NR SA applicable TCs for SNPN-only Ues

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0467  Cat: F (Rel-17)

					Source: China Telecom, MediaTek

Discussion:

3GU 17.6.0

wrong location & CR ver

Decision: 		The document was revised to R5-243962.

R5-243962	Scoping NR SA applicable TCs for SNPN-only Ues

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0467  rev 1 Cat: F (Rel-17)

					Source: China Telecom, MediaTek

(Replaces R5-242684)

Decision: 		The document was agreed.

R5-242704	Addition of applicability for eNPN test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0468  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

wrong location & CR ver

Decision: 		The document was revised to R5-243963.

R5-243963	Addition of applicability for eNPN test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0468  rev 1 Cat: F (Rel-17)

					Source: China Telecom

(Replaces R5-242704)

Decision: 		The document was agreed.

##### 6.3.7.5	TS 38.523-3

##### 6.3.7.6	Discussion Papers, Work Plan, TC lists

#### 6.3.8	Further enhancements on MIMO for NR (UID-960079) NR_feMIMO-UEConTest

##### 6.3.8.1	TS 38.508-1

##### 6.3.8.2	TS 38.508-2

R5-243157	Addition of PICS for FeMIMO

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0643  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

pc_ '

Decision: 		The document was revised to R5-243543.

R5-243543	Addition of PICS for FeMIMO

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0643  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243157)

Decision: 		The document was agreed.

##### 6.3.8.3	TS 38.523-1

R5-243155	Adding 7.1.1.3.17.1 multi-TRP PUSCH repetition type A

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4443  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243156	Adding 7.1.1.3.17.2 multi-TRP PUSCH repetition type B

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4444  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 6.3.8.4	TS 38.523-2

R5-243158	Adding test applicability for multi-TRP PUSCH repetition test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0477  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

RAN5 Chair: pls. add the other specs affected.

TF160 manager: pls. use latest spec!

Decision: 		The document was revised to R5-243544.

R5-243544	Adding test applicability for multi-TRP PUSCH repetition test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0477  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-243158)

Decision: 		The document was agreed.

##### 6.3.8.5	TS 38.523-3

##### 6.3.8.6	Discussion Papers, Work Plan, TC lists

#### 6.3.9	NR Sidelink Relay (UID-960083) NR_SL_relay-UEConTest

##### 6.3.9.1	TS 38.508-1

R5-242574	Update for derivation path of test procedure 4.9.39 and 4.9.40

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3146  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was agreed.

R5-242575	Correction for test procedure 4.9.41 and 4.9.42

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3147  Cat: F (Rel-18)

					Source: ZTE Corporation

Discussion:

r1

overlap with R5-242801.

-> removed updates of 4.9.42 to resolve the overlap.

r2

Decision: 		The document was revised to R5-243545.

R5-243545	Correction for test procedure 4.9.41 and 4.9.42

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3147  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-242575)

Discussion:

reissued as R5-243546 because of title change

Decision: 		The document was withdrawn.

R5-243546	Correction for test procedure 4.9.41

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3216  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

reissued from R5-243545 because of title change

Decision: 		The document was agreed.

R5-242799	Addition of new combination of system information blocks for 5G Prose communication

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3178  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

R5-242800	Editorial update of SIB12 in 4.6.2

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3179  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

R5-242801	Update of Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3180  Cat: F (Rel-18)

					Source: TDIA, CATT

Discussion:

r1

Decision: 		The document was revised to R5-243547.

R5-243547	Update of Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3180  rev 1 Cat: F (Rel-18)

					Source: TDIA, CATT

(Replaces R5-242801)

Decision: 		The document was agreed.

R5-242802	Update of Table 4.6.1-22 RRCSetupComplete

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3181  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

R5-242803	Update of Table 4.6.1A-2A RemoteUEInformationSidelink

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3182  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

R5-242804	Update of Table 4.6.1A-8 uuMessageTransferSidelink

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3183  Cat: F (Rel-18)

					Source: TDIA, CATT

Discussion:

r1

Decision: 		The document was revised to R5-243548.

R5-243548	Update of Table 4.6.1A-8 uuMessageTransferSidelink

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3183  rev 1 Cat: F (Rel-18)

					Source: TDIA, CATT

(Replaces R5-242804)

Decision: 		The document was agreed.

R5-242805	Addition of 5G ProSe discovery and security messages

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3184  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

R5-242806	Update of 5G ProSe direct discovery messages

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3185  Cat: F (Rel-18)

					Source: TDIA,CATT

Decision: 		The document was agreed.

R5-242807	Addition of Default HTTP messages for communication with the 5G ProSe

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3186  Cat: F (Rel-18)

					Source: TDIA, CATT

Discussion:

r1

[x4]

Decision: 		The document was revised to R5-243549.

R5-243549	Addition of Default HTTP messages for communication with the 5G ProSe

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3186  rev 1 Cat: F (Rel-18)

					Source: TDIA, CATT

(Replaces R5-242807)

Decision: 		The document was agreed.

R5-242808	Addition of Default TLS message and information element contents

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3187  Cat: F (Rel-18)

					Source: TDIA, CATT

Discussion:

r1

[x4]

Decision: 		The document was revised to R5-243550.

R5-243550	Addition of Default TLS message and information element contents

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3187  rev 1 Cat: F (Rel-18)

					Source: TDIA, CATT

(Replaces R5-242808)

Decision: 		The document was agreed.

R5-242809	Addition of spec reference of 3GPP TS 33.310, 33.503 and IETF RFC 5246 for 5G ProSe information elements

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3188  Cat: F (Rel-18)

					Source: TDIA, CATT

Discussion:

r1

Decision: 		The document was revised to R5-243551.

R5-243551	Addition of spec reference of 3GPP TS 33.310, 33.503 and IETF RFC 5246 for 5G ProSe information elements

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3188  rev 1 Cat: F (Rel-18)

					Source: TDIA, CATT

(Replaces R5-242809)

Decision: 		The document was agreed.

R5-242810	Update of Default settings of UICC and USIM for 5G ProSe

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3189  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

R5-242811	Update of Test procedure for 5G ProSe U2N Relay Discovery

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3190  Cat: F (Rel-18)

					Source: TDIA, CATT

Decision: 		The document was agreed.

##### 6.3.9.2	TS 38.508-2

##### 6.3.9.3	TS 38.523-1

##### 6.3.9.4	TS 38.523-2

##### 6.3.9.5	TS 38.523-3

##### 6.3.9.6	Discussion Papers, Work Plan, TC lists

#### 6.3.10	NR Sidelink enhancement (UID-960084) NR_SL_enh-UEConTest

##### 6.3.10.1	TS 38.508-1

##### 6.3.10.2	TS 38.508-2

##### 6.3.10.3	TS 38.523-1

##### 6.3.10.4	TS 38.523-2

##### 6.3.10.5	TS 38.523-3

##### 6.3.10.6	Discussion Papers, Work Plan, TC lists

#### 6.3.11	NR QoE management and optimizations for diverse services (UID-970072) NR_QoE-UEConTest

##### 6.3.11.1	TS 38.508-1

##### 6.3.11.2	TS 38.508-2

##### 6.3.11.3	TS 38.523-1

##### 6.3.11.4	TS 38.523-2

##### 6.3.11.5	TS 38.523-3

R5-242310	NR QoE: Addition of NR/5GC Test Model aspects

Type: CR		For: Agreement
					38.523-3 v17.10.0	  CR-3432  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

##### 6.3.11.6	Discussion Papers, Work Plan, TC lists

#### 6.3.12	NR Positioning Enhancements (UID-970075) NR_pos_enh-UEConTest

##### 6.3.12.1	TS 38.508-1

##### 6.3.12.2	TS 37.571-2

##### 6.3.12.3	TS 37.571-3

##### 6.3.12.4	TS 37.571-4

##### 6.3.12.5	TS 37.571-5

R5-242484	Change the GNSS-AcquisitionAssistance IE description for BDS signals

Type: CR		For: Agreement
					37.571-5 v17.4.0	  CR-0231  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was agreed.

##### 6.3.12.6	Discussion Papers, Work Plan, TC lists

#### 6.3.13	Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData) (UID – 970077) MCProtoc16_enh2MCPTT_eMCData2-ConTest

##### 6.3.13.1	TS 36.579-1

##### 6.3.13.2	TS 36.579-2

R5-243274	Correction to MCPTT TC 5.10

Type: CR		For: Agreement
					36.579-2 v16.4.0	  CR-0371  Cat: F (Rel-16)

					Source: NIST

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.3.13.3	TS 36.579-3

##### 6.3.13.4	TS 36.579-4

##### 6.3.13.5	TS 36.579-5

##### 6.3.13.6	TS 36.579-6

R5-242311	Correction of testcase 6.1.1.14

Type: CR		For: Agreement
					36.579-6 v16.4.0	  CR-0106  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242312	Correction of testcase 6.9.1

Type: CR		For: Agreement
					36.579-6 v16.4.0	  CR-0107  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-243271	Correction of MCVideo Location TCs

Type: CR		For: Agreement
					36.579-6 v16.4.0	  CR-0111  Cat: F (Rel-16)

					Source: NIST

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.3.13.7	TS 36.579-7

R5-243267	Addition of new TC 6.5.1 Location Information Request

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0059  Cat: F (Rel-16)

					Source: NIST

Discussion:

r1

Decision: 		The document was revised to R5-243570.

R5-243570	Addition of new TC 6.5.1 Location Information Request

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0059  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-243267)

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-243268	Addition of new TC 6.5.2 Location Report triggering

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0060  Cat: F (Rel-16)

					Source: NIST

Discussion:

r1

Decision: 		The document was revised to R5-243571.

R5-243571	Addition of new TC 6.5.2 Location Report triggering

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0060  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-243268)

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-243269	Addition of new TC 5.8 Rules based affiliation status change

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0061  Cat: F (Rel-16)

					Source: NIST

Discussion:

r1

Decision: 		The document was revised to R5-243572.

R5-243572	Addition of new TC 5.8 Rules based affiliation status change

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0061  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-243269)

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-243270	Addition of new TC 5.9 Location based functional alias status change

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0062  Cat: F (Rel-16)

					Source: NIST

Discussion:

r1

Decision: 		The document was revised to R5-243573.

R5-243573	Addition of new TC 5.9 Location based functional alias status change

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0062  rev 1 Cat: F (Rel-16)

					Source: NIST

(Replaces R5-243270)

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-243272	Correction of MCData TC 6.4.2 Emergency Alert

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0063  Cat: F (Rel-16)

					Source: NIST

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.3.13.8	TS 36.579-8 (pCRs only)

##### 6.3.13.9	TS 36.579-9 (pCRs only)

##### 6.3.13.10	Discussion Papers, Work Plan, TC lists

R5-242587	Discussion on extending RAN5 scope to include MCPTT, MCVideo, and MCData Server Test Cases

Type: discussion		For: Decision
					Source: FisrtNet, NIST, Ericsson, AT&T, Samsung, Element

Abstract:

For 3GPP Release 13, Mission Critical Push To Talk (MCPTT) was defined in core specifications. Consequently, and for commercial need, a Work Item [1] was opened in RAN5 to create conformance test cases. The Work Item created the TS 36.579 family of specifications. The TS 36.579 family of specifications included both UE/Device/Client conformance test cases as well as server conformance test cases. The server test cases, included in TS 36.579-3, were created for situations where an MCPTT server would exist outside of the core network and be controlled by an end user acting as a customer of a carrier. No network or core testing was part of the scope of the Work Item.

The justification of server-side conformance testing in Rel-13 Work Item Description [1] states:

“Furthermore, in order to avoid fragmentation of the compliance testing of the different MCPTT architectural elements, and the possibility of varying interpretation of 3GPP core spec requirements, the industry has identified the need for a 3GPP Conformance test specification for the MCPTT Server. Although this may be seen as outside of the present RAN5 scope, it should be noted that an MCPTT Server-like implementation will be implemented in the UEs to allow them to operate in off-network scenarios. MCPTT Server conformance testing will be a mere extension of this concept, and does not represent a bridge for RAN5 to test the Network.”

In subsequent mission critical services Work Items for Release 14 [2] and Release 15 [3], Mission Critical Video (MCVideo) and Mission Critical Data (MCData) were included in the TS 36.579 family of specifications for UE/Device/Client conformance test cases only. There was not strong Industry need for MCVideo and MCData server test cases.

For Release 16, the Industry needs were changed. The Release 16 mission critical services Work Item Description (WID) [4] included updates for all existing TS 36.579 specifications and included the creation of new specifications TS 36.579-8 for MCVideo server test cases and TS 36.579-9 for MCData server test cases. The WID was endorsed by RAN5 and approved in the RAN Plenary.

Once work began on the Rel-16 Work Item, RAN5 and TF 160 leadership decided that MCPTT, MCVideo, and MCData server test cases were outside the scope of RAN5. This has caused all work for MCPTT, MCVideo, and MCData server test cases to halt within RAN5.

It would be helpful if we can agree on changing the scope for RAN5 which would enable including MCPTT, MCVideo, and MCData server test cases in RAN5 specs.

2	Discussion

There are scenarios in the real world where a server, or a device that can perform as a server, interacts with the core and its mission critical services (MCX) servers. In these cases, a human user controls the device, so technically, it’s an end user device. The most common instance of this is with the use of dispatch interfaces and mobile dispatch devices. A human uses the dispatch interface to enable groups, disable groups, create groups, etc. and interact with the carrier core servers. MCX (MCPTT, MCVideo, and MCData) Server conformance testing will be a mere extension of this concept, and does not represent a bridge for RAN5 to test the Network.

Server side MCX conformance testing specs are critical to bring interoperability among MCX application server vendors/providers and across interconnected MCX systems from various vendors. It can help in reducing implementation cost and rollout time.

Server-side conformance testing is currently not in scope of 3GPP RAN5. It has resulted in pausing all work for MCPTT, MCVideo, and MCData server test cases which were endorsed by RAN5 and approved by RAN plenaries in Rel-13 and Rel-16 WIDs for mission critical services. It will be beneficial to discuss and agree on a way forward to resume work on MCPTT, MCVideo, and MCData server test cases. Following proposals may be potential way forward:

Proposal 1: Accept that some human-controlled devices are able to act as Mission Critical Services servers, exist outside of the core network, and can therefore fit within the current scope of RAN5.

or

Proposal 2: Extend the scope of RAN5 to include Mission Critical Services server conformance testing which would enable incorporating MCPTT, MCVideo, and MCData server test cases in RAN5 specs.

3	Proposal

Proposal 1: Accept that some human-controlled devices are able to act as Mission Critical Services servers, exist outside of the core network, and can therefore fit within the current scope of RAN5.

or

Proposal 2: Extend the scope of RAN5 to include Mission Critical Services server conformance testing which would enable incorporating MCPTT, MCVideo, and MCData server test cases in RAN5 specs.

Discussion:

RAN5 Chair: the ToR RP-210790 should be changed.

But it will be restricted to RAN5 server testing only,

Decision: 		The document was noted.

#### 6.3.14	Further Multi-RAT Dual-Connectivity enhancement (UID-991033) LTE_NR_DC_enh2-UEConTest

##### 6.3.14.1	TS 38.508-1

R5-243174	Common configuration for SN initiated inter-SN CPC EN-DC for 38.508-1

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3201  Cat: F (Rel-18)

					Source: Nokia

Discussion:

r1

Deferred. Still discussion ongoing.

Decision: 		The document was revised to R5-243952.

R5-243952	Common configuration for SN initiated inter-SN CPC EN-DC for 38.508-1

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3201  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-243174)

Decision: 		The document was agreed.

##### 6.3.14.2	TS 38.508-2

R5-243177	Addition of capability clauses for inter-SN CPC testcases for EN-DC and NR-DC

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0644  Cat: F (Rel-18)

					Source: Nokia

Discussion:

r1

Decision: 		The document was revised to R5-243953.

R5-243953	Addition of capability clauses for inter-SN CPC testcases for EN-DC and NR-DC

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0644  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-243177)

Decision: 		The document was agreed.

##### 6.3.14.3	TS 38.523-1

R5-242857	Update of message contents for 8.2.3.18.4 conditional PSCell change

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4423  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r1

to solve some message issues, TF160’s comments.

Decision: 		The document was revised to R5-243580.

R5-243580	Update of message contents for 8.2.3.18.4 conditional PSCell change

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4423  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242857)

Decision: 		The document was agreed.

R5-243172	Addition of new test case 8.2.3.18.6 inter-SN CPC for EN-DC

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4449  Cat: F (Rel-17)

					Source: Nokia

Discussion:

r1

Decision: 		The document was revised to R5-243954.

R5-243954	Addition of new test case 8.2.3.18.6 inter-SN CPC for EN-DC

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4449  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-243172)

Decision: 		The document was agreed.

R5-243175	Addition of new test case 8.2.3.18.7 inter-SN CPC for NR-DC

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4450  Cat: F (Rel-17)

					Source: Nokia

Discussion:

r2

Ongoing discussions.

r4

Decision: 		The document was revised to R5-243955.

R5-243955	Addition of new test case 8.2.3.18.7 inter-SN CPC for NR-DC

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4450  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-243175)

Decision: 		The document was agreed.

##### 6.3.14.4	TS 38.523-2

R5-243176	Addition of applicability clauses for inter-SN CPC testcases for EN-DC and NR-DC

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0480  Cat: F (Rel-17)

					Source: Nokia

Discussion:

r1

Decision: 		The document was revised to R5-243956.

R5-243956	Addition of applicability clauses for inter-SN CPC testcases for EN-DC and NR-DC

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0480  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-243176)

Decision: 		The document was agreed.

##### 6.3.14.5	TS 38.523-3

##### 6.3.14.6	TS 36.508

R5-243173	Common configuration for SN initiated inter-SN CPC EN-DC for 36.508

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1478  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was withdrawn.

##### 6.3.14.7	Discussion Papers, Work Plan, TC lists

#### 6.3.15	Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking (UID-991034) ID_UAS-UEConTest

##### 6.3.15.1	TS 38.508-1

R5-242565	Addition of generci procedure for UUAA-MM

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3144  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Discussion:

Ericsson: .X -> .44

Decision: 		The document was revised to R5-243574.

R5-243574	Addition of generci procedure for UUAA-MM

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3144  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated

(Replaces R5-242565)

Decision: 		The document was agreed.

R5-242566	Correction to Registration request and Registration accept information element for UAS support

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3145  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was withdrawn.

##### 6.3.15.2	TS 38.508-2

##### 6.3.15.3	TS 38.523-1

R5-243124	Updates to UAS TC 9.1.5.2.11

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4441  Cat: F (Rel-17)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243575.

R5-243575	Updates to UAS TC 9.1.5.2.11

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4441  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-243124)

Decision: 		The document was agreed.

R5-243125	Updates to UAS TC 10.1.4.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4442  Cat: F (Rel-17)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243576.

R5-243576	Updates to UAS TC 10.1.4.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4442  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-243125)

Decision: 		The document was agreed.

R5-243169	Editorial correction of testcase 9.1.6.2.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4447  Cat: F (Rel-17)

					Source: Nokia

Decision: 		The document was agreed.

##### 6.3.15.4	TS 38.523-2

##### 6.3.15.5	TS 38.523-3

##### 6.3.15.6	TS 36.508

R5-242738	Correction to UUAA-SM generic test procedure 4.5A.31

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1476  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.15.7	TS 36.523-1

R5-242739	Correction to EPS UAS test cases

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5312  Cat: F (Rel-18)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

##### 6.3.15.8	TS 36.523-2

##### 6.3.15.9	TS 36.523-3

R5-242313	ID_UAS: Test Model aspects

Type: CR		For: Agreement
					36.523-3 v18.3.0	  CR-4820  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

##### 6.3.15.10	Discussion Papers, Work Plan, TC lists

#### 6.3.16	IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS (UID-1000052) ING_5GS-UEConTest

##### 6.3.16.1	TS 38.508-2

R5-242733	Addition of PICS for ING_5GS test cases

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0630  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, China Telecom

Discussion:

TF160 manager: reason not clear; add summary of change

r1

Decision: 		The document was revised to R5-243530.

R5-243530	Addition of PICS for ING_5GS test cases

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0630  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated, China Telecom

(Replaces R5-242733)

Decision: 		The document was agreed.

##### 6.3.16.2	TS 38.523-1

R5-242734	Addition of ING_5GS test case 9.3.1.5

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4398  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242735	Void ING_5GS test case 11.1.10

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4399  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was agreed.

##### 6.3.16.3	TS 38.523-2

R5-242736	Applicability updates to ING_5GS test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0469  Cat: F (Rel-17)

					Source: Qualcomm Incorporated, China Telecom

Discussion:

r1

Decision: 		The document was revised to R5-243960.

R5-243960	Applicability updates to ING_5GS test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0469  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated, China Telecom

(Replaces R5-242736)

Decision: 		The document was agreed.

##### 6.3.16.4	TS 38.523-3

##### 6.3.16.5	Discussion Papers, Work Plan, TC lists

#### 6.3.17	IMS Data Channel (UID-1000055) IMSProtoc17_dataCH-UEContest

##### 6.3.17.1	TS 36.508

##### 6.3.17.2	TS 36.509

##### 6.3.17.3	TS 38.508-1

##### 6.3.17.4	TS 38.509

##### 6.3.17.5	TS 34.229-1

R5-242257	Initial Registration / IMS data channel

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1531  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

title!

Decision: 		The document was withdrawn.

##### 6.3.17.6	TS 34.229-2

##### 6.3.17.7	TS 34.229-3

##### 6.3.17.8	TS 34.229-5

R5-243219	Add new NR IMS data channel testcase - Bootstrap Data Channel

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0596  Cat: F (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was withdrawn.

##### 6.3.17.9	Discussion Papers, Work Plan, TC lists

#### 6.3.18	Rel-18 NR CA and DC; and NR and LTE DC Configurations (UID-1000057) NR_CADC_NR_LTE_DC_R18-UEConTest

##### 6.3.18.1	TS 38.508-1

##### 6.3.18.2	TS 38.508-2

##### 6.3.18.3	TS 38.523-1

##### 6.3.18.4	TS 38.523-2

##### 6.3.18.5	TS 38.523-3

##### 6.3.18.6	Discussion Papers, Work Plan, TC lists

#### 6.3.19	New Rel-18 NR licensed bands and extension of existing NR bands (UID-1000058) NR_lic_bands_BW_R18-UEConTest

##### 6.3.19.1	TS 38.523-3

##### 6.3.19.2	Discussion Papers, Work Plan, TC lists

##### 6.3.19.3	TS 38.508-1

R5-242369	Signalling test frequencies for 3 MHz and band n106

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3121  Cat: F (Rel-18)

					Source: Nokia

Discussion:

r1

Define test frequencies only for Low range.

Decision: 		The document was revised to R5-243566.

R5-243566	Signalling test frequencies for 3 MHz and band n106

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3121  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242369)

Decision: 		The document was agreed.

#### 6.3.20	Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 (UID-1010050) ATSSS_Ph2-UEConTest

##### 6.3.20.1	TS 38.508-1

##### 6.3.20.2	TS 38.508-2

##### 6.3.20.3	TS 38.523-1

R5-242271	Addition to R17 ATSSS testcase 10.5.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4345  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

R&S: in this test case the parallel behaviour is different.

Deferred.

r1

Decision: 		The document was revised to R5-243964.

R5-243964	Addition to R17 ATSSS testcase 10.5.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4345  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242271)

Decision: 		The document was agreed.

R5-242272	Addition to R17 ATSSS testcase 10.5.2.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4346  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

see R5-142271.

Decision: 		The document was revised to R5-243965.

R5-243965	Addition to R17 ATSSS testcase 10.5.2.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4346  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242272)

Decision: 		The document was agreed.

R5-242280	Correction to R17 ATSSS TC 11.9.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4352  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

merged into R5-242703r1

Decision: 		The document was withdrawn.

R5-242281	Correction to R17 ATSSS TC 11.9.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4353  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

merged into R5-242464r1

Decision: 		The document was withdrawn.

R5-242462	 Update to  R17 ATSSS test case 11.9.1

Type: CR		For: Agreement
					38.523-1 v16.14.0	  CR-4363  Cat: F (Rel-16)

					Source: China Telecom

Decision: 		The document was withdrawn.

R5-242464	Update to R17 ATSSS test case 11.9.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4364  Cat: F (Rel-17)

					Source: China Telecom, MediaTek

Discussion:

r1

Decision: 		The document was revised to R5-243522.

R5-243522	Update to R17 ATSSS test case 11.9.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4364  rev 1 Cat: F (Rel-17)

					Source: China Telecom, MediaTek

(Replaces R5-242464)

Decision: 		The document was agreed.

R5-242703	Update to R17 ATSSS test case 11.9.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4397  Cat: F (Rel-17)

					Source: China Telecom, MediaTek

Discussion:

r1

Decision: 		The document was revised to R5-243553.

R5-243553	Update to R17 ATSSS test case 11.9.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4397  rev 1 Cat: F (Rel-17)

					Source: China Telecom, MediaTek

(Replaces R5-242703)

Decision: 		The document was agreed.

R5-242956	Update ATSSS test case 11.9.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4429  Cat: F (Rel-17)

					Source: ZTE

Decision: 		The document was agreed.

R5-242957	Update ATSSS test case 11.9.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4430  Cat: F (Rel-17)

					Source: ZTE

Decision: 		The document was agreed.

R5-243165	New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4445  Cat: F (Rel-17)

					Source: Nokia

Discussion:

r2

Decision: 		The document was revised to R5-243968.

R5-243968	New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4445  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-243165)

Decision: 		The document was agreed.

R5-243166	New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4446  Cat: F (Rel-17)

					Source: Nokia

Discussion:

r2

Decision: 		The document was revised to R5-243969.

R5-243969	New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4446  rev 1 Cat: F (Rel-17)

					Source: Nokia

(Replaces R5-243166)

Decision: 		The document was agreed.

##### 6.3.20.4	TS 38.523-2

R5-242273	Add applicability for Rel-17 ATSSS test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0458  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-243167	Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0478  Cat: F (Rel-17)

					Source: Nokia

Decision: 		The document was agreed.

##### 6.3.20.5	TS 38.523-3

##### 6.3.20.6	Discussion Papers, Work Plan, TC lists

#### 6.3.21	Rel-17 LTE CA Configurations (UID-1010051) LTE_CA_R17-UEConTest

##### 6.3.21.1	TS 36.508

##### 6.3.21.2	TS 36.523-1

##### 6.3.21.3	TS 36.523-2

##### 6.3.21.4	TS 36.523-3

##### 6.3.21.5	Discussion Papers, Work Plan, TC lists

#### 6.3.22	Air-to-ground network for NR (UID-1020087) NR_ATG-UEConTest

##### 6.3.22.1	TS 38.508-1

##### 6.3.22.2	TS 38.508-2

##### 6.3.22.3	TS 38.523-1

##### 6.3.22.4	TS 38.523-2

##### 6.3.22.5	TS 38.523-3

##### 6.3.22.6	Discussion Papers, Work Plan, TC lists

#### 6.3.23	NR support for dedicated spectrum less than 5MHz for FR1 (UID-1030060) NR_FR1_lessthan_5MHz_BW-UEConTest

##### 6.3.23.1	TS 38.508-1

##### 6.3.23.2	TS 38.508-2

##### 6.3.23.3	TS 38.523-1

##### 6.3.23.4	TS 38.523-2

##### 6.3.23.5	TS 38.523-3

##### 6.3.23.6	Discussion Papers, Work Plan, TC lists

#### 6.3.24	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR (UID-1030061) NR_DualTxRx_MUSIM-UEConTest

##### 6.3.24.1	TS 38.508-1

##### 6.3.24.2	TS 38.508-2

##### 6.3.24.3	TS 38.509

##### 6.3.24.4	TS 38.523-1

##### 6.3.24.5	TS 38.523-2

##### 6.3.24.6	TS 38.523-3

##### 6.3.24.7	Discussion Papers, Work Plan, TC lists

#### 6.3.25	Enhanced support of reduced capability NR devices plus CT1 aspects (UID-1030062) NR_redcap_enh_plus_CT1-UEConTest

##### 6.3.25.1	TS 38.508-1

##### 6.3.25.2	TS 38.508-2

##### 6.3.25.3	TS 38.523-1

##### 6.3.25.4	TS 38.523-2

##### 6.3.25.5	TS 38.523-3

##### 6.3.25.6	TS 34.229-2

##### 6.3.25.7	Discussion Papers, Work Plan, TC lists

#### 6.3.26	XR (eXtended Reality) enhancements for NR plus CT1 aspects (UID-1030063) NR_XR_enh_plus_CT1-UEConTest

##### 6.3.26.1	TS 38.508-1

##### 6.3.26.2	TS 38.508-2

##### 6.3.26.3	TS 38.509

##### 6.3.26.4	TS 38.523-1

##### 6.3.26.5	TS 38.523-2

##### 6.3.26.6	TS 38.523-3

##### 6.3.26.7	Discussion Papers, Work Plan, TC lists

#### 6.3.27	Introduction of 900 MHz LTE band in the US (UID-1030064) LTE_900MHz_US-UEConTest

##### 6.3.27.1	TS 36.508

R5-242385	Addition of signalling test frequencies for LTE Band 106 and 3 MHz channel bandwidth

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1475  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 6.3.27.2	TS 36.523-1

R5-242392	Adding less than 5 MHz BW into Bandwidth dependent parameters

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5305  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 6.3.27.3	TS 36.523-2

R5-242393	Addition of band 106 to RF Baseline Implementation Capabilities

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1450  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 6.3.27.4	TS 36.523-3

##### 6.3.27.5	Discussion Papers, Work Plan, TC lists

#### 6.3.28	Further RF requirements enhancement for NR and EN-DC in frequency range 1 (UID-1030066) NR_ENDC_RF_FR1_enh2-UEConTest

##### 6.3.28.1	TS 38.508-1

##### 6.3.28.2	TS 38.508-2

##### 6.3.28.3	TS 38.523-1

##### 6.3.28.4	Discussion Papers, Work Plan, TC lists

#### 6.3.29	Network energy savings for NR (UID-1030067) Netw_Energy_NR-UEConTest

##### 6.3.29.1	TS 38.508-1

##### 6.3.29.2	TS 38.508-2

##### 6.3.29.3	TS 38.523-1

##### 6.3.29.4	TS 38.523-2

##### 6.3.29.5	TS 38.523-3

##### 6.3.29.6	Discussion Papers, Work Plan, TC lists

#### 6.3.30	Further NR coverage enhancements (UID-1030069) NR_cov_enh2-UEConTest

##### 6.3.30.1	TS 38.508-1

##### 6.3.30.2	TS 38.508-2

##### 6.3.30.3	TS 38.523-1

##### 6.3.30.4	TS 38.523-2

##### 6.3.30.5	TS 38.523-3

##### 6.3.30.6	Discussion Papers, Work Plan, TC lists

#### 6.3.31	Multi-carrier enhancements for NR (UID-1030070) NR_MC_enh-UEConTest

##### 6.3.31.1	TS 38.508-1

##### 6.3.31.2	TS 38.508-2

##### 6.3.31.3	TS 38.523-1

##### 6.3.31.4	TS 38.523-2

##### 6.3.31.5	TS 38.523-3

##### 6.3.31.6	Discussion Papers, Work Plan, TC lists

#### 6.3.32	Enhanced NR support for high speed train scenario in frequency range 2 (FR2) (UID-1030071) NR_HST_FR2_enh-UEConTest

##### 6.3.32.1	TS 38.508-1

##### 6.3.32.2	TS 38.508-2

##### 6.3.32.3	TS 38.523-1

##### 6.3.32.4	TS 38.523-2

##### 6.3.32.5	TS 38.523-3

##### 6.3.32.6	Discussion Papers, Work Plan, TC lists

#### 6.3.33	NR MIMO evolution for downlink and uplink  (FR2) (UID-1030072) NR_MIMO_evo_DL_UL-UEConTest

##### 6.3.33.1	TS 38.508-1

##### 6.3.33.2	TS 38.508-2

##### 6.3.33.3	TS 38.523-1

##### 6.3.33.4	TS 38.523-2

##### 6.3.33.5	TS 38.523-3

##### 6.3.33.6	Discussion Papers, Work Plan, TC lists

#### 6.3.34	IoT (Internet of Things) NTN (non-terrestrial network) enhancements plus CT1 aspects (UID-1030073) IoT_NTN_enh_plus_CT1-UEConTest

##### 6.3.34.1	TS 36.508

##### 6.3.34.2	TS 36.509

##### 6.3.34.3	TS 36.523-1

##### 6.3.34.4	TS 36.523-2

R5-243062	Update to R18 IoT NTN enh SIG TCs applicability

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1453  Cat: F (Rel-18)

					Source: CMCC

Discussion:

no contributions under AI 6.3.34, was a place holder

Decision: 		The document was withdrawn.

##### 6.3.34.5	TS 36.523-3

##### 6.3.34.6	Discussion Papers, Work Plan, TC lists

### 6.4	Routine Maintenance for TS 38 Series TEIx_Test

#### 6.4.1	TS 38.508-1

##### 6.4.1.1	Generic Procedures and Test Procedures (Clauses 4.5, 4.5A & 4.9)

R5-242321	Editorial updates to specification references in clauses 4.5, 4.5A & 4.9

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3115  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242576	Editorial correction for generic procedure 4.5.4, 4.5.5 and 4.5.7

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3148  Cat: F (Rel-18)

					Source: ZTE Corporation

Abstract:

Editorial

Discussion:

r1

overlapping with R5-242321.

Is not editorial.

Decision: 		The document was revised to R5-243489.

R5-243489	Editorial correction for generic procedure 4.5.4, 4.5.5 and 4.5.7

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3148  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R5-242576)

Decision: 		The document was agreed.

R5-243168	Editorial correction of Table 4.5.4.2-4: WLAN IPsec_SA_Established

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3200  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was agreed.

##### 6.4.1.2	Default NG-RAN RRC messages and IEs (Clause 4.6)

R5-242139	Correction to RRC message MobilityFromNRCommand

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3108  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

Keysight UK: are test cases impacted?

Decision: 		The document was agreed.

R5-242140	Correction to IE PDCP-Config for UDC

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3109  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242141	Correction to IE MeasIdleConfig for Idle/Inactive measurements

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3110  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242142	Correction to IE CondReconfigToAddModList

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3111  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242256	Add IE NeedForInterruptionInfoNR

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3114  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-242322	Editorial updates to specification references in clause 4.6

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3116  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242401	Add IE AppLayerIdleInactiveConfig

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3123  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242402	Add IE EUTRA-MultiBandInfoListAerial

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3124  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242405	Add IE MBS-NonServingInfoList

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3125  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242406	Add IE MBS-SessionInfoListMulticast

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3126  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242407	Add IE AerialParameters

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3127  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242408	Add IE DL-PRS-MeasurementWithRxFH-RRC-Connected

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3128  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242409	Add IE ERedCapParameters

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3129  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242410	Add IE NCR-Parameters

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3130  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242411	Add IE PosSRS-BWA-RRC-Inactive

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3131  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-242412	Add IEs PosSRS-TxFrequencyHoppingRRC-Connected and PosSRS-TxFrequencyHoppingRRC-Inactive

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3132  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-242413	Update IE SharedSpectrumChAccessParamsPerBand

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3133  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242414	Add IE SharedSpectrumChAccessParamsSidelinkPerBand

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3134  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242415	Add IE SupportedAggBandwidth

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3135  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

TEI18->17

Decision: 		The document was agreed.

R5-242603	Add IE PosSRS-BWA-RRC-Inactive

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3152  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242604	Add IEs PosSRS-TxFrequencyHoppingRRC-Connected and PosSRS-TxFrequencyHoppingRRC-Inactive

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3153  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242648	Add IE Aerial-Config

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3154  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

Tdoc#

Decision: 		The document was revised to R5-243490.

R5-243490	Add IE Aerial-Config

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3154  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242648)

Decision: 		The document was agreed.

R5-242649	Add IE EpochTime

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3155  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

Tdoc#

r1

Decision: 		The document was revised to R5-243491.

R5-243491	Add IE EpochTime

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3155  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-242649)

Decision: 		The document was agreed.

R5-242650	Add IE LTM-TCI-Info

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3156  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242651	Add IE N3C-RelayUE-Info

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3157  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242652	Add IE NeedForInterruptionInfoNR

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3158  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242653	Add IE PDU-SessionID

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3159  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242654	Add IE PUCCH-CSI-Resource

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3160  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242655	Add IE QFI

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3161  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242656	Add IE RACH-ConfigTwoTA

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3162  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242657	Add IE ReferenceConfiguration

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3163  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242658	Add IE SelectedPSCellForCHO-WithSCG

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3164  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242659	Add IE SI-RequestConfigRepetition

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3165  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242660	Add IEs SRS-PosTx-Hopping and SRS-PosResourceSetLinkedForAggBW

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3166  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242661	Add IE TN-AreaId

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3167  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242763	Add IEs SL-BWP-DiscPoolConfig and SL-BWP-DiscPoolConfigCommon

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3172  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242764	Add IEs SL-BWP-PRS-PoolConfig and SL-BWP-PRS-PoolConfigCommon

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3173  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-243087	Correction to IE PDCCH-ConfigCommon

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3198  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

cl. aff.

r1

no RF impact

Decision: 		The document was revised to R5-243493.

R5-243493	Correction to IE PDCCH-ConfigCommon

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3198  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc.

(Replaces R5-243087)

Decision: 		The document was agreed.

R5-243187	Add IE SL-CBR-CommonTxDedicated-SL-PRS-RP-List

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3203  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-243188	Add IE SL-ConfiguredGrantConfigDedicated-SL-PRS-RP

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3204  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

R5-243216	Add IEs SL-DRX-Config, SL-DRX-ConfigGC-BC, SL-DRX-ConfigUC and SL-DRX-ConfigUC-SemiStatic

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3205  Cat: F (Rel-18)

					Source: Ericsson

Discussion:

TEI18->17

Decision: 		The document was revised to R5-243492.

R5-243492	Add IEs SL-DRX-Config, SL-DRX-ConfigGC-BC, SL-DRX-ConfigUC and SL-DRX-ConfigUC-SemiStatic

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3205  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R5-243216)

Decision: 		The document was agreed.

R5-243217	Add IEs SL-FreqSelectionConfig and SL-IndirectPathAddChange

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3206  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

##### 6.4.1.3	Default 5GC NAS messages and IEs (Clause 4.7)

R5-242323	Editorial updates to REGISTRATION REQUEST message

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3117  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

##### 6.4.1.4	Test environment for SIG (Clause 6)

R5-242363	Introduction of signalling test frequencies for band n85

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3119  Cat: F (Rel-18)

					Source: Nokia

Discussion:

TF160 manager: pls. undo the line border

Decision: 		The document was revised to R5-243494.

R5-243494	Introduction of signalling test frequencies for band n85

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3119  rev 1 Cat: F (Rel-18)

					Source: Nokia

(Replaces R5-242363)

Decision: 		The document was agreed.

##### 6.4.1.5	Other clauses, Annexes

R5-243075	Addition of a new section for IDC Configuration

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3197  Cat: F (Rel-18)

					Source: CMCC, CATT, Qualcomm

Discussion:

r3

TF160 manager: 'may' may not be appropriate.

RAN5 Chair: this config shall apply when the pics name is set to TRUE. Remove 'may'.

Decision: 		The document was revised to R5-243581.

R5-243581	Addition of a new section for IDC Configuration

Type: CR		For: Agreement
					38.508-1 v18.2.0	  CR-3197  rev 1 Cat: F (Rel-18)

					Source: CMCC, CATT, Qualcomm

(Replaces R5-243075)

Decision: 		The document was agreed.

#### 6.4.2	TS 38.508-2

R5-242143	Updates to align PICS mnemonics

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0609  Cat: F (Rel-18)

					Source: MediaTek Inc.

Discussion:

overlap

Decision: 		The document was withdrawn.

R5-242325	Updates to align PICS mnemonics

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0616  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242544	Editorial changes to tables of 38.508-2

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0625  Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

Abstract:

Editorial

Discussion:

3GU WIC

r1

Overlapping changes from R5-242461, R5-242242, R5-242709 merged to Table A.4.3.2A.4.1-3 were removed again in the final (un-merged).

Decision: 		The document was revised to R5-243496.

R5-243496	Editorial changes to tables of 38.508-2

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0625  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Technologies Ireland

(Replaces R5-242544)

Discussion:

This CR shall be implemented frist!

Decision: 		The document was agreed.

R5-242583	Addition of UE General Capability of supporting NSAC

Type: CR		For: Agreement
					38.508-2 v17.11.0	  CR-0626  Cat: F (Rel-17)

					Source: CATT

Discussion:

wrong Rel & ver

Decision: 		The document was withdrawn.

R5-242827	Add new PICS for MUSIM gap

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0635  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R5-242832	Add new PICS for RedCap HD-FDD TC

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0636  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-243495.

R5-243495	Add new PICS for RedCap HD-FDD TC

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0636  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-242832)

Decision: 		The document was agreed.

R5-243076	Addition of new PICS for IDC mechanism

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0640  Cat: F (Rel-18)

					Source: CMCC, CATT, Qualcomm

Discussion:

r5

TF160 manager: Note 1 is not needed.

This was agreed.

Decision: 		The document was revised to R5-243582.

R5-243582	Addition of new PICS for IDC mechanism

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0640  rev 1 Cat: F (Rel-18)

					Source: CMCC, CATT, Qualcomm

(Replaces R5-243076)

Decision: 		The document was agreed.

R5-243086	Add new PICS for MUSIM gap

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0641  Cat: F (Rel-18)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-243957.

R5-243957	Add new PICS for MUSIM gap

Type: CR		For: Agreement
					38.508-2 v18.2.0	  CR-0641  rev 1 Cat: F (Rel-18)

					Source: Huawei, HiSilicon

(Replaces R5-243086)

Decision: 		The document was withdrawn.

#### 6.4.3	TS 38.509

#### 6.4.4	TS 38.523-1

##### 6.4.4.1	Clauses 1 - 5

##### 6.4.4.2	Idle Mode (Clause 6)

R5-242144	Correction to Idle mode test cases

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4313  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242145	Correction to eCPSOR TC 6.3.2.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4314  Cat: F (Rel-17)

					Source: MediaTek Inc., Anritsu, ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-242146	Correction to eCPSOR TC 6.3.2.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4315  Cat: F (Rel-17)

					Source: MediaTek Inc., Keysight

Decision: 		The document was agreed.

R5-242147	Correction to eCPSOR TC 6.3.2.5

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4316  Cat: F (Rel-17)

					Source: MediaTek Inc., Anritsu

Decision: 		The document was agreed.

R5-242148	Correction to NR slice TC 6.4.2.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4317  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242149	Correction to CAG TC 6.5.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4318  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242258	Correction to NR CAG Test case 6.5.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4343  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Decision: 		The document was withdrawn.

R5-242270	Corrrection to NR5GC eCPSOR test case 6.3.2.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4344  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Discussion:

covered in R5-242146.

merged with R5-242146.

Decision: 		The document was withdrawn.

R5-242489	Correction to NR5GC testcase 6.3.1.10

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4366  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Discussion:

deferred

r1

Decision: 		The document was revised to R5-243587.

R5-243587	Correction to NR5GC testcase 6.3.1.10

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4366  rev 1 Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

(Replaces R5-242489)

Decision: 		The document was agreed.

R5-242592	Correction to NR5GC testcase 6.1.2.13

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4384  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Discussion:

r1

Decision: 		The document was revised to R5-243497.

R5-243497	Correction to NR5GC testcase 6.1.2.13

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4384  rev 1 Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

(Replaces R5-242592)

Decision: 		The document was agreed.

R5-242793	Corrrection to NR5GC CAG test case 6.5.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4411  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Discussion:

r1

Rohde&Schwarz started a discussion on whether the check should be done in the preamble

Decision: 		The document was revised to R5-243588.

R5-243588	Corrrection to NR5GC CAG test case 6.5.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4411  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK

(Replaces R5-242793)

ACTION:	review & update NR 5GC test cases verification of CAG support
	(action on: TF160, R&S / due by: 2024-08-11)

Decision: 		The document was agreed.

R5-242812	Correction of RRC IDLE TC 6.1.2.23-MFBI

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4412  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242833	Add test applicability for new RedCap HD-FDD TC

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4422  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

wrong title! Resolve .X!

cl. aff-> new!

empty sect.2!

reissued as 243445 because of title change

Decision: 		The document was withdrawn.

R5-243445	Add new RedCap HD-FDD Test Case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4462  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Abstract:

reissued from R5-242833 because of title change

Discussion:

r2

Decision: 		The document was revised to R5-243585.

R5-243585	Add new RedCap HD-FDD Test Case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4462  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-243445)

Decision: 		The document was agreed.

##### 6.4.4.3	Layer 2

###### 6.4.4.3.1	NR Layer 2

6.4.4.3.1.1	Common Test Case Specific Values for Layer 2 (Clause 7.1.0)

6.4.4.3.1.2	MAC

R5-242274	Update 2-step RACH TC 7.1.1.1.8 for HD-FDD UE-PRACH

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4347  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242275	Update 2-step RACH TC 7.1.1.1.9 for HD-FDD UE-PRACH

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4348  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242276	Update URLLC TC 7.1.1.4.1.5 to test RedCap UE

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4349  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242277	Update URLLC TC 7.1.1.4.2.6 to test RedCap UE

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4350  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242278	Update coverage enhancement TC 7.1.1.4.2.7 to test RedCap UE

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4351  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242530	Correction to NR testcase 7.1.1.3.8.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4369  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Decision: 		The document was withdrawn.

R5-242562	Corrections to BWP test case 7.1.1.8.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4379  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

late doc

Deferred.

r1

Decision: 		The document was revised to R5-243966.

R5-243966	Corrections to BWP test case 7.1.1.8.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4379  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242562)

Discussion:

for email agreement

r1

Decision: 		The document was revised to R5-244020.

R5-244020	Corrections to BWP test case 7.1.1.8.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4379  rev 2 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-243966)

Discussion:

email agreed

Decision: 		The document was agreed.

R5-242813	Correction of MAC TC 7.1.1.1.4-Beam Failure

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4413  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r1

RAN5 Vice Chair SIG: pls renumber some steps

Decision: 		The document was revised to R5-243589.

R5-243589	Correction of MAC TC 7.1.1.1.4-Beam Failure

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4413  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242813)

Decision: 		The document was agreed.

R5-243088	Correction of MAC TC 7.1.1.3.2b-ULCGTYPE1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4435  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-243181	Corrections to eMIMO TC 7.1.1.3.10

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4451  Cat: F (Rel-17)

					Source: Lenovo, MCC TF160

Decision: 		The document was agreed.

R5-243183	Corrections to CovEnh TC 7.1.1.1.18

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4452  Cat: F (Rel-17)

					Source: Lenovo, MCC TF160

Decision: 		The document was agreed.

R5-243186	Correction to MAC TC 7.1.1.6.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4453  Cat: F (Rel-17)

					Source: MediaTek Inc., Keysight

Discussion:

offline discussion with Qualcomm, R&S, TF160.

Deferred.

Decision: 		The document was withdrawn.

6.4.4.3.1.3	RLC

6.4.4.3.1.4	PDCP

R5-242150	Correction to UPIP TC 7.1.3.2.6

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4319  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242151	Correction to PDCP TC 7.1.3.4.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4320  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243498.

R5-243498	Correction to PDCP TC 7.1.3.4.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4320  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242151)

Decision: 		The document was agreed.

R5-242152	Correction to PDCP TC 7.1.3.4.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4321  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was revised to R5-243499.

R5-243499	Correction to PDCP TC 7.1.3.4.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4321  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242152)

Decision: 		The document was agreed.

6.4.4.3.1.5	SDAP

##### 6.4.4.4	RRC

###### 6.4.4.4.1	NR RRC

6.4.4.4.1.1	RRC Connection Management Procedures (clause 8.1.1)

R5-242153	Correction to RRC TC 8.1.1.2.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4322  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

merged into R5-242539

Decision: 		The document was withdrawn.

R5-242539	Updates to NR RRC test case 8.1.1.2.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4371  Cat: F (Rel-17)

					Source: MCC TF160, MediaTek

Discussion:

r1

Decision: 		The document was revised to R5-243501.

R5-243501	Updates to NR RRC test case 8.1.1.2.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4371  rev 1 Cat: F (Rel-17)

					Source: MCC TF160, MediaTek

(Replaces R5-242539)

Decision: 		The document was agreed.

R5-242862	Editorial correction for test procedure sequence in 8.1.1.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4424  Cat: F (Rel-17)

					Source: NTT DOCOMO, INC.

Decision: 		The document was agreed.

6.4.4.4.1.2	RRC Reconfiguration (clause 8.1.2)

R5-242828	Correction of MUSIM gap TC 8.1.2.1.6

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4420  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r3

Decision: 		The document was revised to R5-243958.

R5-243958	Correction of MUSIM gap TC 8.1.2.1.6

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4420  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242828)

Decision: 		The document was agreed.

6.4.4.4.1.3	RRC Measurement Configuration Control and Reporting (clause 8.1.3)

6.4.4.4.1.4	RRC Handover (clause 8.1.4)

R5-242561	Corrections to conditional handover test case 8.1.4.4.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4378  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

R-17!

WIC +NR_Mob_enh!

Decision: 		The document was revised to R5-243502.

R5-243502	Corrections to conditional handover test case 8.1.4.4.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4378  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242561)

Decision: 		The document was agreed.

R5-243121	Corrrection to NR5GC test case 8.1.4.4.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4439  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

6.4.4.4.1.5	RRC Others (clause 8.1.5)

R5-242154	Correction to NR Idle measurement TC 8.1.5.11.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4323  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243503.

R5-243503	Correction to NR Idle measurement TC 8.1.5.11.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4323  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242154)

Decision: 		The document was agreed.

R5-242155	Correction to NR Idle measurement TC 8.1.5.11.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4324  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242156	Correction to NR Inactive measurement TC 8.1.5.11.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4325  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242157	Correction to NR Inactive measurement TC 8.1.5.11.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4326  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242327	Updates to NR RRC test case 8.1.5.1.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4360  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242531	Correction to NR5GC testcase 8.1.5.8.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4370  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-242563	Addition of E-UTRA to NR MPS priority Indication test case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4380  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

R-17

r1

Decision: 		The document was revised to R5-243504.

R5-243504	Addition of E-UTRA to NR MPS priority Indication test case

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4380  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242563)

Decision: 		The document was agreed.

R5-242741	Correction to NR RRC UL segmentation test case 8.1.5.9.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4401  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

r1

Decision: 		The document was revised to R5-243505.

R5-243505	Correction to NR RRC UL segmentation test case 8.1.5.9.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4401  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242741)

Decision: 		The document was agreed.

R5-242815	Correction to NR test case 8.1.5.8.1-processing delay

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4414  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r1

Decision: 		The document was revised to R5-243506.

R5-243506	Correction to NR test case 8.1.5.8.1-processing delay

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4414  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242815)

Decision: 		The document was agreed.

6.4.4.4.1.6	RRC SON and MDT support for NR (clause 8.1.6)

R5-242158	Correction to MDT enh. TC 8.1.6.1.2.14

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4327  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242159	Correction to MDT enh. TC 8.1.6.1.3.8

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4328  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243507.

R5-243507	Correction to MDT enh. TC 8.1.6.1.3.8

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4328  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242159)

Decision: 		The document was agreed.

R5-242160	Correction to MDT enh. TC 8.1.6.1.3.9

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4329  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243508.

R5-243508	Correction to MDT enh. TC 8.1.6.1.3.9

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4329  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242160)

Decision: 		The document was agreed.

R5-242161	Correction to MDT enh. TC 8.1.6.1.3.10

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4330  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

offline commets from TF160

Decision: 		The document was revised to R5-243509.

R5-243509	Correction to MDT enh. TC 8.1.6.1.3.10

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4330  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242161)

Decision: 		The document was agreed.

R5-242162	Correction to MDT enh. TC 8.1.6.1.3.11

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4331  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243510.

R5-243510	Correction to MDT enh. TC 8.1.6.1.3.11

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4331  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242162)

Decision: 		The document was agreed.

R5-242163	Correction to MDT enh. TC 8.1.6.1.4.9

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4332  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242164	Correction to MDT enh. TC 8.1.6.4.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4333  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242165	Correction to MDT enh. TC 8.1.6.4.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4334  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242166	Correction to MDT enh. TC 8.1.6.4.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4335  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-243077	Update of test case 8.1.6.1.2.14 for SON_MDT

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4432  Cat: F (Rel-17)

					Source: CMCC, CATT, Qualcomm

Discussion:

R&S: Step 4A, remove the 10s.

r3

Decision: 		The document was revised to R5-243583.

R5-243583	Update of test case 8.1.6.1.2.14 for SON_MDT

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4432  rev 1 Cat: F (Rel-17)

					Source: CMCC, CATT, Qualcomm

(Replaces R5-243077)

Decision: 		The document was agreed.

6.4.4.4.1.7	RRC Non-public networks (clause 8.1.7)

###### 6.4.4.4.2	MR-DC RRC

6.4.4.4.2.1	RRC UE Capability / Others (clause 8.2.1)

R5-242328	Updates to EN-DC RRC test case 8.2.1.1.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4361  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242329	Updates to NE-DC RRC test case 8.2.1.1.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4362  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

6.4.4.4.2.2	RRC Radio Bearer (clause 8.2.2)

R5-242167	Correction to NE-DC TC 8.2.2.8.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4336  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

Changes merged into revision of R5-242816

Decision: 		The document was withdrawn.

R5-242816	Correction to NEDC test case 8.2.2.8.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4415  Cat: F (Rel-17)

					Source: Huawei, HiSilicon, MCC TF160, Mediatek

Discussion:

r1

Decision: 		The document was revised to R5-243511.

R5-243511	Correction to NEDC test case 8.2.2.8.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4415  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon, MCC TF160, Mediatek

(Replaces R5-242816)

Decision: 		The document was agreed.

6.4.4.4.2.3	RRC Measurement / Handovers (clause 8.2.3)

R5-243346	Corrrection to RRC test case 8.2.3.1.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4454  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

R5-243347	Corrrection to RRC test case 8.2.3.2.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4455  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

R5-243348	Corrrection to RRC test case 8.2.3.6.2x

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4456  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

R5-243349	Corrrection to RRC test case 8.2.3.7.2x

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4457  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

R5-243350	Corrrection to RRC test case 8.2.3.8.2x

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4458  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Discussion:

correct reference to non-existing table

Decision: 		The document was revised to R5-243512.

R5-243512	Corrrection to RRC test case 8.2.3.8.2x

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4458  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK

(Replaces R5-243350)

Decision: 		The document was agreed.

R5-243351	Corrrection to RRC test case 8.2.3.12.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4459  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Discussion:

offline comments from Qualcomm.

Decision: 		The document was revised to R5-243513.

R5-243513	Corrrection to RRC test case 8.2.3.12.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4459  rev 1 Cat: F (Rel-17)

					Source: Keysight Technologies UK

(Replaces R5-243351)

Decision: 		The document was agreed.

R5-243352	Corrrection to RRC test case 8.2.3.13.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4460  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

R5-243353	Corrrection to RRC test case 8.2.3.14.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4461  Cat: F (Rel-17)

					Source: Keysight Technologies UK

Decision: 		The document was agreed.

6.4.4.4.2.4	RRC Carrier Aggregation (clause 8.2.4)

R5-242168	Correction to NR CA TC 8.2.4.2.1.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4337  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

6.4.4.4.2.5	RRC Reconfiguration / Radio Link Failure (clause 8.2.5)

6.4.4.4.2.6	RRC Others (clause 8.2.6)

R5-242169	Correction to NE-DC TC 8.2.6.3.5 and 8.2.6.3.6

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4338  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242170	Correction to UPIP TC 8.2.6.4.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4339  Cat: F (Rel-17)

					Source: MediaTek Inc.

Abstract:

Editorial

Decision: 		The document was agreed.

R5-242527	Correction of UPIP TC 8.2.6.4.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4368  Cat: F (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was agreed.

6.4.4.4.2.7	RRC Resume (clause 8.2.7)

##### 6.4.4.5	5GS Mobility Management

###### 6.4.4.5.1	MM Primary authentication and key agreement (clause 9.1.1)

###### 6.4.4.5.2	MM Security mode control, Identification & Generic UE configuration update (clauses 9.1.2, 9.1.3 & 9.1.4)

###### 6.4.4.5.3	MM Registration & De-registration (clauses 9.1.5 & 9.1.6)

R5-242487	Correction to NR5GC testcase 9.1.5.1.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4365  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ, MediaTek, Huawei, Hisilicon

Decision: 		The document was agreed.

###### 6.4.4.5.4	MM Service Request (clause 9.1.7)

R5-242829	Correction of MUSIM NAS TC 9.1.7.3-9.1.7.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4421  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

###### 6.4.4.5.5	MM SMS Over NAS (clause 9.1.8)

###### 6.4.4.5.6	RACS (clause 9.1.9)

R5-242762	Correction to RACS test case 9.1.9.7

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4404  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Decision: 		The document was withdrawn.

R5-242774	Correction to RACS test case 9.1.9.7

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4410  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Decision: 		The document was withdrawn.

###### 6.4.4.5.7	MM Network slice-specific authentication and authorization (clause 9.1.10)

###### 6.4.4.5.8	MM SNPN (clause 9.1.11)

###### 6.4.4.5.9	MM NSAC/NSSRG (clauses 9.1.12 & 9.1.13)

R5-242740	Correction to NSAC test case 9.1.12.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4400  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd

Decision: 		The document was agreed.

###### 6.4.4.5.10	MM Paging Early Indication (clause 9.1.14)

##### 6.4.4.6	5GS Non-3GPP Access Mobility Management (clause 9.2)

##### 6.4.4.7	5GS Inter-system Mobility (clause 9.3)

R5-242761	Correction to eNS test case 9.3.1.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4403  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd, MediaTek

Decision: 		The document was revised to R5-243514.

R5-243514	Correction to eNS test case 9.3.1.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4403  rev 1 Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd, MediaTek

(Replaces R5-242761)

Decision: 		The document was agreed.

##### 6.4.4.8	5GS Session Management

###### 6.4.4.8.1	SM PDU session authentication and authorization (clause 10.1.1)

###### 6.4.4.8.2	SM Network-requested PDU session modification & release (clauses 10.1.2 & 10.1.3)

R5-242937	Addition of NR P-CSCF restoration test case 10.1.2.3

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4428  Cat: F (Rel-17)

					Source: Qualcomm Incorporated, Orange

Decision: 		The document was agreed.

###### 6.4.4.8.3	SM UE-requested PDU session establishment, modification & release (clauses 10.1.4, 10.1.5 & 10.1.6)

###### 6.4.4.8.4	SM NSAC (clauses 10.1.8)

R5-242171	Correction to NSAC TC 10.1.8.4

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4340  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242773	Correction to eNS test case 10.1.8.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4409  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd, MediaTek

Decision: 		The document was agreed.

R5-242958	Correction to eNS test case 10.1.8.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4431  Cat: F (Rel-17)

					Source: Anritsu EMEA Ltd, MediaTek

Decision: 		The document was agreed.

R5-243500	Update the NSAC test case titles

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4464  Cat: F (Rel-17)

					Source: CATT

Discussion:

late doc

TF160 manager: commented about the Rel's.

Decision: 		The document was revised to R5-243590.

R5-243590	Update the NSAC test case titles

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4464  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-243500)

Decision: 		The document was agreed.

##### 6.4.4.9	EN-DC Session Management (clause 10.2)

##### 6.4.4.10	5GS Non-3GPP Access & ATSS Session Management (clauses 10.3 & 10.4)

##### 6.4.4.11	5GS Multilayer and Services

###### 6.4.4.11.1	EPS Fallback (clause 11.1)

R5-242817	Correction to NR 5GC Multilayer TC 11.1.7-Emergency Services Fallback to EPS with redirection

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4416  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

Deferred.

r2

Decision: 		The document was revised to R5-243591.

R5-243591	Correction to NR 5GC Multilayer TC 11.1.7-Emergency Services Fallback to EPS with redirection

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4416  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242817)

Decision: 		The document was agreed.

###### 6.4.4.11.2	5G-SRVCC (clause 11.2)

###### 6.4.4.11.3	Unified Access Control (UAC) (clause 11.3)

R5-242172	Correction to UAC TC 11.3.5

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4341  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242173	Correction to UAC TC 11.3.12

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4342  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r1

Decision: 		The document was revised to R5-243515.

R5-243515	Correction to UAC TC 11.3.12

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4342  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242173)

Decision: 		The document was agreed.

R5-242496	Correction to NR5GC testcase 11.3.12

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4367  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Discussion:

r1

Deferred uplon request of TF160.

Decision: 		The document was revised to R5-243592.

R5-243592	Correction to NR5GC testcase 11.3.12

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4367  rev 1 Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

(Replaces R5-242496)

Decision: 		The document was agreed.

###### 6.4.4.11.4	Emergency Services (clause 11.4)

R5-242818	Correction to NR 5GC Multilayer TC 11.4.12-Emergency Services

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4417  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r4

Decision: 		The document was revised to R5-243593.

R5-243593	Correction to NR 5GC Multilayer TC 11.4.12-Emergency Services

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4417  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242818)

Decision: 		The document was agreed.

###### 6.4.4.11.5	eCall over IMS (clause 11.5)

R5-242819	Correction of NR eCall TC 11.5.5-Limited service

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4418  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

Comments from Qualcomm and TF160 about PDU Session, update wait timer, and Step numbering PRD13 based.

Deferred.

r1

Decision: 		The document was revised to R5-243594.

R5-243594	Correction of NR eCall TC 11.5.5-Limited service

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4418  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242819)

Decision: 		The document was agreed.

R5-242820	Correction of NR eCall TC 11.5.12-Domain Selection

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4419  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

###### 6.4.4.11.6	3GPP PS Data Off (clause 11.6)

###### 6.4.4.11.7	eDRX (clause 11.7)

R5-242326	Updates to eDRX TC 11.7.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4359  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

###### 6.4.4.11.8	Inter-system mobility between untrusted Non-3GPP and 3GPP system (clause 11.8)

###### 6.4.4.11.9	MBS (clause 14)

R5-242594	Correction for MBS TC 14.2.4.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4385  Cat: F (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was revised to R5-243516.

R5-243516	Correction for MBS TC 14.2.4.2.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4385  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-242594)

Decision: 		The document was agreed.

R5-242595	Correction for MBS TC 14.2.4.3.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4386  Cat: F (Rel-17)

					Source: ZTE Corporation

Discussion:

r1

Decision: 		The document was revised to R5-243517.

R5-243517	Correction for MBS TC 14.2.4.3.2

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4386  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-242595)

Decision: 		The document was agreed.

R5-242596	Editorial correction for MBS TC of 14.2.4.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4387  Cat: F (Rel-17)

					Source: ZTE Corporation

Abstract:

Editorial

Discussion:

r1

Decision: 		The document was revised to R5-243518.

R5-243518	Editorial correction for MBS TC of 14.2.4.1

Type: CR		For: Agreement
					38.523-1 v17.6.0	  CR-4387  rev 1 Cat: F (Rel-17)

					Source: ZTE Corporation

(Replaces R5-242596)

Decision: 		The document was agreed.

#### 6.4.5	TS 38.523-2

R5-242174	Editorial update of applicability for TC 8.2.4.2.1.x

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0456  Cat: F (Rel-17)

					Source: MediaTek Inc.

Abstract:

Editorial

Discussion:

r1

Decision: 		The document was revised to R5-243519.

R5-243519	Editorial update of applicability for TC 8.2.4.2.1.x

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0456  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242174)

Decision: 		The document was agreed.

R5-242175	Update applicability of SOR-CMCI TC

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0457  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242279	Addition to legacy test cases applicability for RedCap UE

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0459  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

r2

Decision: 		The document was revised to R5-243595.

R5-243595	Addition to legacy test cases applicability for RedCap UE

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0459  rev 1 Cat: F (Rel-17)

					Source: MediaTek Inc.

(Replaces R5-242279)

Decision: 		The document was agreed.

R5-242330	Updates of clause 4

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0460  Cat: F (Rel-17)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242545	Editorial changes to Applicability tables of 38.523-2

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0461  Cat: F (Rel-17)

					Source: Qualcomm Technologies Ireland

Discussion:

3GU WIC

This CR shall be implemented first!

Decision: 		The document was agreed.

R5-242564	Addition of applicability of E-UTRA to NR MPS priority Indication test case

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0463  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

R-17

Decision: 		The document was revised to R5-243596.

R5-243596	Addition of applicability of E-UTRA to NR MPS priority Indication test case

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0463  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242564)

Decision: 		The document was agreed.

R5-242569	Corrections to applicability of NR supplemental uplink test case

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0464  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

R-17

r1

Decision: 		The document was revised to R5-243520.

R5-243520	Corrections to applicability of NR supplemental uplink test case

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0464  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242569)

Decision: 		The document was agreed.

R5-242570	Corrections to applicability of Network Slice Admision Control mobility management aspects test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0465  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

R-17! ()

Decision: 		The document was revised to R5-243521.

R5-243521	Corrections to applicability of Network Slice Admision Control mobility management aspects test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0465  rev 1 Cat: F (Rel-17)

					Source: Qualcomm Incorporated

(Replaces R5-242570)

Decision: 		The document was agreed.

R5-242662	Update of test applicability of NSAC test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0466  Cat: F (Rel-17)

					Source: CATT

Discussion:

r2

Decision: 		The document was revised to R5-243552.

R5-243552	Update of test applicability of NSAC test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0466  rev 1 Cat: F (Rel-17)

					Source: CATT

(Replaces R5-242662)

Decision: 		The document was agreed.

R5-242814	Update applicability of MAC TC 7.1.1.1.4-Beam Failure

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0471  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R5-242830	Update test applicability for MUSIM TC

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0472  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r4

Decision: 		The document was revised to R5-243959.

R5-243959	Update test applicability for MUSIM TC

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0472  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242830)

Decision: 		The document was agreed.

R5-242834	Add test applicability for new RedCap HD-FDD TC

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0473  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

r3

Decision: 		The document was revised to R5-243586.

R5-243586	Add test applicability for new RedCap HD-FDD TC

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0473  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R5-242834)

Decision: 		The document was agreed.

R5-242938	Applicability updates for P-CSCF restoration test cases

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0475  Cat: F (Rel-17)

					Source: Qualcomm Incorporated, Orange

Decision: 		The document was agreed.

R5-243078	Update of applicability for test cases 8.1.6.1.2.14

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0476  Cat: F (Rel-17)

					Source: CMCC, CATT, Qualcomm

Discussion:

r3

Decision: 		The document was revised to R5-243584.

R5-243584	Update of applicability for test cases 8.1.6.1.2.14

Type: CR		For: Agreement
					38.523-2 v17.6.1	  CR-0476  rev 1 Cat: F (Rel-17)

					Source: CMCC, CATT, Qualcomm

(Replaces R5-243078)

Decision: 		The document was agreed.

#### 6.4.6	TS 38.523-3

R5-242176	Update case title of 8.1.1.2.4 and 8.1.5.9.1

Type: CR		For: Agreement
					38.523-3 v17.10.0	  CR-3428  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242331	Routine maintenance for TS 38.523-3

Type: CR		For: Agreement
					38.523-3 v17.10.0	  CR-3435  Cat: F (Rel-17)

					Source: MCC TF160

Discussion:

r1

Decision: 		The document was revised to R5-243967.

R5-243967	Routine maintenance for TS 38.523-3

Type: CR		For: Agreement
					38.523-3 v17.10.0	  CR-3435  rev 1 Cat: F (Rel-17)

					Source: MCC TF160

(Replaces R5-242331)

Decision: 		The document was agreed.

#### 6.4.7	Discussion Papers, Work Plan, TC lists

R5-242177	Discussion on handling of legacy Rel-16/17 WIs for RedCap UE

Type: discussion		For: Endorsement
					Source: MediaTek Inc.

Abstract:

At RAN5 #102, there were two discussion papers [1][2] noted with following proposals:

R5-240865 [1]:

Proposal 1: Create a new sub-clause 4.3.2 and add an applicability table for R16+, and keep R15 as it is.

Proposal 2: Create a new sub-clause 4.3.2 and add an applicability table for R16+, and keep R15 aligned with R16+ to add all R15 RedCap UE applicable TCs to the table.

The meeting accepted the proposed solution to add a dedicated list of test cases (from Rel-16 onwards) but how to implement needs to be further studied. Any implementation must not impact TTCN reference implementation.

R5-241572 [2]:

Proposal 1:	Contributors can bring a discussion paper with the prose and applicability update for WI with industrial interest.

The meeting accepted the proposal for any update to the spec being contribution driven based on company/industry interest.

2.	Discussion

Based on above meeting conclusions, in this discussion paper below work items with industrial interest indicated are reviewed:

1) R16 NR_UE_pow_sav-UEConTest;

2) R16 NR_L1enh_URLLC-UEConTest;

3) R16 NR_2step_RACH-UEConTest;

4) R17 NR_cov_enh-UEConTest

5) R17 NR_UE_pow_sav_enh_plus_CT-UEConTest

6) R17 NR_SmallData_INACTIVE-UEConTest

The cases which need prose updates are submitted in below CR list during this meeting:

R5-242274	Update 2-step RACH TC 7.1.1.1.8 for HD-FDD UE-PRACH

R5-242275	Update 2-step RACH TC 7.1.1.1.9 for HD-FDD UE-PRACH

R5-242276	Update URLLC TC 7.1.1.4.1.5 to test RedCap UE

R5-242277	Update URLLC TC 7.1.1.4.2.6 to test RedCap UE

R5-242278	Update coverage enhancement TC 7.1.1.4.2.7 to test RedCap UE

To have a uniform applicability table with other vertical UEs, it`s suggested to adopt the proposal 2 of [1], that is creating a new sub-clause 4.3.2 and adding an applicability table for R16+, and keeping R15 aligned with R16+ to add all R15 RedCap UE applicable TCs to the table. Therefore, below CR is submitted to add new clause 4.3.2:

R5-242279	Addition to legacy test cases applicability for RedCap UE	38.523-2

3.	Proposal

Proposal:	RAN5 to accept the handling of legacy Rel-16/17 WIs for RedCap UE in TS 38.523-1/-2.

Decision: 		The document was noted.

R5-242568	Tracking Rel-15 5GS EN-DC and SA NR Maintenance Workplan for protocol testing

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

After completion of Rel-15 5GS, The Rel-15 5GS maintenance WP updates were made as post meeting document to capture open issues in Signalling EN-DC and SA NR. In the past few meetings, majority of SIG open issues have resolved, and the remaining open issues can be tracked separately, as no further progress can be made until open items are resolved.

Discussion:

r1

stop tracking from now on.

TF160 manager: Editor's note not needed for the last 4 test cases.

Decision: 		The document was revised to R5-243488.

R5-243488	Tracking Rel-15 5GS EN-DC and SA NR Maintenance Workplan for protocol testing

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

(Replaces R5-242568)

Decision: 		The document was noted.

R5-242743	TS 38.523-1 Tracker status before RAN5-103

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-242831	Discussion to add new RedCap HD-FDD TC

Type: discussion		For: Endorsement
					Source: Huawei, HiSilicon

Discussion:

r1

noted, the proposal is accepted.

CRs will be reviewed in the normal process.

Decision: 		The document was revised to R5-243484.

R5-243484	Discussion to add new RedCap HD-FDD TC

Type: discussion		For: Endorsement
					Source: Huawei, HiSilicon

(Replaces R5-242831)

Decision: 		The document was noted.

R5-242942	Discussion Paper for Rel-17 eNS_Ph2 WI NSAC Session management tests case applicability

Type: discussion		For: Discussion
					Source: CATT

Abstract:

The Rel-17 work item on Enhancement of Network Slicing Phase 2 was introduced for the purpose of Network Slice Admission Control (NSAC), including the following UE requirements:

-	Restriction on the maximum number of UEs per network slice with a proper cause and a backoff timer.

-	Restriction on the maximum number of PDU sessions per network slice with a proper cause and a backoff timer.

-	Support of subscription-based mechanism ensuring that a UE is only allowed to be registered with compatible network slices.

In R5-241479, Analysis on NSAC / Session management test cases shows:

Observation#2 - None of the SM test case introduced are testing unique UE requirement specific to R-17 eNS_Ph2 WI. This may cause even earlier UE release to PASS this test case but more importantly the UE requirements are not part of R-17 eNS_Ph2 WI

2.	Discussion

Observation: Most updates for R-17 eNS_Ph2 WI on Session management aspects are enhanced based on the existing rejection cause values: #67 "insufficient resources for specific slice and DNN", #67 "insufficient resources for specific slice and DNN", #69 "insufficient resources for specific slice" and Back-off timer…which have already been defined since Rel-15. It is hard to describe the scenario for conformance testing on enhancement of NSAC along. The issue observed in R5-241479 exists.

3.	Proposals

Proposal#1：Update the applicability of test cases in 10.1.8 NSAC / Session management aspects from Rel-17 to Rel-15, related WI code from eNS_Ph2-UEConTest to 5GS_NR_LTE-UEConTest.

Proposal#2：A new Pics based on UE General Capability of supporting NSAC will be defined to limit the test cases to be applied only after this feature is commercialized along with network IOT.

Discussion:

r1

Notal, proposal accepted.

Decision: 		The document was revised to R5-243487.

R5-243487	Discussion Paper for Rel-17 eNS_Ph2 WI NSAC Session management tests case applicability

Type: discussion		For: Discussion
					Source: CATT

(Replaces R5-242942)

Decision: 		The document was noted.

R5-243074	Disc on handling of MDT IDC problem

Type: discussion		For: Endorsement
					Source: CMCC, CATT, Qualcomm

Abstract:

Observation 0: To complete TC 8.1.6.1.2.14, it is still need to confirm UE will detect IDC problem with “T1” configuration.

Observation 1.1: By the agreed CR R5-241635 for TS 38.523-1, the pre-test condition of TC 8.1.6.1.2.14 was set as shown in the picture above. “Victim(DL)” B40 and “Aggressor(UL)” 2.4GHz WiFi have been set as close to each other as possible. Only 1MHz of isolation will lead to inevitable interference from “Aggressor: Channel 1” to “Victim: NR Cell 14/NRf4”.

Observation 1.2: Comparing to the idle “Victim(DL)”, there is no changes happened to the connected “Victim(DL)” considering they suffer the same interference from the unchanged “Aggressor”(the same working frequency and the same power level). If the connected “Victim(DL)” suffers the IDC problem, the idle “Victim(DL)” will also suffer the same IDC problem under the same pre-test conditions.

Proposal 1: When checking whether UE will suspends measurement logging when IDC problem is detected, if UE detects IDC problem in RRC_CONNECTED with some pre-test condition, to configure the IDLE UE with the same pre-test condition as in RRC_CONNECTED to enable the UE detect IDC problem in RRC_IDLE also.

Observation 2.2: Whether UE will initiate the UAI procedure is out of the scope of the TP(Test Purpose) of TC 8.1.6.1.2.14.

Observation 2.3: “UE to initiate the UAI procedure” is the pre-condition to check whether UE detects IDC problem. Once UE initiates the UAI procedure, we can confirm that UE detects IDC problem if UE initiates transmission of the UAI message. Similarly, we can also confirm that UE does NOT detect IDC problem if UE does NOT initiate transmission of the UAI message.

Proposal 2: RAN5 to define a new test mode as “A UE capable of providing IDC assistance information in RRC_CONNECTED will initiate the procedure” if it was configured to do so.

Proposal 3: Instead of Propsal 2 above, RAN5 to define a new PICS “PC_IDC_Report” as “Support providing UEAssistanceInformation message with IDC-Assistance-r16 in RRC_CONNECTED when an IDC problem is detected” if it was configured to do so.

Discussion:

discussion with Anritsu about the interference, whether it is generated on the SS side. This was denied, CMCC claimed it is generated in the UE, depending on the filter.

Another discussion about the UE receiving/detecting the IDC in Idle mode.

Question whether the SS vendors have to set a pics.

Conclusion: will be leaving to the UE vendor how the test case is defined.

Proposal 3 is agreed. "RRC Idle".

Decision: 		The document was revised to R5-243486.

R5-243486	Disc on handling of MDT IDC problem

Type: discussion		For: Endorsement
					Source: CMCC, CATT, Qualcomm

(Replaces R5-243074)

Decision: 		The document was noted.

R5-243117	5GS EPS fallback test coverage

Type: discussion		For: Endorsement
					Source: Ericsson

Abstract:

The EPS fallback is part of NG.114 [1] clause 4.8. Interworking with E-UTRAN connected to EPC.

2	Discussion

The EPS fallback test cases are defined in TS 38.523-1 [2] clause 11.1 5GS / EPS Fallback. The applicability defined in TS 38.523-2 [3] refers to IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS").

The EPS fallback is part of NG.114 [1] clause 4.8.as “The UE and the network must support the EPS fallback procedure for IMS voice with N26 interface as specified in section 4.13.6.1 of 3GPP TS 23.502 [5], 3GPP TS 38.331 [31] and 3GPP TS 38.413 [30].”

Observation 2-1:

The applicability for many EPS fallback test cases refers to IR.92, i.e., E-UTRA.

Proposal 2-1:

It’s proposed to align the EPS fallback test case applicability as NG.114.

Proposal 2-2:

It’s proposed to update and correct the EPS fallback test cases to be aligned with NG.114 requirements.

3	Proposal

It’s proposed to implement the proposals 2-1 and 2-2.

Discussion:

r1

Proposals are accepted but limited to EPS fallback N26.

Decision: 		The document was revised to R5-243485.

R5-243485	5GS EPS fallback test coverage

Type: discussion		For: Endorsement
					Source: Ericsson

(Replaces R5-243117)

Decision: 		The document was noted.

### 6.5	Routine Maintenance for TS 36 Series TEIx_Test

#### 6.5.1	Routine Maintenance for TS 36.508

R5-242178	Update the note in the satellite ephemeris information

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1469  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242179	Update the SIB31 for GSO and NGSO branches

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1470  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242180	Correction to generic procedure 4.5.4 and 4.5.4A

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1471  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242181	Correction to generic test procedure 4.5A.5

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1472  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242182	Correction to IE PhysicalConfigDedicated

Type: CR		For: Agreement
					36.508 v18.4.0	  CR-1473  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

#### 6.5.2	Routine Maintenance for TS 36.509

#### 6.5.3	Routine Maintenance for TS 36.523-1

##### 6.5.3.1	Idle Mode

R5-242821	Correction of SENSE eMTC TC 6.1.1.12-PLMN selection

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5314  Cat: F (Rel-18)

					Source: Huawei, HiSilicon,TDIA

Decision: 		The document was agreed.

R5-242822	Correction of SENSE eMTC TC 6.1.1.13-PLMN selection Exception

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5315  Cat: F (Rel-18)

					Source: Huawei, HiSilicon,TDIA

Decision: 		The document was agreed.

R5-242823	Correction of SENSE eMTC TC 6.1.1.14-Periodic SENSE

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5316  Cat: F (Rel-18)

					Source: Huawei, HiSilicon,TDIA

Decision: 		The document was agreed.

##### 6.5.3.2	Layer 2

###### 6.5.3.2.1	MAC

###### 6.5.3.2.2	RLC

###### 6.5.3.2.3	PDCP

R5-242183	Correction to PDCP TC 7.3.3.x

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5300  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

R5-242184	Correction to ZUC TC 7.3.3.5, 7.3.3.6 and 7.3.4.3

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5301  Cat: F (Rel-18)

					Source: MediaTek Inc.

Decision: 		The document was agreed.

##### 6.5.3.3	RRC

###### 6.5.3.3.1	RRC Part 1 (clauses 8.1 and 8.5)

R5-242121	Correction of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5299  Cat: F (Rel-18)

					Source: Vodafone GmbH, MCC TF 160

Discussion:

sub-AI!

r1

Decision: 		The document was revised to R5-243468.

R5-243468	Correction of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5299  rev 1 Cat: F (Rel-18)

					Source: Vodafone GmbH, MCC TF 160

(Replaces R5-242121)

Decision: 		The document was agreed.

R5-242333	Updates to E-UTRA RRC test case 8.1.2.15

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5304  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

###### 6.5.3.3.2	RRC Part 2 (clause 8.2),

###### 6.5.3.3.3	RRC Part 3 (clause 8.3)

###### 6.5.3.3.4	Inter-RAT (clauses 8.4 & 8.4A)

###### 6.5.3.3.5	RRC LTE MDT (clause 8.6)

###### 6.5.3.3.6	RRC ANR for UTRAN (clause 8.7)

##### 6.5.3.4	EPS Mobility Management

R5-242758	Correction to LTE RACS Test case 9.2.5.1

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5313  Cat: F (Rel-18)

					Source: Anritsu EMEA Ltd

Discussion:

WIC->TEI16

r1

Decision: 		The document was revised to R5-243470.

R5-243470	Correction to LTE RACS Test case 9.2.5.1

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5313  rev 1 Cat: F (Rel-18)

					Source: Anritsu EMEA Ltd

(Replaces R5-242758)

Decision: 		The document was agreed.

R5-242859	Editorial correction for Specific message contents in 9.2.1.1.20

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5320  Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

Decision: 		The document was revised to R5-243471.

R5-243471	Editorial correction for Specific message contents in 9.2.1.1.20

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5320  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

(Replaces R5-242859)

Decision: 		The document was agreed.

R5-242861	Editorial correction for Specific message contents in 9.2.3.1.22

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5321  Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

Discussion:

r1

Decision: 		The document was revised to R5-243472.

R5-243472	Editorial correction for Specific message contents in 9.2.3.1.22

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5321  rev 1 Cat: F (Rel-18)

					Source: NTT DOCOMO, INC.

(Replaces R5-242861)

Decision: 		The document was agreed.

##### 6.5.3.5	EPS Session Management

R5-242940	Addition of EPS P-CSCF restoration test case 10.3.2

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5322  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Orange

Decision: 		The document was agreed.

##### 6.5.3.6	General Tests

R5-242185	Correction to TC 11.2.13

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5302  Cat: F (Rel-18)

					Source: MediaTek Inc., NTT DOCOMO, MCC TF160, ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-242593	Correction to LTE-IRAT test cases 11.2.11

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5310  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

r1

Decision: 		The document was revised to R5-243474.

R5-243474	Correction to LTE-IRAT test cases 11.2.11

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5310  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-242593)

Decision: 		The document was agreed.

##### 6.5.3.7	Interoperability Radio Bearers

##### 6.5.3.8	Multilayer Procedures

R5-242954	Addition of new test case related to SSAC per PLMN 13.5.1.c

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5323  Cat: F (Rel-18)

					Source: NTTDOCOMO, INC.

Discussion:

date, Rel-17!

r1

Rohde&Schwarz commented about step 3, this will make a conformant UE fail.

r2

Decision: 		The document was revised to R5-243476.

R5-243476	Addition of new test case related to SSAC per PLMN 13.5.1.c

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5323  rev 1 Cat: F (Rel-18)

					Source: NTTDOCOMO, INC.

(Replaces R5-242954)

Decision: 		The document was agreed.

##### 6.5.3.9	PWS - ETWS, CMAS

##### 6.5.3.10	Non-3GPP

##### 6.5.3.11	Others (TS 36.523-1 clauses not covered by other AIs under AI 6.5.3, e.g. eMBMS, Home (e)NB, MBMS in LTE, D2D, SC-PTM, NB-IoT, CIoT...)

R5-242332	Updates to NB-IoT RRC test case 22.4.26

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5303  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242470	Correction to NBIOT testcase 22.5.21

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5306  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

r1

TC->?

Keysight UK needed more time.

r2

Decision: 		The document was revised to R5-243477.

R5-243477	Correction to NBIOT testcase 22.5.21

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5306  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-242470)

Decision: 		The document was agreed.

R5-242486	Correction to NBIOT testcase 22.5.1

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5307  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-242524	Correction to NBIOT testcase 22.3.1.2

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5308  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-242526	Correction to NTN test cases 22.1.2, 22.2.13, 22.4.30 and 22.5.23

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5309  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Discussion:

r2

title???

Decision: 		The document was revised to R5-243478.

R5-243478	Correction to NTN test cases 22.1.2, 22.2.13, 22.4.30 and 22.5.23

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5309  rev 1 Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

(Replaces R5-242526)

Discussion:

reissued as R5-243479 because of title change

Decision: 		The document was withdrawn.

R5-243479	Correction to NTN test cases

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5324  Cat: F (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

reissued from R5-243478 because of title change

Decision: 		The document was agreed.

R5-242720	Correction to NTN TA report test case 22.3.1.13

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5311  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz

Discussion:

fix summary of change

Decision: 		The document was revised to R5-243480.

R5-243480	Correction to NTN TA report test case 22.3.1.13

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5311  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Rohde & Schwarz

(Replaces R5-242720)

Discussion:

first agreed, then deferred to include more changes.

TF160: suggested to update PRD15 in order to clarify the usage of the "This CR's revision history:" field.

Decision: 		The document was agreed.

R5-242824	Correction of SENSE NB-IoT TC 22.2.14-PLMN selection

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5317  Cat: F (Rel-18)

					Source: Huawei, HiSilicon,TDIA

Decision: 		The document was agreed.

R5-242825	Correction of SENSE NB-IoT TC 22.2.15-PLMN selection Exception

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5318  Cat: F (Rel-18)

					Source: Huawei, HiSilicon,TDIA

Decision: 		The document was agreed.

R5-242826	Correction of SENSE NB-IoT TC 22.2.16-Periodic SENSE

Type: CR		For: Agreement
					36.523-1 v18.4.0	  CR-5319  Cat: F (Rel-18)

					Source: Huawei, HiSilicon,TDIA

Decision: 		The document was agreed.

#### 6.5.4	Routine Maintenance for TS 36.523-2

R5-242122	Addition of applicability of new test case 8.1.3.6b for redir-policy bit

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1447  Cat: F (Rel-18)

					Source: Vodafone GmbH, MCC TF160

Discussion:

sub-AI!

r1

Decision: 		The document was revised to R5-243469.

R5-243469	Addition of applicability of new test case 8.1.3.6b for redir-policy bit

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1447  rev 1 Cat: F (Rel-18)

					Source: Vodafone GmbH, MCC TF160

(Replaces R5-242122)

Decision: 		The document was agreed.

R5-242186	Correction to case title of TC 11.2.13

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1448  Cat: F (Rel-18)

					Source: MediaTek Inc., NTT DOCOMO, MCC TF160, ROHDE & SCHWARZ

Decision: 		The document was agreed.

R5-242187	Optimize the tables for IoT NTN

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1449  Cat: F (Rel-18)

					Source: MediaTek Inc., Anritsu

Discussion:

TF160 requested to keep the voided paragraph.

Deferred.

r1

Decision: 		The document was revised to R5-243578.

R5-243578	Optimize the tables for IoT NTN

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1449  rev 1 Cat: F (Rel-18)

					Source: MediaTek Inc., Anritsu

(Replaces R5-242187)

Decision: 		The document was agreed.

R5-242941	Applicability updates for EPS P-CSCF restoration test cases

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1451  Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Orange

Decision: 		The document was revised to R5-243473.

R5-243473	Applicability updates for EPS P-CSCF restoration test cases

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1451  rev 1 Cat: F (Rel-18)

					Source: Qualcomm Incorporated, Orange

(Replaces R5-242941)

Decision: 		The document was agreed.

R5-242955	Addition of applicability for new test case related to SSAC per PLMN 13.5.1.c

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1452  Cat: F (Rel-18)

					Source: NTTDOCOMO, INC.

Discussion:

date, Rel-17!

r2

RAN5 Chair: it may be helpful to prepare a WP. Coordinate with TF160.

Decision: 		The document was revised to R5-243577.

R5-243577	Addition of applicability for new test case related to SSAC per PLMN 13.5.1.c

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1452  rev 1 Cat: F (Rel-18)

					Source: NTTDOCOMO, INC.

(Replaces R5-242955)

Decision: 		The document was agreed.

R5-243182	Update of applicability for test case 6.1.2.9

Type: CR		For: Agreement
					36.523-2 v18.4.0	  CR-1454  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was agreed.

#### 6.5.5	Routine Maintenance for TS 36.523-3

R5-242334	NTN-IoT: Addition of NGSO NB-IoT NTN Test Model

Type: CR		For: Agreement
					36.523-3 v18.3.0	  CR-4821  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

R5-242335	Routine maintenance for TS 36.523-3

Type: CR		For: Agreement
					36.523-3 v18.3.0	  CR-4822  Cat: F (Rel-18)

					Source: MCC TF160

Decision: 		The document was agreed.

#### 6.5.6	Discussion Papers, Work Plan, TC lists

R5-242744	TS 36.523-1 Tracker status before RAN5-103

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-242949	Proposal for new test case related to ACB per PLMN

Type: discussion		For: Endorsement
					36.523-1 v..
					Source: NTTDOCOMO, INC.

Abstract:

Japanese network operator has a plan to introcduce an Intercarrier Roaming in Emergency in JAPAN.

Please see as follows. This is published by Japanese Ministry of Internal Affairs and Communications.

000883579.pdf (soumu.go.jp)

Figure 1 : Image of intercarrier roaming in an emergency

As to handling of this Intercarrier Roaming, Japan will use following 3GPP features to add the affected user to Relieving Carrier.

-cell reserved for operator use

-Access Control per PLMN

We introduced the test cases related to cell reserved for operator use.

According to current TS36.523-1[1], there are 9 test cases related to access control without checking ac-BarringPerPLMN-List.

==

MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call

MTSI MO speech call / SSAC in Connected mode / 0% access probability for MTSI MO speech call

MTSI MO video call / SSAC / 0% access probability for MTSI MO video call

MTSI MO video call / SSAC in Connected mode / 0% access probability for MTSI MO video call

Emergency call / Success / SSAC / 0% access probability for MTSI MO speech call

Emergency call / Success / SSAC in Connected mode / 0% access probability for MTSI MO speech call

MTSI MO speech call / SCM / 0% access probability skip for MTSI MO speech cal

MTSI MO video call / SCM / 0% access probability skip for MTSI MO video call

MTSI MO SMS / SCM / 0% access probability skip for MTSI MO SMS over IP

==

So we would like to introduce test cases related to ac-BarringPerPLMN-List for stability of connectivity.

2	Proposals:

I have a proposal to add new test cases related to ac-BarringPerPLMN-List.

Proposal: RAN5 should add a new test case to check access control due to ac-BarringPerPLMN-List.

This test case is applied to Ues supporting E-UTRA.

Decision: 		The document was revised to R5-243467.

R5-243467	Proposal for new test case related to ACB per PLMN

Type: discussion		For: Endorsement
					36.523-1 v..
					Source: NTTDOCOMO, INC.

(Replaces R5-242949)

Discussion:

accepted. Estimation is 12 test cases.

Decision: 		The document was noted.

### 6.6	Other Maintenance TEIx_Test

#### 6.6.1	Routine Maintenance for TS 34.108

#### 6.6.2	Routine Maintenance for TS 34.109

#### 6.6.3	Routine Maintenance for TS 34.123

##### 6.6.3.1	TS 34.123-1

##### 6.6.3.2	TS 34.123-2

##### 6.6.3.3	TS 34.123-3

R5-242188	Removal of GEA2 test case from the ATS

Type: CR		For: Agreement
					34.123-3 v17.4.0	  CR-3678  Cat: F (Rel-17)

					Source: MediaTek Inc.

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-243475	Update of cl 8.5.4.7 Inter RAT handover to UTRAN

Type: CR		For: Agreement
					34.123-3 v17.4.0	  CR-3680  Cat: F (Rel-17)

					Source: MCC TF160, Rohde & Schwarz

Discussion:

late doc

block agreed on Thu

CR# 3679->81 bec. TTCn email meeting had assigned meanwhile.

Decision: 		The document was agreed.

#### 6.6.4	Discussion Papers, Work Plan, TC lists

#### 6.6.6	Routine Maintenance for TS 34.229

##### 6.6.6.1	TS 34.229-1

R5-242623	New MO speech EVS test case 12.23a

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1534  Cat: F (Rel-17)

					Source: Ericsson

Discussion:

Offline comments from TF160

r1

meeting decided to add execution giudlines to avoid redundant testing for test cases 12.23 +a.

Decision: 		The document was revised to R5-243523.

R5-243523	New MO speech EVS test case 12.23a

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1534  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-242623)

Discussion:

first agreed, then wrong WIC

Decision: 		The document was revised to R5-243984.

R5-243984	New MO speech EVS test case 12.23a

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1534  rev 2 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-243523)

Decision: 		The document was agreed.

R5-242624	New MT speech EVS test case 12.24a

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1535  Cat: F (Rel-17)

					Source: Ericsson

Discussion:

r1

same as R5-242623.

Decision: 		The document was revised to R5-243524.

R5-243524	New MT speech EVS test case 12.24a

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1535  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-242624)

Discussion:

first agreed, then wrong WIC

Decision: 		The document was revised to R5-243985.

R5-243985	New MT speech EVS test case 12.24a

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1535  rev 2 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-243524)

Decision: 		The document was agreed.

R5-242625	Applicability of new EVS test cases 12.23a and 12.24a

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1536  Cat: F (Rel-17)

					Source: Ericsson

Decision: 		The document was withdrawn.

R5-242737	Updates to Annex A.5.3

Type: CR		For: Agreement
					34.229-1 v17.0.0	  CR-1537  Cat: F (Rel-17)

					Source: Starpoint, TDIA

Decision: 		The document was agreed.

##### 6.6.6.2	TS 34.229-2

R5-243218	Applicability for new test cases 12.23a and 12.24a

Type: CR		For: Agreement
					34.229-2 v17.0.0	  CR-0341  Cat: F (Rel-17)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243961.

R5-243961	Applicability for new test cases 12.23a and 12.24a

Type: CR		For: Agreement
					34.229-2 v17.0.0	  CR-0341  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-243218)

Discussion:

first agreed, then wrong WIC

Decision: 		The document was revised to R5-243986.

R5-243986	Applicability for new test cases 12.23a and 12.24a

Type: CR		For: Agreement
					34.229-2 v17.0.0	  CR-0341  rev 2 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-243961)

Decision: 		The document was agreed.

##### 6.6.6.3	TS 34.229-3

##### 6.6.6.4	TS 34.229-4

##### 6.6.6.5	TS 34.229-5

R5-242336	Correction of IMSo5G test case 7.23

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0580  Cat: F (Rel-17)

					Source: MCC TF160

Discussion:

r1

Decision: 		The document was revised to R5-243525.

R5-243525	Correction of IMSo5G test case 7.23

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0580  rev 1 Cat: F (Rel-17)

					Source: MCC TF160

(Replaces R5-242336)

Decision: 		The document was agreed.

R5-242618	Updates to Annex A.15.2

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0581  Cat: F (Rel-17)

					Source: Ericsson

Discussion:

r1

Decision: 		The document was revised to R5-243526.

R5-243526	Updates to Annex A.15.2

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0581  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R5-242618)

Decision: 		The document was agreed.

R5-242619	Updates to Annex A.15.2a

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0582  Cat: F (Rel-17)

					Source: Ericsson

Decision: 		The document was agreed.

R5-242620	Updates to Annex A.16.2a

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0583  Cat: F (Rel-17)

					Source: Ericsson

ACTION:	review and address inconsistencies in the application of conditons in TS 34.229-5
	(action on: Ericsson, TF160, Qualcomm / due by: 2024-08-10)

Decision: 		The document was withdrawn.

R5-242621	Updates to test case 7.4a

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0584  Cat: F (Rel-17)

					Source: Ericsson, Rohde & Schwarz

Discussion:

r1

Deferred for comments from TF160.

Orange mentioned also 24a.

Decision: 		The document was revised to R5-243527.

R5-243527	Updates to test case 7.4a

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0584  rev 1 Cat: F (Rel-17)

					Source: Ericsson, Rohde & Schwarz

(Replaces R5-242621)

Decision: 		The document was agreed.

R5-242622	Updates to test case 7.6a

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0585  Cat: F (Rel-17)

					Source: Ericsson, Rohde & Schwarz

Discussion:

r1

Decision: 		The document was revised to R5-243528.

R5-243528	Updates to test case 7.6a

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0585  rev 1 Cat: F (Rel-17)

					Source: Ericsson, Rohde & Schwarz

(Replaces R5-242622)

Decision: 		The document was agreed.

R5-242742	Correction to NR IMS TC 7.24b - Forking

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0586  Cat: F (Rel-17)

					Source: Huawei, Hisilicon

Discussion:

R5!

r1

TF160 requested more time.

Decision: 		The document was revised to R5-243529.

R5-243529	Correction to NR IMS TC 7.24b - Forking

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0586  rev 1 Cat: F (Rel-17)

					Source: Huawei, Hisilicon

(Replaces R5-242742)

Discussion:

for email agreement

Email withdrawn

Decision: 		The document was withdrawn.

R5-242749	Correction to generic procedure A.16.1a

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0587  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242750	Correction to VONR test case 7.10

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0588  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242751	Correction to VONR test case 7.13

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0589  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242752	Correction to test case 7.16

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0590  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242753	Correction to VONR test case 7.17

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0591  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242754	Correction to VONR test case 7.25

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0592  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242755	Correction to test case 8.37

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0593  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-242756	Correction to test case 8.38

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0594  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was agreed.

R5-243094	Correction to NR IMS testcases 7.4a and 7.6a

Type: CR		For: Agreement
					34.229-5 v17.0.0	  CR-0595  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Discussion:

merged into a revision of R5-242621-2.

Decision: 		The document was withdrawn.

##### 6.6.6.6	Discussion Papers, Work Plan, TC lists

#### 6.6.7	Routine Maintenance for TS 37.571

##### 6.6.7.1	TS 37.571-2

R5-242950	Correction to LPP positioning test case 9.3.4.2

Type: CR		For: Agreement
					37.571-2 v17.3.0	  CR-0183  Cat: F (Rel-17)

					Source: NTT DOCOMO Inc., MCC TF160

Discussion:

date!

r1

Decision: 		The document was revised to R5-243569.

R5-243569	Correction to LPP positioning test case 9.3.4.2

Type: CR		For: Agreement
					37.571-2 v17.3.0	  CR-0183  rev 1 Cat: F (Rel-17)

					Source: NTT DOCOMO Inc., MCC TF160

(Replaces R5-242950)

Decision: 		The document was agreed.

##### 6.6.7.2	TS 37.571-3

##### 6.6.7.3	TS 37.571-4

##### 6.6.7.4	TS 37.571-5

##### 6.6.7.5	Discussion Papers, Work Plan, TC lists

#### 6.6.8	Routine Maintenance for TS 51.010

##### 6.6.8.1	TS 51.010-1 (Signalling)

##### 6.6.8.2	TS 51.010-2 (Signalling)

##### 6.6.8.3	TS 51.010-5 (Signalling)

##### 6.6.8.4	TS 51.010-7 (Signalling)

##### 6.6.8.5	Discussion Papers, Work Plan, TC list & CR summary

#### 6.6.9	Routine Maintenance for TS 36.579

##### 6.6.9.1	TS 36.579-1

R5-242337	Clarification of initial conditions and RRC/NAS signalling in clause 5.3

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0344  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242338	Clarification of initial conditions and RRC/NAS signalling in clause 5.3A

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0345  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242339	Clarification of initial conditions and RRC/NAS signalling in clause 5.3B

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0346  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242340	Clarification of initial conditions and RRC/NAS signalling in clause 5.3C

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0347  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242341	Clarifying the System Under Test

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0348  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242342	Corrections of clause 2

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0349  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242343	Corrections of clause 5.5.2.15.2

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0350  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242344	Corrections of clause 5.5.2.2.2

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0351  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242345	Corrections of clause 5.5.2.4

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0352  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242346	Corrections of clause 5.5.2.5.2

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0353  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242347	Corrections of clause 5.5.2.7.1

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0354  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242348	Editorial corrections of clauses 1 and 4

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0355  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242349	Improvement of clause 5.2

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0356  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242350	Improvement of clause 5.4

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0357  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242398	Correction of URI scheme in HTTP POST

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0358  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242400	Corrections of clause 5.5.9.1

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0359  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-243273	Correction to clause 5.5.3.4.2

Type: CR		For: Agreement
					36.579-1 v16.4.0	  CR-0360  Cat: F (Rel-16)

					Source: NIST

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.6.9.2	TS 36.579-2

R5-242351	Common correction of test cases in clauses 5 and 6

Type: CR		For: Agreement
					36.579-2 v16.4.0	  CR-0367  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242352	Correction of test case 6.1.1.15

Type: CR		For: Agreement
					36.579-2 v16.4.0	  CR-0368  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242353	Correction of test case 6.1.1.16

Type: CR		For: Agreement
					36.579-2 v16.4.0	  CR-0369  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242354	Correction of test case 6.3.2

Type: CR		For: Agreement
					36.579-2 v16.4.0	  CR-0370  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.6.9.3	TS 36.579-3

##### 6.6.9.4	TS 36.579-4

R5-242355	Misc updates

Type: CR		For: Agreement
					36.579-4 v16.4.0	  CR-0034  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.6.9.5	TS 36.579-5

R5-242399	Routine maintenance for TS 36.579-5

Type: CR		For: Agreement
					36.579-5 v17.5.0	  CR-0111  Cat: F (Rel-17)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.6.9.6	TS 36.579-6

R5-242356	Common correction of test cases in clause 6

Type: CR		For: Agreement
					36.579-6 v16.4.0	  CR-0108  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242357	Correction of testcase 6.3.1

Type: CR		For: Agreement
					36.579-6 v16.4.0	  CR-0109  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

R5-242358	Correction of testcase 6.3.2

Type: CR		For: Agreement
					36.579-6 v16.4.0	  CR-0110  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.6.9.7	TS 36.579-7

R5-242359	Common correction of test cases in clauses 5 and 6

Type: CR		For: Agreement
					36.579-7 v16.3.0	  CR-0058  Cat: F (Rel-16)

					Source: MCC TF160

Discussion:

block agreed on Thu

Decision: 		The document was agreed.

##### 6.6.9.8	Other Specs

##### 6.6.9.9	Discussion Papers, Work Plan, TC lists

### 6.7	Outgoing liaison statements for provisional approval

### 6.8	AOB

## 7	Closing Joint Session

### 7.1	Agenda for closing session

R5-242102	Agenda - closing session

Type: agenda		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

### 7.2	Pointer CRs

R5-242291	Removal of technical content in TS 34.229-1 v15.10.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					34.229-1 v15.10.0	  CR-1532  Cat: F (Rel-15)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242292	Removal of technical content in TS 34.229-1 v16.7.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					34.229-1 v16.7.0	  CR-1533  Cat: F (Rel-16)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242293	Removal of technical content in TS 34.229-2 v16.10.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					34.229-2 v16.10.0	  CR-0340  Cat: F (Rel-16)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242294	Removal of technical content in TS 34.229-3 v16.8.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					34.229-3 v16.8.0	  CR-0947  Cat: F (Rel-16)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242295	Removal of technical content in TS 34.229-5 v16.10.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					34.229-5 v16.10.0	  CR-0579  Cat: F (Rel-16)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242296	Removal of technical content in TS 36.521-3 v17.1.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					36.521-3 v17.1.0	  CR-3023  Cat: F (Rel-17)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242297	Removal of technical content in TS 36.523-3 v17.7.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					36.523-3 v17.7.0	  CR-4819  Cat: F (Rel-17)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242298	Removal of technical content in TS 36.579-3 v13.3.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					36.579-3 v13.3.0	  CR-0010  Cat: F (Rel-13)

					Source: ETSI Secretariat

Abstract:

block agreement

Decision: 		The document was agreed.

R5-242299	Removal of technical content in TS 36.579-3 v14.0.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					36.579-3 v14.0.0	  CR-0011  Cat: F (Rel-14)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242300	Removal of technical content in TS 36.579-5 v15.1.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					36.579-5 v15.1.0	  CR-0109  Cat: F (Rel-15)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242301	Removal of technical content in TS 36.579-5 v16.0.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					36.579-5 v16.0.0	  CR-0110  Cat: F (Rel-16)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242302	Removal of technical content in TS 36.579-7 v15.5.1 and substitution with pointer to the next Release

Type: CR		For: Agreement
					36.579-7 v15.5.1	  CR-0057  Cat: F (Rel-15)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242303	Removal of technical content in TS 37.571-3 v16.14.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					37.571-3 v16.14.0	  CR-0168  Cat: F (Rel-16)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242304	Removal of technical content in TS 37.571-4 v16.8.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					37.571-4 v16.8.0	  CR-0203  Cat: F (Rel-16)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242305	Removal of technical content in TS 38.521-5 v17.0.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					38.521-5 v17.0.0	  CR-0037  Cat: F (Rel-17)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242306	Removal of technical content in TS 38.522 v15.3.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					38.522 v15.3.0	  CR-0407  Cat: F (Rel-15)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

R5-242307	Removal of technical content in TR 38.903 v16.14.0 and substitution with pointer to the next Release

Type: CR		For: Agreement
					38.903 v16.14.0	  CR-0729  Cat: F (Rel-16)

					Source: ETSI Secretariat

Abstract:

block agreement

Discussion:

block agreed on Fri

Decision: 		The document was agreed.

### 7.3	Open Issues

#### 7.3.1	RF group docs still requiring WG verdict/confirmation - original A.I. retained

#### 7.3.2	Sig group docs still requiring WG verdict/confirmation - original A.I. retained

#### 7.3.3	Other open issues from joint sessions - original A.I. retained

#### 7.3.4	Other

### 7.4	iWD/PRD Updates

#### 7.4.1	iWD-003: Record of RAN5 owned test cases not ready for RAN5 agreement or verifiable on one UE only

#### 7.4.2	PRD17: Guidance to using Work Item Codes with RAN5 test cases

R5-242775	PRD-17 on Guidance to Work Item Codes (post RAN#104 version)

Type: other		For: Approval
					Source: Bureau Veritas ADT (Rapporteur)

Abstract:

Post-meeting

Decision: 		The document was email approved.

#### 7.4.3	PRD20: Status updates E-UTRA CA

R5-242778	PRD20 Completed CA_2A-2A-29A-30A-66A_5DL-1UL without UL CA

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was not treated.

R5-242779	PRD20 Completed CA_2A-2A-29A-66A-66A_5DL-1UL without UL CA

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was not treated.

R5-242780	PRD20 Completed WP_CA_2A-29A-66A-66A_CA_2A-2A-29A-66A_4DL-1UL_DL only FBs

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was not treated.

R5-242781	PRD20 Completed CA_30A-48A 2DL-2UL with UL CA_30A-48A

Type: other		For: Information
					Source: Bureau Veritas ADT, Sporton International

Abstract:

Post-meeting

Decision: 		The document was not treated.

R5-243282	PRD20 on E-UTRA CA configuration handling in RAN5 v1.7.0

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

5.6. 5pm

Decision: 		The document was approved.

#### 7.4.4	PRD21: Status Updates and Completion Declaration Statements (CDS) for NR bands, NR band CBW extensions, 5G NR CADC configurations for all Power Classes

R5-242361	PRD21 CDS: NR FR1 FDD band n85

Type: WI summary		For: Information
					Source: Nokia

Decision: 		The document was noted.

R5-242367	PRD21 CDS: NR FR1 FDD band n106

Type: WI summary		For: Information
					Source: Nokia

Decision: 		The document was agreed.

R5-242436	PRD21 CDS: PC2 EN-DC FR1 3 bands and 4 bands

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was not treated.

R5-242444	PRD21 CDS: NRDC combos between FR1 and FR2

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was not treated.

R5-242445	PRD21 CDS PC3 NRCA 2 bands combos

Type: other		For: Information
					Source: KDDI Corporation

Decision: 		The document was approved.

R5-242447	PRD21 CDS PC3 NRCA CA_n3A-n28A-n77A

Type: other		For: Information
					Source: KDDI Corporation

Decision: 		The document was approved.

R5-242448	PRD21 CDS PC3 NRCA CA_n3A-n28A-n41A-n77A

Type: other		For: Information
					Source: KDDI Corporation

Decision: 		The document was withdrawn.

R5-242453	PRD21 CDS: ENDC combos between FR1 and FR2

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was not treated.

R5-242454	PRD21 CDS PC2 EN-DC 2 bands combos with n41 and n77

Type: other		For: Information
					Source: KDDI Corporation

Decision: 		The document was approved.

R5-242465	PRD21 CDS: PC3 NRCA additional combos FR1

Type: other		For: (not specified)
					Source: Verizon Spain

Decision: 		The document was not treated.

R5-242469	PRD21 CDS: NR band and CBW FR1 n7 and n78 PC1

Type: WI summary		For: Information
					Source: Nokia

Decision: 		The document was noted.

R5-242597	PRD21 CDS PC3 EN-DC DC_18A_n257G

Type: other		For: Information
					Source: KDDI Corporation

Decision: 		The document was approved.

R5-242717	PRD21 CDS: NRCA PC3 8 NR-CA bands

Type: other		For: Approval
					Source: WE Certification Oy, AT&T

Decision: 		The document was approved.

R5-242718	PRD21 CDS: ENDC PC3 6 EN-DC bands

Type: other		For: Approval
					Source: WE Certification Oy, AT&T

Decision: 		The document was approved.

R5-242777	PRD21 CDS NR band TDD FR2 n260 with PC3

Type: other		For: Information
					Source: Bureau Veritas ADT

Decision: 		The document was approved.

R5-242794	PRD21 CDS: PC2 ENDC 3 bands combos with n78 and n79

Type: other		For: Information
					Source: NTT DOCOMO INC.

Discussion:

r1

Decision: 		The document was revised to R5-243988.

R5-243988	PRD21 CDS: PC2 ENDC 3 bands combos with n78 and n79

Type: other		For: Information
					Source: NTT DOCOMO INC.

(Replaces R5-242794)

Decision: 		The document was not treated.

R5-242795	PRD21 CDS: PC3 ENDC 3 bands combos with n78 and n79

Type: other		For: Information
					Source: NTT DOCOMO INC.

Discussion:

r1

Decision: 		The document was revised to R5-243989.

R5-243989	PRD21 CDS: PC3 ENDC 3 bands combos with n78 and n79

Type: other		For: Information
					Source: NTT DOCOMO INC.

(Replaces R5-242795)

Decision: 		The document was not treated.

R5-242996	PRD21 CDS: PC3 NR CA_n78A-n79A with 1UL

Type: other		For: Information
					Source: NTT DOCOMO INC.

Discussion:

r1

Decision: 		The document was revised to R5-243990.

R5-243990	PRD21 CDS: PC3 NR CA_n78A-n79A with 1UL

Type: other		For: Information
					Source: NTT DOCOMO INC.

(Replaces R5-242996)

Decision: 		The document was not treated.

R5-243026	PRD21 CDS: PC2 NR CA_n78A-n79A with 1UL

Type: other		For: Information
					Source: NTT DOCOMO INC.

Discussion:

r1

Decision: 		The document was revised to R5-243991.

R5-243991	PRD21 CDS: PC2 NR CA_n78A-n79A with 1UL

Type: other		For: Information
					Source: NTT DOCOMO INC.

(Replaces R5-243026)

Decision: 		The document was noted.

R5-243033	PRD21 CDS PC3 for 3 bands CA_n1A-n8A-n78A and CA_n3A-n8A-n78A

Type: other		For: (not specified)
					Source: China Unicom

Decision: 		The document was not treated.

R5-243067	PRD21 on NR bands and 5G NR CADC config handling v1.9.0

Type: other		For: Approval
					Source: CMCC

Discussion:

for email approval

Decision: 		The document was email approved.

R5-243069	Disc on handling of PRD21 and PRD20

Type: discussion		For: Endorsement
					Source: CMCC, ZTE, Bureau Veritas, Huawei, Hisilicon

Abstract:

Observation 1.1: Only NR bands and 5G NR CADC configurations should be covered by PRD21.

Observation 1.2: NR NTN satellite operating bands should be covered by PRD21.

Observation 1.3: IoT-NTN satellite operating bands should NOT be covered by PRD21.

Proposal 1: PRD20 to revise the title into “E-UTRA bands and CA configuration handling”, and to trace IoT-NTN bands in PRD20 (word file and excel file). All the IoT-NTN related parts to be removed from PRD21 (word file and excel file) into PRD20. IoT-NTN bands will be handled in an individual sheet in PRD20 excel.

Observation 2: Compared with PRD21, PRD20 has no Columm “Interested deployer” yet.

Proposal 2: Similar as PRD21 excel, to add Column “interested deployer” in PRD20 excel also. Only when there is industry need, the corresponding E-UTRA bands/CA configuration can be tagged as “Ongoing” and related contributions are acceptable in RAN5.

Proposal 3: NR NTN bands to be handled in an individual sheet in PRD21 excel.

Some CA/DC configs which have 3Tx capability, however, are already marked as “completed” status in PRD21 excel. Taking CA_n5A-n77A as an example: CA_n5A-n77A with UL -/CA_n5A-n77A have been completed in PRD21, thus the current status of CA_n5A-n77A in PRD21 excel is not suitable for trace CA_n5A-n77A 3Tx configuration.

Observation 3.1: 3Tx inter-band CA/DC configuration information is not captured in the current PRD21 excel.

Observation 3.2:  PRD21 excel needs to be updated to trace the status of 3Tx inter-band CA/DC configs.

Proposal 4: It is proposed to add new rows in PRD21 excel to trace 3Tx inter-band CA/DC configs.

Discussion:

All proposals are accepted.

Decision: 		The document was noted.

R5-243070	PRD21 CDS: NR band and CBW FR1 n34 PC1.5

Type: other		For: Information
					Source: CMCC

Decision: 		The document was approved.

R5-243071	PRD21 CDS: NR band and CBW FR1 n39 PC1.5

Type: other		For: Information
					Source: CMCC

Decision: 		The document was approved.

R5-243072	PRD21 CDS: NR band and CBW FR1 n40 PC1.5

Type: other		For: Information
					Source: CMCC

Decision: 		The document was approved.

R5-243073	PRD21 CDS: NR band and CBW FR1 n41 PC1

Type: other		For: Information
					Source: CMCC, Nokia, Nokia Shanghai Bell

Discussion:

cannot be completed since TP analysis for A-MPR is not finished yet.

Decision: 		The document was withdrawn.

#### 7.4.5	Other PRD updates

### 7.5	Work Items/ Study Items

#### 7.5.1	Final version of Work Item Proposals

R5-243451	New WID on UE Conformance - Further NR mobility enhancements

Type: WID new		For: Endorsement
					Source: MediaTek Inc., Apple, China Telecom

(Replaces R5-242124)

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243452	New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation

Type: WID new		For: Endorsement
					Source: MediaTek Inc., China Telecom

(Replaces R5-242125)

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243453	New WID on UE Conformance - Introduction of the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) for IoT NTN

Type: WID new		For: Endorsement
					Source: MediaTek Inc., Inmarsat

(Replaces R5-242126)

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243454	New WID on UE Conformance Test Aspects - Expanded and improved NR positioning

Type: WID new		For: Endorsement
					Source: CATT, CAICT

(Replaces R5-242239)

Discussion:

is endorsed

+ 6 months

Decision: 		The document was agreed.

R5-243455	New WID on UE Conformance – NR demodulation performance evolution

Type: WID new		For: Endorsement
					Source: China Telecom, Qualcomm, Samsung

(Replaces R5-242446)

Discussion:

added Ericsson, Orange, Verizon, and NTT DOCOMO as supporting companies.

is endorsed

Decision: 		The document was agreed.

R5-243456	New WID on UE Conformance - Support for UAV (Uncrewed Aerial Vehicles) and Further Architecture Enhancement for UAV and UAM

Type: WID new		For: Information
					Source: Qualcomm Incorporated, Nokia, China Unicom

(Replaces R5-242540)

Discussion:

CMCC asked to support this WI. This was accepted.

Noted.

Decision: 		The document was noted.

R5-243457	New WID on UE Conformance - Mobile Terminated-Small Data Transmission (MT-SDT) for NR

Type: WID new		For: Endorsement
					Source: Qualcomm Incorporated, China Telecom, ZTE

(Replaces R5-242541)

Abstract:

Rel-17 specified MO-SDT to allow small packet transmission for UL-oriented packets. RAN2 has specified the subsequent enhancements under the Rel-18 work item Core part: Mobile Terminated-Small Data Transmission (MT-SDT) for NR, wherein, or DL, MT-SDT (i.e. DL-triggered small data) allows similar benefits, i.e. 1) reducing signalling overhead and UE power consumption by not transitioning to RRC_CONNECTED and reducing latency by allowing fast transmission of (small and infrequent) packets, e.g. for positioning.

Work in RAN2 has been concluded, therefore, it is justified to start work in RAN5.

4	Objective

4.1	Objective of Testing part WI

RAN2 has specified enhancements for following topics under the Rel-18 work item Core part: Mobile Terminated-Small Data Transmission (MT-SDT) for NR:

•	MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RACH procedure based and CG-SDT procedure-based UL response.

•	MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.

The objective for RAN5 is to specify UE conformance specifications for the same.

Discussion:

is endorsed

Coversheet 'source' has to be updated for the plenary.

Lenovo asked to be added.

Decision: 		The document was agreed.

R5-243459	New WID on UE Conformance – Enhanced support of Non-Public Networks Phase 2

Type: WID new		For: Endorsement
					Source: China Telecom, Qualcomm, ZTE

(Replaces R5-242691)

Discussion:

ETSI MCC: meeting header correction

Decision: 		The document was revised to R5-243973.

R5-243973	New WID on UE Conformance – Enhanced support of Non-Public Networks Phase 2

Type: WID new		For: Endorsement
					Source: China Telecom, Qualcomm, ZTE

(Replaces R5-243459)

Discussion:

is endorsed

Decision: 		The document was noted.

R5-243460	New WID on UE Conformance - Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC)

Type: WID new		For: Endorsement
					Source: Apple, ROHDE & SCHWARZ, Vivo

(Replaces R5-242841)

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243461	New WID on UE Conformance - In-Device Co-existence (IDC) enhancements for NR and MR-DC

Type: WID new		For: Endorsement
					Source: CMCC, Xiaomi

(Replaces R5-243079)

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243462	UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs

Type: WID new		For: Endorsement
					Source: Apple, CAICT, Keysight Technologies

(Replaces R5-243222)

Discussion:

is endorsed

Motorola Mobility, Vodafone, and Ericsson were added as supporting companies

Decision: 		The document was agreed.

R5-243463	New WID on UE Conformance  4Rx for NR bands for FWA UEs (up to 2.6GHz) and handheld UEs (1GHz to 2.6GHz)

Type: WID new		For: Endorsement
					Source: ZTE, China Telecom

(Replaces R5-243284)

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243464	New WID on UE Conformance - NR NTN (Non-Terrestrial Networks) enhancements

Type: WID new		For: Endorsement
					Source: Qualcomm Inc, China Unicom, Mediatek

(Replaces R5-243355)

Discussion:

R&S will be added.

Only noted.

Decision: 		The document was noted.

R5-243465	New WID on UE Conformance - Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception

Type: WID new		For: Endorsement
					Source: Qualcomm Inc

(Replaces R5-243356)

Discussion:

R&S asked to be added.

is endorsed

Decision: 		The document was agreed.

R5-243568	New WID on UE Conformance - New bands and bandwidth allocation for LTE based 5G terrestrial broadcast

Type: WID new		For: Information
					Source: Qualcomm Inc

(Replaces R5-242242)

Discussion:

Keysight UK: 38.509->36.509

36.905 -> .904

To be presented at the next meeting.

Decision: 		The document was noted.

#### 7.5.2	Active Work Items/ Study Item: work plans (wp), status reports (sr), Work Item Descriptions (wid)

R5-242114	WI Progress and Target Completion Date Review

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

R5-242189	WP UE Conformance - NR and MR-DC measurement gap enhancements

Type: Work Plan		For: Information
					Source: MediaTek Inc.

Decision: 		The document was noted.

R5-242190	SR UE Conformance - NR and MR-DC measurement gap enhancements

Type: WI status report		For: Information
					Source: MediaTek Inc.

Decision: 		The document was noted.

R5-242237	Work plan: UE Conformance Test Aspects - NR Positioning Enhancement

Type: Work Plan		For: Information
					Source: CATT

Decision: 		The document was noted.

R5-242238	SR UE Conformance Test Aspects - NR Positioning Enhancement

Type: WI status report		For: Information
					Source: CATT

Decision: 		The document was noted.

R5-242376	WP UE Conformance – High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18

Type: Work Plan		For: Information
					Source: Nokia

Decision: 		The document was noted.

R5-242377	SR UE Conformance – High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18

Type: WI status report		For: Information
					Source: Nokia

Decision: 		The document was noted.

R5-242378	Revised WID on UE Conformance - High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18

Type: WID revised		For: Endorsement
					Source: Nokia

Discussion:

CMCC added.

is endorsed.

Decision: 		The document was noted.

R5-242382	WP UE Conformance – Introduction of 900 MHz LTE band in the US

Type: Work Plan		For: Information
					Source: Nokia

Decision: 		The document was noted.

R5-242383	SR UE Conformance – Introduction of 900 MHz LTE band in the US

Type: WI status report		For: Information
					Source: Nokia

Decision: 		The document was noted.

R5-242403	WP UE Conformance - NR QoE management and optimizations for diverse services

Type: Work Plan		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R5-242404	SR UE Conformance - NR QoE management and optimizations for diverse services

Type: WI status report		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R5-242432	WP for UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

Type: Work Plan		For: (not specified)
					Source: China Telecom,Huawei, CATT

Decision: 		The document was noted.

R5-242437	SR UE Conformance Further enhancement on NR demodulation performance RAN5#103

Type: WI status report		For: Discussion
					Source: China Telecom

Decision: 		The document was noted.

R5-242438	Revised WID on UE Conformance Further enhancement on NR demodulation performance

Type: WID revised		For: Endorsement
					Source: China Telecom

Decision: 		The document was withdrawn.

R5-242455	SR for UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

Type: WI status report		For: Information
					Source: China Telecom,Huawei, CATT

Decision: 		The document was noted.

R5-242456	WP for UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS

Type: Work Plan		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-242457	SR for UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS

Type: WI status report		For: Information
					Source: China Telecom,Qualcomm

Decision: 		The document was noted.

R5-242458	WP for UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2

Type: Work Plan		For: Information
					Source: China Telecom,ZTE,MTK

Decision: 		The document was noted.

R5-242459	SR for UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2

Type: WI status report		For: Information
					Source: China Telecom, ZTE, MTK

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243977	Revised WID for UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2

Type: WID revised		For: Endorsement
					Source: China Telecom ,ZTE, MTK

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-242553	SR UE Conformance Test Aspects - Rel-17 Support of Uncrewed Aerial Systems

Type: WI status report		For: Information
					Source: Qualcomm CDMA Technologies

Decision: 		The document was noted.

R5-242554	SR UE Conformance Test Aspects - Rel-17 NR small data transmissions in INACTIVE state

Type: WI status report		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was noted.

R5-242555	WP UE Conformance Test Aspects - Rel-17 Support of Uncrewed Aerial Systems

Type: Work Plan		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was noted.

R5-242556	WP UE Conformance Test Aspects - Rel-17 NR small data transmissions in INACTIVE state

Type: Work Plan		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was noted.

R5-242615	WP UE Conformance – NR support for dedicated spectrum less than 5MHz for FR1

Type: Work Plan		For: Approval
					Source: Nokia

Decision: 		The document was noted.

R5-243974	WP UE Conformance – NR support for dedicated spectrum less than 5MHz for FR1

Type: Work Plan		For: Approval
					Source: Nokia

(Replaces R5-242615)

Decision: 		The document was noted.

R5-242616	SR UE Conformance – NR support for dedicated spectrum less than 5MHz for FR1

Type: WI status report		For: Approval
					Source: Nokia

Decision: 		The document was noted.

R5-242626	WP UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2)

Type: Work Plan		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R5-242627	SR UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2)

Type: WI status report		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R5-242664	WP - UE Conformance - Further enhancements on MIMO for NR

Type: Work Plan		For: Information
					Source: Samsung

Decision: 		The document was noted.

R5-242665	SR - UE Conformance - Further enhancements on MIMO for NR

Type: WI status report		For: Information
					Source: Samsung

Decision: 		The document was noted.

R5-242666	WP - UE Conformance - NR support for high speed train scenario in frequency range 2

Type: Work Plan		For: Information
					Source: Samsung

Decision: 		The document was noted.

R5-242667	SR - UE Conformance - NR support for high speed train scenario in frequency range 2

Type: WI status report		For: Information
					Source: Samsung

Decision: 		The document was noted.

R5-242685	WP UE Conformance – Rel-17 Enhancement of Private Network Support for NG-RAN including CT aspects RAN5#103

Type: Work Plan		For: Information
					Source: China Telecom, Qualcomm

Decision: 		The document was noted.

R5-242686	SR UE Conformance – Rel-17 Enhancement of Private Network Support for NG-RAN including CT aspects RAN5#103

Type: WI status report		For: Information
					Source: China Telecom, Qualcomm

Decision: 		The document was noted.

R5-242687	WP UE Conformance – Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations RAN5#103

Type: Work Plan		For: Information
					Source: China Telecom, Huawei

Decision: 		The document was noted.

R5-242688	SR UE Conformance – Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations RAN5#103

Type: WI status report		For: Information
					Source: China Telecom, Huawei

Decision: 		The document was noted.

R5-242689	WP UE Conformance – Rel-17 LTE CA Configurations RAN5#103

Type: Work Plan		For: Information
					Source: China Telecom, ZTE

Decision: 		The document was noted.

R5-242690	SR UE Conformance – Rel-17 LTE CA Configurations RAN5#103

Type: WI status report		For: Information
					Source: China Telecom, ZTE

Decision: 		The document was noted.

R5-242747	SR of UE Conformance - IMS Data Channel test after RAN5 103

Type: WI status report		For: Information
					Source: Huawei

Decision: 		The document was revised to R5-243993.

R5-243993	SR of UE Conformance - IMS Data Channel test after RAN5 103

Type: WI status report		For: Information
					Source: Huawei

(Replaces R5-242747)

Decision: 		The document was not treated.

R5-242748	WP of UE Conformance - IMS Data Channel test after RAN5 103

Type: Work Plan		For: Information
					Source: Huawei

Decision: 		The document was noted.

R5-242835	WP UE Conformance - Further NR coverage enhancements

Type: Work Plan		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-242836	SR UE Conformance - Further NR coverage enhancements

Type: WI status report		For: Information
					Source: China Telecom

Decision: 		The document was noted.

R5-242837	Work plan: UE Conformance - NR sidelink enhancement

Type: Work Plan		For: Endorsement
					Source: CATT

Decision: 		The document was noted.

R5-242838	SR UE Conformance - NR sidelink enhancement

Type: WI status report		For: Endorsement
					Source: CATT

Decision: 		The document was noted.

R5-242839	Work plan: UE Conformance - NR Sidelink Relay

Type: Work Plan		For: Endorsement
					Source: CATT

Decision: 		The document was noted.

R5-242840	SR UE Conformance - NR Sidelink Relay

Type: WI status report		For: Endorsement
					Source: CATT

Decision: 		The document was noted.

R5-242842	WP UE Conformance – Multi-carrier enhancements for NR

Type: Work Plan		For: Information
					Source: China Telecom, Huawei, HiSilicon

Decision: 		The document was noted.

R5-242843	SR UE Conformance – Multi-carrier enhancements for NR

Type: WI status report		For: Information
					Source: China Telecom, Huawei, HiSilicon

Decision: 		The document was noted.

R5-242847	WP UE Conformance - Further Multi-RAT Dual-Connectivity enhancement

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-242848	SR UE Conformance - Further Multi-RAT Dual-Connectivity enhancement

Type: WI status report		For: Endorsement
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-242849	WP UE Conformance - 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-242850	SR UE Conformance - 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC

Type: WI status report		For: Endorsement
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-242851	WP UE Conformance - Network energy savings for NR

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-242852	SR UE Conformance - Network energy savings for NR

Type: WI status report		For: Endorsement
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-242853	WP UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-242854	SR UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18

Type: WI status report		For: Endorsement
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-243034	SR Rel-18 ATG for NR after RAN#103

Type: WI status report		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243035	WP Rel-18 ATG for NR after RAN#103

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243039	SR NR_Rel-16_CA_DC after RAN#103

Type: WI status report		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243040	WP NR_Rel-16_CA_DC after RAN#103

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243052	Rel-15 5GS WP SIG NE-DC after RAN5#103

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243054	SR Rel-18 IoT NTN enh after RAN#103

Type: WI status report		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243055	WP Rel-18 IoT NTN enh after RAN#103

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243056	Revised WID on UE Conformance - IoT NTN enh plus CT1 aspects

Type: WID revised		For: Endorsement
					Source: CMCC

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243063	SR Rel-18 PC1.5 n34 n39 n40 after RAN5#103

Type: WI status report		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243978	Revised WID - PC1.5 n34 n39 n40

Type: WID revised		For: Endorsement
					Source: CMCC

Discussion:

is endorsed.

Decision: 		The document was noted.

R5-243064	WP Rel-18 PC1.5 n34 n39 n40 after RAN5#103

Type: Work Plan		For: Information
					Source: CMCC

Decision: 		The document was noted.

R5-243080	WP - UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2

Type: Work Plan		For: Information
					Source: Samsung

Decision: 		The document was noted.

R5-243081	WP - UE Conformance - NR MIMO Evolution for Downlink and Uplink

Type: Work Plan		For: Information
					Source: Samsung

Decision: 		The document was noted.

R5-243082	SR - UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2

Type: WI status report		For: Information
					Source: Samsung

Decision: 		The document was noted.

R5-243083	SR - UE Conformance - NR MIMO Evolution for Downlink and Uplink

Type: WI status report		For: Information
					Source: Samsung

Decision: 		The document was noted.

R5-243098	SR of Rel-16 V2X WI after RAN5 103

Type: WI status report		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-243099	WP of Rel-16 V2X WI after RAN5 103

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-243161	WP of Rel-18 NR CA and DC; and NR and LTE DC Configurations

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-243162	SR of Rel-18 NR CA and DC; and NR and LTE DC Configurations

Type: WI status report		For: Endorsement
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-243178	Revised WID on UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects

Type: WID revised		For: Endorsement
					Source: Nokia

Discussion:

r1

38.903 will be removed.

Decision: 		The document was noted.

R5-243975	Revised WID on UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects

Type: WID revised		For: Endorsement
					Source: Nokia

(Replaces R5-243178)

Discussion:

is endorsed

Decision: 		The document was revised to R5-243992.

R5-243992	Revised WID on UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects

Type: WID revised		For: Endorsement
					Source: Nokia

(Replaces R5-243975)

Decision: 		The document was noted.

R5-243179	WP UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects

Type: Work Plan		For: Information
					Source: Nokia

Decision: 		The document was noted.

R5-243180	SR UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects

Type: WI status report		For: Information
					Source: Nokia

Decision: 		The document was noted.

R5-243190	Revised WID on UE Conformance - Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258

Type: WID revised		For: Endorsement
					Source: Huawei, Hisilicon

Discussion:

is endorsed

Decision: 		The document was agreed.

R5-243191	WP of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-243192	SR of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Discussion:

meeting #

Decision: 		The document was revised to R5-243979.

R5-243979	SR of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

(Replaces R5-243192)

Decision: 		The document was noted.

R5-243193	WP of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-243194	SR of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was revised to R5-243981.

R5-243981	SR of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

(Replaces R5-243194)

Decision: 		The document was noted.

R5-243195	WP of Rel-17 NR CA and DC; and NR and LTE DC Configurations

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-243196	SR of Rel-17 NR CA and DC; and NR and LTE DC Configurations

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-243197	WP of New Rel-18 NR licensed bands and extension of existing NR bands

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-243198	SR of New Rel-18 NR licensed bands and extension of existing NR bands

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-243199	WP of Further RF requirements enhancement for NR and EN-DC in frequency range 1

Type: Work Plan		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-243200	SR of Further RF requirements enhancement for NR and EN-DC in frequency range 1

Type: WI status report		For: Endorsement
					Source: Huawei, Hisilicon

Decision: 		The document was noted.

R5-243262	SR - UE Conformance Aspects - NR RRM enhancements

Type: WI status report		For: Approval
					Source: Apple

Decision: 		The document was noted.

R5-243263	WP - UE Conformance Aspects - NR RRM enhancements

Type: Work Plan		For: Approval
					Source: Apple

Decision: 		The document was noted.

R5-243265	SR Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData)

Type: WI status report		For: Information
					Source: NIST

Decision: 		The document was noted.

R5-243266	WP Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData)

Type: Work Plan		For: Information
					Source: NIST

Decision: 		The document was noted.

R5-243275	Revised WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects

Type: WID revised		For: Endorsement
					Source: China Unicom

Decision: 		The document was revised to R5-243976.

R5-243976	Revised WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects

Type: WID revised		For: Endorsement
					Source: China Unicom

(Replaces R5-243275)

Discussion:

is endorsed.

Decision: 		The document was noted.

R5-243276	WP on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects

Type: Work Plan		For: (not specified)
					Source: China Unicom

Decision: 		The document was noted.

R5-243277	SR on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects

Type: WI status report		For: Endorsement
					Source: China Unicom

Decision: 		The document was revised to R5-243982.

R5-243982	SR on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects

Type: WI status report		For: Endorsement
					Source: China Unicom

(Replaces R5-243277)

Decision: 		The document was noted.

R5-243281	WP on UE Conformance - Additional NR bands for UL-MIMO in Rel-18

Type: Work Plan		For: (not specified)
					Source: China Unicom

Decision: 		The document was noted.

R5-243306	SR on UE Conformance - Additional NR bands for UL-MIMO in Rel-18

Type: WI status report		For: Information
					Source: China Unicom

Decision: 		The document was revised to R5-243983.

R5-243983	SR on UE Conformance - Additional NR bands for UL-MIMO in Rel-18

Type: WI status report		For: Information
					Source: China Unicom

(Replaces R5-243306)

Decision: 		The document was noted.

R5-243307	WP on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band

Type: Work Plan		For: (not specified)
					Source: China Unicom

Decision: 		The document was noted.

R5-243308	SR on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band

Type: WI status report		For: Information
					Source: China Unicom

Discussion:

GPP

Decision: 		The document was revised to R5-243980.

R5-243980	SR on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band

Type: WI status report		For: Information
					Source: China Unicom

(Replaces R5-243308)

Decision: 		The document was noted.

R5-243405	SR of NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

Type: WI status report		For: Endorsement
					Source: Apple Benelux B.V., Nokia

Abstract:

Captures Post RAN5#103 status of this Rel-18 WID

Decision: 		The document was noted.

R5-243406	WP for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

Type: Work Plan		For: Information
					Source: Apple Benelux B.V., Nokia

Abstract:

Captures Post RAN5#103 work plan for this Rel-18 WID

Decision: 		The document was noted.

R5-243424	WP - UE Conformance Test Aspects for NR-based Access to Unlicensed Spectrum

Type: Work Plan		For: Approval
					Source: Qualcomm Inc

Decision: 		The document was not treated.

R5-243425	SR - UE Conformance Test Aspects for NR-based Access to Unlicensed Spectrum

Type: WI status report		For: Approval
					Source: Qualcomm Inc

Decision: 		The document was noted.

R5-243426	WP UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN)

Type: Work Plan		For: Approval
					Source: Qualcomm Inc

Decision: 		The document was noted.

R5-243427	SR UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN)

Type: WI status report		For: Approval
					Source: Qualcomm Inc

Decision: 		The document was noted.

R5-243428	WP UE Conformance Test Aspects - Further enhancement on NR demodulation performance

Type: Work Plan		For: Approval
					Source: Qualcomm Inc

Decision: 		The document was noted.

#### 7.5.3	Work Plan updates of recently closed work items

R5-242557	SIG WP for TEI16 - Redirection with MPS Indication

Type: Work Plan		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was not treated.

R5-242558	Rel-15 5GS maintenance SIG WP (NR-SA and ENDC)

Type: Work Plan		For: (not specified)
					Source: Qualcomm CDMA Technologies

Decision: 		The document was not treated.

R5-242953	WP Rel-15 NR TX and RX Test Cases – Part 3: Range 1 and Range 2 Interworking operation with other radios (TS 38.521-3)

Type: WI status report		For: Agreement
					Source: Qualcomm Tech. Netherlands B.V

Decision: 		The document was noted.

R5-243163	Rel-15 5GS maintenance WP for NR RF TX and RX Test Cases (38.521-1)

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-243164	Rel-15 5GS maintenance WP for NR RF TX and RX Test Cases (38.521-2)

Type: Work Plan		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R5-243280	Rel-17 RedCap WP update after RAN5#103

Type: Work Plan		For: (not specified)
					Source: China Unicom

Decision: 		The document was noted.

R5-243410	Maintainence Work Plan for Rel17 TRP TRS

Type: Work Plan		For: Information
					Source: Apple Benelux B.V.

Discussion:

reserved in error

Decision: 		The document was withdrawn.

### 7.6	Docs still needing agreement/endorsement/approval (e.g. Outgoing LS, Reports, New Specs, Info for certification bodies etc.)

R5-243450	MCC TF160 Status Report

Type: report		For: Approval
					Source: MCC TF160

(Replaces R5-242308)

Decision: 		The document was approved.

R5-242745	TS 38.523-1 Tracker status after RAN5-103

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R5-242746	TS 36.523-1 Tracker status after RAN5-103

Type: other		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R5-242776	RAN5#103 summary of changes to RAN5 test cases with potential impact on GCF and PTCRB

Type: report		For: Information
					Source: Bureau Veritas ADT

Abstract:

Post-meeting

Decision: 		The document was not treated.

R5-243972	LS on Development of NB-IoT test cases for Release 15 and Release 16

Type: LS out		For: Approval
					to GSMA TSG Internet of Things (IoT) group, cc TSG WG RAN4, GCF SG, GCF CAG
					Source: TSG WG RAN5

Abstract:

RAN5 would like to thank GSMA TSG Internet of Things (IoT) group for the LS (TSGIoT66a_003) regarding the development of NB-IoT test cases for Release 15 and Release 16. RAN5 received further information and background about the LS from GSMA TSG representatives in 3GPP during the presentation of the LS in RAN5#103 meeting. RAN5 was informed that GSMA TSG IoT Group’s work on NB-IoT NTN test specification and test cases is targeted towards ‘Interoperability’ testing, thus won’t be duplicating conformance test development undertaken by RAN5. It was also clarified that the queries in the LS was limited to Signalling (Protocol) test cases, though RAN4 has been copied.

RAN5 acknowledges GSMA TSG IoT Group’s strong interest in facilitating certification and commercialization of Rel-17 NB-IoT NTN devices.

RAN5 Conformance Test Development covering features and functionalities defined by 3GPP Core Working Groups is closely aligned with commercial deployment needs of the industry. RAN5 has no interest to define test cases for features and functionalities that have very little or no commercial interest. RAN5 experience to date has been not much of industry interest for Rel-15 and Rel-16 NB-IoT in a terrestrial environment. Hence, RAN5 has defined a limited amount of test cases for Rel-15 NB-IoT (no test coverage for EDT) and no test cases for Rel-16 NB-IoT.

If RAN5 can establish real commercial interest for some of the features defined in Rel-15 and Rel-16 NB-IoT for deployment of Rel-17 NB-IoT NTN, then RAN5 will be a position to define test cases for these features. RAN5 has the flexibility to follow most appropriate 3GPP Conformance Work Item process for this task depending on the scope of the work. Also, there is no 3GPP process and/or procedure impediment for RAN5 to add test coverage anytime for features defined in earlier releases.

RAN5 won’t initiate Conformance Test Development work on its own, as in the case of any 3GPP activity, it is contribution driven. Hence, interested proponents of NB-IoT NTN must initiate and drive Rel-15 and Rel-16 NB-IoT conformance test development in RAN5. RAN5 Leadership Team will provide necessary support as required.

Decision: 		The document was approved.

### 7.7	Confirmation of Future RAN5 Matters

R5-242115	Review deadlines for next quarter

Type: other		For: Information
					Source: WG Chairman

Decision: 		The document was noted.

### 7.8	AOB

## Annex A: Contribution documents and status

### A1: List of TDocs

1896 documents were submitted at RAN5#103. Plus 543 informal revisions (not shown here)

| Document | Title | Source | Decision | Replaces | Replaced by |
| --- | --- | --- | --- | --- | --- |
| R5-242100 | Agenda - opening session | WG Chairman | revised |  | R5-243354 |
| R5-242101 | Agenda - midweek session | WG Chairman | noted |  |  |
| R5-242102 | Agenda - closing session | WG Chairman | noted |  |  |
| R5-242103 | RAN5#103 Session Programme | WG Chairman | noted |  |  |
| R5-242104 | RAN5 Leadership Team | WG Chairman | noted |  |  |
| R5-242105 | RAN5#102 WG Minutes | ETSI Secretariat | approved |  |  |
| R5-242106 | RAN5#102 WG Action Points | ETSI Secretariat | noted |  |  |
| R5-242107 | Latest RAN Plenary notes | WG Chairman | noted |  |  |
| R5-242108 | Latest RAN Plenary draft Report | WG Chairman | noted |  |  |
| R5-242109 | Post Plenary Active Work Item update | ETSI Secretariat | noted |  |  |
| R5-242110 | RAN5 SR to RP#103 | WG Chairman | noted |  |  |
| R5-242111 | TF160 SR to RP#103 | WG Chairman | noted |  |  |
| R5-242112 | RAN5#103 LS Template | WG Chairman | noted |  |  |
| R5-242113 | Meeting schedule for 2024-25 | WG Chairman | noted |  |  |
| R5-242114 | WI Progress and Target Completion Date Review | WG Chairman | noted |  |  |
| R5-242115 | Review deadlines for next quarter | WG Chairman | noted |  |  |
| R5-242116 | LS on development of FRMCS conformance test cases by ETSI TTF | TC RT | noted |  |  |
| R5-242117 | LS on development of Server Side Conformance for Mission Critical Services by ETSI TTF | TCCE | noted |  |  |
| R5-242118 | Reply LS on TT work for Rel-18 NR FR1 TRP TRS | TSG WG RAN4 | noted |  |  |
| R5-242119 | LS on Head Phantoms for XR devices OTA testing | TSG WG RAN4 | noted |  |  |
| R5-242120 | Editorial correction to TC 6.2.16 | Mediatek India Technology Pvt. | agreed |  |  |
| R5-242121 | Correction of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit | Vodafone GmbH, MCC TF 160 | revised |  | R5-243468 |
| R5-242122 | Addition of applicability of new test case 8.1.3.6b for redir-policy bit | Vodafone GmbH, MCC TF160 | revised |  | R5-243469 |
| R5-242123 | Addition of Test Case 14.2.6 | Rohde & Schwarz | revised |  | R5-243803 |
| R5-242124 | New WID on UE Conformance - Further NR mobility enhancements | MediaTek Inc., Apple, China Telecom | revised |  | R5-243451 |
| R5-242125 | New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation | MediaTek Inc., China Telecom | revised |  | R5-243452 |
| R5-242126 | New WID on UE Conformance - Introduction of the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) for IoT NTN | MediaTek Inc., Inmarsat | revised |  | R5-243453 |
| R5-242127 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | revised |  | R5-244005 |
| R5-242128 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | revised |  | R5-243920 |
| R5-242129 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | revised |  | R5-243921 |
| R5-242130 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | revised |  | R5-243922 |
| R5-242131 | Correction to HPUE PICS mnemonic | MediaTek Inc. | withdrawn |  |  |
| R5-242132 | Update of NTN-config for NGSO | MediaTek Inc. | withdrawn |  |  |
| R5-242133 | Correction to the note of NR NTN PICS | MediaTek Inc. | agreed |  |  |
| R5-242134 | Correction to NR NTN TC 8.1.3.1.26 | MediaTek Inc. | agreed |  |  |
| R5-242135 | Correction to NR NTN TC 9.4.1.1 | MediaTek Inc. | revised |  | R5-243556 |
| R5-242136 | Correction to NR NTN TC 9.4.1.2 | MediaTek Inc. | revised |  | R5-243557 |
| R5-242137 | Correction to the applicability condition of NR NTN | MediaTek Inc. | revised |  | R5-243565 |
| R5-242138 | Addition of TC 9.1.11.5 into Table 4.3.1-1 | MediaTek Inc. | withdrawn |  |  |
| R5-242139 | Correction to RRC message MobilityFromNRCommand | MediaTek Inc. | agreed |  |  |
| R5-242140 | Correction to IE PDCP-Config for UDC | MediaTek Inc. | agreed |  |  |
| R5-242141 | Correction to IE MeasIdleConfig for Idle/Inactive measurements | MediaTek Inc. | agreed |  |  |
| R5-242142 | Correction to IE CondReconfigToAddModList | MediaTek Inc. | agreed |  |  |
| R5-242143 | Updates to align PICS mnemonics | MediaTek Inc. | withdrawn |  |  |
| R5-242144 | Correction to Idle mode test cases | MediaTek Inc. | agreed |  |  |
| R5-242145 | Correction to eCPSOR TC 6.3.2.2 | MediaTek Inc., Anritsu, ROHDE & SCHWARZ | agreed |  |  |
| R5-242146 | Correction to eCPSOR TC 6.3.2.4 | MediaTek Inc., Keysight | agreed |  |  |
| R5-242147 | Correction to eCPSOR TC 6.3.2.5 | MediaTek Inc., Anritsu | agreed |  |  |
| R5-242148 | Correction to NR slice TC 6.4.2.3 | MediaTek Inc. | agreed |  |  |
| R5-242149 | Correction to CAG TC 6.5.2.1 | MediaTek Inc. | agreed |  |  |
| R5-242150 | Correction to UPIP TC 7.1.3.2.6 | MediaTek Inc. | agreed |  |  |
| R5-242151 | Correction to PDCP TC 7.1.3.4.1 | MediaTek Inc. | revised |  | R5-243498 |
| R5-242152 | Correction to PDCP TC 7.1.3.4.2 | MediaTek Inc. | revised |  | R5-243499 |
| R5-242153 | Correction to RRC TC 8.1.1.2.4 | MediaTek Inc. | withdrawn |  |  |
| R5-242154 | Correction to NR Idle measurement TC 8.1.5.11.1 | MediaTek Inc. | revised |  | R5-243503 |
| R5-242155 | Correction to NR Idle measurement TC 8.1.5.11.2 | MediaTek Inc. | agreed |  |  |
| R5-242156 | Correction to NR Inactive measurement TC 8.1.5.11.3 | MediaTek Inc. | agreed |  |  |
| R5-242157 | Correction to NR Inactive measurement TC 8.1.5.11.4 | MediaTek Inc. | agreed |  |  |
| R5-242158 | Correction to MDT enh. TC 8.1.6.1.2.14 | MediaTek Inc. | agreed |  |  |
| R5-242159 | Correction to MDT enh. TC 8.1.6.1.3.8 | MediaTek Inc. | revised |  | R5-243507 |
| R5-242160 | Correction to MDT enh. TC 8.1.6.1.3.9 | MediaTek Inc. | revised |  | R5-243508 |
| R5-242161 | Correction to MDT enh. TC 8.1.6.1.3.10 | MediaTek Inc. | revised |  | R5-243509 |
| R5-242162 | Correction to MDT enh. TC 8.1.6.1.3.11 | MediaTek Inc. | revised |  | R5-243510 |
| R5-242163 | Correction to MDT enh. TC 8.1.6.1.4.9 | MediaTek Inc. | agreed |  |  |
| R5-242164 | Correction to MDT enh. TC 8.1.6.4.2 | MediaTek Inc. | agreed |  |  |
| R5-242165 | Correction to MDT enh. TC 8.1.6.4.3 | MediaTek Inc. | agreed |  |  |
| R5-242166 | Correction to MDT enh. TC 8.1.6.4.4 | MediaTek Inc. | agreed |  |  |
| R5-242167 | Correction to NE-DC TC 8.2.2.8.3 | MediaTek Inc. | withdrawn |  |  |
| R5-242168 | Correction to NR CA TC 8.2.4.2.1.1 | MediaTek Inc. | agreed |  |  |
| R5-242169 | Correction to NE-DC TC 8.2.6.3.5 and 8.2.6.3.6 | MediaTek Inc. | agreed |  |  |
| R5-242170 | Correction to UPIP TC 8.2.6.4.1 | MediaTek Inc. | agreed |  |  |
| R5-242171 | Correction to NSAC TC 10.1.8.4 | MediaTek Inc. | agreed |  |  |
| R5-242172 | Correction to UAC TC 11.3.5 | MediaTek Inc. | agreed |  |  |
| R5-242173 | Correction to UAC TC 11.3.12 | MediaTek Inc. | revised |  | R5-243515 |
| R5-242174 | Editorial update of applicability for TC 8.2.4.2.1.x | MediaTek Inc. | revised |  | R5-243519 |
| R5-242175 | Update applicability of SOR-CMCI TC | MediaTek Inc. | agreed |  |  |
| R5-242176 | Update case title of 8.1.1.2.4 and 8.1.5.9.1 | MediaTek Inc. | agreed |  |  |
| R5-242177 | Discussion on handling of legacy Rel-16/17 WIs for RedCap UE | MediaTek Inc. | noted |  |  |
| R5-242178 | Update the note in the satellite ephemeris information | MediaTek Inc. | agreed |  |  |
| R5-242179 | Update the SIB31 for GSO and NGSO branches | MediaTek Inc. | agreed |  |  |
| R5-242180 | Correction to generic procedure 4.5.4 and 4.5.4A | MediaTek Inc. | agreed |  |  |
| R5-242181 | Correction to generic test procedure 4.5A.5 | MediaTek Inc. | agreed |  |  |
| R5-242182 | Correction to IE PhysicalConfigDedicated | MediaTek Inc. | agreed |  |  |
| R5-242183 | Correction to PDCP TC 7.3.3.x | MediaTek Inc. | agreed |  |  |
| R5-242184 | Correction to ZUC TC 7.3.3.5, 7.3.3.6 and 7.3.4.3 | MediaTek Inc. | agreed |  |  |
| R5-242185 | Correction to TC 11.2.13 | MediaTek Inc., NTT DOCOMO, MCC TF160, ROHDE & SCHWARZ | agreed |  |  |
| R5-242186 | Correction to case title of TC 11.2.13 | MediaTek Inc., NTT DOCOMO, MCC TF160, ROHDE & SCHWARZ | agreed |  |  |
| R5-242187 | Optimize the tables for IoT NTN | MediaTek Inc., Anritsu | revised |  | R5-243578 |
| R5-242188 | Removal of GEA2 test case from the ATS | MediaTek Inc. | agreed |  | - |
| R5-242189 | WP UE Conformance - NR and MR-DC measurement gap enhancements | MediaTek Inc. | noted |  |  |
| R5-242190 | SR UE Conformance - NR and MR-DC measurement gap enhancements | MediaTek Inc. | noted |  |  |
| R5-242191 | Update TC 14.2.3 with TT analysis results | CATT | revised |  | R5-243784 |
| R5-242192 | Update TC 14.2.4 with TT analysis results | CATT | revised |  | R5-243785 |
| R5-242193 | Update TC 14.2.8 with TT analysis results | CATT | revised |  | R5-243760 |
| R5-242194 | Update TC 14.2.9 with TT analysis results | CATT | revised |  | R5-243761 |
| R5-242195 | Update TC 14.2.10 with TT analysis results | CATT | revised |  | R5-243762 |
| R5-242196 | Update TC 14.3.3 with TT analysis results | CATT | revised |  | R5-243786 |
| R5-242197 | Update TC 14.3.4 with TT analysis results | CATT | revised |  | R5-243787 |
| R5-242198 | Update TC 14.3.7 with TT analysis results | CATT | revised |  | R5-243763 |
| R5-242199 | Update TC 14.3.8 with TT analysis results | CATT | revised |  | R5-243764 |
| R5-242200 | Update TC 15.2.1 with TT analysis results | CATT | revised |  | R5-243788 |
| R5-242201 | Update TC 15.2.2 with TT analysis results | CATT | agreed |  |  |
| R5-242202 | Update TC 15.2.3 with TT analysis results | CATT | revised |  | R5-243789 |
| R5-242203 | Update TC 15.2.4 with TT analysis results | CATT | revised |  | R5-243790 |
| R5-242204 | Update TC 15.2.8 with TT analysis results | CATT | revised |  | R5-243765 |
| R5-242205 | Update TC 15.2.9 with TT analysis results | CATT | revised |  | R5-243766 |
| R5-242206 | Update TC 15.2.10 with TT analysis results | CATT | revised |  | R5-243767 |
| R5-242207 | Update TC 15.3.1 with TT analysis results | CATT | revised |  | R5-243791 |
| R5-242208 | Update TC 15.3.2 with TT analysis results | CATT | agreed |  |  |
| R5-242209 | Update TC 15.3.5 with TT analysis results | CATT | agreed |  |  |
| R5-242210 | Update TC 15.3.6 with TT analysis results | CATT | agreed |  |  |
| R5-242211 | Update TC 16.2.3 with TT analysis results | CATT | agreed |  |  |
| R5-242212 | Update TC 16.2.4 with TT analysis results | CATT | agreed |  |  |
| R5-242213 | Update TC 16.2.7 with TT analysis results | CATT | agreed |  |  |
| R5-242214 | Update TC 16.2.8 with TT analysis results | CATT | agreed |  |  |
| R5-242215 | Update TC 16.3.2 with TT analysis results | CATT | agreed |  |  |
| R5-242216 | Update TC 16.3.4 with TT analysis results | CATT | agreed |  |  |
| R5-242217 | Update NR PRS-based measurement requirements in annex based on the updated TT analysis results | CATT | revised |  | R5-243768 |
| R5-242218 | Update TC 14.2.3 and TC 14.2.10 TT analysis results | CATT | revised |  | R5-243769 |
| R5-242219 | Update TC 14.2.4 TT analysis results | CATT | revised |  | R5-243792 |
| R5-242220 | Update TC 14.2.8 and TC 14.2.9 TT analysis results | CATT | revised |  | R5-243770 |
| R5-242221 | Update TC 14.3.3 and TC 14.3.8 TT analysis results | CATT | revised |  | R5-243771 |
| R5-242222 | Update TC 14.3.4 TT analysis results | CATT | revised |  | R5-243793 |
| R5-242223 | Update TC 14.3.7 TT analysis results | CATT | revised |  | R5-243772 |
| R5-242224 | Update TC 15.2.1 TT analysis results | CATT | agreed |  |  |
| R5-242225 | Update TC 15.2.2 TT analysis results | CATT | agreed |  |  |
| R5-242226 | Update TC 15.2.3 TT analysis results | CATT | revised |  | R5-243794 |
| R5-242227 | Update TC 15.2.4 TT analysis results | CATT | revised |  | R5-243795 |
| R5-242228 | Update TC 15.2.8, 15.2.9 and 15.2.10 TT analysis results | CATT | revised |  | R5-243773 |
| R5-242229 | Update TC 15.3.1 TT analysis results | CATT | agreed |  |  |
| R5-242230 | Update TC 15.3.2 TT analysis results | CATT | agreed |  |  |
| R5-242231 | Update TC 15.3.5 TT analysis results | CATT | agreed |  |  |
| R5-242232 | Update TC 15.3.6 TT analysis results | CATT | agreed |  |  |
| R5-242233 | Update TC 16.2.3, 16.2.7 and 16.2.8 TT analysis results | CATT | agreed |  |  |
| R5-242234 | Update TC 16.2.4 TT analysis results | CATT | agreed |  |  |
| R5-242235 | Update TC 16.3.2 TT analysis results | CATT | agreed |  |  |
| R5-242236 | Update TC 16.3.4 TT analysis results | CATT | agreed |  |  |
| R5-242237 | Work plan: UE Conformance Test Aspects - NR Positioning Enhancement | CATT | noted |  |  |
| R5-242238 | SR UE Conformance Test Aspects - NR Positioning Enhancement | CATT | noted |  |  |
| R5-242239 | New WID on UE Conformance Test Aspects - Expanded and improved NR positioning | CATT, CAICT | revised |  | R5-243454 |
| R5-242240 | Addition of test frequencies for CA_n28A-n40A | KDDI Corporation | agreed |  |  |
| R5-242241 | Addition of test frequencies for CA_n3A-n28A-n41A-n77A | KDDI Corporation | agreed |  |  |
| R5-242242 | Addition of UE capability for CA_n28A-n40A | KDDI Corporation | revised |  | R5-243812 |
| R5-242243 | Addition of UE capability for CA_n3A-n28A-n41A-n77A | KDDI Corporation | agreed |  |  |
| R5-242244 | LS RAN4 on 5G NR FR1 OTA Technical Specifications | GCF CAG | noted |  |  |
| R5-242245 | Addition of reference sensitivity TP analysis for CA_n28A-n40A | KDDI Corporation | revised |  | R5-243820 |
| R5-242246 | Addition of reference sensitivity TP analysis for CA_n3A-n28A-n41A-n77A | KDDI Corporation | agreed |  |  |
| R5-242247 | Addition of delta TIBc for CA_n28A-n40A | KDDI Corporation | agreed |  |  |
| R5-242248 | Addition of delta TIBc for CA_n3A-n28A-n41A-n77A | KDDI Corporation | revised |  | R5-243822 |
| R5-242249 | Addition of reference sensitivity for CA_n28A-n40A | KDDI Corporation | revised |  | R5-243821 |
| R5-242250 | Addition of delta RIBc and reference sensitivity for CA_n3A-n28A-n41A-n77A | KDDI Corporation | agreed |  |  |
| R5-242251 | Addition and correction of RF baseline implementation capabilities for PC2 EN-DC combs within FR1 | KDDI Corporation, MediaTek Inc | revised |  | R5-243609 |
| R5-242252 | Addition of UE maximum output power for new PC2 EN-DC combs within FR1 | KDDI Corporation | agreed |  |  |
| R5-242253 | Addition of reference sensitivity for new PC2 EN-DC combs within FR1 | KDDI Corporation | revised |  | R5-243615 |
| R5-242254 | Addition and correction of reference sensitivity exceptions for PC2 EN-DC combos | KDDI Corporation | agreed |  |  |
| R5-242255 | Editorial correction of UE capability for R16 EN-DC combs within FR1 | KDDI Corporation | agreed |  |  |
| R5-242256 | Add IE NeedForInterruptionInfoNR | Ericsson | withdrawn |  |  |
| R5-242257 | Initial Registration / IMS data channel | China Telecom | withdrawn |  |  |
| R5-242258 | Correction to NR CAG Test case 6.5.2.1 | Anritsu EMEA Ltd | withdrawn |  |  |
| R5-242259 | Pending UL MIMO update in 38.903 | Keysight Technologies | agreed |  |  |
| R5-242260 | Pending UL MIMO update in 38.521-2 | Keysight Technologies | agreed |  |  |
| R5-242261 | Discussion on MU and TT analysis for FR2c | Keysight Technologies UK Ltd | revised |  | R5-243724 |
| R5-242262 | FR2c MU definition in 38.903 | Keysight Technologies | revised |  | R5-243730 |
| R5-242263 | FR2c MU - Tx test cases update in 38.521-2 | Keysight Technologies | revised |  | R5-243726 |
| R5-242264 | Discussion on MU and TT analysis for PC5 | Keysight Technologies UK Ltd | noted |  |  |
| R5-242265 | PC5 FR2 MU definition in 38.903 | Keysight Technologies | revised |  | R5-243723 |
| R5-242266 | PC5 FR2 MU - Tx test cases update in 38.521-2 | Keysight Technologies | revised |  | R5-243721 |
| R5-242267 | PC5 FR2 MU - Rx test cases update in 38.521-2 | Keysight Technologies | agreed |  |  |
| R5-242268 | PC5 FR2 MU - Annex F update in 38.521-2 | Keysight Technologies | revised |  | R5-243722 |
| R5-242269 | PC5 FR2 MU - Updates in 38.521-3 | Keysight Technologies | revised |  | R5-243728 |
| R5-242270 | Corrrection to NR5GC eCPSOR test case 6.3.2.4 | Keysight Technologies UK | withdrawn |  |  |
| R5-242271 | Addition to R17 ATSSS testcase 10.5.2.1 | MediaTek Inc. | revised |  | R5-243964 |
| R5-242272 | Addition to R17 ATSSS testcase 10.5.2.2 | MediaTek Inc. | revised |  | R5-243965 |
| R5-242273 | Add applicability for Rel-17 ATSSS test cases | MediaTek Inc. | agreed |  |  |
| R5-242274 | Update 2-step RACH TC 7.1.1.1.8 for HD-FDD UE-PRACH | MediaTek Inc. | agreed |  |  |
| R5-242275 | Update 2-step RACH TC 7.1.1.1.9 for HD-FDD UE-PRACH | MediaTek Inc. | agreed |  |  |
| R5-242276 | Update URLLC TC 7.1.1.4.1.5 to test RedCap UE | MediaTek Inc. | agreed |  |  |
| R5-242277 | Update URLLC TC 7.1.1.4.2.6 to test RedCap UE | MediaTek Inc. | agreed |  |  |
| R5-242278 | Update coverage enhancement TC 7.1.1.4.2.7 to test RedCap UE | MediaTek Inc. | agreed |  |  |
| R5-242279 | Addition to legacy test cases applicability for RedCap UE | MediaTek Inc. | revised |  | R5-243595 |
| R5-242280 | Correction to R17 ATSSS TC 11.9.1 | MediaTek Inc. | withdrawn |  |  |
| R5-242281 | Correction to R17 ATSSS TC 11.9.2 | MediaTek Inc. | withdrawn |  |  |
| R5-242282 | Correction of Additional spurious emission test requirement for NS_45 | MediaTek Beijing Inc. | revised |  | R5-243807 |
| R5-242283 | Editorial Correction of test configuration table of Reference sensitivity level for 4DL CA | MediaTek Beijing Inc. | agreed |  |  |
| R5-242284 | Update of Rx test cases to add 50MHz for NR band n7 | MediaTek Beijing Inc. | revised |  | R5-243625 |
| R5-242285 | Editorial correction to test description of Clause 6.5A and 6.5B | MediaTek Beijing Inc. | agreed |  |  |
| R5-242286 | Update of Operating bands for UL MIMO band n66 | MediaTek Beijing Inc. | withdrawn |  |  |
| R5-242287 | Addition of n66 UL MIMO MOP | MediaTek Beijing Inc. | agreed |  |  |
| R5-242288 | Addition of n66 UL MIMO MPR | MediaTek Beijing Inc. | agreed |  |  |
| R5-242289 | Editorial correction of n66 with UL MIMO capabilities | MediaTek Beijing Inc. | agreed | - |  |
| R5-242290 | Proposal for new test cases related to P-CSCF Restoration procedures | Orange, Qualcomm | withdrawn |  |  |
| R5-242291 | Removal of technical content in TS 34.229-1 v15.10.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242292 | Removal of technical content in TS 34.229-1 v16.7.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242293 | Removal of technical content in TS 34.229-2 v16.10.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242294 | Removal of technical content in TS 34.229-3 v16.8.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242295 | Removal of technical content in TS 34.229-5 v16.10.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242296 | Removal of technical content in TS 36.521-3 v17.1.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242297 | Removal of technical content in TS 36.523-3 v17.7.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242298 | Removal of technical content in TS 36.579-3 v13.3.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242299 | Removal of technical content in TS 36.579-3 v14.0.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242300 | Removal of technical content in TS 36.579-5 v15.1.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242301 | Removal of technical content in TS 36.579-5 v16.0.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242302 | Removal of technical content in TS 36.579-7 v15.5.1 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242303 | Removal of technical content in TS 37.571-3 v16.14.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242304 | Removal of technical content in TS 37.571-4 v16.8.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242305 | Removal of technical content in TS 38.521-5 v17.0.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242306 | Removal of technical content in TS 38.522 v15.3.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242307 | Removal of technical content in TR 38.903 v16.14.0 and substitution with pointer to the next Release | ETSI Secretariat | agreed |  |  |
| R5-242308 | MCC TF160 Status Report | MCC TF160 | revised |  | R5-243450 |
| R5-242309 | RAN5 PRD12 version 6.10 | MCC TF160 | approved |  |  |
| R5-242310 | NR QoE: Addition of NR/5GC Test Model aspects | MCC TF160 | agreed |  |  |
| R5-242311 | Correction of testcase 6.1.1.14 | MCC TF160 | agreed |  |  |
| R5-242312 | Correction of testcase 6.9.1 | MCC TF160 | agreed |  |  |
| R5-242313 | ID_UAS: Test Model aspects | MCC TF160 | agreed |  |  |
| R5-242314 | Updates to 5G V2X test case 12.1.7.1 | MCC TF160 | agreed |  |  |
| R5-242315 | Updates to 5G V2X test case 12.2.9.1 | MCC TF160 | agreed |  |  |
| R5-242316 | Corrections to SDT TC 7.1.1.13.1 | MCC TF160, Lenovo | agreed |  |  |
| R5-242317 | Corrections to SDT TC 7.1.1.13.2 | MCC TF160, Lenovo | agreed |  |  |
| R5-242318 | Corrections to SDT TC 7.1.1.13.3 | MCC TF160, Lenovo | agreed |  |  |
| R5-242319 | NR SDT: Addition of NR/5GC Test Model aspects | MCC TF160 | revised |  | R5-243579 |
| R5-242320 | NR NTN: Addition of GSO Test Model | MCC TF160 | agreed |  |  |
| R5-242321 | Editorial updates to specification references in clauses 4.5, 4.5A & 4.9 | MCC TF160 | agreed |  |  |
| R5-242322 | Editorial updates to specification references in clause 4.6 | MCC TF160 | agreed |  |  |
| R5-242323 | Editorial updates to REGISTRATION REQUEST message | MCC TF160 | agreed |  |  |
| R5-242324 | Deletion of PICS pc_pdsch_MappingTypeA | MCC TF160 | agreed |  |  |
| R5-242325 | Updates to align PICS mnemonics | MCC TF160 | agreed |  |  |
| R5-242326 | Updates to eDRX TC 11.7.2 | MCC TF160 | agreed |  |  |
| R5-242327 | Updates to NR RRC test case 8.1.5.1.1 | MCC TF160 | agreed |  |  |
| R5-242328 | Updates to EN-DC RRC test case 8.2.1.1.1 | MCC TF160 | agreed |  |  |
| R5-242329 | Updates to NE-DC RRC test case 8.2.1.1.2 | MCC TF160 | agreed |  |  |
| R5-242330 | Updates of clause 4 | MCC TF160 | agreed |  |  |
| R5-242331 | Routine maintenance for TS 38.523-3 | MCC TF160 | revised |  | R5-243967 |
| R5-242332 | Updates to NB-IoT RRC test case 22.4.26 | MCC TF160 | agreed |  |  |
| R5-242333 | Updates to E-UTRA RRC test case 8.1.2.15 | MCC TF160 | agreed |  |  |
| R5-242334 | NTN-IoT: Addition of NGSO NB-IoT NTN Test Model | MCC TF160 | agreed |  |  |
| R5-242335 | Routine maintenance for TS 36.523-3 | MCC TF160 | agreed |  |  |
| R5-242336 | Correction of IMSo5G test case 7.23 | MCC TF160 | revised |  | R5-243525 |
| R5-242337 | Clarification of initial conditions and RRC/NAS signalling in clause 5.3 | MCC TF160 | agreed |  |  |
| R5-242338 | Clarification of initial conditions and RRC/NAS signalling in clause 5.3A | MCC TF160 | agreed |  |  |
| R5-242339 | Clarification of initial conditions and RRC/NAS signalling in clause 5.3B | MCC TF160 | agreed |  |  |
| R5-242340 | Clarification of initial conditions and RRC/NAS signalling in clause 5.3C | MCC TF160 | agreed |  |  |
| R5-242341 | Clarifying the System Under Test | MCC TF160 | agreed |  |  |
| R5-242342 | Corrections of clause 2 | MCC TF160 | agreed |  |  |
| R5-242343 | Corrections of clause 5.5.2.15.2 | MCC TF160 | agreed |  |  |
| R5-242344 | Corrections of clause 5.5.2.2.2 | MCC TF160 | agreed |  |  |
| R5-242345 | Corrections of clause 5.5.2.4 | MCC TF160 | agreed |  |  |
| R5-242346 | Corrections of clause 5.5.2.5.2 | MCC TF160 | agreed |  |  |
| R5-242347 | Corrections of clause 5.5.2.7.1 | MCC TF160 | agreed |  |  |
| R5-242348 | Editorial corrections of clauses 1 and 4 | MCC TF160 | agreed |  |  |
| R5-242349 | Improvement of clause 5.2 | MCC TF160 | agreed |  |  |
| R5-242350 | Improvement of clause 5.4 | MCC TF160 | agreed |  |  |
| R5-242351 | Common correction of test cases in clauses 5 and 6 | MCC TF160 | agreed |  |  |
| R5-242352 | Correction of test case 6.1.1.15 | MCC TF160 | agreed |  |  |
| R5-242353 | Correction of test case 6.1.1.16 | MCC TF160 | agreed |  |  |
| R5-242354 | Correction of test case 6.3.2 | MCC TF160 | agreed |  |  |
| R5-242355 | Misc updates | MCC TF160 | agreed |  |  |
| R5-242356 | Common correction of test cases in clause 6 | MCC TF160 | agreed |  |  |
| R5-242357 | Correction of testcase 6.3.1 | MCC TF160 | agreed |  |  |
| R5-242358 | Correction of testcase 6.3.2 | MCC TF160 | agreed |  |  |
| R5-242359 | Common correction of test cases in clauses 5 and 6 | MCC TF160 | agreed |  |  |
| R5-242360 | Correction to Table 6.F-1 | Ericsson | withdrawn |  |  |
| R5-242361 | PRD21 CDS: NR FR1 FDD band n85 | Nokia | noted |  |  |
| R5-242362 | Introduction of test channel bandwidths and test frequencies for band n85 | Nokia | revised |  | R5-243602 |
| R5-242363 | Introduction of signalling test frequencies for band n85 | Nokia | revised |  | R5-243494 |
| R5-242364 | Introduction of band n85 for physical layer baseline implementation capabilities | Nokia | revised |  | R5-243611 |
| R5-242365 | Adding transmitter requirements for band n85 | Nokia | agreed |  |  |
| R5-242366 | Adding receiver requirements for band n85 | Nokia | revised |  | R5-243651 |
| R5-242367 | PRD21 CDS: NR FR1 FDD band n106 | Nokia | agreed |  |  |
| R5-242368 | Introduction of test channel bandwidths and test frequencies for band n106 | Nokia | agreed |  |  |
| R5-242369 | Signalling test frequencies for 3 MHz and band n106 | Nokia | revised |  | R5-243566 |
| R5-242370 | Introduction of band n106 for physical layer baseline implementation capabilities | Nokia | agreed |  |  |
| R5-242371 | Introduction of band n106 to NR band group for FR1 | Nokia | agreed |  |  |
| R5-242372 | Introduction of new band n106 into clause 5 | Nokia | revised |  | R5-243643 |
| R5-242373 | Adding transmitter requirements for band n106 | Nokia | revised |  | R5-243640 |
| R5-242374 | Adding receiver requirements for band n106 | Nokia | revised |  | R5-243641 |
| R5-242375 | Correction to reference sensitivity test requirements for CA_n25A-n77A, CA_n25A-n78A and CA_n66A-n78A | Nokia | revised |  | R5-243635 |
| R5-242376 | WP UE Conformance – High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 | Nokia | noted |  |  |
| R5-242377 | SR UE Conformance – High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 | Nokia | noted |  |  |
| R5-242378 | Revised WID on UE Conformance - High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands in Rel-18 | Nokia | noted |  |  |
| R5-242379 | Introduction of PC1 transmitter requirements for bands n7, n41 and n78 | Nokia | revised |  | R5-243877 |
| R5-242380 | Introduction of bands n7 and n78 PC1 for physical layer baseline implementation capabilities | Nokia | agreed |  |  |
| R5-242381 | Adding reference sensitivity requirements for UEs that support optional symmetric UL/DL in band n71 | Nokia | revised |  | R5-243642 |
| R5-242382 | WP UE Conformance – Introduction of 900 MHz LTE band in the US | Nokia | noted |  |  |
| R5-242383 | SR UE Conformance – Introduction of 900 MHz LTE band in the US | Nokia | noted |  |  |
| R5-242384 | Addition of test frequencies for LTE Band 106 | Nokia | agreed |  |  |
| R5-242385 | Addition of signalling test frequencies for LTE Band 106 and 3 MHz channel bandwidth | Nokia | agreed |  |  |
| R5-242386 | Addition of band 106 into frequency bands and channel arrangements | Nokia | agreed |  |  |
| R5-242387 | Addition of band 106 transmitter requirements | Nokia | agreed |  |  |
| R5-242388 | Addition of band 106 receiver requirements | Nokia | agreed |  |  |
| R5-242389 | Addition of test case applicability statements for band 106 | Nokia | agreed |  |  |
| R5-242390 | Addition of band groups for band 106 | Nokia | agreed |  |  |
| R5-242391 | Update to spurious emissions UE coexistence as part of introduction of LTE Band 106 | Nokia | agreed |  |  |
| R5-242392 | Adding less than 5 MHz BW into Bandwidth dependent parameters | Nokia | agreed |  |  |
| R5-242393 | Addition of band 106 to RF Baseline Implementation Capabilities | Nokia | agreed |  |  |
| R5-242394 | Addition of 3 MHz CBW test frequencies for NR Bands n26, n28, n85 and n100 | Nokia | revised |  | R5-243601 |
| R5-242395 | Introduction of dedicated spectrum less than 5MHz for FR1 to clause 5 and annex A | Nokia | revised |  | R5-243639 |
| R5-242396 | Addition of 3 MHz channel bandwidth for transmitter requirements | Nokia | revised |  | R5-243637 |
| R5-242397 | Addition of 3 MHz channel bandwidth for receiver requirements | Nokia | revised |  | R5-243638 |
| R5-242398 | Correction of URI scheme in HTTP POST | MCC TF160 | agreed |  |  |
| R5-242399 | Routine maintenance for TS 36.579-5 | MCC TF160 | agreed |  |  |
| R5-242400 | Corrections of clause 5.5.9.1 | MCC TF160 | agreed |  |  |
| R5-242401 | Add IE AppLayerIdleInactiveConfig | Ericsson | agreed |  |  |
| R5-242402 | Add IE EUTRA-MultiBandInfoListAerial | Ericsson | agreed |  |  |
| R5-242403 | WP UE Conformance - NR QoE management and optimizations for diverse services | Ericsson | noted |  |  |
| R5-242404 | SR UE Conformance - NR QoE management and optimizations for diverse services | Ericsson | noted |  |  |
| R5-242405 | Add IE MBS-NonServingInfoList | Ericsson | agreed |  |  |
| R5-242406 | Add IE MBS-SessionInfoListMulticast | Ericsson | agreed |  |  |
| R5-242407 | Add IE AerialParameters | Ericsson | agreed |  |  |
| R5-242408 | Add IE DL-PRS-MeasurementWithRxFH-RRC-Connected | Ericsson | agreed |  |  |
| R5-242409 | Add IE ERedCapParameters | Ericsson | agreed |  |  |
| R5-242410 | Add IE NCR-Parameters | Ericsson | agreed |  |  |
| R5-242411 | Add IE PosSRS-BWA-RRC-Inactive | Ericsson | withdrawn |  |  |
| R5-242412 | Add IEs PosSRS-TxFrequencyHoppingRRC-Connected and PosSRS-TxFrequencyHoppingRRC-Inactive | Ericsson | withdrawn |  |  |
| R5-242413 | Update IE SharedSpectrumChAccessParamsPerBand | Ericsson | agreed |  |  |
| R5-242414 | Add IE SharedSpectrumChAccessParamsSidelinkPerBand | Ericsson | agreed |  |  |
| R5-242415 | Add IE SupportedAggBandwidth | Ericsson | agreed |  |  |
| R5-242416 | Editorial correction to the wrong citation of rel-15 RRM cases | Mediatek India Technology Pvt. | agreed |  |  |
| R5-242417 | Editorial correction to the wrong message content of NTN RA cases | Mediatek India Technology Pvt. | agreed |  |  |
| R5-242418 | Addition of Editors note for 8.2.1.1.1 | Mediatek India Technology Pvt., Qualcomm Inc. | agreed |  |  |
| R5-242419 | Addition of Test configuration for satellite access | Mediatek India Technology Pvt., CMCC | agreed |  |  |
| R5-242420 | Addition of Antenna configuration for satellite access | Mediatek India Technology Pvt., CMCC | agreed |  |  |
| R5-242421 | Editorial correction to the RMC of 8.12.1.1.2 | Mediatek India Technology Pvt. | agreed |  |  |
| R5-242422 | Correction and update to the RMC and minimum test time of NB-IoT NTN demod cases | Mediatek India Technology Pvt. | revised |  | R5-243924 |
| R5-242423 | Update NB-IoT NTN timing advance test cases | MediaTek Beijing Inc. | revised |  | R5-243908 |
| R5-242424 | Update TT for NB-IoT NTN timing accuracy test cases | MediaTek Beijing Inc. | agreed |  |  |
| R5-242425 | Update NB-IoT NTN test case TC 13.4.1.1 | MediaTek Beijing Inc. | revised |  | R5-244019 |
| R5-242426 | Update TT analysis for NB-IoT NTN timing accuracy test cases | MediaTek Beijing Inc. | agreed |  |  |
| R5-242427 | Discussion on minimum test time simulation methods for NB-IoT NTN | Mediatek India Technology Pvt. | noted |  |  |
| R5-242428 | Discussion on NTN DCQR cases | Mediatek India Technology Pvt. | noted |  |  |
| R5-242429 | Update of test case 6.5F.3.3 and 6.5F.2.4.2 | Ericsson | agreed |  |  |
| R5-242430 | Update groups of bands for satellite access | Mediatek India Technology Pvt. | revised |  | R5-243888 |
| R5-242431 | Correction to setup for SIB31 and SIB31-NB | Mediatek India Technology Pvt. | revised |  | R5-243889 |
| R5-242432 | WP for UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR | China Telecom,Huawei, CATT | noted |  |  |
| R5-242433 | Update test requirement for Redcap inter frequency test cases | MediaTek Beijing Inc. | revised |  | R5-243907 |
| R5-242434 | Fix the release information for a NR CA PC2 band configuration | Verizon | agreed |  |  |
| R5-242435 | Update reference sensitivity test cases for ENDC band configurations with PC2 UL | Verizon | agreed |  |  |
| R5-242436 | PRD21 CDS: PC2 EN-DC FR1 3 bands and 4 bands | Verizon Spain | available |  |  |
| R5-242437 | SR UE Conformance Further enhancement on NR demodulation performance RAN5#103 | China Telecom | noted |  |  |
| R5-242438 | Revised WID on UE Conformance Further enhancement on NR demodulation performance | China Telecom | withdrawn |  |  |
| R5-242439 | TT analysis for FR1 CQI reporting test cases with inter-cell interference | China Telecom, Qualcomm | agreed |  |  |
| R5-242440 | Completion of FDD CQI reporting test cases with inter-cell interference | China Telecom | revised |  | R5-243691 |
| R5-242441 | Completion of TDD CQI reporting test cases with inter-cell interference | China Telecom | revised |  | R5-243692 |
| R5-242442 | Addition of MU and TT for CQI reporting test cases with inter-cell interference | China Telecom | revised |  | R5-243693 |
| R5-242443 | Correction of test applicability rule for CQI reporting test cases with inter-cell interference | China Telecom | agreed |  |  |
| R5-242444 | PRD21 CDS: NRDC combos between FR1 and FR2 | Verizon Spain | available |  |  |
| R5-242445 | PRD21 CDS PC3 NRCA 2 bands combos | KDDI Corporation | approved |  |  |
| R5-242446 | New WID on UE Conformance – NR demodulation performance evolution | China Telecom, Qualcomm, Samsung | revised |  | R5-243455 |
| R5-242447 | PRD21 CDS PC3 NRCA CA_n3A-n28A-n77A | KDDI Corporation | approved |  |  |
| R5-242448 | PRD21 CDS PC3 NRCA CA_n3A-n28A-n41A-n77A | KDDI Corporation | withdrawn |  |  |
| R5-242449 | Update for additional ENDC band configuration | Verizon Spain | agreed |  |  |
| R5-242450 | Update for additional ENDC band configurations | Verizon Spain | agreed |  |  |
| R5-242451 | Update for additional ENDC band configurations | Verizon | agreed |  |  |
| R5-242452 | Update for additional ENDC band configurations | Verizon Spain | agreed |  |  |
| R5-242453 | PRD21 CDS: ENDC combos between FR1 and FR2 | Verizon Spain | available |  |  |
| R5-242454 | PRD21 CDS PC2 EN-DC 2 bands combos with n41 and n77 | KDDI Corporation | approved |  |  |
| R5-242455 | SR for UE Conformance - Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR | China Telecom,Huawei, CATT | noted |  |  |
| R5-242456 | WP for UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS | China Telecom | noted |  |  |
| R5-242457 | SR for UE Conformance - IMS voice service support and network usability guarantee for UE’s E-UTRA capability disabled scenario in 5GS | China Telecom,Qualcomm | noted |  |  |
| R5-242458 | WP for UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | China Telecom,ZTE,MTK | noted |  |  |
| R5-242459 | SR for UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | China Telecom, ZTE, MTK | agreed |  | - |
| R5-242460 | Update for additional NRCA band configurations | Verizon, Ericsson | agreed |  |  |
| R5-242461 | Update for additional NRCA band configurations | Verizon Spain | revised |  | R5-243608 |
| R5-242462 | Update to R17 ATSSS test case 11.9.1 | China Telecom | withdrawn |  |  |
| R5-242463 | Update for additional NRCA band configurations | Verizon | agreed |  |  |
| R5-242464 | Update to R17 ATSSS test case 11.9.2 | China Telecom, MediaTek | revised |  | R5-243522 |
| R5-242465 | PRD21 CDS: PC3 NRCA additional combos FR1 | Verizon Spain | available |  |  |
| R5-242466 | Addition of n41 PC1 MOP TC | CMCC, Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-242467 | Addition of n41 PC1 MPR TC | CMCC, Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-242468 | Update n41 PC1 UTRA ACLR TC | CMCC, Nokia, Nokia Shanghai Bell | agreed |  |  |
| R5-242469 | PRD21 CDS: NR band and CBW FR1 n7 and n78 PC1 | Nokia | noted |  |  |
| R5-242470 | Correction to NBIOT testcase 22.5.21 | ROHDE & SCHWARZ | revised |  | R5-243477 |
| R5-242471 | Addition of new UE Rx-Tx time difference reporting delay test case 15.4.1 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-242472 | Addition of new UE Rx-Tx time difference reporting delay test case 15.4.2 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-242473 | Addition of new UE Rx-Tx time difference reporting delay test case 15.4.3 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-242474 | Addition of new UE Rx-Tx time difference reporting delay test case 15.4.4 in RRC_INACTIVE state | CATT | agreed |  |  |
| R5-242475 | Addition of new NR PRS-RSRPP measurement period test case 17.4.1 in RRC_INACTIVE | CATT | agreed |  |  |
| R5-242476 | Addition of new NR PRS-RSRPP measurement period test case 17.4.2 in RRC_INACTIVE | CATT | agreed |  |  |
| R5-242477 | Addition of new NR PRS-RSRPP measurement period test case 17.4.3 in RRC_INACTIVE | CATT | agreed |  |  |
| R5-242478 | Addition of new NR PRS-RSRPP measurement period test case 17.4.4 in RRC_INACTIVE | CATT | agreed |  |  |
| R5-242479 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.1 in RRC_INACTIVE | CATT | agreed |  |  |
| R5-242480 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.2 in RRC_INACTIVE | CATT | agreed |  |  |
| R5-242481 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.3 in RRC_INACTIVE | CATT | agreed |  |  |
| R5-242482 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.4 in RRC_INACTIVE | CATT | agreed |  |  |
| R5-242483 | Addition of test applicabilities for Rel-17 PRS-RSRPP and UE Rx-Tx time difference test cases | CATT, Rohde&Schwarz | revised |  | R5-243802 |
| R5-242484 | Change the GNSS-AcquisitionAssistance IE description for BDS signals | CATT | agreed |  |  |
| R5-242485 | Addition of test case 7.4F, Maximum input level for shared spectrum channel access | Ericsson | revised |  | R5-243886 |
| R5-242486 | Correction to NBIOT testcase 22.5.1 | ROHDE & SCHWARZ | agreed |  |  |
| R5-242487 | Correction to NR5GC testcase 9.1.5.1.4 | ROHDE & SCHWARZ, MediaTek, Huawei, Hisilicon | agreed |  |  |
| R5-242488 | Addition of test case 6.3F.4.4, Aggregate power tolerance for shared spectrum channel access | Ericsson | revised |  | R5-243885 |
| R5-242489 | Correction to NR5GC testcase 6.3.1.10 | ROHDE & SCHWARZ | revised |  | R5-243587 |
| R5-242490 | Updates to editors note in NR-U Tx test cases | Ericsson | revised |  | R5-243646 |
| R5-242491 | Addition of MU and TT for new NR-U test cases | Ericsson | revised |  | R5-243884 |
| R5-242492 | Addition of test case 6.2A.4.1.1, 4Rx FR1 CQI reporting under AWGN for Scell on band with shared spectrum access for CA (2DLCA) | Ericsson | agreed |  |  |
| R5-242493 | Core spec alignment of general clause for CSI reporting requirements | Ericsson | revised |  | R5-243694 |
| R5-242494 | Addition of applicability for new NR-U test cases | Ericsson | agreed |  |  |
| R5-242495 | Addition of new NR-U test cases | Ericsson | agreed |  |  |
| R5-242496 | Correction to NR5GC testcase 11.3.12 | ROHDE & SCHWARZ | revised |  | R5-243592 |
| R5-242497 | Addition of reference sensitivity TP analysis for new PC2 EN-DC combs within FR1 | KDDI Corporation | agreed |  |  |
| R5-242498 | Addition of n41 PC1 RF baseline ICS | CMCC, Nokia, Nokia Shanghai Bell | revised |  | R5-243610 |
| R5-242499 | Update to R18 IoT NTN test cases applicability | CMCC, Keysight, ROHDE & SCHWARZ, Qualcomm | revised |  | R5-243835 |
| R5-242500 | Introduction of new NR NTN test case - 6.5.1 Occupied bandwidth | Keysight Technologies | agreed |  |  |
| R5-242501 | Introduction of new NR NTN test case - 6.4.2.1 Error Vector Magnitude | Keysight Technologies | agreed |  |  |
| R5-242502 | Introduction of new NR NTN test case - 6.4.2.1a Error Vector Magnitude including symbols with transient period | Keysight Technologies | revised |  | R5-243896 |
| R5-242503 | Introduction of new NR NTN test case - 6.4.2.2 Carrier leakage | Keysight Technologies | revised |  | R5-243673 |
| R5-242504 | Introduction of new NR NTN test case - 6.4.2.3 In-band emissions | Keysight Technologies | revised |  | R5-243674 |
| R5-242505 | Introduction of new NR NTN test case - 6.4.2.4 EVM equalizer spectrum flatness | Keysight Technologies | revised |  | R5-243675 |
| R5-242506 | Introduction of new NR NTN test case - 6.4.2.5 EVM equalizer spectrum flatness for Pi2 BPSK | Keysight Technologies | agreed |  |  |
| R5-242507 | Update of NR NTN test case - 6.2.1 UE maximum output power | Keysight Technologies | revised |  | R5-243676 |
| R5-242508 | Update of NR NTN test case - 6.2.2 UE maximum output power reduction | Keysight Technologies | agreed |  |  |
| R5-242509 | Update of NR NTN test case - 6.2.3 UE additional maximum output power reduction | Keysight Technologies | revised |  | R5-243677 |
| R5-242510 | Update of NR NTN test case - 6.2.4 Configured transmitted power | Keysight Technologies | agreed |  |  |
| R5-242511 | Update MU and TT for NR NTN test cases | Keysight Technologies | revised |  | R5-243678 |
| R5-242512 | Update applicability for NR NTN test cases | Keysight Technologies | agreed |  |  |
| R5-242513 | Update MU and TT for NR NTN test cases | Keysight Technologies | withdrawn |  | - |
| R5-242514 | On phase continuity requirements for DMRS bundling for NR NTN | Keysight Technologies UK Ltd | noted |  |  |
| R5-242515 | Test frequency parameters corrections for CA n48B | Keysight Technologies | agreed |  |  |
| R5-242516 | Corrected MU and TT definition for FR1 UL MIMO test case 6.5D.2.3 | Keysight Technologies | agreed |  |  |
| R5-242517 | Introduction of new SUL test case 7.7C | Keysight Technologies | revised |  | R5-243897 |
| R5-242518 | Discussion on introducing new SUL test case 7.7C | Keysight Technologies UK Ltd | noted |  |  |
| R5-242519 | Absolute power control - test requirements correction | Keysight Technologies | agreed |  |  |
| R5-242520 | EVM in transient periods - test case definition update | Keysight Technologies | withdrawn |  |  |
| R5-242521 | Clarification for Rx test requirements for PC1.5 | Keysight Technologies, Huawei | agreed |  |  |
| R5-242522 | MU and TT defintion for TxD EVM test case 6.4G.2.1 | Keysight Technologies, Rohde&Schwarz | revised |  | R5-243898 |
| R5-242523 | Correction on applicability for PC1.5 in Rx tests | Keysight Technologies, Huawei | agreed |  |  |
| R5-242524 | Correction to NBIOT testcase 22.3.1.2 | ROHDE & SCHWARZ | agreed |  |  |
| R5-242525 | TP analysis for NR-U test case 6.4F.2.2, carrier leakage | Ericsson | agreed |  |  |
| R5-242526 | Correction to NTN test cases 22.1.2, 22.2.13, 22.4.30 and 22.5.23 | ROHDE & SCHWARZ | revised |  | R5-243478 |
| R5-242527 | Correction of UPIP TC 8.2.6.4.4 | ZTE Corporation | agreed |  |  |
| R5-242528 | Apply interlaced RB allocation in test configuration table in test case 6.4F.2.2 | Ericsson | revised |  | R5-243623 |
| R5-242529 | Addition of MU and TT for NR-U test case | Ericsson | revised |  | R5-243648 |
| R5-242530 | Correction to NR testcase 7.1.1.3.8.1 | ROHDE & SCHWARZ | withdrawn |  |  |
| R5-242531 | Correction to NR5GC testcase 8.1.5.8.1 | ROHDE & SCHWARZ | agreed |  |  |
| R5-242532 | Discussion regarding handling of missing test frequencies for UL CA combinations. | Ericsson | revised |  | R5-243605 |
| R5-242533 | Clarification of Test frequency limitations for CA in Annex C | Ericsson | revised |  | R5-243882 |
| R5-242534 | Correction of clause structure of CA test frequencies | Ericsson | agreed |  |  |
| R5-242535 | Addition of signalling test frequencies for CA_n48C | Ericsson, Verizon, MCC TF160 | revised |  | R5-243532 |
| R5-242536 | Addition of TP analysis for CA_n48C | Ericsson | agreed |  |  |
| R5-242537 | Addition of checklist for intra band CA in Annex B | Ericsson | revised |  | R5-243883 |
| R5-242538 | Addition of CA_n48C reference sensitivity requirement | Ericsson | agreed |  |  |
| R5-242539 | Updates to NR RRC test case 8.1.1.2.4 | MCC TF160, MediaTek | revised |  | R5-243501 |
| R5-242540 | New WID on UE Conformance - Support for UAV (Uncrewed Aerial Vehicles) and Further Architecture Enhancement for UAV and UAM | Qualcomm Incorporated, Nokia, China Unicom | revised |  | R5-243456 |
| R5-242541 | New WID on UE Conformance - Mobile Terminated-Small Data Transmission (MT-SDT) for NR | Qualcomm Incorporated, China Telecom, ZTE | revised |  | R5-243457 |
| R5-242542 | Introduction of new TC NR-NTN - TC TAI(s) not allowed / forbidden TAI(s) | Qualcomm Technologies Ireland | revised |  | R5-243558 |
| R5-242543 | Added Common test environment for NR-NTN test cases | Qualcomm Technologies Ireland | revised |  | R5-243554 |
| R5-242544 | Editorial changes to tables of 38.508-2 | Qualcomm Technologies Ireland | revised |  | R5-243496 |
| R5-242545 | Editorial changes to Applicability tables of 38.523-2 | Qualcomm Technologies Ireland | agreed |  |  |
| R5-242546 | Updates to SIB19 and ephemeris information for NGSO and GSO condition | Qualcomm Technologies Ireland, MediaTek Inc. | revised |  | R5-243597 |
| R5-242547 | Introduction of new TC NR-NTN - ntn-UlSyncValidityDuration, T430 Expiry | Qualcomm Technologies Ireland | revised |  | R5-243559 |
| R5-242548 | Updates to NTN test cases with missing NGSO configuration | Qualcomm Technologies Ireland | revised |  | R5-243560 |
| R5-242549 | Addition of new NR-NTN conditional handover condEventA4 test case | Qualcomm Technologies Ireland | revised |  | R5-243598 |
| R5-242550 | Addition of new NR NTN conditional handover CondEventD1 test case | Qualcomm Technologies Ireland | revised |  | R5-243561 |
| R5-242551 | Addition of new NR NTN conditional handover CondEventT1 test case | Qualcomm Technologies Ireland | revised |  | R5-243599 |
| R5-242552 | Addition of applicability for new NR NTN test cases | Qualcomm Technologies Ireland | withdrawn |  |  |
| R5-242553 | SR UE Conformance Test Aspects - Rel-17 Support of Uncrewed Aerial Systems | Qualcomm CDMA Technologies | noted |  |  |
| R5-242554 | SR UE Conformance Test Aspects - Rel-17 NR small data transmissions in INACTIVE state | Qualcomm CDMA Technologies | noted |  |  |
| R5-242555 | WP UE Conformance Test Aspects - Rel-17 Support of Uncrewed Aerial Systems | Qualcomm CDMA Technologies | noted |  |  |
| R5-242556 | WP UE Conformance Test Aspects - Rel-17 NR small data transmissions in INACTIVE state | Qualcomm CDMA Technologies | noted |  |  |
| R5-242557 | SIG WP for TEI16 - Redirection with MPS Indication | Qualcomm CDMA Technologies | reserved |  |  |
| R5-242558 | Rel-15 5GS maintenance SIG WP (NR-SA and ENDC) | Qualcomm CDMA Technologies | reserved |  |  |
| R5-242559 | Discussion paper – Way forward for R-17 legacy testing for NR-NTN only devices | Qualcomm Incorporated | noted |  |  |
| R5-242560 | Progress and Way Forward on SW Implementation on Machine Readable PICS and Applicability | Qualcomm Incorporated | revised |  | R5-243567 |
| R5-242561 | Corrections to conditional handover test case 8.1.4.4.1 | Qualcomm Incorporated | revised |  | R5-243502 |
| R5-242562 | Corrections to BWP test case 7.1.1.8.1 | Qualcomm Incorporated | revised |  | R5-243966 |
| R5-242563 | Addition of E-UTRA to NR MPS priority Indication test case | Qualcomm Incorporated | revised |  | R5-243504 |
| R5-242564 | Addition of applicability of E-UTRA to NR MPS priority Indication test case | Qualcomm Incorporated | revised |  | R5-243596 |
| R5-242565 | Addition of generci procedure for UUAA-MM | Qualcomm Incorporated | revised |  | R5-243574 |
| R5-242566 | Correction to Registration request and Registration accept information element for UAS support | Qualcomm Incorporated | withdrawn |  |  |
| R5-242567 | Challenges and Proposals for Testing eSIM-only Devices | Qualcomm Incorporated | revised |  | R5-243466 |
| R5-242568 | Tracking Rel-15 5GS EN-DC and SA NR Maintenance Workplan for protocol testing | Qualcomm Incorporated | revised |  | R5-243488 |
| R5-242569 | Corrections to applicability of NR supplemental uplink test case | Qualcomm Incorporated | revised |  | R5-243520 |
| R5-242570 | Corrections to applicability of Network Slice Admision Control mobility management aspects test cases | Qualcomm Incorporated | revised |  | R5-243521 |
| R5-242571 | General updates of clause 5 for R17 new CBW configurations | China Unicom, Nokia, CAICT | agreed |  |  |
| R5-242572 | General updates of TS 38.521-1 clause 5 for R16 CA configurations | China Unicom, KDDI | agreed |  |  |
| R5-242573 | General updates of TS 38.521-1 clause 5 for R17 CA configurations | China Unicom, Verizon, KDDI, WE Certification Oy, AT&T | revised |  | R5-243636 |
| R5-242574 | Update for derivation path of test procedure 4.9.39 and 4.9.40 | ZTE Corporation | agreed |  |  |
| R5-242575 | Correction for test procedure 4.9.41 and 4.9.42 | ZTE Corporation | revised |  | R5-243545 |
| R5-242576 | Editorial correction for generic procedure 4.5.4, 4.5.5 and 4.5.7 | ZTE Corporation | revised |  | R5-243489 |
| R5-242577 | Update to NTN DCQR cases | Mediatek India Technology Pvt. | revised |  | R5-243887 |
| R5-242578 | Correction to TC titles in applicability table of RRM event triggered reporting TCs with measurement gaps | Anritsu | revised |  | R5-243923 |
| R5-242579 | Correction to TC titles for event triggered reporting with measurement gap | Anritsu | agreed |  |  |
| R5-242580 | Correction to 7.6.15.3 FR2 SA event triggered reporting tests with concurrent gaps including TT | Anritsu | revised |  | R5-243744 |
| R5-242581 | Introduction of Test Tolerance analysis for 7.6.15.3 FR2 SA event triggered reporting tests for concurrent gaps | Anritsu | revised |  | R5-243745 |
| R5-242582 | Editorial correction for V2X TC 12.1.2.1 | ZTE Corporation | agreed |  |  |
| R5-242583 | Addition of UE General Capability of supporting NSAC | CATT | withdrawn |  |  |
| R5-242584 | Correction for V2X TC 12.1.2.2 | ZTE Corporation | agreed |  |  |
| R5-242585 | Correction for V2X TC 12.1.3.2 | ZTE Corporation | agreed |  |  |
| R5-242586 | Deletion of the Editors note in 8.3.1.1.1 | Mediatek India Technology Pvt., CMCC, Rohde&Schwarz | revised |  | R5-243925 |
| R5-242587 | Discussion on extending RAN5 scope to include MCPTT, MCVideo, and MCData Server Test Cases | FisrtNet, NIST, Ericsson, AT&T, Samsung, Element | noted |  |  |
| R5-242588 | Update to the simulation method of minimun test time | Mediatek India Technology Pvt. | agreed |  |  |
| R5-242589 | Update the minimum test time for 8.3.1.1.1 | Mediatek India Technology Pvt. | withdrawn |  |  |
| R5-242590 | Addition of test frequencies for new EN-DC comb within FR2 | KDDI Corporation | agreed |  |  |
| R5-242591 | Addition of UE capability for new EN-DC comb within FR2 | KDDI Corporation | withdrawn |  |  |
| R5-242592 | Correction to NR5GC testcase 6.1.2.13 | ROHDE & SCHWARZ | revised |  | R5-243497 |
| R5-242593 | Correction to LTE-IRAT test cases 11.2.11 | ROHDE & SCHWARZ | revised |  | R5-243474 |
| R5-242594 | Correction for MBS TC 14.2.4.2.1 | ZTE Corporation | revised |  | R5-243516 |
| R5-242595 | Correction for MBS TC 14.2.4.3.2 | ZTE Corporation | revised |  | R5-243517 |
| R5-242596 | Editorial correction for MBS TC of 14.2.4.1 | ZTE Corporation | revised |  | R5-243518 |
| R5-242597 | PRD21 CDS PC3 EN-DC DC_18A_n257G | KDDI Corporation | approved |  |  |
| R5-242598 | Editorial correction for Editor Note and reference section number in 4.3.1 | CAICT | agreed |  |  |
| R5-242599 | Core requirements alignment for UE co-existence of Inter-band within FR1 | CAICT | revised |  | R5-243665 |
| R5-242600 | Discussion on the Informative note for High test channel bandwidth | CAICT | revised |  | R5-243891 |
| R5-242601 | Addition of CBW 70MHz 90MHz and 100MHz for n40 | CAICT | agreed |  |  |
| R5-242602 | Correction to Table 6.F-1 | Ericsson | revised |  | R5-243647 |
| R5-242603 | Add IE PosSRS-BWA-RRC-Inactive | Ericsson | agreed |  |  |
| R5-242604 | Add IEs PosSRS-TxFrequencyHoppingRRC-Connected and PosSRS-TxFrequencyHoppingRRC-Inactive | Ericsson | agreed |  |  |
| R5-242605 | New WID on UE Conformance - Protocol enhancements for Mission Critical Services (MCPTT, MCVideo, MCData) for Rel-17 | Ericsson, FirstNet, NIST, AT&T, Element, Samsung | revised |  | R5-243458 |
| R5-242606 | Completion of NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case including test tolerance | Nokia | revised |  | R5-243751 |
| R5-242607 | Completion of NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case including test tolerance | Nokia | revised |  | R5-243749 |
| R5-242608 | Addition of Test Tolerances analysis for NR-U redirection test cases | Nokia | revised |  | R5-243752 |
| R5-242609 | Editorial correction to 7.6.3.6 test case | Nokia | agreed |  |  |
| R5-242610 | Editorial Correction of FR2 feMIMO test case | Nokia | revised |  | R5-243715 |
| R5-242611 | Incorrect clause number for test case 7.6.13.1 | Nokia | agreed |  |  |
| R5-242612 | Addition of cell configuration mapping for HST FR2 test cases | Nokia | agreed |  |  |
| R5-242613 | Addition of Test Case: UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA | Nokia | revised |  | R5-243804 |
| R5-242614 | Addition of UE Rx-Tx time difference measurements in RRC_INACTIVE test case | Nokia | revised |  | R5-243805 |
| R5-242615 | WP UE Conformance – NR support for dedicated spectrum less than 5MHz for FR1 | Nokia | noted |  | R5-243974 |
| R5-242616 | SR UE Conformance – NR support for dedicated spectrum less than 5MHz for FR1 | Nokia | noted |  |  |
| R5-242617 | GCF 3GPP TCL after GCF CAG#78 | Ericsson | noted |  |  |
| R5-242618 | Updates to Annex A.15.2 | Ericsson | revised |  | R5-243526 |
| R5-242619 | Updates to Annex A.15.2a | Ericsson | agreed |  |  |
| R5-242620 | Updates to Annex A.16.2a | Ericsson | withdrawn |  |  |
| R5-242621 | Updates to test case 7.4a | Ericsson, Rohde & Schwarz | revised |  | R5-243527 |
| R5-242622 | Updates to test case 7.6a | Ericsson, Rohde & Schwarz | revised |  | R5-243528 |
| R5-242623 | New MO speech EVS test case 12.23a | Ericsson | revised |  | R5-243523 |
| R5-242624 | New MT speech EVS test case 12.24a | Ericsson | revised |  | R5-243524 |
| R5-242625 | Applicability of new EVS test cases 12.23a and 12.24a | Ericsson | withdrawn |  |  |
| R5-242626 | WP UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2) | Ericsson | noted |  |  |
| R5-242627 | SR UE Conformance - Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2) | Ericsson | noted |  |  |
| R5-242628 | FR2 RRM test cases: Known Issue List | Ericsson | revised |  | R5-243731 |
| R5-242629 | Test Tolerance analysis for SRS carrier based switching test cases | Ericsson | agreed |  |  |
| R5-242630 | Test Tolerance analysis for event triggered reporting test without gap under non-DRX test case 7.6.2.10 | Ericsson | revised |  | R5-243759 |
| R5-242631 | Test Tolerance analysis for event triggered reporting test without gap under DRX test case 7.6.2.11 | Ericsson | agreed |  |  |
| R5-242632 | Correction of interruptions at NR SRS carrier based switching test case 4.5.2.8 including Test Tolerance | Ericsson | revised |  | R5-243754 |
| R5-242633 | Correction of interruptions at E-UTRA SRS carrier based switching test case 4.5.2.9 including Test Tolerance | Ericsson | revised |  | R5-243755 |
| R5-242634 | Correction of FR2 interruptions at SRS carrier based switching test case 5.5.2.7 and 5.5.2.8 | Ericsson | revised |  | R5-243706 |
| R5-242635 | Correction of SA FR1 interruptions at NR SRS carrier based switching test case 6.5.2.2 including Test Tolerance | Ericsson | revised |  | R5-243756 |
| R5-242636 | Correction of NR SA event triggered reporting tests with additional mandatory gap pattern test case 6.6.2.9 including Test Tolerance | Ericsson | agreed |  |  |
| R5-242637 | Correction of NR SA event triggered reporting tests for FR1 without gap when DRX is used test case 6.6.2.10 including Test Tolerance | Ericsson | agreed |  |  |
| R5-242638 | Correction of NR SA event triggered reporting tests for FR1 without gap when DRX is not used test case 6.6.2.11 including Test Tolerance | Ericsson | agreed |  |  |
| R5-242639 | Addition of FR2 SA event triggered reporting test without gap under non-DRX test case 7.6.2.10 including Test Tolerance | Ericsson | revised |  | R5-243757 |
| R5-242640 | Addition of FR2 SA event triggered reporting test without gap under DRX test case 7.6.2.11 including Test Tolerance | Ericsson | revised |  | R5-243758 |
| R5-242641 | Test Tolerance correction in Annex F for RRM enhancement test cases | Ericsson | agreed |  |  |
| R5-242642 | Correction of NR_U SA Intra-frequency measurements test cases | Ericsson | withdrawn |  |  |
| R5-242643 | Correction of NR_U SA Inter_RAT measurements test cases | Ericsson | agreed |  |  |
| R5-242644 | Correction of NR_U SA L1-RSRP measurements test cases | Ericsson | revised |  | R5-243748 |
| R5-242645 | Applicabilty correction of RRM enhancements test cases | Ericsson | revised |  | R5-243718 |
| R5-242646 | Cell configuration correction in Annex E for SA FR2 RRM enhancement test cases | Ericsson | agreed |  |  |
| R5-242647 | Minimum conformance requirements for FR2 SA event triggered reporting without gap | Ericsson | agreed |  |  |
| R5-242648 | Add IE Aerial-Config | Ericsson | revised |  | R5-243490 |
| R5-242649 | Add IE EpochTime | Ericsson | revised |  | R5-243491 |
| R5-242650 | Add IE LTM-TCI-Info | Ericsson | agreed |  |  |
| R5-242651 | Add IE N3C-RelayUE-Info | Ericsson | agreed |  |  |
| R5-242652 | Add IE NeedForInterruptionInfoNR | Ericsson | agreed |  |  |
| R5-242653 | Add IE PDU-SessionID | Ericsson | agreed |  |  |
| R5-242654 | Add IE PUCCH-CSI-Resource | Ericsson | agreed |  |  |
| R5-242655 | Add IE QFI | Ericsson | agreed |  |  |
| R5-242656 | Add IE RACH-ConfigTwoTA | Ericsson | agreed |  |  |
| R5-242657 | Add IE ReferenceConfiguration | Ericsson | agreed |  |  |
| R5-242658 | Add IE SelectedPSCellForCHO-WithSCG | Ericsson | agreed |  |  |
| R5-242659 | Add IE SI-RequestConfigRepetition | Ericsson | agreed |  |  |
| R5-242660 | Add IEs SRS-PosTx-Hopping and SRS-PosResourceSetLinkedForAggBW | Ericsson | agreed |  |  |
| R5-242661 | Add IE TN-AreaId | Ericsson | agreed |  |  |
| R5-242662 | Update of test applicability of NSAC test cases | CATT | revised |  | R5-243552 |
| R5-242663 | Update Test Configuration for NS17 Band 28 3MHz | Rakuten Mobile, Inc | agreed |  |  |
| R5-242664 | WP - UE Conformance - Further enhancements on MIMO for NR | Samsung | noted |  |  |
| R5-242665 | SR - UE Conformance - Further enhancements on MIMO for NR | Samsung | noted |  |  |
| R5-242666 | WP - UE Conformance - NR support for high speed train scenario in frequency range 2 | Samsung | noted |  |  |
| R5-242667 | SR - UE Conformance - NR support for high speed train scenario in frequency range 2 | Samsung | noted |  |  |
| R5-242668 | Additional annex Channel Profile of FR2 HST-DPS | Samsung | agreed |  |  |
| R5-242669 | Additional applicability for Inter-cell SSB based L1-RSRP measurements | Samsung | revised |  | R5-243716 |
| R5-242670 | Addition test case for Inter-cell SSB based L1-RSRP Reporting | Samsung | revised |  | R5-243880 |
| R5-242671 | Adding TRP specific Minimum conformance requirements | Samsung | revised |  | R5-243705 |
| R5-242672 | Additional test case for Enhanced Beam correspondence for PC6 | Samsung | revised |  | R5-243652 |
| R5-242673 | Additional demodulation test case for FR2 high speed train | Samsung | revised |  | R5-243881 |
| R5-242674 | Additional applicability for HST FR2 test cases | Samsung | revised |  | R5-243717 |
| R5-242675 | Update UE pre-test configurations for test case 6.5.3.1 | China Telecom | agreed |  |  |
| R5-242676 | Update test case 6.5.3.2 | China Telecom | agreed |  |  |
| R5-242677 | Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN | China Telecom | revised |  | R5-243540 |
| R5-242678 | Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 Enpn | China Telecom | revised |  | R5-243541 |
| R5-242679 | Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 Enpn | China Telecom | agreed |  |  |
| R5-242680 | Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 Enpn | China Telecom | revised |  | R5-243951 |
| R5-242681 | Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN | China Telecom | agreed |  |  |
| R5-242682 | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN | China Telecom | revised |  | R5-243542 |
| R5-242683 | Addition of new test case 9.1.11.7 for Rel-17 eNPN | China Telecom | agreed |  |  |
| R5-242684 | Scoping NR SA applicable TCs for SNPN-only Ues | China Telecom, MediaTek | revised |  | R5-243962 |
| R5-242685 | WP UE Conformance – Rel-17 Enhancement of Private Network Support for NG-RAN including CT aspects RAN5#103 | China Telecom, Qualcomm | noted |  |  |
| R5-242686 | SR UE Conformance – Rel-17 Enhancement of Private Network Support for NG-RAN including CT aspects RAN5#103 | China Telecom, Qualcomm | noted |  |  |
| R5-242687 | WP UE Conformance – Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations RAN5#103 | China Telecom, Huawei | noted |  |  |
| R5-242688 | SR UE Conformance – Rel-18 High Power UE for NR CA and DC; and NR and LTE DC Configurations RAN5#103 | China Telecom, Huawei | noted |  |  |
| R5-242689 | WP UE Conformance – Rel-17 LTE CA Configurations RAN5#103 | China Telecom, ZTE | noted |  |  |
| R5-242690 | SR UE Conformance – Rel-17 LTE CA Configurations RAN5#103 | China Telecom, ZTE | noted |  |  |
| R5-242691 | New WID on UE Conformance – Enhanced support of Non-Public Networks Phase 2 | China Telecom, Qualcomm, ZTE | revised |  | R5-243459 |
| R5-242692 | Discussion on minimum test time simulation results for 8.3.1.1.1 | Mediatek India Technology Pvt. | revised |  | R5-243890 |
| R5-242693 | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | withdrawn |  |  |
| R5-242694 | Addition of test applicability for Rel-16 RRM EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | revised |  | R5-243719 |
| R5-242695 | Addition of new test case 5.6.7.1 EN-DC event triggered reporting test without gap under non-DRX | Sporton | revised |  | R5-243707 |
| R5-242696 | Addition of new test case 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used | Sporton | revised |  | R5-243708 |
| R5-242697 | Addition of new test case 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Sporton | revised |  | R5-243709 |
| R5-242698 | Addition of new test case 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Sporton | revised |  | R5-243710 |
| R5-242699 | Addition of MU and test tolerance in Annex F for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | revised |  | R5-244002 |
| R5-242700 | Addition of test tolerance analysis for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | revised |  | R5-244004 |
| R5-242701 | Addition of Inter-band DL CA and CA_30A-48A band for A-MPR & Spurious emission test cases | Sporton, Bureau Veritas ADT | agreed |  |  |
| R5-242702 | Addition of test point analysis for Inter-band DL CA and UL CA_30A-48A for A-MPR & Spurious emission | Sporton | withdrawn |  |  |
| R5-242703 | Update to R17 ATSSS test case 11.9.1 | China Telecom, MediaTek | revised |  | R5-243553 |
| R5-242704 | Addition of applicability for eNPN test cases | China Telecom | revised |  | R5-243963 |
| R5-242705 | Addition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A Tx test cases | WE Certification Oy, AT&T | revised |  | R5-243655 |
| R5-242706 | Addition of reference sensitivity tests for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | revised |  | R5-243656 |
| R5-242707 | Definition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A applicability to protocol testing | WE Certification Oy, AT&T | withdrawn |  |  |
| R5-242708 | Definition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A applicability to protocol testing | WE Certification Oy, AT&T | agreed |  |  |
| R5-242709 | Addition of capability declaration for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | revised |  | R5-243811 |
| R5-242710 | Test point analysis for spurious emissions CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | agreed |  |  |
| R5-242711 | Addition of DC_66A_n66 BCS1, DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A to Ch5 | WE Certification Oy, AT&T | agreed |  |  |
| R5-242712 | Addition of test point analysis for Inter-band DL CA and UL CA_30A-48A for A-MPR & Spurious emission | Sporton | agreed |  |  |
| R5-242713 | Addition of capability declaration for DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A | WE Certification Oy, AT&T | revised |  | R5-243657 |
| R5-242714 | Definition of DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A applicability to protocol testing | WE Certification Oy, AT&T | agreed |  |  |
| R5-242715 | Correction of test bandwidth for CA_n2(2A) protocol testing | WE Certification Oy, AT&T, Anritsu | withdrawn |  | - |
| R5-242716 | Correction of Reference Sensitivity test for DC_30A-n77A | WE Certification Oy, AT&T, Anritsu | revised |  | R5-243670 |
| R5-242717 | PRD21 CDS: NRCA PC3 8 NR-CA bands | WE Certification Oy, AT&T | approved |  |  |
| R5-242718 | PRD21 CDS: ENDC PC3 6 EN-DC bands | WE Certification Oy, AT&T | approved |  |  |
| R5-242719 | Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs | NTT DOCOMO INC. | withdrawn |  |  |
| R5-242720 | Correction to NTN TA report test case 22.3.1.13 | Qualcomm Incorporated, Rohde & Schwarz | revised |  | R5-243480 |
| R5-242721 | Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs | NTT DOCOMO INC. | withdrawn |  |  |
| R5-242722 | Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs | NTT DOCOMO INC. | withdrawn |  |  |
| R5-242723 | Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs | NTT DOCOMO INC. | withdrawn |  |  |
| R5-242724 | Correct typo for clause 6.5B.3.2.1.4 and 6.5B.3.4.1.4.1 | SGS Wireless | agreed |  |  |
| R5-242725 | Correction condition C111 for 38.521-1 TC6.4.2.5 Rel-16 | SGS Wireless | withdrawn |  |  |
| R5-242726 | Correction condition F011 for 38.533 TC16.4.1.1 | SGS Wireless | revised |  | R5-243910 |
| R5-242727 | Correct Table 4.5.2.1.4.1-2 & Table 4.5.2.2.4.1-2 & Table 4.5.2.3.4.1-2 & Table 4.5.2.4.4.1-2 & Table 4.5.2.5.4.1-2 of test frequency | SGS Wireless | agreed |  |  |
| R5-242728 | Correct Table A.4.5-1 item 60 due to it doesnt match reference | SGS Wireless | revised |  | R5-243911 |
| R5-242729 | On the QoQZ MU for n262 | ROHDE & SCHWARZ | noted |  |  |
| R5-242730 | Update of the QoQZ MU | ROHDE & SCHWARZ | agreed |  |  |
| R5-242731 | Removal of the reference to efficiency calibration in QoQZ | ROHDE & SCHWARZ | agreed |  |  |
| R5-242732 | Addition of MOP requirements for 2UL CA 3A-42A | NTT DOCOMO INC. | agreed |  |  |
| R5-242733 | Addition of PICS for ING_5GS test cases | Qualcomm Incorporated, China Telecom | revised |  | R5-243530 |
| R5-242734 | Addition of ING_5GS test case 9.3.1.5 | Qualcomm Incorporated | agreed |  |  |
| R5-242735 | Void ING_5GS test case 11.1.10 | Qualcomm Incorporated | agreed |  | - |
| R5-242736 | Applicability updates to ING_5GS test cases | Qualcomm Incorporated, China Telecom | revised |  | R5-243960 |
| R5-242737 | Updates to Annex A.5.3 | Starpoint, TDIA | agreed |  |  |
| R5-242738 | Correction to UUAA-SM generic test procedure 4.5A.31 | Qualcomm Incorporated | agreed |  |  |
| R5-242739 | Correction to EPS UAS test cases | Qualcomm Incorporated | agreed |  |  |
| R5-242740 | Correction to NSAC test case 9.1.12.3 | Anritsu EMEA Ltd | agreed |  |  |
| R5-242741 | Correction to NR RRC UL segmentation test case 8.1.5.9.1 | Qualcomm Incorporated | revised |  | R5-243505 |
| R5-242742 | Correction to NR IMS TC 7.24b - Forking | Huawei, Hisilicon | revised |  | R5-243529 |
| R5-242743 | TS 38.523-1 Tracker status before RAN5-103 | Huawei, Hisilicon | noted |  |  |
| R5-242744 | TS 36.523-1 Tracker status before RAN5-103 | Huawei, Hisilicon | noted |  |  |
| R5-242745 | TS 38.523-1 Tracker status after RAN5-103 | Huawei, Hisilicon | reserved |  |  |
| R5-242746 | TS 36.523-1 Tracker status after RAN5-103 | Huawei, Hisilicon | reserved |  |  |
| R5-242747 | SR of UE Conformance - IMS Data Channel test after RAN5 103 | Huawei | revised |  | R5-243993 |
| R5-242748 | WP of UE Conformance - IMS Data Channel test after RAN5 103 | Huawei | noted |  |  |
| R5-242749 | Correction to generic procedure A.16.1a | Qualcomm Incorporated | agreed |  |  |
| R5-242750 | Correction to VONR test case 7.10 | Qualcomm Incorporated | agreed |  |  |
| R5-242751 | Correction to VONR test case 7.13 | Qualcomm Incorporated | agreed |  |  |
| R5-242752 | Correction to test case 7.16 | Qualcomm Incorporated | agreed |  |  |
| R5-242753 | Correction to VONR test case 7.17 | Qualcomm Incorporated | agreed |  |  |
| R5-242754 | Correction to VONR test case 7.25 | Qualcomm Incorporated | agreed |  |  |
| R5-242755 | Correction to test case 8.37 | Qualcomm Incorporated | agreed |  |  |
| R5-242756 | Correction to test case 8.38 | Qualcomm Incorporated | agreed |  |  |
| R5-242757 | Correction to NR NTN TA report test case 7.1.1.10.4 | Qualcomm Incorporated | agreed |  |  |
| R5-242758 | Correction to LTE RACS Test case 9.2.5.1 | Anritsu EMEA Ltd | revised |  | R5-243470 |
| R5-242759 | Addition of NR NTN PICS | Qualcomm Incorporated | withdrawn |  |  |
| R5-242760 | Addition of system information combination for inter frequency NTN scenario | Qualcomm Incorporated | revised |  | R5-243555 |
| R5-242761 | Correction to eNS test case 9.3.1.4 | Anritsu EMEA Ltd, MediaTek | revised |  | R5-243514 |
| R5-242762 | Correction to RACS test case 9.1.9.7 | Anritsu EMEA Ltd | withdrawn |  |  |
| R5-242763 | Add IEs SL-BWP-DiscPoolConfig and SL-BWP-DiscPoolConfigCommon | Ericsson | agreed |  |  |
| R5-242764 | Add IEs SL-BWP-PRS-PoolConfig and SL-BWP-PRS-PoolConfigCommon | Ericsson | agreed |  |  |
| R5-242765 | Addition of NR NTN PICS | Qualcomm Incorporated | agreed |  |  |
| R5-242766 | Applicability updates to NR NTN test cases | Qualcomm Incorporated | revised |  | R5-243950 |
| R5-242767 | Addition of NR NTN cell reselection test case 6.1.2.18a | Qualcomm Incorporated | revised |  | R5-243562 |
| R5-242768 | Addition of NR NTN cell reselection test case 6.1.2.18b | Qualcomm Incorporated | revised |  | R5-243563 |
| R5-242769 | Addition of NR NTN MAC test case 7.1.1.5.6 | Qualcomm Incorporated | agreed |  |  |
| R5-242770 | Addition of NR NTN RLC t-reassembly timer test case 7.1.2.3.9a | Qualcomm Incorporated | revised |  | R5-243564 |
| R5-242771 | Update of Logical Channel Config Table 4.6.3-66 | Qualcomm Incorporated | revised |  | R5-243536 |
| R5-242772 | Update channel access configurations for shared spectrum | Qualcomm Incorporated | agreed |  |  |
| R5-242773 | Correction to eNS test case 10.1.8.1 | Anritsu EMEA Ltd, MediaTek | agreed |  |  |
| R5-242774 | Correction to RACS test case 9.1.9.7 | Anritsu EMEA Ltd | withdrawn |  |  |
| R5-242775 | PRD-17 on Guidance to Work Item Codes (post RAN#104 version) | Bureau Veritas ADT (Rapporteur) | email approved |  |  |
| R5-242776 | RAN5#103 summary of changes to RAN5 test cases with potential impact on GCF and PTCRB | Bureau Veritas ADT | reserved |  |  |
| R5-242777 | PRD21 CDS NR band TDD FR2 n260 with PC3 | Bureau Veritas ADT | approved |  |  |
| R5-242778 | PRD20 Completed CA_2A-2A-29A-30A-66A_5DL-1UL without UL CA | Bureau Veritas ADT, Sporton International | available |  |  |
| R5-242779 | PRD20 Completed CA_2A-2A-29A-66A-66A_5DL-1UL without UL CA | Bureau Veritas ADT, Sporton International | available |  |  |
| R5-242780 | PRD20 Completed WP_CA_2A-29A-66A-66A_CA_2A-2A-29A-66A_4DL-1UL_DL only FBs | Bureau Veritas ADT, Sporton International | available |  |  |
| R5-242781 | PRD20 Completed CA_30A-48A 2DL-2UL with UL CA_30A-48A | Bureau Veritas ADT, Sporton International | available |  |  |
| R5-242782 | Update of Receiver test cases for multiple CA combos | Bureau Veritas ADT, Sporton International | agreed |  |  |
| R5-242783 | Removal of definition and abbreviation for IXIT and PIXIT | Bureau Veritas ADT | agreed |  |  |
| R5-242784 | Removal of defination and abbreviation for IXIT and PIXIT | Bureau Veritas ADT, CATT | agreed |  |  |
| R5-242785 | Correction to IE name higherPowerLimit used in test cases | Bureau Veritas ADT | agreed |  |  |
| R5-242786 | Update of Channel bandwidth in Default NG-RAN RRC message and information elements contents | Bureau Veritas ADT | agreed |  |  |
| R5-242787 | Update of Rx antenna ports associated declaration | Bureau Veritas ADT | agreed |  |  |
| R5-242788 | Update of editors note for test frequencies | Bureau Veritas ADT | revised |  | R5-243603 |
| R5-242789 | Correction to applicability of 5G test cases | Bureau Veritas ADT, Keysight | revised |  | R5-243720 |
| R5-242790 | Clarification On Beam Correspondence requirement for EN-DC | Apple AB Denmark | agreed |  |  |
| R5-242791 | Introducing framework for Beam Correspondence during Initial Access in IDLE related Tx Power tests | Apple AB Denmark | revised |  | R5-243841 |
| R5-242792 | On Beam Correspondence initial access in Idle and FR2 RRM RACH Test | Apple AB Denmark | revised |  | R5-243702 |
| R5-242793 | Corrrection to NR5GC CAG test case 6.5.2.1 | Keysight Technologies UK | revised |  | R5-243588 |
| R5-242794 | PRD21 CDS: PC2 ENDC 3 bands combos with n78 and n79 | NTT DOCOMO INC. | revised |  | R5-243988 |
| R5-242795 | PRD21 CDS: PC3 ENDC 3 bands combos with n78 and n79 | NTT DOCOMO INC. | revised |  | R5-243989 |
| R5-242796 | Update to FR1 EN-DC Configurations for n78 and n79 | NTT DOCOMO INC. | agreed |  |  |
| R5-242797 | Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC3 | NTT DOCOMO INC. | revised |  | R5-243825 |
| R5-242798 | Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC2 | NTT DOCOMO INC. | revised |  | R5-243828 |
| R5-242799 | Addition of new combination of system information blocks for 5G Prose communication | TDIA, CATT | agreed |  |  |
| R5-242800 | Editorial update of SIB12 in 4.6.2 | TDIA, CATT | agreed |  |  |
| R5-242801 | Update of Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side | TDIA, CATT | revised |  | R5-243547 |
| R5-242802 | Update of Table 4.6.1-22 RRCSetupComplete | TDIA, CATT | agreed |  |  |
| R5-242803 | Update of Table 4.6.1A-2A RemoteUEInformationSidelink | TDIA, CATT | agreed |  |  |
| R5-242804 | Update of Table 4.6.1A-8 uuMessageTransferSidelink | TDIA, CATT | revised |  | R5-243548 |
| R5-242805 | Addition of 5G ProSe discovery and security messages | TDIA, CATT | agreed |  |  |
| R5-242806 | Update of 5G ProSe direct discovery messages | TDIA,CATT | agreed |  |  |
| R5-242807 | Addition of Default HTTP messages for communication with the 5G ProSe | TDIA, CATT | revised |  | R5-243549 |
| R5-242808 | Addition of Default TLS message and information element contents | TDIA, CATT | revised |  | R5-243550 |
| R5-242809 | Addition of spec reference of 3GPP TS 33.310, 33.503 and IETF RFC 5246 for 5G ProSe information elements | TDIA, CATT | revised |  | R5-243551 |
| R5-242810 | Update of Default settings of UICC and USIM for 5G ProSe | TDIA, CATT | agreed |  |  |
| R5-242811 | Update of Test procedure for 5G ProSe U2N Relay Discovery | TDIA, CATT | agreed |  |  |
| R5-242812 | Correction of RRC IDLE TC 6.1.2.23-MFBI | Huawei, HiSilicon | agreed |  |  |
| R5-242813 | Correction of MAC TC 7.1.1.1.4-Beam Failure | Huawei, HiSilicon | revised |  | R5-243589 |
| R5-242814 | Update applicability of MAC TC 7.1.1.1.4-Beam Failure | Huawei, HiSilicon | agreed |  |  |
| R5-242815 | Correction to NR test case 8.1.5.8.1-processing delay | Huawei, HiSilicon | revised |  | R5-243506 |
| R5-242816 | Correction to NEDC test case 8.2.2.8.3 | Huawei, HiSilicon, MCC TF160, Mediatek | revised |  | R5-243511 |
| R5-242817 | Correction to NR 5GC Multilayer TC 11.1.7-Emergency Services Fallback to EPS with redirection | Huawei, HiSilicon | revised |  | R5-243591 |
| R5-242818 | Correction to NR 5GC Multilayer TC 11.4.12-Emergency Services | Huawei, HiSilicon | revised |  | R5-243593 |
| R5-242819 | Correction of NR eCall TC 11.5.5-Limited service | Huawei, HiSilicon | revised |  | R5-243594 |
| R5-242820 | Correction of NR eCall TC 11.5.12-Domain Selection | Huawei, HiSilicon | withdrawn |  |  |
| R5-242821 | Correction of SENSE eMTC TC 6.1.1.12-PLMN selection | Huawei, HiSilicon,TDIA | agreed |  |  |
| R5-242822 | Correction of SENSE eMTC TC 6.1.1.13-PLMN selection Exception | Huawei, HiSilicon,TDIA | agreed |  |  |
| R5-242823 | Correction of SENSE eMTC TC 6.1.1.14-Periodic SENSE | Huawei, HiSilicon,TDIA | agreed |  |  |
| R5-242824 | Correction of SENSE NB-IoT TC 22.2.14-PLMN selection | Huawei, HiSilicon,TDIA | agreed |  |  |
| R5-242825 | Correction of SENSE NB-IoT TC 22.2.15-PLMN selection Exception | Huawei, HiSilicon,TDIA | agreed |  |  |
| R5-242826 | Correction of SENSE NB-IoT TC 22.2.16-Periodic SENSE | Huawei, HiSilicon,TDIA | agreed |  |  |
| R5-242827 | Add new PICS for MUSIM gap | Huawei, HiSilicon | withdrawn |  |  |
| R5-242828 | Correction of MUSIM gap TC 8.1.2.1.6 | Huawei, HiSilicon | revised |  | R5-243958 |
| R5-242829 | Correction of MUSIM NAS TC 9.1.7.3-9.1.7.4 | Huawei, HiSilicon | withdrawn |  |  |
| R5-242830 | Update test applicability for MUSIM TC | Huawei, HiSilicon | revised |  | R5-243959 |
| R5-242831 | Discussion to add new RedCap HD-FDD TC | Huawei, HiSilicon | revised |  | R5-243484 |
| R5-242832 | Add new PICS for RedCap HD-FDD TC | Huawei, HiSilicon | revised |  | R5-243495 |
| R5-242833 | Add test applicability for new RedCap HD-FDD TC | Huawei, HiSilicon | withdrawn |  | - |
| R5-242834 | Add test applicability for new RedCap HD-FDD TC | Huawei, HiSilicon | revised |  | R5-243586 |
| R5-242835 | WP UE Conformance - Further NR coverage enhancements | China Telecom | noted |  |  |
| R5-242836 | SR UE Conformance - Further NR coverage enhancements | China Telecom | noted |  |  |
| R5-242837 | Work plan: UE Conformance - NR sidelink enhancement | CATT | noted |  |  |
| R5-242838 | SR UE Conformance - NR sidelink enhancement | CATT | noted |  |  |
| R5-242839 | Work plan: UE Conformance - NR Sidelink Relay | CATT | noted |  |  |
| R5-242840 | SR UE Conformance - NR Sidelink Relay | CATT | noted |  |  |
| R5-242841 | New WID on UE Conformance - Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC) | Apple, ROHDE & SCHWARZ, Vivo | revised |  | R5-243460 |
| R5-242842 | WP UE Conformance – Multi-carrier enhancements for NR | China Telecom, Huawei, HiSilicon | noted |  |  |
| R5-242843 | SR UE Conformance – Multi-carrier enhancements for NR | China Telecom, Huawei, HiSilicon | noted |  |  |
| R5-242844 | Addition of test case 6.3H.3.1 Minimum output power for inter-band UL CA with UL MIMO | TTA | revised |  | R5-243626 |
| R5-242845 | Addition of test case 6.5H.3.1 Occupied bandwidth for inter-band UL CA with UL MIMO | TTA | revised |  | R5-243627 |
| R5-242846 | Correction to Test Bands Selection Condition for Performance test case 5.3.2.2.5 | TTA | agreed |  |  |
| R5-242847 | WP UE Conformance - Further Multi-RAT Dual-Connectivity enhancement | Huawei, HiSilicon | noted |  |  |
| R5-242848 | SR UE Conformance - Further Multi-RAT Dual-Connectivity enhancement | Huawei, HiSilicon | noted |  |  |
| R5-242849 | WP UE Conformance - 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC | Huawei, HiSilicon | noted |  |  |
| R5-242850 | SR UE Conformance - 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC | Huawei, HiSilicon | noted |  |  |
| R5-242851 | WP UE Conformance - Network energy savings for NR | Huawei, HiSilicon | noted |  |  |
| R5-242852 | SR UE Conformance - Network energy savings for NR | Huawei, HiSilicon | noted |  |  |
| R5-242853 | WP UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 | Huawei, HiSilicon | noted |  |  |
| R5-242854 | SR UE Conformance - Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 | Huawei, HiSilicon | noted |  |  |
| R5-242855 | Update of 7.3B.2 reference sensitivity for inter-band EN-DC | Huawei, HiSilicon | revised |  | R5-243672 |
| R5-242856 | Update of UE capability name for higher power limit | Huawei, HiSilicon | withdrawn |  |  |
| R5-242857 | Update of message contents for 8.2.3.18.4 conditional PSCell change | Huawei, HiSilicon | revised |  | R5-243580 |
| R5-242858 | Correction to SUL n83 test frequencies in Table 4.3.1.1.1.83-1 | Qualcomm Tech. Netherlands B.V | agreed |  |  |
| R5-242859 | Editorial correction for Specific message contents in 9.2.1.1.20 | NTT DOCOMO, INC. | revised |  | R5-243471 |
| R5-242860 | Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A | NTT DOCOMO INC. | withdrawn |  |  |
| R5-242861 | Editorial correction for Specific message contents in 9.2.3.1.22 | NTT DOCOMO, INC. | revised |  | R5-243472 |
| R5-242862 | Editorial correction for test procedure sequence in 8.1.1.3 | NTT DOCOMO, INC. | agreed |  |  |
| R5-242863 | Correction to SDT RRM test case 6.2.1 with TT | Huawei, HiSilicon | revised |  | R5-244001 |
| R5-242864 | Correction to SDT RRM test case 7.2.1.1 with TT | Huawei, HiSilicon | withdrawn |  |  |
| R5-242865 | Addition of PICS for MCE RRM test cases | Huawei, HiSilicon | agreed |  |  |
| R5-242866 | Addition of applicability for MCE RRM test cases | Huawei, HiSilicon | revised |  | R5-243914 |
| R5-242867 | Addition of MCE RRM test case 6.5.7D.1 | Huawei, HiSilicon | agreed |  |  |
| R5-242868 | Addition of MCE RRM test case 6.5.7D.2 | Huawei, HiSilicon | agreed |  |  |
| R5-242869 | Addition of MCE RRM test case 6.5.7D.3 | Huawei, HiSilicon | agreed |  |  |
| R5-242870 | Correction to Annex E for MCE RRM test cases | Huawei, HiSilicon | agreed |  |  |
| R5-242871 | Correction to defalut configuration of R15 RACH RRM test cases | Huawei, HiSilicon | withdrawn |  |  |
| R5-242872 | Correction to R15 L1-RSRP absolute accuracy RRM test cases | Huawei, HiSilicon | agreed |  |  |
| R5-242873 | Correction to R15 RACH RRM test cases and RMCs | Huawei, HiSilicon | revised |  | R5-243916 |
| R5-242874 | Correction to R15 reestablishment RRM test case 6.3.2.1.1 | Huawei, HiSilicon | revised |  | R5-243917 |
| R5-242875 | Correction to R15 SCell activation test cases | Huawei, HiSilicon, Starpoint | agreed |  |  |
| R5-242876 | Correction to R16 relaxed measurement RRM test cases | Huawei, HiSilicon, Starpoint | agreed |  |  |
| R5-242877 | Correction to applicability for RedCap RRM test cases | Huawei, HiSilicon | revised |  | R5-243915 |
| R5-242878 | Correction to RedCap RRM test case 16.1.1.5 and 16.1.1.7 | Huawei, HiSilicon, Starpoint | agreed |  |  |
| R5-242879 | Correction to RedCap RRM test case 16.1.2.5 and 16.1.2.6 | Huawei, HiSilicon, Starpoint, Qualcomm | revised |  | R5-243918 |
| R5-242880 | Correction to RedCap RRM test case 16.2.1.1 with TT | Huawei, HiSilicon | revised |  | R5-244014 |
| R5-242881 | Correction to RedCap RRM test case 16.2.1.2 with TT | Huawei, HiSilicon | revised |  | R5-244015 |
| R5-242882 | Correction to RedCap RRM test case 17.2.1.1 with TT | Huawei, HiSilicon | withdrawn |  |  |
| R5-242883 | Correction to RedCap RRM test case 17.3.1.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-242884 | Correction to RedCap RRM test case 17.3.1.2 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-242885 | Correction to RedCap RRM test case 17.3.2.3.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-242886 | Correction to RedCap RRM test case 17.4.1.1 with TT | Huawei, HiSilicon | revised |  | R5-243774 |
| R5-242887 | Correction to RedCap RRM test case 17.4.3.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-242888 | Correction to RedCap RRM test case 17.5.1.5 with TT | Huawei, HiSilicon | withdrawn |  |  |
| R5-242889 | Correction to RedCap RRM test case 17.5.1.6 with TT | Huawei, HiSilicon | withdrawn |  |  |
| R5-242890 | Correction to RedCap RRM test case 17.5.1.7 with TT | Huawei, HiSilicon | withdrawn |  |  |
| R5-242891 | Correction to RedCap RRM test case 17.5.1.8 with TT | Huawei, HiSilicon | revised |  | R5-243701 |
| R5-242892 | Correction to RedCap RRM test case 17.5.4.1.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-242893 | Correction to RedCap RRM test case 17.5.4.2.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-242894 | Correction to RedCap RRM test case 17.5.5.1.1 with TT | Huawei, HiSilicon | revised |  | R5-243775 |
| R5-242895 | Correction to RedCap RRM test case 17.5.5.2.1 with TT | Huawei, HiSilicon | agreed |  |  |
| R5-242896 | Correction to RedCap RRM test case 17.7.1.1 with TT | Huawei, HiSilicon | revised |  | R5-243776 |
| R5-242897 | Correction to RedCap RRM test case 17.7.1.2 with TT | Huawei, HiSilicon | revised |  | R5-243777 |
| R5-242898 | Correction to RedCap RRM test case 17.7.2.1 with TT | Huawei, HiSilicon | revised |  | R5-243778 |
| R5-242899 | Correction to RedCap RRM test case 17.7.2.2 with TT | Huawei, HiSilicon | revised |  | R5-243779 |
| R5-242900 | Correction to RedCap RRM test case 17.7.3.1 with TT | Huawei, HiSilicon | revised |  | R5-243780 |
| R5-242901 | Correction to RedCap RRM test case 17.7.3.2 with TT | Huawei, HiSilicon | revised |  | R5-243781 |
| R5-242902 | Correction to RedCap RRM test case 17.7.4.1 with TT | Huawei, HiSilicon | revised |  | R5-243782 |
| R5-242903 | Addition of RedCap RRM test case 17.7.4.2 with TT | Huawei, HiSilicon | revised |  | R5-243783 |
| R5-242904 | Correction to Annex E for RedCap RRM test cases | Huawei, HiSilicon | agreed |  |  |
| R5-242905 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | revised |  | R5-243746 |
| R5-242906 | TT analysis for RedCap RRM test case 16.2.1.1 | Huawei, HiSilicon | revised |  | R5-243747 |
| R5-242907 | TT analysis for RedCap RRM test case 16.2.1.2 | Huawei, HiSilicon | revised |  | R5-244017 |
| R5-242908 | TT analysis for RedCap RRM test case 17.2.1.1 | Huawei, HiSilicon | withdrawn |  |  |
| R5-242909 | Correction to L2U redirection RRM test case 6.3.12 | Huawei, HiSilicon | agreed |  |  |
| R5-242910 | Correction to LTE V2V and V2X RRM test cases | Huawei, HiSilicon | revised |  | R5-243809 |
| R5-242911 | Addition of NR sidelink SIG test case 12.1.1.1 | Huawei, HiSilicon | revised |  | R5-243534 |
| R5-242912 | Correction to NR sidelink SIG test case 12.2.5.1 | Huawei, HiSilicon | agreed |  |  |
| R5-242913 | Correction to NR sidelink SIG test case 12.2.5.2 | Huawei, HiSilicon | revised |  | R5-243535 |
| R5-242914 | Addition of applicability for NR sidelink SIG test cases | Huawei, HiSilicon | agreed |  | - |
| R5-242915 | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | revised |  | R5-243732 |
| R5-242916 | Addition of REFSENS TP analysis for 2CC ENDC for n78A and n79A | NTT DOCOMO INC. | agreed |  |  |
| R5-242917 | Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A | NTT DOCOMO INC. | revised |  | R5-243827 |
| R5-242918 | Correction to initial condition for SUL Tx switching TC 6.3C.3.2.4.1 | Qualcomm Tech. Netherlands B.V | agreed |  |  |
| R5-242919 | Addition of TE and UE connection diagram for 3Tx NR CA and EN-DC | Huawei, HiSilicon | agreed |  |  |
| R5-242920 | Addition of physical layer baseline implementation capabilities for 3Tx NR CA and EN-DC | Huawei, HiSilicon | revised |  | R5-243606 |
| R5-242921 | Update of configured transmitted power requirements for 3Tx NR CA | Huawei, HiSilicon | agreed |  |  |
| R5-242922 | Addition of new test case 6.2H.3.1 MOP for inter-band CA with MIMO | Huawei, HiSilicon | agreed |  |  |
| R5-242923 | Addition of new test cases 6.2H.3.x for inter-band CA with MIMO | Huawei, HiSilicon | agreed |  |  |
| R5-242924 | Addition of new test case 6.2L.3.1 MOP for inter-band CA with TxD | Huawei, HiSilicon | agreed |  |  |
| R5-242925 | Addition of new test cases 6.2L.3.x for inter-band CA with TxD | Huawei, HiSilicon | revised |  | R5-243628 |
| R5-242926 | Addition of MU and TT analysis for 3Tx NR CA test cases | Huawei, HiSilicon | agreed |  |  |
| R5-242927 | Addition of general informationfor 3Tx NR CA | Huawei, HiSilicon | revised |  | R5-243878 |
| R5-242928 | Addition of new test case 6.2H.1.3 MOP for inter-band EN-DC with MIMO | Huawei, HiSilicon | agreed |  |  |
| R5-242929 | Addition of new test cases 6.2H.x.3 for inter-band EN-DC with MIMO | Huawei, HiSilicon | agreed |  |  |
| R5-242930 | Addition of new test case 6.2L.1.3 MOP for inter-band EN-DC with TxD | Huawei, HiSilicon | agreed |  |  |
| R5-242931 | Addition of new test cases 6.2L.x.3 for inter-band EN-DC with TxD | Huawei, HiSilicon | agreed |  |  |
| R5-242932 | Addition of applicabilities for 3Tx NR CA and EN-DC test cases | Huawei, HiSilicon | agreed |  |  |
| R5-242933 | Addition of test point analysis for 6.2H.3.1 MOP for inter-band CA with MIMO | Huawei, HiSilicon | agreed |  |  |
| R5-242934 | Addition of test point analysis for 6.2L.3.1 MOP for inter-band CA with TxD | Huawei, HiSilicon | agreed |  |  |
| R5-242935 | Addition of test point analysis for 6.2H.1.3 MOP for inter-band EN-DC with MIMO | Huawei, HiSilicon | agreed |  |  |
| R5-242936 | Addition of test point analysis for 6.2L.1.3 MOP for inter-band EN-DC with TxD | Huawei, HiSilicon | agreed |  |  |
| R5-242937 | Addition of NR P-CSCF restoration test case 10.1.2.3 | Qualcomm Incorporated, Orange | agreed |  |  |
| R5-242938 | Applicability updates for P-CSCF restoration test cases | Qualcomm Incorporated, Orange | agreed |  |  |
| R5-242939 | Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC3 | NTT DOCOMO INC. | withdrawn |  | - |
| R5-242940 | Addition of EPS P-CSCF restoration test case 10.3.2 | Qualcomm Incorporated, Orange | agreed |  |  |
| R5-242941 | Applicability updates for EPS P-CSCF restoration test cases | Qualcomm Incorporated, Orange | revised |  | R5-243473 |
| R5-242942 | Discussion Paper for Rel-17 eNS_Ph2 WI NSAC Session management tests case applicability | CATT | revised |  | R5-243487 |
| R5-242943 | Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC2 | NTT DOCOMO INC. | revised |  | R5-243826 |
| R5-242944 | Update to FR1 CA Configurations for n78A-n79A with 1UL | NTT DOCOMO INC. | agreed |  |  |
| R5-242945 | Core spec alignment Missing DMRS configuration restriction for intra ULCA | Qualcomm Tech. Netherlands B.V | agreed |  |  |
| R5-242946 | Updates to additional Spurious emissions TC 6.5.3.3 | Qualcomm Tech. Netherlands B.V | withdrawn |  |  |
| R5-242947 | Updates to additional spurious emissions TC 6.5.3.3 | Qualcomm Tech. Netherlands B.V | agreed |  |  |
| R5-242948 | Addition of REFSENS for PC3 CA n78A-n79A with 1UL | NTT DOCOMO INC. | revised |  | R5-243824 |
| R5-242949 | Proposal for new test case related to ACB per PLMN | NTTDOCOMO, INC. | revised |  | R5-243467 |
| R5-242950 | Correction to LPP positioning test case 9.3.4.2 | NTT DOCOMO Inc., MCC TF160 | revised |  | R5-243569 |
| R5-242951 | Adding new NTN test case 6.3.4 Power control | Qualcomm Tech. Netherlands B.V | revised |  | R5-243679 |
| R5-242952 | Addition of REFSENS for PC2 CA n78A-n79A with 1UL | NTT DOCOMO INC. | agreed |  |  |
| R5-242953 | WP Rel-15 NR TX and RX Test Cases – Part 3: Range 1 and Range 2 Interworking operation with other radios (TS 38.521-3) | Qualcomm Tech. Netherlands B.V | noted |  |  |
| R5-242954 | Addition of new test case related to SSAC per PLMN 13.5.1.c | NTTDOCOMO, INC. | revised |  | R5-243476 |
| R5-242955 | Addition of applicability for new test case related to SSAC per PLMN 13.5.1.c | NTTDOCOMO, INC. | revised |  | R5-243577 |
| R5-242956 | Update ATSSS test case 11.9.3 | ZTE | agreed |  |  |
| R5-242957 | Update ATSSS test case 11.9.4 | ZTE | agreed |  |  |
| R5-242958 | Correction to eNS test case 10.1.8.2 | Anritsu EMEA Ltd, MediaTek | agreed |  |  |
| R5-242959 | Addition of Test Case 14.2.5 | Rohde & Schwarz | revised |  | R5-243857 |
| R5-242960 | Addition of Test Case 14.2.7 | Rohde & Schwarz | revised |  | R5-243858 |
| R5-242961 | Addition of Test Case 14.3.5 | Rohde & Schwarz | revised |  | R5-243859 |
| R5-242962 | Addition of Test Case 14.3.6 | Rohde & Schwarz | revised |  | R5-243860 |
| R5-242963 | Addition of Test Case 14.4.1 | Rohde & Schwarz | revised |  | R5-243861 |
| R5-242964 | Addition of Test Case 14.4.2 | Rohde & Schwarz | revised |  | R5-243862 |
| R5-242965 | Addition of Test Case 14.5.1 | Rohde & Schwarz | revised |  | R5-243863 |
| R5-242966 | Addition of Test Case 14.5.2 | Rohde & Schwarz | revised |  | R5-243864 |
| R5-242967 | Addition of Test Case 15.2.5 | Rohde & Schwarz | revised |  | R5-243865 |
| R5-242968 | Addition of Test Case 15.2.6 | Rohde & Schwarz | revised |  | R5-243866 |
| R5-242969 | Addition of Test Case 15.2.7 | Rohde & Schwarz | revised |  | R5-243867 |
| R5-242970 | Addition of Test Case 15.3.3 | Rohde & Schwarz | revised |  | R5-243868 |
| R5-242971 | Addition of Test Case 15.3.4 | Rohde & Schwarz | revised |  | R5-243869 |
| R5-242972 | Addition of Test Case 16.2.5 | Rohde & Schwarz | revised |  | R5-243870 |
| R5-242973 | Addition of Test Case 16.2.6 | Rohde & Schwarz | revised |  | R5-243871 |
| R5-242974 | Addition of Test Case 16.3.3 | Rohde & Schwarz | revised |  | R5-243872 |
| R5-242975 | Addition of Test Case 16.4.1 | Rohde & Schwarz | revised |  | R5-243873 |
| R5-242976 | Addition of Test Case 16.4.2 | Rohde & Schwarz | revised |  | R5-243874 |
| R5-242977 | Addition of Test Case 16.5.1 | Rohde & Schwarz | revised |  | R5-243875 |
| R5-242978 | Addition of Test Case 16.5.2 | Rohde & Schwarz | revised |  | R5-243876 |
| R5-242979 | Editorial correction of references to TS 38.101-1 in section 7 | Rohde & Schwarz | agreed |  |  |
| R5-242980 | Editorial correction of references to TS 38.101-1 in section 6.7 | Rohde & Schwarz | agreed |  |  |
| R5-242981 | Editorial correction of references to TS 38.101-1 in section 6.6 | Rohde & Schwarz | agreed |  |  |
| R5-242982 | Editorial correction of references to TS 38.101-1 in section 5 | Rohde & Schwarz | agreed |  |  |
| R5-242983 | Editorial correction of references to TS 38.101-1 in section 4 | Rohde & Schwarz | agreed |  |  |
| R5-242984 | Addition of missing applicability to RedCap SS-RSRP test cases | Rohde & Schwarz | agreed |  |  |
| R5-242985 | Corrections to R17 RedCap 18.3.1.x test cases | Rohde & Schwarz | revised |  | R5-243713 |
| R5-242986 | Editorial correction to title of R17 RedCap test case 16.6.4.7 | Rohde & Schwarz | agreed |  |  |
| R5-242987 | Test procedure correction for R17 RedCap NCD-SSB test cases | Rohde & Schwarz | agreed |  |  |
| R5-242988 | Addition of SA FR1 NR-U RSSI minimum conformance requirements | Rohde & Schwarz | agreed |  |  |
| R5-242989 | Addition of SA FR1 NR-U RSSI measurement accuracy test case 11.6.5.3 | Rohde & Schwarz | agreed |  |  |
| R5-242990 | Addition of SA FR1 NR-U SSB based L1-RSRP measurement test case 11.6.4.1 | Rohde & Schwarz | agreed |  |  |
| R5-242991 | Addition of SA FR1 NR-U channel occupancy inter-frequency test case 11.6.6.3 | Rohde & Schwarz | agreed |  |  |
| R5-242992 | Correction to frequencies table for RRM FR2 SA test cases | Rohde & Schwarz | agreed |  |  |
| R5-242993 | Corrections to message contents for SA FR2 event triggered intra-frequency test cases | Rohde & Schwarz | agreed |  |  |
| R5-242994 | Corrections to message contents for SA FR2 timing test cases | Rohde & Schwarz | agreed |  |  |
| R5-242995 | Addition of Test Applicability UE Rx-Tx time difference measurement accuracy in RRC_INACTIVE state in FR2 SA | Nokia | revised |  | R5-243806 |
| R5-242996 | PRD21 CDS: PC3 NR CA_n78A-n79A with 1UL | NTT DOCOMO INC. | revised |  | R5-243990 |
| R5-242997 | MU discussion on FR2c | Anritsu | revised |  | R5-243725 |
| R5-242998 | On testability issue of FR2c OBW | Anritsu | noted |  |  |
| R5-242999 | Update for FR2c MU | Anritsu | revised |  | R5-243831 |
| R5-243000 | Update for FR2c MU | Anritsu | withdrawn |  |  |
| R5-243001 | Update for FR2c MU | Anritsu | revised |  | R5-243832 |
| R5-243002 | QoQZ for QZ size 40 cm | Anritsu | noted |  |  |
| R5-243003 | QoQZ for QZ size 40 cm | Anritsu | withdrawn |  |  |
| R5-243004 | Correction to PCC DL allocation of CA_XA-YA-ZA in 7.6.3A.5 | Anritsu | withdrawn |  |  |
| R5-243005 | Correction to test requirement of CA_2A-2A-66A-66A in 7.3A.9 | Anritsu | agreed |  |  |
| R5-243006 | Correction to test requirement of CA_2A-2A-12A-66A-66A in 7.3A.10 | Anritsu | agreed |  |  |
| R5-243007 | Correction to K1 for RRM TDD SCS 15 kHz | Anritsu | revised |  | R5-243833 |
| R5-243008 | Clarification of test frequency for n5 CBW 25 MHz | Anritsu | agreed |  |  |
| R5-243009 | Correction to note for inter-band CA including n48 | Anritsu | agreed |  |  |
| R5-243010 | Correction to test configuration and test requirements for NS_49 A-MPR | Anritsu, China Unicom | agreed |  |  |
| R5-243011 | Correction to test configuration and test requirements for NS_21 A-MPR | Anritsu | agreed |  |  |
| R5-243012 | Correction to notes for NS_21 A-SEM | Anritsu | agreed |  |  |
| R5-243013 | Update for EVM including symbols with transient period | Anritsu, Keysight | revised |  | R5-243839 |
| R5-243014 | Clarification of antenna array assumptions for in-band measurement | Anritsu | revised |  | R5-243663 |
| R5-243015 | Correction to notes for FR1 EN-DC Spurious emission band UE co-existence | Anritsu | agreed |  |  |
| R5-243016 | Correction to PDSCH Reference Channel for FDD HST | Anritsu | revised |  | R5-243697 |
| R5-243017 | Correction to PUCCH Resource ID in 2Rx FR1 RI reporting | Anritsu | revised |  | R5-243695 |
| R5-243018 | Correction to message exception in non-contention based random access | Anritsu | withdrawn |  |  |
| R5-243019 | Correction to K1 in 4.6.4.3 and 6.6.4.3 for FDD and TDD SCS 30 kHz | Anritsu | agreed |  |  |
| R5-243020 | Correction to DCI trigger in 6.6.4.3 for FDD and TDD SCS 15 kHz | Anritsu | agreed |  |  |
| R5-243021 | Correction to test parameters in 16.5.1.7 | Anritsu | withdrawn |  |  |
| R5-243022 | Correction to LCRB of CA_n5-n48-n66 in 7.3A.0.5 | Anritsu | revised |  | R5-243834 |
| R5-243023 | Correction to notes for support of SCS and CBW in CA configurations | Anritsu | revised |  | R5-243912 |
| R5-243024 | Correction to test parameters in FR1 HST DPS | Anritsu | revised |  | R5-243913 |
| R5-243025 | Correction to SMTC settings in 16.6.2.9 and 16.6.2.10 | Anritsu | agreed |  |  |
| R5-243026 | PRD21 CDS: PC2 NR CA_n78A-n79A with 1UL | NTT DOCOMO INC. | revised |  | R5-243991 |
| R5-243027 | Addition of delta TIB into 6.2A.4.0.2.3 for CA_n3A-n8A-n78A | China Unicom | agreed |  |  |
| R5-243028 | Addition of delta RIB into 7.3A.0.3.2.3 for CA_n3A-n8A-n78A | China Unicom | revised |  | R5-243632 |
| R5-243029 | Addition of 3DL CA minimun requirements of reference sensitivity exceptions into 7.3A.0.5 for CA_n3A-n8A-n78A | China Unicom | agreed |  |  |
| R5-243030 | Addition of 3DL CA reference sensitivity TC into 7.3A.2 for CA_n3A-n8A-n78A | China Unicom | revised |  | R5-243617 |
| R5-243031 | Update inter-band NR CA configuration for CA_n3A-n8A-n78A | China Unicom | revised |  | R5-243600 |
| R5-243032 | Update reference sensitivity TC for CA_n3A-n8A-n78A | China Unicom | revised |  | R5-243616 |
| R5-243033 | PRD21 CDS PC3 for 3 bands CA_n1A-n8A-n78A and CA_n3A-n8A-n78A | China Unicom | available |  |  |
| R5-243034 | SR Rel-18 ATG for NR after RAN#103 | CMCC | noted |  |  |
| R5-243035 | WP Rel-18 ATG for NR after RAN#103 | CMCC | noted |  |  |
| R5-243036 | Update to ATG Definitions and Symbols | CMCC | revised |  | R5-243906 |
| R5-243037 | Update to ATG RF Tx TCs | CMCC | agreed |  |  |
| R5-243038 | Update to ATG RF Rx TCs | CMCC, CAICT | agreed |  |  |
| R5-243039 | SR NR_Rel-16_CA_DC after RAN#103 | CMCC | noted |  |  |
| R5-243040 | WP NR_Rel-16_CA_DC after RAN#103 | CMCC | noted |  |  |
| R5-243041 | Update to R16 NR CADC configuration test cases applicability | CMCC | revised |  | R5-243714 |
| R5-243042 | Update to R17 NR CADC configuration test cases applicability | CMCC | withdrawn |  |  |
| R5-243043 | Update to R18 NR CADC configuration test cases applicability | CMCC | withdrawn |  |  |
| R5-243044 | Update REFSENSE TP analysis for CA_n79C | CMCC | agreed |  |  |
| R5-243045 | Update to REFSENSE TC for CA_n79C | CMCC | agreed |  |  |
| R5-243046 | Update delta TIBc of CA_n39A-n41A for simultaneous RxTx capability | CMCC | agreed |  |  |
| R5-243047 | Update delta RIBc of CA_n39A-n41A for simultaneous RxTx capability | CMCC | agreed |  |  |
| R5-243048 | Update REFSENSE exceptions due to UL harmonic interference for CA_n39A-n41A | CMCC | revised |  | R5-243837 |
| R5-243049 | Update REFSENSE exceptions due to CBI for CA_n39A-n41A | CMCC | agreed |  |  |
| R5-243050 | Update REFSENSE TC 7.3A.1_1 for CA_n39A-n41A | CMCC | revised |  | R5-243838 |
| R5-243051 | Update REFSENSE TP analysis for CA_n39A-n41A | CMCC | revised |  | R5-243836 |
| R5-243052 | Rel-15 5GS WP SIG NE-DC after RAN5#103 | CMCC | noted |  |  |
| R5-243053 | Editorial corrections for IoT NTN RRM TCs | CMCC, MediaTek | agreed |  |  |
| R5-243054 | SR Rel-18 IoT NTN enh after RAN#103 | CMCC | noted |  |  |
| R5-243055 | WP Rel-18 IoT NTN enh after RAN#103 | CMCC | noted |  |  |
| R5-243056 | Revised WID on UE Conformance - IoT NTN enh plus CT1 aspects | CMCC | agreed |  |  |
| R5-243057 | Addition of new RRM TC 13.5.1 for IoT NTN enh | CMCC | revised |  | R5-243688 |
| R5-243058 | Addition of new RRM TC 13.5.2 for IoT NTN enh | CMCC | revised |  | R5-243689 |
| R5-243059 | Addition of new RRM TC 13.5.3 for IoT NTN enh | CMCC | revised |  | R5-243690 |
| R5-243060 | Addition of cell configuration mapping for IoT NTN RRM TCs | CMCC | agreed |  |  |
| R5-243061 | Update to R18 IoT NTN enh RRM TCs applicability | CMCC | withdrawn |  |  |
| R5-243062 | Update to R18 IoT NTN enh SIG TCs applicability | CMCC | withdrawn |  |  |
| R5-243063 | SR Rel-18 PC1.5 n34 n39 n40 after RAN5#103 | CMCC | noted |  | - |
| R5-243064 | WP Rel-18 PC1.5 n34 n39 n40 after RAN5#103 | CMCC | noted |  |  |
| R5-243065 | Addition of PC1.5 n39 A-MPR | CMCC | agreed |  |  |
| R5-243066 | TP analysis for PC1.5 n39 A-MPR and A-SE | CMCC | revised |  | R5-243614 |
| R5-243067 | PRD21 on NR bands and 5G NR CADC config handling v1.9.0 | CMCC | email approved |  |  |
| R5-243068 | Update NR band and CADC configs status in ICS Annex B | CMCC | agreed |  |  |
| R5-243069 | Disc on handling of PRD21 and PRD20 | CMCC, ZTE, Bureau Veritas, Huawei, Hisilicon | noted |  |  |
| R5-243070 | PRD21 CDS: NR band and CBW FR1 n34 PC1.5 | CMCC | approved |  |  |
| R5-243071 | PRD21 CDS: NR band and CBW FR1 n39 PC1.5 | CMCC | approved |  |  |
| R5-243072 | PRD21 CDS: NR band and CBW FR1 n40 PC1.5 | CMCC | approved |  |  |
| R5-243073 | PRD21 CDS: NR band and CBW FR1 n41 PC1 | CMCC, Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R5-243074 | Disc on handling of MDT IDC problem | CMCC, CATT, Qualcomm | revised |  | R5-243486 |
| R5-243075 | Addition of a new section for IDC Configuration | CMCC, CATT, Qualcomm | revised |  | R5-243581 |
| R5-243076 | Addition of new PICS for IDC mechanism | CMCC, CATT, Qualcomm | revised |  | R5-243582 |
| R5-243077 | Update of test case 8.1.6.1.2.14 for SON_MDT | CMCC, CATT, Qualcomm | revised |  | R5-243583 |
| R5-243078 | Update of applicability for test cases 8.1.6.1.2.14 | CMCC, CATT, Qualcomm | revised |  | R5-243584 |
| R5-243079 | New WID on UE Conformance - In-Device Co-existence (IDC) enhancements for NR and MR-DC | CMCC, Xiaomi | revised |  | R5-243461 |
| R5-243080 | WP - UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2 | Samsung | noted |  |  |
| R5-243081 | WP - UE Conformance - NR MIMO Evolution for Downlink and Uplink | Samsung | noted |  |  |
| R5-243082 | SR - UE Conformance - Enhanced NR support for high speed train scenario in frequency range 2 | Samsung | noted |  |  |
| R5-243083 | SR - UE Conformance - NR MIMO Evolution for Downlink and Uplink | Samsung | noted |  |  |
| R5-243084 | Correction of SDT TC 7.1.1.13.5-cg-SDT-TATimer | Huawei, HiSilicon, MCC TF160 | revised |  | R5-243537 |
| R5-243085 | Correction of SDT TC 8.1.5.13.1-cg-SDT | Huawei, HiSilicon, MCC TF160 | revised |  | R5-243538 |
| R5-243086 | Add new PICS for MUSIM gap | Huawei, HiSilicon | revised |  | R5-243957 |
| R5-243087 | Correction to IE PDCCH-ConfigCommon | MediaTek Inc. | revised |  | R5-243493 |
| R5-243088 | Correction of MAC TC 7.1.1.3.2b-ULCGTYPE1 | Huawei, HiSilicon | agreed |  |  |
| R5-243089 | Correction to PDSCH TDD Test Cases | Rohde & Schwarz | agreed |  |  |
| R5-243090 | Addition of missing TRS configurations | Rohde & Schwarz | revised |  | R5-243604 |
| R5-243091 | Correction to Rx fast spherical coverage method | Rohde & Schwarz | revised |  | R5-243664 |
| R5-243092 | Correction to Inter-band EN-DC spurious emission requirements | Rohde & Schwarz, Oppo | revised |  | R5-243840 |
| R5-243093 | Correction to URLLC message exceptions | Rohde & Schwarz | agreed |  |  |
| R5-243094 | Correction to NR IMS testcases 7.4a and 7.6a | ROHDE & SCHWARZ | withdrawn |  |  |
| R5-243095 | Removal of Editors note for applicability of RedCap SS-RSRP test cases | Rohde & Schwarz | agreed |  |  |
| R5-243096 | Core spec alignment for R17 RedCap NCD-SSB test cases | Rohde & Schwarz | withdrawn |  |  |
| R5-243097 | Addition of SA FR1 NR-U channel occupancy intra-frequency test cases | Rohde & Schwarz | agreed |  |  |
| R5-243098 | SR of Rel-16 V2X WI after RAN5 103 | Huawei, HiSilicon | noted |  |  |
| R5-243099 | WP of Rel-16 V2X WI after RAN5 103 | Huawei, HiSilicon | noted |  |  |
| R5-243100 | On the MU for n259 | ROHDE & SCHWARZ | noted |  |  |
| R5-243101 | Documentation of MU for n259 | ROHDE & SCHWARZ | revised |  | R5-243829 |
| R5-243102 | Update of MU for n259 | ROHDE & SCHWARZ | revised |  | R5-243727 |
| R5-243103 | On the MU and TT for CA and MIMO test cases | ROHDE & SCHWARZ | noted |  |  |
| R5-243104 | Documentation of MU for UL MIMO | ROHDE & SCHWARZ | agreed |  |  |
| R5-243105 | Update of MU and TT for CA and UL MIMO | ROHDE & SCHWARZ | revised |  | R5-243830 |
| R5-243106 | Update of status of FR2 CA and UL MIMO test cases | ROHDE & SCHWARZ | agreed |  |  |
| R5-243107 | Update of applicability of FR2 CA and MIMO test cases | ROHDE & SCHWARZ | revised |  | R5-243729 |
| R5-243108 | Addition of power tolerance CA test cases for FR2 | ROHDE & SCHWARZ | agreed |  |  |
| R5-243109 | Applicability for FR2 NSA power tolerance CA test cases | ROHDE & SCHWARZ | agreed |  |  |
| R5-243110 | Update of MU and TT for FR1 CSI reporting | ROHDE & SCHWARZ | agreed |  |  |
| R5-243111 | Editorial correction of UE capability in PDSCH repetition test cases | ROHDE & SCHWARZ | agreed |  |  |
| R5-243112 | Correction of narrow band blocking for CA | ROHDE & SCHWARZ, Anritsu | revised |  | R5-243808 |
| R5-243113 | Correction of sr-PUCCH-ResourceIndex | ROHDE & SCHWARZ | withdrawn |  |  |
| R5-243114 | Correction of MOP for NB1 and NB2 test case | ROHDE & SCHWARZ | agreed |  |  |
| R5-243115 | Update of frequency error test cases | ROHDE & SCHWARZ | agreed |  |  |
| R5-243116 | Update of test case 8.3.1.1.1 | ROHDE & SCHWARZ | agreed |  |  |
| R5-243117 | 5GS EPS fallback test coverage | Ericsson | revised |  | R5-243485 |
| R5-243118 | Correction to NR CA shared spectrum test case 7.1.1.10.3 | Qualcomm Incorporated | agreed |  |  |
| R5-243119 | Correction to NR shared spectrum test case 8.1.5.6.6.1 | Qualcomm Incorporated | agreed |  |  |
| R5-243120 | Correction to NR shared spectrum test case 8.2.5.7.1 | Qualcomm Incorporated | agreed |  |  |
| R5-243121 | Corrrection to NR5GC test case 8.1.4.4.1 | Keysight Technologies UK | agreed |  |  |
| R5-243122 | Correction to NR shared spectrum test case 8.2.5.7.2 | Qualcomm Incorporated | agreed |  |  |
| R5-243123 | Addition of PC2 for n8 UL MIMO into 6.2D.1 MOP TC | China Unicom | agreed |  |  |
| R5-243124 | Updates to UAS TC 9.1.5.2.11 | Ericsson | revised |  | R5-243575 |
| R5-243125 | Updates to UAS TC 10.1.4.2 | Ericsson | revised |  | R5-243576 |
| R5-243126 | Adding clarification of trace mode in emission test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243127 | Correction to general RB allocation in 6.1A | Huawei, HiSilicon | agreed |  |  |
| R5-243128 | Cleaning up test applicability for ULFPTx in test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243129 | Cleaning up test applicability for ULFPTx | Huawei, HiSilicon | agreed |  |  |
| R5-243130 | Editorial correction of Annex F for R17 FR1 test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243131 | Adding test point information of 6.5H.1.1 | Huawei, HiSilicon | agreed |  |  |
| R5-243132 | Addition of PICS for TxD for NR FR1 bands | Huawei, HiSilicon, Bureau Veritas | agreed |  |  |
| R5-243133 | Updating applicability for Rx test cases | Huawei, HiSilicon | withdrawn |  |  |
| R5-243134 | Addition of NR V2X MOP for inter-band concurrent operation | Huawei, HiSilicon | agreed |  |  |
| R5-243135 | Addition of TP analysis for NR V2X MOP | Huawei, HiSilicon | agreed |  |  |
| R5-243136 | Update of NR V2X MPR test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243137 | Update of NR V2X AMPR test cases | Huawei, HiSilicon | revised |  | R5-243629 |
| R5-243138 | Addition of NR V2X NR configured transmit power test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243139 | Addition of TP analysis for NR V2X Configured transmit power | Huawei, HiSilicon | agreed |  |  |
| R5-243140 | Update to NR V2X minimum output power test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243141 | Update to NR V2X transmit off power test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243142 | Addition of NR V2X ON/OFF time mask test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243143 | Addition of TP analysis for NR V2X ON OFF time mask | Huawei, HiSilicon | agreed |  |  |
| R5-243144 | Update to test applicability of V2X test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243145 | Addition of NR V2X MOP minimum requirement for sidelink enhancement | Huawei, HiSilicon | agreed |  |  |
| R5-243146 | Update to NR V2X MPR test cases to add PC2 and intra-band concurrent operation | Huawei, HiSilicon | agreed |  |  |
| R5-243147 | Addition of TP analysis for NR V2X MPR for PC2 and intra-band concurrent operation | Huawei, HiSilicon | revised |  | R5-243621 |
| R5-243148 | Update to NR V2X AMPR test cases to add PC2 and intra-band concurrent operation | Huawei, HiSilicon | agreed |  |  |
| R5-243149 | Addition of TP analysis for NR V2X AMPR for PC2 and intra-band concurrent operation | Huawei, HiSilicon | revised |  | R5-243622 |
| R5-243150 | Update to HST scheme A and B test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243151 | Adding RMC for feMIMO test cases | Huawei, HiSilicon, Samsung | agreed |  |  |
| R5-243152 | Update to applicability of FeMIMO test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243153 | Update to Unified TCI state test cases | Huawei, HiSilicon, Samsung | revised |  | R5-243703 |
| R5-243154 | Correcting propagation AWGN in test cases without Noc | Huawei, HiSilicon | revised |  | R5-243704 |
| R5-243155 | Adding 7.1.1.3.17.1 multi-TRP PUSCH repetition type A | Huawei, HiSilicon | agreed |  |  |
| R5-243156 | Adding 7.1.1.3.17.2 multi-TRP PUSCH repetition type B | Huawei, HiSilicon | agreed |  |  |
| R5-243157 | Addition of PICS for FeMIMO | Huawei, HiSilicon | revised |  | R5-243543 |
| R5-243158 | Adding test applicability for multi-TRP PUSCH repetition test cases | Huawei, HiSilicon | revised |  | R5-243544 |
| R5-243159 | Discussion on beam correspondence for initial access | Huawei, HiSilicon | revised |  | R5-243823 |
| R5-243160 | Draft CR for introducing beamlock function in IDLE mode | Huawei, HiSilicon | withdrawn |  |  |
| R5-243161 | WP of Rel-18 NR CA and DC; and NR and LTE DC Configurations | Huawei, HiSilicon | noted |  |  |
| R5-243162 | SR of Rel-18 NR CA and DC; and NR and LTE DC Configurations | Huawei, HiSilicon | noted |  |  |
| R5-243163 | Rel-15 5GS maintenance WP for NR RF TX and RX Test Cases (38.521-1) | Huawei, HiSilicon | noted |  |  |
| R5-243164 | Rel-15 5GS maintenance WP for NR RF TX and RX Test Cases (38.521-2) | Huawei, HiSilicon | noted |  |  |
| R5-243165 | New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource | Nokia | revised |  | R5-243968 |
| R5-243166 | New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session | Nokia | revised |  | R5-243969 |
| R5-243167 | Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2 | Nokia | agreed |  |  |
| R5-243168 | Editorial correction of Table 4.5.4.2-4: WLAN IPsec_SA_Established | Nokia | agreed |  |  |
| R5-243169 | Editorial correction of testcase 9.1.6.2.3 | Nokia | agreed |  |  |
| R5-243170 | Modification of testcase 8.1.5.13.3 for SDT-SRB2-Indication | Nokia | revised |  | R5-243539 |
| R5-243171 | Modification of testcase 8.1.5.13.3 applicability clauses | Nokia | agreed |  |  |
| R5-243172 | Addition of new test case 8.2.3.18.6 inter-SN CPC for EN-DC | Nokia | revised |  | R5-243954 |
| R5-243173 | Common configuration for SN initiated inter-SN CPC EN-DC for 36.508 | Nokia | withdrawn |  |  |
| R5-243174 | Common configuration for SN initiated inter-SN CPC EN-DC for 38.508-1 | Nokia | revised |  | R5-243952 |
| R5-243175 | Addition of new test case 8.2.3.18.7 inter-SN CPC for NR-DC | Nokia | revised |  | R5-243955 |
| R5-243176 | Addition of applicability clauses for inter-SN CPC testcases for EN-DC and NR-DC | Nokia | revised |  | R5-243956 |
| R5-243177 | Addition of capability clauses for inter-SN CPC testcases for EN-DC and NR-DC | Nokia | revised |  | R5-243953 |
| R5-243178 | Revised WID on UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects | Nokia | noted |  | R5-243975 |
| R5-243179 | WP UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects | Nokia | noted |  |  |
| R5-243180 | SR UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects | Nokia | noted |  |  |
| R5-243181 | Corrections to eMIMO TC 7.1.1.3.10 | Lenovo, MCC TF160 | agreed |  |  |
| R5-243182 | Update of applicability for test case 6.1.2.9 | Ericsson | agreed |  |  |
| R5-243183 | Corrections to CovEnh TC 7.1.1.1.18 | Lenovo, MCC TF160 | agreed |  |  |
| R5-243184 | Update of message contents for CQI reporting in CA | ROHDE & SCHWARZ | withdrawn |  |  |
| R5-243185 | Applicability updates to NR shared spectrum test cases | Qualcomm Incorporated | agreed |  |  |
| R5-243186 | Correction to MAC TC 7.1.1.6.1 | MediaTek Inc., Keysight | withdrawn |  |  |
| R5-243187 | Add IE SL-CBR-CommonTxDedicated-SL-PRS-RP-List | Ericsson | agreed |  |  |
| R5-243188 | Add IE SL-ConfiguredGrantConfigDedicated-SL-PRS-RP | Ericsson | agreed |  |  |
| R5-243189 | Addition of CA_n1A-n8A-n78A,CA_n3A-n8A-n78A into NR inter-band CA within FR1 three bands | China Unicom | revised |  | R5-243607 |
| R5-243190 | Revised WID on UE Conformance - Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258 | Huawei, Hisilicon | agreed |  |  |
| R5-243191 | WP of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258 | Huawei, Hisilicon | noted |  |  |
| R5-243192 | SR of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258 | Huawei, Hisilicon | revised |  | R5-243979 |
| R5-243193 | WP of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL | Huawei, Hisilicon | noted |  |  |
| R5-243194 | SR of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL | Huawei, Hisilicon | revised |  | R5-243981 |
| R5-243195 | WP of Rel-17 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | noted |  |  |
| R5-243196 | SR of Rel-17 NR CA and DC; and NR and LTE DC Configurations | Huawei, Hisilicon | noted |  |  |
| R5-243197 | WP of New Rel-18 NR licensed bands and extension of existing NR bands | Huawei, Hisilicon | noted |  |  |
| R5-243198 | SR of New Rel-18 NR licensed bands and extension of existing NR bands | Huawei, Hisilicon | noted |  |  |
| R5-243199 | WP of Further RF requirements enhancement for NR and EN-DC in frequency range 1 | Huawei, Hisilicon | noted |  |  |
| R5-243200 | SR of Further RF requirements enhancement for NR and EN-DC in frequency range 1 | Huawei, Hisilicon | noted |  |  |
| R5-243201 | Updating AMPR requirements for PC3 UE supporting power boosting | Huawei, HiSilicon | agreed |  |  |
| R5-243202 | Updating AMPR testing for CA_n41A-n79A | Huawei, HiSilicon | agreed |  |  |
| R5-243203 | Updating the TP analysis for AMPR testing for CA_n41A-n79A | Huawei, HiSilicon | revised |  | R5-243624 |
| R5-243204 | Updating description in test configuration table of test case 7.3B.2.3_1.1 | Huawei, HiSilicon | agreed |  |  |
| R5-243205 | Updating AMPR testing for NS_43 for band n8 | Huawei, HiSilicon | agreed |  |  |
| R5-243206 | Updating TP analysis for AMPR testing for NS_43 | Huawei, HiSilicon | revised |  | R5-243613 |
| R5-243207 | Updating PC2 AMPR testing for NS_100 for band n8 | Huawei, HiSilicon | revised |  | R5-243631 |
| R5-243208 | Updating general clause for Uplink switching across up to four bands | Huawei, HiSilicon | agreed |  |  |
| R5-243209 | Adding new test case Time mask for switching across three uplink bands | Huawei, HiSilicon | revised |  | R5-243658 |
| R5-243210 | Adding new test case Time mask for switching across four uplink bands | Huawei, HiSilicon | revised |  | R5-243659 |
| R5-243211 | Adding new SUL test case Time mask for switching across three uplink bands | Huawei, HiSilicon | revised |  | R5-243620 |
| R5-243212 | Adding new SUL test case Time mask for switching across four uplink bands | Huawei, HiSilicon | revised |  | R5-243644 |
| R5-243213 | Updating MU and TT for Time mask for switching across up to four bands testing | Huawei, HiSilicon | revised |  | R5-243645 |
| R5-243214 | Adding TP analysis for new FR1 test case 6.3A.3.6 and 6.3A.3.7 | Huawei, Hisilicon | revised |  | R5-243618 |
| R5-243215 | Adding TP analysis for new SUL test case 6.3C.3.6 | Huawei, Hisilicon | revised |  | R5-243619 |
| R5-243216 | Add IEs SL-DRX-Config, SL-DRX-ConfigGC-BC, SL-DRX-ConfigUC and SL-DRX-ConfigUC-SemiStatic | Ericsson | revised |  | R5-243492 |
| R5-243217 | Add IEs SL-FreqSelectionConfig and SL-IndirectPathAddChange | Ericsson | agreed |  |  |
| R5-243218 | Applicability for new test cases 12.23a and 12.24a | Ericsson | revised |  | R5-243961 |
| R5-243219 | Add new NR IMS data channel testcase - Bootstrap Data Channel | Huawei, Hisilicon | withdrawn |  |  |
| R5-243220 | Addition of MU and TT analysis for 3Tx EN-DC test cases | Huawei, HiSilicon | agreed |  |  |
| R5-243221 | Addition of general information for 3Tx EN-DC | Huawei, HiSilicon | agreed |  |  |
| R5-243222 | UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | Apple, CAICT, Keysight Technologies | revised |  | R5-243462 |
| R5-243223 | Addition of delta TIBc for R16 inter-band EN-DC within FR1 | KT Corp. | revised |  | R5-243666 |
| R5-243224 | CR to TS 38.551 on Annex A.2.3 | Apple (UK) Limited | revised |  | R5-243800 |
| R5-243225 | Addition of delta RIBc for R16 inter-band EN-DC within FR1 | KT Corp. | revised |  | R5-243667 |
| R5-243226 | Update to applicability of V2X test cases | Huawei, HiSilicon | withdrawn |  |  |
| R5-243227 | CR to TS 38.551 on Annex C, editorial | Apple (UK) Limited | agreed |  |  |
| R5-243228 | Update to Annex F for V2X test cases | Huawei, HiSilicon | revised |  | R5-243630 |
| R5-243229 | Addition of minimum requirements of reference sensitivity exceptions for R16 inter-band EN-DC within FR1 | KT Corp. | revised |  | R5-243668 |
| R5-243230 | CR to TS 38.551 on Annex G, editorial | Apple (UK) Limited | agreed |  |  |
| R5-243231 | Update of R16 Configuration for inter-band EN-DC | KT Corp. | revised |  | R5-243669 |
| R5-243232 | CR to TS 38.551 on EUT positioning | Apple (UK) Limited | agreed |  |  |
| R5-243233 | CR to TS 38.551 on section 6.1.2 | Apple (UK) Limited | agreed |  |  |
| R5-243234 | Proposal for new test cases related to P-CSCF Restoration procedures | Orange, Qualcomm | noted |  |  |
| R5-243235 | Addition of Clause 4.5.6.4.0 minimum conformance requirements for simultaneous RRC-based active BWP switch on multiple CCs | Apple | agreed |  |  |
| R5-243236 | Addition of Clause 4.6.8.0 minimum conformance requirements for CSI-RS based intra-frequency measurement | Apple | agreed |  |  |
| R5-243237 | Addition of Clause 4.6.9.0 minimum conformance requirements for CSI-RS based inter-frequency measurement | Apple | agreed |  |  |
| R5-243238 | Addition of Clause 5.5.3.0 minimum conformance requirements for SCell activation and deactivation delay | Apple | agreed |  |  |
| R5-243239 | Addition of Clause 5.5.6.5.0 minimum conformance requirements for simultaneous RRC-based active BWP switch on multiple CC | Apple | agreed |  |  |
| R5-243240 | Addition of message contents to test case 4.7.9.1 | Apple, Huawei, Hisilicon | revised |  | R5-243815 |
| R5-243241 | Addition of message contents to test case 4.7.9.2 | Apple, Huawei, Hisilicon | revised |  | R5-243842 |
| R5-243242 | Addition of message contents to test case 4.7.10.1 | Apple, Huawei, Hisilicon | revised |  | R5-243843 |
| R5-243243 | Addition of message contents to test case 4.7.10.2 | Apple, Huawei, Hisilicon | revised |  | R5-243844 |
| R5-243244 | Addition of message contents to test case 5.7.8.1 | Apple, Huawei, Hisilicon | revised |  | R5-243845 |
| R5-243245 | Addition of message contents to test case 5.7.8.2 | Apple, Huawei, Hisilicon | revised |  | R5-243846 |
| R5-243246 | Addition of message contents to test case 5.7.9.1 | Apple, Huawei, Hisilicon | revised |  | R5-243847 |
| R5-243247 | Addition of message contents to test case 5.7.9.2 | Apple, Huawei, Hisilicon | revised |  | R5-243848 |
| R5-243248 | Addition of message contents to test case 6.7.11.1 | Apple, Huawei, Hisilicon | revised |  | R5-243849 |
| R5-243249 | Addition of message contents to test case 6.7.11.2 | Apple, Huawei, Hisilicon | revised |  | R5-243850 |
| R5-243250 | Addition of message contents to test case 6.7.12.1 | Apple, Huawei, Hisilicon | revised |  | R5-243851 |
| R5-243251 | Addition of message contents to test case 6.7.12.2 | Apple, Huawei, Hisilicon | revised |  | R5-243852 |
| R5-243252 | Addition of message contents to test case 7.7.8.1 | Apple, Huawei, Hisilicon | revised |  | R5-243853 |
| R5-243253 | Addition of message contents to test case 7.7.8.2 | Apple, Huawei, Hisilicon | revised |  | R5-243854 |
| R5-243254 | Addition of message contents to test case 7.7.9.1 | Apple, Huawei, Hisilicon | revised |  | R5-243855 |
| R5-243255 | Addition of message contents to test case 7.7.9.2 | Apple, Huawei, Hisilicon | revised |  | R5-243856 |
| R5-243256 | Correction of spurious emission minimum conformance requirements for UE co-ex to align with RAN4 specs | Apple, Keysight | agreed |  |  |
| R5-243257 | Correction of spurious emission minimum conformance requirements for UE co-ex for intra-band contiguous CA | Apple, Keysight | revised |  | R5-243633 |
| R5-243258 | Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL | Apple, Keysight | revised |  | R5-243649 |
| R5-243259 | Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL Rel16 | Apple, Keysight | revised |  | R5-243816 |
| R5-243260 | Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-16 | Apple, Keysight | revised |  | R5-243817 |
| R5-243261 | Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-17 | Apple, Keysight | revised |  | R5-243634 |
| R5-243262 | SR - UE Conformance Aspects - NR RRM enhancements | Apple | noted |  |  |
| R5-243263 | WP - UE Conformance Aspects - NR RRM enhancements | Apple | noted |  |  |
| R5-243264 | Draft CR for introducing beamlock function in IDLE mode | Huawei, HiSilicon | not pursued |  |  |
| R5-243265 | SR Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData) | NIST | noted |  |  |
| R5-243266 | WP Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData) | NIST | noted |  |  |
| R5-243267 | Addition of new TC 6.5.1 Location Information Request | NIST | revised |  | R5-243570 |
| R5-243268 | Addition of new TC 6.5.2 Location Report triggering | NIST | revised |  | R5-243571 |
| R5-243269 | Addition of new TC 5.8 Rules based affiliation status change | NIST | revised |  | R5-243572 |
| R5-243270 | Addition of new TC 5.9 Location based functional alias status change | NIST | revised |  | R5-243573 |
| R5-243271 | Correction of MCVideo Location TCs | NIST | agreed |  |  |
| R5-243272 | Correction of MCData TC 6.4.2 Emergency Alert | NIST | agreed |  |  |
| R5-243273 | Correction to clause 5.5.3.4.2 | NIST | agreed |  |  |
| R5-243274 | Correction to MCPTT TC 5.10 | NIST | agreed |  |  |
| R5-243275 | Revised WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects | China Unicom | revised |  | R5-243976 |
| R5-243276 | WP on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects | China Unicom | noted |  |  |
| R5-243277 | SR on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects | China Unicom | revised |  | R5-243982 |
| R5-243278 | Editorial corrections to GSO frequency error test cases | Keysight Technologies UK Ltd | revised |  | R5-243686 |
| R5-243279 | Updates to Reference sensitivity for SUL | Keysight Technologies UK Ltd | agreed |  |  |
| R5-243280 | Rel-17 RedCap WP update after RAN5#103 | China Unicom | noted |  |  |
| R5-243281 | WP on UE Conformance - Additional NR bands for UL-MIMO in Rel-18 | China Unicom | noted |  |  |
| R5-243282 | PRD20 on E-UTRA CA configuration handling in RAN5 v1.7.0 | ZTE Corporation | approved |  |  |
| R5-243283 | Update Annex B for PRD20 E-UTRA CA list v170 | ZTE Corporation | revised |  | R5-243810 |
| R5-243284 | New WID on UE Conformance 4Rx for NR bands for FWA UEs (up to 2.6GHz) and handheld UEs (1GHz to 2.6GHz) | ZTE, China Telecom | revised |  | R5-243463 |
| R5-243285 | Corrections on 6.2F.3 for UE A-MPR requirements for shared spectrum access | ZTE Corporation | agreed |  |  |
| R5-243286 | Corrections on 4.3.1 for NTN FR1 test channel bandwidth | ZTE Corporation | agreed |  |  |
| R5-243287 | Update on A.4.1 and A.4.3.1 for FR1-NTN implementation types and RF baseline capabilities | ZTE Corporation | agreed |  |  |
| R5-243288 | Corrections on 5.1 for NTN FR1 frequency range designation | ZTE Corporation | agreed |  |  |
| R5-243289 | Corrections on 6.2.3 for NTN UE additional maximum output power reduction | ZTE Corporation | agreed |  |  |
| R5-243290 | Corrections on 6.5.3.3 for NTN UE additional spurious emissions | ZTE Corporation | agreed |  |  |
| R5-243291 | Updates on Annex for satellite access conformance test | ZTE Corporation | revised |  | R5-243680 |
| R5-243292 | Corrections on 4.0 for NTN tested bands selection criteria | ZTE Corporation | agreed |  |  |
| R5-243293 | Addition to 4.3.1 for FR2 Redcap UE test channel bandwidth | ZTE Corporation | revised |  | R5-243750 |
| R5-243294 | Corrections on 4.3.1.4.1.2 for test frequency for DC_38A_n78A | ZTE Corporation | agreed |  |  |
| R5-243295 | Corrections on 6.2 and 6.2A for almost contiguous A-MPR requirements for HPUE | ZTE Corporation | revised |  | R5-243818 |
| R5-243296 | Corrections on 6.2.3 for UE additional maximum output power reduction requirements | ZTE Corporation | agreed |  |  |
| R5-243297 | Corrections on 6.5.2.2 for UE spectrum emission mask | ZTE Corporation | agreed |  |  |
| R5-243298 | Corrections on 6.5.3.3 for UE additional spurious emissions | ZTE Corporation | agreed |  |  |
| R5-243299 | Corrections on clause 6 for removal of power class capable UE denotation | ZTE Corporation | agreed |  |  |
| R5-243300 | Cleanups on 7.3.2 for reference sensitivity power level test | ZTE Corporation | agreed |  |  |
| R5-243301 | Updates on 3.1 and 4.3 for Definitions and Specification suffix information | ZTE Corporation | revised |  | R5-243819 |
| R5-243302 | Corrections on 6.2.2 and 6.2A.2.0.2 for UE MPR requirements | ZTE Corporation | revised |  | R5-243660 |
| R5-243303 | Corrections on 7.1 for general description to receiver characteristics | ZTE Corporation | revised |  | R5-243662 |
| R5-243304 | Corrections on 6.2B.1.3 for UE MOP for configuration DC_38A_n78A | ZTE Corporation | revised |  | R5-243671 |
| R5-243305 | Discussion on spec quality related issues | ZTE Corporation | noted |  |  |
| R5-243306 | SR on UE Conformance - Additional NR bands for UL-MIMO in Rel-18 | China Unicom | revised |  | R5-243983 |
| R5-243307 | WP on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band | China Unicom | noted |  |  |
| R5-243308 | SR on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band | China Unicom | revised |  | R5-243980 |
| R5-243309 | Clarification of QoQZ Validation Procedure | Keysight Technologies UK Ltd, ETS-Lindgren | agreed |  |  |
| R5-243310 | Clarification of Calibration and Ripple Test Procedures | Keysight Technologies UK Ltd, ETS-Lindgren | agreed |  |  |
| R5-243311 | Clarification of Calibration and Ripple Test Procedures | Keysight Technologies UK Ltd, ETS-Lindgren | revised |  | R5-243797 |
| R5-243312 | Clarification of Calibration and Ripple Test Procedures | Keysight Technologies UK Ltd, ETS-Lindgren | agreed |  |  |
| R5-243313 | Clarification of Channel Model Coordinate System and Probe Placement | Keysight Technologies UK Ltd, OPPO | agreed |  |  |
| R5-243314 | Clarification of polynomial interpolation | Keysight Technologies UK Ltd, Rohde & Schwarz | revised |  | R5-243798 |
| R5-243315 | On NTC MTSU and TT for 40cm QZ and FR2d | Keysight Technologies UK Ltd | noted |  |  |
| R5-243316 | On Testing with Battery | Keysight Technologies UK Ltd | revised |  | R5-243801 |
| R5-243317 | On Testing with Battery | Keysight Technologies UK Ltd | withdrawn |  |  |
| R5-243318 | On Testing with Battery | Keysight Technologies UK Ltd | withdrawn |  |  |
| R5-243319 | Update of Refsens TC for RedCap UE | China Unicom | agreed |  |  |
| R5-243320 | Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases 10.4.2.3, 10.4.2.4, 10.4.2.5, 10.4.2.6 | Qualcomm Technologies Int | revised |  | R5-243893 |
| R5-243321 | Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases with DRX 10.4.2.7, 10.4.2.8, 10.4.2.9, 10.4.2.10 | Qualcomm Technologies Int | revised |  | R5-243894 |
| R5-243322 | Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1 | Qualcomm Technologies Int | revised |  | R5-243736 |
| R5-243323 | Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2 | Qualcomm Technologies Int | revised |  | R5-243737 |
| R5-243324 | Test Tolerance analysis for NR-U RSRQ intra-frequency accuracy test cases 10.5.2.1 and 11.6.2.1 | Qualcomm Technologies Int | revised |  | R5-243738 |
| R5-243325 | Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2 | Qualcomm Technologies Int | revised |  | R5-243739 |
| R5-243326 | Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3 | Qualcomm Technologies Int | revised |  | R5-243740 |
| R5-243327 | Test Tolerance analysis for NR-U SINR inter-frequency accuracy test cases 10.5.3.2 and 11.6.3.2 | Qualcomm Technologies Int | revised |  | R5-243741 |
| R5-243328 | Test Tolerance analysis for NR-U L1-RSRP accuracy test case 10.5.4.1 | Qualcomm Technologies Int | revised |  | R5-243742 |
| R5-243329 | Addition of EN-DC FR1 NR-U Random Access and Handover test cases | Qualcomm Technologies Int | agreed |  |  |
| R5-243330 | Update Table A.4.5-1 with additonal NB-NTN capabilities | Qualcomm Technologies Int | withdrawn |  |  |
| R5-243331 | Update applicability for NB-NTN Demod TC 8.3.1.1.1 | Qualcomm Technologies Int | withdrawn |  |  |
| R5-243332 | Update applicability for NB-NTN RRM test cases | Qualcomm Technologies Int | withdrawn |  |  |
| R5-243333 | Update applicability for NB-NTN Demod TC 8.3.1.1.1 | Qualcomm Technologies Int | revised |  | R5-243905 |
| R5-243334 | Update applicability for NB-NTN Channel quality reporting accuracy testcases | Qualcomm Technologies Int | revised |  | R5-243687 |
| R5-243335 | Correction to RRM RedCap TC 16.5.1.7 | Qualcomm Technologies Int, Anritsu | revised |  | R5-243712 |
| R5-243336 | Addition of suffix information for Carrier Aggregation (CA) with UL MIMO | China Unicom | revised |  | R5-243879 |
| R5-243337 | Correction on Rel-17 HST conditions in ServingCellConfigCommon | Keysight Technologies | agreed |  |  |
| R5-243338 | Update for PMI TypeII codebook test cases | Keysight Technologies | agreed |  |  |
| R5-243339 | Update mTT for missing DL RMC | Keysight Technologies | revised |  | R5-243698 |
| R5-243340 | Correction to FR1 NSA Event Trigger TC 4.6.1.8 | Keysight Technologies | revised |  | R5-243909 |
| R5-243341 | Corrections to NR FR1 RedCap E-UTRA cell re-selection TCs 16.1.2.1, 16.1.2.3 | Keysight Technologies | agreed |  |  |
| R5-243342 | Editorial correction in Redcap 16.6.2.1 | Keysight Technologies | agreed |  |  |
| R5-243343 | Correction to FR1 RedCap E-UTRA - NR inter - RAT Handover TC 18.2.1.1 | Keysight Technologies | agreed |  |  |
| R5-243344 | Editorial correction in Annex E for FR1 RedCap TCs 16.6.2.3, 16.6.2.4 | Keysight Technologies | agreed |  |  |
| R5-243345 | Discussion on PRACH FR2 power measurement | Keysight Technologies UK Ltd | noted |  |  |
| R5-243346 | Corrrection to RRC test case 8.2.3.1.2 | Keysight Technologies UK | agreed |  |  |
| R5-243347 | Corrrection to RRC test case 8.2.3.2.2 | Keysight Technologies UK | agreed |  |  |
| R5-243348 | Corrrection to RRC test case 8.2.3.6.2x | Keysight Technologies UK | agreed |  |  |
| R5-243349 | Corrrection to RRC test case 8.2.3.7.2x | Keysight Technologies UK | agreed |  |  |
| R5-243350 | Corrrection to RRC test case 8.2.3.8.2x | Keysight Technologies UK | revised |  | R5-243512 |
| R5-243351 | Corrrection to RRC test case 8.2.3.12.2 | Keysight Technologies UK | revised |  | R5-243513 |
| R5-243352 | Corrrection to RRC test case 8.2.3.13.2 | Keysight Technologies UK | agreed |  |  |
| R5-243353 | Corrrection to RRC test case 8.2.3.14.3 | Keysight Technologies UK | agreed |  | - |
| R5-243354 | Agenda - opening session | WG Chairman | approved | R5-242100 |  |
| R5-243355 | New WID on UE Conformance - NR NTN (Non-Terrestrial Networks) enhancements | Qualcomm Inc, China Unicom, Mediatek | revised |  | R5-243464 |
| R5-243356 | New WID on UE Conformance - Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception | Qualcomm Inc | revised |  | R5-243465 |
| R5-243357 | New WID on UE Conformance - New bands and bandwidth allocation for LTE based 5G terrestrial broadcast | Qualcomm Inc | revised |  | R5-243568 |
| R5-243358 | Discussion on introducing the WID for LTE based 5G terrestrial broadcast | Qualcomm Inc | noted |  |  |
| R5-243359 | Addition of NTN Timing Advance test 14.3.2.1 | Qualcomm France | agreed |  |  |
| R5-243360 | Addition of NTN distance-based CHO tests 14.2.1.5 and 14.2.1.6 | Qualcomm France | agreed |  |  |
| R5-243361 | Addition of NTN inter-frequency Measurement Initiation Cell Reselection tests 14.1.7 and 14.1.8 | Qualcomm France | agreed |  |  |
| R5-243362 | Addition of NTN L1-RSRP Measurement Accuracy tests 14.6.4.1 and 14.6.4.2 | Qualcomm France | agreed |  |  |
| R5-243363 | Update to NTN general sections | Qualcomm France | agreed |  |  |
| R5-243364 | Update to NR-U Cases 10.4.1.1 and 10.4.1.4 including TTs | Qualcomm France | agreed |  |  |
| R5-243365 | Update to NR-U Timing Accuracy Test 10.2.1.1 including TTs | Qualcomm France | agreed |  |  |
| R5-243366 | Update to NR-U Timing Advance Test 10.2.2.1 including TTs | Qualcomm France | agreed |  |  |
| R5-243367 | Update to NR-U L1-RSRP measurement reporting tests 10.4.3.x including TTs | Qualcomm France | agreed |  |  |
| R5-243368 | Update to SCell activation and deactivation tests 10.3.3.x including TTs | Qualcomm France | agreed |  |  |
| R5-243369 | Addition of NR-U EN-DC PSCell addition and release delay test case including TTs | Qualcomm France | agreed |  |  |
| R5-243370 | Addition of NR-U SA FR1 UE transmit timing and timing advance test cases including TTs | Qualcomm France | agreed |  |  |
| R5-243371 | Addition of NR-U SA FR1 Scell activation and deactivation test cases including TTs | Qualcomm France | agreed |  |  |
| R5-243372 | Update to Annex E and F for NR-U RRM tests | Qualcomm France | revised |  | R5-243733 |
| R5-243373 | Removal of interruption during SCell operation NR-U tests | Qualcomm France | agreed |  |  |
| R5-243374 | Update to NSA measurement accuracy NR-U tests including TT | Qualcomm France | revised |  | R5-243734 |
| R5-243375 | Update to SA measurement accuracy NR-U tests including TT | Qualcomm France | revised |  | R5-243735 |
| R5-243376 | Update to NSA event-triggered measurement NR-U tests including TT | Qualcomm France | revised |  | R5-243892 |
| R5-243377 | Addition of testing methodology for Shared Spectrum | Qualcomm France | agreed |  |  |
| R5-243378 | Clean up of pending items in section 10 NR-U tests | Qualcomm France | agreed |  |  |
| R5-243379 | Update to Annex H for NR-U | Qualcomm France | agreed |  |  |
| R5-243380 | Clean up of pending items in section 11 NR-U tests | Qualcomm France | agreed |  |  |
| R5-243381 | Editorial update to Annex D for NR-U | Qualcomm France | agreed |  |  |
| R5-243382 | Update to OCNG modeling for NR-U | Qualcomm France | agreed |  |  |
| R5-243383 | Correction to gap-based measurement tests for RedCap | Qualcomm France | revised |  | R5-243904 |
| R5-243384 | Correction to stationary relaxed measurement criterion tests | Qualcomm France | agreed |  |  |
| R5-243385 | Correction to stationary relaxed measurement criterion inter-RAT tests | Qualcomm France | withdrawn |  |  |
| R5-243386 | Correction to re-establishment test cases for RedCap | Qualcomm France | revised |  | R5-243903 |
| R5-243387 | Correction to event-triggered measurement test cases for RedCap | Qualcomm France | withdrawn |  |  |
| R5-243388 | Editorial correction to section 18 title | Qualcomm France | agreed |  |  |
| R5-243389 | Update to gap-based measurement reporting tests for HST | Qualcomm France | revised |  | R5-243711 |
| R5-243390 | Update to Rel.17 HST CA measurement reporting tests | Qualcomm France | revised |  | R5-243901 |
| R5-243391 | Correction to gap-based measurement tests | Qualcomm France | agreed |  |  |
| R5-243392 | Correction to SUL test cases | Qualcomm France | revised |  | R5-243902 |
| R5-243393 | Correction to Handover tests | Qualcomm France | agreed |  |  |
| R5-243394 | Addition of applicability table for NR NTN RRM tests | Qualcomm France | agreed |  |  |
| R5-243395 | Update to NR-U test applicability | Qualcomm France | agreed |  |  |
| R5-243396 | Applicability correction to gap-based measurement tests for RedCap | Qualcomm France | agreed |  |  |
| R5-243397 | Applicability correction to gap-based measurement tests | Qualcomm France | agreed |  |  |
| R5-243398 | Correction to NTN PICS | Qualcomm France | agreed |  | - |
| R5-243399 | Addition of TTs for NR-U intra-frequency measurement reporting test | Qualcomm France | revised |  | R5-243895 |
| R5-243400 | Addition of TTs for NR-U Timing Accuracy Test | Qualcomm France | agreed |  |  |
| R5-243401 | Addition of TTs for NR-U Timing Advance Test | Qualcomm France | agreed |  |  |
| R5-243402 | Addition of TTs for NR-U L1-RSRP measurement reporting tests | Qualcomm France | revised |  | R5-243743 |
| R5-243403 | Addition of TTs for SCell activation and deactivation tests | Qualcomm France | agreed |  |  |
| R5-243404 | Addition of TTs for PSCell addition and release test | Qualcomm France | revised |  | R5-243753 |
| R5-243405 | SR of NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 | Apple Benelux B.V., Nokia | noted |  |  |
| R5-243406 | WP for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 | Apple Benelux B.V., Nokia | noted |  |  |
| R5-243407 | Updates across TS 38.561 V17.0.0 | Apple Benelux B.V. | revised |  | R5-243799 |
| R5-243408 | Editorial updates within TRP tests | Apple Benelux B.V. | agreed |  |  |
| R5-243409 | Editorial updates within TRS tests | Apple Benelux B.V. | agreed |  |  |
| R5-243410 | Maintainence Work Plan for Rel17 TRP TRS | Apple Benelux B.V. | withdrawn |  |  |
| R5-243411 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | withdrawn |  | - |
| R5-243412 | Updates to FR2 RF phase continuity test | Apple Benelux B.V. | revised |  | R5-243661 |
| R5-243413 | ICS updates pertaining to RF phase continuity conformance tests | Apple Benelux B.V. | withdrawn |  | - |
| R5-243414 | Applicability updates for Phase continuity tests | Apple Benelux B.V. | revised |  | R5-243813 |
| R5-243415 | Annex F updates for NTN RF test on Tx on-off time mask | Apple Benelux B.V. | revised |  | R5-243681 |
| R5-243416 | Annex F updates for NTN RF test on Spectrum emission mask | Apple Benelux B.V. | revised |  | R5-243814 |
| R5-243417 | Annex F updates for NTN RF test on ACLR | Apple Benelux B.V. | agreed |  |  |
| R5-243418 | MU TT updates for NTN RF tests | Apple Benelux B.V. | withdrawn |  |  |
| R5-243419 | Common clause updates related to NTN band n254 | Apple Benelux B.V., Keysight | revised |  | R5-243682 |
| R5-243420 | MOP requirement updates related to NTN band n254 | Apple Benelux B.V., Keysight | revised |  | R5-243683 |
| R5-243421 | A-MPR requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | revised |  | R5-243684 |
| R5-243422 | REFSENS requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | agreed |  |  |
| R5-243423 | IBB requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | revised |  | R5-243685 |
| R5-243424 | WP - UE Conformance Test Aspects for NR-based Access to Unlicensed Spectrum | Qualcomm Inc | available |  |  |
| R5-243425 | SR - UE Conformance Test Aspects for NR-based Access to Unlicensed Spectrum | Qualcomm Inc | noted |  |  |
| R5-243426 | WP UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN) | Qualcomm Inc | noted |  |  |
| R5-243427 | SR UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN) | Qualcomm Inc | noted |  |  |
| R5-243428 | WP UE Conformance Test Aspects - Further enhancement on NR demodulation performance | Qualcomm Inc | noted |  |  |
| R5-243429 | Simulation results to derive min test time for nb-ntn demod test cases | Qualcomm Inc | noted |  |  |
| R5-243430 | Update to nb-ntn demod test case to update min test time | Qualcomm Inc | revised |  | R5-243926 |
| R5-243431 | Simulation results to derive min test time for nr-ntn pdsch demod test cases | Qualcomm Inc | noted |  |  |
| R5-243432 | Addition of NSA SDR test case for DL 1024QAM | Qualcomm Inc | revised |  | R5-243696 |
| R5-243433 | Addition of MU/TT analysis for inter-cell CRS interference | Qualcomm Inc | agreed |  |  |
| R5-243434 | Update to inter-cell CRS interference test case | Qualcomm Inc | agreed |  |  |
| R5-243435 | Corrections to the FRC Table A.3.2.2.2-29 | Qualcomm Inc | agreed |  |  |
| R5-243436 | Corrections to Table G.1.5-1 for entries corresponding to FR1.15-1 | Qualcomm Inc | revised |  | R5-243699 |
| R5-243437 | Applicability spec update for inter-cell CRS interference test case | Qualcomm Inc | agreed |  |  |
| R5-243438 | Addition of connection diagram for CSI test cases for inter-cell interference scenarios | Qualcomm Inc | revised |  | R5-243900 |
| R5-243439 | Appllicability spec update to allow skipping 2DLCA and 3DCLA PDSCH Demodulation test cases | Qualcomm Inc | agreed |  | - |
| R5-243440 | Applicability spec update for NR-NTN test cases | Qualcomm Inc | agreed |  |  |
| R5-243441 | Update to nrn-ntn pdsch demod test case to update min test time | Qualcomm Inc | revised |  | R5-243899 |
| R5-243442 | Update for PC1.5 n39 A-SE | CMCC | agreed |  |  |
| R5-243443 | Correction to RedCap inter-RAT measurement test cases | Huawei, HiSilicon, Qualcomm | revised |  | R5-243919 |
| R5-243444 | Editorial correction to applicability of TC 12.1.2.1 | ZTE Corporation | agreed | - | - |
| R5-243445 | Add new RedCap HD-FDD Test Case | Huawei, HiSilicon | revised | - | R5-243585 |
| R5-243446 | ICS updates pertaining to RF phase continuity conformance tests | Apple Benelux B.V. | revised | - | R5-243612 |
| R5-243447 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | revised | - | R5-243650 |
| R5-243448 | Update Test Point analysis for NR NTN test cases | Keysight Technologies | agreed | - | - |
| R5-243449 | LS regarding the development of NB-IoT test cases for Release 15 and Release 16 | GSMA TSGIoT | noted | - | - |
| R5-243450 | MCC TF160 Status Report | MCC TF160 | approved | R5-242308 | - |
| R5-243451 | New WID on UE Conformance - Further NR mobility enhancements | MediaTek Inc., Apple, China Telecom | agreed | R5-242124 | - |
| R5-243452 | New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation | MediaTek Inc., China Telecom | agreed | R5-242125 | - |
| R5-243453 | New WID on UE Conformance - Introduction of the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) for IoT NTN | MediaTek Inc., Inmarsat | agreed | R5-242126 | - |
| R5-243454 | New WID on UE Conformance Test Aspects - Expanded and improved NR positioning | CATT, CAICT | agreed | R5-242239 | - |
| R5-243455 | New WID on UE Conformance – NR demodulation performance evolution | China Telecom, Qualcomm, Samsung | agreed | R5-242446 | - |
| R5-243456 | New WID on UE Conformance - Support for UAV (Uncrewed Aerial Vehicles) and Further Architecture Enhancement for UAV and UAM | Qualcomm Incorporated, Nokia, China Unicom | noted | R5-242540 | - |
| R5-243457 | New WID on UE Conformance - Mobile Terminated-Small Data Transmission (MT-SDT) for NR | Qualcomm Incorporated, China Telecom, ZTE | agreed | R5-242541 | - |
| R5-243458 | New WID on UE Conformance - Protocol enhancements for Mission Critical Services (MCPTT, MCVideo, MCData) for Rel-17 | Ericsson, FirstNet, NIST, AT&T, Element, Samsung | noted | R5-242605 | - |
| R5-243459 | New WID on UE Conformance – Enhanced support of Non-Public Networks Phase 2 | China Telecom, Qualcomm, ZTE | revised | R5-242691 | R5-243973 |
| R5-243460 | New WID on UE Conformance - Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC) | Apple, ROHDE & SCHWARZ, Vivo | agreed | R5-242841 | - |
| R5-243461 | New WID on UE Conformance - In-Device Co-existence (IDC) enhancements for NR and MR-DC | CMCC, Xiaomi | agreed | R5-243079 | - |
| R5-243462 | UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | Apple, CAICT, Keysight Technologies | agreed | R5-243222 | - |
| R5-243463 | New WID on UE Conformance 4Rx for NR bands for FWA UEs (up to 2.6GHz) and handheld UEs (1GHz to 2.6GHz) | ZTE, China Telecom | agreed | R5-243284 | - |
| R5-243464 | New WID on UE Conformance - NR NTN (Non-Terrestrial Networks) enhancements | Qualcomm Inc, China Unicom, Mediatek | noted | R5-243355 | - |
| R5-243465 | New WID on UE Conformance - Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception | Qualcomm Inc | agreed | R5-243356 | - |
| R5-243466 | Challenges and Proposals for Testing eSIM-only Devices | Qualcomm Incorporated | noted | R5-242567 | - |
| R5-243467 | Proposal for new test case related to ACB per PLMN | NTTDOCOMO, INC. | noted | R5-242949 | - |
| R5-243468 | Correction of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit | Vodafone GmbH, MCC TF 160 | agreed | R5-242121 | - |
| R5-243469 | Addition of applicability of new test case 8.1.3.6b for redir-policy bit | Vodafone GmbH, MCC TF160 | agreed | R5-242122 | - |
| R5-243470 | Correction to LTE RACS Test case 9.2.5.1 | Anritsu EMEA Ltd | agreed | R5-242758 | - |
| R5-243471 | Editorial correction for Specific message contents in 9.2.1.1.20 | NTT DOCOMO, INC. | agreed | R5-242859 | - |
| R5-243472 | Editorial correction for Specific message contents in 9.2.3.1.22 | NTT DOCOMO, INC. | agreed | R5-242861 | - |
| R5-243473 | Applicability updates for EPS P-CSCF restoration test cases | Qualcomm Incorporated, Orange | agreed | R5-242941 | - |
| R5-243474 | Correction to LTE-IRAT test cases 11.2.11 | ROHDE & SCHWARZ | agreed | R5-242593 | - |
| R5-243475 | Update of cl 8.5.4.7 Inter RAT handover to UTRAN | MCC TF160, Rohde & Schwarz | agreed | - | - |
| R5-243476 | Addition of new test case related to SSAC per PLMN 13.5.1.c | NTTDOCOMO, INC. | agreed | R5-242954 | - |
| R5-243477 | Correction to NBIOT testcase 22.5.21 | ROHDE & SCHWARZ | agreed | R5-242470 | - |
| R5-243478 | Correction to NTN test cases 22.1.2, 22.2.13, 22.4.30 and 22.5.23 | ROHDE & SCHWARZ | withdrawn | R5-242526 | - |
| R5-243479 | Correction to NTN test cases | ROHDE & SCHWARZ | agreed | - | - |
| R5-243480 | Correction to NTN TA report test case 22.3.1.13 | Qualcomm Incorporated, Rohde & Schwarz | agreed | R5-242720 | - |
| R5-243481 | Addition of DCI information for semi static channel access in shared spectrum | Qualcomm Incorporated | agreed | - | - |
| R5-243482 | Common configuration change for SDT condition | Nokia, Huawei | agreed | - | - |
| R5-243483 | Configuration fix for testcase 8.1.5.13.2 | Nokia | agreed | - | - |
| R5-243484 | Discussion to add new RedCap HD-FDD TC | Huawei, HiSilicon | noted | R5-242831 | - |
| R5-243485 | 5GS EPS fallback test coverage | Ericsson | noted | R5-243117 | - |
| R5-243486 | Disc on handling of MDT IDC problem | CMCC, CATT, Qualcomm | noted | R5-243074 | - |
| R5-243487 | Discussion Paper for Rel-17 eNS_Ph2 WI NSAC Session management tests case applicability | CATT | noted | R5-242942 | - |
| R5-243488 | Tracking Rel-15 5GS EN-DC and SA NR Maintenance Workplan for protocol testing | Qualcomm Incorporated | noted | R5-242568 | - |
| R5-243489 | Editorial correction for generic procedure 4.5.4, 4.5.5 and 4.5.7 | ZTE Corporation | agreed | R5-242576 | - |
| R5-243490 | Add IE Aerial-Config | Ericsson | agreed | R5-242648 | - |
| R5-243491 | Add IE EpochTime | Ericsson | agreed | R5-242649 | - |
| R5-243492 | Add IEs SL-DRX-Config, SL-DRX-ConfigGC-BC, SL-DRX-ConfigUC and SL-DRX-ConfigUC-SemiStatic | Ericsson | agreed | R5-243216 | - |
| R5-243493 | Correction to IE PDCCH-ConfigCommon | MediaTek Inc. | agreed | R5-243087 | - |
| R5-243494 | Introduction of signalling test frequencies for band n85 | Nokia | agreed | R5-242363 | - |
| R5-243495 | Add new PICS for RedCap HD-FDD TC | Huawei, HiSilicon | agreed | R5-242832 | - |
| R5-243496 | Editorial changes to tables of 38.508-2 | Qualcomm Technologies Ireland | agreed | R5-242544 | - |
| R5-243497 | Correction to NR5GC testcase 6.1.2.13 | ROHDE & SCHWARZ | agreed | R5-242592 | - |
| R5-243498 | Correction to PDCP TC 7.1.3.4.1 | MediaTek Inc. | agreed | R5-242151 | - |
| R5-243499 | Correction to PDCP TC 7.1.3.4.2 | MediaTek Inc. | agreed | R5-242152 | - |
| R5-243500 | Update the NSAC test case titles | CATT | revised | - | R5-243590 |
| R5-243501 | Updates to NR RRC test case 8.1.1.2.4 | MCC TF160, MediaTek | agreed | R5-242539 | - |
| R5-243502 | Corrections to conditional handover test case 8.1.4.4.1 | Qualcomm Incorporated | agreed | R5-242561 | - |
| R5-243503 | Correction to NR Idle measurement TC 8.1.5.11.1 | MediaTek Inc. | agreed | R5-242154 | - |
| R5-243504 | Addition of E-UTRA to NR MPS priority Indication test case | Qualcomm Incorporated | agreed | R5-242563 | - |
| R5-243505 | Correction to NR RRC UL segmentation test case 8.1.5.9.1 | Qualcomm Incorporated | agreed | R5-242741 | - |
| R5-243506 | Correction to NR test case 8.1.5.8.1-processing delay | Huawei, HiSilicon | agreed | R5-242815 | - |
| R5-243507 | Correction to MDT enh. TC 8.1.6.1.3.8 | MediaTek Inc. | agreed | R5-242159 | - |
| R5-243508 | Correction to MDT enh. TC 8.1.6.1.3.9 | MediaTek Inc. | agreed | R5-242160 | - |
| R5-243509 | Correction to MDT enh. TC 8.1.6.1.3.10 | MediaTek Inc. | agreed | R5-242161 | - |
| R5-243510 | Correction to MDT enh. TC 8.1.6.1.3.11 | MediaTek Inc. | agreed | R5-242162 | - |
| R5-243511 | Correction to NEDC test case 8.2.2.8.3 | Huawei, HiSilicon, MCC TF160, Mediatek | agreed | R5-242816 | - |
| R5-243512 | Corrrection to RRC test case 8.2.3.8.2x | Keysight Technologies UK | agreed | R5-243350 | - |
| R5-243513 | Corrrection to RRC test case 8.2.3.12.2 | Keysight Technologies UK | agreed | R5-243351 | - |
| R5-243514 | Correction to eNS test case 9.3.1.4 | Anritsu EMEA Ltd, MediaTek | agreed | R5-242761 | - |
| R5-243515 | Correction to UAC TC 11.3.12 | MediaTek Inc. | agreed | R5-242173 | - |
| R5-243516 | Correction for MBS TC 14.2.4.2.1 | ZTE Corporation | agreed | R5-242594 | - |
| R5-243517 | Correction for MBS TC 14.2.4.3.2 | ZTE Corporation | agreed | R5-242595 | - |
| R5-243518 | Editorial correction for MBS TC of 14.2.4.1 | ZTE Corporation | agreed | R5-242596 | - |
| R5-243519 | Editorial update of applicability for TC 8.2.4.2.1.x | MediaTek Inc. | agreed | R5-242174 | - |
| R5-243520 | Corrections to applicability of NR supplemental uplink test case | Qualcomm Incorporated | agreed | R5-242569 | - |
| R5-243521 | Corrections to applicability of Network Slice Admision Control mobility management aspects test cases | Qualcomm Incorporated | agreed | R5-242570 | - |
| R5-243522 | Update to R17 ATSSS test case 11.9.2 | China Telecom, MediaTek | agreed | R5-242464 | - |
| R5-243523 | New MO speech EVS test case 12.23a | Ericsson | revised | R5-242623 | R5-243984 |
| R5-243524 | New MT speech EVS test case 12.24a | Ericsson | revised | R5-242624 | R5-243985 |
| R5-243525 | Correction of IMSo5G test case 7.23 | MCC TF160 | agreed | R5-242336 | - |
| R5-243526 | Updates to Annex A.15.2 | Ericsson | agreed | R5-242618 | - |
| R5-243527 | Updates to test case 7.4a | Ericsson, Rohde & Schwarz | agreed | R5-242621 | - |
| R5-243528 | Updates to test case 7.6a | Ericsson, Rohde & Schwarz | agreed | R5-242622 | - |
| R5-243529 | Correction to NR IMS TC 7.24b - Forking | Huawei, Hisilicon | withdrawn | R5-242742 | - |
| R5-243530 | Addition of PICS for ING_5GS test cases | Qualcomm Incorporated, China Telecom | agreed | R5-242733 | - |
| R5-243532 | Addition of signalling test frequencies for CA_n48C | Ericsson, Verizon, MCC TF160 | withdrawn | R5-242535 | - |
| R5-243533 | Addition of signalling test frequencies for CA_n48C and CA_n5B | Ericsson, Verizon, MCC TF160 | agreed | - | - |
| R5-243534 | Addition of NR sidelink SIG test case 12.1.1.1 | Huawei, HiSilicon | agreed | R5-242911 | - |
| R5-243535 | Correction to NR sidelink SIG test case 12.2.5.2 | Huawei, HiSilicon | agreed | R5-242913 | - |
| R5-243536 | Update of Logical Channel Config Table 4.6.3-66 | Qualcomm Incorporated | agreed | R5-242771 | - |
| R5-243537 | Correction of SDT TC 7.1.1.13.5-cg-SDT-TATimer | Huawei, HiSilicon, MCC TF160 | agreed | R5-243084 | - |
| R5-243538 | Correction of SDT TC 8.1.5.13.1-cg-SDT | Huawei, HiSilicon, MCC TF160 | agreed | R5-243085 | - |
| R5-243539 | Modification of testcase 8.1.5.13.3 for SDT-SRB2-Indication | Nokia | agreed | R5-243170 | - |
| R5-243540 | Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN | China Telecom | agreed | R5-242677 | - |
| R5-243541 | Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 Enpn | China Telecom | agreed | R5-242678 | - |
| R5-243542 | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN | China Telecom | agreed | R5-242682 | - |
| R5-243543 | Addition of PICS for FeMIMO | Huawei, HiSilicon | agreed | R5-243157 | - |
| R5-243544 | Adding test applicability for multi-TRP PUSCH repetition test cases | Huawei, HiSilicon | agreed | R5-243158 | - |
| R5-243545 | Correction for test procedure 4.9.41 and 4.9.42 | ZTE Corporation | withdrawn | R5-242575 | - |
| R5-243546 | Correction for test procedure 4.9.41 | ZTE Corporation | agreed | - | - |
| R5-243547 | Update of Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side | TDIA, CATT | agreed | R5-242801 | - |
| R5-243548 | Update of Table 4.6.1A-8 uuMessageTransferSidelink | TDIA, CATT | agreed | R5-242804 | - |
| R5-243549 | Addition of Default HTTP messages for communication with the 5G ProSe | TDIA, CATT | agreed | R5-242807 | - |
| R5-243550 | Addition of Default TLS message and information element contents | TDIA, CATT | agreed | R5-242808 | - |
| R5-243551 | Addition of spec reference of 3GPP TS 33.310, 33.503 and IETF RFC 5246 for 5G ProSe information elements | TDIA, CATT | agreed | R5-242809 | - |
| R5-243552 | Update of test applicability of NSAC test cases | CATT | agreed | R5-242662 | - |
| R5-243553 | Update to R17 ATSSS test case 11.9.1 | China Telecom, MediaTek | agreed | R5-242703 | - |
| R5-243554 | Added Common test environment for NR-NTN test cases | Qualcomm Technologies Ireland | agreed | R5-242543 | - |
| R5-243555 | Addition of system information combination for inter frequency NTN scenario | Qualcomm Incorporated | agreed | R5-242760 | - |
| R5-243556 | Correction to NR NTN TC 9.4.1.1 | MediaTek Inc. | agreed | R5-242135 | - |
| R5-243557 | Correction to NR NTN TC 9.4.1.2 | MediaTek Inc. | agreed | R5-242136 | - |
| R5-243558 | Introduction of new TC NR-NTN - TC TAI(s) not allowed / forbidden TAI(s) | Qualcomm Technologies Ireland | agreed | R5-242542 | - |
| R5-243559 | Introduction of new TC NR-NTN - ntn-UlSyncValidityDuration, T430 Expiry | Qualcomm Technologies Ireland | agreed | R5-242547 | - |
| R5-243560 | Updates to NTN test cases with missing NGSO configuration | Qualcomm Technologies Ireland | agreed | R5-242548 | - |
| R5-243561 | Addition of new NR NTN conditional handover CondEventD1 test case | Qualcomm Technologies Ireland | agreed | R5-242550 | - |
| R5-243562 | Addition of NR NTN cell reselection test case 6.1.2.18a | Qualcomm Incorporated | agreed | R5-242767 | - |
| R5-243563 | Addition of NR NTN cell reselection test case 6.1.2.18b | Qualcomm Incorporated | agreed | R5-242768 | - |
| R5-243564 | Addition of NR NTN RLC t-reassembly timer test case 7.1.2.3.9a | Qualcomm Incorporated | agreed | R5-242770 | - |
| R5-243565 | Correction to the applicability condition of NR NTN | MediaTek Inc. | agreed | R5-242137 | - |
| R5-243566 | Signalling test frequencies for 3 MHz and band n106 | Nokia | agreed | R5-242369 | - |
| R5-243567 | Progress and Way Forward on SW Implementation on Machine Readable PICS and Applicability | Qualcomm Incorporated | revised | R5-242560 | R5-243970 |
| R5-243568 | New WID on UE Conformance - New bands and bandwidth allocation for LTE based 5G terrestrial broadcast | Qualcomm Inc | noted | R5-242242 | - |
| R5-243569 | Correction to LPP positioning test case 9.3.4.2 | NTT DOCOMO Inc., MCC TF160 | agreed | R5-242950 | - |
| R5-243570 | Addition of new TC 6.5.1 Location Information Request | NIST | agreed | R5-243267 | - |
| R5-243571 | Addition of new TC 6.5.2 Location Report triggering | NIST | agreed | R5-243268 | - |
| R5-243572 | Addition of new TC 5.8 Rules based affiliation status change | NIST | agreed | R5-243269 | - |
| R5-243573 | Addition of new TC 5.9 Location based functional alias status change | NIST | agreed | R5-243270 | - |
| R5-243574 | Addition of generci procedure for UUAA-MM | Qualcomm Incorporated | agreed | R5-242565 | - |
| R5-243575 | Updates to UAS TC 9.1.5.2.11 | Ericsson | agreed | R5-243124 | - |
| R5-243576 | Updates to UAS TC 10.1.4.2 | Ericsson | agreed | R5-243125 | - |
| R5-243577 | Addition of applicability for new test case related to SSAC per PLMN 13.5.1.c | NTTDOCOMO, INC. | agreed | R5-242955 | - |
| R5-243578 | Optimize the tables for IoT NTN | MediaTek Inc., Anritsu | agreed | R5-242187 | - |
| R5-243579 | NR SDT: Addition of NR/5GC Test Model aspects | MCC TF160 | agreed | R5-242319 | - |
| R5-243580 | Update of message contents for 8.2.3.18.4 conditional PSCell change | Huawei, HiSilicon | agreed | R5-242857 | - |
| R5-243581 | Addition of a new section for IDC Configuration | CMCC, CATT, Qualcomm | agreed | R5-243075 | - |
| R5-243582 | Addition of new PICS for IDC mechanism | CMCC, CATT, Qualcomm | agreed | R5-243076 | - |
| R5-243583 | Update of test case 8.1.6.1.2.14 for SON_MDT | CMCC, CATT, Qualcomm | agreed | R5-243077 | - |
| R5-243584 | Update of applicability for test cases 8.1.6.1.2.14 | CMCC, CATT, Qualcomm | agreed | R5-243078 | - |
| R5-243585 | Add new RedCap HD-FDD Test Case | Huawei, HiSilicon | agreed | R5-243445 | - |
| R5-243586 | Add test applicability for new RedCap HD-FDD TC | Huawei, HiSilicon | agreed | R5-242834 | - |
| R5-243587 | Correction to NR5GC testcase 6.3.1.10 | ROHDE & SCHWARZ | agreed | R5-242489 | - |
| R5-243588 | Corrrection to NR5GC CAG test case 6.5.2.1 | Keysight Technologies UK | agreed | R5-242793 | - |
| R5-243589 | Correction of MAC TC 7.1.1.1.4-Beam Failure | Huawei, HiSilicon | agreed | R5-242813 | - |
| R5-243590 | Update the NSAC test case titles | CATT | agreed | R5-243500 | - |
| R5-243591 | Correction to NR 5GC Multilayer TC 11.1.7-Emergency Services Fallback to EPS with redirection | Huawei, HiSilicon | agreed | R5-242817 | - |
| R5-243592 | Correction to NR5GC testcase 11.3.12 | ROHDE & SCHWARZ | agreed | R5-242496 | - |
| R5-243593 | Correction to NR 5GC Multilayer TC 11.4.12-Emergency Services | Huawei, HiSilicon | agreed | R5-242818 | - |
| R5-243594 | Correction of NR eCall TC 11.5.5-Limited service | Huawei, HiSilicon | agreed | R5-242819 | - |
| R5-243595 | Addition to legacy test cases applicability for RedCap UE | MediaTek Inc. | agreed | R5-242279 | - |
| R5-243596 | Addition of applicability of E-UTRA to NR MPS priority Indication test case | Qualcomm Incorporated | agreed | R5-242564 | - |
| R5-243597 | Updates to SIB19 and ephemeris information for NGSO and GSO condition | Qualcomm Technologies Ireland, MediaTek Inc. | agreed | R5-242546 | - |
| R5-243598 | Addition of new NR-NTN conditional handover condEventA4 test case | Qualcomm Technologies Ireland | agreed | R5-242549 | - |
| R5-243599 | Addition of new NR NTN conditional handover CondEventT1 test case | Qualcomm Technologies Ireland | agreed | R5-242551 | - |
| R5-243600 | Update inter-band NR CA configuration for CA_n3A-n8A-n78A | China Unicom | agreed | R5-243031 | - |
| R5-243601 | Addition of 3 MHz CBW test frequencies for NR Bands n26, n28, n85 and n100 | Nokia | agreed | R5-242394 | - |
| R5-243602 | Introduction of test channel bandwidths and test frequencies for band n85 | Nokia | agreed | R5-242362 | - |
| R5-243603 | Update of editors note for test frequencies | Bureau Veritas ADT | agreed | R5-242788 | - |
| R5-243604 | Addition of missing TRS configurations | Rohde & Schwarz | agreed | R5-243090 | - |
| R5-243605 | Discussion regarding handling of missing test frequencies for UL CA combinations. | Ericsson | noted | R5-242532 | - |
| R5-243606 | Addition of physical layer baseline implementation capabilities for 3Tx NR CA and EN-DC | Huawei, HiSilicon | agreed | R5-242920 | - |
| R5-243607 | Addition of CA_n1A-n8A-n78A,CA_n3A-n8A-n78A into NR inter-band CA within FR1 three bands | China Unicom | agreed | R5-243189 | - |
| R5-243608 | Update for additional NRCA band configurations | Verizon Spain | agreed | R5-242461 | - |
| R5-243609 | Addition and correction of RF baseline implementation capabilities for PC2 EN-DC combs within FR1 | KDDI Corporation, MediaTek Inc | agreed | R5-242251 | - |
| R5-243610 | Addition of n41 PC1 RF baseline ICS | CMCC, Nokia, Nokia Shanghai Bell | agreed | R5-242498 | - |
| R5-243611 | Introduction of band n85 for physical layer baseline implementation capabilities | Nokia | agreed | R5-242364 | - |
| R5-243612 | ICS updates pertaining to RF phase continuity conformance tests | Apple Benelux B.V. | agreed | R5-243446 | - |
| R5-243613 | Updating TP analysis for AMPR testing for NS_43 | Huawei, HiSilicon | agreed | R5-243206 | - |
| R5-243614 | TP analysis for PC1.5 n39 A-MPR and A-SE | CMCC | agreed | R5-243066 | - |
| R5-243615 | Addition of reference sensitivity for new PC2 EN-DC combs within FR1 | KDDI Corporation | agreed | R5-242253 | - |
| R5-243616 | Update reference sensitivity TC for CA_n3A-n8A-n78A | China Unicom | agreed | R5-243032 | - |
| R5-243617 | Addition of 3DL CA reference sensitivity TC into 7.3A.2 for CA_n3A-n8A-n78A | China Unicom | agreed | R5-243030 | - |
| R5-243618 | Adding TP analysis for new FR1 test case 6.3A.3.6 and 6.3A.3.7 | Huawei, Hisilicon | agreed | R5-243214 | - |
| R5-243619 | Adding TP analysis for new SUL test case 6.3C.3.6 | Huawei, Hisilicon | revised | R5-243215 | R5-243796 |
| R5-243620 | Adding new SUL test case Time mask for switching across three uplink bands | Huawei, HiSilicon | agreed | R5-243211 | - |
| R5-243621 | Addition of TP analysis for NR V2X MPR for PC2 and intra-band concurrent operation | Huawei, HiSilicon | agreed | R5-243147 | - |
| R5-243622 | Addition of TP analysis for NR V2X AMPR for PC2 and intra-band concurrent operation | Huawei, HiSilicon | agreed | R5-243149 | - |
| R5-243623 | Apply interlaced RB allocation in test configuration table in test case 6.4F.2.2 | Ericsson | agreed | R5-242528 | - |
| R5-243624 | Updating the TP analysis for AMPR testing for CA_n41A-n79A | Huawei, HiSilicon | agreed | R5-243203 | - |
| R5-243625 | Update of Rx test cases to add 50MHz for NR band n7 | MediaTek Beijing Inc. | agreed | R5-242284 | - |
| R5-243626 | Addition of test case 6.3H.3.1 Minimum output power for inter-band UL CA with UL MIMO | TTA | agreed | R5-242844 | - |
| R5-243627 | Addition of test case 6.5H.3.1 Occupied bandwidth for inter-band UL CA with UL MIMO | TTA | agreed | R5-242845 | - |
| R5-243628 | Addition of new test cases 6.2L.3.x for inter-band CA with TxD | Huawei, HiSilicon | agreed | R5-242925 | - |
| R5-243629 | Update of NR V2X AMPR test cases | Huawei, HiSilicon | agreed | R5-243137 | - |
| R5-243630 | Update to Annex F for V2X test cases | Huawei, HiSilicon | agreed | R5-243228 | - |
| R5-243631 | Updating PC2 AMPR testing for NS_100 for band n8 | Huawei, HiSilicon | agreed | R5-243207 | - |
| R5-243632 | Addition of delta RIB into 7.3A.0.3.2.3 for CA_n3A-n8A-n78A | China Unicom | agreed | R5-243028 | - |
| R5-243633 | Correction of spurious emission minimum conformance requirements for UE co-ex for intra-band contiguous CA | Apple, Keysight | agreed | R5-243257 | - |
| R5-243634 | Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-17 | Apple, Keysight | agreed | R5-243261 | - |
| R5-243635 | Correction to reference sensitivity test requirements for CA_n25A-n77A, CA_n25A-n78A and CA_n66A-n78A | Nokia | agreed | R5-242375 | - |
| R5-243636 | General updates of TS 38.521-1 clause 5 for R17 CA configurations | China Unicom, Verizon, KDDI, WE Certification Oy, AT&T | agreed | R5-242573 | - |
| R5-243637 | Addition of 3 MHz channel bandwidth for transmitter requirements | Nokia | agreed | R5-242396 | - |
| R5-243638 | Addition of 3 MHz channel bandwidth for receiver requirements | Nokia | agreed | R5-242397 | - |
| R5-243639 | Introduction of dedicated spectrum less than 5MHz for FR1 to clause 5 and annex A | Nokia | agreed | R5-242395 | - |
| R5-243640 | Adding transmitter requirements for band n106 | Nokia | agreed | R5-242373 | - |
| R5-243641 | Adding receiver requirements for band n106 | Nokia | agreed | R5-242374 | - |
| R5-243642 | Adding reference sensitivity requirements for UEs that support optional symmetric UL/DL in band n71 | Nokia | agreed | R5-242381 | - |
| R5-243643 | Introduction of new band n106 into clause 5 | Nokia | agreed | R5-242372 | - |
| R5-243644 | Adding new SUL test case Time mask for switching across four uplink bands | Huawei, HiSilicon | agreed | R5-243212 | - |
| R5-243645 | Updating MU and TT for Time mask for switching across up to four bands testing | Huawei, HiSilicon | agreed | R5-243213 | - |
| R5-243646 | Updates to editors note in NR-U Tx test cases | Ericsson | agreed | R5-242490 | - |
| R5-243647 | Correction to Table 6.F-1 | Ericsson | agreed | R5-242602 | - |
| R5-243648 | Addition of MU and TT for NR-U test case | Ericsson | agreed | R5-242529 | - |
| R5-243649 | Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL | Apple, Keysight | agreed | R5-243258 | - |
| R5-243650 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | agreed | R5-243447 | - |
| R5-243651 | Adding receiver requirements for band n85 | Nokia | agreed | R5-242366 | - |
| R5-243652 | Additional test case for Enhanced Beam correspondence for PC6 | Samsung | agreed | R5-242672 | - |
| R5-243653 | Addition of REFSENS TP analysis for PC3 | NTT DOCOMO INC. | agreed | - | - |
| R5-243654 | Correction of test bandwidth for CA_n2(2A) | WE Certification Oy, AT&T, Anritsu | agreed | - | - |
| R5-243655 | Addition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A Tx test cases | WE Certification Oy, AT&T | agreed | R5-242705 | - |
| R5-243656 | Addition of reference sensitivity tests for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | agreed | R5-242706 | - |
| R5-243657 | Addition of capability declaration for DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A | WE Certification Oy, AT&T | agreed | R5-242713 | - |
| R5-243658 | Adding new test case Time mask for switching across three uplink bands | Huawei, HiSilicon | agreed | R5-243209 | - |
| R5-243659 | Adding new test case Time mask for switching across four uplink bands | Huawei, HiSilicon | agreed | R5-243210 | - |
| R5-243660 | Corrections on 6.2.2 and 6.2A.2.0.2 for UE MPR requirements | ZTE Corporation | agreed | R5-243302 | - |
| R5-243661 | Updates to FR2 RF phase continuity test | Apple Benelux B.V. | agreed | R5-243412 | - |
| R5-243662 | Corrections on 7.1 for general description to receiver characteristics | ZTE Corporation | agreed | R5-243303 | - |
| R5-243663 | Clarification of antenna array assumptions for in-band measurement | Anritsu | agreed | R5-243014 | - |
| R5-243664 | Correction to Rx fast spherical coverage method | Rohde & Schwarz | agreed | R5-243091 | - |
| R5-243665 | Core requirements alignment for UE co-existence of Inter-band within FR1 | CAICT | agreed | R5-242599 | - |
| R5-243666 | Addition of delta TIBc for R16 inter-band EN-DC within FR1 | KT Corp. | agreed | R5-243223 | - |
| R5-243667 | Addition of delta RIBc for R16 inter-band EN-DC within FR1 | KT Corp. | agreed | R5-243225 | - |
| R5-243668 | Addition of minimum requirements of reference sensitivity exceptions for R16 inter-band EN-DC within FR1 | KT Corp. | agreed | R5-243229 | - |
| R5-243669 | Update of R16 Configuration for inter-band EN-DC | KT Corp. | agreed | R5-243231 | - |
| R5-243670 | Correction of Reference Sensitivity test for DC_30A-n77A | WE Certification Oy, AT&T, Anritsu | agreed | R5-242716 | - |
| R5-243671 | Corrections on 6.2B.1.3 for UE MOP for configuration DC_38A_n78A | ZTE Corporation | agreed | R5-243304 | - |
| R5-243672 | Update of 7.3B.2 reference sensitivity for inter-band EN-DC | Huawei, HiSilicon | agreed | R5-242855 | - |
| R5-243673 | Introduction of new NR NTN test case - 6.4.2.2 Carrier leakage | Keysight Technologies | agreed | R5-242503 | - |
| R5-243674 | Introduction of new NR NTN test case - 6.4.2.3 In-band emissions | Keysight Technologies | agreed | R5-242504 | - |
| R5-243675 | Introduction of new NR NTN test case - 6.4.2.4 EVM equalizer spectrum flatness | Keysight Technologies | agreed | R5-242505 | - |
| R5-243676 | Update of NR NTN test case - 6.2.1 UE maximum output power | Keysight Technologies | agreed | R5-242507 | - |
| R5-243677 | Update of NR NTN test case - 6.2.3 UE additional maximum output power reduction | Keysight Technologies | agreed | R5-242509 | - |
| R5-243678 | Update MU and TT for NR NTN test cases | Keysight Technologies | agreed | R5-242511 | - |
| R5-243679 | Adding new NTN test case 6.3.4 Power control | Qualcomm Tech. Netherlands B.V | agreed | R5-242951 | - |
| R5-243680 | Updates on Annex for satellite access conformance test | ZTE Corporation | agreed | R5-243291 | - |
| R5-243681 | Annex F updates for NTN RF test on Tx on-off time mask | Apple Benelux B.V. | agreed | R5-243415 | - |
| R5-243682 | Common clause updates related to NTN band n254 | Apple Benelux B.V., Keysight | revised | R5-243419 | R5-243987 |
| R5-243683 | MOP requirement updates related to NTN band n254 | Apple Benelux B.V., Keysight | agreed | R5-243420 | - |
| R5-243684 | A-MPR requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | agreed | R5-243421 | - |
| R5-243685 | IBB requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | agreed | R5-243423 | - |
| R5-243686 | Editorial corrections to GSO frequency error test cases | Keysight Technologies UK Ltd | withdrawn | R5-243278 | - |
| R5-243687 | Update applicability for NB-NTN Channel quality reporting accuracy testcases | Qualcomm Technologies Int | agreed | R5-243334 | - |
| R5-243688 | Addition of new RRM TC 13.5.1 for IoT NTN enh | CMCC | agreed | R5-243057 | - |
| R5-243689 | Addition of new RRM TC 13.5.2 for IoT NTN enh | CMCC | agreed | R5-243058 | - |
| R5-243690 | Addition of new RRM TC 13.5.3 for IoT NTN enh | CMCC | agreed | R5-243059 | - |
| R5-243691 | Completion of FDD CQI reporting test cases with inter-cell interference | China Telecom | agreed | R5-242440 | - |
| R5-243692 | Completion of TDD CQI reporting test cases with inter-cell interference | China Telecom | agreed | R5-242441 | - |
| R5-243693 | Addition of MU and TT for CQI reporting test cases with inter-cell interference | China Telecom | agreed | R5-242442 | - |
| R5-243694 | Core spec alignment of general clause for CSI reporting requirements | Ericsson | agreed | R5-242493 | - |
| R5-243695 | Correction to PUCCH Resource ID in 2Rx FR1 RI reporting | Anritsu | agreed | R5-243017 | - |
| R5-243696 | Addition of NSA SDR test case for DL 1024QAM | Qualcomm Inc | agreed | R5-243432 | - |
| R5-243697 | Correction to PDSCH Reference Channel for FDD HST | Anritsu | agreed | R5-243016 | - |
| R5-243698 | Update mTT for missing DL RMC | Keysight Technologies | agreed | R5-243339 | - |
| R5-243699 | Corrections to Table G.1.5-1 for entries corresponding to FR1.15-1 | Qualcomm Inc | agreed | R5-243436 | - |
| R5-243700 | LS on phase continuity requirements for DMRS bundling for NR NTN | TSG WG RAN5 | withdrawn | - | - |
| R5-243701 | Correction to RedCap RRM test case 17.5.1.8 with TT | Huawei, HiSilicon | withdrawn | R5-242891 | - |
| R5-243702 | On Beam Correspondence initial access in Idle and FR2 RRM RACH Test | Apple AB Denmark | noted | R5-242792 | - |
| R5-243703 | Update to Unified TCI state test cases | Huawei, HiSilicon, Samsung | agreed | R5-243153 | - |
| R5-243704 | Correcting propagation AWGN in test cases without Noc | Huawei, HiSilicon | agreed | R5-243154 | - |
| R5-243705 | Adding TRP specific Minimum conformance requirements | Samsung | agreed | R5-242671 | - |
| R5-243706 | Correction of FR2 interruptions at SRS carrier based switching test case 5.5.2.7 and 5.5.2.8 | Ericsson | agreed | R5-242634 | - |
| R5-243707 | Addition of new test case 5.6.7.1 EN-DC event triggered reporting test without gap under non-DRX | Sporton | agreed | R5-242695 | - |
| R5-243708 | Addition of new test case 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used | Sporton | agreed | R5-242696 | - |
| R5-243709 | Addition of new test case 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Sporton | agreed | R5-242697 | - |
| R5-243710 | Addition of new test case 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Sporton | agreed | R5-242698 | - |
| R5-243711 | Update to gap-based measurement reporting tests for HST | Qualcomm France | agreed | R5-243389 | - |
| R5-243712 | Correction to RRM RedCap TC 16.5.1.7 | Qualcomm Technologies Int, Anritsu | agreed | R5-243335 | - |
| R5-243713 | Corrections to R17 RedCap 18.3.1.x test cases | Rohde & Schwarz | agreed | R5-242985 | - |
| R5-243714 | Update to R16 NR CADC configuration test cases applicability | CMCC | withdrawn | R5-243041 | - |
| R5-243715 | Editorial Correction of FR2 feMIMO test case | Nokia | agreed | R5-242610 | - |
| R5-243716 | Additional applicability for Inter-cell SSB based L1-RSRP measurements | Samsung | agreed | R5-242669 | - |
| R5-243717 | Additional applicability for HST FR2 test cases | Samsung | agreed | R5-242674 | - |
| R5-243718 | Applicabilty correction of RRM enhancements test cases | Ericsson | agreed | R5-242645 | - |
| R5-243719 | Addition of test applicability for Rel-16 RRM EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | agreed | R5-242694 | - |
| R5-243720 | Correction to applicability of 5G test cases | Bureau Veritas ADT, Keysight | revised | R5-242789 | R5-244010 |
| R5-243721 | PC5 FR2 MU - Tx test cases update in 38.521-2 | Keysight Technologies | agreed | R5-242266 | - |
| R5-243722 | PC5 FR2 MU - Annex F update in 38.521-2 | Keysight Technologies | agreed | R5-242268 | - |
| R5-243723 | PC5 FR2 MU definition in 38.903 | Keysight Technologies | agreed | R5-242265 | - |
| R5-243724 | Discussion on MU and TT analysis for FR2c | Keysight Technologies UK Ltd | noted | R5-242261 | - |
| R5-243725 | MU discussion on FR2c | Anritsu | noted | R5-242997 | - |
| R5-243726 | FR2c MU - Tx test cases update in 38.521-2 | Keysight Technologies | agreed | R5-242263 | - |
| R5-243727 | Update of MU for n259 | ROHDE & SCHWARZ | agreed | R5-243102 | - |
| R5-243728 | PC5 FR2 MU - Updates in 38.521-3 | Keysight Technologies | agreed | R5-242269 | - |
| R5-243729 | Update of applicability of FR2 CA and MIMO test cases | ROHDE & SCHWARZ | agreed | R5-243107 | - |
| R5-243730 | FR2c MU definition in 38.903 | Keysight Technologies | agreed | R5-242262 | - |
| R5-243731 | FR2 RRM test cases: Known Issue List | Ericsson | noted | R5-242628 | - |
| R5-243732 | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | revised | R5-242915 | R5-244003 |
| R5-243733 | Update to Annex E and F for NR-U RRM tests | Qualcomm France | revised | R5-243372 | R5-243994 |
| R5-243734 | Update to NSA measurement accuracy NR-U tests including TT | Qualcomm France | revised | R5-243374 | R5-243995 |
| R5-243735 | Update to SA measurement accuracy NR-U tests including TT | Qualcomm France | agreed | R5-243375 | - |
| R5-243736 | Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1 | Qualcomm Technologies Int | revised | R5-243322 | R5-243996 |
| R5-243737 | Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2 | Qualcomm Technologies Int | revised | R5-243323 | R5-243997 |
| R5-243738 | Test Tolerance analysis for NR-U RSRQ intra-frequency accuracy test cases 10.5.2.1 and 11.6.2.1 | Qualcomm Technologies Int | agreed | R5-243324 | - |
| R5-243739 | Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2 | Qualcomm Technologies Int | revised | R5-243325 | R5-243998 |
| R5-243740 | Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3 | Qualcomm Technologies Int | revised | R5-243326 | R5-243999 |
| R5-243741 | Test Tolerance analysis for NR-U SINR inter-frequency accuracy test cases 10.5.3.2 and 11.6.3.2 | Qualcomm Technologies Int | agreed | R5-243327 | - |
| R5-243742 | Test Tolerance analysis for NR-U L1-RSRP accuracy test case 10.5.4.1 | Qualcomm Technologies Int | agreed | R5-243328 | - |
| R5-243743 | Addition of TTs for NR-U L1-RSRP measurement reporting tests | Qualcomm France | revised | R5-243402 | R5-244000 |
| R5-243744 | Correction to 7.6.15.3 FR2 SA event triggered reporting tests with concurrent gaps including TT | Anritsu | agreed | R5-242580 | - |
| R5-243745 | Introduction of Test Tolerance analysis for 7.6.15.3 FR2 SA event triggered reporting tests for concurrent gaps | Anritsu | agreed | R5-242581 | - |
| R5-243746 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | revised | R5-242905 | R5-244012 |
| R5-243747 | TT analysis for RedCap RRM test case 16.2.1.1 | Huawei, HiSilicon | revised | R5-242906 | R5-244016 |
| R5-243748 | Correction of NR_U SA L1-RSRP measurements test cases | Ericsson | agreed | R5-242644 | - |
| R5-243749 | Completion of NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case including test tolerance | Nokia | agreed | R5-242607 | - |
| R5-243750 | Addition to 4.3.1 for FR2 Redcap UE test channel bandwidth | ZTE Corporation | agreed | R5-243293 | - |
| R5-243751 | Completion of NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case including test tolerance | Nokia | agreed | R5-242606 | - |
| R5-243752 | Addition of Test Tolerances analysis for NR-U redirection test cases | Nokia | agreed | R5-242608 | - |
| R5-243753 | Addition of TTs for PSCell addition and release test | Qualcomm France | agreed | R5-243404 | - |
| R5-243754 | Correction of interruptions at NR SRS carrier based switching test case 4.5.2.8 including Test Tolerance | Ericsson | agreed | R5-242632 | - |
| R5-243755 | Correction of interruptions at E-UTRA SRS carrier based switching test case 4.5.2.9 including Test Tolerance | Ericsson | agreed | R5-242633 | - |
| R5-243756 | Correction of SA FR1 interruptions at NR SRS carrier based switching test case 6.5.2.2 including Test Tolerance | Ericsson | agreed | R5-242635 | - |
| R5-243757 | Addition of FR2 SA event triggered reporting test without gap under non-DRX test case 7.6.2.10 including Test Tolerance | Ericsson | agreed | R5-242639 | - |
| R5-243758 | Addition of FR2 SA event triggered reporting test without gap under DRX test case 7.6.2.11 including Test Tolerance | Ericsson | agreed | R5-242640 | - |
| R5-243759 | Test Tolerance analysis for event triggered reporting test without gap under non-DRX test case 7.6.2.10 | Ericsson | agreed | R5-242630 | - |
| R5-243760 | Update TC 14.2.8 with TT analysis results | CATT | agreed | R5-242193 | - |
| R5-243761 | Update TC 14.2.9 with TT analysis results | CATT | agreed | R5-242194 | - |
| R5-243762 | Update TC 14.2.10 with TT analysis results | CATT | agreed | R5-242195 | - |
| R5-243763 | Update TC 14.3.7 with TT analysis results | CATT | agreed | R5-242198 | - |
| R5-243764 | Update TC 14.3.8 with TT analysis results | CATT | agreed | R5-242199 | - |
| R5-243765 | Update TC 15.2.8 with TT analysis results | CATT | agreed | R5-242204 | - |
| R5-243766 | Update TC 15.2.9 with TT analysis results | CATT | agreed | R5-242205 | - |
| R5-243767 | Update TC 15.2.10 with TT analysis results | CATT | agreed | R5-242206 | - |
| R5-243768 | Update NR PRS-based measurement requirements in annex based on the updated TT analysis results | CATT | agreed | R5-242217 | - |
| R5-243769 | Update TC 14.2.3 and TC 14.2.10 TT analysis results | CATT | agreed | R5-242218 | - |
| R5-243770 | Update TC 14.2.8 and TC 14.2.9 TT analysis results | CATT | agreed | R5-242220 | - |
| R5-243771 | Update TC 14.3.3 and TC 14.3.8 TT analysis results | CATT | agreed | R5-242221 | - |
| R5-243772 | Update TC 14.3.7 TT analysis results | CATT | agreed | R5-242223 | - |
| R5-243773 | Update TC 15.2.8, 15.2.9 and 15.2.10 TT analysis results | CATT | agreed | R5-242228 | - |
| R5-243774 | Correction to RedCap RRM test case 17.4.1.1 with TT | Huawei, HiSilicon | agreed | R5-242886 | - |
| R5-243775 | Correction to RedCap RRM test case 17.5.5.1.1 with TT | Huawei, HiSilicon | agreed | R5-242894 | - |
| R5-243776 | Correction to RedCap RRM test case 17.7.1.1 with TT | Huawei, HiSilicon | agreed | R5-242896 | - |
| R5-243777 | Correction to RedCap RRM test case 17.7.1.2 with TT | Huawei, HiSilicon | agreed | R5-242897 | - |
| R5-243778 | Correction to RedCap RRM test case 17.7.2.1 with TT | Huawei, HiSilicon | agreed | R5-242898 | - |
| R5-243779 | Correction to RedCap RRM test case 17.7.2.2 with TT | Huawei, HiSilicon | agreed | R5-242899 | - |
| R5-243780 | Correction to RedCap RRM test case 17.7.3.1 with TT | Huawei, HiSilicon | agreed | R5-242900 | - |
| R5-243781 | Correction to RedCap RRM test case 17.7.3.2 with TT | Huawei, HiSilicon | agreed | R5-242901 | - |
| R5-243782 | Correction to RedCap RRM test case 17.7.4.1 with TT | Huawei, HiSilicon | agreed | R5-242902 | - |
| R5-243783 | Addition of RedCap RRM test case 17.7.4.2 with TT | Huawei, HiSilicon | agreed | R5-242903 | - |
| R5-243784 | Update TC 14.2.3 with TT analysis results | CATT | agreed | R5-242191 | - |
| R5-243785 | Update TC 14.2.4 with TT analysis results | CATT | agreed | R5-242192 | - |
| R5-243786 | Update TC 14.3.3 with TT analysis results | CATT | agreed | R5-242196 | - |
| R5-243787 | Update TC 14.3.4 with TT analysis results | CATT | agreed | R5-242197 | - |
| R5-243788 | Update TC 15.2.1 with TT analysis results | CATT | agreed | R5-242200 | - |
| R5-243789 | Update TC 15.2.3 with TT analysis results | CATT | agreed | R5-242202 | - |
| R5-243790 | Update TC 15.2.4 with TT analysis results | CATT | agreed | R5-242203 | - |
| R5-243791 | Update TC 15.3.1 with TT analysis results | CATT | agreed | R5-242207 | - |
| R5-243792 | Update TC 14.2.4 TT analysis results | CATT | agreed | R5-242219 | - |
| R5-243793 | Update TC 14.3.4 TT analysis results | CATT | agreed | R5-242222 | - |
| R5-243794 | Update TC 15.2.3 TT analysis results | CATT | agreed | R5-242226 | - |
| R5-243795 | Update TC 15.2.4 TT analysis results | CATT | agreed | R5-242227 | - |
| R5-243796 | Adding TP analysis for new SUL test case 6.3C.3.6 | Huawei, Hisilicon | agreed | R5-243619 | - |
| R5-243797 | Clarification of Calibration and Ripple Test Procedures | Keysight Technologies UK Ltd, ETS-Lindgren | agreed | R5-243311 | - |
| R5-243798 | Clarification of polynomial interpolation | Keysight Technologies UK Ltd, Rohde & Schwarz | agreed | R5-243314 | - |
| R5-243799 | Updates across TS 38.561 V17.0.0 | Apple Benelux B.V. | agreed | R5-243407 | - |
| R5-243800 | CR to TS 38.551 on Annex A.2.3 | Apple (UK) Limited | agreed | R5-243224 | - |
| R5-243801 | On Testing with Battery | Keysight Technologies UK Ltd | withdrawn | R5-243316 | - |
| R5-243802 | Addition of test applicabilities for Rel-17 PRS-RSRPP and UE Rx-Tx time difference test cases | CATT, Rohde&Schwarz | agreed | R5-242483 | - |
| R5-243803 | Addition of Test Case 14.2.6 | Rohde & Schwarz | agreed | R5-242123 | - |
| R5-243804 | Addition of Test Case: UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA | Nokia | agreed | R5-242613 | - |
| R5-243805 | Addition of UE Rx-Tx time difference measurements in RRC_INACTIVE test case | Nokia | agreed | R5-242614 | - |
| R5-243806 | Addition of Test Applicability UE Rx-Tx time difference measurement accuracy in RRC_INACTIVE state in FR2 SA | Nokia | agreed | R5-242995 | - |
| R5-243807 | Correction of Additional spurious emission test requirement for NS_45 | MediaTek Beijing Inc. | agreed | R5-242282 | - |
| R5-243808 | Correction of narrow band blocking for CA | ROHDE & SCHWARZ, Anritsu | agreed | R5-243112 | - |
| R5-243809 | Correction to LTE V2V and V2X RRM test cases | Huawei, HiSilicon | agreed | R5-242910 | - |
| R5-243810 | Update Annex B for PRD20 E-UTRA CA list v170 | ZTE Corporation | agreed | R5-243283 | - |
| R5-243811 | Addition of capability declaration for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | agreed | R5-242709 | - |
| R5-243812 | Addition of UE capability for CA_n28A-n40A | KDDI Corporation | agreed | R5-242242 | - |
| R5-243813 | Applicability updates for Phase continuity tests | Apple Benelux B.V. | agreed | R5-243414 | - |
| R5-243814 | Annex F updates for NTN RF test on Spectrum emission mask | Apple Benelux B.V. | agreed | R5-243416 | - |
| R5-243815 | Addition of message contents to test case 4.7.9.1 | Apple, Huawei, Hisilicon | agreed | R5-243240 | - |
| R5-243816 | Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL Rel16 | Apple, Keysight | agreed | R5-243259 | - |
| R5-243817 | Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-16 | Apple, Keysight | agreed | R5-243260 | - |
| R5-243818 | Corrections on 6.2 and 6.2A for almost contiguous A-MPR requirements for HPUE | ZTE Corporation | agreed | R5-243295 | - |
| R5-243819 | Updates on 3.1 and 4.3 for Definitions and Specification suffix information | ZTE Corporation | agreed | R5-243301 | - |
| R5-243820 | Addition of reference sensitivity TP analysis for CA_n28A-n40A | KDDI Corporation | agreed | R5-242245 | - |
| R5-243821 | Addition of reference sensitivity for CA_n28A-n40A | KDDI Corporation | agreed | R5-242249 | - |
| R5-243822 | Addition of delta TIBc for CA_n3A-n28A-n41A-n77A | KDDI Corporation | agreed | R5-242248 | - |
| R5-243823 | Discussion on beam correspondence for initial access | Huawei, HiSilicon | noted | R5-243159 | - |
| R5-243824 | Addition of REFSENS for PC3 CA n78A-n79A with 1UL | NTT DOCOMO INC. | agreed | R5-242948 | - |
| R5-243825 | Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC3 | NTT DOCOMO INC. | agreed | R5-242797 | - |
| R5-243826 | Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC2 | NTT DOCOMO INC. | agreed | R5-242943 | - |
| R5-243827 | Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A | NTT DOCOMO INC. | agreed | R5-242917 | - |
| R5-243828 | Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC2 | NTT DOCOMO INC. | agreed | R5-242798 | - |
| R5-243829 | Documentation of MU for n259 | ROHDE & SCHWARZ | agreed | R5-243101 | - |
| R5-243830 | Update of MU and TT for CA and UL MIMO | ROHDE & SCHWARZ | agreed | R5-243105 | - |
| R5-243831 | Update for FR2c MU | Anritsu | agreed | R5-242999 | - |
| R5-243832 | Update for FR2c MU | Anritsu | agreed | R5-243001 | - |
| R5-243833 | Correction to K1 for RRM TDD SCS 15 kHz | Anritsu | agreed | R5-243007 | - |
| R5-243834 | Correction to LCRB of CA_n5-n48-n66 in 7.3A.0.5 | Anritsu | agreed | R5-243022 | - |
| R5-243835 | Update to R18 IoT NTN test cases applicability | CMCC, Keysight, ROHDE & SCHWARZ, Qualcomm | agreed | R5-242499 | - |
| R5-243836 | Update REFSENSE TP analysis for CA_n39A-n41A | CMCC | agreed | R5-243051 | - |
| R5-243837 | Update REFSENSE exceptions due to UL harmonic interference for CA_n39A-n41A | CMCC | agreed | R5-243048 | - |
| R5-243838 | Update REFSENSE TC 7.3A.1_1 for CA_n39A-n41A | CMCC | agreed | R5-243050 | - |
| R5-243839 | Update for EVM including symbols with transient period | Anritsu, Keysight | agreed | R5-243013 | - |
| R5-243840 | Correction to Inter-band EN-DC spurious emission requirements | Rohde & Schwarz, Oppo | agreed | R5-243092 | - |
| R5-243841 | Introducing framework for Beam Correspondence during Initial Access in IDLE related Tx Power tests | Apple AB Denmark | agreed | R5-242791 | - |
| R5-243842 | Addition of message contents to test case 4.7.9.2 | Apple, Huawei, Hisilicon | agreed | R5-243241 | - |
| R5-243843 | Addition of message contents to test case 4.7.10.1 | Apple, Huawei, Hisilicon | agreed | R5-243242 | - |
| R5-243844 | Addition of message contents to test case 4.7.10.2 | Apple, Huawei, Hisilicon | agreed | R5-243243 | - |
| R5-243845 | Addition of message contents to test case 5.7.8.1 | Apple, Huawei, Hisilicon | agreed | R5-243244 | - |
| R5-243846 | Addition of message contents to test case 5.7.8.2 | Apple, Huawei, Hisilicon | agreed | R5-243245 | - |
| R5-243847 | Addition of message contents to test case 5.7.9.1 | Apple, Huawei, Hisilicon | agreed | R5-243246 | - |
| R5-243848 | Addition of message contents to test case 5.7.9.2 | Apple, Huawei, Hisilicon | agreed | R5-243247 | - |
| R5-243849 | Addition of message contents to test case 6.7.11.1 | Apple, Huawei, Hisilicon | agreed | R5-243248 | - |
| R5-243850 | Addition of message contents to test case 6.7.11.2 | Apple, Huawei, Hisilicon | agreed | R5-243249 | - |
| R5-243851 | Addition of message contents to test case 6.7.12.1 | Apple, Huawei, Hisilicon | agreed | R5-243250 | - |
| R5-243852 | Addition of message contents to test case 6.7.12.2 | Apple, Huawei, Hisilicon | agreed | R5-243251 | - |
| R5-243853 | Addition of message contents to test case 7.7.8.1 | Apple, Huawei, Hisilicon | agreed | R5-243252 | - |
| R5-243854 | Addition of message contents to test case 7.7.8.2 | Apple, Huawei, Hisilicon | agreed | R5-243253 | - |
| R5-243855 | Addition of message contents to test case 7.7.9.1 | Apple, Huawei, Hisilicon | agreed | R5-243254 | - |
| R5-243856 | Addition of message contents to test case 7.7.9.2 | Apple, Huawei, Hisilicon | agreed | R5-243255 | - |
| R5-243857 | Addition of Test Case 14.2.5 | Rohde & Schwarz | agreed | R5-242959 | - |
| R5-243858 | Addition of Test Case 14.2.7 | Rohde & Schwarz | agreed | R5-242960 | - |
| R5-243859 | Addition of Test Case 14.3.5 | Rohde & Schwarz | agreed | R5-242961 | - |
| R5-243860 | Addition of Test Case 14.3.6 | Rohde & Schwarz | agreed | R5-242962 | - |
| R5-243861 | Addition of Test Case 14.4.1 | Rohde & Schwarz | agreed | R5-242963 | - |
| R5-243862 | Addition of Test Case 14.4.2 | Rohde & Schwarz | agreed | R5-242964 | - |
| R5-243863 | Addition of Test Case 14.5.1 | Rohde & Schwarz | agreed | R5-242965 | - |
| R5-243864 | Addition of Test Case 14.5.2 | Rohde & Schwarz | agreed | R5-242966 | - |
| R5-243865 | Addition of Test Case 15.2.5 | Rohde & Schwarz | agreed | R5-242967 | - |
| R5-243866 | Addition of Test Case 15.2.6 | Rohde & Schwarz | agreed | R5-242968 | - |
| R5-243867 | Addition of Test Case 15.2.7 | Rohde & Schwarz | agreed | R5-242969 | - |
| R5-243868 | Addition of Test Case 15.3.3 | Rohde & Schwarz | agreed | R5-242970 | - |
| R5-243869 | Addition of Test Case 15.3.4 | Rohde & Schwarz | agreed | R5-242971 | - |
| R5-243870 | Addition of Test Case 16.2.5 | Rohde & Schwarz | agreed | R5-242972 | - |
| R5-243871 | Addition of Test Case 16.2.6 | Rohde & Schwarz | agreed | R5-242973 | - |
| R5-243872 | Addition of Test Case 16.3.3 | Rohde & Schwarz | agreed | R5-242974 | - |
| R5-243873 | Addition of Test Case 16.4.1 | Rohde & Schwarz | agreed | R5-242975 | - |
| R5-243874 | Addition of Test Case 16.4.2 | Rohde & Schwarz | agreed | R5-242976 | - |
| R5-243875 | Addition of Test Case 16.5.1 | Rohde & Schwarz | agreed | R5-242977 | - |
| R5-243876 | Addition of Test Case 16.5.2 | Rohde & Schwarz | agreed | R5-242978 | - |
| R5-243877 | Introduction of PC1 transmitter requirements for bands n7, n41 and n78 | Nokia | agreed | R5-242379 | - |
| R5-243878 | Addition of general informationfor 3Tx NR CA | Huawei, HiSilicon | agreed | R5-242927 | - |
| R5-243879 | Addition of suffix information for Carrier Aggregation (CA) with UL MIMO | China Unicom | agreed | R5-243336 | - |
| R5-243880 | Addition test case for Inter-cell SSB based L1-RSRP Reporting | Samsung | agreed | R5-242670 | - |
| R5-243881 | Additional demodulation test case for FR2 high speed train | Samsung | agreed | R5-242673 | - |
| R5-243882 | Clarification of Test frequency limitations for CA in Annex C | Ericsson | agreed | R5-242533 | - |
| R5-243883 | Addition of checklist for intra band CA in Annex B | Ericsson | agreed | R5-242537 | - |
| R5-243884 | Addition of MU and TT for new NR-U test cases | Ericsson | agreed | R5-242491 | - |
| R5-243885 | Addition of test case 6.3F.4.4, Aggregate power tolerance for shared spectrum channel access | Ericsson | agreed | R5-242488 | - |
| R5-243886 | Addition of test case 7.4F, Maximum input level for shared spectrum channel access | Ericsson | agreed | R5-242485 | - |
| R5-243887 | Update to NTN DCQR cases | Mediatek India Technology Pvt. | agreed | R5-242577 | - |
| R5-243888 | Update groups of bands for satellite access | Mediatek India Technology Pvt. | withdrawn | R5-242430 | - |
| R5-243889 | Correction to setup for SIB31 and SIB31-NB | Mediatek India Technology Pvt. | withdrawn | R5-242431 | - |
| R5-243890 | Discussion on minimum test time simulation results for 8.3.1.1.1 | Mediatek India Technology Pvt. | noted | R5-242692 | - |
| R5-243891 | Discussion on the Informative note for High test channel bandwidth | CAICT | noted | R5-242600 | - |
| R5-243892 | Update to NSA event-triggered measurement NR-U tests including TT | Qualcomm France | agreed | R5-243376 | - |
| R5-243893 | Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases 10.4.2.3, 10.4.2.4, 10.4.2.5, 10.4.2.6 | Qualcomm Technologies Int | agreed | R5-243320 | - |
| R5-243894 | Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases with DRX 10.4.2.7, 10.4.2.8, 10.4.2.9, 10.4.2.10 | Qualcomm Technologies Int | agreed | R5-243321 | - |
| R5-243895 | Addition of TTs for NR-U intra-frequency measurement reporting test | Qualcomm France | agreed | R5-243399 | - |
| R5-243896 | Introduction of new NR NTN test case - 6.4.2.1a Error Vector Magnitude including symbols with transient period | Keysight Technologies | agreed | R5-242502 | - |
| R5-243897 | Introduction of new SUL test case 7.7C | Keysight Technologies | agreed | R5-242517 | - |
| R5-243898 | MU and TT defintion for TxD EVM test case 6.4G.2.1 | Keysight Technologies, Rohde&Schwarz | agreed | R5-242522 | - |
| R5-243899 | Update to nrn-ntn pdsch demod test case to update min test time | Qualcomm Inc | agreed | R5-243441 | - |
| R5-243900 | Addition of connection diagram for CSI test cases for inter-cell interference scenarios | Qualcomm Inc | agreed | R5-243438 | - |
| R5-243901 | Update to Rel.17 HST CA measurement reporting tests | Qualcomm France | agreed | R5-243390 | - |
| R5-243902 | Correction to SUL test cases | Qualcomm France | agreed | R5-243392 | - |
| R5-243903 | Correction to re-establishment test cases for RedCap | Qualcomm France | agreed | R5-243386 | - |
| R5-243904 | Correction to gap-based measurement tests for RedCap | Qualcomm France | agreed | R5-243383 | - |
| R5-243905 | Update applicability for NB-NTN Demod TC 8.3.1.1.1 | Qualcomm Technologies Int | agreed | R5-243333 | - |
| R5-243906 | Update to ATG Definitions and Symbols | CMCC | revised | R5-243036 | R5-244009 |
| R5-243907 | Update test requirement for Redcap inter frequency test cases | MediaTek Beijing Inc. | revised | R5-242433 | R5-244013 |
| R5-243908 | Update NB-IoT NTN timing advance test cases | MediaTek Beijing Inc. | revised | R5-242423 | R5-244018 |
| R5-243909 | Correction to FR1 NSA Event Trigger TC 4.6.1.8 | Keysight Technologies | agreed | R5-243340 | - |
| R5-243910 | Correction condition F011 for 38.533 TC16.4.1.1 | SGS Wireless | agreed | R5-242726 | - |
| R5-243911 | Correct Table A.4.5-1 item 60 due to it doesnt match reference | SGS Wireless | agreed | R5-242728 | - |
| R5-243912 | Correction to notes for support of SCS and CBW in CA configurations | Anritsu | agreed | R5-243023 | - |
| R5-243913 | Correction to test parameters in FR1 HST DPS | Anritsu | agreed | R5-243024 | - |
| R5-243914 | Addition of applicability for MCE RRM test cases | Huawei, HiSilicon | agreed | R5-242866 | - |
| R5-243915 | Correction to applicability for RedCap RRM test cases | Huawei, HiSilicon | revised | R5-242877 | R5-244011 |
| R5-243916 | Correction to R15 RACH RRM test cases and RMCs | Huawei, HiSilicon | agreed | R5-242873 | - |
| R5-243917 | Correction to R15 reestablishment RRM test case 6.3.2.1.1 | Huawei, HiSilicon | agreed | R5-242874 | - |
| R5-243918 | Correction to RedCap RRM test case 16.1.2.5 and 16.1.2.6 | Huawei, HiSilicon, Starpoint, Qualcomm | agreed | R5-242879 | - |
| R5-243919 | Correction to RedCap inter-RAT measurement test cases | Huawei, HiSilicon, Qualcomm | agreed | R5-243443 | - |
| R5-243920 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | revised | R5-242128 | R5-244006 |
| R5-243921 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | revised | R5-242129 | R5-244007 |
| R5-243922 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | revised | R5-242130 | R5-244008 |
| R5-243923 | Correction to TC titles in applicability table of RRM event triggered reporting TCs with measurement gaps | Anritsu | agreed | R5-242578 | - |
| R5-243924 | Correction and update to the RMC and minimum test time of NB-IoT NTN demod cases | Mediatek India Technology Pvt. | agreed | R5-242422 | - |
| R5-243925 | Deletion of the Editors note in 8.3.1.1.1 | Mediatek India Technology Pvt., CMCC, Rohde&Schwarz | withdrawn | R5-242586 | - |
| R5-243926 | Update to nb-ntn demod test case to update min test time | Qualcomm Inc | withdrawn | R5-243430 | - |
| R5-243950 | Applicability updates to NR NTN test cases | Qualcomm Incorporated | agreed | R5-242766 | - |
| R5-243951 | Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 Enpn | China Telecom | agreed | R5-242680 | - |
| R5-243952 | Common configuration for SN initiated inter-SN CPC EN-DC for 38.508-1 | Nokia | agreed | R5-243174 | - |
| R5-243953 | Addition of capability clauses for inter-SN CPC testcases for EN-DC and NR-DC | Nokia | agreed | R5-243177 | - |
| R5-243954 | Addition of new test case 8.2.3.18.6 inter-SN CPC for EN-DC | Nokia | agreed | R5-243172 | - |
| R5-243955 | Addition of new test case 8.2.3.18.7 inter-SN CPC for NR-DC | Nokia | agreed | R5-243175 | - |
| R5-243956 | Addition of applicability clauses for inter-SN CPC testcases for EN-DC and NR-DC | Nokia | agreed | R5-243176 | - |
| R5-243957 | Add new PICS for MUSIM gap | Huawei, HiSilicon | withdrawn | R5-243086 | - |
| R5-243958 | Correction of MUSIM gap TC 8.1.2.1.6 | Huawei, HiSilicon | agreed | R5-242828 | - |
| R5-243959 | Update test applicability for MUSIM TC | Huawei, HiSilicon | agreed | R5-242830 | - |
| R5-243960 | Applicability updates to ING_5GS test cases | Qualcomm Incorporated, China Telecom | agreed | R5-242736 | - |
| R5-243961 | Applicability for new test cases 12.23a and 12.24a | Ericsson | revised | R5-243218 | R5-243986 |
| R5-243962 | Scoping NR SA applicable TCs for SNPN-only Ues | China Telecom, MediaTek | agreed | R5-242684 | - |
| R5-243963 | Addition of applicability for eNPN test cases | China Telecom | agreed | R5-242704 | - |
| R5-243964 | Addition to R17 ATSSS testcase 10.5.2.1 | MediaTek Inc. | agreed | R5-242271 | - |
| R5-243965 | Addition to R17 ATSSS testcase 10.5.2.2 | MediaTek Inc. | agreed | R5-242272 | - |
| R5-243966 | Corrections to BWP test case 7.1.1.8.1 | Qualcomm Incorporated | revised | R5-242562 | R5-244020 |
| R5-243967 | Routine maintenance for TS 38.523-3 | MCC TF160 | agreed | R5-242331 | - |
| R5-243968 | New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource | Nokia | agreed | R5-243165 | - |
| R5-243969 | New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session | Nokia | agreed | R5-243166 | - |
| R5-243970 | Progress and Way Forward on SW Implementation on Machine Readable PICS and Applicability | Qualcomm Incorporated | noted | R5-243567 | - |
| R5-243972 | LS on Development of NB-IoT test cases for Release 15 and Release 16 | TSG WG RAN5 | approved | - | - |
| R5-243973 | New WID on UE Conformance – Enhanced support of Non-Public Networks Phase 2 | China Telecom, Qualcomm, ZTE | noted | R5-243459 | - |
| R5-243974 | WP UE Conformance – NR support for dedicated spectrum less than 5MHz for FR1 | Nokia | noted | R5-242615 | - |
| R5-243975 | Revised WID on UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects | Nokia | revised | R5-243178 | R5-243992 |
| R5-243976 | Revised WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects | China Unicom | noted | R5-243275 | - |
| R5-243977 | Revised WID for UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | China Telecom ,ZTE, MTK | agreed | - | - |
| R5-243978 | Revised WID - PC1.5 n34 n39 n40 | CMCC | noted | - | - |
| R5-243979 | SR of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258 | Huawei, Hisilicon | noted | R5-243192 | - |
| R5-243980 | SR on UE Conformance - High power UE (power class 2) for NR FR1 FDD single band | China Unicom | noted | R5-243308 | - |
| R5-243981 | SR of Rel-17 Power Class 2 UE for NR inter-band CA/DC with or without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL | Huawei, Hisilicon | noted | R5-243194 | - |
| R5-243982 | SR on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects | China Unicom | noted | R5-243277 | - |
| R5-243983 | SR on UE Conformance - Additional NR bands for UL-MIMO in Rel-18 | China Unicom | noted | R5-243306 | - |
| R5-243984 | New MO speech EVS test case 12.23a | Ericsson | agreed | R5-243523 | - |
| R5-243985 | New MT speech EVS test case 12.24a | Ericsson | agreed | R5-243524 | - |
| R5-243986 | Applicability for new test cases 12.23a and 12.24a | Ericsson | agreed | R5-243961 | - |
| R5-243987 | Common clause updates related to NTN band n254 | Apple Benelux B.V., Keysight | agreed | R5-243682 | - |
| R5-243988 | PRD21 CDS: PC2 ENDC 3 bands combos with n78 and n79 | NTT DOCOMO INC. | available | R5-242794 | - |
| R5-243989 | PRD21 CDS: PC3 ENDC 3 bands combos with n78 and n79 | NTT DOCOMO INC. | available | R5-242795 | - |
| R5-243990 | PRD21 CDS: PC3 NR CA_n78A-n79A with 1UL | NTT DOCOMO INC. | available | R5-242996 | - |
| R5-243991 | PRD21 CDS: PC2 NR CA_n78A-n79A with 1UL | NTT DOCOMO INC. | noted | R5-243026 | - |
| R5-243992 | Revised WID on UE Conformance - XR (eXtended Reality) enhancements for NR plus CT1 aspects | Nokia | noted | R5-243975 | - |
| R5-243993 | SR of UE Conformance - IMS Data Channel test after RAN5 103 | Huawei | reserved | R5-242747 | - |
| R5-243994 | Update to Annex E and F for NR-U RRM tests | Qualcomm France | agreed | R5-243733 | - |
| R5-243995 | Update to NSA measurement accuracy NR-U tests including TT | Qualcomm France | agreed | R5-243734 | - |
| R5-243996 | Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1 | Qualcomm Technologies Int | agreed | R5-243736 | - |
| R5-243997 | Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2 | Qualcomm Technologies Int | agreed | R5-243737 | - |
| R5-243998 | Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2 | Qualcomm Technologies Int | agreed | R5-243739 | - |
| R5-243999 | Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3 | Qualcomm Technologies Int | agreed | R5-243740 | - |
| R5-244000 | Addition of TTs for NR-U L1-RSRP measurement reporting tests | Qualcomm France | revised | R5-243743 | R5-244021 |
| R5-244001 | Correction to SDT RRM test case 6.2.1 with TT | Huawei, HiSilicon | agreed | R5-242863 | - |
| R5-244002 | Addition of MU and test tolerance in Annex F for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | agreed | R5-242699 | - |
| R5-244003 | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | agreed | R5-243732 | - |
| R5-244004 | Addition of test tolerance analysis for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | agreed | R5-242700 | - |
| R5-244005 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | agreed | R5-242127 | - |
| R5-244006 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | agreed | R5-243920 | - |
| R5-244007 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | agreed | R5-243921 | - |
| R5-244008 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | agreed | R5-243922 | - |
| R5-244009 | Update to ATG Definitions and Symbols | CMCC | agreed | R5-243906 | - |
| R5-244010 | Correction to applicability of 5G test cases | Bureau Veritas ADT, Keysight | agreed | R5-243720 | - |
| R5-244011 | Correction to applicability for RedCap RRM test cases | Huawei, HiSilicon | agreed | R5-243915 | - |
| R5-244012 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | agreed | R5-243746 | - |
| R5-244013 | Update test requirement for Redcap inter frequency test cases | MediaTek Beijing Inc. | agreed | R5-243907 | - |
| R5-244014 | Correction to RedCap RRM test case 16.2.1.1 with TT | Huawei, HiSilicon | agreed | R5-242880 | - |
| R5-244015 | Correction to RedCap RRM test case 16.2.1.2 with TT | Huawei, HiSilicon | agreed | R5-242881 | - |
| R5-244016 | TT analysis for RedCap RRM test case 16.2.1.1 | Huawei, HiSilicon | agreed | R5-243747 | - |
| R5-244017 | TT analysis for RedCap RRM test case 16.2.1.2 | Huawei, HiSilicon | agreed | R5-242907 | - |
| R5-244018 | Update NB-IoT NTN timing advance test cases | MediaTek Beijing Inc. | agreed | R5-243908 | - |
| R5-244019 | Update NB-IoT NTN test case TC 13.4.1.1 | MediaTek Beijing Inc. | agreed | R5-242425 | - |
| R5-244020 | Corrections to BWP test case 7.1.1.8.1 | Qualcomm Incorporated | agreed | R5-243966 | - |
| R5-244021 | Addition of TTs for NR-U L1-RSRP measurement reporting tests | Qualcomm France | agreed | R5-244000 | - |

## Annex B: List of change requests

1613 CRs and their final revisions at RAN5#103 (+543 intermediate revisions are not shown)

| Document | Title | Source | Spec | CR | Rev | Rel | Cat | WI | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R5-242188 | Removal of GEA2 test case from the ATS | MediaTek Inc. | 34.123-3 | 3678 | - | Rel-17 | F | TEI_Test | agreed |
| R5-243475 | Update of cl 8.5.4.7 Inter RAT handover to UTRAN | MCC TF160, Rohde & Schwarz | 34.123-3 | 3680 | - | Rel-17 | F | TEI9_Test | agreed |
| R5-242257 | Initial Registration / IMS data channel | China Telecom | 34.229-1 | 1531 | - | Rel-17 | F | IMSProtoc17_dataCH-UEContest | withdrawn |
| R5-242291 | Removal of technical content in TS 34.229-1 v15.10.0 and substitution with pointer to the next Release | ETSI Secretariat | 34.229-1 | 1532 | - | Rel-15 | F | TEI15_Test | agreed |
| R5-242292 | Removal of technical content in TS 34.229-1 v16.7.0 and substitution with pointer to the next Release | ETSI Secretariat | 34.229-1 | 1533 | - | Rel-16 | F | TEI16_Test | agreed |
| R5-242623 | New MO speech EVS test case 12.23a | Ericsson | 34.229-1 | 1534 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243523 | New MO speech EVS test case 12.23a | Ericsson | 34.229-1 | 1534 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243984 | New MO speech EVS test case 12.23a | Ericsson | 34.229-1 | 1534 | 2 | Rel-17 | F | TEI15_Test | agreed |
| R5-242624 | New MT speech EVS test case 12.24a | Ericsson | 34.229-1 | 1535 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243524 | New MT speech EVS test case 12.24a | Ericsson | 34.229-1 | 1535 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243985 | New MT speech EVS test case 12.24a | Ericsson | 34.229-1 | 1535 | 2 | Rel-17 | F | TEI15_Test | agreed |
| R5-242625 | Applicability of new EVS test cases 12.23a and 12.24a | Ericsson | 34.229-1 | 1536 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-242737 | Updates to Annex A.5.3 | Starpoint, TDIA | 34.229-1 | 1537 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242293 | Removal of technical content in TS 34.229-2 v16.10.0 and substitution with pointer to the next Release | ETSI Secretariat | 34.229-2 | 0340 | - | Rel-16 | F | TEI16_Test | agreed |
| R5-243218 | Applicability for new test cases 12.23a and 12.24a | Ericsson | 34.229-2 | 0341 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243961 | Applicability for new test cases 12.23a and 12.24a | Ericsson | 34.229-2 | 0341 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243986 | Applicability for new test cases 12.23a and 12.24a | Ericsson | 34.229-2 | 0341 | 2 | Rel-17 | F | TEI15_Test | agreed |
| R5-242294 | Removal of technical content in TS 34.229-3 v16.8.0 and substitution with pointer to the next Release | ETSI Secretariat | 34.229-3 | 0947 | - | Rel-16 | F | TEI16_Test | agreed |
| R5-242295 | Removal of technical content in TS 34.229-5 v16.10.0 and substitution with pointer to the next Release | ETSI Secretariat | 34.229-5 | 0579 | - | Rel-16 | F | TEI16_Test | agreed |
| R5-242336 | Correction of IMSo5G test case 7.23 | MCC TF160 | 34.229-5 | 0580 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243525 | Correction of IMSo5G test case 7.23 | MCC TF160 | 34.229-5 | 0580 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242618 | Updates to Annex A.15.2 | Ericsson | 34.229-5 | 0581 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243526 | Updates to Annex A.15.2 | Ericsson | 34.229-5 | 0581 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242619 | Updates to Annex A.15.2a | Ericsson | 34.229-5 | 0582 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242620 | Updates to Annex A.16.2a | Ericsson | 34.229-5 | 0583 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-242621 | Updates to test case 7.4a | Ericsson, Rohde & Schwarz | 34.229-5 | 0584 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243527 | Updates to test case 7.4a | Ericsson, Rohde & Schwarz | 34.229-5 | 0584 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242622 | Updates to test case 7.6a | Ericsson, Rohde & Schwarz | 34.229-5 | 0585 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243528 | Updates to test case 7.6a | Ericsson, Rohde & Schwarz | 34.229-5 | 0585 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242742 | Correction to NR IMS TC 7.24b - Forking | Huawei, Hisilicon | 34.229-5 | 0586 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243529 | Correction to NR IMS TC 7.24b - Forking | Huawei, Hisilicon | 34.229-5 | 0586 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-242749 | Correction to generic procedure A.16.1a | Qualcomm Incorporated | 34.229-5 | 0587 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242750 | Correction to VONR test case 7.10 | Qualcomm Incorporated | 34.229-5 | 0588 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242751 | Correction to VONR test case 7.13 | Qualcomm Incorporated | 34.229-5 | 0589 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242752 | Correction to test case 7.16 | Qualcomm Incorporated | 34.229-5 | 0590 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242753 | Correction to VONR test case 7.17 | Qualcomm Incorporated | 34.229-5 | 0591 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242754 | Correction to VONR test case 7.25 | Qualcomm Incorporated | 34.229-5 | 0592 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242755 | Correction to test case 8.37 | Qualcomm Incorporated | 34.229-5 | 0593 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242756 | Correction to test case 8.38 | Qualcomm Incorporated | 34.229-5 | 0594 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243094 | Correction to NR IMS testcases 7.4a and 7.6a | ROHDE & SCHWARZ | 34.229-5 | 0595 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-243219 | Add new NR IMS data channel testcase - Bootstrap Data Channel | Huawei, Hisilicon | 34.229-5 | 0596 | - | Rel-17 | F | IMSProtoc17_dataCH-UEContest | withdrawn |
| R5-242178 | Update the note in the satellite ephemeris information | MediaTek Inc. | 36.508 | 1469 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-242179 | Update the SIB31 for GSO and NGSO branches | MediaTek Inc. | 36.508 | 1470 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-242180 | Correction to generic procedure 4.5.4 and 4.5.4A | MediaTek Inc. | 36.508 | 1471 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-242181 | Correction to generic test procedure 4.5A.5 | MediaTek Inc. | 36.508 | 1472 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-242182 | Correction to IE PhysicalConfigDedicated | MediaTek Inc. | 36.508 | 1473 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-242384 | Addition of test frequencies for LTE Band 106 | Nokia | 36.508 | 1474 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242385 | Addition of signalling test frequencies for LTE Band 106 and 3 MHz channel bandwidth | Nokia | 36.508 | 1475 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242738 | Correction to UUAA-SM generic test procedure 4.5A.31 | Qualcomm Incorporated | 36.508 | 1476 | - | Rel-18 | F | ID_UAS-UEConTest | agreed |
| R5-243113 | Correction of sr-PUCCH-ResourceIndex | ROHDE & SCHWARZ | 36.508 | 1477 | - | Rel-18 | F | TEI8_Test | withdrawn |
| R5-243173 | Common configuration for SN initiated inter-SN CPC EN-DC for 36.508 | Nokia | 36.508 | 1478 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | withdrawn |
| R5-242282 | Correction of Additional spurious emission test requirement for NS_45 | MediaTek Beijing Inc. | 36.521-1 | 5502 | - | Rel-18 | F | TEI15_Test, LTE_TDD_2400_US-UEConTest | revised |
| R5-243807 | Correction of Additional spurious emission test requirement for NS_45 | MediaTek Beijing Inc. | 36.521-1 | 5502 | 1 | Rel-18 | F | TEI15_Test, LTE_TDD_2400_US-UEConTest | agreed |
| R5-242283 | Editorial Correction of test configuration table of Reference sensitivity level for 4DL CA | MediaTek Beijing Inc. | 36.521-1 | 5503 | - | Rel-18 | F | TEI13_Test, LTE_CA_Rel13-UEConTest | agreed |
| R5-242386 | Addition of band 106 into frequency bands and channel arrangements | Nokia | 36.521-1 | 5504 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242387 | Addition of band 106 transmitter requirements | Nokia | 36.521-1 | 5505 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242388 | Addition of band 106 receiver requirements | Nokia | 36.521-1 | 5506 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242421 | Editorial correction to the RMC of 8.12.1.1.2 | Mediatek India Technology Pvt. | 36.521-1 | 5507 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | agreed |
| R5-242663 | Update Test Configuration for NS17 Band 28 3MHz | Rakuten Mobile, Inc | 36.521-1 | 5508 | - | Rel-18 | F | TEI15_Test | agreed |
| R5-242701 | Addition of Inter-band DL CA and CA_30A-48A band for A-MPR & Spurious emission test cases | Sporton, Bureau Veritas ADT | 36.521-1 | 5509 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-242702 | Addition of test point analysis for Inter-band DL CA and UL CA_30A-48A for A-MPR & Spurious emission | Sporton | 36.521-1 | 5510 | - | Rel-18 | F | LTE_CA_R17-UEConTest | withdrawn |
| R5-242732 | Addition of MOP requirements for 2UL CA 3A-42A | NTT DOCOMO INC. | 36.521-1 | 5511 | - | Rel-18 | F | TEI14_Test | agreed |
| R5-242782 | Update of Receiver test cases for multiple CA combos | Bureau Veritas ADT, Sporton International | 36.521-1 | 5512 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-243004 | Correction to PCC DL allocation of CA_XA-YA-ZA in 7.6.3A.5 | Anritsu | 36.521-1 | 5513 | - | Rel-18 | F | TEI12_Test, LTE_CA_Rel12_3DL-UEConTest | withdrawn |
| R5-243005 | Correction to test requirement of CA_2A-2A-66A-66A in 7.3A.9 | Anritsu | 36.521-1 | 5514 | - | Rel-18 | F | TEI14_Test, LTE_CA_R14-UEConTest | agreed |
| R5-243006 | Correction to test requirement of CA_2A-2A-12A-66A-66A in 7.3A.10 | Anritsu | 36.521-1 | 5515 | - | Rel-18 | F | TEI15_Test, LTE_CA_R15-UEConTest | agreed |
| R5-243112 | Correction of narrow band blocking for CA | ROHDE & SCHWARZ, Anritsu | 36.521-1 | 5516 | - | Rel-18 | F | TEI10_Test, LTE_CA_Rel12_3DL-UEConTest | revised |
| R5-243808 | Correction of narrow band blocking for CA | ROHDE & SCHWARZ, Anritsu | 36.521-1 | 5516 | 1 | Rel-18 | F | TEI10_Test, LTE_CA_Rel12_3DL-UEConTest | agreed |
| R5-242389 | Addition of test case applicability statements for band 106 | Nokia | 36.521-2 | 1028 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242499 | Update to R18 IoT NTN test cases applicability | CMCC, Keysight, ROHDE & SCHWARZ, Qualcomm | 36.521-2 | 1029 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243835 | Update to R18 IoT NTN test cases applicability | CMCC, Keysight, ROHDE & SCHWARZ, Qualcomm | 36.521-2 | 1029 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242728 | Correct Table A.4.5-1 item 60 due to it doesnt match reference | SGS Wireless | 36.521-2 | 1030 | - | Rel-18 | F | TEI14_Test | revised |
| R5-243911 | Correct Table A.4.5-1 item 60 due to it doesnt match reference | SGS Wireless | 36.521-2 | 1030 | 1 | Rel-18 | F | TEI14_Test | agreed |
| R5-242783 | Removal of definition and abbreviation for IXIT and PIXIT | Bureau Veritas ADT | 36.521-2 | 1031 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-243061 | Update to R18 IoT NTN enh RRM TCs applicability | CMCC | 36.521-2 | 1032 | - | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | withdrawn |
| R5-243283 | Update Annex B for PRD20 E-UTRA CA list v170 | ZTE Corporation | 36.521-2 | 1033 | - | Rel-18 | F | LTE_CA_R17-UEConTest | revised |
| R5-243810 | Update Annex B for PRD20 E-UTRA CA list v170 | ZTE Corporation | 36.521-2 | 1033 | 1 | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-243330 | Update Table A.4.5-1 with additonal NB-NTN capabilities | Qualcomm Technologies Int | 36.521-2 | 1034 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-243331 | Update applicability for NB-NTN Demod TC 8.3.1.1.1 | Qualcomm Technologies Int | 36.521-2 | 1035 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-243332 | Update applicability for NB-NTN RRM test cases | Qualcomm Technologies Int | 36.521-2 | 1036 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-242120 | Editorial correction to TC 6.2.16 | Mediatek India Technology Pvt. | 36.521-3 | 3022 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | agreed |
| R5-242296 | Removal of technical content in TS 36.521-3 v17.1.0 and substitution with pointer to the next Release | ETSI Secretariat | 36.521-3 | 3023 | - | Rel-17 | F | TEI17_Test | agreed |
| R5-242390 | Addition of band groups for band 106 | Nokia | 36.521-3 | 3024 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242416 | Editorial correction to the wrong citation of rel-15 RRM cases | Mediatek India Technology Pvt. | 36.521-3 | 3025 | - | Rel-18 | F | TEI15_Test, NB_IOTenh2-UEConTest | agreed |
| R5-242417 | Editorial correction to the wrong message content of NTN RA cases | Mediatek India Technology Pvt. | 36.521-3 | 3026 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242419 | Addition of Test configuration for satellite access | Mediatek India Technology Pvt., CMCC | 36.521-3 | 3027 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242420 | Addition of Antenna configuration for satellite access | Mediatek India Technology Pvt., CMCC | 36.521-3 | 3028 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242423 | Update NB-IoT NTN timing advance test cases | MediaTek Beijing Inc. | 36.521-3 | 3029 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243908 | Update NB-IoT NTN timing advance test cases | MediaTek Beijing Inc. | 36.521-3 | 3029 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-244018 | Update NB-IoT NTN timing advance test cases | MediaTek Beijing Inc. | 36.521-3 | 3029 | 2 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242424 | Update TT for NB-IoT NTN timing accuracy test cases | MediaTek Beijing Inc. | 36.521-3 | 3030 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242425 | Update NB-IoT NTN test case TC 13.4.1.1 | MediaTek Beijing Inc. | 36.521-3 | 3031 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-244019 | Update NB-IoT NTN test case TC 13.4.1.1 | MediaTek Beijing Inc. | 36.521-3 | 3031 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242430 | Update groups of bands for satellite access | Mediatek India Technology Pvt. | 36.521-3 | 3032 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243888 | Update groups of bands for satellite access | Mediatek India Technology Pvt. | 36.521-3 | 3032 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-242431 | Correction to setup for SIB31 and SIB31-NB | Mediatek India Technology Pvt. | 36.521-3 | 3033 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243889 | Correction to setup for SIB31 and SIB31-NB | Mediatek India Technology Pvt. | 36.521-3 | 3033 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-242577 | Update to NTN DCQR cases | Mediatek India Technology Pvt. | 36.521-3 | 3034 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243887 | Update to NTN DCQR cases | Mediatek India Technology Pvt. | 36.521-3 | 3034 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242909 | Correction to L2U redirection RRM test case 6.3.12 | Huawei, HiSilicon | 36.521-3 | 3035 | - | Rel-18 | F | TEI10_Test, LTE-UEConTest_RF | agreed |
| R5-242910 | Correction to LTE V2V and V2X RRM test cases | Huawei, HiSilicon | 36.521-3 | 3036 | - | Rel-18 | F | TEI14_Test, LTE_V2X-UEConTest | revised |
| R5-243809 | Correction to LTE V2V and V2X RRM test cases | Huawei, HiSilicon | 36.521-3 | 3036 | 1 | Rel-18 | F | TEI14_Test, LTE_V2X-UEConTest | agreed |
| R5-243053 | Editorial corrections for IoT NTN RRM TCs | CMCC, MediaTek | 36.521-3 | 3037 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-243057 | Addition of new RRM TC 13.5.1 for IoT NTN enh | CMCC | 36.521-3 | 3038 | - | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | revised |
| R5-243688 | Addition of new RRM TC 13.5.1 for IoT NTN enh | CMCC | 36.521-3 | 3038 | 1 | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | agreed |
| R5-243058 | Addition of new RRM TC 13.5.2 for IoT NTN enh | CMCC | 36.521-3 | 3039 | - | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | revised |
| R5-243689 | Addition of new RRM TC 13.5.2 for IoT NTN enh | CMCC | 36.521-3 | 3039 | 1 | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | agreed |
| R5-243059 | Addition of new RRM TC 13.5.3 for IoT NTN enh | CMCC | 36.521-3 | 3040 | - | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | revised |
| R5-243690 | Addition of new RRM TC 13.5.3 for IoT NTN enh | CMCC | 36.521-3 | 3040 | 1 | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | agreed |
| R5-243060 | Addition of cell configuration mapping for IoT NTN RRM TCs | CMCC | 36.521-3 | 3041 | - | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | agreed |
| R5-243334 | Update applicability for NB-NTN Channel quality reporting accuracy testcases | Qualcomm Technologies Int | 36.521-3 | 3042 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243687 | Update applicability for NB-NTN Channel quality reporting accuracy testcases | Qualcomm Technologies Int | 36.521-3 | 3042 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242285 | Editorial correction to test description of Clause 6.5A and 6.5B | MediaTek Beijing Inc. | 36.521-4 | 0018 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242418 | Addition of Editors note for 8.2.1.1.1 | Mediatek India Technology Pvt., Qualcomm Inc. | 36.521-4 | 0019 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242422 | Correction and update to the RMC and minimum test time of NB-IoT NTN demod cases | Mediatek India Technology Pvt. | 36.521-4 | 0020 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243924 | Correction and update to the RMC and minimum test time of NB-IoT NTN demod cases | Mediatek India Technology Pvt. | 36.521-4 | 0020 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242586 | Deletion of the Editors note in 8.3.1.1.1 | Mediatek India Technology Pvt., CMCC, Rohde&Schwarz | 36.521-4 | 0021 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243925 | Deletion of the Editors note in 8.3.1.1.1 | Mediatek India Technology Pvt., CMCC, Rohde&Schwarz | 36.521-4 | 0021 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-242588 | Update to the simulation method of minimun test time | Mediatek India Technology Pvt. | 36.521-4 | 0022 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242589 | Update the minimum test time for 8.3.1.1.1 | Mediatek India Technology Pvt. | 36.521-4 | 0023 | - | Rel-18 | F | LTE_NBIOT_eMTC_NTN_req-UEConTest, TEI18_Test | withdrawn |
| R5-243114 | Correction of MOP for NB1 and NB2 test case | ROHDE & SCHWARZ | 36.521-4 | 0024 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-243115 | Update of frequency error test cases | ROHDE & SCHWARZ | 36.521-4 | 0025 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-243116 | Update of test case 8.3.1.1.1 | ROHDE & SCHWARZ | 36.521-4 | 0026 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-243278 | Editorial corrections to GSO frequency error test cases | Keysight Technologies UK Ltd | 36.521-4 | 0027 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243686 | Editorial corrections to GSO frequency error test cases | Keysight Technologies UK Ltd | 36.521-4 | 0027 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-243333 | Update applicability for NB-NTN Demod TC 8.3.1.1.1 | Qualcomm Technologies Int | 36.521-4 | 0028 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243905 | Update applicability for NB-NTN Demod TC 8.3.1.1.1 | Qualcomm Technologies Int | 36.521-4 | 0028 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-243430 | Update to nb-ntn demod test case to update min test time | Qualcomm Inc | 36.521-4 | 0029 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | revised |
| R5-243926 | Update to nb-ntn demod test case to update min test time | Qualcomm Inc | 36.521-4 | 0029 | 1 | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | withdrawn |
| R5-242121 | Correction of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit | Vodafone GmbH, MCC TF 160 | 36.523-1 | 5299 | - | Rel-18 | F | TEI18_Test | revised |
| R5-243468 | Correction of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit | Vodafone GmbH, MCC TF 160 | 36.523-1 | 5299 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-242183 | Correction to PDCP TC 7.3.3.x | MediaTek Inc. | 36.523-1 | 5300 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-242184 | Correction to ZUC TC 7.3.3.5, 7.3.3.6 and 7.3.4.3 | MediaTek Inc. | 36.523-1 | 5301 | - | Rel-18 | F | TEI11_Test | agreed |
| R5-242185 | Correction to TC 11.2.13 | MediaTek Inc., NTT DOCOMO, MCC TF160, ROHDE & SCHWARZ | 36.523-1 | 5302 | - | Rel-18 | F | TEI11_Test | agreed |
| R5-242332 | Updates to NB-IoT RRC test case 22.4.26 | MCC TF160 | 36.523-1 | 5303 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | agreed |
| R5-242333 | Updates to E-UTRA RRC test case 8.1.2.15 | MCC TF160 | 36.523-1 | 5304 | - | Rel-18 | F | TEI8_Test | agreed |
| R5-242392 | Adding less than 5 MHz BW into Bandwidth dependent parameters | Nokia | 36.523-1 | 5305 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242470 | Correction to NBIOT testcase 22.5.21 | ROHDE & SCHWARZ | 36.523-1 | 5306 | - | Rel-18 | F | TEI14_Test, NB_IOTenh-UEConTest | revised |
| R5-243477 | Correction to NBIOT testcase 22.5.21 | ROHDE & SCHWARZ | 36.523-1 | 5306 | 1 | Rel-18 | F | TEI14_Test, NB_IOTenh-UEConTest | agreed |
| R5-242486 | Correction to NBIOT testcase 22.5.1 | ROHDE & SCHWARZ | 36.523-1 | 5307 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | agreed |
| R5-242524 | Correction to NBIOT testcase 22.3.1.2 | ROHDE & SCHWARZ | 36.523-1 | 5308 | - | Rel-18 | F | TEI13_Test, NB_IOT-UEConTest | agreed |
| R5-242526 | Correction to NTN test cases 22.1.2, 22.2.13, 22.4.30 and 22.5.23 | ROHDE & SCHWARZ | 36.523-1 | 5309 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-243478 | Correction to NTN test cases 22.1.2, 22.2.13, 22.4.30 and 22.5.23 | ROHDE & SCHWARZ | 36.523-1 | 5309 | 1 | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | withdrawn |
| R5-242593 | Correction to LTE-IRAT test cases 11.2.11 | ROHDE & SCHWARZ | 36.523-1 | 5310 | - | Rel-18 | F | TEI9_Test | revised |
| R5-243474 | Correction to LTE-IRAT test cases 11.2.11 | ROHDE & SCHWARZ | 36.523-1 | 5310 | 1 | Rel-18 | F | TEI9_Test | agreed |
| R5-242720 | Correction to NTN TA report test case 22.3.1.13 | Qualcomm Incorporated, Rohde & Schwarz | 36.523-1 | 5311 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-243480 | Correction to NTN TA report test case 22.3.1.13 | Qualcomm Incorporated, Rohde & Schwarz | 36.523-1 | 5311 | 1 | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-242739 | Correction to EPS UAS test cases | Qualcomm Incorporated | 36.523-1 | 5312 | - | Rel-18 | F | ID_UAS-UEConTest | agreed |
| R5-242758 | Correction to LTE RACS Test case 9.2.5.1 | Anritsu EMEA Ltd | 36.523-1 | 5313 | - | Rel-18 | F | TEI16_Test, RACS-UEConTest | revised |
| R5-243470 | Correction to LTE RACS Test case 9.2.5.1 | Anritsu EMEA Ltd | 36.523-1 | 5313 | 1 | Rel-18 | F | TEI16_Test, RACS-UEConTest | agreed |
| R5-242821 | Correction of SENSE eMTC TC 6.1.1.12-PLMN selection | Huawei, HiSilicon,TDIA | 36.523-1 | 5314 | - | Rel-18 | F | TEI18_Test, SENSE-UEContest | agreed |
| R5-242822 | Correction of SENSE eMTC TC 6.1.1.13-PLMN selection Exception | Huawei, HiSilicon,TDIA | 36.523-1 | 5315 | - | Rel-18 | F | TEI18_Test, SENSE-UEContest | agreed |
| R5-242823 | Correction of SENSE eMTC TC 6.1.1.14-Periodic SENSE | Huawei, HiSilicon,TDIA | 36.523-1 | 5316 | - | Rel-18 | F | TEI18_Test, SENSE-UEContest | agreed |
| R5-242824 | Correction of SENSE NB-IoT TC 22.2.14-PLMN selection | Huawei, HiSilicon,TDIA | 36.523-1 | 5317 | - | Rel-18 | F | TEI18_Test, SENSE-UEContest | agreed |
| R5-242825 | Correction of SENSE NB-IoT TC 22.2.15-PLMN selection Exception | Huawei, HiSilicon,TDIA | 36.523-1 | 5318 | - | Rel-18 | F | TEI18_Test, SENSE-UEContest | agreed |
| R5-242826 | Correction of SENSE NB-IoT TC 22.2.16-Periodic SENSE | Huawei, HiSilicon,TDIA | 36.523-1 | 5319 | - | Rel-18 | F | TEI18_Test, SENSE-UEContest | agreed |
| R5-242859 | Editorial correction for Specific message contents in 9.2.1.1.20 | NTT DOCOMO, INC. | 36.523-1 | 5320 | - | Rel-18 | F | TEI8_Test | revised |
| R5-243471 | Editorial correction for Specific message contents in 9.2.1.1.20 | NTT DOCOMO, INC. | 36.523-1 | 5320 | 1 | Rel-18 | F | TEI8_Test | agreed |
| R5-242861 | Editorial correction for Specific message contents in 9.2.3.1.22 | NTT DOCOMO, INC. | 36.523-1 | 5321 | - | Rel-18 | F | TEI8_Test | revised |
| R5-243472 | Editorial correction for Specific message contents in 9.2.3.1.22 | NTT DOCOMO, INC. | 36.523-1 | 5321 | 1 | Rel-18 | F | TEI8_Test | agreed |
| R5-242940 | Addition of EPS P-CSCF restoration test case 10.3.2 | Qualcomm Incorporated, Orange | 36.523-1 | 5322 | - | Rel-18 | F | TEI9_Test | agreed |
| R5-242954 | Addition of new test case related to SSAC per PLMN 13.5.1.c | NTTDOCOMO, INC. | 36.523-1 | 5323 | - | Rel-18 | F | TEI12_Test | revised |
| R5-243476 | Addition of new test case related to SSAC per PLMN 13.5.1.c | NTTDOCOMO, INC. | 36.523-1 | 5323 | 1 | Rel-18 | F | TEI12_Test | agreed |
| R5-243479 | Correction to NTN test cases | ROHDE & SCHWARZ | 36.523-1 | 5324 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-242122 | Addition of applicability of new test case 8.1.3.6b for redir-policy bit | Vodafone GmbH, MCC TF160 | 36.523-2 | 1447 | - | Rel-18 | F | TEI18_Test | revised |
| R5-243469 | Addition of applicability of new test case 8.1.3.6b for redir-policy bit | Vodafone GmbH, MCC TF160 | 36.523-2 | 1447 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-242186 | Correction to case title of TC 11.2.13 | MediaTek Inc., NTT DOCOMO, MCC TF160, ROHDE & SCHWARZ | 36.523-2 | 1448 | - | Rel-18 | F | TEI11_Test | agreed |
| R5-242187 | Optimize the tables for IoT NTN | MediaTek Inc., Anritsu | 36.523-2 | 1449 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | revised |
| R5-243578 | Optimize the tables for IoT NTN | MediaTek Inc., Anritsu | 36.523-2 | 1449 | 1 | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-242393 | Addition of band 106 to RF Baseline Implementation Capabilities | Nokia | 36.523-2 | 1450 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242941 | Applicability updates for EPS P-CSCF restoration test cases | Qualcomm Incorporated, Orange | 36.523-2 | 1451 | - | Rel-18 | F | TEI9_Test | revised |
| R5-243473 | Applicability updates for EPS P-CSCF restoration test cases | Qualcomm Incorporated, Orange | 36.523-2 | 1451 | 1 | Rel-18 | F | TEI9_Test | agreed |
| R5-242955 | Addition of applicability for new test case related to SSAC per PLMN 13.5.1.c | NTTDOCOMO, INC. | 36.523-2 | 1452 | - | Rel-18 | F | TEI12_Test | revised |
| R5-243577 | Addition of applicability for new test case related to SSAC per PLMN 13.5.1.c | NTTDOCOMO, INC. | 36.523-2 | 1452 | 1 | Rel-18 | F | TEI12_Test | agreed |
| R5-243062 | Update to R18 IoT NTN enh SIG TCs applicability | CMCC | 36.523-2 | 1453 | - | Rel-18 | F | IoT_NTN_enh_plus_CT1-UEConTest | withdrawn |
| R5-243182 | Update of applicability for test case 6.1.2.9 | Ericsson | 36.523-2 | 1454 | - | Rel-18 | F | TEI14_Test, LTE_feMTC-UEConTest | agreed |
| R5-242297 | Removal of technical content in TS 36.523-3 v17.7.0 and substitution with pointer to the next Release | ETSI Secretariat | 36.523-3 | 4819 | - | Rel-17 | F | TEI17_Test | agreed |
| R5-242313 | ID_UAS: Test Model aspects | MCC TF160 | 36.523-3 | 4820 | - | Rel-18 | F | ID_UAS-UEConTest | agreed |
| R5-242334 | NTN-IoT: Addition of NGSO NB-IoT NTN Test Model | MCC TF160 | 36.523-3 | 4821 | - | Rel-18 | F | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest | agreed |
| R5-242335 | Routine maintenance for TS 36.523-3 | MCC TF160 | 36.523-3 | 4822 | - | Rel-18 | F | TEI17_Test | agreed |
| R5-242337 | Clarification of initial conditions and RRC/NAS signalling in clause 5.3 | MCC TF160 | 36.579-1 | 0344 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242338 | Clarification of initial conditions and RRC/NAS signalling in clause 5.3A | MCC TF160 | 36.579-1 | 0345 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242339 | Clarification of initial conditions and RRC/NAS signalling in clause 5.3B | MCC TF160 | 36.579-1 | 0346 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242340 | Clarification of initial conditions and RRC/NAS signalling in clause 5.3C | MCC TF160 | 36.579-1 | 0347 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242341 | Clarifying the System Under Test | MCC TF160 | 36.579-1 | 0348 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242342 | Corrections of clause 2 | MCC TF160 | 36.579-1 | 0349 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242343 | Corrections of clause 5.5.2.15.2 | MCC TF160 | 36.579-1 | 0350 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242344 | Corrections of clause 5.5.2.2.2 | MCC TF160 | 36.579-1 | 0351 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242345 | Corrections of clause 5.5.2.4 | MCC TF160 | 36.579-1 | 0352 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242346 | Corrections of clause 5.5.2.5.2 | MCC TF160 | 36.579-1 | 0353 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242347 | Corrections of clause 5.5.2.7.1 | MCC TF160 | 36.579-1 | 0354 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242348 | Editorial corrections of clauses 1 and 4 | MCC TF160 | 36.579-1 | 0355 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242349 | Improvement of clause 5.2 | MCC TF160 | 36.579-1 | 0356 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242350 | Improvement of clause 5.4 | MCC TF160 | 36.579-1 | 0357 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242398 | Correction of URI scheme in HTTP POST | MCC TF160 | 36.579-1 | 0358 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242400 | Corrections of clause 5.5.9.1 | MCC TF160 | 36.579-1 | 0359 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-243273 | Correction to clause 5.5.3.4.2 | NIST | 36.579-1 | 0360 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-242351 | Common correction of test cases in clauses 5 and 6 | MCC TF160 | 36.579-2 | 0367 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242352 | Correction of test case 6.1.1.15 | MCC TF160 | 36.579-2 | 0368 | - | Rel-16 | F | TEI14_Test, MCPTT-ConTest | agreed |
| R5-242353 | Correction of test case 6.1.1.16 | MCC TF160 | 36.579-2 | 0369 | - | Rel-16 | F | TEI14_Test, MCPTT-ConTest | agreed |
| R5-242354 | Correction of test case 6.3.2 | MCC TF160 | 36.579-2 | 0370 | - | Rel-16 | F | TEI14_Test, MCPTT-ConTest | agreed |
| R5-243274 | Correction to MCPTT TC 5.10 | NIST | 36.579-2 | 0371 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-242298 | Removal of technical content in TS 36.579-3 v13.3.0 and substitution with pointer to the next Release | ETSI Secretariat | 36.579-3 | 0010 | - | Rel-13 | F | TEI13_Test | agreed |
| R5-242299 | Removal of technical content in TS 36.579-3 v14.0.0 and substitution with pointer to the next Release | ETSI Secretariat | 36.579-3 | 0011 | - | Rel-14 | F | TEI14_Test | agreed |
| R5-242355 | Misc updates | MCC TF160 | 36.579-4 | 0034 | - | Rel-16 | F | TEI14_Test, MCPTT-ConTest | agreed |
| R5-242300 | Removal of technical content in TS 36.579-5 v15.1.0 and substitution with pointer to the next Release | ETSI Secretariat | 36.579-5 | 0109 | - | Rel-15 | F | TEI15_Test | agreed |
| R5-242301 | Removal of technical content in TS 36.579-5 v16.0.0 and substitution with pointer to the next Release | ETSI Secretariat | 36.579-5 | 0110 | - | Rel-16 | F | TEI16_Test | agreed |
| R5-242399 | Routine maintenance for TS 36.579-5 | MCC TF160 | 36.579-5 | 0111 | - | Rel-17 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242311 | Correction of testcase 6.1.1.14 | MCC TF160 | 36.579-6 | 0106 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-242312 | Correction of testcase 6.9.1 | MCC TF160 | 36.579-6 | 0107 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-242356 | Common correction of test cases in clause 6 | MCC TF160 | 36.579-6 | 0108 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-242357 | Correction of testcase 6.3.1 | MCC TF160 | 36.579-6 | 0109 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-242358 | Correction of testcase 6.3.2 | MCC TF160 | 36.579-6 | 0110 | - | Rel-16 | F | TEI15_Test, MCenhUEConTest | agreed |
| R5-243271 | Correction of MCVideo Location TCs | NIST | 36.579-6 | 0111 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-242302 | Removal of technical content in TS 36.579-7 v15.5.1 and substitution with pointer to the next Release | ETSI Secretariat | 36.579-7 | 0057 | - | Rel-15 | F | TEI15_Test | agreed |
| R5-242359 | Common correction of test cases in clauses 5 and 6 | MCC TF160 | 36.579-7 | 0058 | - | Rel-16 | F | TEI14_Test, MCImp-UEConTest | agreed |
| R5-243267 | Addition of new TC 6.5.1 Location Information Request | NIST | 36.579-7 | 0059 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-243570 | Addition of new TC 6.5.1 Location Information Request | NIST | 36.579-7 | 0059 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-243268 | Addition of new TC 6.5.2 Location Report triggering | NIST | 36.579-7 | 0060 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-243571 | Addition of new TC 6.5.2 Location Report triggering | NIST | 36.579-7 | 0060 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-243269 | Addition of new TC 5.8 Rules based affiliation status change | NIST | 36.579-7 | 0061 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-243572 | Addition of new TC 5.8 Rules based affiliation status change | NIST | 36.579-7 | 0061 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-243270 | Addition of new TC 5.9 Location based functional alias status change | NIST | 36.579-7 | 0062 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | revised |
| R5-243573 | Addition of new TC 5.9 Location based functional alias status change | NIST | 36.579-7 | 0062 | 1 | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-243272 | Correction of MCData TC 6.4.2 Emergency Alert | NIST | 36.579-7 | 0063 | - | Rel-16 | F | MCProtoc16_enh2MCPTT_eMCData2-ConTest | agreed |
| R5-242426 | Update TT analysis for NB-IoT NTN timing accuracy test cases | MediaTek Beijing Inc. | 36.903 | 0462 | - | Rel-18 | F | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest | agreed |
| R5-242712 | Addition of test point analysis for Inter-band DL CA and UL CA_30A-48A for A-MPR & Spurious emission | Sporton | 36.905 | 0273 | - | Rel-18 | F | LTE_CA_R17-UEConTest | agreed |
| R5-243310 | Clarification of Calibration and Ripple Test Procedures | Keysight Technologies UK Ltd, ETS-Lindgren | 37.544 | 0033 | - | Rel-16 | F | TEI14_Test, LTE_MIMO_OTA-UEConTest | agreed |
| R5-243317 | On Testing with Battery | Keysight Technologies UK Ltd | 37.544 | 0034 | - | Rel-16 | F | TEI14_Test, LTE_MIMO_OTA-UEConTest | withdrawn |
| R5-242123 | Addition of Test Case 14.2.6 | Rohde & Schwarz | 37.571-1 | 0472 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243803 | Addition of Test Case 14.2.6 | Rohde & Schwarz | 37.571-1 | 0472 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242191 | Update TC 14.2.3 with TT analysis results | CATT | 37.571-1 | 0473 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243784 | Update TC 14.2.3 with TT analysis results | CATT | 37.571-1 | 0473 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242192 | Update TC 14.2.4 with TT analysis results | CATT | 37.571-1 | 0474 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243785 | Update TC 14.2.4 with TT analysis results | CATT | 37.571-1 | 0474 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242193 | Update TC 14.2.8 with TT analysis results | CATT | 37.571-1 | 0475 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243760 | Update TC 14.2.8 with TT analysis results | CATT | 37.571-1 | 0475 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242194 | Update TC 14.2.9 with TT analysis results | CATT | 37.571-1 | 0476 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243761 | Update TC 14.2.9 with TT analysis results | CATT | 37.571-1 | 0476 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242195 | Update TC 14.2.10 with TT analysis results | CATT | 37.571-1 | 0477 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243762 | Update TC 14.2.10 with TT analysis results | CATT | 37.571-1 | 0477 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242196 | Update TC 14.3.3 with TT analysis results | CATT | 37.571-1 | 0478 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243786 | Update TC 14.3.3 with TT analysis results | CATT | 37.571-1 | 0478 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242197 | Update TC 14.3.4 with TT analysis results | CATT | 37.571-1 | 0479 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243787 | Update TC 14.3.4 with TT analysis results | CATT | 37.571-1 | 0479 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242198 | Update TC 14.3.7 with TT analysis results | CATT | 37.571-1 | 0480 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243763 | Update TC 14.3.7 with TT analysis results | CATT | 37.571-1 | 0480 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242199 | Update TC 14.3.8 with TT analysis results | CATT | 37.571-1 | 0481 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243764 | Update TC 14.3.8 with TT analysis results | CATT | 37.571-1 | 0481 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242200 | Update TC 15.2.1 with TT analysis results | CATT | 37.571-1 | 0482 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243788 | Update TC 15.2.1 with TT analysis results | CATT | 37.571-1 | 0482 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242201 | Update TC 15.2.2 with TT analysis results | CATT | 37.571-1 | 0483 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242202 | Update TC 15.2.3 with TT analysis results | CATT | 37.571-1 | 0484 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243789 | Update TC 15.2.3 with TT analysis results | CATT | 37.571-1 | 0484 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242203 | Update TC 15.2.4 with TT analysis results | CATT | 37.571-1 | 0485 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243790 | Update TC 15.2.4 with TT analysis results | CATT | 37.571-1 | 0485 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242204 | Update TC 15.2.8 with TT analysis results | CATT | 37.571-1 | 0486 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243765 | Update TC 15.2.8 with TT analysis results | CATT | 37.571-1 | 0486 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242205 | Update TC 15.2.9 with TT analysis results | CATT | 37.571-1 | 0487 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243766 | Update TC 15.2.9 with TT analysis results | CATT | 37.571-1 | 0487 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242206 | Update TC 15.2.10 with TT analysis results | CATT | 37.571-1 | 0488 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243767 | Update TC 15.2.10 with TT analysis results | CATT | 37.571-1 | 0488 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242207 | Update TC 15.3.1 with TT analysis results | CATT | 37.571-1 | 0489 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243791 | Update TC 15.3.1 with TT analysis results | CATT | 37.571-1 | 0489 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242208 | Update TC 15.3.2 with TT analysis results | CATT | 37.571-1 | 0490 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242209 | Update TC 15.3.5 with TT analysis results | CATT | 37.571-1 | 0491 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242210 | Update TC 15.3.6 with TT analysis results | CATT | 37.571-1 | 0492 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242211 | Update TC 16.2.3 with TT analysis results | CATT | 37.571-1 | 0493 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242212 | Update TC 16.2.4 with TT analysis results | CATT | 37.571-1 | 0494 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242213 | Update TC 16.2.7 with TT analysis results | CATT | 37.571-1 | 0495 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242214 | Update TC 16.2.8 with TT analysis results | CATT | 37.571-1 | 0496 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242215 | Update TC 16.3.2 with TT analysis results | CATT | 37.571-1 | 0497 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242216 | Update TC 16.3.4 with TT analysis results | CATT | 37.571-1 | 0498 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242217 | Update NR PRS-based measurement requirements in annex based on the updated TT analysis results | CATT | 37.571-1 | 0499 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243768 | Update NR PRS-based measurement requirements in annex based on the updated TT analysis results | CATT | 37.571-1 | 0499 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242471 | Addition of new UE Rx-Tx time difference reporting delay test case 15.4.1 in RRC_INACTIVE state | CATT | 37.571-1 | 0500 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242472 | Addition of new UE Rx-Tx time difference reporting delay test case 15.4.2 in RRC_INACTIVE state | CATT | 37.571-1 | 0501 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242473 | Addition of new UE Rx-Tx time difference reporting delay test case 15.4.3 in RRC_INACTIVE state | CATT | 37.571-1 | 0502 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242474 | Addition of new UE Rx-Tx time difference reporting delay test case 15.4.4 in RRC_INACTIVE state | CATT | 37.571-1 | 0503 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242475 | Addition of new NR PRS-RSRPP measurement period test case 17.4.1 in RRC_INACTIVE | CATT | 37.571-1 | 0504 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242476 | Addition of new NR PRS-RSRPP measurement period test case 17.4.2 in RRC_INACTIVE | CATT | 37.571-1 | 0505 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242477 | Addition of new NR PRS-RSRPP measurement period test case 17.4.3 in RRC_INACTIVE | CATT | 37.571-1 | 0506 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242478 | Addition of new NR PRS-RSRPP measurement period test case 17.4.4 in RRC_INACTIVE | CATT | 37.571-1 | 0507 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242479 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.1 in RRC_INACTIVE | CATT | 37.571-1 | 0508 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242480 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.2 in RRC_INACTIVE | CATT | 37.571-1 | 0509 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242481 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.3 in RRC_INACTIVE | CATT | 37.571-1 | 0510 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242482 | Addition of new NR PRS-RSRPP measurement accuracy test case 17.5.4 in RRC_INACTIVE | CATT | 37.571-1 | 0511 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242613 | Addition of Test Case: UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA | Nokia | 37.571-1 | 0512 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243804 | Addition of Test Case: UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA | Nokia | 37.571-1 | 0512 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242614 | Addition of UE Rx-Tx time difference measurements in RRC_INACTIVE test case | Nokia | 37.571-1 | 0513 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243805 | Addition of UE Rx-Tx time difference measurements in RRC_INACTIVE test case | Nokia | 37.571-1 | 0513 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242959 | Addition of Test Case 14.2.5 | Rohde & Schwarz | 37.571-1 | 0514 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243857 | Addition of Test Case 14.2.5 | Rohde & Schwarz | 37.571-1 | 0514 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242960 | Addition of Test Case 14.2.7 | Rohde & Schwarz | 37.571-1 | 0515 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243858 | Addition of Test Case 14.2.7 | Rohde & Schwarz | 37.571-1 | 0515 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242961 | Addition of Test Case 14.3.5 | Rohde & Schwarz | 37.571-1 | 0516 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243859 | Addition of Test Case 14.3.5 | Rohde & Schwarz | 37.571-1 | 0516 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242962 | Addition of Test Case 14.3.6 | Rohde & Schwarz | 37.571-1 | 0517 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243860 | Addition of Test Case 14.3.6 | Rohde & Schwarz | 37.571-1 | 0517 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242963 | Addition of Test Case 14.4.1 | Rohde & Schwarz | 37.571-1 | 0518 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243861 | Addition of Test Case 14.4.1 | Rohde & Schwarz | 37.571-1 | 0518 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242964 | Addition of Test Case 14.4.2 | Rohde & Schwarz | 37.571-1 | 0519 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243862 | Addition of Test Case 14.4.2 | Rohde & Schwarz | 37.571-1 | 0519 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242965 | Addition of Test Case 14.5.1 | Rohde & Schwarz | 37.571-1 | 0520 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243863 | Addition of Test Case 14.5.1 | Rohde & Schwarz | 37.571-1 | 0520 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242966 | Addition of Test Case 14.5.2 | Rohde & Schwarz | 37.571-1 | 0521 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243864 | Addition of Test Case 14.5.2 | Rohde & Schwarz | 37.571-1 | 0521 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242967 | Addition of Test Case 15.2.5 | Rohde & Schwarz | 37.571-1 | 0522 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243865 | Addition of Test Case 15.2.5 | Rohde & Schwarz | 37.571-1 | 0522 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242968 | Addition of Test Case 15.2.6 | Rohde & Schwarz | 37.571-1 | 0523 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243866 | Addition of Test Case 15.2.6 | Rohde & Schwarz | 37.571-1 | 0523 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242969 | Addition of Test Case 15.2.7 | Rohde & Schwarz | 37.571-1 | 0524 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243867 | Addition of Test Case 15.2.7 | Rohde & Schwarz | 37.571-1 | 0524 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242970 | Addition of Test Case 15.3.3 | Rohde & Schwarz | 37.571-1 | 0525 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243868 | Addition of Test Case 15.3.3 | Rohde & Schwarz | 37.571-1 | 0525 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242971 | Addition of Test Case 15.3.4 | Rohde & Schwarz | 37.571-1 | 0526 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243869 | Addition of Test Case 15.3.4 | Rohde & Schwarz | 37.571-1 | 0526 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242972 | Addition of Test Case 16.2.5 | Rohde & Schwarz | 37.571-1 | 0527 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243870 | Addition of Test Case 16.2.5 | Rohde & Schwarz | 37.571-1 | 0527 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242973 | Addition of Test Case 16.2.6 | Rohde & Schwarz | 37.571-1 | 0528 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243871 | Addition of Test Case 16.2.6 | Rohde & Schwarz | 37.571-1 | 0528 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242974 | Addition of Test Case 16.3.3 | Rohde & Schwarz | 37.571-1 | 0529 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243872 | Addition of Test Case 16.3.3 | Rohde & Schwarz | 37.571-1 | 0529 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242975 | Addition of Test Case 16.4.1 | Rohde & Schwarz | 37.571-1 | 0530 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243873 | Addition of Test Case 16.4.1 | Rohde & Schwarz | 37.571-1 | 0530 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242976 | Addition of Test Case 16.4.2 | Rohde & Schwarz | 37.571-1 | 0531 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243874 | Addition of Test Case 16.4.2 | Rohde & Schwarz | 37.571-1 | 0531 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242977 | Addition of Test Case 16.5.1 | Rohde & Schwarz | 37.571-1 | 0532 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243875 | Addition of Test Case 16.5.1 | Rohde & Schwarz | 37.571-1 | 0532 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242978 | Addition of Test Case 16.5.2 | Rohde & Schwarz | 37.571-1 | 0533 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243876 | Addition of Test Case 16.5.2 | Rohde & Schwarz | 37.571-1 | 0533 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242950 | Correction to LPP positioning test case 9.3.4.2 | NTT DOCOMO Inc., MCC TF160 | 37.571-2 | 0183 | - | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243569 | Correction to LPP positioning test case 9.3.4.2 | NTT DOCOMO Inc., MCC TF160 | 37.571-2 | 0183 | 1 | Rel-17 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242303 | Removal of technical content in TS 37.571-3 v16.14.0 and substitution with pointer to the next Release | ETSI Secretariat | 37.571-3 | 0168 | - | Rel-16 | F | TEI16_Test | agreed |
| R5-242483 | Addition of test applicabilities for Rel-17 PRS-RSRPP and UE Rx-Tx time difference test cases | CATT, Rohde&Schwarz | 37.571-3 | 0169 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243802 | Addition of test applicabilities for Rel-17 PRS-RSRPP and UE Rx-Tx time difference test cases | CATT, Rohde&Schwarz | 37.571-3 | 0169 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242995 | Addition of Test Applicability UE Rx-Tx time difference measurement accuracy in RRC_INACTIVE state in FR2 SA | Nokia | 37.571-3 | 0170 | - | Rel-17 | F | NR_pos_enh-UEConTest | revised |
| R5-243806 | Addition of Test Applicability UE Rx-Tx time difference measurement accuracy in RRC_INACTIVE state in FR2 SA | Nokia | 37.571-3 | 0170 | 1 | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242304 | Removal of technical content in TS 37.571-4 v16.8.0 and substitution with pointer to the next Release | ETSI Secretariat | 37.571-4 | 0203 | - | Rel-16 | F | TEI16_Test | agreed |
| R5-242484 | Change the GNSS-AcquisitionAssistance IE description for BDS signals | CATT | 37.571-5 | 0231 | - | Rel-17 | F | NR_pos_enh-UEConTest | agreed |
| R5-242132 | Update of NTN-config for NGSO | MediaTek Inc. | 38.508-1 | 3107 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-242139 | Correction to RRC message MobilityFromNRCommand | MediaTek Inc. | 38.508-1 | 3108 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242140 | Correction to IE PDCP-Config for UDC | MediaTek Inc. | 38.508-1 | 3109 | - | Rel-18 | F | TEI17_Test, NR_UDC-UEConTest | agreed |
| R5-242141 | Correction to IE MeasIdleConfig for Idle/Inactive measurements | MediaTek Inc. | 38.508-1 | 3110 | - | Rel-18 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-242142 | Correction to IE CondReconfigToAddModList | MediaTek Inc. | 38.508-1 | 3111 | - | Rel-18 | F | TEI16_Test, NR_Mob_enh-UEConTest | agreed |
| R5-242240 | Addition of test frequencies for CA_n28A-n40A | KDDI Corporation | 38.508-1 | 3112 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242241 | Addition of test frequencies for CA_n3A-n28A-n41A-n77A | KDDI Corporation | 38.508-1 | 3113 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242256 | Add IE NeedForInterruptionInfoNR | Ericsson | 38.508-1 | 3114 | - | Rel-18 | F | TEI18_Test | withdrawn |
| R5-242321 | Editorial updates to specification references in clauses 4.5, 4.5A & 4.9 | MCC TF160 | 38.508-1 | 3115 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242322 | Editorial updates to specification references in clause 4.6 | MCC TF160 | 38.508-1 | 3116 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242323 | Editorial updates to REGISTRATION REQUEST message | MCC TF160 | 38.508-1 | 3117 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242362 | Introduction of test channel bandwidths and test frequencies for band n85 | Nokia | 38.508-1 | 3118 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | revised |
| R5-243602 | Introduction of test channel bandwidths and test frequencies for band n85 | Nokia | 38.508-1 | 3118 | 1 | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-242363 | Introduction of signalling test frequencies for band n85 | Nokia | 38.508-1 | 3119 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | revised |
| R5-243494 | Introduction of signalling test frequencies for band n85 | Nokia | 38.508-1 | 3119 | 1 | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-242368 | Introduction of test channel bandwidths and test frequencies for band n106 | Nokia | 38.508-1 | 3120 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-242369 | Signalling test frequencies for 3 MHz and band n106 | Nokia | 38.508-1 | 3121 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243566 | Signalling test frequencies for 3 MHz and band n106 | Nokia | 38.508-1 | 3121 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-242394 | Addition of 3 MHz CBW test frequencies for NR Bands n26, n28, n85 and n100 | Nokia | 38.508-1 | 3122 | - | Rel-18 | F | NR_FR1_lessthan_5MHz_BW-UEConTest | revised |
| R5-243601 | Addition of 3 MHz CBW test frequencies for NR Bands n26, n28, n85 and n100 | Nokia | 38.508-1 | 3122 | 1 | Rel-18 | F | NR_FR1_lessthan_5MHz_BW-UEConTest | agreed |
| R5-242401 | Add IE AppLayerIdleInactiveConfig | Ericsson | 38.508-1 | 3123 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242402 | Add IE EUTRA-MultiBandInfoListAerial | Ericsson | 38.508-1 | 3124 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242405 | Add IE MBS-NonServingInfoList | Ericsson | 38.508-1 | 3125 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242406 | Add IE MBS-SessionInfoListMulticast | Ericsson | 38.508-1 | 3126 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242407 | Add IE AerialParameters | Ericsson | 38.508-1 | 3127 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242408 | Add IE DL-PRS-MeasurementWithRxFH-RRC-Connected | Ericsson | 38.508-1 | 3128 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242409 | Add IE ERedCapParameters | Ericsson | 38.508-1 | 3129 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242410 | Add IE NCR-Parameters | Ericsson | 38.508-1 | 3130 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242411 | Add IE PosSRS-BWA-RRC-Inactive | Ericsson | 38.508-1 | 3131 | - | Rel-18 | F | TEI18_Test | withdrawn |
| R5-242412 | Add IEs PosSRS-TxFrequencyHoppingRRC-Connected and PosSRS-TxFrequencyHoppingRRC-Inactive | Ericsson | 38.508-1 | 3132 | - | Rel-18 | F | TEI18_Test | withdrawn |
| R5-242413 | Update IE SharedSpectrumChAccessParamsPerBand | Ericsson | 38.508-1 | 3133 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242414 | Add IE SharedSpectrumChAccessParamsSidelinkPerBand | Ericsson | 38.508-1 | 3134 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242415 | Add IE SupportedAggBandwidth | Ericsson | 38.508-1 | 3135 | - | Rel-18 | F | TEI17_Test | agreed |
| R5-242450 | Update for additional ENDC band configurations | Verizon Spain | 38.508-1 | 3136 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242460 | Update for additional NRCA band configurations | Verizon, Ericsson | 38.508-1 | 3137 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242515 | Test frequency parameters corrections for CA n48B | Keysight Technologies | 38.508-1 | 3138 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242533 | Clarification of Test frequency limitations for CA in Annex C | Ericsson | 38.508-1 | 3139 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243882 | Clarification of Test frequency limitations for CA in Annex C | Ericsson | 38.508-1 | 3139 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242534 | Correction of clause structure of CA test frequencies | Ericsson | 38.508-1 | 3140 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242535 | Addition of signalling test frequencies for CA_n48C | Ericsson, Verizon, MCC TF160 | 38.508-1 | 3141 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243532 | Addition of signalling test frequencies for CA_n48C | Ericsson, Verizon, MCC TF160 | 38.508-1 | 3141 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | withdrawn |
| R5-242543 | Added Common test environment for NR-NTN test cases | Qualcomm Technologies Ireland | 38.508-1 | 3142 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243554 | Added Common test environment for NR-NTN test cases | Qualcomm Technologies Ireland | 38.508-1 | 3142 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242546 | Updates to SIB19 and ephemeris information for NGSO and GSO condition | Qualcomm Technologies Ireland, MediaTek Inc. | 38.508-1 | 3143 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243597 | Updates to SIB19 and ephemeris information for NGSO and GSO condition | Qualcomm Technologies Ireland, MediaTek Inc. | 38.508-1 | 3143 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242565 | Addition of generci procedure for UUAA-MM | Qualcomm Incorporated | 38.508-1 | 3144 | - | Rel-18 | F | ID_UAS-UEConTest | revised |
| R5-243574 | Addition of generci procedure for UUAA-MM | Qualcomm Incorporated | 38.508-1 | 3144 | 1 | Rel-18 | F | ID_UAS-UEConTest | agreed |
| R5-242566 | Correction to Registration request and Registration accept information element for UAS support | Qualcomm Incorporated | 38.508-1 | 3145 | - | Rel-18 | F | ID_UAS-UEConTest | withdrawn |
| R5-242574 | Update for derivation path of test procedure 4.9.39 and 4.9.40 | ZTE Corporation | 38.508-1 | 3146 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242575 | Correction for test procedure 4.9.41 and 4.9.42 | ZTE Corporation | 38.508-1 | 3147 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-243545 | Correction for test procedure 4.9.41 and 4.9.42 | ZTE Corporation | 38.508-1 | 3147 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | withdrawn |
| R5-242576 | Editorial correction for generic procedure 4.5.4, 4.5.5 and 4.5.7 | ZTE Corporation | 38.508-1 | 3148 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243489 | Editorial correction for generic procedure 4.5.4, 4.5.5 and 4.5.7 | ZTE Corporation | 38.508-1 | 3148 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242590 | Addition of test frequencies for new EN-DC comb within FR2 | KDDI Corporation | 38.508-1 | 3149 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242598 | Editorial correction for Editor Note and reference section number in 4.3.1 | CAICT | 38.508-1 | 3150 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242601 | Addition of CBW 70MHz 90MHz and 100MHz for n40 | CAICT | 38.508-1 | 3151 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-242603 | Add IE PosSRS-BWA-RRC-Inactive | Ericsson | 38.508-1 | 3152 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242604 | Add IEs PosSRS-TxFrequencyHoppingRRC-Connected and PosSRS-TxFrequencyHoppingRRC-Inactive | Ericsson | 38.508-1 | 3153 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242648 | Add IE Aerial-Config | Ericsson | 38.508-1 | 3154 | - | Rel-18 | F | TEI18_Test | revised |
| R5-243490 | Add IE Aerial-Config | Ericsson | 38.508-1 | 3154 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-242649 | Add IE EpochTime | Ericsson | 38.508-1 | 3155 | - | Rel-18 | F | TEI18_Test | revised |
| R5-243491 | Add IE EpochTime | Ericsson | 38.508-1 | 3155 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-242650 | Add IE LTM-TCI-Info | Ericsson | 38.508-1 | 3156 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242651 | Add IE N3C-RelayUE-Info | Ericsson | 38.508-1 | 3157 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242652 | Add IE NeedForInterruptionInfoNR | Ericsson | 38.508-1 | 3158 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242653 | Add IE PDU-SessionID | Ericsson | 38.508-1 | 3159 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242654 | Add IE PUCCH-CSI-Resource | Ericsson | 38.508-1 | 3160 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242655 | Add IE QFI | Ericsson | 38.508-1 | 3161 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242656 | Add IE RACH-ConfigTwoTA | Ericsson | 38.508-1 | 3162 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242657 | Add IE ReferenceConfiguration | Ericsson | 38.508-1 | 3163 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242658 | Add IE SelectedPSCellForCHO-WithSCG | Ericsson | 38.508-1 | 3164 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242659 | Add IE SI-RequestConfigRepetition | Ericsson | 38.508-1 | 3165 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242660 | Add IEs SRS-PosTx-Hopping and SRS-PosResourceSetLinkedForAggBW | Ericsson | 38.508-1 | 3166 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242661 | Add IE TN-AreaId | Ericsson | 38.508-1 | 3167 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242708 | Definition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A applicability to protocol testing | WE Certification Oy, AT&T | 38.508-1 | 3168 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242714 | Definition of DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A applicability to protocol testing | WE Certification Oy, AT&T | 38.508-1 | 3169 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242715 | Correction of test bandwidth for CA_n2(2A) protocol testing | WE Certification Oy, AT&T, Anritsu | 38.508-1 | 3170 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | withdrawn |
| R5-242760 | Addition of system information combination for inter frequency NTN scenario | Qualcomm Incorporated | 38.508-1 | 3171 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243555 | Addition of system information combination for inter frequency NTN scenario | Qualcomm Incorporated | 38.508-1 | 3171 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242763 | Add IEs SL-BWP-DiscPoolConfig and SL-BWP-DiscPoolConfigCommon | Ericsson | 38.508-1 | 3172 | - | Rel-18 | F | TEI17_Test | agreed |
| R5-242764 | Add IEs SL-BWP-PRS-PoolConfig and SL-BWP-PRS-PoolConfigCommon | Ericsson | 38.508-1 | 3173 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-242771 | Update of Logical Channel Config Table 4.6.3-66 | Qualcomm Incorporated | 38.508-1 | 3174 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243536 | Update of Logical Channel Config Table 4.6.3-66 | Qualcomm Incorporated | 38.508-1 | 3174 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242772 | Update channel access configurations for shared spectrum | Qualcomm Incorporated | 38.508-1 | 3175 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242786 | Update of Channel bandwidth in Default NG-RAN RRC message and information elements contents | Bureau Veritas ADT | 38.508-1 | 3176 | - | Rel-18 | F | TEI15_Test | agreed |
| R5-242788 | Update of editors note for test frequencies | Bureau Veritas ADT | 38.508-1 | 3177 | - | Rel-18 | F | TEI15_Test | revised |
| R5-243603 | Update of editors note for test frequencies | Bureau Veritas ADT | 38.508-1 | 3177 | 1 | Rel-18 | F | TEI15_Test | agreed |
| R5-242799 | Addition of new combination of system information blocks for 5G Prose communication | TDIA, CATT | 38.508-1 | 3178 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242800 | Editorial update of SIB12 in 4.6.2 | TDIA, CATT | 38.508-1 | 3179 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242801 | Update of Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side | TDIA, CATT | 38.508-1 | 3180 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-243547 | Update of Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side | TDIA, CATT | 38.508-1 | 3180 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242802 | Update of Table 4.6.1-22 RRCSetupComplete | TDIA, CATT | 38.508-1 | 3181 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242803 | Update of Table 4.6.1A-2A RemoteUEInformationSidelink | TDIA, CATT | 38.508-1 | 3182 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242804 | Update of Table 4.6.1A-8 uuMessageTransferSidelink | TDIA, CATT | 38.508-1 | 3183 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-243548 | Update of Table 4.6.1A-8 uuMessageTransferSidelink | TDIA, CATT | 38.508-1 | 3183 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242805 | Addition of 5G ProSe discovery and security messages | TDIA, CATT | 38.508-1 | 3184 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242806 | Update of 5G ProSe direct discovery messages | TDIA,CATT | 38.508-1 | 3185 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242807 | Addition of Default HTTP messages for communication with the 5G ProSe | TDIA, CATT | 38.508-1 | 3186 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-243549 | Addition of Default HTTP messages for communication with the 5G ProSe | TDIA, CATT | 38.508-1 | 3186 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242808 | Addition of Default TLS message and information element contents | TDIA, CATT | 38.508-1 | 3187 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-243550 | Addition of Default TLS message and information element contents | TDIA, CATT | 38.508-1 | 3187 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242809 | Addition of spec reference of 3GPP TS 33.310, 33.503 and IETF RFC 5246 for 5G ProSe information elements | TDIA, CATT | 38.508-1 | 3188 | - | Rel-18 | F | NR_SL_relay-UEConTest | revised |
| R5-243551 | Addition of spec reference of 3GPP TS 33.310, 33.503 and IETF RFC 5246 for 5G ProSe information elements | TDIA, CATT | 38.508-1 | 3188 | 1 | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242810 | Update of Default settings of UICC and USIM for 5G ProSe | TDIA, CATT | 38.508-1 | 3189 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242811 | Update of Test procedure for 5G ProSe U2N Relay Discovery | TDIA, CATT | 38.508-1 | 3190 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242858 | Correction to SUL n83 test frequencies in Table 4.3.1.1.1.83-1 | Qualcomm Tech. Netherlands B.V | 38.508-1 | 3191 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242871 | Correction to defalut configuration of R15 RACH RRM test cases | Huawei, HiSilicon | 38.508-1 | 3192 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-242919 | Addition of TE and UE connection diagram for 3Tx NR CA and EN-DC | Huawei, HiSilicon | 38.508-1 | 3193 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-243007 | Correction to K1 for RRM TDD SCS 15 kHz | Anritsu | 38.508-1 | 3194 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243833 | Correction to K1 for RRM TDD SCS 15 kHz | Anritsu | 38.508-1 | 3194 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243008 | Clarification of test frequency for n5 CBW 25 MHz | Anritsu | 38.508-1 | 3195 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-243031 | Update inter-band NR CA configuration for CA_n3A-n8A-n78A | China Unicom | 38.508-1 | 3196 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243600 | Update inter-band NR CA configuration for CA_n3A-n8A-n78A | China Unicom | 38.508-1 | 3196 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243075 | Addition of a new section for IDC Configuration | CMCC, CATT, Qualcomm | 38.508-1 | 3197 | - | Rel-18 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-243581 | Addition of a new section for IDC Configuration | CMCC, CATT, Qualcomm | 38.508-1 | 3197 | 1 | Rel-18 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-243087 | Correction to IE PDCCH-ConfigCommon | MediaTek Inc. | 38.508-1 | 3198 | - | Rel-18 | F | TEI17_Test, NR_UE_pow_sav_enh_plus_CT-UEConTest | revised |
| R5-243493 | Correction to IE PDCCH-ConfigCommon | MediaTek Inc. | 38.508-1 | 3198 | 1 | Rel-18 | F | TEI17_Test, NR_UE_pow_sav_enh_plus_CT-UEConTest | agreed |
| R5-243090 | Addition of missing TRS configurations | Rohde & Schwarz | 38.508-1 | 3199 | - | Rel-18 | F | TEI16_Test, NR_perf_enh-UEConTest | revised |
| R5-243604 | Addition of missing TRS configurations | Rohde & Schwarz | 38.508-1 | 3199 | 1 | Rel-18 | F | TEI16_Test, NR_perf_enh-UEConTest | agreed |
| R5-243168 | Editorial correction of Table 4.5.4.2-4: WLAN IPsec_SA_Established | Nokia | 38.508-1 | 3200 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-243174 | Common configuration for SN initiated inter-SN CPC EN-DC for 38.508-1 | Nokia | 38.508-1 | 3201 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-243952 | Common configuration for SN initiated inter-SN CPC EN-DC for 38.508-1 | Nokia | 38.508-1 | 3201 | 1 | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-243184 | Update of message contents for CQI reporting in CA | ROHDE & SCHWARZ | 38.508-1 | 3202 | - | Rel-18 | F | TEI16_Test, NR_perf_enh-UEConTest | withdrawn |
| R5-243187 | Add IE SL-CBR-CommonTxDedicated-SL-PRS-RP-List | Ericsson | 38.508-1 | 3203 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-243188 | Add IE SL-ConfiguredGrantConfigDedicated-SL-PRS-RP | Ericsson | 38.508-1 | 3204 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-243216 | Add IEs SL-DRX-Config, SL-DRX-ConfigGC-BC, SL-DRX-ConfigUC and SL-DRX-ConfigUC-SemiStatic | Ericsson | 38.508-1 | 3205 | - | Rel-18 | F | TEI17_Test | revised |
| R5-243492 | Add IEs SL-DRX-Config, SL-DRX-ConfigGC-BC, SL-DRX-ConfigUC and SL-DRX-ConfigUC-SemiStatic | Ericsson | 38.508-1 | 3205 | 1 | Rel-18 | F | TEI17_Test | agreed |
| R5-243217 | Add IEs SL-FreqSelectionConfig and SL-IndirectPathAddChange | Ericsson | 38.508-1 | 3206 | - | Rel-18 | F | TEI18_Test | agreed |
| R5-243286 | Corrections on 4.3.1 for NTN FR1 test channel bandwidth | ZTE Corporation | 38.508-1 | 3207 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243293 | Addition to 4.3.1 for FR2 Redcap UE test channel bandwidth | ZTE Corporation | 38.508-1 | 3208 | - | Rel-18 | F | TEI17_Test, NR_redcap_enh_plus_CT1-UEConTest | revised |
| R5-243750 | Addition to 4.3.1 for FR2 Redcap UE test channel bandwidth | ZTE Corporation | 38.508-1 | 3208 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_enh_plus_CT1-UEConTest | agreed |
| R5-243294 | Corrections on 4.3.1.4.1.2 for test frequency for DC_38A_n78A | ZTE Corporation | 38.508-1 | 3209 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243337 | Correction on Rel-17 HST conditions in ServingCellConfigCommon | Keysight Technologies | 38.508-1 | 3210 | - | Rel-18 | F | TEI17_Test, NR_HST_FR1_enh-UEConTest | agreed |
| R5-243438 | Addition of connection diagram for CSI test cases for inter-cell interference scenarios | Qualcomm Inc | 38.508-1 | 3211 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-243900 | Addition of connection diagram for CSI test cases for inter-cell interference scenarios | Qualcomm Inc | 38.508-1 | 3211 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-243481 | Addition of DCI information for semi static channel access in shared spectrum | Qualcomm Incorporated | 38.508-1 | 3212 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243482 | Common configuration change for SDT condition | Nokia, Huawei | 38.508-1 | 3213 | - | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-243654 | Correction of test bandwidth for CA_n2(2A) | WE Certification Oy, AT&T, Anritsu | 38.508-1 | 3214 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-243533 | Addition of signalling test frequencies for CA_n48C and CA_n5B | Ericsson, Verizon, MCC TF160 | 38.508-1 | 3215 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243546 | Correction for test procedure 4.9.41 | ZTE Corporation | 38.508-1 | 3216 | - | Rel-18 | F | NR_SL_relay-UEConTest | agreed |
| R5-242131 | Correction to HPUE PICS mnemonic | MediaTek Inc. | 38.508-2 | 0607 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | withdrawn |
| R5-242133 | Correction to the note of NR NTN PICS | MediaTek Inc. | 38.508-2 | 0608 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242143 | Updates to align PICS mnemonics | MediaTek Inc. | 38.508-2 | 0609 | - | Rel-18 | F | TEI17_Test | withdrawn |
| R5-242242 | Addition of UE capability for CA_n28A-n40A | KDDI Corporation | 38.508-2 | 0610 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243812 | Addition of UE capability for CA_n28A-n40A | KDDI Corporation | 38.508-2 | 0610 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242243 | Addition of UE capability for CA_n3A-n28A-n41A-n77A | KDDI Corporation | 38.508-2 | 0611 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242251 | Addition and correction of RF baseline implementation capabilities for PC2 EN-DC combs within FR1 | KDDI Corporation, MediaTek Inc | 38.508-2 | 0612 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-243609 | Addition and correction of RF baseline implementation capabilities for PC2 EN-DC combs within FR1 | KDDI Corporation, MediaTek Inc | 38.508-2 | 0612 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242255 | Editorial correction of UE capability for R16 EN-DC combs within FR1 | KDDI Corporation | 38.508-2 | 0613 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242289 | Editorial correction of n66 with UL MIMO capabilities | MediaTek Beijing Inc. | 38.508-2 | 0614 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | agreed |
| R5-242324 | Deletion of PICS pc_pdsch_MappingTypeA | MCC TF160 | 38.508-2 | 0615 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242325 | Updates to align PICS mnemonics | MCC TF160 | 38.508-2 | 0616 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242364 | Introduction of band n85 for physical layer baseline implementation capabilities | Nokia | 38.508-2 | 0617 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | revised |
| R5-243611 | Introduction of band n85 for physical layer baseline implementation capabilities | Nokia | 38.508-2 | 0617 | 1 | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-242370 | Introduction of band n106 for physical layer baseline implementation capabilities | Nokia | 38.508-2 | 0618 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-242380 | Introduction of bands n7 and n78 PC1 for physical layer baseline implementation capabilities | Nokia | 38.508-2 | 0619 | - | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | agreed |
| R5-242434 | Fix the release information for a NR CA PC2 band configuration | Verizon | 38.508-2 | 0620 | - | Rel-18 | F | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-UEConTest | agreed |
| R5-242449 | Update for additional ENDC band configuration | Verizon Spain | 38.508-2 | 0621 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242451 | Update for additional ENDC band configurations | Verizon | 38.508-2 | 0622 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242461 | Update for additional NRCA band configurations | Verizon Spain | 38.508-2 | 0623 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243608 | Update for additional NRCA band configurations | Verizon Spain | 38.508-2 | 0623 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242498 | Addition of n41 PC1 RF baseline ICS | CMCC, Nokia, Nokia Shanghai Bell | 38.508-2 | 0624 | - | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | revised |
| R5-243610 | Addition of n41 PC1 RF baseline ICS | CMCC, Nokia, Nokia Shanghai Bell | 38.508-2 | 0624 | 1 | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | agreed |
| R5-242544 | Editorial changes to tables of 38.508-2 | Qualcomm Technologies Ireland | 38.508-2 | 0625 | - | Rel-18 | F | TEI18_Test | revised |
| R5-243496 | Editorial changes to tables of 38.508-2 | Qualcomm Technologies Ireland | 38.508-2 | 0625 | 1 | Rel-18 | F | TEI18_Test | agreed |
| R5-242583 | Addition of UE General Capability of supporting NSAC | CATT | 38.508-2 | 0626 | - | Rel-17 | F | TEI15_Test | withdrawn |
| R5-242591 | Addition of UE capability for new EN-DC comb within FR2 | KDDI Corporation | 38.508-2 | 0627 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | withdrawn |
| R5-242709 | Addition of capability declaration for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | 38.508-2 | 0628 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243811 | Addition of capability declaration for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | 38.508-2 | 0628 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242713 | Addition of capability declaration for DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A | WE Certification Oy, AT&T | 38.508-2 | 0629 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-243657 | Addition of capability declaration for DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A | WE Certification Oy, AT&T | 38.508-2 | 0629 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242733 | Addition of PICS for ING_5GS test cases | Qualcomm Incorporated, China Telecom | 38.508-2 | 0630 | - | Rel-18 | F | ING_5GS-UEConTest | revised |
| R5-243530 | Addition of PICS for ING_5GS test cases | Qualcomm Incorporated, China Telecom | 38.508-2 | 0630 | 1 | Rel-18 | F | ING_5GS-UEConTest | agreed |
| R5-242759 | Addition of NR NTN PICS | Qualcomm Incorporated | 38.508-2 | 0631 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-242765 | Addition of NR NTN PICS | Qualcomm Incorporated | 38.508-2 | 0632 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242784 | Removal of defination and abbreviation for IXIT and PIXIT | Bureau Veritas ADT, CATT | 38.508-2 | 0633 | - | Rel-18 | F | TEI15_Test | agreed |
| R5-242787 | Update of Rx antenna ports associated declaration | Bureau Veritas ADT | 38.508-2 | 0634 | - | Rel-18 | F | TEI15_Test | agreed |
| R5-242827 | Add new PICS for MUSIM gap | Huawei, HiSilicon | 38.508-2 | 0635 | - | Rel-18 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | withdrawn |
| R5-242832 | Add new PICS for RedCap HD-FDD TC | Huawei, HiSilicon | 38.508-2 | 0636 | - | Rel-18 | F | TEI17_Test | revised |
| R5-243495 | Add new PICS for RedCap HD-FDD TC | Huawei, HiSilicon | 38.508-2 | 0636 | 1 | Rel-18 | F | TEI17_Test | agreed |
| R5-242865 | Addition of PICS for MCE RRM test cases | Huawei, HiSilicon | 38.508-2 | 0637 | - | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-242920 | Addition of physical layer baseline implementation capabilities for 3Tx NR CA and EN-DC | Huawei, HiSilicon | 38.508-2 | 0638 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | revised |
| R5-243606 | Addition of physical layer baseline implementation capabilities for 3Tx NR CA and EN-DC | Huawei, HiSilicon | 38.508-2 | 0638 | 1 | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-243068 | Update NR band and CADC configs status in ICS Annex B | CMCC | 38.508-2 | 0639 | - | Rel-18 | F | TEI17_Test | agreed |
| R5-243076 | Addition of new PICS for IDC mechanism | CMCC, CATT, Qualcomm | 38.508-2 | 0640 | - | Rel-18 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-243582 | Addition of new PICS for IDC mechanism | CMCC, CATT, Qualcomm | 38.508-2 | 0640 | 1 | Rel-18 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-243086 | Add new PICS for MUSIM gap | Huawei, HiSilicon | 38.508-2 | 0641 | - | Rel-18 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | revised |
| R5-243957 | Add new PICS for MUSIM gap | Huawei, HiSilicon | 38.508-2 | 0641 | 1 | Rel-18 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | withdrawn |
| R5-243132 | Addition of PICS for TxD for NR FR1 bands | Huawei, HiSilicon, Bureau Veritas | 38.508-2 | 0642 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-243157 | Addition of PICS for FeMIMO | Huawei, HiSilicon | 38.508-2 | 0643 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-243543 | Addition of PICS for FeMIMO | Huawei, HiSilicon | 38.508-2 | 0643 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-243177 | Addition of capability clauses for inter-SN CPC testcases for EN-DC and NR-DC | Nokia | 38.508-2 | 0644 | - | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-243953 | Addition of capability clauses for inter-SN CPC testcases for EN-DC and NR-DC | Nokia | 38.508-2 | 0644 | 1 | Rel-18 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-243189 | Addition of CA_n1A-n8A-n78A,CA_n3A-n8A-n78A into NR inter-band CA within FR1 three bands | China Unicom | 38.508-2 | 0645 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243607 | Addition of CA_n1A-n8A-n78A,CA_n3A-n8A-n78A into NR inter-band CA within FR1 three bands | China Unicom | 38.508-2 | 0645 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243287 | Update on A.4.1 and A.4.3.1 for FR1-NTN implementation types and RF baseline capabilities | ZTE Corporation | 38.508-2 | 0646 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243398 | Correction to NTN PICS | Qualcomm France | 38.508-2 | 0647 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243446 | ICS updates pertaining to RF phase continuity conformance tests | Apple Benelux B.V. | 38.508-2 | 0648 | - | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | revised |
| R5-243612 | ICS updates pertaining to RF phase continuity conformance tests | Apple Benelux B.V. | 38.508-2 | 0648 | 1 | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | agreed |
| R5-243413 | ICS updates pertaining to RF phase continuity conformance tests | Apple Benelux B.V. | 38.509 | 0084 | - | Rel-17 | F | TEI17_Test, NR_cov_enh-UEConTest | withdrawn |
| R5-242247 | Addition of delta TIBc for CA_n28A-n40A | KDDI Corporation | 38.521-1 | 2736 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242248 | Addition of delta TIBc for CA_n3A-n28A-n41A-n77A | KDDI Corporation | 38.521-1 | 2737 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243822 | Addition of delta TIBc for CA_n3A-n28A-n41A-n77A | KDDI Corporation | 38.521-1 | 2737 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242249 | Addition of reference sensitivity for CA_n28A-n40A | KDDI Corporation | 38.521-1 | 2738 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243821 | Addition of reference sensitivity for CA_n28A-n40A | KDDI Corporation | 38.521-1 | 2738 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242250 | Addition of delta RIBc and reference sensitivity for CA_n3A-n28A-n41A-n77A | KDDI Corporation | 38.521-1 | 2739 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242284 | Update of Rx test cases to add 50MHz for NR band n7 | MediaTek Beijing Inc. | 38.521-1 | 2740 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | revised |
| R5-243625 | Update of Rx test cases to add 50MHz for NR band n7 | MediaTek Beijing Inc. | 38.521-1 | 2740 | 1 | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-242286 | Update of Operating bands for UL MIMO band n66 | MediaTek Beijing Inc. | 38.521-1 | 2741 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | withdrawn |
| R5-242287 | Addition of n66 UL MIMO MOP | MediaTek Beijing Inc. | 38.521-1 | 2742 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | agreed |
| R5-242288 | Addition of n66 UL MIMO MPR | MediaTek Beijing Inc. | 38.521-1 | 2743 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | agreed |
| R5-242360 | Correction to Table 6.F-1 | Ericsson | 38.521-1 | 2744 | - | Rel-18 | F | NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-242365 | Adding transmitter requirements for band n85 | Nokia | 38.521-1 | 2745 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-242366 | Adding receiver requirements for band n85 | Nokia | 38.521-1 | 2746 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | revised |
| R5-243651 | Adding receiver requirements for band n85 | Nokia | 38.521-1 | 2746 | 1 | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-242372 | Introduction of new band n106 into clause 5 | Nokia | 38.521-1 | 2747 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243643 | Introduction of new band n106 into clause 5 | Nokia | 38.521-1 | 2747 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-242373 | Adding transmitter requirements for band n106 | Nokia | 38.521-1 | 2748 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243640 | Adding transmitter requirements for band n106 | Nokia | 38.521-1 | 2748 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-242374 | Adding receiver requirements for band n106 | Nokia | 38.521-1 | 2749 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243641 | Adding receiver requirements for band n106 | Nokia | 38.521-1 | 2749 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-242375 | Correction to reference sensitivity test requirements for CA_n25A-n77A, CA_n25A-n78A and CA_n66A-n78A | Nokia | 38.521-1 | 2750 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243635 | Correction to reference sensitivity test requirements for CA_n25A-n77A, CA_n25A-n78A and CA_n66A-n78A | Nokia | 38.521-1 | 2750 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242379 | Introduction of PC1 transmitter requirements for bands n7, n41 and n78 | Nokia | 38.521-1 | 2751 | - | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | revised |
| R5-243877 | Introduction of PC1 transmitter requirements for bands n7, n41 and n78 | Nokia | 38.521-1 | 2751 | 1 | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | agreed |
| R5-242381 | Adding reference sensitivity requirements for UEs that support optional symmetric UL/DL in band n71 | Nokia | 38.521-1 | 2752 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243642 | Adding reference sensitivity requirements for UEs that support optional symmetric UL/DL in band n71 | Nokia | 38.521-1 | 2752 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-242391 | Update to spurious emissions UE coexistence as part of introduction of LTE Band 106 | Nokia | 38.521-1 | 2753 | - | Rel-18 | F | LTE_900MHz_US-UEConTest | agreed |
| R5-242395 | Introduction of dedicated spectrum less than 5MHz for FR1 to clause 5 and annex A | Nokia | 38.521-1 | 2754 | - | Rel-18 | F | NR_FR1_lessthan_5MHz_BW-UEConTest | revised |
| R5-243639 | Introduction of dedicated spectrum less than 5MHz for FR1 to clause 5 and annex A | Nokia | 38.521-1 | 2754 | 1 | Rel-18 | F | NR_FR1_lessthan_5MHz_BW-UEConTest | agreed |
| R5-242396 | Addition of 3 MHz channel bandwidth for transmitter requirements | Nokia | 38.521-1 | 2755 | - | Rel-18 | F | NR_FR1_lessthan_5MHz_BW-UEConTest | revised |
| R5-243637 | Addition of 3 MHz channel bandwidth for transmitter requirements | Nokia | 38.521-1 | 2755 | 1 | Rel-18 | F | NR_FR1_lessthan_5MHz_BW-UEConTest | agreed |
| R5-242397 | Addition of 3 MHz channel bandwidth for receiver requirements | Nokia | 38.521-1 | 2756 | - | Rel-18 | F | NR_FR1_lessthan_5MHz_BW-UEConTest | revised |
| R5-243638 | Addition of 3 MHz channel bandwidth for receiver requirements | Nokia | 38.521-1 | 2756 | 1 | Rel-18 | F | NR_FR1_lessthan_5MHz_BW-UEConTest | agreed |
| R5-242463 | Update for additional NRCA band configurations | Verizon | 38.521-1 | 2757 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242466 | Addition of n41 PC1 MOP TC | CMCC, Nokia, Nokia Shanghai Bell | 38.521-1 | 2758 | - | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | agreed |
| R5-242467 | Addition of n41 PC1 MPR TC | CMCC, Nokia, Nokia Shanghai Bell | 38.521-1 | 2759 | - | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | agreed |
| R5-242468 | Update n41 PC1 UTRA ACLR TC | CMCC, Nokia, Nokia Shanghai Bell | 38.521-1 | 2760 | - | Rel-18 | F | LTE_NR_HPUE_FWVM_R18-UEConTest | agreed |
| R5-242485 | Addition of test case 7.4F, Maximum input level for shared spectrum channel access | Ericsson | 38.521-1 | 2761 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243886 | Addition of test case 7.4F, Maximum input level for shared spectrum channel access | Ericsson | 38.521-1 | 2761 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242488 | Addition of test case 6.3F.4.4, Aggregate power tolerance for shared spectrum channel access | Ericsson | 38.521-1 | 2762 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243885 | Addition of test case 6.3F.4.4, Aggregate power tolerance for shared spectrum channel access | Ericsson | 38.521-1 | 2762 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242490 | Updates to editors note in NR-U Tx test cases | Ericsson | 38.521-1 | 2763 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243646 | Updates to editors note in NR-U Tx test cases | Ericsson | 38.521-1 | 2763 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242491 | Addition of MU and TT for new NR-U test cases | Ericsson | 38.521-1 | 2764 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243884 | Addition of MU and TT for new NR-U test cases | Ericsson | 38.521-1 | 2764 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242516 | Corrected MU and TT definition for FR1 UL MIMO test case 6.5D.2.3 | Keysight Technologies | 38.521-1 | 2765 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242517 | Introduction of new SUL test case 7.7C | Keysight Technologies | 38.521-1 | 2766 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243897 | Introduction of new SUL test case 7.7C | Keysight Technologies | 38.521-1 | 2766 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242519 | Absolute power control - test requirements correction | Keysight Technologies | 38.521-1 | 2767 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242520 | EVM in transient periods - test case definition update | Keysight Technologies | 38.521-1 | 2768 | - | Rel-18 | F | TEI16_Test | withdrawn |
| R5-242521 | Clarification for Rx test requirements for PC1.5 | Keysight Technologies, Huawei | 38.521-1 | 2769 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-242522 | MU and TT defintion for TxD EVM test case 6.4G.2.1 | Keysight Technologies, Rohde&Schwarz | 38.521-1 | 2770 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | revised |
| R5-243898 | MU and TT defintion for TxD EVM test case 6.4G.2.1 | Keysight Technologies, Rohde&Schwarz | 38.521-1 | 2770 | 1 | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-242528 | Apply interlaced RB allocation in test configuration table in test case 6.4F.2.2 | Ericsson | 38.521-1 | 2771 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243623 | Apply interlaced RB allocation in test configuration table in test case 6.4F.2.2 | Ericsson | 38.521-1 | 2771 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242529 | Addition of MU and TT for NR-U test case | Ericsson | 38.521-1 | 2772 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243648 | Addition of MU and TT for NR-U test case | Ericsson | 38.521-1 | 2772 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242538 | Addition of CA_n48C reference sensitivity requirement | Ericsson | 38.521-1 | 2773 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242571 | General updates of clause 5 for R17 new CBW configurations | China Unicom, Nokia, CAICT | 38.521-1 | 2774 | - | Rel-18 | F | TEI17_Test, NR_lic_bands_BW_R17-UEConTest | agreed |
| R5-242572 | General updates of TS 38.521-1 clause 5 for R16 CA configurations | China Unicom, KDDI | 38.521-1 | 2775 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242573 | General updates of TS 38.521-1 clause 5 for R17 CA configurations | China Unicom, Verizon, KDDI, WE Certification Oy, AT&T | 38.521-1 | 2776 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243636 | General updates of TS 38.521-1 clause 5 for R17 CA configurations | China Unicom, Verizon, KDDI, WE Certification Oy, AT&T | 38.521-1 | 2776 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242602 | Correction to Table 6.F-1 | Ericsson | 38.521-1 | 2777 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243647 | Correction to Table 6.F-1 | Ericsson | 38.521-1 | 2777 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242705 | Addition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A Tx test cases | WE Certification Oy, AT&T | 38.521-1 | 2778 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243655 | Addition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A Tx test cases | WE Certification Oy, AT&T | 38.521-1 | 2778 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242706 | Addition of reference sensitivity tests for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | 38.521-1 | 2779 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243656 | Addition of reference sensitivity tests for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | 38.521-1 | 2779 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242707 | Definition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A applicability to protocol testing | WE Certification Oy, AT&T | 38.521-1 | 2780 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | withdrawn |
| R5-242844 | Addition of test case 6.3H.3.1 Minimum output power for inter-band UL CA with UL MIMO | TTA | 38.521-1 | 2781 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | revised |
| R5-243626 | Addition of test case 6.3H.3.1 Minimum output power for inter-band UL CA with UL MIMO | TTA | 38.521-1 | 2781 | 1 | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242845 | Addition of test case 6.5H.3.1 Occupied bandwidth for inter-band UL CA with UL MIMO | TTA | 38.521-1 | 2782 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | revised |
| R5-243627 | Addition of test case 6.5H.3.1 Occupied bandwidth for inter-band UL CA with UL MIMO | TTA | 38.521-1 | 2782 | 1 | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242918 | Correction to initial condition for SUL Tx switching TC 6.3C.3.2.4.1 | Qualcomm Tech. Netherlands B.V | 38.521-1 | 2783 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242921 | Update of configured transmitted power requirements for 3Tx NR CA | Huawei, HiSilicon | 38.521-1 | 2784 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242922 | Addition of new test case 6.2H.3.1 MOP for inter-band CA with MIMO | Huawei, HiSilicon | 38.521-1 | 2785 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242923 | Addition of new test cases 6.2H.3.x for inter-band CA with MIMO | Huawei, HiSilicon | 38.521-1 | 2786 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242924 | Addition of new test case 6.2L.3.1 MOP for inter-band CA with TxD | Huawei, HiSilicon | 38.521-1 | 2787 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242925 | Addition of new test cases 6.2L.3.x for inter-band CA with TxD | Huawei, HiSilicon | 38.521-1 | 2788 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | revised |
| R5-243628 | Addition of new test cases 6.2L.3.x for inter-band CA with TxD | Huawei, HiSilicon | 38.521-1 | 2788 | 1 | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242926 | Addition of MU and TT analysis for 3Tx NR CA test cases | Huawei, HiSilicon | 38.521-1 | 2789 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242927 | Addition of general informationfor 3Tx NR CA | Huawei, HiSilicon | 38.521-1 | 2790 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | revised |
| R5-243878 | Addition of general informationfor 3Tx NR CA | Huawei, HiSilicon | 38.521-1 | 2790 | 1 | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242944 | Update to FR1 CA Configurations for n78A-n79A with 1UL | NTT DOCOMO INC. | 38.521-1 | 2791 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242945 | Core spec alignment Missing DMRS configuration restriction for intra ULCA | Qualcomm Tech. Netherlands B.V | 38.521-1 | 2792 | - | Rel-18 | F | TEI16_Test, NR_RF_FR1-UEConTest | agreed |
| R5-242948 | Addition of REFSENS for PC3 CA n78A-n79A with 1UL | NTT DOCOMO INC. | 38.521-1 | 2793 | - | Rel-18 | F | TEI15_Test | revised |
| R5-243824 | Addition of REFSENS for PC3 CA n78A-n79A with 1UL | NTT DOCOMO INC. | 38.521-1 | 2793 | 1 | Rel-18 | F | TEI15_Test | agreed |
| R5-242952 | Addition of REFSENS for PC2 CA n78A-n79A with 1UL | NTT DOCOMO INC. | 38.521-1 | 2794 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-243009 | Correction to note for inter-band CA including n48 | Anritsu | 38.521-1 | 2795 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-243010 | Correction to test configuration and test requirements for NS_49 A-MPR | Anritsu, China Unicom | 38.521-1 | 2796 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243011 | Correction to test configuration and test requirements for NS_21 A-MPR | Anritsu | 38.521-1 | 2797 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-243012 | Correction to notes for NS_21 A-SEM | Anritsu | 38.521-1 | 2798 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-243013 | Update for EVM including symbols with transient period | Anritsu, Keysight | 38.521-1 | 2799 | - | Rel-18 | F | TEI16_Test | revised |
| R5-243839 | Update for EVM including symbols with transient period | Anritsu, Keysight | 38.521-1 | 2799 | 1 | Rel-18 | F | TEI16_Test | agreed |
| R5-243022 | Correction to LCRB of CA_n5-n48-n66 in 7.3A.0.5 | Anritsu | 38.521-1 | 2800 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243834 | Correction to LCRB of CA_n5-n48-n66 in 7.3A.0.5 | Anritsu | 38.521-1 | 2800 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-243023 | Correction to notes for support of SCS and CBW in CA configurations | Anritsu | 38.521-1 | 2801 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243912 | Correction to notes for support of SCS and CBW in CA configurations | Anritsu | 38.521-1 | 2801 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243027 | Addition of delta TIB into 6.2A.4.0.2.3 for CA_n3A-n8A-n78A | China Unicom | 38.521-1 | 2802 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243028 | Addition of delta RIB into 7.3A.0.3.2.3 for CA_n3A-n8A-n78A | China Unicom | 38.521-1 | 2803 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243632 | Addition of delta RIB into 7.3A.0.3.2.3 for CA_n3A-n8A-n78A | China Unicom | 38.521-1 | 2803 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243029 | Addition of 3DL CA minimun requirements of reference sensitivity exceptions into 7.3A.0.5 for CA_n3A-n8A-n78A | China Unicom | 38.521-1 | 2804 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243030 | Addition of 3DL CA reference sensitivity TC into 7.3A.2 for CA_n3A-n8A-n78A | China Unicom | 38.521-1 | 2805 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243617 | Addition of 3DL CA reference sensitivity TC into 7.3A.2 for CA_n3A-n8A-n78A | China Unicom | 38.521-1 | 2805 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243036 | Update to ATG Definitions and Symbols | CMCC | 38.521-1 | 2806 | - | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-243906 | Update to ATG Definitions and Symbols | CMCC | 38.521-1 | 2806 | 1 | Rel-18 | F | NR_ATG-UEConTest | revised |
| R5-244009 | Update to ATG Definitions and Symbols | CMCC | 38.521-1 | 2806 | 2 | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-243037 | Update to ATG RF Tx TCs | CMCC | 38.521-1 | 2807 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-243038 | Update to ATG RF Rx TCs | CMCC, CAICT | 38.521-1 | 2808 | - | Rel-18 | F | NR_ATG-UEConTest | agreed |
| R5-243045 | Update to REFSENSE TC for CA_n79C | CMCC | 38.521-1 | 2809 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243046 | Update delta TIBc of CA_n39A-n41A for simultaneous RxTx capability | CMCC | 38.521-1 | 2810 | - | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | agreed |
| R5-243047 | Update delta RIBc of CA_n39A-n41A for simultaneous RxTx capability | CMCC | 38.521-1 | 2811 | - | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | agreed |
| R5-243048 | Update REFSENSE exceptions due to UL harmonic interference for CA_n39A-n41A | CMCC | 38.521-1 | 2812 | - | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | revised |
| R5-243837 | Update REFSENSE exceptions due to UL harmonic interference for CA_n39A-n41A | CMCC | 38.521-1 | 2812 | 1 | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | agreed |
| R5-243049 | Update REFSENSE exceptions due to CBI for CA_n39A-n41A | CMCC | 38.521-1 | 2813 | - | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | agreed |
| R5-243050 | Update REFSENSE TC 7.3A.1_1 for CA_n39A-n41A | CMCC | 38.521-1 | 2814 | - | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | revised |
| R5-243838 | Update REFSENSE TC 7.3A.1_1 for CA_n39A-n41A | CMCC | 38.521-1 | 2814 | 1 | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | agreed |
| R5-243065 | Addition of PC1.5 n39 A-MPR | CMCC | 38.521-1 | 2815 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | agreed |
| R5-243123 | Addition of PC2 for n8 UL MIMO into 6.2D.1 MOP TC | China Unicom | 38.521-1 | 2816 | - | Rel-18 | F | NR_bands_UL_MIMO_R18-UEConTest | agreed |
| R5-243126 | Adding clarification of trace mode in emission test cases | Huawei, HiSilicon | 38.521-1 | 2817 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243127 | Correction to general RB allocation in 6.1A | Huawei, HiSilicon | 38.521-1 | 2818 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243128 | Cleaning up test applicability for ULFPTx in test cases | Huawei, HiSilicon | 38.521-1 | 2819 | - | Rel-18 | F | TEI16_Test, NR_eMIMO-UEConTest | agreed |
| R5-243130 | Editorial correction of Annex F for R17 FR1 test cases | Huawei, HiSilicon | 38.521-1 | 2820 | - | Rel-18 | F | TEI17_Test, NR_RF_FR1_enh-UEConTest | agreed |
| R5-243134 | Addition of NR V2X MOP for inter-band concurrent operation | Huawei, HiSilicon | 38.521-1 | 2821 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243136 | Update of NR V2X MPR test cases | Huawei, HiSilicon | 38.521-1 | 2822 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243137 | Update of NR V2X AMPR test cases | Huawei, HiSilicon | 38.521-1 | 2823 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-243629 | Update of NR V2X AMPR test cases | Huawei, HiSilicon | 38.521-1 | 2823 | 1 | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243138 | Addition of NR V2X NR configured transmit power test cases | Huawei, HiSilicon | 38.521-1 | 2824 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243140 | Update to NR V2X minimum output power test cases | Huawei, HiSilicon | 38.521-1 | 2825 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243141 | Update to NR V2X transmit off power test cases | Huawei, HiSilicon | 38.521-1 | 2826 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243142 | Addition of NR V2X ON/OFF time mask test cases | Huawei, HiSilicon | 38.521-1 | 2827 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243145 | Addition of NR V2X MOP minimum requirement for sidelink enhancement | Huawei, HiSilicon | 38.521-1 | 2828 | - | Rel-18 | F | NR_SL_enh-UEConTest | agreed |
| R5-243146 | Update to NR V2X MPR test cases to add PC2 and intra-band concurrent operation | Huawei, HiSilicon | 38.521-1 | 2829 | - | Rel-18 | F | NR_SL_enh-UEConTest | agreed |
| R5-243148 | Update to NR V2X AMPR test cases to add PC2 and intra-band concurrent operation | Huawei, HiSilicon | 38.521-1 | 2830 | - | Rel-18 | F | NR_SL_enh-UEConTest | agreed |
| R5-243201 | Updating AMPR requirements for PC3 UE supporting power boosting | Huawei, HiSilicon | 38.521-1 | 2831 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243202 | Updating AMPR testing for CA_n41A-n79A | Huawei, HiSilicon | 38.521-1 | 2832 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243205 | Updating AMPR testing for NS_43 for band n8 | Huawei, HiSilicon | 38.521-1 | 2833 | - | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | agreed |
| R5-243207 | Updating PC2 AMPR testing for NS_100 for band n8 | Huawei, HiSilicon | 38.521-1 | 2834 | - | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | revised |
| R5-243631 | Updating PC2 AMPR testing for NS_100 for band n8 | Huawei, HiSilicon | 38.521-1 | 2834 | 1 | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | agreed |
| R5-243208 | Updating general clause for Uplink switching across up to four bands | Huawei, HiSilicon | 38.521-1 | 2835 | - | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-243209 | Adding new test case Time mask for switching across three uplink bands | Huawei, HiSilicon | 38.521-1 | 2836 | - | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243658 | Adding new test case Time mask for switching across three uplink bands | Huawei, HiSilicon | 38.521-1 | 2836 | 1 | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-243210 | Adding new test case Time mask for switching across four uplink bands | Huawei, HiSilicon | 38.521-1 | 2837 | - | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243659 | Adding new test case Time mask for switching across four uplink bands | Huawei, HiSilicon | 38.521-1 | 2837 | 1 | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-243211 | Adding new SUL test case Time mask for switching across three uplink bands | Huawei, HiSilicon | 38.521-1 | 2838 | - | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243620 | Adding new SUL test case Time mask for switching across three uplink bands | Huawei, HiSilicon | 38.521-1 | 2838 | 1 | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-243212 | Adding new SUL test case Time mask for switching across four uplink bands | Huawei, HiSilicon | 38.521-1 | 2839 | - | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243644 | Adding new SUL test case Time mask for switching across four uplink bands | Huawei, HiSilicon | 38.521-1 | 2839 | 1 | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-243213 | Updating MU and TT for Time mask for switching across up to four bands testing | Huawei, HiSilicon | 38.521-1 | 2840 | - | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243645 | Updating MU and TT for Time mask for switching across up to four bands testing | Huawei, HiSilicon | 38.521-1 | 2840 | 1 | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-243228 | Update to Annex F for V2X test cases | Huawei, HiSilicon | 38.521-1 | 2841 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-243630 | Update to Annex F for V2X test cases | Huawei, HiSilicon | 38.521-1 | 2841 | 1 | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243256 | Correction of spurious emission minimum conformance requirements for UE co-ex to align with RAN4 specs | Apple, Keysight | 38.521-1 | 2842 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243257 | Correction of spurious emission minimum conformance requirements for UE co-ex for intra-band contiguous CA | Apple, Keysight | 38.521-1 | 2843 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243633 | Correction of spurious emission minimum conformance requirements for UE co-ex for intra-band contiguous CA | Apple, Keysight | 38.521-1 | 2843 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-243258 | Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL | Apple, Keysight | 38.521-1 | 2844 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243649 | Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL | Apple, Keysight | 38.521-1 | 2844 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243259 | Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL Rel16 | Apple, Keysight | 38.521-1 | 2845 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243816 | Correction of spurious emission minimum conformance requirements for UE co-ex for inter-band 2UL Rel16 | Apple, Keysight | 38.521-1 | 2845 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243260 | Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-16 | Apple, Keysight | 38.521-1 | 2846 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243817 | Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-16 | Apple, Keysight | 38.521-1 | 2846 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243261 | Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-17 | Apple, Keysight | 38.521-1 | 2847 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243634 | Correction of spurious emission test requirements for UE co-ex for CA (2UL CA) Rel-17 | Apple, Keysight | 38.521-1 | 2847 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-243279 | Updates to Reference sensitivity for SUL | Keysight Technologies UK Ltd | 38.521-1 | 2848 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243285 | Corrections on 6.2F.3 for UE A-MPR requirements for shared spectrum access | ZTE Corporation | 38.521-1 | 2849 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243295 | Corrections on 6.2 and 6.2A for almost contiguous A-MPR requirements for HPUE | ZTE Corporation | 38.521-1 | 2850 | - | Rel-18 | F | TEI16_Test, LTE_NR_B41_Bn41_PC29dBm-UEConTest | revised |
| R5-243818 | Corrections on 6.2 and 6.2A for almost contiguous A-MPR requirements for HPUE | ZTE Corporation | 38.521-1 | 2850 | 1 | Rel-18 | F | TEI16_Test, LTE_NR_B41_Bn41_PC29dBm-UEConTest | agreed |
| R5-243296 | Corrections on 6.2.3 for UE additional maximum output power reduction requirements | ZTE Corporation | 38.521-1 | 2851 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243297 | Corrections on 6.5.2.2 for UE spectrum emission mask | ZTE Corporation | 38.521-1 | 2852 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243298 | Corrections on 6.5.3.3 for UE additional spurious emissions | ZTE Corporation | 38.521-1 | 2853 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243299 | Corrections on clause 6 for removal of power class capable UE denotation | ZTE Corporation | 38.521-1 | 2854 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243300 | Cleanups on 7.3.2 for reference sensitivity power level test | ZTE Corporation | 38.521-1 | 2855 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243301 | Updates on 3.1 and 4.3 for Definitions and Specification suffix information | ZTE Corporation | 38.521-1 | 2856 | - | Rel-18 | F | TEI17_Test, NR_RF_FR1_enh-UEConTest | revised |
| R5-243819 | Updates on 3.1 and 4.3 for Definitions and Specification suffix information | ZTE Corporation | 38.521-1 | 2856 | 1 | Rel-18 | F | TEI17_Test, NR_RF_FR1_enh-UEConTest | agreed |
| R5-243319 | Update of Refsens TC for RedCap UE | China Unicom | 38.521-1 | 2857 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243336 | Addition of suffix information for Carrier Aggregation (CA) with UL MIMO | China Unicom | 38.521-1 | 2858 | - | Rel-18 | F | TEI17_Test, NR_RF_FR1_enh-UEConTest | revised |
| R5-243879 | Addition of suffix information for Carrier Aggregation (CA) with UL MIMO | China Unicom | 38.521-1 | 2858 | 1 | Rel-18 | F | TEI17_Test, NR_RF_FR1_enh-UEConTest | agreed |
| R5-243442 | Update for PC1.5 n39 A-SE | CMCC | 38.521-1 | 2859 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | agreed |
| R5-243447 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | 38.521-1 | 2860 | - | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | revised |
| R5-243650 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | 38.521-1 | 2860 | 1 | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | agreed |
| R5-242260 | Pending UL MIMO update in 38.521-2 | Keysight Technologies | 38.521-2 | 1041 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242263 | FR2c MU - Tx test cases update in 38.521-2 | Keysight Technologies | 38.521-2 | 1042 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243726 | FR2c MU - Tx test cases update in 38.521-2 | Keysight Technologies | 38.521-2 | 1042 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242266 | PC5 FR2 MU - Tx test cases update in 38.521-2 | Keysight Technologies | 38.521-2 | 1043 | - | Rel-18 | F | NR_FR2_FWA_Bn257_Bn258-UEConTest | revised |
| R5-243721 | PC5 FR2 MU - Tx test cases update in 38.521-2 | Keysight Technologies | 38.521-2 | 1043 | 1 | Rel-18 | F | NR_FR2_FWA_Bn257_Bn258-UEConTest | agreed |
| R5-242267 | PC5 FR2 MU - Rx test cases update in 38.521-2 | Keysight Technologies | 38.521-2 | 1044 | - | Rel-18 | F | NR_FR2_FWA_Bn257_Bn258-UEConTest | agreed |
| R5-242268 | PC5 FR2 MU - Annex F update in 38.521-2 | Keysight Technologies | 38.521-2 | 1045 | - | Rel-18 | F | NR_FR2_FWA_Bn257_Bn258-UEConTest | revised |
| R5-243722 | PC5 FR2 MU - Annex F update in 38.521-2 | Keysight Technologies | 38.521-2 | 1045 | 1 | Rel-18 | F | NR_FR2_FWA_Bn257_Bn258-UEConTest | agreed |
| R5-242672 | Additional test case for Enhanced Beam correspondence for PC6 | Samsung | 38.521-2 | 1046 | - | Rel-18 | F | NR_HST_FR2-UEConTest | revised |
| R5-243652 | Additional test case for Enhanced Beam correspondence for PC6 | Samsung | 38.521-2 | 1046 | 1 | Rel-18 | F | NR_HST_FR2-UEConTest | agreed |
| R5-242791 | Introducing framework for Beam Correspondence during Initial Access in IDLE related Tx Power tests | Apple AB Denmark | 38.521-2 | 1047 | - | Rel-18 | F | NR_RF_FR2_req_Ph3-UEConTest | revised |
| R5-243841 | Introducing framework for Beam Correspondence during Initial Access in IDLE related Tx Power tests | Apple AB Denmark | 38.521-2 | 1047 | 1 | Rel-18 | F | NR_RF_FR2_req_Ph3-UEConTest | agreed |
| R5-242999 | Update for FR2c MU | Anritsu | 38.521-2 | 1048 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | revised |
| R5-243831 | Update for FR2c MU | Anritsu | 38.521-2 | 1048 | 1 | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-243014 | Clarification of antenna array assumptions for in-band measurement | Anritsu | 38.521-2 | 1049 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243663 | Clarification of antenna array assumptions for in-band measurement | Anritsu | 38.521-2 | 1049 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243091 | Correction to Rx fast spherical coverage method | Rohde & Schwarz | 38.521-2 | 1050 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243664 | Correction to Rx fast spherical coverage method | Rohde & Schwarz | 38.521-2 | 1050 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243102 | Update of MU for n259 | ROHDE & SCHWARZ | 38.521-2 | 1051 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243727 | Update of MU for n259 | ROHDE & SCHWARZ | 38.521-2 | 1051 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243105 | Update of MU and TT for CA and UL MIMO | ROHDE & SCHWARZ | 38.521-2 | 1052 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243830 | Update of MU and TT for CA and UL MIMO | ROHDE & SCHWARZ | 38.521-2 | 1052 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243302 | Corrections on 6.2.2 and 6.2A.2.0.2 for UE MPR requirements | ZTE Corporation | 38.521-2 | 1053 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243660 | Corrections on 6.2.2 and 6.2A.2.0.2 for UE MPR requirements | ZTE Corporation | 38.521-2 | 1053 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243303 | Corrections on 7.1 for general description to receiver characteristics | ZTE Corporation | 38.521-2 | 1054 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243662 | Corrections on 7.1 for general description to receiver characteristics | ZTE Corporation | 38.521-2 | 1054 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243309 | Clarification of QoQZ Validation Procedure | Keysight Technologies UK Ltd, ETS-Lindgren | 38.521-2 | 1055 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243411 | Updates to FR1 RF phase continuity test | Apple Benelux B.V. | 38.521-2 | 1056 | - | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | withdrawn |
| R5-243412 | Updates to FR2 RF phase continuity test | Apple Benelux B.V. | 38.521-2 | 1057 | - | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | revised |
| R5-243661 | Updates to FR2 RF phase continuity test | Apple Benelux B.V. | 38.521-2 | 1057 | 1 | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | agreed |
| R5-242252 | Addition of UE maximum output power for new PC2 EN-DC combs within FR1 | KDDI Corporation | 38.521-3 | 1756 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242253 | Addition of reference sensitivity for new PC2 EN-DC combs within FR1 | KDDI Corporation | 38.521-3 | 1757 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-243615 | Addition of reference sensitivity for new PC2 EN-DC combs within FR1 | KDDI Corporation | 38.521-3 | 1757 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242254 | Addition and correction of reference sensitivity exceptions for PC2 EN-DC combos | KDDI Corporation | 38.521-3 | 1758 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242269 | PC5 FR2 MU - Updates in 38.521-3 | Keysight Technologies | 38.521-3 | 1759 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243728 | PC5 FR2 MU - Updates in 38.521-3 | Keysight Technologies | 38.521-3 | 1759 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242452 | Update for additional ENDC band configurations | Verizon Spain | 38.521-3 | 1760 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242599 | Core requirements alignment for UE co-existence of Inter-band within FR1 | CAICT | 38.521-3 | 1761 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243665 | Core requirements alignment for UE co-existence of Inter-band within FR1 | CAICT | 38.521-3 | 1761 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242711 | Addition of DC_66A_n66 BCS1, DC_66A-66A_n66A, DC_2A-2A_n66A, DC_66A-66A-66A_n77A, DC_66A-66A-66A_n77(2A) and DC_66A-66A_n2A to Ch5 | WE Certification Oy, AT&T | 38.521-3 | 1762 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242716 | Correction of Reference Sensitivity test for DC_30A-n77A | WE Certification Oy, AT&T, Anritsu | 38.521-3 | 1763 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | revised |
| R5-243670 | Correction of Reference Sensitivity test for DC_30A-n77A | WE Certification Oy, AT&T, Anritsu | 38.521-3 | 1763 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242719 | Addition of new test case 6.4B.1.4_1.4 Frequency Error for Inter-band EN-DC including FR2 for 5 NR CCs | NTT DOCOMO INC. | 38.521-3 | 1764 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | withdrawn |
| R5-242721 | Addition of new test case 6.4B.1.4_1.5 Frequency Error for Inter-band EN-DC including FR2 for 6 NR CCs | NTT DOCOMO INC. | 38.521-3 | 1765 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | withdrawn |
| R5-242722 | Addition of new test case 6.4B.1.4_1.6 Frequency Error for Inter-band EN-DC including FR2 for 7 NR CCs | NTT DOCOMO INC. | 38.521-3 | 1766 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | withdrawn |
| R5-242723 | Addition of new test case 6.4B.1.4_1.7 Frequency Error for Inter-band EN-DC including FR2 for 8 NR CCs | NTT DOCOMO INC. | 38.521-3 | 1767 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | withdrawn |
| R5-242724 | Correct typo for clause 6.5B.3.2.1.4 and 6.5B.3.4.1.4.1 | SGS Wireless | 38.521-3 | 1768 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242785 | Correction to IE name higherPowerLimit used in test cases | Bureau Veritas ADT | 38.521-3 | 1769 | - | Rel-18 | F | TEI17_Test, Power_Limit_CA_DC-UEConTest | agreed |
| R5-242790 | Clarification On Beam Correspondence requirement for EN-DC | Apple AB Denmark | 38.521-3 | 1770 | - | Rel-18 | F | NR_RF_FR2_req_Ph3-UEConTest | agreed |
| R5-242796 | Update to FR1 EN-DC Configurations for n78 and n79 | NTT DOCOMO INC. | 38.521-3 | 1771 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242797 | Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC3 | NTT DOCOMO INC. | 38.521-3 | 1772 | - | Rel-18 | F | TEI15_Test | revised |
| R5-243825 | Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC3 | NTT DOCOMO INC. | 38.521-3 | 1772 | 1 | Rel-18 | F | TEI15_Test | agreed |
| R5-242798 | Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC2 | NTT DOCOMO INC. | 38.521-3 | 1773 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-243828 | Addition of REFSENS for 3CC EN-DC for n78A and n79A in PC2 | NTT DOCOMO INC. | 38.521-3 | 1773 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242855 | Update of 7.3B.2 reference sensitivity for inter-band EN-DC | Huawei, HiSilicon | 38.521-3 | 1774 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243672 | Update of 7.3B.2 reference sensitivity for inter-band EN-DC | Huawei, HiSilicon | 38.521-3 | 1774 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242856 | Update of UE capability name for higher power limit | Huawei, HiSilicon | 38.521-3 | 1775 | - | Rel-18 | F | TEI17_Test, Power_Limit_CA_DC-UEConTest | withdrawn |
| R5-242860 | Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A | NTT DOCOMO INC. | 38.521-3 | 1776 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | withdrawn |
| R5-242928 | Addition of new test case 6.2H.1.3 MOP for inter-band EN-DC with MIMO | Huawei, HiSilicon | 38.521-3 | 1777 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242929 | Addition of new test cases 6.2H.x.3 for inter-band EN-DC with MIMO | Huawei, HiSilicon | 38.521-3 | 1778 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242930 | Addition of new test case 6.2L.1.3 MOP for inter-band EN-DC with TxD | Huawei, HiSilicon | 38.521-3 | 1779 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242931 | Addition of new test cases 6.2L.x.3 for inter-band EN-DC with TxD | Huawei, HiSilicon | 38.521-3 | 1780 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-243000 | Update for FR2c MU | Anritsu | 38.521-3 | 1781 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | withdrawn |
| R5-243015 | Correction to notes for FR1 EN-DC Spurious emission band UE co-existence | Anritsu | 38.521-3 | 1782 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243092 | Correction to Inter-band EN-DC spurious emission requirements | Rohde & Schwarz, Oppo | 38.521-3 | 1783 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243840 | Correction to Inter-band EN-DC spurious emission requirements | Rohde & Schwarz, Oppo | 38.521-3 | 1783 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243106 | Update of status of FR2 CA and UL MIMO test cases | ROHDE & SCHWARZ | 38.521-3 | 1784 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243108 | Addition of power tolerance CA test cases for FR2 | ROHDE & SCHWARZ | 38.521-3 | 1785 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243204 | Updating description in test configuration table of test case 7.3B.2.3_1.1 | Huawei, HiSilicon | 38.521-3 | 1786 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243220 | Addition of MU and TT analysis for 3Tx EN-DC test cases | Huawei, HiSilicon | 38.521-3 | 1787 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-243221 | Addition of general information for 3Tx EN-DC | Huawei, HiSilicon | 38.521-3 | 1788 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-243223 | Addition of delta TIBc for R16 inter-band EN-DC within FR1 | KT Corp. | 38.521-3 | 1789 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243666 | Addition of delta TIBc for R16 inter-band EN-DC within FR1 | KT Corp. | 38.521-3 | 1789 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243225 | Addition of delta RIBc for R16 inter-band EN-DC within FR1 | KT Corp. | 38.521-3 | 1790 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243667 | Addition of delta RIBc for R16 inter-band EN-DC within FR1 | KT Corp. | 38.521-3 | 1790 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243229 | Addition of minimum requirements of reference sensitivity exceptions for R16 inter-band EN-DC within FR1 | KT Corp. | 38.521-3 | 1791 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243668 | Addition of minimum requirements of reference sensitivity exceptions for R16 inter-band EN-DC within FR1 | KT Corp. | 38.521-3 | 1791 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243231 | Update of R16 Configuration for inter-band EN-DC | KT Corp. | 38.521-3 | 1792 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243669 | Update of R16 Configuration for inter-band EN-DC | KT Corp. | 38.521-3 | 1792 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243304 | Corrections on 6.2B.1.3 for UE MOP for configuration DC_38A_n78A | ZTE Corporation | 38.521-3 | 1793 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243671 | Corrections on 6.2B.1.3 for UE MOP for configuration DC_38A_n78A | ZTE Corporation | 38.521-3 | 1793 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242440 | Completion of FDD CQI reporting test cases with inter-cell interference | China Telecom | 38.521-4 | 0826 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-243691 | Completion of FDD CQI reporting test cases with inter-cell interference | China Telecom | 38.521-4 | 0826 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-242441 | Completion of TDD CQI reporting test cases with inter-cell interference | China Telecom | 38.521-4 | 0827 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-243692 | Completion of TDD CQI reporting test cases with inter-cell interference | China Telecom | 38.521-4 | 0827 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-242442 | Addition of MU and TT for CQI reporting test cases with inter-cell interference | China Telecom | 38.521-4 | 0828 | - | Rel-18 | F | NR_demod_enh2-UEConTest | revised |
| R5-243693 | Addition of MU and TT for CQI reporting test cases with inter-cell interference | China Telecom | 38.521-4 | 0828 | 1 | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-242492 | Addition of test case 6.2A.4.1.1, 4Rx FR1 CQI reporting under AWGN for Scell on band with shared spectrum access for CA (2DLCA) | Ericsson | 38.521-4 | 0829 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242493 | Core spec alignment of general clause for CSI reporting requirements | Ericsson | 38.521-4 | 0830 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243694 | Core spec alignment of general clause for CSI reporting requirements | Ericsson | 38.521-4 | 0830 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242668 | Additional annex Channel Profile of FR2 HST-DPS | Samsung | 38.521-4 | 0831 | - | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-242673 | Additional demodulation test case for FR2 high speed train | Samsung | 38.521-4 | 0832 | - | Rel-18 | F | NR_HST_FR2-UEConTest | revised |
| R5-243881 | Additional demodulation test case for FR2 high speed train | Samsung | 38.521-4 | 0832 | 1 | Rel-18 | F | NR_HST_FR2-UEConTest | agreed |
| R5-243016 | Correction to PDSCH Reference Channel for FDD HST | Anritsu | 38.521-4 | 0833 | - | Rel-18 | F | TEI16_Test, NR_HST-UEConTest | revised |
| R5-243697 | Correction to PDSCH Reference Channel for FDD HST | Anritsu | 38.521-4 | 0833 | 1 | Rel-18 | F | TEI16_Test, NR_HST-UEConTest | agreed |
| R5-243017 | Correction to PUCCH Resource ID in 2Rx FR1 RI reporting | Anritsu | 38.521-4 | 0834 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243695 | Correction to PUCCH Resource ID in 2Rx FR1 RI reporting | Anritsu | 38.521-4 | 0834 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243024 | Correction to test parameters in FR1 HST DPS | Anritsu | 38.521-4 | 0835 | - | Rel-18 | F | TEI16_Test, NR_HST-UEConTest | revised |
| R5-243913 | Correction to test parameters in FR1 HST DPS | Anritsu | 38.521-4 | 0835 | 1 | Rel-18 | F | TEI16_Test, NR_HST-UEConTest | agreed |
| R5-243089 | Correction to PDSCH TDD Test Cases | Rohde & Schwarz | 38.521-4 | 0836 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243093 | Correction to URLLC message exceptions | Rohde & Schwarz | 38.521-4 | 0837 | - | Rel-18 | F | TEI16_Test, NR_L1enh_URLLC-UEConTest | agreed |
| R5-243110 | Update of MU and TT for FR1 CSI reporting | ROHDE & SCHWARZ | 38.521-4 | 0838 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243111 | Editorial correction of UE capability in PDSCH repetition test cases | ROHDE & SCHWARZ | 38.521-4 | 0839 | - | Rel-18 | F | TEI16_Test, NR_L1enh_URLLC-UEConTest | agreed |
| R5-243150 | Update to HST scheme A and B test cases | Huawei, HiSilicon | 38.521-4 | 0840 | - | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-243151 | Adding RMC for feMIMO test cases | Huawei, HiSilicon, Samsung | 38.521-4 | 0841 | - | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-243338 | Update for PMI TypeII codebook test cases | Keysight Technologies | 38.521-4 | 0842 | - | Rel-18 | F | TEI16_Test, NR_perf_enh-UEConTest | agreed |
| R5-243339 | Update mTT for missing DL RMC | Keysight Technologies | 38.521-4 | 0843 | - | Rel-18 | F | TEI16_Test | revised |
| R5-243698 | Update mTT for missing DL RMC | Keysight Technologies | 38.521-4 | 0843 | 1 | Rel-18 | F | TEI16_Test | agreed |
| R5-243432 | Addition of NSA SDR test case for DL 1024QAM | Qualcomm Inc | 38.521-4 | 0844 | - | Rel-18 | F | TEI17_Test, NR_DL1024QAM_FR1-UEConTest | revised |
| R5-243696 | Addition of NSA SDR test case for DL 1024QAM | Qualcomm Inc | 38.521-4 | 0844 | 1 | Rel-18 | F | TEI17_Test, NR_DL1024QAM_FR1-UEConTest | agreed |
| R5-243434 | Update to inter-cell CRS interference test case | Qualcomm Inc | 38.521-4 | 0845 | - | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-243435 | Corrections to the FRC Table A.3.2.2.2-29 | Qualcomm Inc | 38.521-4 | 0846 | - | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-243436 | Corrections to Table G.1.5-1 for entries corresponding to FR1.15-1 | Qualcomm Inc | 38.521-4 | 0847 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243699 | Corrections to Table G.1.5-1 for entries corresponding to FR1.15-1 | Qualcomm Inc | 38.521-4 | 0847 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242305 | Removal of technical content in TS 38.521-5 v17.0.0 and substitution with pointer to the next Release | ETSI Secretariat | 38.521-5 | 0037 | - | Rel-17 | F | TEI17_Test | agreed |
| R5-242500 | Introduction of new NR NTN test case - 6.5.1 Occupied bandwidth | Keysight Technologies | 38.521-5 | 0038 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242501 | Introduction of new NR NTN test case - 6.4.2.1 Error Vector Magnitude | Keysight Technologies | 38.521-5 | 0039 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242502 | Introduction of new NR NTN test case - 6.4.2.1a Error Vector Magnitude including symbols with transient period | Keysight Technologies | 38.521-5 | 0040 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243896 | Introduction of new NR NTN test case - 6.4.2.1a Error Vector Magnitude including symbols with transient period | Keysight Technologies | 38.521-5 | 0040 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242503 | Introduction of new NR NTN test case - 6.4.2.2 Carrier leakage | Keysight Technologies | 38.521-5 | 0041 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243673 | Introduction of new NR NTN test case - 6.4.2.2 Carrier leakage | Keysight Technologies | 38.521-5 | 0041 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242504 | Introduction of new NR NTN test case - 6.4.2.3 In-band emissions | Keysight Technologies | 38.521-5 | 0042 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243674 | Introduction of new NR NTN test case - 6.4.2.3 In-band emissions | Keysight Technologies | 38.521-5 | 0042 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242505 | Introduction of new NR NTN test case - 6.4.2.4 EVM equalizer spectrum flatness | Keysight Technologies | 38.521-5 | 0043 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243675 | Introduction of new NR NTN test case - 6.4.2.4 EVM equalizer spectrum flatness | Keysight Technologies | 38.521-5 | 0043 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242506 | Introduction of new NR NTN test case - 6.4.2.5 EVM equalizer spectrum flatness for Pi2 BPSK | Keysight Technologies | 38.521-5 | 0044 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242507 | Update of NR NTN test case - 6.2.1 UE maximum output power | Keysight Technologies | 38.521-5 | 0045 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243676 | Update of NR NTN test case - 6.2.1 UE maximum output power | Keysight Technologies | 38.521-5 | 0045 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242508 | Update of NR NTN test case - 6.2.2 UE maximum output power reduction | Keysight Technologies | 38.521-5 | 0046 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242509 | Update of NR NTN test case - 6.2.3 UE additional maximum output power reduction | Keysight Technologies | 38.521-5 | 0047 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243677 | Update of NR NTN test case - 6.2.3 UE additional maximum output power reduction | Keysight Technologies | 38.521-5 | 0047 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242510 | Update of NR NTN test case - 6.2.4 Configured transmitted power | Keysight Technologies | 38.521-5 | 0048 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242511 | Update MU and TT for NR NTN test cases | Keysight Technologies | 38.521-5 | 0049 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243678 | Update MU and TT for NR NTN test cases | Keysight Technologies | 38.521-5 | 0049 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242946 | Updates to additional Spurious emissions TC 6.5.3.3 | Qualcomm Tech. Netherlands B.V | 38.521-5 | 0050 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-242947 | Updates to additional spurious emissions TC 6.5.3.3 | Qualcomm Tech. Netherlands B.V | 38.521-5 | 0051 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242951 | Adding new NTN test case 6.3.4 Power control | Qualcomm Tech. Netherlands B.V | 38.521-5 | 0052 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243679 | Adding new NTN test case 6.3.4 Power control | Qualcomm Tech. Netherlands B.V | 38.521-5 | 0052 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243288 | Corrections on 5.1 for NTN FR1 frequency range designation | ZTE Corporation | 38.521-5 | 0053 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243289 | Corrections on 6.2.3 for NTN UE additional maximum output power reduction | ZTE Corporation | 38.521-5 | 0054 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243290 | Corrections on 6.5.3.3 for NTN UE additional spurious emissions | ZTE Corporation | 38.521-5 | 0055 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243291 | Updates on Annex for satellite access conformance test | ZTE Corporation | 38.521-5 | 0056 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243680 | Updates on Annex for satellite access conformance test | ZTE Corporation | 38.521-5 | 0056 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243415 | Annex F updates for NTN RF test on Tx on-off time mask | Apple Benelux B.V. | 38.521-5 | 0057 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243681 | Annex F updates for NTN RF test on Tx on-off time mask | Apple Benelux B.V. | 38.521-5 | 0057 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243416 | Annex F updates for NTN RF test on Spectrum emission mask | Apple Benelux B.V. | 38.521-5 | 0058 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243814 | Annex F updates for NTN RF test on Spectrum emission mask | Apple Benelux B.V. | 38.521-5 | 0058 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243417 | Annex F updates for NTN RF test on ACLR | Apple Benelux B.V. | 38.521-5 | 0059 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243419 | Common clause updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0060 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243682 | Common clause updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0060 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243987 | Common clause updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0060 | 2 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-243420 | MOP requirement updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0061 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243683 | MOP requirement updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0061 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-243421 | A-MPR requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0062 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243684 | A-MPR requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0062 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-243422 | REFSENS requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0063 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-243423 | IBB requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0064 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | revised |
| R5-243685 | IBB requirements updates related to NTN band n254 | Apple Benelux B.V., Keysight | 38.521-5 | 0064 | 1 | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-243441 | Update to nrn-ntn pdsch demod test case to update min test time | Qualcomm Inc | 38.521-5 | 0065 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243899 | Update to nrn-ntn pdsch demod test case to update min test time | Qualcomm Inc | 38.521-5 | 0065 | 1 | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242306 | Removal of technical content in TS 38.522 v15.3.0 and substitution with pointer to the next Release | ETSI Secretariat | 38.522 | 0407 | - | Rel-15 | F | TEI15_Test | agreed |
| R5-242443 | Correction of test applicability rule for CQI reporting test cases with inter-cell interference | China Telecom | 38.522 | 0408 | - | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-242494 | Addition of applicability for new NR-U test cases | Ericsson | 38.522 | 0409 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242512 | Update applicability for NR NTN test cases | Keysight Technologies | 38.522 | 0410 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242523 | Correction on applicability for PC1.5 in Rx tests | Keysight Technologies, Huawei | 38.522 | 0411 | - | Rel-18 | F | TEI17_Test, NR_RF_TxD-UEConTest | agreed |
| R5-242578 | Correction to TC titles in applicability table of RRM event triggered reporting TCs with measurement gaps | Anritsu | 38.522 | 0412 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-243923 | Correction to TC titles in applicability table of RRM event triggered reporting TCs with measurement gaps | Anritsu | 38.522 | 0412 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-242610 | Editorial Correction of FR2 feMIMO test case | Nokia | 38.522 | 0413 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-243715 | Editorial Correction of FR2 feMIMO test case | Nokia | 38.522 | 0413 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-242645 | Applicabilty correction of RRM enhancements test cases | Ericsson | 38.522 | 0414 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243718 | Applicabilty correction of RRM enhancements test cases | Ericsson | 38.522 | 0414 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242669 | Additional applicability for Inter-cell SSB based L1-RSRP measurements | Samsung | 38.522 | 0415 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-243716 | Additional applicability for Inter-cell SSB based L1-RSRP measurements | Samsung | 38.522 | 0415 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-242674 | Additional applicability for HST FR2 test cases | Samsung | 38.522 | 0416 | - | Rel-18 | F | NR_HST_FR2-UEConTest | revised |
| R5-243717 | Additional applicability for HST FR2 test cases | Samsung | 38.522 | 0416 | 1 | Rel-18 | F | NR_HST_FR2-UEConTest | agreed |
| R5-242694 | Addition of test applicability for Rel-16 RRM EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.522 | 0417 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243719 | Addition of test applicability for Rel-16 RRM EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.522 | 0417 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242725 | Correction condition C111 for 38.521-1 TC6.4.2.5 Rel-16 | SGS Wireless | 38.522 | 0418 | - | Rel-18 | F | NR_feMIMO-UEConTest | withdrawn |
| R5-242726 | Correction condition F011 for 38.533 TC16.4.1.1 | SGS Wireless | 38.522 | 0419 | - | Rel-18 | F | TEI17_Test, NR_redcap_enh_plus_CT1-UEConTest | revised |
| R5-243910 | Correction condition F011 for 38.533 TC16.4.1.1 | SGS Wireless | 38.522 | 0419 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_enh_plus_CT1-UEConTest | agreed |
| R5-242789 | Correction to applicability of 5G test cases | Bureau Veritas ADT, Keysight | 38.522 | 0420 | - | Rel-18 | F | TEI15_Test | revised |
| R5-243720 | Correction to applicability of 5G test cases | Bureau Veritas ADT, Keysight | 38.522 | 0420 | 1 | Rel-18 | F | TEI15_Test | revised |
| R5-244010 | Correction to applicability of 5G test cases | Bureau Veritas ADT, Keysight | 38.522 | 0420 | 2 | Rel-18 | F | TEI15_Test | agreed |
| R5-242846 | Correction to Test Bands Selection Condition for Performance test case 5.3.2.2.5 | TTA | 38.522 | 0421 | - | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-242866 | Addition of applicability for MCE RRM test cases | Huawei, HiSilicon | 38.522 | 0422 | - | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243914 | Addition of applicability for MCE RRM test cases | Huawei, HiSilicon | 38.522 | 0422 | 1 | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-242877 | Correction to applicability for RedCap RRM test cases | Huawei, HiSilicon | 38.522 | 0423 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243915 | Correction to applicability for RedCap RRM test cases | Huawei, HiSilicon | 38.522 | 0423 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-244011 | Correction to applicability for RedCap RRM test cases | Huawei, HiSilicon | 38.522 | 0423 | 2 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242932 | Addition of applicabilities for 3Tx NR CA and EN-DC test cases | Huawei, HiSilicon | 38.522 | 0424 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242984 | Addition of missing applicability to RedCap SS-RSRP test cases | Rohde & Schwarz | 38.522 | 0425 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243041 | Update to R16 NR CADC configuration test cases applicability | CMCC | 38.522 | 0426 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243714 | Update to R16 NR CADC configuration test cases applicability | CMCC | 38.522 | 0426 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | withdrawn |
| R5-243042 | Update to R17 NR CADC configuration test cases applicability | CMCC | 38.522 | 0427 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | withdrawn |
| R5-243043 | Update to R18 NR CADC configuration test cases applicability | CMCC | 38.522 | 0428 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R18-UEConTest | withdrawn |
| R5-243107 | Update of applicability of FR2 CA and MIMO test cases | ROHDE & SCHWARZ | 38.522 | 0429 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243729 | Update of applicability of FR2 CA and MIMO test cases | ROHDE & SCHWARZ | 38.522 | 0429 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243109 | Applicability for FR2 NSA power tolerance CA test cases | ROHDE & SCHWARZ | 38.522 | 0430 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243129 | Cleaning up test applicability for ULFPTx | Huawei, HiSilicon | 38.522 | 0431 | - | Rel-18 | F | TEI16_Test, NR_eMIMO-UEConTest | agreed |
| R5-243133 | Updating applicability for Rx test cases | Huawei, HiSilicon | 38.522 | 0432 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-243144 | Update to test applicability of V2X test cases | Huawei, HiSilicon | 38.522 | 0433 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243152 | Update to applicability of FeMIMO test cases | Huawei, HiSilicon | 38.522 | 0434 | - | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-243226 | Update to applicability of V2X test cases | Huawei, HiSilicon | 38.522 | 0435 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | withdrawn |
| R5-243292 | Corrections on 4.0 for NTN tested bands selection criteria | ZTE Corporation | 38.522 | 0436 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243394 | Addition of applicability table for NR NTN RRM tests | Qualcomm France | 38.522 | 0437 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243395 | Update to NR-U test applicability | Qualcomm France | 38.522 | 0438 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243396 | Applicability correction to gap-based measurement tests for RedCap | Qualcomm France | 38.522 | 0439 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243397 | Applicability correction to gap-based measurement tests | Qualcomm France | 38.522 | 0440 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243414 | Applicability updates for Phase continuity tests | Apple Benelux B.V. | 38.522 | 0441 | - | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | revised |
| R5-243813 | Applicability updates for Phase continuity tests | Apple Benelux B.V. | 38.522 | 0441 | 1 | Rel-18 | F | TEI17_Test, NR_cov_enh-UEConTest | agreed |
| R5-243437 | Applicability spec update for inter-cell CRS interference test case | Qualcomm Inc | 38.522 | 0442 | - | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-243439 | Appllicability spec update to allow skipping 2DLCA and 3DCLA PDSCH Demodulation test cases | Qualcomm Inc | 38.522 | 0443 | - | Rel-18 | F | TEI16_Test, NR_perf_enh-UEConTest | agreed |
| R5-243440 | Applicability spec update for NR-NTN test cases | Qualcomm Inc | 38.522 | 0444 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242134 | Correction to NR NTN TC 8.1.3.1.26 | MediaTek Inc. | 38.523-1 | 4310 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242135 | Correction to NR NTN TC 9.4.1.1 | MediaTek Inc. | 38.523-1 | 4311 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243556 | Correction to NR NTN TC 9.4.1.1 | MediaTek Inc. | 38.523-1 | 4311 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242136 | Correction to NR NTN TC 9.4.1.2 | MediaTek Inc. | 38.523-1 | 4312 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243557 | Correction to NR NTN TC 9.4.1.2 | MediaTek Inc. | 38.523-1 | 4312 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242144 | Correction to Idle mode test cases | MediaTek Inc. | 38.523-1 | 4313 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242145 | Correction to eCPSOR TC 6.3.2.2 | MediaTek Inc., Anritsu, ROHDE & SCHWARZ | 38.523-1 | 4314 | - | Rel-17 | F | TEI17_Test, eCPSOR_CON-UEConTest | agreed |
| R5-242146 | Correction to eCPSOR TC 6.3.2.4 | MediaTek Inc., Keysight | 38.523-1 | 4315 | - | Rel-17 | F | TEI17_Test, eCPSOR_CON-UEConTest | agreed |
| R5-242147 | Correction to eCPSOR TC 6.3.2.5 | MediaTek Inc., Anritsu | 38.523-1 | 4316 | - | Rel-17 | F | TEI17_Test, eCPSOR_CON-UEConTest | agreed |
| R5-242148 | Correction to NR slice TC 6.4.2.3 | MediaTek Inc. | 38.523-1 | 4317 | - | Rel-17 | F | TEI17_Test, NR_slice-UEConTest | agreed |
| R5-242149 | Correction to CAG TC 6.5.2.1 | MediaTek Inc. | 38.523-1 | 4318 | - | Rel-17 | F | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest | agreed |
| R5-242150 | Correction to UPIP TC 7.1.3.2.6 | MediaTek Inc. | 38.523-1 | 4319 | - | Rel-17 | F | TEI17_Test, UPIP_SEC_LTE-RAN-UEConTest | agreed |
| R5-242151 | Correction to PDCP TC 7.1.3.4.1 | MediaTek Inc. | 38.523-1 | 4320 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243498 | Correction to PDCP TC 7.1.3.4.1 | MediaTek Inc. | 38.523-1 | 4320 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242152 | Correction to PDCP TC 7.1.3.4.2 | MediaTek Inc. | 38.523-1 | 4321 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243499 | Correction to PDCP TC 7.1.3.4.2 | MediaTek Inc. | 38.523-1 | 4321 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242153 | Correction to RRC TC 8.1.1.2.4 | MediaTek Inc. | 38.523-1 | 4322 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-242154 | Correction to NR Idle measurement TC 8.1.5.11.1 | MediaTek Inc. | 38.523-1 | 4323 | - | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | revised |
| R5-243503 | Correction to NR Idle measurement TC 8.1.5.11.1 | MediaTek Inc. | 38.523-1 | 4323 | 1 | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-242155 | Correction to NR Idle measurement TC 8.1.5.11.2 | MediaTek Inc. | 38.523-1 | 4324 | - | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-242156 | Correction to NR Inactive measurement TC 8.1.5.11.3 | MediaTek Inc. | 38.523-1 | 4325 | - | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-242157 | Correction to NR Inactive measurement TC 8.1.5.11.4 | MediaTek Inc. | 38.523-1 | 4326 | - | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-242158 | Correction to MDT enh. TC 8.1.6.1.2.14 | MediaTek Inc. | 38.523-1 | 4327 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242159 | Correction to MDT enh. TC 8.1.6.1.3.8 | MediaTek Inc. | 38.523-1 | 4328 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-243507 | Correction to MDT enh. TC 8.1.6.1.3.8 | MediaTek Inc. | 38.523-1 | 4328 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242160 | Correction to MDT enh. TC 8.1.6.1.3.9 | MediaTek Inc. | 38.523-1 | 4329 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-243508 | Correction to MDT enh. TC 8.1.6.1.3.9 | MediaTek Inc. | 38.523-1 | 4329 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242161 | Correction to MDT enh. TC 8.1.6.1.3.10 | MediaTek Inc. | 38.523-1 | 4330 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-243509 | Correction to MDT enh. TC 8.1.6.1.3.10 | MediaTek Inc. | 38.523-1 | 4330 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242162 | Correction to MDT enh. TC 8.1.6.1.3.11 | MediaTek Inc. | 38.523-1 | 4331 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-243510 | Correction to MDT enh. TC 8.1.6.1.3.11 | MediaTek Inc. | 38.523-1 | 4331 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242163 | Correction to MDT enh. TC 8.1.6.1.4.9 | MediaTek Inc. | 38.523-1 | 4332 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242164 | Correction to MDT enh. TC 8.1.6.4.2 | MediaTek Inc. | 38.523-1 | 4333 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242165 | Correction to MDT enh. TC 8.1.6.4.3 | MediaTek Inc. | 38.523-1 | 4334 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242166 | Correction to MDT enh. TC 8.1.6.4.4 | MediaTek Inc. | 38.523-1 | 4335 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-242167 | Correction to NE-DC TC 8.2.2.8.3 | MediaTek Inc. | 38.523-1 | 4336 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-242168 | Correction to NR CA TC 8.2.4.2.1.1 | MediaTek Inc. | 38.523-1 | 4337 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242169 | Correction to NE-DC TC 8.2.6.3.5 and 8.2.6.3.6 | MediaTek Inc. | 38.523-1 | 4338 | - | Rel-17 | F | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest | agreed |
| R5-242170 | Correction to UPIP TC 8.2.6.4.1 | MediaTek Inc. | 38.523-1 | 4339 | - | Rel-17 | F | TEI17_Test, UPIP_SEC_LTE-RAN-UEConTest | agreed |
| R5-242171 | Correction to NSAC TC 10.1.8.4 | MediaTek Inc. | 38.523-1 | 4340 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-242172 | Correction to UAC TC 11.3.5 | MediaTek Inc. | 38.523-1 | 4341 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242173 | Correction to UAC TC 11.3.12 | MediaTek Inc. | 38.523-1 | 4342 | - | Rel-17 | F | TEI15_Test | revised |
| R5-243515 | Correction to UAC TC 11.3.12 | MediaTek Inc. | 38.523-1 | 4342 | 1 | Rel-17 | F | TEI15_Test | agreed |
| R5-242258 | Correction to NR CAG Test case 6.5.2.1 | Anritsu EMEA Ltd | 38.523-1 | 4343 | - | Rel-17 | F | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest | withdrawn |
| R5-242270 | Corrrection to NR5GC eCPSOR test case 6.3.2.4 | Keysight Technologies UK | 38.523-1 | 4344 | - | Rel-17 | F | TEI17_Test, eCPSOR_CON-UEConTest | withdrawn |
| R5-242271 | Addition to R17 ATSSS testcase 10.5.2.1 | MediaTek Inc. | 38.523-1 | 4345 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-243964 | Addition to R17 ATSSS testcase 10.5.2.1 | MediaTek Inc. | 38.523-1 | 4345 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-242272 | Addition to R17 ATSSS testcase 10.5.2.2 | MediaTek Inc. | 38.523-1 | 4346 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-243965 | Addition to R17 ATSSS testcase 10.5.2.2 | MediaTek Inc. | 38.523-1 | 4346 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-242274 | Update 2-step RACH TC 7.1.1.1.8 for HD-FDD UE-PRACH | MediaTek Inc. | 38.523-1 | 4347 | - | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242275 | Update 2-step RACH TC 7.1.1.1.9 for HD-FDD UE-PRACH | MediaTek Inc. | 38.523-1 | 4348 | - | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242276 | Update URLLC TC 7.1.1.4.1.5 to test RedCap UE | MediaTek Inc. | 38.523-1 | 4349 | - | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242277 | Update URLLC TC 7.1.1.4.2.6 to test RedCap UE | MediaTek Inc. | 38.523-1 | 4350 | - | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242278 | Update coverage enhancement TC 7.1.1.4.2.7 to test RedCap UE | MediaTek Inc. | 38.523-1 | 4351 | - | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242280 | Correction to R17 ATSSS TC 11.9.1 | MediaTek Inc. | 38.523-1 | 4352 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-242281 | Correction to R17 ATSSS TC 11.9.2 | MediaTek Inc. | 38.523-1 | 4353 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-242314 | Updates to 5G V2X test case 12.1.7.1 | MCC TF160 | 38.523-1 | 4354 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242315 | Updates to 5G V2X test case 12.2.9.1 | MCC TF160 | 38.523-1 | 4355 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242316 | Corrections to SDT TC 7.1.1.13.1 | MCC TF160, Lenovo | 38.523-1 | 4356 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-242317 | Corrections to SDT TC 7.1.1.13.2 | MCC TF160, Lenovo | 38.523-1 | 4357 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-242318 | Corrections to SDT TC 7.1.1.13.3 | MCC TF160, Lenovo | 38.523-1 | 4358 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-242326 | Updates to eDRX TC 11.7.2 | MCC TF160 | 38.523-1 | 4359 | - | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242327 | Updates to NR RRC test case 8.1.5.1.1 | MCC TF160 | 38.523-1 | 4360 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242328 | Updates to EN-DC RRC test case 8.2.1.1.1 | MCC TF160 | 38.523-1 | 4361 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242329 | Updates to NE-DC RRC test case 8.2.1.1.2 | MCC TF160 | 38.523-1 | 4362 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242462 | Update to R17 ATSSS test case 11.9.1 | China Telecom | 38.523-1 | 4363 | - | Rel-16 | F | ATSSS_Ph2-UEConTest | withdrawn |
| R5-242464 | Update to R17 ATSSS test case 11.9.2 | China Telecom, MediaTek | 38.523-1 | 4364 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-243522 | Update to R17 ATSSS test case 11.9.2 | China Telecom, MediaTek | 38.523-1 | 4364 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-242487 | Correction to NR5GC testcase 9.1.5.1.4 | ROHDE & SCHWARZ, MediaTek, Huawei, Hisilicon | 38.523-1 | 4365 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242489 | Correction to NR5GC testcase 6.3.1.10 | ROHDE & SCHWARZ | 38.523-1 | 4366 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243587 | Correction to NR5GC testcase 6.3.1.10 | ROHDE & SCHWARZ | 38.523-1 | 4366 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242496 | Correction to NR5GC testcase 11.3.12 | ROHDE & SCHWARZ | 38.523-1 | 4367 | - | Rel-17 | F | TEI15_Test | revised |
| R5-243592 | Correction to NR5GC testcase 11.3.12 | ROHDE & SCHWARZ | 38.523-1 | 4367 | 1 | Rel-17 | F | TEI15_Test | agreed |
| R5-242527 | Correction of UPIP TC 8.2.6.4.4 | ZTE Corporation | 38.523-1 | 4368 | - | Rel-17 | F | TEI17_Test, UPIP_SEC_LTE-RAN-UEConTest | agreed |
| R5-242530 | Correction to NR testcase 7.1.1.3.8.1 | ROHDE & SCHWARZ | 38.523-1 | 4369 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-242531 | Correction to NR5GC testcase 8.1.5.8.1 | ROHDE & SCHWARZ | 38.523-1 | 4370 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242539 | Updates to NR RRC test case 8.1.1.2.4 | MCC TF160, MediaTek | 38.523-1 | 4371 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243501 | Updates to NR RRC test case 8.1.1.2.4 | MCC TF160, MediaTek | 38.523-1 | 4371 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242542 | Introduction of new TC NR-NTN - TC TAI(s) not allowed / forbidden TAI(s) | Qualcomm Technologies Ireland | 38.523-1 | 4372 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243558 | Introduction of new TC NR-NTN - TC TAI(s) not allowed / forbidden TAI(s) | Qualcomm Technologies Ireland | 38.523-1 | 4372 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242547 | Introduction of new TC NR-NTN - ntn-UlSyncValidityDuration, T430 Expiry | Qualcomm Technologies Ireland | 38.523-1 | 4373 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243559 | Introduction of new TC NR-NTN - ntn-UlSyncValidityDuration, T430 Expiry | Qualcomm Technologies Ireland | 38.523-1 | 4373 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242548 | Updates to NTN test cases with missing NGSO configuration | Qualcomm Technologies Ireland | 38.523-1 | 4374 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243560 | Updates to NTN test cases with missing NGSO configuration | Qualcomm Technologies Ireland | 38.523-1 | 4374 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242549 | Addition of new NR-NTN conditional handover condEventA4 test case | Qualcomm Technologies Ireland | 38.523-1 | 4375 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243598 | Addition of new NR-NTN conditional handover condEventA4 test case | Qualcomm Technologies Ireland | 38.523-1 | 4375 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242550 | Addition of new NR NTN conditional handover CondEventD1 test case | Qualcomm Technologies Ireland | 38.523-1 | 4376 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243561 | Addition of new NR NTN conditional handover CondEventD1 test case | Qualcomm Technologies Ireland | 38.523-1 | 4376 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242551 | Addition of new NR NTN conditional handover CondEventT1 test case | Qualcomm Technologies Ireland | 38.523-1 | 4377 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243599 | Addition of new NR NTN conditional handover CondEventT1 test case | Qualcomm Technologies Ireland | 38.523-1 | 4377 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242561 | Corrections to conditional handover test case 8.1.4.4.1 | Qualcomm Incorporated | 38.523-1 | 4378 | - | Rel-17 | F | TEI16_Test, NR_Mob_enh-UEConTest | revised |
| R5-243502 | Corrections to conditional handover test case 8.1.4.4.1 | Qualcomm Incorporated | 38.523-1 | 4378 | 1 | Rel-17 | F | TEI16_Test, NR_Mob_enh-UEConTest | agreed |
| R5-242562 | Corrections to BWP test case 7.1.1.8.1 | Qualcomm Incorporated | 38.523-1 | 4379 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243966 | Corrections to BWP test case 7.1.1.8.1 | Qualcomm Incorporated | 38.523-1 | 4379 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-244020 | Corrections to BWP test case 7.1.1.8.1 | Qualcomm Incorporated | 38.523-1 | 4379 | 2 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242563 | Addition of E-UTRA to NR MPS priority Indication test case | Qualcomm Incorporated | 38.523-1 | 4380 | - | Rel-17 | F | TEI16_Test | revised |
| R5-243504 | Addition of E-UTRA to NR MPS priority Indication test case | Qualcomm Incorporated | 38.523-1 | 4380 | 1 | Rel-17 | F | TEI16_Test | agreed |
| R5-242582 | Editorial correction for V2X TC 12.1.2.1 | ZTE Corporation | 38.523-1 | 4381 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242584 | Correction for V2X TC 12.1.2.2 | ZTE Corporation | 38.523-1 | 4382 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242585 | Correction for V2X TC 12.1.3.2 | ZTE Corporation | 38.523-1 | 4383 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242592 | Correction to NR5GC testcase 6.1.2.13 | ROHDE & SCHWARZ | 38.523-1 | 4384 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243497 | Correction to NR5GC testcase 6.1.2.13 | ROHDE & SCHWARZ | 38.523-1 | 4384 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242594 | Correction for MBS TC 14.2.4.2.1 | ZTE Corporation | 38.523-1 | 4385 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-243516 | Correction for MBS TC 14.2.4.2.1 | ZTE Corporation | 38.523-1 | 4385 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-242595 | Correction for MBS TC 14.2.4.3.2 | ZTE Corporation | 38.523-1 | 4386 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-243517 | Correction for MBS TC 14.2.4.3.2 | ZTE Corporation | 38.523-1 | 4386 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-242596 | Editorial correction for MBS TC of 14.2.4.1 | ZTE Corporation | 38.523-1 | 4387 | - | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | revised |
| R5-243518 | Editorial correction for MBS TC of 14.2.4.1 | ZTE Corporation | 38.523-1 | 4387 | 1 | Rel-17 | F | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest | agreed |
| R5-242675 | Update UE pre-test configurations for test case 6.5.3.1 | China Telecom | 38.523-1 | 4388 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242676 | Update test case 6.5.3.2 | China Telecom | 38.523-1 | 4389 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242677 | Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4390 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-243540 | Addition of new test case 6.3.3.1 for Steering of Roaming in Rel-17 eNPN | China Telecom | 38.523-1 | 4390 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242678 | Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 Enpn | China Telecom | 38.523-1 | 4391 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-243541 | Addition of new test case 6.3.3.2 for Steering of Roaming in Rel-17 Enpn | China Telecom | 38.523-1 | 4391 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242679 | Addition of new test case 6.3.3.3 for Steering of Roaming in Rel-17 Enpn | China Telecom | 38.523-1 | 4392 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242680 | Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 Enpn | China Telecom | 38.523-1 | 4393 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-243951 | Addition of new test case 6.3.3.4 for Steering of Roaming in Rel-17 Enpn | China Telecom | 38.523-1 | 4393 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242681 | Addition of new test case 9.1.11.4 for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4394 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242682 | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4395 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-243542 | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN | China Telecom | 38.523-1 | 4395 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242683 | Addition of new test case 9.1.11.7 for Rel-17 eNPN | China Telecom | 38.523-1 | 4396 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242703 | Update to R17 ATSSS test case 11.9.1 | China Telecom, MediaTek | 38.523-1 | 4397 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-243553 | Update to R17 ATSSS test case 11.9.1 | China Telecom, MediaTek | 38.523-1 | 4397 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-242734 | Addition of ING_5GS test case 9.3.1.5 | Qualcomm Incorporated | 38.523-1 | 4398 | - | Rel-17 | F | ING_5GS-UEConTest | agreed |
| R5-242735 | Void ING_5GS test case 11.1.10 | Qualcomm Incorporated | 38.523-1 | 4399 | - | Rel-17 | F | ING_5GS-UEConTest | agreed |
| R5-242740 | Correction to NSAC test case 9.1.12.3 | Anritsu EMEA Ltd | 38.523-1 | 4400 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-242741 | Correction to NR RRC UL segmentation test case 8.1.5.9.1 | Qualcomm Incorporated | 38.523-1 | 4401 | - | Rel-17 | F | TEI16_Test, RACS-UEConTest | revised |
| R5-243505 | Correction to NR RRC UL segmentation test case 8.1.5.9.1 | Qualcomm Incorporated | 38.523-1 | 4401 | 1 | Rel-17 | F | TEI16_Test, RACS-UEConTest | agreed |
| R5-242757 | Correction to NR NTN TA report test case 7.1.1.10.4 | Qualcomm Incorporated | 38.523-1 | 4402 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242761 | Correction to eNS test case 9.3.1.4 | Anritsu EMEA Ltd, MediaTek | 38.523-1 | 4403 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | revised |
| R5-243514 | Correction to eNS test case 9.3.1.4 | Anritsu EMEA Ltd, MediaTek | 38.523-1 | 4403 | 1 | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-242762 | Correction to RACS test case 9.1.9.7 | Anritsu EMEA Ltd | 38.523-1 | 4404 | - | Rel-17 | F | TEI16_Test, RACS-UEConTest | withdrawn |
| R5-242767 | Addition of NR NTN cell reselection test case 6.1.2.18a | Qualcomm Incorporated | 38.523-1 | 4405 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243562 | Addition of NR NTN cell reselection test case 6.1.2.18a | Qualcomm Incorporated | 38.523-1 | 4405 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242768 | Addition of NR NTN cell reselection test case 6.1.2.18b | Qualcomm Incorporated | 38.523-1 | 4406 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243563 | Addition of NR NTN cell reselection test case 6.1.2.18b | Qualcomm Incorporated | 38.523-1 | 4406 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242769 | Addition of NR NTN MAC test case 7.1.1.5.6 | Qualcomm Incorporated | 38.523-1 | 4407 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242770 | Addition of NR NTN RLC t-reassembly timer test case 7.1.2.3.9a | Qualcomm Incorporated | 38.523-1 | 4408 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243564 | Addition of NR NTN RLC t-reassembly timer test case 7.1.2.3.9a | Qualcomm Incorporated | 38.523-1 | 4408 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242773 | Correction to eNS test case 10.1.8.1 | Anritsu EMEA Ltd, MediaTek | 38.523-1 | 4409 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-242774 | Correction to RACS test case 9.1.9.7 | Anritsu EMEA Ltd | 38.523-1 | 4410 | - | Rel-17 | F | TEI17_Test, RACS-UEConTest | withdrawn |
| R5-242793 | Corrrection to NR5GC CAG test case 6.5.2.1 | Keysight Technologies UK | 38.523-1 | 4411 | - | Rel-17 | F | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest | revised |
| R5-243588 | Corrrection to NR5GC CAG test case 6.5.2.1 | Keysight Technologies UK | 38.523-1 | 4411 | 1 | Rel-17 | F | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest | agreed |
| R5-242812 | Correction of RRC IDLE TC 6.1.2.23-MFBI | Huawei, HiSilicon | 38.523-1 | 4412 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242813 | Correction of MAC TC 7.1.1.1.4-Beam Failure | Huawei, HiSilicon | 38.523-1 | 4413 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243589 | Correction of MAC TC 7.1.1.1.4-Beam Failure | Huawei, HiSilicon | 38.523-1 | 4413 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242815 | Correction to NR test case 8.1.5.8.1-processing delay | Huawei, HiSilicon | 38.523-1 | 4414 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243506 | Correction to NR test case 8.1.5.8.1-processing delay | Huawei, HiSilicon | 38.523-1 | 4414 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242816 | Correction to NEDC test case 8.2.2.8.3 | Huawei, HiSilicon, MCC TF160, Mediatek | 38.523-1 | 4415 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243511 | Correction to NEDC test case 8.2.2.8.3 | Huawei, HiSilicon, MCC TF160, Mediatek | 38.523-1 | 4415 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242817 | Correction to NR 5GC Multilayer TC 11.1.7-Emergency Services Fallback to EPS with redirection | Huawei, HiSilicon | 38.523-1 | 4416 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243591 | Correction to NR 5GC Multilayer TC 11.1.7-Emergency Services Fallback to EPS with redirection | Huawei, HiSilicon | 38.523-1 | 4416 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242818 | Correction to NR 5GC Multilayer TC 11.4.12-Emergency Services | Huawei, HiSilicon | 38.523-1 | 4417 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243593 | Correction to NR 5GC Multilayer TC 11.4.12-Emergency Services | Huawei, HiSilicon | 38.523-1 | 4417 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242819 | Correction of NR eCall TC 11.5.5-Limited service | Huawei, HiSilicon | 38.523-1 | 4418 | - | Rel-17 | F | TEI16_Test, NR_EIEI-UEConTest | revised |
| R5-243594 | Correction of NR eCall TC 11.5.5-Limited service | Huawei, HiSilicon | 38.523-1 | 4418 | 1 | Rel-17 | F | TEI16_Test, NR_EIEI-UEConTest | agreed |
| R5-242820 | Correction of NR eCall TC 11.5.12-Domain Selection | Huawei, HiSilicon | 38.523-1 | 4419 | - | Rel-17 | F | TEI16_Test, NR_EIEI-UEConTest | withdrawn |
| R5-242828 | Correction of MUSIM gap TC 8.1.2.1.6 | Huawei, HiSilicon | 38.523-1 | 4420 | - | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | revised |
| R5-243958 | Correction of MUSIM gap TC 8.1.2.1.6 | Huawei, HiSilicon | 38.523-1 | 4420 | 1 | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | agreed |
| R5-242829 | Correction of MUSIM NAS TC 9.1.7.3-9.1.7.4 | Huawei, HiSilicon | 38.523-1 | 4421 | - | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | withdrawn |
| R5-242833 | Add test applicability for new RedCap HD-FDD TC | Huawei, HiSilicon | 38.523-1 | 4422 | - | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-242857 | Update of message contents for 8.2.3.18.4 conditional PSCell change | Huawei, HiSilicon | 38.523-1 | 4423 | - | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-243580 | Update of message contents for 8.2.3.18.4 conditional PSCell change | Huawei, HiSilicon | 38.523-1 | 4423 | 1 | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-242862 | Editorial correction for test procedure sequence in 8.1.1.3 | NTT DOCOMO, INC. | 38.523-1 | 4424 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242911 | Addition of NR sidelink SIG test case 12.1.1.1 | Huawei, HiSilicon | 38.523-1 | 4425 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-243534 | Addition of NR sidelink SIG test case 12.1.1.1 | Huawei, HiSilicon | 38.523-1 | 4425 | 1 | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242912 | Correction to NR sidelink SIG test case 12.2.5.1 | Huawei, HiSilicon | 38.523-1 | 4426 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242913 | Correction to NR sidelink SIG test case 12.2.5.2 | Huawei, HiSilicon | 38.523-1 | 4427 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | revised |
| R5-243535 | Correction to NR sidelink SIG test case 12.2.5.2 | Huawei, HiSilicon | 38.523-1 | 4427 | 1 | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242937 | Addition of NR P-CSCF restoration test case 10.1.2.3 | Qualcomm Incorporated, Orange | 38.523-1 | 4428 | - | Rel-17 | F | TEI15_Test | agreed |
| R5-242956 | Update ATSSS test case 11.9.3 | ZTE | 38.523-1 | 4429 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-242957 | Update ATSSS test case 11.9.4 | ZTE | 38.523-1 | 4430 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-242958 | Correction to eNS test case 10.1.8.2 | Anritsu EMEA Ltd, MediaTek | 38.523-1 | 4431 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-243077 | Update of test case 8.1.6.1.2.14 for SON_MDT | CMCC, CATT, Qualcomm | 38.523-1 | 4432 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-243583 | Update of test case 8.1.6.1.2.14 for SON_MDT | CMCC, CATT, Qualcomm | 38.523-1 | 4432 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-243084 | Correction of SDT TC 7.1.1.13.5-cg-SDT-TATimer | Huawei, HiSilicon, MCC TF160 | 38.523-1 | 4433 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-243537 | Correction of SDT TC 7.1.1.13.5-cg-SDT-TATimer | Huawei, HiSilicon, MCC TF160 | 38.523-1 | 4433 | 1 | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-243085 | Correction of SDT TC 8.1.5.13.1-cg-SDT | Huawei, HiSilicon, MCC TF160 | 38.523-1 | 4434 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-243538 | Correction of SDT TC 8.1.5.13.1-cg-SDT | Huawei, HiSilicon, MCC TF160 | 38.523-1 | 4434 | 1 | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-243088 | Correction of MAC TC 7.1.1.3.2b-ULCGTYPE1 | Huawei, HiSilicon | 38.523-1 | 4435 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243118 | Correction to NR CA shared spectrum test case 7.1.1.10.3 | Qualcomm Incorporated | 38.523-1 | 4436 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-243119 | Correction to NR shared spectrum test case 8.1.5.6.6.1 | Qualcomm Incorporated | 38.523-1 | 4437 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-243120 | Correction to NR shared spectrum test case 8.2.5.7.1 | Qualcomm Incorporated | 38.523-1 | 4438 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-243121 | Corrrection to NR5GC test case 8.1.4.4.1 | Keysight Technologies UK | 38.523-1 | 4439 | - | Rel-17 | F | TEI16_Test, NR_Mob_enh-UEConTest | agreed |
| R5-243122 | Correction to NR shared spectrum test case 8.2.5.7.2 | Qualcomm Incorporated | 38.523-1 | 4440 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-243124 | Updates to UAS TC 9.1.5.2.11 | Ericsson | 38.523-1 | 4441 | - | Rel-17 | F | ID_UAS-UEConTest | revised |
| R5-243575 | Updates to UAS TC 9.1.5.2.11 | Ericsson | 38.523-1 | 4441 | 1 | Rel-17 | F | ID_UAS-UEConTest | agreed |
| R5-243125 | Updates to UAS TC 10.1.4.2 | Ericsson | 38.523-1 | 4442 | - | Rel-17 | F | ID_UAS-UEConTest | revised |
| R5-243576 | Updates to UAS TC 10.1.4.2 | Ericsson | 38.523-1 | 4442 | 1 | Rel-17 | F | ID_UAS-UEConTest | agreed |
| R5-243155 | Adding 7.1.1.3.17.1 multi-TRP PUSCH repetition type A | Huawei, HiSilicon | 38.523-1 | 4443 | - | Rel-17 | F | NR_feMIMO-UEConTest | agreed |
| R5-243156 | Adding 7.1.1.3.17.2 multi-TRP PUSCH repetition type B | Huawei, HiSilicon | 38.523-1 | 4444 | - | Rel-17 | F | NR_feMIMO-UEConTest | agreed |
| R5-243165 | New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource | Nokia | 38.523-1 | 4445 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-243968 | New testcase 10.5.1.1 UE establishing initial PDN connection as a user plane resource | Nokia | 38.523-1 | 4445 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-243166 | New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session | Nokia | 38.523-1 | 4446 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | revised |
| R5-243969 | New testcase 10.5.1.2 UE establishing a PDN connection as a user plane resource of an already established MA PDU session | Nokia | 38.523-1 | 4446 | 1 | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-243169 | Editorial correction of testcase 9.1.6.2.3 | Nokia | 38.523-1 | 4447 | - | Rel-17 | F | ID_UAS-UEConTest | agreed |
| R5-243170 | Modification of testcase 8.1.5.13.3 for SDT-SRB2-Indication | Nokia | 38.523-1 | 4448 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-243539 | Modification of testcase 8.1.5.13.3 for SDT-SRB2-Indication | Nokia | 38.523-1 | 4448 | 1 | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-243172 | Addition of new test case 8.2.3.18.6 inter-SN CPC for EN-DC | Nokia | 38.523-1 | 4449 | - | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-243954 | Addition of new test case 8.2.3.18.6 inter-SN CPC for EN-DC | Nokia | 38.523-1 | 4449 | 1 | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-243175 | Addition of new test case 8.2.3.18.7 inter-SN CPC for NR-DC | Nokia | 38.523-1 | 4450 | - | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-243955 | Addition of new test case 8.2.3.18.7 inter-SN CPC for NR-DC | Nokia | 38.523-1 | 4450 | 1 | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-243181 | Corrections to eMIMO TC 7.1.1.3.10 | Lenovo, MCC TF160 | 38.523-1 | 4451 | - | Rel-17 | F | TEI16_Test, NR_eMIMO-UEConTest | agreed |
| R5-243183 | Corrections to CovEnh TC 7.1.1.1.18 | Lenovo, MCC TF160 | 38.523-1 | 4452 | - | Rel-17 | F | TEI17_Test, NR_cov_enh-UEConTest | agreed |
| R5-243186 | Correction to MAC TC 7.1.1.6.1 | MediaTek Inc., Keysight | 38.523-1 | 4453 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-243346 | Corrrection to RRC test case 8.2.3.1.2 | Keysight Technologies UK | 38.523-1 | 4454 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243347 | Corrrection to RRC test case 8.2.3.2.2 | Keysight Technologies UK | 38.523-1 | 4455 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243348 | Corrrection to RRC test case 8.2.3.6.2x | Keysight Technologies UK | 38.523-1 | 4456 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243349 | Corrrection to RRC test case 8.2.3.7.2x | Keysight Technologies UK | 38.523-1 | 4457 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243350 | Corrrection to RRC test case 8.2.3.8.2x | Keysight Technologies UK | 38.523-1 | 4458 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243512 | Corrrection to RRC test case 8.2.3.8.2x | Keysight Technologies UK | 38.523-1 | 4458 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243351 | Corrrection to RRC test case 8.2.3.12.2 | Keysight Technologies UK | 38.523-1 | 4459 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243513 | Corrrection to RRC test case 8.2.3.12.2 | Keysight Technologies UK | 38.523-1 | 4459 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243352 | Corrrection to RRC test case 8.2.3.13.2 | Keysight Technologies UK | 38.523-1 | 4460 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243353 | Corrrection to RRC test case 8.2.3.14.3 | Keysight Technologies UK | 38.523-1 | 4461 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243445 | Add new RedCap HD-FDD Test Case | Huawei, HiSilicon | 38.523-1 | 4462 | - | Rel-17 | F | TEI17_Test | revised |
| R5-243585 | Add new RedCap HD-FDD Test Case | Huawei, HiSilicon | 38.523-1 | 4462 | 1 | Rel-17 | F | TEI17_Test | agreed |
| R5-243483 | Configuration fix for testcase 8.1.5.13.2 | Nokia | 38.523-1 | 4463 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-243500 | Update the NSAC test case titles | CATT | 38.523-1 | 4464 | - | Rel-17 | F | TEI15_Test | revised |
| R5-243590 | Update the NSAC test case titles | CATT | 38.523-1 | 4464 | 1 | Rel-17 | F | TEI15_Test | agreed |
| R5-242137 | Correction to the applicability condition of NR NTN | MediaTek Inc. | 38.523-2 | 0454 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243565 | Correction to the applicability condition of NR NTN | MediaTek Inc. | 38.523-2 | 0454 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242138 | Addition of TC 9.1.11.5 into Table 4.3.1-1 | MediaTek Inc. | 38.523-2 | 0455 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | withdrawn |
| R5-242174 | Editorial update of applicability for TC 8.2.4.2.1.x | MediaTek Inc. | 38.523-2 | 0456 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243519 | Editorial update of applicability for TC 8.2.4.2.1.x | MediaTek Inc. | 38.523-2 | 0456 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242175 | Update applicability of SOR-CMCI TC | MediaTek Inc. | 38.523-2 | 0457 | - | Rel-17 | F | TEI17_Test, eCPSOR_CON-UEConTest | agreed |
| R5-242273 | Add applicability for Rel-17 ATSSS test cases | MediaTek Inc. | 38.523-2 | 0458 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-242279 | Addition to legacy test cases applicability for RedCap UE | MediaTek Inc. | 38.523-2 | 0459 | - | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243595 | Addition to legacy test cases applicability for RedCap UE | MediaTek Inc. | 38.523-2 | 0459 | 1 | Rel-17 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242330 | Updates of clause 4 | MCC TF160 | 38.523-2 | 0460 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242545 | Editorial changes to Applicability tables of 38.523-2 | Qualcomm Technologies Ireland | 38.523-2 | 0461 | - | Rel-17 | F | TEI17_Test | agreed |
| R5-242552 | Addition of applicability for new NR NTN test cases | Qualcomm Technologies Ireland | 38.523-2 | 0462 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-242564 | Addition of applicability of E-UTRA to NR MPS priority Indication test case | Qualcomm Incorporated | 38.523-2 | 0463 | - | Rel-17 | F | TEI16_Test | revised |
| R5-243596 | Addition of applicability of E-UTRA to NR MPS priority Indication test case | Qualcomm Incorporated | 38.523-2 | 0463 | 1 | Rel-17 | F | TEI16_Test | agreed |
| R5-242569 | Corrections to applicability of NR supplemental uplink test case | Qualcomm Incorporated | 38.523-2 | 0464 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243520 | Corrections to applicability of NR supplemental uplink test case | Qualcomm Incorporated | 38.523-2 | 0464 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242570 | Corrections to applicability of Network Slice Admision Control mobility management aspects test cases | Qualcomm Incorporated | 38.523-2 | 0465 | - | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | revised |
| R5-243521 | Corrections to applicability of Network Slice Admision Control mobility management aspects test cases | Qualcomm Incorporated | 38.523-2 | 0465 | 1 | Rel-17 | F | TEI17_Test, eNS_Ph2-UEConTest | agreed |
| R5-242662 | Update of test applicability of NSAC test cases | CATT | 38.523-2 | 0466 | - | Rel-17 | F | TEI15_Test | revised |
| R5-243552 | Update of test applicability of NSAC test cases | CATT | 38.523-2 | 0466 | 1 | Rel-17 | F | TEI15_Test | agreed |
| R5-242684 | Scoping NR SA applicable TCs for SNPN-only Ues | China Telecom, MediaTek | 38.523-2 | 0467 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-243962 | Scoping NR SA applicable TCs for SNPN-only Ues | China Telecom, MediaTek | 38.523-2 | 0467 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242704 | Addition of applicability for eNPN test cases | China Telecom | 38.523-2 | 0468 | - | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | revised |
| R5-243963 | Addition of applicability for eNPN test cases | China Telecom | 38.523-2 | 0468 | 1 | Rel-17 | F | NG_RAN_PRN_enh_plus_CT-UEConTest | agreed |
| R5-242736 | Applicability updates to ING_5GS test cases | Qualcomm Incorporated, China Telecom | 38.523-2 | 0469 | - | Rel-17 | F | ING_5GS-UEConTest | revised |
| R5-243960 | Applicability updates to ING_5GS test cases | Qualcomm Incorporated, China Telecom | 38.523-2 | 0469 | 1 | Rel-17 | F | ING_5GS-UEConTest | agreed |
| R5-242766 | Applicability updates to NR NTN test cases | Qualcomm Incorporated | 38.523-2 | 0470 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | revised |
| R5-243950 | Applicability updates to NR NTN test cases | Qualcomm Incorporated | 38.523-2 | 0470 | 1 | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242814 | Update applicability of MAC TC 7.1.1.1.4-Beam Failure | Huawei, HiSilicon | 38.523-2 | 0471 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242830 | Update test applicability for MUSIM TC | Huawei, HiSilicon | 38.523-2 | 0472 | - | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | revised |
| R5-243959 | Update test applicability for MUSIM TC | Huawei, HiSilicon | 38.523-2 | 0472 | 1 | Rel-17 | F | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest | agreed |
| R5-242834 | Add test applicability for new RedCap HD-FDD TC | Huawei, HiSilicon | 38.523-2 | 0473 | - | Rel-17 | F | TEI17_Test | revised |
| R5-243586 | Add test applicability for new RedCap HD-FDD TC | Huawei, HiSilicon | 38.523-2 | 0473 | 1 | Rel-17 | F | TEI17_Test | agreed |
| R5-242914 | Addition of applicability for NR sidelink SIG test cases | Huawei, HiSilicon | 38.523-2 | 0474 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242938 | Applicability updates for P-CSCF restoration test cases | Qualcomm Incorporated, Orange | 38.523-2 | 0475 | - | Rel-17 | F | TEI15_Test | agreed |
| R5-243078 | Update of applicability for test cases 8.1.6.1.2.14 | CMCC, CATT, Qualcomm | 38.523-2 | 0476 | - | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | revised |
| R5-243584 | Update of applicability for test cases 8.1.6.1.2.14 | CMCC, CATT, Qualcomm | 38.523-2 | 0476 | 1 | Rel-17 | F | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest | agreed |
| R5-243158 | Adding test applicability for multi-TRP PUSCH repetition test cases | Huawei, HiSilicon | 38.523-2 | 0477 | - | Rel-17 | F | NR_feMIMO-UEConTest | revised |
| R5-243544 | Adding test applicability for multi-TRP PUSCH repetition test cases | Huawei, HiSilicon | 38.523-2 | 0477 | 1 | Rel-17 | F | NR_feMIMO-UEConTest | agreed |
| R5-243167 | Addition of applicability clauses for testcases 10.5.1.1 and 10.5.1.2 | Nokia | 38.523-2 | 0478 | - | Rel-17 | F | ATSSS_Ph2-UEConTest | agreed |
| R5-243171 | Modification of testcase 8.1.5.13.3 applicability clauses | Nokia | 38.523-2 | 0479 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-243176 | Addition of applicability clauses for inter-SN CPC testcases for EN-DC and NR-DC | Nokia | 38.523-2 | 0480 | - | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | revised |
| R5-243956 | Addition of applicability clauses for inter-SN CPC testcases for EN-DC and NR-DC | Nokia | 38.523-2 | 0480 | 1 | Rel-17 | F | LTE_NR_DC_enh2-UEConTest | agreed |
| R5-243185 | Applicability updates to NR shared spectrum test cases | Qualcomm Incorporated | 38.523-2 | 0481 | - | Rel-17 | F | NR_unlic-UEConTest | agreed |
| R5-243444 | Editorial correction to applicability of TC 12.1.2.1 | ZTE Corporation | 38.523-2 | 0482 | - | Rel-17 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-242176 | Update case title of 8.1.1.2.4 and 8.1.5.9.1 | MediaTek Inc. | 38.523-3 | 3428 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242310 | NR QoE: Addition of NR/5GC Test Model aspects | MCC TF160 | 38.523-3 | 3432 | - | Rel-17 | F | NR_QoE-UEConTest | agreed |
| R5-242319 | NR SDT: Addition of NR/5GC Test Model aspects | MCC TF160 | 38.523-3 | 3433 | - | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-243579 | NR SDT: Addition of NR/5GC Test Model aspects | MCC TF160 | 38.523-3 | 3433 | 1 | Rel-17 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-242320 | NR NTN: Addition of GSO Test Model | MCC TF160 | 38.523-3 | 3434 | - | Rel-17 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-242331 | Routine maintenance for TS 38.523-3 | MCC TF160 | 38.523-3 | 3435 | - | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243967 | Routine maintenance for TS 38.523-3 | MCC TF160 | 38.523-3 | 3435 | 1 | Rel-17 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242127 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | 38.533 | 3092 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-244005 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | 38.533 | 3092 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-242128 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | 38.533 | 3093 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-243920 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | 38.533 | 3093 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-244006 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | 38.533 | 3093 | 2 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-242371 | Introduction of band n106 to NR band group for FR1 | Nokia | 38.533 | 3094 | - | Rel-18 | F | NR_lic_bands_BW_R18-UEConTest | agreed |
| R5-242433 | Update test requirement for Redcap inter frequency test cases | MediaTek Beijing Inc. | 38.533 | 3095 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243907 | Update test requirement for Redcap inter frequency test cases | MediaTek Beijing Inc. | 38.533 | 3095 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-244013 | Update test requirement for Redcap inter frequency test cases | MediaTek Beijing Inc. | 38.533 | 3095 | 2 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242579 | Correction to TC titles for event triggered reporting with measurement gap | Anritsu | 38.533 | 3096 | - | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-242580 | Correction to 7.6.15.3 FR2 SA event triggered reporting tests with concurrent gaps including TT | Anritsu | 38.533 | 3097 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-243744 | Correction to 7.6.15.3 FR2 SA event triggered reporting tests with concurrent gaps including TT | Anritsu | 38.533 | 3097 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-242606 | Completion of NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case including test tolerance | Nokia | 38.533 | 3098 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243751 | Completion of NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case including test tolerance | Nokia | 38.533 | 3098 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242607 | Completion of NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case including test tolerance | Nokia | 38.533 | 3099 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243749 | Completion of NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case including test tolerance | Nokia | 38.533 | 3099 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242609 | Editorial correction to 7.6.3.6 test case | Nokia | 38.533 | 3100 | - | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-242611 | Incorrect clause number for test case 7.6.13.1 | Nokia | 38.533 | 3101 | - | Rel-18 | F | TEI17_Test, NR_IIOT_URLLC_enh-UEConTest | agreed |
| R5-242612 | Addition of cell configuration mapping for HST FR2 test cases | Nokia | 38.533 | 3102 | - | Rel-18 | F | NR_HST_FR2-UEConTest | agreed |
| R5-242632 | Correction of interruptions at NR SRS carrier based switching test case 4.5.2.8 including Test Tolerance | Ericsson | 38.533 | 3103 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243754 | Correction of interruptions at NR SRS carrier based switching test case 4.5.2.8 including Test Tolerance | Ericsson | 38.533 | 3103 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242633 | Correction of interruptions at E-UTRA SRS carrier based switching test case 4.5.2.9 including Test Tolerance | Ericsson | 38.533 | 3104 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243755 | Correction of interruptions at E-UTRA SRS carrier based switching test case 4.5.2.9 including Test Tolerance | Ericsson | 38.533 | 3104 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242634 | Correction of FR2 interruptions at SRS carrier based switching test case 5.5.2.7 and 5.5.2.8 | Ericsson | 38.533 | 3105 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243706 | Correction of FR2 interruptions at SRS carrier based switching test case 5.5.2.7 and 5.5.2.8 | Ericsson | 38.533 | 3105 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242635 | Correction of SA FR1 interruptions at NR SRS carrier based switching test case 6.5.2.2 including Test Tolerance | Ericsson | 38.533 | 3106 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243756 | Correction of SA FR1 interruptions at NR SRS carrier based switching test case 6.5.2.2 including Test Tolerance | Ericsson | 38.533 | 3106 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242636 | Correction of NR SA event triggered reporting tests with additional mandatory gap pattern test case 6.6.2.9 including Test Tolerance | Ericsson | 38.533 | 3107 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242637 | Correction of NR SA event triggered reporting tests for FR1 without gap when DRX is used test case 6.6.2.10 including Test Tolerance | Ericsson | 38.533 | 3108 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242638 | Correction of NR SA event triggered reporting tests for FR1 without gap when DRX is not used test case 6.6.2.11 including Test Tolerance | Ericsson | 38.533 | 3109 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242639 | Addition of FR2 SA event triggered reporting test without gap under non-DRX test case 7.6.2.10 including Test Tolerance | Ericsson | 38.533 | 3110 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243757 | Addition of FR2 SA event triggered reporting test without gap under non-DRX test case 7.6.2.10 including Test Tolerance | Ericsson | 38.533 | 3110 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242640 | Addition of FR2 SA event triggered reporting test without gap under DRX test case 7.6.2.11 including Test Tolerance | Ericsson | 38.533 | 3111 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243758 | Addition of FR2 SA event triggered reporting test without gap under DRX test case 7.6.2.11 including Test Tolerance | Ericsson | 38.533 | 3111 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242641 | Test Tolerance correction in Annex F for RRM enhancement test cases | Ericsson | 38.533 | 3112 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242642 | Correction of NR_U SA Intra-frequency measurements test cases | Ericsson | 38.533 | 3113 | - | Rel-18 | F | NR_unlic-UEConTest | withdrawn |
| R5-242643 | Correction of NR_U SA Inter_RAT measurements test cases | Ericsson | 38.533 | 3114 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242644 | Correction of NR_U SA L1-RSRP measurements test cases | Ericsson | 38.533 | 3115 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243748 | Correction of NR_U SA L1-RSRP measurements test cases | Ericsson | 38.533 | 3115 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242646 | Cell configuration correction in Annex E for SA FR2 RRM enhancement test cases | Ericsson | 38.533 | 3116 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242647 | Minimum conformance requirements for FR2 SA event triggered reporting without gap | Ericsson | 38.533 | 3117 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242670 | Addition test case for Inter-cell SSB based L1-RSRP Reporting | Samsung | 38.533 | 3118 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-243880 | Addition test case for Inter-cell SSB based L1-RSRP Reporting | Samsung | 38.533 | 3118 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-242671 | Adding TRP specific Minimum conformance requirements | Samsung | 38.533 | 3119 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-243705 | Adding TRP specific Minimum conformance requirements | Samsung | 38.533 | 3119 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-242693 | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.533 | 3120 | - | Rel-18 | F | NR_RRM_enh-UEConTest | withdrawn |
| R5-242695 | Addition of new test case 5.6.7.1 EN-DC event triggered reporting test without gap under non-DRX | Sporton | 38.533 | 3121 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243707 | Addition of new test case 5.6.7.1 EN-DC event triggered reporting test without gap under non-DRX | Sporton | 38.533 | 3121 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242696 | Addition of new test case 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used | Sporton | 38.533 | 3122 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243708 | Addition of new test case 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used | Sporton | 38.533 | 3122 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242697 | Addition of new test case 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Sporton | 38.533 | 3123 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243709 | Addition of new test case 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Sporton | 38.533 | 3123 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242698 | Addition of new test case 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Sporton | 38.533 | 3124 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243710 | Addition of new test case 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Sporton | 38.533 | 3124 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242699 | Addition of MU and test tolerance in Annex F for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.533 | 3125 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-244002 | Addition of MU and test tolerance in Annex F for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.533 | 3125 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242727 | Correct Table 4.5.2.1.4.1-2 & Table 4.5.2.2.4.1-2 & Table 4.5.2.3.4.1-2 & Table 4.5.2.4.4.1-2 & Table 4.5.2.5.4.1-2 of test frequency | SGS Wireless | 38.533 | 3126 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242863 | Correction to SDT RRM test case 6.2.1 with TT | Huawei, HiSilicon | 38.533 | 3127 | - | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | revised |
| R5-244001 | Correction to SDT RRM test case 6.2.1 with TT | Huawei, HiSilicon | 38.533 | 3127 | 1 | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | agreed |
| R5-242864 | Correction to SDT RRM test case 7.2.1.1 with TT | Huawei, HiSilicon | 38.533 | 3128 | - | Rel-18 | F | NR_SmallData_INACTIVE-UEConTest | withdrawn |
| R5-242867 | Addition of MCE RRM test case 6.5.7D.1 | Huawei, HiSilicon | 38.533 | 3129 | - | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-242868 | Addition of MCE RRM test case 6.5.7D.2 | Huawei, HiSilicon | 38.533 | 3130 | - | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-242869 | Addition of MCE RRM test case 6.5.7D.3 | Huawei, HiSilicon | 38.533 | 3131 | - | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-242870 | Correction to Annex E for MCE RRM test cases | Huawei, HiSilicon | 38.533 | 3132 | - | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-242872 | Correction to R15 L1-RSRP absolute accuracy RRM test cases | Huawei, HiSilicon | 38.533 | 3133 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242873 | Correction to R15 RACH RRM test cases and RMCs | Huawei, HiSilicon | 38.533 | 3134 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243916 | Correction to R15 RACH RRM test cases and RMCs | Huawei, HiSilicon | 38.533 | 3134 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242874 | Correction to R15 reestablishment RRM test case 6.3.2.1.1 | Huawei, HiSilicon | 38.533 | 3135 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243917 | Correction to R15 reestablishment RRM test case 6.3.2.1.1 | Huawei, HiSilicon | 38.533 | 3135 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242875 | Correction to R15 SCell activation test cases | Huawei, HiSilicon, Starpoint | 38.533 | 3136 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242876 | Correction to R16 relaxed measurement RRM test cases | Huawei, HiSilicon, Starpoint | 38.533 | 3137 | - | Rel-18 | F | TEI16_Test, NR_UE_pow_sav-UEConTest | agreed |
| R5-242878 | Correction to RedCap RRM test case 16.1.1.5 and 16.1.1.7 | Huawei, HiSilicon, Starpoint | 38.533 | 3138 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242879 | Correction to RedCap RRM test case 16.1.2.5 and 16.1.2.6 | Huawei, HiSilicon, Starpoint, Qualcomm | 38.533 | 3139 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243918 | Correction to RedCap RRM test case 16.1.2.5 and 16.1.2.6 | Huawei, HiSilicon, Starpoint, Qualcomm | 38.533 | 3139 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242880 | Correction to RedCap RRM test case 16.2.1.1 with TT | Huawei, HiSilicon | 38.533 | 3140 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-244014 | Correction to RedCap RRM test case 16.2.1.1 with TT | Huawei, HiSilicon | 38.533 | 3140 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242881 | Correction to RedCap RRM test case 16.2.1.2 with TT | Huawei, HiSilicon | 38.533 | 3141 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-244015 | Correction to RedCap RRM test case 16.2.1.2 with TT | Huawei, HiSilicon | 38.533 | 3141 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242882 | Correction to RedCap RRM test case 17.2.1.1 with TT | Huawei, HiSilicon | 38.533 | 3142 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-242883 | Correction to RedCap RRM test case 17.3.1.1 with TT | Huawei, HiSilicon | 38.533 | 3143 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242884 | Correction to RedCap RRM test case 17.3.1.2 with TT | Huawei, HiSilicon | 38.533 | 3144 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242885 | Correction to RedCap RRM test case 17.3.2.3.1 with TT | Huawei, HiSilicon | 38.533 | 3145 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242886 | Correction to RedCap RRM test case 17.4.1.1 with TT | Huawei, HiSilicon | 38.533 | 3146 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243774 | Correction to RedCap RRM test case 17.4.1.1 with TT | Huawei, HiSilicon | 38.533 | 3146 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242887 | Correction to RedCap RRM test case 17.4.3.1 with TT | Huawei, HiSilicon | 38.533 | 3147 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242888 | Correction to RedCap RRM test case 17.5.1.5 with TT | Huawei, HiSilicon | 38.533 | 3148 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-242889 | Correction to RedCap RRM test case 17.5.1.6 with TT | Huawei, HiSilicon | 38.533 | 3149 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-242890 | Correction to RedCap RRM test case 17.5.1.7 with TT | Huawei, HiSilicon | 38.533 | 3150 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-242891 | Correction to RedCap RRM test case 17.5.1.8 with TT | Huawei, HiSilicon | 38.533 | 3151 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243701 | Correction to RedCap RRM test case 17.5.1.8 with TT | Huawei, HiSilicon | 38.533 | 3151 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-242892 | Correction to RedCap RRM test case 17.5.4.1.1 with TT | Huawei, HiSilicon | 38.533 | 3152 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242893 | Correction to RedCap RRM test case 17.5.4.2.1 with TT | Huawei, HiSilicon | 38.533 | 3153 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242894 | Correction to RedCap RRM test case 17.5.5.1.1 with TT | Huawei, HiSilicon | 38.533 | 3154 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243775 | Correction to RedCap RRM test case 17.5.5.1.1 with TT | Huawei, HiSilicon | 38.533 | 3154 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242895 | Correction to RedCap RRM test case 17.5.5.2.1 with TT | Huawei, HiSilicon | 38.533 | 3155 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242896 | Correction to RedCap RRM test case 17.7.1.1 with TT | Huawei, HiSilicon | 38.533 | 3156 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243776 | Correction to RedCap RRM test case 17.7.1.1 with TT | Huawei, HiSilicon | 38.533 | 3156 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242897 | Correction to RedCap RRM test case 17.7.1.2 with TT | Huawei, HiSilicon | 38.533 | 3157 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243777 | Correction to RedCap RRM test case 17.7.1.2 with TT | Huawei, HiSilicon | 38.533 | 3157 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242898 | Correction to RedCap RRM test case 17.7.2.1 with TT | Huawei, HiSilicon | 38.533 | 3158 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243778 | Correction to RedCap RRM test case 17.7.2.1 with TT | Huawei, HiSilicon | 38.533 | 3158 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242899 | Correction to RedCap RRM test case 17.7.2.2 with TT | Huawei, HiSilicon | 38.533 | 3159 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243779 | Correction to RedCap RRM test case 17.7.2.2 with TT | Huawei, HiSilicon | 38.533 | 3159 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242900 | Correction to RedCap RRM test case 17.7.3.1 with TT | Huawei, HiSilicon | 38.533 | 3160 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243780 | Correction to RedCap RRM test case 17.7.3.1 with TT | Huawei, HiSilicon | 38.533 | 3160 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242901 | Correction to RedCap RRM test case 17.7.3.2 with TT | Huawei, HiSilicon | 38.533 | 3161 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243781 | Correction to RedCap RRM test case 17.7.3.2 with TT | Huawei, HiSilicon | 38.533 | 3161 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242902 | Correction to RedCap RRM test case 17.7.4.1 with TT | Huawei, HiSilicon | 38.533 | 3162 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243782 | Correction to RedCap RRM test case 17.7.4.1 with TT | Huawei, HiSilicon | 38.533 | 3162 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242903 | Addition of RedCap RRM test case 17.7.4.2 with TT | Huawei, HiSilicon | 38.533 | 3163 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243783 | Addition of RedCap RRM test case 17.7.4.2 with TT | Huawei, HiSilicon | 38.533 | 3163 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242904 | Correction to Annex E for RedCap RRM test cases | Huawei, HiSilicon | 38.533 | 3164 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242905 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | 38.533 | 3165 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243746 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | 38.533 | 3165 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-244012 | Correction to Annex F for RedCap RRM test cases | Huawei, HiSilicon | 38.533 | 3165 | 2 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242915 | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.533 | 3166 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243732 | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.533 | 3166 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-244003 | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.533 | 3166 | 2 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242979 | Editorial correction of references to TS 38.101-1 in section 7 | Rohde & Schwarz | 38.533 | 3167 | - | Rel-18 | F | TEI16_Test, NR_CLI-UEConTest | agreed |
| R5-242980 | Editorial correction of references to TS 38.101-1 in section 6.7 | Rohde & Schwarz | 38.533 | 3168 | - | Rel-18 | F | TEI16_Test, NR_eMIMO-UEConTest | agreed |
| R5-242981 | Editorial correction of references to TS 38.101-1 in section 6.6 | Rohde & Schwarz | 38.533 | 3169 | - | Rel-18 | F | TEI17_Test, NR_IIOT_URLLC_enh-UEConTest | agreed |
| R5-242982 | Editorial correction of references to TS 38.101-1 in section 5 | Rohde & Schwarz | 38.533 | 3170 | - | Rel-18 | F | TEI16_Test, NR_CLI-UEConTest | agreed |
| R5-242983 | Editorial correction of references to TS 38.101-1 in section 4 | Rohde & Schwarz | 38.533 | 3171 | - | Rel-18 | F | TEI16_Test, NR_eMIMO-UEConTest | agreed |
| R5-242985 | Corrections to R17 RedCap 18.3.1.x test cases | Rohde & Schwarz | 38.533 | 3172 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243713 | Corrections to R17 RedCap 18.3.1.x test cases | Rohde & Schwarz | 38.533 | 3172 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242986 | Editorial correction to title of R17 RedCap test case 16.6.4.7 | Rohde & Schwarz | 38.533 | 3173 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242987 | Test procedure correction for R17 RedCap NCD-SSB test cases | Rohde & Schwarz | 38.533 | 3174 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242988 | Addition of SA FR1 NR-U RSSI minimum conformance requirements | Rohde & Schwarz | 38.533 | 3175 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242989 | Addition of SA FR1 NR-U RSSI measurement accuracy test case 11.6.5.3 | Rohde & Schwarz | 38.533 | 3176 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242990 | Addition of SA FR1 NR-U SSB based L1-RSRP measurement test case 11.6.4.1 | Rohde & Schwarz | 38.533 | 3177 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242991 | Addition of SA FR1 NR-U channel occupancy inter-frequency test case 11.6.6.3 | Rohde & Schwarz | 38.533 | 3178 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242992 | Correction to frequencies table for RRM FR2 SA test cases | Rohde & Schwarz | 38.533 | 3179 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242993 | Corrections to message contents for SA FR2 event triggered intra-frequency test cases | Rohde & Schwarz | 38.533 | 3180 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242994 | Corrections to message contents for SA FR2 timing test cases | Rohde & Schwarz | 38.533 | 3181 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243018 | Correction to message exception in non-contention based random access | Anritsu | 38.533 | 3182 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-243019 | Correction to K1 in 4.6.4.3 and 6.6.4.3 for FDD and TDD SCS 30 kHz | Anritsu | 38.533 | 3183 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243020 | Correction to DCI trigger in 6.6.4.3 for FDD and TDD SCS 15 kHz | Anritsu | 38.533 | 3184 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243021 | Correction to test parameters in 16.5.1.7 | Anritsu | 38.533 | 3185 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-243025 | Correction to SMTC settings in 16.6.2.9 and 16.6.2.10 | Anritsu | 38.533 | 3186 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243095 | Removal of Editors note for applicability of RedCap SS-RSRP test cases | Rohde & Schwarz | 38.533 | 3187 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243096 | Core spec alignment for R17 RedCap NCD-SSB test cases | Rohde & Schwarz | 38.533 | 3188 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-243097 | Addition of SA FR1 NR-U channel occupancy intra-frequency test cases | Rohde & Schwarz | 38.533 | 3189 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243153 | Update to Unified TCI state test cases | Huawei, HiSilicon, Samsung | 38.533 | 3190 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-243703 | Update to Unified TCI state test cases | Huawei, HiSilicon, Samsung | 38.533 | 3190 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-243154 | Correcting propagation AWGN in test cases without Noc | Huawei, HiSilicon | 38.533 | 3191 | - | Rel-18 | F | NR_feMIMO-UEConTest | revised |
| R5-243704 | Correcting propagation AWGN in test cases without Noc | Huawei, HiSilicon | 38.533 | 3191 | 1 | Rel-18 | F | NR_feMIMO-UEConTest | agreed |
| R5-243235 | Addition of Clause 4.5.6.4.0 minimum conformance requirements for simultaneous RRC-based active BWP switch on multiple CCs | Apple | 38.533 | 3192 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243236 | Addition of Clause 4.6.8.0 minimum conformance requirements for CSI-RS based intra-frequency measurement | Apple | 38.533 | 3193 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243237 | Addition of Clause 4.6.9.0 minimum conformance requirements for CSI-RS based inter-frequency measurement | Apple | 38.533 | 3194 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243238 | Addition of Clause 5.5.3.0 minimum conformance requirements for SCell activation and deactivation delay | Apple | 38.533 | 3195 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243239 | Addition of Clause 5.5.6.5.0 minimum conformance requirements for simultaneous RRC-based active BWP switch on multiple CC | Apple | 38.533 | 3196 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243240 | Addition of message contents to test case 4.7.9.1 | Apple, Huawei, Hisilicon | 38.533 | 3197 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243815 | Addition of message contents to test case 4.7.9.1 | Apple, Huawei, Hisilicon | 38.533 | 3197 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243241 | Addition of message contents to test case 4.7.9.2 | Apple, Huawei, Hisilicon | 38.533 | 3198 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243842 | Addition of message contents to test case 4.7.9.2 | Apple, Huawei, Hisilicon | 38.533 | 3198 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243242 | Addition of message contents to test case 4.7.10.1 | Apple, Huawei, Hisilicon | 38.533 | 3199 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243843 | Addition of message contents to test case 4.7.10.1 | Apple, Huawei, Hisilicon | 38.533 | 3199 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243243 | Addition of message contents to test case 4.7.10.2 | Apple, Huawei, Hisilicon | 38.533 | 3200 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243844 | Addition of message contents to test case 4.7.10.2 | Apple, Huawei, Hisilicon | 38.533 | 3200 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243244 | Addition of message contents to test case 5.7.8.1 | Apple, Huawei, Hisilicon | 38.533 | 3201 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243845 | Addition of message contents to test case 5.7.8.1 | Apple, Huawei, Hisilicon | 38.533 | 3201 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243245 | Addition of message contents to test case 5.7.8.2 | Apple, Huawei, Hisilicon | 38.533 | 3202 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243846 | Addition of message contents to test case 5.7.8.2 | Apple, Huawei, Hisilicon | 38.533 | 3202 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243246 | Addition of message contents to test case 5.7.9.1 | Apple, Huawei, Hisilicon | 38.533 | 3203 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243847 | Addition of message contents to test case 5.7.9.1 | Apple, Huawei, Hisilicon | 38.533 | 3203 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243247 | Addition of message contents to test case 5.7.9.2 | Apple, Huawei, Hisilicon | 38.533 | 3204 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243848 | Addition of message contents to test case 5.7.9.2 | Apple, Huawei, Hisilicon | 38.533 | 3204 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243248 | Addition of message contents to test case 6.7.11.1 | Apple, Huawei, Hisilicon | 38.533 | 3205 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243849 | Addition of message contents to test case 6.7.11.1 | Apple, Huawei, Hisilicon | 38.533 | 3205 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243249 | Addition of message contents to test case 6.7.11.2 | Apple, Huawei, Hisilicon | 38.533 | 3206 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243850 | Addition of message contents to test case 6.7.11.2 | Apple, Huawei, Hisilicon | 38.533 | 3206 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243250 | Addition of message contents to test case 6.7.12.1 | Apple, Huawei, Hisilicon | 38.533 | 3207 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243851 | Addition of message contents to test case 6.7.12.1 | Apple, Huawei, Hisilicon | 38.533 | 3207 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243251 | Addition of message contents to test case 6.7.12.2 | Apple, Huawei, Hisilicon | 38.533 | 3208 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243852 | Addition of message contents to test case 6.7.12.2 | Apple, Huawei, Hisilicon | 38.533 | 3208 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243252 | Addition of message contents to test case 7.7.8.1 | Apple, Huawei, Hisilicon | 38.533 | 3209 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243853 | Addition of message contents to test case 7.7.8.1 | Apple, Huawei, Hisilicon | 38.533 | 3209 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243253 | Addition of message contents to test case 7.7.8.2 | Apple, Huawei, Hisilicon | 38.533 | 3210 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243854 | Addition of message contents to test case 7.7.8.2 | Apple, Huawei, Hisilicon | 38.533 | 3210 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243254 | Addition of message contents to test case 7.7.9.1 | Apple, Huawei, Hisilicon | 38.533 | 3211 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243855 | Addition of message contents to test case 7.7.9.1 | Apple, Huawei, Hisilicon | 38.533 | 3211 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243255 | Addition of message contents to test case 7.7.9.2 | Apple, Huawei, Hisilicon | 38.533 | 3212 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243856 | Addition of message contents to test case 7.7.9.2 | Apple, Huawei, Hisilicon | 38.533 | 3212 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-243329 | Addition of EN-DC FR1 NR-U Random Access and Handover test cases | Qualcomm Technologies Int | 38.533 | 3213 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243335 | Correction to RRM RedCap TC 16.5.1.7 | Qualcomm Technologies Int, Anritsu | 38.533 | 3214 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243712 | Correction to RRM RedCap TC 16.5.1.7 | Qualcomm Technologies Int, Anritsu | 38.533 | 3214 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243340 | Correction to FR1 NSA Event Trigger TC 4.6.1.8 | Keysight Technologies | 38.533 | 3215 | - | Rel-18 | F | TEI17_Test, NR_HST_FR1_enh-UEConTest | revised |
| R5-243909 | Correction to FR1 NSA Event Trigger TC 4.6.1.8 | Keysight Technologies | 38.533 | 3215 | 1 | Rel-18 | F | TEI17_Test, NR_HST_FR1_enh-UEConTest | agreed |
| R5-243341 | Corrections to NR FR1 RedCap E-UTRA cell re-selection TCs 16.1.2.1, 16.1.2.3 | Keysight Technologies | 38.533 | 3216 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243342 | Editorial correction in Redcap 16.6.2.1 | Keysight Technologies | 38.533 | 3217 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243343 | Correction to FR1 RedCap E-UTRA - NR inter - RAT Handover TC 18.2.1.1 | Keysight Technologies | 38.533 | 3218 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243344 | Editorial correction in Annex E for FR1 RedCap TCs 16.6.2.3, 16.6.2.4 | Keysight Technologies | 38.533 | 3219 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243359 | Addition of NTN Timing Advance test 14.3.2.1 | Qualcomm France | 38.533 | 3220 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243360 | Addition of NTN distance-based CHO tests 14.2.1.5 and 14.2.1.6 | Qualcomm France | 38.533 | 3221 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243361 | Addition of NTN inter-frequency Measurement Initiation Cell Reselection tests 14.1.7 and 14.1.8 | Qualcomm France | 38.533 | 3222 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243362 | Addition of NTN L1-RSRP Measurement Accuracy tests 14.6.4.1 and 14.6.4.2 | Qualcomm France | 38.533 | 3223 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243363 | Update to NTN general sections | Qualcomm France | 38.533 | 3224 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243364 | Update to NR-U Cases 10.4.1.1 and 10.4.1.4 including TTs | Qualcomm France | 38.533 | 3225 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243365 | Update to NR-U Timing Accuracy Test 10.2.1.1 including TTs | Qualcomm France | 38.533 | 3226 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243366 | Update to NR-U Timing Advance Test 10.2.2.1 including TTs | Qualcomm France | 38.533 | 3227 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243367 | Update to NR-U L1-RSRP measurement reporting tests 10.4.3.x including TTs | Qualcomm France | 38.533 | 3228 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243368 | Update to SCell activation and deactivation tests 10.3.3.x including TTs | Qualcomm France | 38.533 | 3229 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243369 | Addition of NR-U EN-DC PSCell addition and release delay test case including TTs | Qualcomm France | 38.533 | 3230 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243370 | Addition of NR-U SA FR1 UE transmit timing and timing advance test cases including TTs | Qualcomm France | 38.533 | 3231 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243371 | Addition of NR-U SA FR1 Scell activation and deactivation test cases including TTs | Qualcomm France | 38.533 | 3232 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243372 | Update to Annex E and F for NR-U RRM tests | Qualcomm France | 38.533 | 3233 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243733 | Update to Annex E and F for NR-U RRM tests | Qualcomm France | 38.533 | 3233 | 1 | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243994 | Update to Annex E and F for NR-U RRM tests | Qualcomm France | 38.533 | 3233 | 2 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243373 | Removal of interruption during SCell operation NR-U tests | Qualcomm France | 38.533 | 3234 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243374 | Update to NSA measurement accuracy NR-U tests including TT | Qualcomm France | 38.533 | 3235 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243734 | Update to NSA measurement accuracy NR-U tests including TT | Qualcomm France | 38.533 | 3235 | 1 | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243995 | Update to NSA measurement accuracy NR-U tests including TT | Qualcomm France | 38.533 | 3235 | 2 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243375 | Update to SA measurement accuracy NR-U tests including TT | Qualcomm France | 38.533 | 3236 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243735 | Update to SA measurement accuracy NR-U tests including TT | Qualcomm France | 38.533 | 3236 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243376 | Update to NSA event-triggered measurement NR-U tests including TT | Qualcomm France | 38.533 | 3237 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243892 | Update to NSA event-triggered measurement NR-U tests including TT | Qualcomm France | 38.533 | 3237 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243377 | Addition of testing methodology for Shared Spectrum | Qualcomm France | 38.533 | 3238 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243378 | Clean up of pending items in section 10 NR-U tests | Qualcomm France | 38.533 | 3239 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243379 | Update to Annex H for NR-U | Qualcomm France | 38.533 | 3240 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243380 | Clean up of pending items in section 11 NR-U tests | Qualcomm France | 38.533 | 3241 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243381 | Editorial update to Annex D for NR-U | Qualcomm France | 38.533 | 3242 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243382 | Update to OCNG modeling for NR-U | Qualcomm France | 38.533 | 3243 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243383 | Correction to gap-based measurement tests for RedCap | Qualcomm France | 38.533 | 3244 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243904 | Correction to gap-based measurement tests for RedCap | Qualcomm France | 38.533 | 3244 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243384 | Correction to stationary relaxed measurement criterion tests | Qualcomm France | 38.533 | 3245 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243385 | Correction to stationary relaxed measurement criterion inter-RAT tests | Qualcomm France | 38.533 | 3246 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-243386 | Correction to re-establishment test cases for RedCap | Qualcomm France | 38.533 | 3247 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243903 | Correction to re-establishment test cases for RedCap | Qualcomm France | 38.533 | 3247 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243387 | Correction to event-triggered measurement test cases for RedCap | Qualcomm France | 38.533 | 3248 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-243388 | Editorial correction to section 18 title | Qualcomm France | 38.533 | 3249 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243389 | Update to gap-based measurement reporting tests for HST | Qualcomm France | 38.533 | 3250 | - | Rel-18 | F | TEI17_Test, NR_HST_FR1_enh-UEConTest | revised |
| R5-243711 | Update to gap-based measurement reporting tests for HST | Qualcomm France | 38.533 | 3250 | 1 | Rel-18 | F | TEI17_Test, NR_HST_FR1_enh-UEConTest | agreed |
| R5-243390 | Update to Rel.17 HST CA measurement reporting tests | Qualcomm France | 38.533 | 3251 | - | Rel-18 | F | TEI17_Test, NR_HST_FR1_enh-UEConTest | revised |
| R5-243901 | Update to Rel.17 HST CA measurement reporting tests | Qualcomm France | 38.533 | 3251 | 1 | Rel-18 | F | TEI17_Test, NR_HST_FR1_enh-UEConTest | agreed |
| R5-243391 | Correction to gap-based measurement tests | Qualcomm France | 38.533 | 3252 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243392 | Correction to SUL test cases | Qualcomm France | 38.533 | 3253 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243902 | Correction to SUL test cases | Qualcomm France | 38.533 | 3253 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243393 | Correction to Handover tests | Qualcomm France | 38.533 | 3254 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243443 | Correction to RedCap inter-RAT measurement test cases | Huawei, HiSilicon, Qualcomm | 38.533 | 3255 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243919 | Correction to RedCap inter-RAT measurement test cases | Huawei, HiSilicon, Qualcomm | 38.533 | 3255 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-243224 | CR to TS 38.551 on Annex A.2.3 | Apple (UK) Limited | 38.551 | 0010 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | revised |
| R5-243800 | CR to TS 38.551 on Annex A.2.3 | Apple (UK) Limited | 38.551 | 0010 | 1 | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-243227 | CR to TS 38.551 on Annex C, editorial | Apple (UK) Limited | 38.551 | 0011 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-243230 | CR to TS 38.551 on Annex G, editorial | Apple (UK) Limited | 38.551 | 0012 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-243232 | CR to TS 38.551 on EUT positioning | Apple (UK) Limited | 38.551 | 0013 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-243233 | CR to TS 38.551 on section 6.1.2 | Apple (UK) Limited | 38.551 | 0014 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-243312 | Clarification of Calibration and Ripple Test Procedures | Keysight Technologies UK Ltd, ETS-Lindgren | 38.551 | 0015 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-243313 | Clarification of Channel Model Coordinate System and Probe Placement | Keysight Technologies UK Ltd, OPPO | 38.551 | 0016 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | agreed |
| R5-243316 | On Testing with Battery | Keysight Technologies UK Ltd | 38.551 | 0017 | - | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | revised |
| R5-243801 | On Testing with Battery | Keysight Technologies UK Ltd | 38.551 | 0017 | 1 | Rel-17 | F | TEI17_Test, NR_MIMO_OTA-UEConTest | withdrawn |
| R5-243311 | Clarification of Calibration and Ripple Test Procedures | Keysight Technologies UK Ltd, ETS-Lindgren | 38.561 | 0006 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | revised |
| R5-243797 | Clarification of Calibration and Ripple Test Procedures | Keysight Technologies UK Ltd, ETS-Lindgren | 38.561 | 0006 | 1 | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | agreed |
| R5-243314 | Clarification of polynomial interpolation | Keysight Technologies UK Ltd, Rohde & Schwarz | 38.561 | 0007 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | revised |
| R5-243798 | Clarification of polynomial interpolation | Keysight Technologies UK Ltd, Rohde & Schwarz | 38.561 | 0007 | 1 | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | agreed |
| R5-243318 | On Testing with Battery | Keysight Technologies UK Ltd | 38.561 | 0008 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | withdrawn |
| R5-243407 | Updates across TS 38.561 V17.0.0 | Apple Benelux B.V. | 38.561 | 0009 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | revised |
| R5-243799 | Updates across TS 38.561 V17.0.0 | Apple Benelux B.V. | 38.561 | 0009 | 1 | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | agreed |
| R5-243408 | Editorial updates within TRP tests | Apple Benelux B.V. | 38.561 | 0010 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | agreed |
| R5-243409 | Editorial updates within TRS tests | Apple Benelux B.V. | 38.561 | 0011 | - | Rel-17 | F | TEI17_Test, NR_FR1_TRP_TRS-UEConTest | agreed |
| R5-242129 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | 38.903 | 0705 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-243921 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | 38.903 | 0705 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-244007 | Addition of TT for MG enhancements TC 6.6.18.3 | MediaTek Inc. | 38.903 | 0705 | 2 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-242130 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | 38.903 | 0706 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-243922 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | 38.903 | 0706 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-244008 | Addition of TT for MG enhancements TC 6.6.18.4 | MediaTek Inc. | 38.903 | 0706 | 2 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-242218 | Update TC 14.2.3 and TC 14.2.10 TT analysis results | CATT | 38.903 | 0707 | - | Rel-18 | F | NR_pos_enh-UEConTest | revised |
| R5-243769 | Update TC 14.2.3 and TC 14.2.10 TT analysis results | CATT | 38.903 | 0707 | 1 | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242219 | Update TC 14.2.4 TT analysis results | CATT | 38.903 | 0708 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243792 | Update TC 14.2.4 TT analysis results | CATT | 38.903 | 0708 | 1 | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242220 | Update TC 14.2.8 and TC 14.2.9 TT analysis results | CATT | 38.903 | 0709 | - | Rel-18 | F | NR_pos_enh-UEConTest | revised |
| R5-243770 | Update TC 14.2.8 and TC 14.2.9 TT analysis results | CATT | 38.903 | 0709 | 1 | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242221 | Update TC 14.3.3 and TC 14.3.8 TT analysis results | CATT | 38.903 | 0710 | - | Rel-18 | F | NR_pos_enh-UEConTest | revised |
| R5-243771 | Update TC 14.3.3 and TC 14.3.8 TT analysis results | CATT | 38.903 | 0710 | 1 | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242222 | Update TC 14.3.4 TT analysis results | CATT | 38.903 | 0711 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243793 | Update TC 14.3.4 TT analysis results | CATT | 38.903 | 0711 | 1 | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242223 | Update TC 14.3.7 TT analysis results | CATT | 38.903 | 0712 | - | Rel-18 | F | NR_pos_enh-UEConTest | revised |
| R5-243772 | Update TC 14.3.7 TT analysis results | CATT | 38.903 | 0712 | 1 | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242224 | Update TC 15.2.1 TT analysis results | CATT | 38.903 | 0713 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242225 | Update TC 15.2.2 TT analysis results | CATT | 38.903 | 0714 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242226 | Update TC 15.2.3 TT analysis results | CATT | 38.903 | 0715 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243794 | Update TC 15.2.3 TT analysis results | CATT | 38.903 | 0715 | 1 | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242227 | Update TC 15.2.4 TT analysis results | CATT | 38.903 | 0716 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | revised |
| R5-243795 | Update TC 15.2.4 TT analysis results | CATT | 38.903 | 0716 | 1 | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242228 | Update TC 15.2.8, 15.2.9 and 15.2.10 TT analysis results | CATT | 38.903 | 0717 | - | Rel-18 | F | NR_pos_enh-UEConTest | revised |
| R5-243773 | Update TC 15.2.8, 15.2.9 and 15.2.10 TT analysis results | CATT | 38.903 | 0717 | 1 | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242229 | Update TC 15.3.1 TT analysis results | CATT | 38.903 | 0718 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242230 | Update TC 15.3.2 TT analysis results | CATT | 38.903 | 0719 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242231 | Update TC 15.3.5 TT analysis results | CATT | 38.903 | 0720 | - | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242232 | Update TC 15.3.6 TT analysis results | CATT | 38.903 | 0721 | - | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242233 | Update TC 16.2.3, 16.2.7 and 16.2.8 TT analysis results | CATT | 38.903 | 0722 | - | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242234 | Update TC 16.2.4 TT analysis results | CATT | 38.903 | 0723 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242235 | Update TC 16.3.2 TT analysis results | CATT | 38.903 | 0724 | - | Rel-18 | F | TEI16_Test, NR_pos-UEConTest | agreed |
| R5-242236 | Update TC 16.3.4 TT analysis results | CATT | 38.903 | 0725 | - | Rel-18 | F | NR_pos_enh-UEConTest | agreed |
| R5-242259 | Pending UL MIMO update in 38.903 | Keysight Technologies | 38.903 | 0726 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242262 | FR2c MU definition in 38.903 | Keysight Technologies | 38.903 | 0727 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243730 | FR2c MU definition in 38.903 | Keysight Technologies | 38.903 | 0727 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242265 | PC5 FR2 MU definition in 38.903 | Keysight Technologies | 38.903 | 0728 | - | Rel-18 | F | NR_FR2_FWA_Bn257_Bn258-UEConTest | revised |
| R5-243723 | PC5 FR2 MU definition in 38.903 | Keysight Technologies | 38.903 | 0728 | 1 | Rel-18 | F | NR_FR2_FWA_Bn257_Bn258-UEConTest | agreed |
| R5-242307 | Removal of technical content in TR 38.903 v16.14.0 and substitution with pointer to the next Release | ETSI Secretariat | 38.903 | 0729 | - | Rel-16 | F | TEI16_Test | agreed |
| R5-242439 | TT analysis for FR1 CQI reporting test cases with inter-cell interference | China Telecom, Qualcomm | 38.903 | 0730 | - | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-242581 | Introduction of Test Tolerance analysis for 7.6.15.3 FR2 SA event triggered reporting tests for concurrent gaps | Anritsu | 38.903 | 0731 | - | Rel-18 | F | NR_MG_enh-UEConTest | revised |
| R5-243745 | Introduction of Test Tolerance analysis for 7.6.15.3 FR2 SA event triggered reporting tests for concurrent gaps | Anritsu | 38.903 | 0731 | 1 | Rel-18 | F | NR_MG_enh-UEConTest | agreed |
| R5-242608 | Addition of Test Tolerances analysis for NR-U redirection test cases | Nokia | 38.903 | 0732 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243752 | Addition of Test Tolerances analysis for NR-U redirection test cases | Nokia | 38.903 | 0732 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242629 | Test Tolerance analysis for SRS carrier based switching test cases | Ericsson | 38.903 | 0733 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242630 | Test Tolerance analysis for event triggered reporting test without gap under non-DRX test case 7.6.2.10 | Ericsson | 38.903 | 0734 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-243759 | Test Tolerance analysis for event triggered reporting test without gap under non-DRX test case 7.6.2.10 | Ericsson | 38.903 | 0734 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242631 | Test Tolerance analysis for event triggered reporting test without gap under DRX test case 7.6.2.11 | Ericsson | 38.903 | 0735 | - | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242700 | Addition of test tolerance analysis for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.903 | 0736 | - | Rel-18 | F | NR_RRM_enh-UEConTest | revised |
| R5-244004 | Addition of test tolerance analysis for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | Sporton | 38.903 | 0736 | 1 | Rel-18 | F | NR_RRM_enh-UEConTest | agreed |
| R5-242730 | Update of the QoQZ MU | ROHDE & SCHWARZ | 38.903 | 0737 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242731 | Removal of the reference to efficiency calibration in QoQZ | ROHDE & SCHWARZ | 38.903 | 0738 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242906 | TT analysis for RedCap RRM test case 16.2.1.1 | Huawei, HiSilicon | 38.903 | 0739 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-243747 | TT analysis for RedCap RRM test case 16.2.1.1 | Huawei, HiSilicon | 38.903 | 0739 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-244016 | TT analysis for RedCap RRM test case 16.2.1.1 | Huawei, HiSilicon | 38.903 | 0739 | 2 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242907 | TT analysis for RedCap RRM test case 16.2.1.2 | Huawei, HiSilicon | 38.903 | 0740 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | revised |
| R5-244017 | TT analysis for RedCap RRM test case 16.2.1.2 | Huawei, HiSilicon | 38.903 | 0740 | 1 | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | agreed |
| R5-242908 | TT analysis for RedCap RRM test case 17.2.1.1 | Huawei, HiSilicon | 38.903 | 0741 | - | Rel-18 | F | TEI17_Test, NR_redcap_plus_ARCH-UEConTest | withdrawn |
| R5-243001 | Update for FR2c MU | Anritsu | 38.903 | 0742 | - | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | revised |
| R5-243832 | Update for FR2c MU | Anritsu | 38.903 | 0742 | 1 | Rel-18 | F | TEI16_Test, NR_bands_BW_R16-UEConTest | agreed |
| R5-243003 | QoQZ for QZ size 40 cm | Anritsu | 38.903 | 0743 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | withdrawn |
| R5-243101 | Documentation of MU for n259 | ROHDE & SCHWARZ | 38.903 | 0744 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243829 | Documentation of MU for n259 | ROHDE & SCHWARZ | 38.903 | 0744 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243104 | Documentation of MU for UL MIMO | ROHDE & SCHWARZ | 38.903 | 0745 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243320 | Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases 10.4.2.3, 10.4.2.4, 10.4.2.5, 10.4.2.6 | Qualcomm Technologies Int | 38.903 | 0746 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243893 | Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases 10.4.2.3, 10.4.2.4, 10.4.2.5, 10.4.2.6 | Qualcomm Technologies Int | 38.903 | 0746 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243321 | Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases with DRX 10.4.2.7, 10.4.2.8, 10.4.2.9, 10.4.2.10 | Qualcomm Technologies Int | 38.903 | 0747 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243894 | Test Tolerance analysis for NR-U inter-frequency measurement reporting test cases with DRX 10.4.2.7, 10.4.2.8, 10.4.2.9, 10.4.2.10 | Qualcomm Technologies Int | 38.903 | 0747 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243322 | Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1 | Qualcomm Technologies Int | 38.903 | 0748 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243736 | Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1 | Qualcomm Technologies Int | 38.903 | 0748 | 1 | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243996 | Test Tolerance analysis for NR-U RSRP intra-frequency accuracy test cases 10.5.1.1 and 11.6.1.1 | Qualcomm Technologies Int | 38.903 | 0748 | 2 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243323 | Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2 | Qualcomm Technologies Int | 38.903 | 0749 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243737 | Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2 | Qualcomm Technologies Int | 38.903 | 0749 | 1 | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243997 | Test Tolerance analysis for NR-U RSRP inter-frequency accuracy test case 10.5.1.2 | Qualcomm Technologies Int | 38.903 | 0749 | 2 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243324 | Test Tolerance analysis for NR-U RSRQ intra-frequency accuracy test cases 10.5.2.1 and 11.6.2.1 | Qualcomm Technologies Int | 38.903 | 0750 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243738 | Test Tolerance analysis for NR-U RSRQ intra-frequency accuracy test cases 10.5.2.1 and 11.6.2.1 | Qualcomm Technologies Int | 38.903 | 0750 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243325 | Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2 | Qualcomm Technologies Int | 38.903 | 0751 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243739 | Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2 | Qualcomm Technologies Int | 38.903 | 0751 | 1 | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243998 | Test Tolerance analysis for NR-U RSRQ inter-frequency accuracy test cases 10.5.2.2 and 11.6.2.2 | Qualcomm Technologies Int | 38.903 | 0751 | 2 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243326 | Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3 | Qualcomm Technologies Int | 38.903 | 0752 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243740 | Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3 | Qualcomm Technologies Int | 38.903 | 0752 | 1 | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243999 | Test Tolerance analysis for NR-U SINR intra-frequency accuracy test cases 10.5.3.1 and 10.5.3.3 | Qualcomm Technologies Int | 38.903 | 0752 | 2 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243327 | Test Tolerance analysis for NR-U SINR inter-frequency accuracy test cases 10.5.3.2 and 11.6.3.2 | Qualcomm Technologies Int | 38.903 | 0753 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243741 | Test Tolerance analysis for NR-U SINR inter-frequency accuracy test cases 10.5.3.2 and 11.6.3.2 | Qualcomm Technologies Int | 38.903 | 0753 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243328 | Test Tolerance analysis for NR-U L1-RSRP accuracy test case 10.5.4.1 | Qualcomm Technologies Int | 38.903 | 0754 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243742 | Test Tolerance analysis for NR-U L1-RSRP accuracy test case 10.5.4.1 | Qualcomm Technologies Int | 38.903 | 0754 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243399 | Addition of TTs for NR-U intra-frequency measurement reporting test | Qualcomm France | 38.903 | 0755 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243895 | Addition of TTs for NR-U intra-frequency measurement reporting test | Qualcomm France | 38.903 | 0755 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243400 | Addition of TTs for NR-U Timing Accuracy Test | Qualcomm France | 38.903 | 0756 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243401 | Addition of TTs for NR-U Timing Advance Test | Qualcomm France | 38.903 | 0757 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243402 | Addition of TTs for NR-U L1-RSRP measurement reporting tests | Qualcomm France | 38.903 | 0758 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243743 | Addition of TTs for NR-U L1-RSRP measurement reporting tests | Qualcomm France | 38.903 | 0758 | 1 | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-244000 | Addition of TTs for NR-U L1-RSRP measurement reporting tests | Qualcomm France | 38.903 | 0758 | 2 | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-244021 | Addition of TTs for NR-U L1-RSRP measurement reporting tests | Qualcomm France | 38.903 | 0758 | 3 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243403 | Addition of TTs for SCell activation and deactivation tests | Qualcomm France | 38.903 | 0759 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243404 | Addition of TTs for PSCell addition and release test | Qualcomm France | 38.903 | 0760 | - | Rel-18 | F | NR_unlic-UEConTest | revised |
| R5-243753 | Addition of TTs for PSCell addition and release test | Qualcomm France | 38.903 | 0760 | 1 | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-243418 | MU TT updates for NTN RF tests | Apple Benelux B.V. | 38.903 | 0761 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-243433 | Addition of MU/TT analysis for inter-cell CRS interference | Qualcomm Inc | 38.903 | 0762 | - | Rel-18 | F | NR_demod_enh2-UEConTest | agreed |
| R5-242245 | Addition of reference sensitivity TP analysis for CA_n28A-n40A | KDDI Corporation | 38.905 | 0882 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243820 | Addition of reference sensitivity TP analysis for CA_n28A-n40A | KDDI Corporation | 38.905 | 0882 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242246 | Addition of reference sensitivity TP analysis for CA_n3A-n28A-n41A-n77A | KDDI Corporation | 38.905 | 0883 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242429 | Update of test case 6.5F.3.3 and 6.5F.2.4.2 | Ericsson | 38.905 | 0884 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242435 | Update reference sensitivity test cases for ENDC band configurations with PC2 UL | Verizon | 38.905 | 0885 | - | Rel-18 | F | ENDC_PC2_R17_xLTE_yNR-UEConTest | agreed |
| R5-242495 | Addition of new NR-U test cases | Ericsson | 38.905 | 0886 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242497 | Addition of reference sensitivity TP analysis for new PC2 EN-DC combs within FR1 | KDDI Corporation | 38.905 | 0887 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242513 | Update MU and TT for NR NTN test cases | Keysight Technologies | 38.905 | 0888 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | withdrawn |
| R5-242525 | TP analysis for NR-U test case 6.4F.2.2, carrier leakage | Ericsson | 38.905 | 0889 | - | Rel-18 | F | NR_unlic-UEConTest | agreed |
| R5-242536 | Addition of TP analysis for CA_n48C | Ericsson | 38.905 | 0890 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-242537 | Addition of checklist for intra band CA in Annex B | Ericsson | 38.905 | 0891 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243883 | Addition of checklist for intra band CA in Annex B | Ericsson | 38.905 | 0891 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-242710 | Test point analysis for spurious emissions CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A | WE Certification Oy, AT&T | 38.905 | 0892 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R17-UEConTest | agreed |
| R5-242916 | Addition of REFSENS TP analysis for 2CC ENDC for n78A and n79A | NTT DOCOMO INC. | 38.905 | 0893 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242917 | Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A | NTT DOCOMO INC. | 38.905 | 0894 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-243827 | Addition of REFSENS TP analysis for 3CC ENDC for n78A and n79A | NTT DOCOMO INC. | 38.905 | 0894 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-242933 | Addition of test point analysis for 6.2H.3.1 MOP for inter-band CA with MIMO | Huawei, HiSilicon | 38.905 | 0895 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242934 | Addition of test point analysis for 6.2L.3.1 MOP for inter-band CA with TxD | Huawei, HiSilicon | 38.905 | 0896 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242935 | Addition of test point analysis for 6.2H.1.3 MOP for inter-band EN-DC with MIMO | Huawei, HiSilicon | 38.905 | 0897 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242936 | Addition of test point analysis for 6.2L.1.3 MOP for inter-band EN-DC with TxD | Huawei, HiSilicon | 38.905 | 0898 | - | Rel-18 | F | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest | agreed |
| R5-242939 | Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC3 | NTT DOCOMO INC. | 38.905 | 0899 | - | Rel-18 | F | TEI15_Test | withdrawn |
| R5-242943 | Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC2 | NTT DOCOMO INC. | 38.905 | 0900 | - | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | revised |
| R5-243826 | Addition of REFSENS TP analysis for CA_n78-n79 with 1UL in PC2 | NTT DOCOMO INC. | 38.905 | 0900 | 1 | Rel-18 | F | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest | agreed |
| R5-243032 | Update reference sensitivity TC for CA_n3A-n8A-n78A | China Unicom | 38.905 | 0901 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | revised |
| R5-243616 | Update reference sensitivity TC for CA_n3A-n8A-n78A | China Unicom | 38.905 | 0901 | 1 | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243044 | Update REFSENSE TP analysis for CA_n79C | CMCC | 38.905 | 0902 | - | Rel-18 | F | NR_CADC_NR_LTE_DC_R16-UEConTest | agreed |
| R5-243051 | Update REFSENSE TP analysis for CA_n39A-n41A | CMCC | 38.905 | 0903 | - | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | revised |
| R5-243836 | Update REFSENSE TP analysis for CA_n39A-n41A | CMCC | 38.905 | 0903 | 1 | Rel-18 | F | LTE_NR_Simult_RxTx_R18-UEConTest | agreed |
| R5-243066 | TP analysis for PC1.5 n39 A-MPR and A-SE | CMCC | 38.905 | 0904 | - | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | revised |
| R5-243614 | TP analysis for PC1.5 n39 A-MPR and A-SE | CMCC | 38.905 | 0904 | 1 | Rel-18 | F | HPUE_NR_FR1_TDD_R18-UEConTest | agreed |
| R5-243131 | Adding test point information of 6.5H.1.1 | Huawei, HiSilicon | 38.905 | 0905 | - | Rel-18 | F | TEI17_Test, NR_RF_FR1_enh-UEConTest | agreed |
| R5-243135 | Addition of TP analysis for NR V2X MOP | Huawei, HiSilicon | 38.905 | 0906 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243139 | Addition of TP analysis for NR V2X Configured transmit power | Huawei, HiSilicon | 38.905 | 0907 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243143 | Addition of TP analysis for NR V2X ON OFF time mask | Huawei, HiSilicon | 38.905 | 0908 | - | Rel-18 | F | 5G_V2X_NRSL_eV2XARC-UEConTest | agreed |
| R5-243147 | Addition of TP analysis for NR V2X MPR for PC2 and intra-band concurrent operation | Huawei, HiSilicon | 38.905 | 0909 | - | Rel-18 | F | NR_SL_enh-UEConTest | revised |
| R5-243621 | Addition of TP analysis for NR V2X MPR for PC2 and intra-band concurrent operation | Huawei, HiSilicon | 38.905 | 0909 | 1 | Rel-18 | F | NR_SL_enh-UEConTest | agreed |
| R5-243149 | Addition of TP analysis for NR V2X AMPR for PC2 and intra-band concurrent operation | Huawei, HiSilicon | 38.905 | 0910 | - | Rel-18 | F | NR_SL_enh-UEConTest | revised |
| R5-243622 | Addition of TP analysis for NR V2X AMPR for PC2 and intra-band concurrent operation | Huawei, HiSilicon | 38.905 | 0910 | 1 | Rel-18 | F | NR_SL_enh-UEConTest | agreed |
| R5-243203 | Updating the TP analysis for AMPR testing for CA_n41A-n79A | Huawei, HiSilicon | 38.905 | 0911 | - | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | revised |
| R5-243624 | Updating the TP analysis for AMPR testing for CA_n41A-n79A | Huawei, HiSilicon | 38.905 | 0911 | 1 | Rel-18 | F | TEI15_Test, 5GS_NR_LTE-UEConTest | agreed |
| R5-243206 | Updating TP analysis for AMPR testing for NS_43 | Huawei, HiSilicon | 38.905 | 0912 | - | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | revised |
| R5-243613 | Updating TP analysis for AMPR testing for NS_43 | Huawei, HiSilicon | 38.905 | 0912 | 1 | Rel-18 | F | HPUE_NR_FR1_FDD_R18-UEConTest | agreed |
| R5-243214 | Adding TP analysis for new FR1 test case 6.3A.3.6 and 6.3A.3.7 | Huawei, Hisilicon | 38.905 | 0913 | - | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243618 | Adding TP analysis for new FR1 test case 6.3A.3.6 and 6.3A.3.7 | Huawei, Hisilicon | 38.905 | 0913 | 1 | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-243215 | Adding TP analysis for new SUL test case 6.3C.3.6 | Huawei, Hisilicon | 38.905 | 0914 | - | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243619 | Adding TP analysis for new SUL test case 6.3C.3.6 | Huawei, Hisilicon | 38.905 | 0914 | 1 | Rel-18 | F | NR_MC_enh-UEConTest | revised |
| R5-243796 | Adding TP analysis for new SUL test case 6.3C.3.6 | Huawei, Hisilicon | 38.905 | 0914 | 2 | Rel-18 | F | NR_MC_enh-UEConTest | agreed |
| R5-243448 | Update Test Point analysis for NR NTN test cases | Keysight Technologies | 38.905 | 0915 | - | Rel-18 | F | NR_NTN_solutions_plus_CT-UEConTest | agreed |
| R5-243653 | Addition of REFSENS TP analysis for PC3 | NTT DOCOMO INC. | 38.905 | 0916 | - | Rel-18 | F | TEI15_Test | agreed |

## Annex C: Lists of liaisons

### C1: Incoming liaison statements

6 incoming LSs at RAN5#103

| Document | Original | Title | From | Decision | Reply TDoc |
| --- | --- | --- | --- | --- | --- |
| R5-242116 | RT(24)093032 | LS on development of FRMCS conformance test cases by ETSI TTF | TC RT | noted | (none) |
| R5-242117 | TCCE(24)000026r2 | LS on development of Server Side Conformance for Mission Critical Services by ETSI TTF | TCCE | noted | (none) |
| R5-242118 | R4-2406079 | Reply LS on TT work for Rel-18 NR FR1 TRP TRS | TSG WG RAN4 | noted | (none) |
| R5-242119 | R4-2406085 | LS on Head Phantoms for XR devices OTA testing | TSG WG RAN4 | noted | (none) |
| R5-242244 | CAG-24-217r3 | LS RAN4 on 5G NR FR1 OTA Technical Specifications | GCF CAG | noted | (none) |
| R5-243449 | TSGIoT66a_003v3 | LS regarding the development of NB-IoT test cases for Release 15 and Release 16 | GSMA TSGIoT | noted | (none) |

### C2: Outgoing liaison statements

1 outgoing LSs at RAN5#103

| Document | Title | To | Cc | reply to i/c LS |
| --- | --- | --- | --- | --- |
| R5-243972 | LS on Development of NB-IoT test cases for Release 15 and Release 16 | GSMA TSG Internet of Things (IoT) group | TSG WG RAN4, GCF SG, GCF CAG |  |

## Annex D: List of agreed/approved new and revised Work Items

11 new WIDs were endorsed at RAN5#103, 3 WIDs revised

| Document | Title | Source | new/revised |
| --- | --- | --- | --- |
| R5-243451 | New WID on UE Conformance - Further NR mobility enhancements | MediaTek Inc., Apple, China Telecom | WID new |
| R5-243452 | New WID on UE Conformance - Introduction of FDD LTE band (L+S band) for IoT NTN operation | MediaTek Inc., China Telecom | WID new |
| R5-243453 | New WID on UE Conformance - Introduction of the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) for IoT NTN | MediaTek Inc., Inmarsat | WID new |
| R5-243454 | New WID on UE Conformance Test Aspects - Expanded and improved NR positioning | CATT, CAICT | WID new |
| R5-243455 | New WID on UE Conformance – NR demodulation performance evolution | China Telecom, Qualcomm, Samsung | WID new |
| R5-243457 | New WID on UE Conformance - Mobile Terminated-Small Data Transmission (MT-SDT) for NR | Qualcomm Incorporated, China Telecom, ZTE | WID new |
| R5-243460 | New WID on UE Conformance - Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC) | Apple, ROHDE & SCHWARZ, Vivo | WID new |
| R5-243461 | New WID on UE Conformance - In-Device Co-existence (IDC) enhancements for NR and MR-DC | CMCC, Xiaomi | WID new |
| R5-243462 | UE Conformance - Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | Apple, CAICT, Keysight Technologies | WID new |
| R5-243463 | New WID on UE Conformance 4Rx for NR bands for FWA UEs (up to 2.6GHz) and handheld UEs (1GHz to 2.6GHz) | ZTE, China Telecom | WID new |
| R5-243465 | New WID on UE Conformance - Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception | Qualcomm Inc | WID new |
| R5-243056 | Revised WID on UE Conformance - IoT NTN enh plus CT1 aspects | CMCC | WID revised |
| R5-243190 | Revised WID on UE Conformance - Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258 | Huawei, Hisilicon | WID revised |
| R5-243977 | Revised WID for UE Conformance - Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | China Telecom ,ZTE, MTK | WID revised |

## Annex E: List of draft Technical Specifications and Reports

None

## Annex F: List of action items

## Joint:

## Action Points at RAN5#103

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#103.01 | JOINT | Ensure withdrawn CR's are not processed for implementation into RAN5 specification. [MCC to undo the changes implemented into TS38.521-3 from withdrawn CR in R5-235958] | MCC | R5-235958 | RAN5#104 | Pending |

## SIG:

## Action Points at RAN5#103

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#103.01 | SIG | Review and update spec language in e-SIM related procedures | Qualcomm, Anritsu, Keysight, R&S, MediaTek, TF160, CATT | R5-243466 | RAN5#104 | Pending |
| AP#103.02 | SIG | Review and address inconsistencies in the application of conditions in TS 34.229-5 | Ericsson, Qualcomm, TF160 | R5-242620 | RAN5#104 | Pending |
| AP#103.03 | SIG | Review and update (if needed) NR 5GC CAG test cases verification of CAG support as part of the Registration procedure in the test sequence | R&S, TF160 | R5-243588 | RAN5#104 | Pending |

## Action Points at RAN5#102

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#102.01 | SIG | Review and update TP4 of TS 38.523-1 TC 9.1.5.1.4 Initial registration / 5GS services / MICO mode / TAI list handling to make it a testable requirement | R&S, TF160, Keysight, MediaTek | R5-241687 | RAN5#103 | Closed R5-242487 |
| AP#102.02 | SIG | Resolve open issues captured in Editor’s note for TS 38.523-1 Emergency Call TCs 11.1.7 and 11.4.12 | Qualcomm, Huawei, TF160, R&S, Anritsu | R5-241691 R5-241692 | RAN5#103 | Closed R5-243591 R5-243593 |

## Action Points at RAN5#101

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#101.01 | SIG | Investigate on the solution to how to treat the capability of ER-NSSAI based on the network vendors' support. | CMCC, Qualcomm, CATT, Huawei, Hisilicon, MCC TF160, Ericsson, MediaTek | R5-236570 | RAN5#102 | Closed R5-240663 |
| AP#101.02 | SIG | Investigate on the solution of which release/WIs current test cases in 10.1.8 (NSAC) should be allocated. | CATT, CMCC, Qualcomm, MCC TF160, ZTE | R5-236570 | RAN5#103 | Closed R5-240663 R5-243487 R5-243590 R5-243552 |
| AP#101.03 | SIG | Investigate to decide on choosing dynamic/static/mixed signalling test environments for NGSO NTN testing. | Mediatek, MCC TF160，Qualcomm, Keysight, R&S, Anritsu | R5-237381 | RAN5#102 | Closed R5-240162 |
| AP#101.04 | SIG | Investigate on a proper title of TS38.508-1 cl4.7 to show the content of this sub-clause including NAS and V2X messages. | MCC TF160, CATT, Huawei, Hisilicon | R5-237433 | RAN5#102 | Closed R5-240432 |

## RF:

## Action Points at RAN5#103

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#103.21 | RF | Define Spurious Emissions QoQZ MU for 40cm QZ and NTC | Anritsu, Keysight, R&S | R5-243002 | RAN5#104 | Open |
| AP#103.22 | RF | Define QoQZ MU for 40cm QZ and ETC (up to FR2d) | Anritsu, Keysight, R&S | R5-243002 | RAN5#105 | Open |

## Action Points at RAN5#102

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#102.21 | RF | Update Annex C how to calculate test frequencies for UL CA (Intraband NC) | E///, KEYS, R&S, Qualcomm, Huawei, CMCC, China Unicom | R5-241364 R5-242533 | RAN5#103 | Closed |

## Action Points at RAN5#101

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#101.22 | RF | Check the simplication of UE co-existence for UL CA in TS38.521-1/-3, TS36.521-1 | China Unicom, CAICT, CMCC, China Telecom, Huawei, Nokia | R5-236443 R5-241367/8 R5-243260/1 | RAN5#106 | Open |

## Action Points at RAN5#99

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#99.21 | RF | Define MTSU for regulatory test cases (MOP, EIRP, TRP, Tx OFF power, Freq error, OBW, SEM, ACLR, Ref Sense, ACS, Inband Blocking, Tx/Rx Spurious emissions) for FR2 Band n259 | Anritsu, R&S, KEYS, DCM | R5-232983 R5-233174 R5-233971-5 R5-235140 R5-234891-3 R5-237725/7 R5-237731 R5-236067 R5-237733 R5-237734 R5-237723 R5-241083/4/5/6 R5-240401/2/3/4/5 R5-241344/5 R5-243100-3102 R5-242261-63 R5-242997-243001 | RAN5#103 | Closed |

## Action Points at RAN5#97

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#97.25 | RF | Investigate and come up with testability proposal to ensure 1x2 channel in FR2 RRM tests is achieved with UE receiving signal power on both its Rx branches | Keysight, R&S, Anritsu, Qualcomm | R5-226658 R5-230934 R5-232673 R5-234680 R5-237848 R5-241237 | RAN5#104 | Open |
| AP#97.27 | RF | Update on-on transient period 6.4.2.1a in TS38.521-1 to address issues identified in R5-226794 | Anritsu, Qualcomm, Ericsson, R&S, Apple | R5-226794 R5-230058 R5-230217 R5-230218 R5-232987 R5-234898/9 R5-234178 R5-237653 R5-237859 R5-241088 R5-241445 R5-240618 R5-241356 R5-242520 R5-243013 | RAN5#103 | Closed |

## Annex G: Void

## Annex H: List of participants

267 delegates and officials attended the RAN5#103 meeting. (online attendees are not recoded)

| TITLE | Family Name | Given Name | Email | Organization Represented |
| --- | --- | --- | --- | --- |
| Dr. | Mikaeil | Ahmed | ahmed.mikaeil@tcl.com | TCL |
| Mr. | Agarwal | Deepak | deepak@cewit.org.in | CEWiT |
| Mr. | Aoyagi | Kenichiro | kenichiro.aoyagi@rakuten.com | Rakuten Mobile, Inc |
| Mr. | Balasubramanian | Vijay | vijayb@qti.qualcomm.com | Arriver Software AB |
| Mr. | BASAIER | JIALADE | basejld@chinaunicom.cn | VSENS |
| Mr. | Bhise | Parikshit | parikshit.bhise@rohde-schwarz.com | ROHDE & SCHWARZ |
| Mr. | Borsato | Ronald | rb354e@att.com | AT&T |
| Dr. | Cao | Jeffrey (Jianfei) | caojianfei@oppo.com | Reflection B.V. |
| Mr. | Chen | Ben | chenben@bjtu.edu.cn | BJTU |
| Miss | chen | jingjing | chenjingjing@chinamobile.com | China Mobile Group Device Co. |
| Miss | Chen | Liping | liping.chen@hughes.com | EchoStar |
| Mr. | Chen | Wenhong | chenwenhong@oppo.com | OPPO Beijing |
| Dr. | Chen | Xiang (Steven) | steven.x.chen@apple.com | Apple Switzerland AG |
| Mr. | Chen | Xiaozhong | chenxiaozhong@catt.cn | CATT |
| Dr. | Chen | Zhe | tom.chenzhe@samsung.com | Samsung Electronics Iberia SA |
| Mr. | Chen | Ziwei | chenziwei@chinamobile.com | China Mobile (Suzhou) Software |
| Mr. | Cheng | Ivan | IvanCheng@sporton.com.tw | Sporton International Inc |
| Mr. | Cheruvu | Bharadwaj Kumar | bcheruvu@qti.qualcomm.com | QUALCOMM Europe Inc. - Italy |
| Mr. | Choi | Jin | HyungjinChoi@eurofins.com | Eurofins KCTL |
| Mr. | Coroescu | Liviu | liviu.coroescu@rohde-schwarz.com | ROHDE & SCHWARZ |
| Mr. | Cui | Shengjiang | cuishengjiang@oppo.com | OPPO Beijing |
| Mr. | Das | Siddharth | siddharth.1.das@nokia.com | Nokia Solutions & Networks (I) |
| Mr. | Doczkat | Martin | martin.doczkat@fcc.gov | FCC |
| Ms. | Dong | Xufei | dongxf37@chinaunicom.cn | CITC |
| Mr. | Du | Hao | duhao.txyjy@vivo.com | vivo Software Technology |
| Mr. | Feng | Sanjun | fengsanjun@vivo.com | vivo Mobile Communication (H) |
| Miss | Fernandez Martos | Flores | flores_fernandez@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Fortes Lopez | Jose M. | Jose.Fortes@rohde-schwarz.com | ROHDE & SCHWARZ |
| Mr. | fu | danbo | Danbo.Fu@mediatek.com | MediaTek Beijing Inc. |
| Mrs. | Fu | Yanze | yanze.fu@samsung.com | Samsung Electronics Czech |
| Mr. | GAO | Lei | gaol8@chinatelecom.cn | Esurfing IoT |
| Mr. | Genoud | Olivier | olivier.genoud@etsi.org | ETSI |
| Mr. | Gowda | Pradeep | pgowda@qti.qualcomm.com | Qualcomm Incorporated |
| Mr. | Grant | Marc | marc.grant@att.com | AT&T |
| Mr. | Grunditz | Håkan | hakan.grunditz@ericsson.com | Ericsson Hungary Ltd |
| Ms. | Gu | Chunying | guchunying@huawei.com | Huawei Technologies Sweden AB |
| Mr. | Hagenfeldt | Calle | calle.hagenfeldt@ericsson.com | Ericsson Limited |
| Dr. | HAN | Xingyu | xingyu.han@samsung.com | Samsung R&D Institute UK |
| Miss | Hao | Yuxin | haoyuxin@huawei.com | Huawei Telecommunication India |
| Dr. | Harris | Paul | paul.harris@viavisolutions.com | VIAVI Solutions |
| Dr. | Hertel | Thorsten | thorsten.hertel@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Hitomi | Shinya | hitomishinya@murata.com | Murata Manufacturing Co Ltd. |
| Mr. | hu | Youjun | hu.youjun1@zte.com.cn | ZTE |
| Dr. | Jha | Satish | satish.jha@firstnet.gov | FirstNet |
| Dr. | Jiang | Zheng | jiangzheng@hotmail.com | E-surfing Digital |
| Mr. | Jing | Xiangwei | jing.xiangwei@zte.com.cn | Sanechips |
| Mr. | John | Jacob | Jacob.John@motorola.com | Motorola Mobility España SA |
| Mr. | Jönsson | Bo | bo.jonsson@ericsson.com | Ericsson LM |
| Mr. | Kalahasti | Narendra | narendra.kalahasti@anritsu.com | Anritsu EMEA Ltd |
| Mr. | Kamei | Takuto | takuto.kamei.uk@nttdocomo.com | DOCOMO Communications Lab. |
| Mr. | Kanchan | Mohit | mohit.kanchan@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Kangas | Matti | matti.kangas@nokia.com | Nokia France |
| Mr. | Karlsson | Petter | petter.karlsson@ericsson.com | Ericsson Limited |
| Mr. | Kato | Shun | syun.katou.fv@nttdocomo.com | NTT DOCOMO INC. |
| Mr. | Kim | Taekhoon | kuhn.kim@samsung.com | Samsung Research America |
| Mr. | Kitagawa | Ryu | ryuu.kitagawa.pn@nttdocomo.com | NTT Advanced Technology Corpor |
| Mr. | Kolodziej | Kuba | kuba.kolodziej@ericsson.com | Ericsson España S.A. |
| Mr. | Kwak | Phil | pkkwak@ktl.re.kr | Korea Testing Laboratory |
| Mr. | Lai | Kenny | kenny.lai@sgs.com | SGS Wireless |
| Ms. | Lam | Maria | maria.lam@verizonwireless.com | Verizon Switzerland AG |
| Mr. | Li | Bozhi | bozhi.li@samsung.com | Samsung Nanjing |
| Ms. | LI | PEI | lip104@chinaunicom.cn | CU Digital Technology |
| Mr. | Li | Qiming | li_qiming@apple.com | Apple (Guizhou) |
| Mr. | LI | Xiaoqiang | ben@cybercore.cn | Cybercore |
| Miss | Li | Yan | liyanwx@chinamobile.com | China Mobile (Suzhou) Software |
| Miss | Li | Ying | liying@srtc.org.cn | SRTC |
| Mr. | Li | Ziyang | li.ziyang1@zte.com.cn | ZTE Japan K.K. |
| Dr. | Lim | JongHyuk | jhlim78@korea.kr | National Radio Research Agency |
| Mr. | Lim | Suhwan | suhlim@meta.com | Facebook Japan G.K. |
| Mr. | Lin | Hsuanli | hsuanli.lin@mediatek.com | MediaTek (Chengdu) Inc. |
| Dr. | Lin | Kevin | kevin.lin@oppo.com | Chengdu OPPO Telecommunication |
| Mr. | Lin | Licheng | licheng.lin@mediatek.com | MediaTek (Wuhan) Inc. |
| Ms. | Lin | Pei | linp@chinatelecom.cn | CTSI |
| Mr. | Liu | Bo | liubo1@chinatelecom.cn | Chinatelecom Cloud |
| Miss | Liu | Dan | dan1992.liu@samsung.com | Samsung R&D Institute India |
| Dr. | Liu | Jialing | jialing.liu@futurewei.com | Futurewei Technologies |
| Mr. | Liu | Jiaxiang | liujiaxiang6@chinatelecom.cn | China Telecommunications |
| Mrs. | Liu | Zhe | zhe.liu@oppo.com | OPPO |
| Mr. | Lo | Anthony | alo2@futurewei.com | Futurewei |
| Mr. | Lodigiani | Luca | luca.lodigiani@inmarsat.com | Inmarsat |
| Dr. | LU | Feng | ho-ro@kddi.com | KDDI Corporation |
| Dr. | Lu | Jonathan | jonathan.lu@fcc.gov | FCC |
| Miss | Lu | Li | lu.li@zte.com.cn | ZTE Korea Limited |
| Dr. | M | Sheeba Kumari | sheebak@tejasnetworks.com | Tejas Network Limited |
| Dr. | Ma | Jinwen | jinwen.ma@verizonwireless.com | Verizon Spain |
| Mr. | Ma | Wei | ma.wei4@zte.com.cn | ZTE FRANCE SASU |
| Dr. | Ma | Zhifeng | ma.zhifeng@zte.com.cn | ZXNE |
| Mr. | Martens | Dick | d.martens@strict.nl | Union Inter. Chemins de Fer |
| Mr. | Masal | Abhijeet | abhijeetmasal@cewit.org.in | CEWiT |
| Dr. | Menzel | Edwin | edwin.menzel@rohde-schwarz.com | ROHDE & SCHWARZ |
| Mr. | Mohammed | Abdul Rasheed | rasheed@motorola.com | Motorola Mobility España SA |
| Mr. | Mohan | Ashwin | ashwin_mohan@apple.com | Apple Benelux B.V. |
| Mr. | Moore | Lawrence | lawrence.moore@ericsson.com | Nanjing Ericsson Panda Com Ltd |
| Mr. | Morihira | Ryo | ry-morihira@kddi.com | KDDI Corporation |
| Mr. | NAKAMURA | Kazuo | k-nakamura@nict.go.jp | NICT |
| Mr. | Nakatogawa | Tsuyoshi | nakatogawa.t-iq@nhk.or.jp | NHK |
| Mr. | Ogawa | Muneaki | ogawa-muneaki@sptvjsat.com | SKY Perfect JSAT Corporation |
| Mr. | Oguma | Yuta | yuuta.oguma.yt@nttdocomo.com | NTT |
| Dr. | Panaitopol | Dorin | dorin.panaitopol@thalesgroup.com | THALES |
| Dr. | Parikh | Hemish | hparikh@qti.qualcomm.com | Qualcomm Tech. Netherlands B.V |
| Mr. | Petrovic | Niels | niels.petrovic@rohde-schwarz.com | ROHDE & SCHWARZ |
| Miss | Pi | Qiping | piqp@docomolabs-beijing.com.cn | DOCOMO Beijing Labs |
| Mr. | Polaki | Kiran | k.polaki@samsung.com | Samsung R&D Institute UK |
| Mr. | Qiu | Haijie | haijie.qiu@samsung.com | SAMSUNG R&D INSTITUTE JAPAN |
| Ms. | Raghavan | Manasa | manasa_raghavan@apple.com | Apple Inc |
| Dr. | Ramamurthi | Vishwanath (Vishwa) | vishwanath.ramamurthi@verizonwireless.com | Verizon UK Ltd |
| Mr. | Ruiz | Emilio | emilio_ruiz@keysight.com | Keysight Technologies UK Ltd |
| Miss | Rutkowski | Kimberly | kim.rutkowski@mvg-world.com | MVG Industries |
| Mr. | Saini | Jyotirmay | jsaini@iitk.ac.in | IIT Kanpur |
| Mr. | Sato | Seiji | s-sato@arib.or.jp | ARIB |
| Mr. | Säynäjäkangas | Tuomo | tuomo.saynajakangas@nokia.com | Nokia Belgium |
| Mr. | Schumacher | Joseph | joseph.schumacher@att.com | AT&T Services, Inc. |
| Mr. | Sekino | Keita | ke-sekino@kddi.com | KDDI Corporation |
| Mr. | Setsu | Masafumi | Masafumi.Setsu@anritsu.com | Anritsu Corporation |
| Mr. | Sha | Wenhao | shawenhao@srtc.org.cn | SRTC |
| Mr. | Sheoran | Deepak | Deepak_k_sheoran@apple.com | Apple Solutions |
| Miss | Shi | Yu | shiyu19@chinaunicom.cn | China Unicom |
| Dr. | SHI | Zhihua | szh@oppo.com | Hangzhou Douku |
| Mr. | Sigovich | Ingbert | ingbert.sigovich@etsi.org | ETSI |
| Mr. | Sinsathitchai | Wisuit | wsinsathitchai@apple.com | Apple AB Denmark |
| Dr. | Song | Dan | songdan@chinamobile.com | China Mobile Com. Corporation |
| Dr. | Song | Lei | lei.song@verizon.com | Verizon Sweden |
| Dr. | Sun | Chengjun | chengjun.sun@samsung.com | BEIJING SAMSUNG TELECOM R&D |
| Miss | Sun | Huifang | sunhf50@chinaunicom.cn | China Unicom |
| Dr. | Sun | Roy | r.sun@cablelabs.com | CableLabs |
| Mr. | Sun | Yanliang | yanliang.sun@vivo.com | Tomorrow's Creation Labs |
| Mr. | Suzuki | Yasuki | ui-suzuki@kddi.com | KDDI Corporation |
| Dr. | Szini | Istvan | istvan@apple.com | Apple (UK) Limited |
| Mr. | Szpila | Rafal | Rafal.Szpila@orange.com | Orange Romania |
| Mr. | Takano | Masahiro | masahiro.takano.ru@nttdocomo.com | DOCOMO Beijing Labs |
| Mr. | Takeda | Hiroki | ho-takeda@kddi.com | KDDI Corporation |
| Mr. | Tang | Runsen | runsen.tang@samsung.com | Samsung Electronics GmbH |
| Miss | Tao | Amy | amy.tao@bureauveritas.com | Bureau Veritas ADT |
| Mr. | Tesserault | Guillaume | guillaume.tesserault@orange.com | Orange UK |
| Dr. | Thompson | Steve | steve.thompson@phywireless.com | PHY Wireless |
| Dr. | Tian | Wenqiang | tianwenqiang@oppo.com | OnePlus |
| Miss | Tian | Xiaoyang | tianxiaoyang@catt.cn | CATT |
| Mr. | Toril | Adan | adan_toril@keysight.com | Keysight Technologies UK Ltd |
| Mr. | Tugnawat | Yogesh | yogesht@qti.qualcomm.com | Qualcomm Incorporated |
| Mr. | Ubicini | Massimiliano | massimiliano.ubicini@telecomitalia.it | TELECOM ITALIA S.p.A. |
| Mr. | Vogedes | Jerome | jv0145@att.com | AT&T Labs, Inc |
| Mr. | WANG | FEI | wangfei@chinamobile.com | China Mobile International Ltd |
| Dr. | Wang | He | h0809.wang@samsung.com | Harman GmbH |
| Mrs. | Wang | Lili | lili008.wang@samsung.com | Samsung Shenzhen |
| Miss | Wang | Lulu | wangll958@chinaunicom.cn | BTPDI |
| Ms. | Wang | Miao | ada.wang@mediatek.com | Shanghai Chen Si Electronics |
| Dr. | Wang | Ruixin | ruixin.wang@vivo.com | vivo Mobile Com. (Chongqing) |
| Mr. | Wang | Xinrong | xinrong_wang@apple.com | Apple R&D |
| Dr. | Wang | Xuesong | cedar.wang@hisilicon.com | HUAWEI Technologies Japan K.K. |
| Dr. | Wang | Yanru | yanru.wang@samsung.com | Samsung Electronics Polska |
| Mr. | Wang | Zhanyuan | wangzhanyuan@vivo.com | vivo Wisdom Technology |
| Mr. | Watanabe | Shingo | sg-watanabe@kddi.com | KDDI Corporation |
| Dr. | wei | xusheng | xusheng.wei@vivo.com | vivo Mobile Communication Co., |
| Mr. | Weiler | Niclas | niclas.weiler@ericsson.com | Ericsson Telecomunicazioni SpA |
| Mr. | Wu | Jingzhou | wujingzhou@chinatelecom.cn | China Telecom Corporation Ltd. |
| Ms. | Xie | Fang | xiefang@chinamobile.com | China Mobile (Suzhou) Software |
| Dr. | XIONG | QI | q1005.xiong@samsung.com | Samsung Electronics France SA |
| Miss | Xu | Jiali | xu.jiali@zte.com.cn | ZTE Corporation |
| Dr. | Xu | Jin | jin.xu@bupt.edu.cn | BUPT |
| Mr. | Xue | Fei | xue.fei25@zte.com.cn | ZTE Corporation |
| Mr. | Yamashita | Osamu | osamu.yamashita@anritsu.com | Anritsu Corporation |
| Dr. | Yang | Ang | ang.yang@vivo.com | iQoo |
| Miss | Yang | Lu | yangl75@chinatelecom.cn | China Telecomunication Corp. |
| Dr. | Yang | Ning | yangning@oppo.com | OPPO Beijing |
| Mr. | Yang | Qian | qian9.yang@vivo.com | vivo Mobile Communication (S) |
| Mr. | Yang | Tuo | yangtuo@chinamobile.com | China Mobile E-Commerce Co. |
| Mr. | Yang | Yunchuan | yc0301.yang@samsung.com | Samsung Guangzhou Mobile R&D |
| Ms. | yang | zhaobing | yangzhaobing@hisilicon.com | Huawei Tech.(UK) Co.. Ltd |
| Mr. | Yao | Kun | yao.kun1@zte.com.cn | Nubia Technology Co.,Ltd |
| Mr. | Yashima | Takuya | takuya.yashima.zu@nttdocomo.com | NTT DOCOMO INC.. |
| Mr. | Yin | Hang | yinh6@chinatelecom.cn | E-surfing Digital |
| Mr. | Yu | Seungjun | seungjun.yu@kt.com | KT Corp. |
| Dr. | Yu | Tsang-wei (Ato) | Ato.Yu@mediatek.com | MediaTek Inc. |
| Dr. | Zhang | Dawei | dawei_zhang@apple.com | Apple France |
| Mr. | Zhang | Jian | zhangjian23@hihonor.com | Honor |
| Dr. | Zhang | Jingwen | zhangjingwen@chinamobile.com | China Mobile Group Device Co. |
| Mr. | Zhang | Miaoqi | zhangmiaoqi@chinamobile.com | CMDI |
| Mr. | ZHANG | Shichang | shichangzhang@oppo.com | Hangzhou Mengyuxiang |
| Mr. | Zhang | Shuang(Allen) | allen.zhang@mediatek.com | Mediatek India Technology Pvt. |
| Mr. | Zhang | Wenfeng | zhang.wenfeng@zohomail.com | Guangdong OPPO Mobile Telecom. |
| Miss | Zhang | Xiaoran | zhangxiaoran@chinamobile.com | China Mobile Com. Corporation |
| Miss | Zhang | Yaping | zhangyaping13@huawei.com | Huawei Device Co., Ltd |
| Miss | zhang | yuanyuan | tina55.zhang@samsung.com | Samsung Electronics Co., Ltd |
| Ms. | Zhang | Yufeng | zhangyufeng@caict.ac.cn | CAICT. |
| Mr. | Zhao | Huawei | derrick.chao@zte.com.cn | ZTE Photonics |
| Ms. | ZHAO | JING | zhaoj16@chinatelecom.cn | China Telecom Corporation Ltd. |
| Mr. | Zhao | Luyang | zhaoluyang@chinamobile.com | China Mobile (Hangzhou) Inf. |
| Dr. | Zhao | Mingyang | zhaomy65@chinaunicom.cn | Unicompay |
| Miss | Zhao | Ya | zhaoya@hisilicon.com | Huawei Technologies Japan K.K. |
| Mr. | Zhao | Zheng | zheng.zhao@verizonwireless.com | Verizon Denmark |
| Mr. | Zhao | Zhenshan | zhaozhenshan@oppo.com | Guangdong OPPO Mobile Telecom. |
| Miss | Zheng | Minhua | minhua.zheng@vivo.com | Nanjing vivo Software Tech. |
| Mr. | Zheng | Yi | zhengyi@chinamobile.com | China Mobile (Hangzhou) Inf. |
| Mr. | Zhou | Daiwei | daiwei.zhou@mediatek.com | MediaTek (Shenzhen) Inc. |
| Mr. | Zhou | Shuai | shuai.zhou@vivo.com | VIVO TECH GmbH |
| Mr. | Zhou | Wubin | zhou.wubin@zte.com.cn | ShenZhen Zhongxing Shitong |
| Miss | Zhu | Siting | zhusiting@caict.ac.cn | CAICT. |
| Mr. | Zhu | Sunlin | sunlin.zhu@samsung.com | Samsung Electronics Nordic AB |
| Mr. | Ziqi | Liu | liuziqi@vivo.com | GUANGDONG GENIUS TECHNOLOGY CO |
| Mr. | Zuo | Zhisong | zuozhisong@oppo.com | Shenzhen Heytap |

## Annex I: List of future meetings