3GPP TSG-RAN5 Meeting # 104	R5-244143

# Maastricht, Netherlands,  19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0849 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability rule for Enhanced Receiver Type 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS38.101-4 has introduced applicability rule for Enhanced Receiver Type 2 requirements in 5.1.1.3 and 5.1.1.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of applicability rule for Enhanced Receiver Type 2 requirements in 5.1.1.3 and 5.1.1.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no applicability rule for Enhanced Receiver Type 2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3 5.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |
| Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |
| Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |
| Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |
| Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |
| Maximum number of TCI states in Single-DCI based inter-slot TDM (maxNumberTCI-states-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 | The requirements apply only when maxNumberTCI-states-r16 = 2. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR1 FDD | PDSCH | Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |
| UE PDSCH processing capability #2 (pdsch-ProcessingType2) | FR1 FDD | PDSCH | Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |
| Pre-emption indication for DL (pre-EmptIndication-DL) | FR1 FDD | PDSCH | Clause 5.2.2.1.8 Clause 5.2.3.1.8 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.8 Clause 5.2.3.2.8 |  |
| 1024QAM modulation for PDSCH for FR1 (pdsch-1024QAM-FR1-r17 or pdsch-1024QAM-2MIMO-FR1-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-8) Clause 5.2.3.1.1 (Test 1-8) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-12) Clause 5.2.3.2.1 (Test 1-12) |  |
|  |  | SDR | Clause 5.5.1 Clause 5.5A.1 | 1024QAM MCS indexes are used only if UE supports 1024QAM for FR1 DL. |
| DRX Adaptation (drx-adaptation-r16) | FR1 FDD | PDCCH | Clause 5.3.2.1.3 | If the Test 1 in Clause 5.3.2.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.2.1.1. |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.3 | If the Test 1 in Clause 5.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.2.2.1. |
|  | FR1 FDD | PDCCH | Clause 5.3.3.1.3 | If the Test 1 in Clause 5.3.3.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.3.1.1. |
|  | FR1 TDD | PDCCH | Clause 5.3.3.2.3 | If the Test 1 in Clause 5.3.3.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.3.2.1. |
| Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ( CRS-IM-DSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.18 Clause 5.2.3.1.17 | UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.19 Clause 5.2.3.2.18 |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signalling on LTE channel bandwidth (CRS-IM-nonDSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 2-1) Clause 5.2.3.1.18 (Test 2-1) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-1) Clause 5.2.3.2.19 (Test 2-1) |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signalling on LTE channel bandwidth (CRS-IM-nonDSS-NWA-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 1-1) Clause 5.2.3.1.18 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.1.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.1.19. If the Test 2-1 in Clause 5.2.3.1.18 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.1.18. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-1) Clause 5.2.3.2.19 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.2.20. If the Test 2-1 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.2.19. |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signalling on LTE channel bandwidth (crs-IM-nonDSS-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-2) Clause 5.2.3.2.19 (Test 2-2) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signalling on LTE channel bandwidth (crs-IM-nonDSS-NWA-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-2) Clause 5.2.3.2.19 (Test 1-2) | If the Test 2-2 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.2.2.20. If the Test 2-2 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.3.2.19. |
| Support for SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH (sfn-SchemeA-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.20 Clause 5.2.3.1.19 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.21 Clause 5.2.3.2.20 |  |
| Support for SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH (sfn-SchemeB-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.21 Clause 5.2.3.1.20 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.22 Clause 5.2.3.2.21 |  |
| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML), when co-scheduled UE(s)’ modulation order is explicitly signalled by DCI index 1-5 in Table 7.3.1.2.2-12 of TS38.212 [10]. (advReceiver-MU-MIMO-r18) | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-1) Clause 5.2.3.1.16 (Test 3-1, Test 4-1) | If the Test 2-2 in Clause 5.2.2.1.16 is passed, the test coverage can be considered fulfilled without executing Test 2-1 in clause 5.2.2.1.16. If the Test 3-2 in Clause 5.2.3.1.16 is passed, the test coverage can be considered fulfilled without executing Test 3-1 in clause 5.2.3.1.16. If the Test 4-2 in Clause 5.2.3.1.16 is passed, the test coverage can be considered fulfilled without executing Test 4-1 in clause 5.2.3.1.16. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-1) Clause 5.2.3.2.17 (Test 3-1, Test 4-1) | If the Test 2-2 in Clause 5.2.2.2.17 is passed, the test coverage can be considered fulfilled without executing Test 2-1 in clause 5.2.2.2.17. If the Test 3-2 in Clause 5.2.3.2.17 is passed, the test coverage can be considered fulfilled without executing Test 3-1 in clause 5.2.3.2.17. If the Test 4-2 in Clause 5.2.3.2.17 is passed, the test coverage can be considered fulfilled without executing Test 4-1 in clause 5.2.3.2.17. |
| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML), when the co-scheduled UE information with DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 [10] is signalled. | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-2) Clause 5.2.3.1.16 (Test 3-2, Test 4-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-2) Clause 5.2.3.2.17 (Test 3-2, Test 4-2) |  |

5.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 5.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only.

Table 5.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| 256QAM modulation scheme for PDSCH for FR1 (pdsch-256QAM-FR1) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-3) Clause 5.2.3.1.1 (Test 1-3) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-3) Clause 5.2.3.2.1 (Test 1-3) |  |
| PDSCH mapping type B (pdsch-MappingTypeB) | FR1 FDD | PDSCH | Clause 5.2.2.1.3 Clause 5.2.3.1.3 Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.3 Clause 5.2.3.2.3 Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |
| Rate-matching around LTE CRS (rateMatchingLTE-CRS) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 Clause 5.2.3.1.4 | For UEs supporting “Alternative additional DMRS position for co-existence with LTE CRS”, if Test 1-2 is tested, the test coverage can be considered fulfilled without executing Test 1-1. Otherwise, only Test 1-1 is tested. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 Clause 5.2.3.2.4 |  |
| Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Tests 1-1, 1-2) Clause 5.2.3.1.1 (Tests 3-1, 4-1, 5-1) Clause 5.2.3.1.4 (Tests 1-1, 1-2) Clause 5.2.3.1.16 (Test 3-1, 3-2, 4-1, 4-2) | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies UE capability on maximum number of NZP-CSI-RS ports |
|  | FR1 TDD | PDSCH | Clause 5.2.3.2.1 (Tests 3-1, 4-1, 5-1) |  |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Tests 2-1, 2-2, 3-1) Clause 5.2.2.1.2 Clause 5.2.3.1.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1) Clause 5.2.3.1.2 Clause 5.2.3.2.17 (Test 3-1, 3-2, 4-1, 4-2) | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Tests 2-1, 2-2, 3-1) Clause 5.2.2.2.2 Clause 5.2.3.2.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1) Clause 5.2.3.2.2 |  |
| Support number of active TCI states per BWP per CC, including control and data (maxNumberActiveTCI-PerBWP) | FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-2) Clause 5.2.3.1.10 (Test 1-2) | The requirements apply only when maxNumberActiveTCI-PerBWP is other than n1. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-2) Clause 5.2.3.2.10 (Test 1-2) |  |
| Support for maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC) | FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-2) Clause 5.2.3.1.10 (Test 1-2) Clause 5.2.2.1.11 Clause 5.2.2.1.12 Clause 5.2.2.1.13 Clause 5.2.2.1.14 Clause 5.2.3.1.11 Clause 5.2.3.1.12 Clause 5.2.3.1.13 Clause 5.2.3.1.14 | The requirements apply only when maxSimultaneousResourceSetsPerCC ≥ 2 |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-2) Clause 5.2.3.2.10 (Test 1-2) Clause 5.2.2.2.11 Clause 5.2.2.2.12 Clause 5.2.2.2.13 Clause 5.2.2.2.14 Clause 5.2.3.2.11 Clause 5.2.3.2.12 Clause 5.2.3.2.13 Clause 5.2.3.2.14 |  |

<End of change>