3GPP TSG-RAN5 Meeting #104	R5-244162

# Maastricht, Netherlands, 19th Aug 2024 - 23th Aug 2024

Agenda item:	5.3.6.10

Source:	Keysight Technologies

Title:	Discussion on MU and TT analysis for PC5

Document for:	Discussion and Endorsement

# Introduction

The aim of this contribution is to progress in the MTSU and Test Tolerance definition for PC5 for the following RF test cases defined in TS 38.521-2 [5]:

- Error vector magnitude

  - Essentially a resubmission of proposal in [7] with some minor corrections on values after offline discussions.

- Spurious emissions tests up to 87GHz.

  - Agreed in last meeting to extend values up to 87GHz despite only needed for FR2c, what is so far out of scope in RAN5 WI for PC5.

# Discussion

## Error vector magnitude

To compute the EVM MTSU in FR2, it is considered both the EVM due to TE noise floor and the TE EVM in addition to the accepted TE noise floor. Regarding the second, in consonance to proposal made in [3], the following values are proposed.

| Parameter | Average EVM level (FR2a) |
| --- | --- |
| 50MHz | 2.50% |
| 100MHz | 3.00% |
| 200MHz | 4.25% |
| 400MHz | 6.00% |

Table 1. Permitted TE EVM in addition to the accepted TE noise floor.

Proposal 1. For PC5 in FR2a, in addition to the TE noise floor, consider the TE EVM defined in Table 1.

Considering assumed TE noise floor in FR2a = -10.6 dBm/400 MHz per polarization considering the minimum output power per EVM test point in case of PC5 assuming the highest MPR case by case, the following table summarizes the resulting EVM due to available SNR:

| Test Config |  |  | 50MHz |  | 100MHz |  | 200MHz |  | 400MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Modulation | RB alloc. | SNR (dB) | EVM | SNR (dB) | EVM | SNR (dB) | EVM | SNR (dB) | EVM |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 45.93 | 0.51% | 42.92 | 0.71% | 39.91 | 1.01% | 36.90 | 1.43% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 42.43 | 0.76% | 39.42 | 1.07% | 36.41 | 1.51% | 31.90 | 2.54% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 45.93 | 0.51% | 42.92 | 0.71% | 39.91 | 1.01% | 36.90 | 1.43% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 42.43 | 0.76% | 39.42 | 1.07% | 36.41 | 1.51% | 31.90 | 2.54% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 40.93 | 0.90% | 37.92 | 1.27% | 34.91 | 1.80% | 28.40 | 3.80% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 39.43 | 1.07% | 36.42 | 1.51% | 33.41 | 2.14% | 28.40 | 3.80% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 36.93 | 1.42% | 33.92 | 2.01% | 30.91 | 2.85% | 25.40 | 5.37% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 35.43 | 1.69% | 32.42 | 2.39% | 29.41 | 3.38% | 25.40 | 5.37% |
| 9 | CP-OFDM QPSK | Inner_Full | 39.43 | 1.07% | 36.42 | 1.51% | 33.41 | 2.14% | 27.90 | 4.03% |
| 10 | CP-OFDM QPSK | Outer_Full | 38.93 | 1.13% | 35.92 | 1.60% | 32.91 | 2.26% | 27.90 | 4.03% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 36.93 | 1.42% | 33.92 | 2.01% | 30.91 | 2.85% | 25.40 | 5.37% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 36.93 | 1.42% | 33.92 | 2.01% | 30.91 | 2.85% | 25.40 | 5.37% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 33.43 | 2.13% | 30.42 | 3.01% | 27.41 | 4.26% | 22.90 | 7.16% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 33.43 | 2.13% | 30.42 | 3.01% | 27.41 | 4.26% | 22.90 | 7.16% |

Table 2. Available SNR and equivalent EVM due to assumed TE noise floor for PUSCH, PC5 in FR2a.

Proposal 2. For PC5 in FR2a, consider EVM defined in Table 2 as EVM due to TE noise floor.

Next step is to compute EVM MTSU as root sum squared of EVM due to TE noise floor and the TE EVM in addition to the accepted TE noise floor. Once EVM MTSU is computed, final step is to compute the required TT following the formula captured in [1], annex F.1.2.  Resulting MTSU and TT to apply for PC5 is then summarized in next table:

| Test Config |  |  | 50MHz |  | 100MHz |  | 200MHz |  | 400MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Modulation | RB alloc. | MTSU | TT | MTSU | TT | MTSU | TT | MTSU | TT |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.55% | 0.00% | 3.08% | 0.00% | 4.37% | 0.00% | 6.17% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.61% | 0.00% | 3.18% | 0.00% | 4.51% | 0.00% | 6.52% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.55% | 0.00% | 3.08% | 0.00% | 4.37% | 0.00% | 6.17% | 0.00% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.61% | 0.00% | 3.18% | 0.00% | 4.51% | 0.00% | 6.52% | 0.00% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 2.66% | 0.00% | 3.26% | 0.00% | 4.61% | 0.00% | 7.10% | 1.88% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 2.72% | 0.00% | 3.36% | 0.00% | 4.76% | 0.00% | 7.10% | 1.88% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.88% | 0.00% | 3.61% | 0.78% | 5.12% | 1.50% | 8.05% | 3.35% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 3.02% | 0.00% | 3.84% | 0.87% | 5.43% | 1.67% | 8.05% | 3.35% |
| 9 | CP-OFDM QPSK | Inner_Full | 2.72% | 0.00% | 3.36% | 0.00% | 4.76% | 0.00% | 7.23% | 1.43% |
| 10 | CP-OFDM QPSK | Outer_Full | 2.74% | 0.00% | 3.40% | 0.00% | 4.81% | 0.00% | 7.23% | 1.43% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 2.88% | 0.00% | 3.61% | 0.00% | 5.12% | 1.01% | 8.05% | 2.37% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 2.88% | 0.00% | 3.61% | 0.00% | 5.12% | 1.01% | 8.05% | 2.37% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 3.28% | 0.65% | 4.25% | 1.06% | 6.02% | 2.01% | 9.34% | 4.30% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 3.28% | 0.65% | 4.25% | 1.06% | 6.02% | 2.01% | 9.34% | 4.30% |

Table 3. Available SNR and equivalent EVM due to assumed TE noise floor for PUSCH, PC5 in FR2a.

Based on previous results, all test points would be testable except test points 13 and 14 for 400MHz where relative EVM measurement increase is 53% > 50% limit agreed in formula captured in [1], annex F.1.2.

Proposal 3. For PC5 in FR2a, for EVM PUSCH test points, consider MTSU and TT defined in Table 3.

Observation 1. With MTSU and TT proposals made in this document, EVM measurement for PUSCH, PC5 in FR2a, all test points would be testable except test points 13 and 14 for 400MHz.

To be noted that differences versus proposal made in [7] in last meeting are due to consider now the MBR, driving now in the 2 untestable test points, and other minor differences in values for BW < 400MHz are due to calculate noise floor for them as -10.6+10*log10(BW/400MHz) [dBm/BW] rather than just assuming -3dB offset every time BW is half the previous one.

## Spurious emissions tests

During previous meeting, proposal 8 made in [7] was endorsed:

Prooposal 8. For RX Spurious test case at any spurious frequency and for TX Spurious test cases at frequencies f < 12.75GHz and 57 GHz ≤ f, for PC5 operating in FR2a, assume the same influence of noise assumed for PC3 and same MTSU assumed for PC1.

And associated CRs in last meeting defined MU up to 80GHz, as currently defined for FR2a for other power classes.

During discussions it was observed that being strict, for the PC5 bands applicable in RAN5 so far, n257 and n258, it is by now not needed to define MTSU beyond 66GHz. However, it was agreed to accept proposed values up to 80GHz in last meeting and indeed to extend values up to 87GHz in this meeting in order to be ready for a future when n259 (FR2c) support were extended for PC5 despite it is known to be out of scope of current PC5 WI.

In this context, For the frequency range 80GHz < f ≤ 87GHz, for both TX and RX Spurious test cases, it is proposed to assume for PC5 the same influence of noise assume for PC3 and to recalculate resulting MTSU accordingly, considering some other MU factors are taken from PC1.

Proposal 4. For RX and TX Spurious test cases, for the frequency range 80GHz < f ≤ 87GHz, assume the same influence of noise assumed for PC3 and to recalculate resulting MTSU accordingly, considering some other MU factors are taken from PC1.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. With MTSU and TT proposals made in this document, EVM measurement for PUSCH, PC5 in FR2a, all test points would be testable except test points 13 and 14 for 400MHz.

Proposal 1. For PC5 in FR2a, in addition to the TE noise floor, consider the TE EVM defined in Table 1.

Proposal 2. For PC5 in FR2a, consider EVM defined in Table 2 as EVM due to TE noise floor.

Proposal 3. For PC5 in FR2a, for EVM PUSCH test points, consider MTSU and TT defined in Table 3.

Proposal 4. For RX and TX Spurious test cases, for the frequency range 80GHz < f ≤ 87GHz, assume the same influence of noise assumed for PC3 and to recalculate resulting MTSU accordingly, considering some other MU factors are taken from PC1.

# References

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, v18.3.0 (2024-06)

- TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, v18.3.0 (2024-06)

- R5-221625: "Discussion on MU and TT proposal for FR2 EVM", Keysight, RAN5#94-e

- R5-227984, “Discussion on FR2 PC1 MU”, Keysight, RAN5#97

- R5-232158, “On FR2 PC5 MU analysis”, Keysight, RAN5#99

- R5-233976, “PC5 MU - Discussion on influence of noise”, Keysight, RAN5#100

- R5-242264, “Discussion on MU and TT analysis for PC5”, Keysight, RAN5#103