3GPP TSG-RAN5 Meeting #104	R5-244167

# Maastricht, Netherlands, 19th Aug 2024 - 23th Aug 2024

Agenda item:	5.4.20

Source:	Keysight Technologies

Title:	Discussion on MU and TT analysis for FR2c and FR2b

Document for:	Discussion and Endorsement

# Introduction

During RAN5#103, action point on MU and TT definition for FR2c, AP#99.21, was closed. However, it was left pending the definition of MU and TT for OBW test case for bandwidths greater than 100MHz. In addition, during associated discussions, it was observed that values for OFF power test case for FR2b were not concluded and therefore declared as not testable, but it was requested it to be concluded for the Japanese regulation.

The aim of this contribution is to make a proposal for OBW for BW ≥ 200MHz on FR2c (band n259) and for OFF power test case on FR2b (band n260).

# Discussion

## OBW test in FR2c

For CBW = 200MHz, in consonance to our former proposal made in [1], based on our test system analysis, required MTSU for FR2c in OBW test is +/- 1.3 %CBW.

Proposal 1. For OBW test in FR2c, define MTSU as +/- 1.3 %CBW for CBW = 200 MHz.

For CBW = 400MHz, for the assumed noise floor and the analysis made in [2], it was concluded that CBW is not testable under current measurement assumption of using measurement bandwidth MBW = 1.5*CBW. It was proposed there to evaluate a MBW down to 1.3*CBW in order to make the case testable.

For CBW=400MHz in FR2c, the OBW test point results into a minimum SNR = 17.7dB = Min Peak EIRP – MBR – MPR – T(MPR) – Noise Floor = 18.7-0.5-3-2 – (-4.5dBm/400MHz).

MTSU value proposed in [2] for MBW = 1.3*CBW and CBW = 400MHz is 1.3%. However, Figure 2 in [2], considering the 97.5-tile curve, for SNR ≈ 17.7dB suggests that MTSU should be greater or equal than 1.4%.

Observation 1. Figure 2 in [2], considering the 97.5-tile curve for MBW = 1.3*CBW, for SNR ≈17.5dB suggests that MTSU should be greater or equal than 1.4%CBW.

In order to have some minor margin, it is proposed to define MTSU as +/-1.5%CBW.

Proposal 2. For OBW test in FR2c, for CBW = 400MHz, apply MBW = 1.3*MHz and define MTSU as +/- 1.5 %CBW.

## Tx OFF Power test in FR2b

Relaxation currently captured for Tx OFF power test case definition in TS 38.521-2 [4] for FR2b, was defined as a provisional value in [3] waiting “for further assessment of SNR”. However, value was never revisited and eventually in TR 38.903 [5], for FR2b it was indicated “Proposed not to test” the Tx OFF power test.

As explained in the introduction, given relaxation and noise floor has been consistently defined for FR2c in Tx OFF power test case, the same will be done for FR2b in this section.

It must be observed the proximity between FR2b and FR2c. Indeed, there is certain overlap between n260 (FR2b) and n259 (FR2c). Therefore, for FR2b and Tx OFF power test, it is proposed to follow an equivalent approach to the one followed in FR2c.

Observation 2. FR2b and FR2c are very close. Indeed, there is certain overlap between n260 (FR2b) and n259 (FR2c).

In FR2c, the noise floor assumption for Tx OFF power test is the same as the one assumed for Minimum output power test case. The same is proposed then for FR2b.

Observation 3. In FR2c, the noise floor assumption for Tx OFF power test is the same as the one assumed for Minimum output power test case.

Proposal 3. For Tx OFF power test case in FR2b, assume TE noise floor = -5.5dBm/400MHz, the same already assumed for FR2b for Minimum output power test case.

Based on previous proposal, the resulting and proposed relaxation would be 35.5dB + ChBW, with ChBW = 10*log10(CBW[MHz]/400MHz).

Proposal 4. For Tx OFF power test case in FR2b, apply a relaxation of 35.5dB + 10*log10(CBW[MHz]/400MHz).

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. Figure 2 in [2], considering the 97.5-tile curve for MBW = 1.3*CBW, for SNR ≈17.5dB suggests that MTSU should be greater or equal than 1.4%CBW.

Observation 2. FR2b and FR2c are very close. Indeed, there is certain overlap between n260 (FR2b) and n259 (FR2c).

Observation 3. In FR2c, the noise floor assumption for Tx OFF power test is the same as the one assumed for Minimum output power test case.

Proposal 1. For OBW test in FR2c, define MTSU as +/- 1.3 %CBW for CBW = 200 MHz.

Proposal 2. For OBW test in FR2c, for CBW = 400MHz, apply MBW = 1.3*MHz and define MTSU as +/- 1.5 %CBW.

Proposal 3. For Tx OFF power test case in FR2b, assume TE noise floor = -5.5dBm/400MHz, the same already assumed for FR2b for Minimum output power test case.

Proposal 4. For Tx OFF power test case in FR2b, apply a relaxation of 35.5dB + 10*log10(CBW[MHz]/400MHz).

# References

- R5-243724, “Discussion on MU and TT analysis for FR2c”, Keysight, RAN5#103

- R5-242998, “On testability issue of FR2c OBW”, Anritsu, RAN5#103

- R5-188229, “SNR estimation for priority 1 and 2 FR2 Tx test cases”, Anritsu, RAN5#81

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, v18.3.0 (2024-06)

- TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, v18.3.0 (2024-06)