3GPP TSG-RAN5 Meeting #104	R5-244176

# Maastricht, Netherlands, 19th Aug 2024 - 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1065 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC6 FR2 MU - Annex F and M update in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress has been made on the MU factors analysis for PC6 in R5-244172 and R5-244448. MUs have been defined for certain test cases in TR 38.903 in R5-244173. Test cases in 38.521-2 should be udpated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated MU and TT in annex F for the following test cases for PC6: -MOP-EIRP -MOP-TRP -REFESENS – EIS -Added applicable measurement grids in Annex M (leverage of PC1 grids with 6x6 array configuration). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete for PC6. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2, F.1.3, F.3.2, F.3.3, M.2.1.6, M.3.1.1.6, M.3.2.1.6, M.4.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.08 dB (FR2a, NTC testing) ±5.28 dB (FR2b, NTC testing) ±6.64 dB (FR2c, NTC testing) ±5.35 dB (FR2a, ETC testing) ±5.55 dB (FR2b, ETC testing) ±6.78 (FR2c, ETC testing) PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.40 dB (FR2b, NTC testing) ±5.60 dB (FR2a, ETC testing) ±5.67 dB (FR2b, ETC testing) PC5 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.60 dB (FR2a, ETC testing) PC6 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.31 dB (FR2a, NTC testing) ±5.58 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Max Device size ≤ 30 cm ±4.61 dB (FR2a, NTC testing) ±4.81 dB (FR2b, NTC testing) ±6.16 dB (FR2c, NTC testing) ±4.85 dB (FR2a, ETC testing) ±5.07 dB (FR2b, ETC testing) TBD (FR2c, ETC testing) PC1 Max TRP Max Device size ≤ 30 cm ±4.64 dB (FR2a, NTC testing) ±4.78 dB (FR2b, NTC testing) ±4.90 dB (FR2a, ETC testing) ±5.04 dB (FR2b, ETC testing) PC5 Max TRP Max Device size ≤ 30 cm ±4.64 dB (FR2a, NTC testing) ±4.90 dB (FR2a, ETC testing) PC6 Max TRP Max Device size ≤ 30 cm ±4.64 dB (FR2a, NTC testing) ±4.90 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |

## <<< Skip unchanged sections >>>

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 Max Device size ≤ 30 cm ±5.36 dB (FR2a, FR2b, NTC testing) ±6.34 dB (FR2c NTC testing) ±5.61 dB (FR2a, FR2b, ETC testing) ±6.48 (FR2c ETC testing) PC1 Max Device size ≤ 30 cm ±5.58 dB (FR2a, FR2b, NTC testing) ±5.83 dB (FR2a, FR2b, ETC testing) PC5 Max Device size ≤ 30 cm ±5.58 dB (FR2a, NTC testing) ±5.83 dB (FR2a, ETC testing) PC6 Max Device size ≤ 30 cm ±5.56 dB (FR2a, NTC testing) ±5.81 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.19-1 in TR 38.903) |

## <<< Skip unchanged sections >>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.99 dB (FR2a, NTC) 2.99 dB (FR2b, NTC) 3.80 dB (FR2c, NTC) 3.15 dB (FR2a, ETC) 3.15 dB (FR2b, ETC) 3.89 (FR2c, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.12 dB (FR2a, NTC) 3.12 dB (FR2b, NTC) 3.28 dB (FR2a, ETC) 3.28 dB (FR2b, ETC) PC5 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.12 dB (FR2a, NTC) 3.28 dB (FR2a, ETC) Max EIRP 0 dB PC6 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.11 dB (FR2a, NTC) 3.27 dB (FR2a, ETC) Max EIRP 0 dB | PC3 Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) TT = 0.60 x (MTSUIFF - 0.3) (FR2c) PC1 Minimum peak EIRP TT = 0.60 x (MTSUIFF – 0.13) (FR2a) TT = 0.60 x (MTSUIFF – 0.20) (FR2b) PC5, PC6 Minimum peak EIRP TT = 0.60 x (MTSUIFF – 0.13) (FR2a) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.77 dB (FR2a, NTC) 2.89 dB (FR2b, NTC) 3.70 dB (FR2c, NTC) 2.91 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) TBD (FR2c, ETC) PC1 Max TRP IFF (Max Device size ≤ 30 cm) 2.78 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 2.94 dB (FR2a, ETC) 3.03 dB (FR2b, ETC) PC5 Max TRP IFF (Max Device size ≤ 30 cm) 2.78 dB (FR2a, NTC) 2.94 dB (FR2a, ETC) PC6 Max TRP IFF (Max Device size ≤ 30 cm) 2.78 dB (FR2a, NTC) 2.94 dB (FR2a, ETC) | Max TRP TT = 0.60 x MTSUIFF |

## <<< Skip unchanged sections >>>

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 IFF (Max Device size ≤ 30 cm) 2.41 dB (FR2a, FR2b, NTC) 2.52 dB (FR2a, FR2b, ETC) 2.85 dB (FR2c, NTC) 2.92 (FR2c, ETC) PC1 IFF (Max Device size ≤ 30 cm) 2.51 dB (FR2a, FR2b, NTC) 2.62 dB (FR2a, FR2b, ETC) PC5 IFF (Max Device size ≤ 30 cm) 2.51 dB (FR2a, NTC) 2.62 dB (FR2a, ETC) PC6 IFF (Max Device size ≤ 30 cm) 2.50 dB (FR2a, NTC) 2.62 dB (FR2a, ETC) | TT = 0.45 x MTSUIFF |

<<< Skip Unchanged Sections >>>

### M.2.1.6	Power class 6 devices

The same antenna array assumptions and measurement grids as in Clause M.2.1.1 with an antenna array configuration of 6 x 6 apply.

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use for beam peak search the following measurement grids leading to a systematic error of “Beam Peak Search” of 0.7 dB:

-	Constant density grid (using the charged particle implementation) with at least 750 grid points.

-	Constant step size grid with at least 1106 grid points, corresponding to an angular step size of 7.5º.

<<< Skip Unchanged Sections >>>

#### M.3.1.1.6	Power class 6 devices

The same antenna array assumptions and measurement grids as in Clause M.3.1.1.1 with an antenna array configuration of 6 x 6 apply.

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use the following recommendation in terms of min. number of grid points, standard deviation, and mean error for spherical coverage grids:

-	constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB.

-	constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB.

<<< Skip Unchanged Sections >>>

#### M.3.2.1.6	Power class 6 devices

The same antenna array assumptions and measurement grids as in Clause M.3.2.1.1 with an antenna array configuration of 6 x 6 apply.

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use the following recommendation in terms of min. number of grid points, standard deviation, and mean error for spherical coverage grids:

-	constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB

-	constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB.

-	the MU element ‘Systematic error related to EIS spherical coverage’ is the DL step size, i.e., 0.2dB

Choice of grids among these 2 types of grids is up to test system implementation.

There is no need to have the Rx beam peak placed on a measurement grid point.

For constant step size measurement grids, the CCDF analyses require the PDFs to be scaled by sin(theta) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1.

<<< Skip Unchanged Sections >>>

### M.4.1.6	Power class 6 devices

The same antenna array assumptions and measurement grids as in Clause M.4.1.1 with an antenna array configuration of 6 x 6 apply.

In order to make a reasonable trade-off between measurement uncertainties, at least the following number of points shall be included in the measurement grid for TRP measurements PC1 UEs based on the assumption that the standard deviation does not exceed 0.25dB. If the re-positioning concept is not applied to TRP test cases:

-	150 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.13 dB

-	13 latitudes and 24 longitudes (266 unique grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.20dB with the allowance to skip and interpolate measurements at the pole at =180°, see Annex M.4.4.

-	13 latitudes and 24 longitudes (266 unique grid points) for constant step size grid – Clenshaw Curtis weights integration approach, with standard deviation of 0.15dB with the allowance to skip and interpolate measurements at the pole at =180°, see Annex M.4.4.

Either of the following minimum number of grid points for TRP procedures apply if the re-positioning is applied:

-	150 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.13 dB with the allowance to skip and interpolate measurements for ≥150°, see Annex M.4.4

-	13 latitudes and 24 longitudes (266 unique grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.19dB with the allowance to skip and interpolate measurements for ≥150°, see Annex M.4.4

-	13 latitudes and 24 longitudes (266 unique grid points) for constant step size grid – Clenshaw-Curtis weights integration approach, with standard deviation of 0.04dB with the allowance to skip and interpolate measurements for ≥150°, see Annex M.4.4.

## <<< END OF CHANGES >>>