3GPP TSG-RAN5 Meeting #104	R5-244189

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3233 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RRCRelease message contents values for 3GPP MT-SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Arriver Software AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MT_SDT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 38.331 Rel-18, Clause 6.2.2, MT-SDT Information Element values corresponding to the RRCRelease message for 3GPP SDT test cases need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added RRCRelease message Information elements values for SDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Environment aspect of the work plan will remain unfulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.6.1-16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-2 CR 0654 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### –	RRCRelease

Table 4.6.1-16: RRCRelease

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| redirectedCarrierInfo |  | Not present |  |  |
| cellReselectionPriorities |  | Not present |  |  |
| suspendConfig |  | Not present |  |  |
| suspendConfig SEQUENCE { |  |  |  | NR_RRC_INACTIVE |
| fullI-RNTI |  | I-RNTI-Value |  |  |
| shortI-RNTI |  | ShortI-RNTI-Value |  |  |
| ran-PagingCycle |  | rf32 |  |  |
| ran-NotificationAreaInfo CHOICE { |  |  |  |  |
| cellList SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell { |  | 1 entry |  |  |
| PLMN-RAN-AreaCellList[1] SEQUENCE { |  |  | entry 1 |  |
| plmn-Identity |  | Not present |  |  |
| ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity { |  | 1 entry |  |  |
| CellIdentity[1] |  | CellIdentity | entry 1 Cellidentity for the used cell. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| t380 |  | Not present |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| sl-UEIdentityRemote-r17 |  | Not present |  |  |
| sl-UEIdentityRemote-r17 |  | RNTI-Value |  | L2RemoteUE |
| sdt-Config-r17 |  | Not present |  |  |
| sdt-Config-r17 CHOICE { |  |  |  | SDT |
| setup SEQUENCE { |  |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { |  | 1 entry |  |  |
| DRB-Identity |  | DRB-Identity using condition DRB1 |  |  |
| } |  |  |  |  |
| sdt-SRB2-Indication-r17 |  | Not present |  |  |
|  |  | allowed |  | pc_srb_SDT_r17 |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { |  | 1 entry |  |  |
| logicalChannelIdentity-r17 |  | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 |  | true |  |  |
| allowedCG-List-r17 |  | Not present |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 |  | 1 entry |  | pc_cg-SDT-PeriodicityExt-r18 |
| cg-MT-SDT-MaxDurationToNext-CG-Occasion-r18 |  | FFS |  | pc_cg-SDT-PeriodicityExt-r18 |
| } |  |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  | Not present |  |  |
| cg-SDT-TimeAlignmentTimer-r17 |  | ms750 |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 |  | 66 |  |  |
| cg-SDT-TA-ValiditationConfig-r17 |  | Not present |  |  |
| cg-SDT-CS-RNTI-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sdt-DRB-ContinueROHC-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| srs-PosRRC-Inactive-r17 |  | Not present |  |  |
| ran-ExtendedPagingCycle-r17 |  | Not present |  |  |
| } |  |  |  |  |
| deprioritisationReq |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR_RRC_INACTIVE | NR RRC state RRC_INACTIVE |
| SDT | For SDT test cases |
| L2RemoteUE | For L2 U2N Remote UE test cases. |
| MT-SDT | For MT-SDT test cases |

<End of modified section>