3GPP TSG-RAN5 Meeting #104	R5-244209

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4478 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new test case 8.1.3.1.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case for Measurement configuration control and reporting for video / QoE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case 8.1.3.1.25 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 8.1.3.1.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0484 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

##### 8.1.3.1.25	Measurement configuration control and reporting for video / QoE

8.1.3.1.25.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state configured with application layer measurement and reporting and video call is ongoing }

ensure that {

when { UE receives an application layer measurement report container from upper layers}

then { UE sends a MeasurementReportAppLayer message }

}

8.1.3.1.25.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clause 5.7.16.2.

[TS 38.331 clause 5.7.16.2]

A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement and reporting, i.e. when at least one measConfigAppLayer and SRB4 and/or SRB5 have been configured by the network.

Upon initiating the procedure, the UE shall:

…

1>	for each measConfigAppLayerId received from upper layers:

2>	if the UE AS has received application layer measurement report container from upper layers which has not been transmitted; and

…

2>	set the measConfigAppLayerId in the MeasurementReportAppLayer message to the value of the measConfigAppLayerId received together with application layer measurement report information;

…

1>	for each encoded MeasurementReportAppLayer message generated above which contains either only encapsulated application layer measurement reports or both encapsulated application layer measurement reports and RAN visible application layer measurement reports:

2>	if reportingSRB is not configured for the measConfigAppLayerId:

…

3>	else:

4>	submit the MeasurementReportAppLayer message to lower layers for transmission via SRB4.

8.1.3.1.25.3	Test description

8.1.3.1.25.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

-	None.

Preamble:

-	The UE is in state RRC_IDLE using generic procedure parameter Connectivity (NR) according to TS 38.508-1 [4], Table 4.5.1-1.

8.1.3.1.25.3.2	Test procedure sequence

Table 8.1.3.1.25.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Make the UE attempt an IMS voice call. | - | - | - | - |
| 2-6 | Steps 2-6 of generic procedure specified in Table 4.9.24.2.2-1 of TS 38.508-1 [4] are performed. | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2, SRB4 and DRB(s). The RRCReconfiguration message is configured using RRCReconfiguration-SRB2-SRB4-DRB(n, m) where n and m are the number of DRB(s) configured with RLC-AM and RLC-UM respectively. Two RLC-UM for video. |  | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 8 | The UE transmits a RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9-13 | Steps 9-13 of generic procedure specified in Table 4.9.24.2.2-1 of TS 38.508-1 [4] are performed. | - | - | - | - |
| 14 | Check: Does the UE transmit a MeasurementReportAppLayer message? | --> | NR RRC: MeasurementReportAppLayer | 1 | P |
| 15 | SS releases the call. (Annex A.8 of TS 34.229-5 [41]). | - | - | - | - |
| 16 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |

8.1.3.1.25.3.3	Specific message contents

None.

<End of modified section>