3GPP TSG-RAN5 Meeting #104	R5-244336

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0042 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update and correction of minimum test time in NTN demod case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current minimum test time of TC 8.3.1.1.1 is missing and there are several implementation error in Table G.3.5-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The definitions of minimum test time of 8.3.1.1.1 is added to Annex G.3.5 (refer to R5-177114, the mainly difference between NB-IoT and NB-IoT NTN is the introduced K-offset) Test 1 - The period of NPDCCH UE-Specific SearchSpace is 96ms according to G=1.5 and Rmax=64. - Number of repetition is 32 for each NPDCCH and NPDSCH. Isf=3, so 128ms (4 x 32) is needed for 1 NPDSCH. Then 2 periods of NPDCCH UE-Specific SearchSpace (96 x 2 = 192ms) and K-offset (8ms based on Table 8.3.1.1-2) total 200ms is needed to get one HARQ-ACK. - Given SI-periodicity 640ms, the overall period is a least common multiple of 200ms and 640ms -> 3200ms. If taking into account the collision between SIB2-NB/SIB31-NB and NPDCCH scheduling, 8 samples can be got in this period. To gather an MNAS number of 669 samples, 267600ms(3200 x 669 / 8) is needed. To shorten the test time, MNAS value is selected from the row with the biggest ER that does not exceed 1% from Table G.4.4-1. Test 2 - The period of NPDCCH UE-Specific SearchSpace is 768ms according to G=1.5 and Rmax=256. - Number of repetition is 128 for NPDCCH and NPDSCH. Isf=3, so 512ms (4 x 128) is needed for 1 NPDSCH. Then 2 periods of NPDCCH UE-Specific SearchSpace (2 x 768 = 1536ms) and K-offset (8ms based on Table 8.3.1.1-2) total 1544ms is needed to get one HARQ-ACK. - SI-periodicity doesn’t need to be taken into account because measuring target is non-anchor carrier. Then the overall period is 1544ms. To gather an MNAS number of 758 samples, 1170352ms(1544 x 758) is needed. To shorten the test time, MNAS value is selected from the row with the biggest ER that does not exceed 1% from Table G.4.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Minimum test time of TC 8.3.1.1.1 remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-234333 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

G.3.5	Minimum Test time

<<< skip unchanged part>>>

Table G.3.5-1: Minimum Test time for PDSCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) |  |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | TDD |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 |  | 267600 | N/A |
| 8.3.1.1.1 Test 2 | R.NB.2 FDD (200kHz, QPSK,1/3) (1x1) NTN TDLA100-10 |  | 1170352 | N/A |
| 8.3.1.1.2 Test 1 | R.NB.6 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 3609 | 3466000 | N/A |
| 8.3.1.1.2 Test 2 | R.NB.6-1 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 3609 | 5544000 | N/A |

<<< END OF CHANGES 1>>>