3GPP TSG-RAN5 Meeting #104	R5-244380

# Maastricht, Netherlands, 19th Aug 2024 - 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3253 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on rules to calcualate Low and High DL frequency in case of asymmetric UL and DL BW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In case of assymetric CBW, in section C.2.3.2, step 3, it is defined the procedure to follow to adjust DL frequency if BWDL does not fit within the band DL frequency range. However, the conditions are defined considering only the DL carrier frequency FDL but without considering also the CBWDL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that equations C.2.3.2-Eq7 to C.2.3.2-Eq8c apply if FDL_LowRange - CBWDL/2 is smaller than the lowest frequency of the band. Clarified that equations C.2.3.2-Eq9 to C.2.3.2-Eq12 apply if FDL_HighRange + CBWDL/2 is higher than the highest frequency of the band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### C.2.3.2	Determination of Low-, Mid- and High-Range for asymmetric uplink and downlink bandwidth combinations

The following procedure is used to determine the test frequencies for Low-, Mid- and High-Range for bands supporting asymmetric UL and DL bandwidth combinations.

1.	Calculate uplink carrier centre frequencies:

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq1 |
| --- | --- |
| FUL_MidRange = Round((FUL_Low + BWUL_Band/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq2 |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq3 |

For those FDD frequency bands with two UL sub-bands, for Low- and Mid-Range, use lower sub-band and for High-Range use higher sub-band. In these cases, BWUL_Band refers to the sub-band.

2.	Calculate the downlink frequencies:

Calculate the DL carrier centre frequencies from the UL frequencies in step 1.

| FDL_LowRange = FUL_LowRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq4 |
| --- | --- |
| FDL_MidRange = FUL_MidRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq5 |
| FDL_HighRange = FUL_HighRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq6 |

For those FDD frequency bands where TX-RX carrier centre frequency separation can have multiple values, FTx-Rx_separation shall be chosen using the following criteria:

- For Low-Range, use FTx-Rx_separation that minimizes FDL_LowRange.for each SCS.

- For Mid-Range, use FTx-Rx_separation closer to FDL_LowRange + (FDL_HighRange – FDL_LowRange)/2.

- For High-Range, use FTx-Rx_separation that maximizes FDL_HighRange for each SCS.

3.	Check that the calculated centre test frequencies in step 2 for the BWDL fits within the bands DL frequency range:

If FDL_LowRange - CBWDL/2 is smaller than the lowest frequency of the band then recalculate the minimum FDL_LowRange and modify the associated FUL_LowRange, FDL_MidRange and FUL_MidRange as:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq7 |
| --- | --- |
| FUL_LowRange = FDL_LowRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq8a |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8b |

If FDL_HighRange + CBWDL/2 is larger than the higher frequency of the band then recalculate the maximum FDL_HighRange and modify the associated FUL_HighRange, FDL_MidRange and FUL_MidRange as:

| FDL_HighRange = Floor((FDL_Low + BWDL_Band – CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq9 |
| --- | --- |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq10 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq11 |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq12 |

## <<< END OF CHANGES >>>