Title:	Discussion on test point selection for NS_04 A-MPR in FR1

# Background

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

The purpose of this paper is to provide an analysis for the parameters in test configuration table and a selection of test points for test environment, test frequencies, channel bandwidths, SCS, RB allocation and modulation.

# Discussion

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] and for FR1. In general, for A-MPR the emissions minimum requirements are SEM and/or Spurious. The proposals below are basically the same as what have been proposed for NS_35 in [4]. However, some additional test frequencies are required for NS_04 A-MPR tests.

## Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

## Test Frequencies

The selection of the general test frequencies can be based on the agreed proposals for SEM [2], i.e. Low and High frequency.

PROPOSAL 2: Select Low range and High range as general test frequencies for NS_04 A-MPR in FR1.

## Additional Test Frequencies for NS_04 (Power Class 1.5, 2 and 3)

NS_04 can be signalled for a UE supporting band n41, where FUL_low = 2496 MHz and where FUL_high = 2690 MHz. The Maximum transmission bandwidth for NR is shown in figure 1. The additional frequencies are in addition to the test frequency Low.

Figure 1: Maximum transmission badwidth.

The following channel bandwidths (BWChannel) are supported for NR band n41:

Figure 2: Channel bandwiths for some NR bands.

The A -MPR requirement for NS_04 is shown in the figure 3 and 4  below.

Figure 3: A-MPR requirements for NS_04.

Figure 4: Parameters for the A-MPR NS_04.

The requirements for NS_04 are given in Figure 2 with certain parameters defined in Figure 3. The requirements are divided into 3 regions: IMD3 region, regrowth region and MPR (A-MPR = 0).

Figure 4: Parameters for the A-MPR.

Figure 4 shows an example of the division into regions for 10 MHz channel bandwidth, based on the equations in Figure 3. The additional frequencies separate the regions. For 10 MHz channel bandwidth, the additional frequencies are 2505 and 2511 MHz.

Table 1: Aditional test frequencies

| BWChannel (MHz) | IMD3 region (MHz) | Regrowth region (MHz) | MPR region (MHz) |
| --- | --- | --- | --- |
| 10 | FC < 2505 | 2505 ≤ FC < 2511 | 2511 ≤ FC |
| 15 | FC < 2512.5 | 2512.5 ≤ FC < 2513.5 | 2513.5 ≤ FC |
| 20 | FC < 2520 | IMD3 region overlaps Regrowth region | 2520 ≤ FC |
| 30 | FC < 2535 | IMD3 region overlaps Regrowth region | 2535 ≤ FC |
| 40 | FC < 2550 | IMD3 region overlaps Regrowth region | 2550 ≤ FC |
| 50 | FC < 2565 | IMD3 region overlaps Regrowth region | 2565 ≤ FC |
| 60 | FC < 2580 | IMD3 region overlaps Regrowth region | 2580 ≤ FC |
| 70 | FC < 2595 | IMD3 region overlaps Regrowth region | 2595 ≤ FC |
| 80 | FC < 2610 | IMD3 region overlaps Regrowth region | 2610 ≤ FC |
| 90 | FC < 2625 | IMD3 region overlaps Regrowth region | 2625 ≤ FC |
| 100 | FC < 2640 | IMD3 region overlaps Regrowth region | 2640 ≤ FC |
|  | PC3_A2, PC2_A4, PC1.5_A6 | PC3_A1. PC2_A3, PC1.5_A5 | (A-MPR=0) |

Table 1 lists the additional frequencies for different channel bandwidths and regions. For channel bandwidths greater than 15 MHz, the IMD3 region overlaps the Regrowth region. All of these additional test frequencies fit the channel raester with the exception of 2513.5 MHz for 15 MHz channel bandwidth. Then select for SCS = 15 kHz 2513.505 MHz and for, SCS = 30 kHz, SCS = 60 kHz 2513.52 MHz as additional test frequencies.

Figure 6: NS_04 A-MPR requirements for different modulations and power classes (PC).

Table 2: RB allocation and test frequencies

|  | IMD3 region (MHz) | Regrowth region (MHz) | MPR region (MHz) |
| --- | --- | --- | --- |
| Proposed RB allocation | Edge_1RB_Left | Edge_1RB_Left | Edge_1RB_Left |
| Test frequency | Low (see TS 38.805-1) | Additional test frequency (see Table 1) | Additional test frequency (see Table 1) |
| Requirement | PC3_A2, PC2_A4, PC1.5_A6 | PC3_A1. PC2_A3, PC1.5_A5 | (A-MPR=0) |

Table 2 shows the proposed test frequencies and RB allocations. Like all other A-MPR test cases, the starting point is to test Low and High frequencies with Edge_1RB allocation and Outer_Full. The same principle shall also apply to the additional frequencies.

PROPOSAL 3-1: For PC 1.5, 2 and 3 select additional frequencies (Fc) according to Table 1 and test the A-MPR requirements with both Edge_1RB and Outer_Full allocation.

## A-MPR regions for NS_04 (Power Class 1)

The RB allocations that require A-MPR can be divided into the following regions:

Edge region: narrow allocations at lower channel edge, with spectral regrowth hitting the additional SEM; this region requires the largest A-MPR values.

Regrowth region: wide allocations at or near lower channel edge with spectral regrowth hitting the additional spurious mask

IMD3 region: narrow allocations near the lower channel edge with IMD3 (of allocation and its image) hitting the additional spurious mask

CIM3 region: narrow allocations near the upper channel edge with CIM3 hitting the additional spurious mask.

The regrowth and IMD3 regions were already present in the NS_04 A-MPR definition for PC3, PC2, and PC1.5. For PC1, there are two new regions, the edge region and CIM3 region. The allocation regions are depicted in Figure 7, following the way how the regions have been specified for PC3 / NS_04.

Figure 7: Example of the allocation regions; the edge region is too small to be visible;
“MPR” indicates the region where no A-MPR is applicable.

Table 3 shows the maximum required A-MPR for each waveform and modulation in each allocation region. These A-MPR values should be applied unless a frequency offset from lower band edge prevents violating the additional spurious emission mask. N/A in the table means that no A-MPR is needed.

Table 3: A-MPR values for PC1

| Modulation/Waveform |  | A-MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Edge region | IMD3 region | CIM3 region | Regrowth region |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 11 | ≤ 2 | N/A | ≤ 3.5 |
|  | QPSK | ≤ 11 | ≤ 2 | N/A | ≤ 4.0 |
|  | 16 QAM | ≤ 11 | ≤ 2.5 | ≤ 2.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 11 | ≤ 3 | ≤ 3.0 | ≤ 4.0 |
|  | 256 QAM | ≤ 11 | N/A | N/A | N/A |
| CP-OFDM | QPSK | ≤ 11 | ≤ 2.5 | ≤ 10.5 | ≤ 6.0 |
|  | 16 QAM | ≤ 11 | ≤ 3 | ≤ 10.5 | ≤ 6.0 |
|  | 64 QAM | ≤ 11 | N/A | ≤ 10.5 | ≤ 6.0 |
|  | 256 QAM | ≤ 11 | N/A | ≤ 11 | N/A |

For PC1, NS_04 A-MPR is defined as A-MPR = max(MPR, A-MPRregrowth, A-MPRIMD3, A-MPRCIM3, A-MPRedge).

Taking DFT-s-OFDM 16 QAM modulation as an example, the following test points are defined. Similar test points are specified for each modulation. Test ID selection is marked for A-MPR region as following in tables 4 and 5:

- no colour	=> edge region (MPR apply in this case), test IDs 15 and 17

- green colour	=> IMD3 region, test ID 16

- turquoise colour	=> CIM3 region, test ID 18

- yellow colour	=> regrowth region, test ID 19 for 10 MHz, test ID 20 for 100 MHz with 30 kHz, test ID 21 for 100 MHz with 60 kHz

Table 4: Test Configuration table for NS_04 for PC1 (DFT-s-OFDM 16 QAM)

| A-MPR test parameters for NS_04 for PC1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | Freq |  |  | Channel BW | SCS | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |

| 15 | Low | Default | Default |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| --- | --- | --- | --- | --- | --- | --- |
| 16 | Low | Default | Default |  | DFT-s-OFDM 16 QAM | Inner_1RB_Left |
| 17 | High | Default | Default |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 18 | High | 100 MHz | 30 kHz |  | DFT-s-OFDM 16 QAM | 50@200 |
| 19 (Note 4) | Low | 10 MHz | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 20 (Note 4) | Low | 100 MHz | 30 kHz |  | DFT-s-OFDM 16 QAM | 135@33 |
| 21 (Note 4) | Low | 100 MHz | 60 kHz |  | DFT-s-OFDM 16 QAM | 64@16 |

Table 5: UE Power Class 1 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) |  | A-MPR (dB) |  |  |  | ΔTC,c (dB) | PCMAX,c (dBm) |  | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | <50 MHz CBW | ≥50 MHz CBW | regrowth | IMD3 | CIM3 | edge |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 15 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 2.5 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 16 | 31 | 2 | 2 | 0 | 2.5 | 0 | 0 | 0 | 28.5 | 28.5 | 2.5 | 3 | 33+TT | 25.5-TT | 25.5-TT |
| 17 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 2 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 18 | 31 | 2 | 2 | 0 | 0 | 2 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 19 | 31 | 1 | 1 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 20 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 21 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |

RB allocations are selected so that proper region according to figure 7 is covered. Principle for RB allocation is according to core requirements in TS 38.101-1 V18.6.0 (2024-06).

For regrowth region, A-MPR value is selected first, then check Required TGBW to identify RB allocation.

A-MPRregrowth is obtained from Table 6.2.3.2-3 in terms of total guard bandwidth (TGBW). The TGBW is defined as the frequency distance between the RB allocation and the additional spurious emission limit defined in Table 6.5.3.3.1-1, i.e.,

TGBW=falloc,low-2496 MHz,

where

falloc,low=FC-BWChannel2+BWGB+RBstart∙12 SCS

is the lower edge frequency of the RB allocation, FC is the channel centre frequency, BWChannel is the channel bandwidth, and BWGB is the minimum guard bandwidth defined in Table 5.3.3-1.

Table 6.2.3.2-3: A-MPRregrowth for NS_04 (Power Class 1)

| TGBW range | A-MPRregrowth *(dB) |
| --- | --- |
| G0dB(BWalloc) TGBW | 0 |
| G1dB(BWalloc) TGBW < G0dB(BWalloc) | 1 |
| G2dB(BWalloc) TGBW < G1dB(BWalloc) | 2 |
| G3dB(BWalloc) TGBW < G2dB(BWalloc) | 3 |
| G4dB(BWalloc) TGBW < G3dB(BWalloc) | 4 |
| G5dB(BWalloc) TGBW < G4dB(BWalloc) | 5 |
| TGBW < G5dB(BWalloc) | 6 |

Each function GA(BWalloc) defines the required minimum total guard bandwidth for A-MPR value A and is defined as

GA(BWalloc)=max⁡(0,  C2BWalloc100MHz2+C1BWalloc100MHz+C0),

where BWalloc=LCRB∙12 SCS is the allocation bandwidth, and C2, C1, and C0 are obtained from Table 6.2.3.2-4 for each combination of waveform, modulation, and back-off value A.

Table 6.2.3.2-4: Polynomial coefficients for determining the required 
total guard bandwidth for each value of A-MPRregrowth (Power Class 1)

| Waveform | Modulation | Back-off value A [dB] / Polynomial coefficients C2, C1, C0 [MHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| DFT-s-OFDM | Pi/2-BPSK | -3, 40, 10 | -91, 98, 0 | -248, 143, -9 | -2148, 604, -40 | 0,0,0 | N/A | N/A |
|  | QPSK | -27, 71, 1 | -49, 73, 0 | -76, 76, -3 | -108, 62, -3 | 0,0,0 | N/A | N/A |
|  | 16-QAM | -34, 81, -1 (NOTE 1) | -57, 92, -5 | -69, 82, -5 | -119, 79, -5 | 0,0,0 | N/A | N/A |
|  | 64-QAM | -23, 74, -1 (NOTE 1) | -61, 91, -4 (NOTE 1) | -84, 92, -5 | -116, 80, -5 | 0,0,0 | N/A | N/A |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| CP-OFDM | QPSK | -32, 81, -2 (NOTE 1) | -36, 77, -3 | -40, 70, -3 | -52, 67, -3 | -67, 60, -3 | -101, 56, -4 | 0,0,0 |
|  | 16-QAM | -31, 80, -2 (NOTE 1) | -32, 72, -2 (NOTE 1) | -36, 66, -2 | -48, 62, -2 | -66, 58, -2 | -108, 60, -4 | 0,0,0 |
|  | 64-QAM | -28, 78, -2 (NOTE 1) | -33, 73, -2 (NOTE 1) | -36, 66, -2 (NOTE 1) | -48, 62, -2 | -65, 58, -2 | -124, 67, -5 | 0,0,0 |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Back-off value A is less than floor(MPR), thus this polynomial is irrelevant. NOTE 2: For 256-QAM, always A-MPRregrowth = 0. |  |  |  |  |  |  |  |  |

PROPOSAL 3-2: For PC1, select test points to cover all A-MPR regions according to clause 2.4. Test the A-MPR requirements with both Edge_1RB, Inner_1RB, Outer_Full and specific allocations.

## Test Channel Bandwidth

Minimum requirements for NS_04 applies for all channel bandwidths. Select channel bandwidth to test as for SEM [2]

PROPOSAL 4: Select Lowest and Highest test channel bandwidth for NS_04 A-MPR in FR1.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. lowest and highest supported subcarrier spacing.

PROPOSAL 5: Select Lowest and Highest supported subcarrier spacing for NS_04 A-MPR in FR1.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms).

PROPOSAL 6: Test all UL modulation waveforms supported by the UE for NS_04 A-MPR in FR1.

## Uplink RB allocation

For parts of the band where A-MPR=0, the selection of UL RB allocation can be based on the agreed proposals for SEM [2].

PROPOSAL 7: For NS_04 where A-MPR=0 base the uplink RB allocation on the agreed proposal for SEM.

2.9.	Number of Test Points

The number of test points for NS_04 AMPR is equal to

- 120 test points for Power Class 1.5, 2 and 3

- 149 test points for Power Class 1.

2.10.	Exceptions for A-MPR UL-MIMO test

The requirements of A-MPR for UL-MIMO is the same as the requirements of single antenna A-MPR test case 6.2.3, therefor the test point selection in proposal 1~5 and 7 can be reused without any change.

CP-OFDM is mandatory in case UL-MIMO transmission with two layer data, only CP-OFDM test points selected in proposal 6 are valid for UL-MIMO.

PROPOSAL 8: Test points selected in proposals 1~5 and 7 can be reused by A-MPR UL-MIMO testing, and only CP-OFDM test points selected in proposal 6 are reused for UL-MIMO.

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_04 A-MPR in FR1 test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select Low range and High range test frequencies for NS_04 A-MPR in FR1.

PROPOSAL 3-1:  For PC 1.5, 2 and 3, select additional frequencies (Fc) according to Table 1 and test the A-MPR requirements with both Edge_1RB and Outer_Full allocation.

PROPOSAL 3-2:  For PC 1, select test points to cover all A-MPR regions according to clause 2.4. Test the A-MPR requirements with both Edge_1RB, Inner_1RB, Outer_Full and specific allocations.

PROPOSAL 4: Select Lowest and Highest test channel bandwidth for NS_04 A-MPR in FR1.

PROPOSAL 5: Select Lowest and Highest supported subcarrier spacing for NS_04 A-MPR in FR1.

PROPOSAL 6: Test all UL modulation waveforms supported by the UE for NS_04 A-MPR in FR1

PROPOSAL 7: For NS_04 A-MPR base the uplink RB allocation on the agreed proposal for SEM.

PROPOSAL 8: Test points selected in proposals 1~5 and 7 can be reused by A-MPR UL-MIMO testing, and only CP-OFDM test points selected in proposal 6 are reused for UL-MIMO.

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- R5-184752, “Discussion on test point selection for NS_35 A-MPR in FR1”, (Ericsson)