3GPP TSG-RAN5 Meeting #104	R5-244417

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0361 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to default message and other information elements content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a need to align the MCX test specifications with the latest version (June 2024) of the MCX core specifications to take into account core Working Groups decisions: - Baseline upgrade of MCPTT/MCVideo/MCData-Info (24.379 F.1.2, 24.281 F.1.2, 24.282 D.1). - Baseline upgrade of MCPTT/MCVideo/MCData Location-Info (24.379 F.3, 24.281 F.3, 24.282 D.4). - Baseline upgrade of MCData messages (24.282 clause 15). - Baseline upgrade of MCPTT media plane control messages (24.380). - Baseline upgrade of MCPTT/MCVideo/MCData Service Configuration (24.484 clauses 8.4, 9.4, 10.4). - Baseline upgrade of MCVideo Transmission Control Messages. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Missing elements added to tables in clause 5.5.3.2. - Update of tables in clause 5.5.3.4. NOTE 1: the changes of element names are not backward compatible (EnterSpecificAreaType renamed to EnterSpecificArea and ExitSpecificAreaType renamed to ExitSpecificArea in Tables 5.5.3.4.2-1, 5.5.3.4.2-2, 5.5.3.4.2-3) - Missing fields added to FD SIGNALLING PAYLOAD in tables 5.5.3.8.5-1 and 5.5.3.8.6-1. - New field 'Invited MCPTT User Identity' added to Connect message; clarifications added to clause 5.5.6.1; editorial changes. - Missing fields added to the service configuration documents. NOTE 2: There are new mandatory elements in the MCPTT Service Configuration document; as values the default values from 24.380 Table 11.1.4-3 and Table 11.2.4-3 are used. - MCVideo Transmission Control Messages updated to Rel-18; clarifications added to clause 5.5.11.0. NOTE 3: In Tables 5.5.11.2.7-1 (Media Transmission Notification), 5.5.11.3.1-1 (Transmission End Request), 5.5.11.3.2-1 (Transmission End Response) and 5.5.11.2.15-1 (Transmission End Notify) the "User ID" is replaced by the "User Id of the Transmitting User" which is a non-backward compatible change as the fields have different field ids. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.2, 5.5.3.4, 5.5.3.8.5, 5.5.3.8.6, 5.5.6, 5.5.8.4, 5.5.8.8, 5.5.8.12, 5.5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

#### 5.5.3.2	MCS Info Lists

##### 5.5.3.2.1	MCS Info Lists from the UE

##### -	MCPTT

Table 5.5.3.2.1-1: MCPTT-Info from the UE

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-access-token> with mcpttString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCPTT client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG, GROUPCONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
|  | "first-to-answer" |  |  | FIRST-TO-ANSWER AND INVITE_REFER |
| mcptt-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_A_ID | The URI of the group |  | (GROUP-CALL OR CHAT-GROUP-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_B | The URI of the invited MCPTT Client |  | PRIVATE-CALL AND INVITE_REFER |
|  | encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_A |  |  | POC-SETTINGS-EVENT |
| mcptt-calling-user-id | not present or encrypted (NOTE 2) <mcptt-calling-user-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  |  |
|  | not present |  |  | CONFIG, GROUPCONFIG, POC-SETTINGS-EVENT |
| mcptt-called-party-id | not present |  |  |  |
|  | not present or encrypted (NOTE 2) <mcptt-called-party-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  | INVITE-RSP |
| mcptt-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "true" |  |  | EMERGENCY-CALL AND INVITE_REFER |
| alert-ind | not present or encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to pc_MCX_EmergencyIndWithAlertInd |  |  | EMERGENCY-CALL AND INVITE_REFER |
| imminentperil-ind | not present or encrypted (NOTE 2) <imminentperil-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) < imminentperil -ind> with mcpttBoolean set to "true" |  |  | IMMPERIL-CALL AND INVITE_REFER |
| broadcast-ind | not present or “false” |  |  |  |
|  | "true" |  |  | BROADCAST-CALL |
| mc-org | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | not present |  |  |  |
|  | px_MCPTT_Group_A_ID if mcptt-request-uri contains a temporary group identity; otherwise, not present | if the <mcptt-request-uri> element contains a group identity then this element can include an MCPTT group ID associated with the group identity in the <mcptt-request-uri> element. E.g. if the <mcptt-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCPTT group ID | TS 24.379 [9] clause F.1.3 | GROUP-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcptt-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | The UUID URN of the MCPTT Client | RFC 4122 [106] TS 24.379 [9] clause 4.10 | (GROUP-CALL OR CHAT-GROUP-CALL OR EMERGENCY-CALL OR IMMPERIL-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) |  |  | (PRIVATE-CALL OR FIRST-TO-ANSWER) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | in general mcptt-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.379 [9] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in SIP PUBLISH for MCPTT service settings only, according to TS 24.379 [9] clause 7.2.3 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| gw-mcptt-usage | not present | Rel-18 |  |  |
| anyExt | not present |  | TS 24.379 [9], clause F.1.3 |  |
| anyExt |  | anyExt shall not contain any further elements than listed below unless specified otherwise in the specific message content of a test case | TS 24.379 [9], clause F.1.3 | FUNCTIONAL_ALIAS |
| functional-alias-URI | encrypted (NOTE 2) <functional-alias-URI> with mcpttURI set to px_MCPTT_ID_FA_A | set to the value of the functional alias that is used together with the "mcptt-calling-user-id" |  |  |
| NOTE 1: The SS shall check the mcptt-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-1A |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| REGISTER_PUBLISH | MCPTT-Info in SIP REGISTER or SIP PUBLISH request for service authorisation |
| INVITE_REFER | MCPTT-Info in SIP INVITE or SIP REFER request for call establishment |
| INVITE-RSP | MCPTT-Info in SIP response to a SIP INVITE NOTE: INVITE-RSP is inherited from the SIP response, i.e. it shall be considered as true whenever set for the SIP response |
| FUNCTIONAL_ALIAS | An active Functional Alias is used |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-1A: Encrypted MCPTT info parameter sent by the UE

| Derivation Path: TS 24.379 [9] clauses F.1.2, F.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcptt parameter |  |  |  |

##### -	MCVideo

Table 5.5.3.2.1-2: MCVideo-Info from the UE

| Derivation Path: TS 24.281 [86] Clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcvideo-access-token> with mcvideoString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCVideo client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG GROUPCONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
| mcvideo-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_Group_A_ID | The URI of the group |  | (GROUP-CALL OR CHAT-GROUP-CALL) AND INVITE_REFER |
|  | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_User_B_ID | The URI of the invited MCVideo Client |  | PRIVATE-CALL AND INVITE_REFER |
|  | Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_User_A_ID |  |  | POC-SETTINGS-EVENT |
| mcvideo-calling-user-id | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_ID_User_A |  |  |  |
|  | not present |  |  | CONFIG, GROUPCONFIG, POC-SETTINGS-EVENT |
| mcvideo-called-party-id | not present |  |  |  |
|  | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_ID_User_A |  |  | INVITE-RSP |
| mcvideo-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or encrypted (NOTE 2) <emergency-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <emergency-ind> with mcvideoBoolean set to true |  |  | EMERGENCY-CALL AND INVITE-REFER |
| alert-ind | not present or encrypted (NOTE 2) <alert-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <alert-ind> with mcvideoBoolean set to pc_MCX_EmergencyIndWithAlertInd |  |  | EMERGENCY-CALL AND INVITE_REFER |
| imminentperil-ind | not present or encrypted (NOTE 2) <imminentperil-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <imminentperil-ind> with mcvideoBoolean set to true |  |  | IMMPERIL-CALL AND INVITE-REFER |
| broadcast-ind | not present or “false” |  |  |  |
|  | "true" |  |  | BROADCAST-CALL |
| mc-org | not present |  |  |  |
| associated-group-id | not present |  |  |  |
|  | px_MCVideo_Group_A_ID if mcvideo-request-uri contains a temporary group identity; otherwise, not present | if the <mcvideo-request-uri> element contains a group identity then this element can include an MCVideo group ID associated with the group identity in the <mcvideo-request-uri> element. E.g. if the <mcvideo-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCVideo group ID | TS 24.281 [86] clause F.1.3 | GROUP-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcvideo-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | The UUID URN of the MCVIDEO Client | RFC 4122 [106] TS 24.281 [86] clause 4.9 | (GROUP-CALL OR CHAT-GROUP-CALL OR EMERGENCY-CALL OR IMMPERIL-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | in general mcvideo-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.281 [86] clause 4.9 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | mcvideo-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.281 [86] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.281 [86] clause 4.9 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | mcvideo-client-id is mandatory in SIP PUBLISH for MCVideo service settings only, according to TS 24.281 [86] clause 7.2.3 | RFC 4122 [106] TS 24.281 [86] clause 4.9 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| multiple-devices-ind | not present |  |  |  |
| video-pull-url | not present |  |  |  |
| gw-mcvideo-usage | not present | Rel-18 |  |  |
| anyExt | not present |  | TS 24.281 [86] clause F.1.3 |  |
| NOTE 1: The SS shall check the mcvideo-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-2A |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| REGISTER_PUBLISH | MCVideo-Info in SIP REGISTER or SIP PUBLISH request for service authorisation |
| INVITE_REFER | MCVideo-Info in SIP INVITE or SIP REFER request for call establishment |
| INVITE-RSP | MCVideo-Info in SIP response to a SIP INVITE |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-2A: Encrypted MCVideo info parameter sent by the UE

| Derivation Path: TS 24.281 [86] clauses F.1.2, F.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcvideo parameter |  |  |  |

##### -	MCData

Table 5.5.3.2.1-3: MCData-Info from the UE

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-access-token> with mcdataString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCData client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG GROUPCONFIG |
| request-type | not present |  |  |  |
|  | "one-to-one-sds" |  |  | MCD_1to1 |
|  | “group-sds” |  |  | MCD_grp |
| mcdata-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID |  |  | MCD_grp |
|  | Encrypted (NOTE 2) <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID |  |  | POC-SETTINGS-EVENT |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-called-party-id | not present |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |
| alert-ind | not present |  |  |  |
| originated-by | not present |  |  |  |
| mcdata-client-id | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) |  |  | MCD_grp |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) |  |  | CONFIG AND PUBLISH |
|  | not present or encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) | in general mcdata-client-id is not mandatory (e.g. for SIP SUBSCRIBE) |  | (CONFIG OR GROUPCONFIG) AND NOT REGISTER (NOTE 3) |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) | mcdata-client-id is mandatory in SIP PUBLISH for MCData service settings only, according to TS 24.282 [87] clause 7.2.3 | RFC 4122 [106] | POC-SETTINGS-EVENT |
| mcdata-controller-psi | not present |  |  |  |
| gw-mcdata-usage | not present | Rel-18 |  |  |
| anyExt | not present |  |  |  |
| anyExt |  | anyExt shall not contain any further elements than listed below unless specified otherwise in the specific message content of a test case |  | PRE_ESTABLISHED_SESSION OR FUNCTIONAL_ALIAS |
| pre-established-session-ind | “true” | TS 24.282 [87], Clause 18.3.2.1 |  | PRE_ESTABLISHED_SESSION |
| functional-alias-URI | encrypted (NOTE 2) <functional-alias-URI> with mcdataURI set to px_MCData_ID_FA_A | set to the value of the functional alias that is used together with the "mcdata-calling-user-id" |  | FUNCTIONAL_ALIAS |
| NOTE 1: The SS shall check the mcdata-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-3A NOTE 3: In contrast to MCPTT and MCVideo for MCData TS 24.282 [87] clause 7.2.1 does not specify the client-id to be included in the REGISTER request. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A goup MCData call |
| REGISTER | MCData-Info in SIP REGISTER request for service authorisation |
| PUBLISH | MCData-Info in SIP PUBLISH request for service authorisation |
| PRE_ESTABLISHED_SESSION | A pre-established sessions is being established |
| FUNCTIONAL_ALIAS | An active Functional Alias is used |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-3A: Encrypted MCData info parameter sent by the UE

| Derivation Path: TS 24.282 [87] clauses D.1.2, D.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcdata parameter |  |  |  |

##### 5.5.3.2.2	MCS Info Lists from the SS

##### -	MCPTT

Table 5.5.3.2.2-1: MCPTT-Info from the SS

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-access-token | not present |  |  |  |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL |
|  | "private" |  |  | PRIVATE-CALL |
|  | “chat” |  |  | CHAT-GROUP-CALL |
|  | "first-to-answer" |  |  | FIRST-TO-ANSWER |
| mcptt-request-uri | Encrypted (NOTE 1) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_A | The URI of the called user |  |  |
| mcptt-calling-user-id | Encrypted (NOTE 1) <mcptt-calling-user-id> with mcpttURI set to px_MCPTT_ID_User_B | The URI of the calling user |  |  |
| mcptt-called-party-id | not present |  |  |  |
| mcptt-calling-group-id | not present |  |  |  |
|  | Encrypted (NOTE 1) <mcptt-calling-group-id> with mcpttURI set to px_MCPTT_Group_A_ID | The URI of the group |  | GROUP-CALL OR CHAT-GROUP-CALL |
| required | not present |  |  |  |
| emergency-ind | not present |  |  |  |
|  | Encrypted (NOTE 1) <emergency-ind> with mcpttBoolean set to "true" |  |  | EMERGENCY-CALL |
| alert-ind | not present |  |  |  |
|  | Encrypted (NOTE 1) <alert-ind> with mcpttBoolean set to "false" |  |  | EMERGENCY-CALL |
| imminentperil-ind | not present |  |  |  |
|  | Encrypted (NOTE 1) <imminentperil-ind> with mcpttBoolean set to "true" |  |  | IMMPERIL-CALL |
| broadcast-ind | not present |  |  |  |
|  | "true" |  |  | BROADCAST-CALL |
| mc-org | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | not present |  |  |  |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcptt-client-id | not present |  |  |  |
| alert-ind-rcvd | not present |  |  |  |
| gw-mcptt-usage | not present | Rel-18 |  |  |
| anyExt | not present |  | TS 24.379 [9], clause F.1.3 |  |
| NOTE 1: Encrypted element as described in Table 5.5.3.2.2-1A |  |  |  |  |

Table 5.5.3.2.2-1A: Encrypted MCPTT info parameter sent by the SS

| Derivation Path: TS 24.379 [9] clauses F.1.2, F.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-2 containing encrypted element content of the mcptt parameter |  |  |  |

##### -	MCVideo

Table 5.5.3.2.2-2: MCVideo-Info from the SS

| Derivation Path: TS 24.281 [86] Clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL |
|  | "private" |  |  | PRIVATE-CALL |
|  | "chat" |  |  | CHAT-GROUP-CALL |
| mcvideo-request-uri | Encrypted (NOTE 1) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_ID_User_A | The URI of the called user |  |  |
| mcvideo-calling-user-id | Encrypted (NOTE 1) <mcvideo-calling-user-id> with mcvideoURI set to px_MCVideo_ID_User_B | The URI of the calling user |  |  |
| mcvideo-called-party-id | not present |  |  |  |
| mcvideo-calling-group-id | not present |  |  |  |
|  | Encrypted (NOTE 1) <mcvideo-calling-group-id> with mcvideoURI set to px_MCVideo_Group_A_ID | The URI of the group |  | GROUP-CALL OR CHAT-GROUP-CALL |
| required | not present |  |  |  |
| emergency-ind | Encrypted (NOTE 1) <emergency-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 1) <emergency-ind> with mcvideoBoolean set to "true" |  |  | EMERGENCY-CALL |
| alert-ind | not present |  |  |  |
|  | Encrypted (NOTE 1) <alert-ind> with mcvideoBoolean set to "false" |  |  | EMERGENCY-CALL |
| imminentperil-ind | not present |  |  |  |
|  | Encrypted (NOTE 1) <imminentperil-ind> with mcvideoBoolean set to "true" |  |  | IMMPERIL-CALL |
| broadcast-ind | not present |  |  |  |
|  | "true" |  |  | BROADCAST-CALL |
| mc-org" | not present |  |  |  |
| associated-group-id | not present |  |  |  |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcvideo-client-id | not present |  |  |  |
| alert-ind-rcvd | not present |  |  |  |
| multiple-devices-ind | not present |  |  |  |
| video-pull-url | not present |  |  |  |
| gw-mcvideo-usage | not present | Rel-18 |  |  |
| anyExt | not present |  | TS 24.281 [86] clause F.1.3 |  |
| NOTE 1: Encrypted element as described in Table 5.5.3.2.2-2A |  |  |  |  |

Table 5.5.3.2.2-2A: Encrypted MCVideo info parameter sent by the SS

| Derivation Path: TS 24.281 [86] clauses F.1.2, F.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-2 containing encrypted element content of the mcvideo parameter |  |  |  |

##### -	MCData

Table 5.5.3.2.2-3: MCData-Info from the SS

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |  |
| mcdata-info |  |  |  |  |  |
| mcdata-Params |  |  |  |  |  |
| mcdata-access-token | not present |  |  |  |  |
| request-type | not present |  |  |  |  |
|  | "one-to-one-sds" |  |  | MCD_1to1 |  |
|  | “group-sds” |  |  | MCD_grp |  |
| mcdata-request-uri | Encrypted (NOTE 1) <mcdata-request-uri> with mcdataURI set to px_MCData_ID_User_A |  |  |  |  |
| mcdata-calling-user-id | Encrypted (NOTE 1) <mcdata-calling-user-id> with mcdataURI set to px_MCData_ID_User_B |  |  |  |  |
| mcdata-called-party-id | not present |  |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |  |
|  | Encrypted (NOTE 1) <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID |  |  | MCD_grp |  |
| alert-ind | not present |  |  |  |  |
| originated-by | not present |  |  |  |  |
| mcdata-client-id | not present |  |  |  |  |
|  | Encrypted (NOTE 1) <mcdata-client-id> with mcdataString set to px_MCX_Client_B_ID |  |  | MCD_grp |  |
| mcdata-controller-psi | not present |  |  |  |  |
| gw-mcdata-usage | not present | Rel-18 |  |  |  |
| anyExt | not present |  |  |  |
| NOTE 1: Encrypted element as described in Table 5.5.3.2.2-3A |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A group MCData call |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.2-3A: Encrypted MCData info parameter sent by the SS

| Derivation Path: TS 24.282 [87] clauses D.1.2, D.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.232 containing encrypted element content of the mcdata parameter |  |  |  |

<End of modification>

<Start of modification>

#### 5.5.3.4	Location-info

##### 5.5.3.4.1	Location-info (Report from the UE)

##### -	MCPTT

Table 5.5.3.4.1-1: Location-info (Report from the UE) for MCPTT

| Derivation Path: TS 24.379 [9] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | not present | Attribute is used to return the value in the <RequestId> attribute in the <Request> element. Only present in response to a Location-Info Request. |  |  |
| ReportType attribute | "Emergency" | Required The <ReportType> attribute has two values "Emergency" and "NonEmergency" used to inform whether the client is sending the report in an emergency situation or not. |  |  |
| TriggerID | not present | An element which can occur multiple times. Contains the value of the <TriggerId> attribute associated with a trigger that has fired. Only present if a trigger is the cause of the Location-info Report. |  |  |
| mcptt-reporting-uri | not present | Rel-18 |  |  |
| CurrentLocation |  | A mandatory element that contains the location information |  |  |
| CurrentServingEcgi | Encrypted (NOTE 2) <CurrentServingEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| NeighbouringEcgi | Encrypted (NOTE 2) <NeighbouringEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbmsSaId | Encrypted (NOTE 2) <MbmsSaId> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbsfnArea | Encrypted (NOTE 2) <MbsfnArea> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| CurrentCoordinate | if present | This is optional depending on the configuration sent by the SS |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with any content |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with any content |  |  |  |
| NOTE 1: Encrypted sub-element of <CurrentCoordinate> as described in Table 5.5.3.4.1-1A NOTE 2: Encrypted sub-element of <CurrentLocation> as described in Table 5.5.3.4.1-1B |  |  |  |  |

Table 5.5.3.4.1-1A: Encrypted sub-element of <CurrentCoordinate> sent by the UE

| Derivation Path: TS 24.379 [9] clause F.3.2 (tCoordinateType ) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

Table 5.5.3.4.1-1B: Encrypted sub-element of <CurrentLocation> sent by the UE

| Derivation Path: TS 24.379 [9] clause F.3.2 (tCurrentLocationType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentLocation> |  |  |  |

##### -	MCVideo

Table 5.5.3.4.1-2: Location-info (Report from the UE) for MCVideo

| Derivation Path: TS 24.281 [86] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | not present | Attribute is used to return the value in the <RequestId> attribute in the <Request> element. Only present in response to a Location-Info Request. |  |  |
| ReportType attribute | "Emergency" | Required The <ReportType> attribute has two values "Emergency" and "NonEmergency" used to inform whether the client is sending the report in an emergency situation or not. |  |  |
| TriggerID | not present | An element which can occur multiple times. Contains the value of the <TriggerId> attribute associated with a trigger that has fired. Only present if a trigger is the cause of the Location-info Report. |  |  |
| mcvideo-reporting-uri | not present | Rel-18 |  |  |
| CurrentLocation |  | A mandatory element that contains the location information |  |  |
| CurrentServingEcgi | Encrypted (NOTE 2) <CurrentServingEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| NeighbouringEcgi | Encrypted (NOTE 2) <NeighbouringEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbmsSaId | Encrypted (NOTE 2) <MbmsSaId> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbsfnArea | Encrypted (NOTE 2) <MbsfnArea> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| CurrentCoordinate | if present | This is optional depending on the configuration sent by the SS |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with any content |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with any content |  |  |  |
| NOTE 1: Encrypted sub-element of <CurrentCoordinate> as described in Table 5.5.3.4.1-2A NOTE 2: Encrypted sub-element of <CurrentLocation> as described in Table 5.5.3.4.1-2B |  |  |  |  |

Table 5.5.3.4.1-2A: Encrypted sub-element of <CurrentCoordinate> sent by the UE

| Derivation Path: TS 24.281 [86] clause F.3.2 (tCoordinateType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

Table 5.5.3.4.1-2B: Encrypted sub-element of <CurrentLocation> sent by the UE

| Derivation Path: TS 24.281 [86] clause F.3.2 (tCurrentLocationType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentLocation> |  |  |  |

##### -	MCData

Table 5.5.3.4.1-3: Location-info (Report from the UE) for MCData

| Derivation Path: TS 24.282 [87] clause D.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | not present | Attribute is used to return the value in the <RequestId> attribute in the <Request> element. Only present in response to a Location-Info Request. |  |  |
| ReportType attribute | "Emergency" | Required The <ReportType> attribute has two values "Emergency" and "NonEmergency" used to inform whether the client is sending the report in an emergency situation or not. |  |  |
| TriggerID | not present | An element which can occur multiple times. Contains the value of the <TriggerId> attribute associated with a trigger that has fired. Only present if a trigger is the cause of the Location-info Report. |  |  |
| mcdata-reporting-uri | not present | Rel-18 |  |  |
| CurrentLocation |  | A mandatory element that contains the location information |  |  |
| CurrentServingEcgi | Encrypted (NOTE 2) <CurrentServingEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| NeighbouringEcgi | Encrypted (NOTE 2) <NeighbouringEcgi> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbmsSaId | Encrypted (NOTE 2) <MbmsSaId> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| MbsfnArea | Encrypted (NOTE 2) <MbsfnArea> with any content if present | This is optional depending on the configuration sent by the SS |  |  |
| CurrentCoordinate | if present | This is optional depending on the configuration sent by the SS |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with any content |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with any content |  |  |  |
| NOTE 1: Encrypted sub-element of <CurrentCoordinate> as described in Table 5.5.3.4.1-2A NOTE 2: Encrypted sub-element of <CurrentLocation> as described in Table 5.5.3.4.1-2B |  |  |  |  |

Table 5.5.3.4.1-3A: Encrypted sub-element of <CurrentCoordinate> sent by the UE

| Derivation Path: TS 24.282 [87] clause d.4.2 (tCoordinateType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

Table 5.5.3.4.1-3B: Encrypted sub-element of <CurrentLocation> sent by the UE

| Derivation Path: TS 24.282 [87] clause D.4.2 (tCurrentLocationType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the sub-element of <CurrentLocation> |  |  |  |

##### 5.5.3.4.2	Location-info (Configuration sent by the SS)

##### -	MCPTT

Table 5.5.3.4.2-1: Location-info (Configuration sent by the SS) for MCPTT

| Derivation Path: TS 24.379 [9] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| ConfigScope | "Full" | The MCPTT Client shall replace any previous configuration. |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "10" | A mandatory element specifying the minimum time the MCPTT client needs to wait between sending location reports. The value is given in seconds |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "5" | A mandatory element specifying the minimum time the MCPTT client needs to wait between sending location reports. The value is given in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McpttSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificArea | not present |  |  |  |
| ExitSpecificArea | not present |  |  |  |
| anyExt |  | mandatory for Rel-15 and above |  |  |
| EmergencyTriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McpttSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificArea | not present |  |  |  |
| ExitSpecificArea | not present |  |  |  |

##### -	MCVideo

Table 5.5.3.4.2-2: Location-info (Configuration sent by the SS) for MCVideo

| Derivation Path: TS 24.281 [86] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| ConfigScope | "Full" | The MCVideo Client shall replace any previous configuration. |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "10" | A mandatory element specifying the minimum time the MCVIdeo client needs to wait between sending location reports. The value is given in seconds |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "5" | A mandatory element specifying the minimum time the MCVideo client needs to wait between sending location reports. The value is given in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McvideoSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificArea | not present |  |  |  |
| ExitSpecificArea | not present |  |  |  |

##### -	MCData

Table 5.5.3.4.2-3: Location-info (Configuration sent by the SS) for MCData

| Derivation Path: TS 24.282 [87] clause D.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| ConfigScope | "Full" | The MCData Client shall replace any previous configuration. |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "10" | A mandatory element specifying the minimum time the MCData client needs to wait between sending location reports. The value is given in seconds |  |  |
| EmergencyLocationInformation" |  |  |  |  |
| ServingEcgi | present | An optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported |  |  |
| NeighbouringEcgi | present | An optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported |  |  |
| MbmsSaId | present | An optional element specifying that the serving MBMS Service Area Id needs to be reported; |  |  |
| MbsfnArea | present | An optional element specifying that the MBSFN area Id needs to be reported; |  |  |
| GeographicalCoordinate | present | An optional element specifying that the geographical coordinate specified in clause 6.1 in 3GPP TS 23.032 [65] needs to be reported |  |  |
| minimumIntervalLength | "5" | A mandatory element specifying the minimum time the MCData client needs to wait between sending location reports. The value is given in seconds |  |  |
| TriggeringCriteria |  |  |  |  |
| CellChange | not present |  |  |  |
| TrackingAreaChange | not present |  |  |  |
| PlmnChange | not present |  |  |  |
| MbmsSaChange | not present |  |  |  |
| MbsfnAreaChange | not present |  |  |  |
| PeriodicReport | not present |  |  |  |
| TravelledDistance | not present |  |  |  |
| McdataSignallingEvent | not present |  |  |  |
| GeographicalAreaChange |  |  |  |  |
| AnyAreaChange | not present |  |  |  |
| EnterSpecificArea | not present |  |  |  |
| ExitSpecificArea | not present |  |  |  |

##### 5.5.3.4.3	Location-info (Request sent by the SS)

##### -	MCPTT

Table 5.5.3.4.3-1: Location-info (Request sent by the SS) for MCPTT

| Derivation Path: TS 24.379 [9] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Request |  |  |  |  |
| RequestID | "1" | The RequestID that the MCPTT Client will reference in the Report |  |  |
| refresh | not present | Rel-18 |  |  |

##### -	MCVideo

Table 5.5.3.4.3-2: Location-info (Request sent by the SS) for MCVideo

| Derivation Path: TS 24.281 [96] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Request |  |  |  |  |
| RequestID | "1" | The RequestID that the MCVideo Client will reference in the Report |  |  |
| refresh | not present | Rel-18 |  |  |

##### -	MCData

Table 5.5.3.4.3-3: Location-info (Request sent by the SS) for MCData

| Derivation Path: TS 24.282 [87] clause D.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Request |  |  |  |  |
| RequestID | "1" | The RequestID that the MCData Client will reference in the Report |  |  |
| refresh | not present | Rel-18 |  |  |

##### 5.5.3.4.4	Location-info (Report from the SS)

##### -	MCPTT

Table 5.5.3.4.4-1: Location-info (Report from the SS) for MCPTT

| Derivation Path: TS 24.379 [9] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | not present |  |  |  |
| ReportType attribute | "Emergency" |  |  |  |
| TriggerID | not present |  |  |  |
| mcptt-reporting-uri | not present | Rel-18 |  |  |
| CurrentLocation |  |  |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with content as specified by the test case |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with content as specified by the test case |  |  |  |
| NOTE 1: Encrypted tCoordinateType element as described in Table 5.5.3.4.4-1A |  |  |  |  |

Table 5.5.3.4.4-1A: Encrypted sub-element of <CurrentCoordinate> sent by the SS

| Derivation Path: TS 24.379 [9] clause F.3.2 (tCoordinateType ) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-2 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

##### -	MCVideo

Table 5.5.3.4.4-2: Location-info (Report from the SS) for MCVideo

| Derivation Path: TS 24.281 [86] clause F.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | not present |  |  |  |
| ReportType attribute | "Emergency" |  |  |  |
| TriggerID | not present |  |  |  |
| mcvideo-reporting-uri | not present | Rel-18 |  |  |
| CurrentLocation |  |  |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with content as specified by the test case |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with content as specified by the test case |  |  |  |
| NOTE 1: Encrypted sub-element of <CurrentCoordinate> as described in Table 5.5.3.4.1-2A |  |  |  |  |

Table 5.5.3.4.4-2A: Encrypted sub-element of <CurrentCoordinate> sent by the SS

| Derivation Path: TS 24.281 [86] clause F.3.2 (tCoordinateType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-2 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

##### -	MCData

Table 5.5.3.4.4-3: Location-info (Report from the SS) for MCData

| Derivation Path: TS 24.282 [87] clause D.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | not present |  |  |  |
| ReportType attribute | "Emergency" |  |  |  |
| TriggerID | not present |  |  |  |
| mcdata-reporting-uri | not present | Rel-18 |  |  |
| CurrentLocation |  |  |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted (NOTE 1) <longitude> with content as specified by the test case |  |  |  |
| latitude | Encrypted (NOTE 1) <latitude> with content as specified by the test case |  |  |  |
| NOTE 1: Encrypted sub-element of <CurrentCoordinate> as described in Table 5.5.3.4.1-2A |  |  |  |  |

Table 5.5.3.4.4-3A: Encrypted sub-element of <CurrentCoordinate> sent by the SS

| Derivation Path: 24.282 [87] clause D.4.2 (tCoordinateType) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-2 containing encrypted element content of the sub-element of <CurrentCoordinate> |  |  |  |

<End of modification>

<Start of modification>

##### 5.5.3.8.5	FD SIGNALLING PAYLOAD message from the UE

Table 5.5.3.8.5-1: FD SIGNALLING PAYLOAD message from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD signalling payload message identity | '00000010'B | FD SIGNALLING PAYLOAD | TS 24.282 [87] clause 15.2.2 |  |
| Date and time | Any allowed value | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Any allowed value | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Any allowed value | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| InReplyTo message ID | Not present |  | TS 24.282 [87] clause 15.2.11 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| FD disposition request type | “0001” | FILE DOWNLOAD COMPLETED UPDATE | TS 24.282 [87] clause 15.2.4 |  |
| Mandatory download | Not present | Not present indicates a Non-Mandatory download | TS 24.282 [87] clause 15.2.16 |  |
|  | '0001'B | MANDATORY DOWNLOAD |  | FD_MSRP |
| Payload |  |  | TS 24.282 [87] clause 15.2.13 | FD_HTTP |
| Length of Payload contents | Length of the payload contents |  |  |  |
| Payload content type | “00000100” | FILEURL |  |  |
| Payload contents | same URL as assigned by the SS in the HTTP 201 (Created) response to the HTTP POST request |  |  |  |
| Metadata | if present | Metadata is optional | TS 24.282 [87] clause 15.2.17 | FD_HTTP |
| file-selector | Any allowed value |  |  |  |
| file-date | Any allowed value |  |  |  |
| file-availability | Any allowed value |  |  |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| User location | Any allowed value if present | Rel-18 | TS 24.282 [87] clause 15.2.25 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |
| Application metadata container | Any allowed value if present | Rel-17 | TS 24.282 [87] clause 15.2.28 |  |

<End of modification>

<Start of modification>

##### 5.5.3.8.6	FD SIGNALLING PAYLOAD message from the SS

Table 5.5.3.8.6-1: FD SIGNALLING PAYLOAD message from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD signalling payload message identity | '00000010'B | FD SIGNALLING PAYLOAD | TS 24.282 [87] clause 15.2.2 |  |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | '01010101010101010101010101010101'O | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | '01010101010101010101010101010101'O | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| InReplyTo message ID | Not present |  | TS 24.282 [87] clause 15.2.11 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| FD disposition request type | '0001'B | FILE DOWNLOAD COMPLETED UPDATE | TS 24.282 [87] clause 15.2.4 |  |
| Mandatory download | Not present | Not present indicates a Non-Mandatory download | TS 24.282 [87] clause 15.2.16 |  |
|  | '0001'B | MANDATORY DOWNLOAD |  | FD_MSRP |
| Length of Payload contents | Length of the payload contents |  |  |  |
| Payload content type | “00000100” | FILEURL |  |  |
| Payload contents | tsc_MCData_MSF_URI & "/" & sub-path | URL identifying the location of the stored file; sub-path is arbitrarily selected by the SS and shall be different for every file upload of a test case |  |  |
| Metadata |  | NOTE 1 | TS 24.282 [87] clause 15.2.17 | FD_HTTP |
| file-selector |  |  | RFC 5547 [124] |  |
| filename | name of the file | e.g. "TestFile.txt" |  |  |
| filesize | size of the file |  |  |  |
| type | type of the file | e.g. "text/plain" |  |  |
| hash |  |  |  |  |
| algorithm | "sha-1" |  |  |  |
| value | hash value of the file |  |  |  |
| file-date |  |  | RFC 5547 [124] |  |
| date-param[1] |  |  |  |  |
| type | "creation" |  |  |  |
| date-time | date and time when the file has been created | e.g. "Mon, 20 Dec 2021 15:01:31 +0100" | RFC 5322 [109] |  |
| file-availability | Date and time until which the file is available | e.g. "Fri, 30 Dec 2050 23:59:59 +0100" | TS 24.282 [87] table 15.2.17-1 |  |
| file-description | "Test file" |  | TS 24.282 [87] table 15.2.17-1 |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |
| User location | Not present | Rel-18 | TS 24.282 [87] clause 15.2.25 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |
| Application metadata container | Not present | Rel-17 | TS 24.282 [87] clause 15.2.28 |  |
| NOTE 1: file-selector, file-date, file-availability and file-description are concatenated using CRLF (carriage-return/line-feed) as separator |  |  |  |  |

<End of modification>

<Start of modification>

### 5.5.6	Default MCPTT media plane control messages and other information elements

#### 5.5.6.1	General

The media plane control protocols messages specified in the present document are based on those specified in TS 24.380 [10] which in term are based on the RTCP Application Packets (RTCP: APP), as defined in IETF RFC 3550 [76].

The following conditions apply throughout clause 5.5.6:

Table 5.5.6.1-1: Conditions

| Condition | Explanation |
| --- | --- |
| FA | IE for when an active Functional Alias is used |
| Multi-Talker | IE for when a Multi Talker call is active |
| ACK | Message requests a Floor Ack |
| NOTE: For further conditions see table 5.5.1-1 |  |

For MCPTT media plane control different SSRCs (Synchronization SouRCes) need to be distinguished. Table 5.5.6.1-2 specifies the SSRCs as used in the default MCPTT media plane control messages for the case that there is no multiplexing of media plane control channels.

NOTE 1:	Multiplexing of media plane control channels has been introduced in Rel-18 of TS 24.379 [9] and TS 24.380 [10] and may be specified in Rel-18 and above test cases.

Table 5.5.6.1-2: SSRCs in MCPTT media plane control messages
(No multiplexing of media plane control channels)

| SSRC (NOTE 1) | Description | Value |
| --- | --- | --- |
| Audio SSRC of the client (NOTE 1) | SSRC to be used by the client (Client A) in the audio stream | Arbitrarily selected by the SS and assigned to the client when the floor is granted (NOTE 2) using the mc_ssrc fmtp attribute in case of implicit grant or by a Floor Granted message otherwise (NOTE 3) |
| Audio SSRC of a remote client (NOTE 1) | SSRC of the audio stream of a remote client (Client B) | Arbitrarily selected by the SS (NOTE 2) |
| RTCP SSRC of the client (NOTE 1) | SSRC used by the client (Client A) in the RTCP header of the MCPTT media plane control messages sent to the SS | The client may use any value, value is not checked by the SS (NOTE 4). |
| RTCP SSRC of the SS (NOTE 1) | SSRC used by the SS in the RTCP header of the MCPTT media plane control messages sent to the client | Arbitrarily selected by the SS (NOTE 4) |
| NOTE 1: The terms "Audio SSRC" and "RTCP SSRC" are as introduced in Rel-18 of TS 24.380 [10]. NOTE 2: Different SSRC values shall be selected by the SS for audio streams from different clients There is no need to consider collision detection and resolution according to IETF RFC 3550 [76]. NOTE 3: Even though the client may use the a=ssrc SDP attribute to indicate which SSRC it would like to use in the audio stream, according to TS 24.380 [10] the client has to use the value provided by the server in the "mc_ssrc" fmtp attribute of the SDP answer or in the "Audio SSRC of Granted Participant" field of the Floor Granted message. NOTE 4: In clause 4.3.3.1 TS 24.380 [10] clarifies in Rel-18 that "the SSRC of the RTCP header is used to enable multiplexing of media plane control channels"; in clauses 14.2.7 and 14.3.8 it is clarified that the "mc_floor_ssrc" fmtp attribute is used to indicate support of multiplexing and to exchange of the SSRC values to be used in the RTCP header. It is assumed that RTCP SSRC values have no meaning in case of no multiplexing. |  |  |

#### 5.5.6.2	Floor Request

Table 5.5.6.2-1: Floor Request

| Derivation Path: 24.380 [10], Table 8.2.4-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00000 | Floor Request |  |
| SSRC | RTCP SSRC of the client |  |  |
|  | The SSRC of the message sender |  | OFF-NETWORK |
| name | MCPT |  |  |
| Floor priority | Not present or Any allowed value | If present, a value between ‘0’ and ‘255’ where '0' is the lowest priority If the Floor Priority field is not included in the message the default priority (='0') is used as the Floor Priority value The max floor priority that can be requested in a Floor Request message is negotiated between the MCPTT client and the controlling MCPTT function using the "mc_priority" fmtp parameter e.g. at call setup |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT User ID of the floor participant requesting the floor. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 10000x0000000000 | Normal call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise |  |
|  | 01000x0000000000 | Broadcast group call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | BROADCAST-CALL |
|  | 00010x0000000000 | Emergency call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | EMERGENCY-CALL |
|  | 00001x0000000000 | Imminent Peril call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | IMMPERIL-CALL |
| Functional Alias | Not present |  |  |
|  | px_MCPTT_ID_FA_A | Functional Alias = URI | FA |
| Location | optional |  |  |
| Location Type | Any allowed value | See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present or Any allowed value | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |
| Location |  |  | REL-15 |
| Location Type | Any allowed value | See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present or Any allowed value | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |

| Condition | Explanation |
| --- | --- |
| REL-15 | In effect when PICS "PICS FFS" is in effect |

#### 5.5.6.3	Floor Granted

Table 5.5.6.3-1: Floor Granted

| Derivation Path: 24.380 [10], Table 8.2.5-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00001 | Floor Granted with acknowledgment not required |  |
|  | 10001 | Floor Granted with acknowledgment required | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server |  |
|  | The SSRC of the message sender | The SSRC of the floor arbitrator | OFF-NETWORK |
| name | MCPT |  |  |
| Duration |  |  |  |
| Duration | "00000000 10000000" | 128 sec (an arbitrary value) |  |
| Audio SSRC of Granted Participant | Audio SSRC of the client |  |  |
| Floor priority | Not present | If the Floor Priority field is not included in the message the default priority (='0') is used as the Floor Priority value |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT User ID of the floor participant granted the floor. |  |
| Queue Size | Not present |  |  |
| Queue Size | "0" | the number of queued MCPTT clients in the MCPTT call | OFF-NETWORK |
| SSRC of queued floor participant | Not present |  |  |
| Queued User ID | Not present |  |  |
| Queue Info | Not present |  |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0100010000000000 | Broadcast group call, queueing supported | BROADCAST-CALL |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 0000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.4	Floor Deny

Table 5.5.6.4-1: Floor Deny

| Derivation Path: 24.380 [10], Table 8.2.6-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00011 | Floor Deny with acknowledgment not required |  |
|  | 10011 | Floor Deny with acknowledgment required | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server |  |
|  | The SSRC of the message sender | The SSRC of the floor arbitrator | OFF-NETWORK |
| name | MCPT |  |  |
| Reject Cause |  |  |  |
| Reject Cause | "1" | Cause #1 - Another MCPTT client has permission |  |
| Reject Phrase | "Another MCPTT client has permission" | An additional text string explaining the reason for rejecting the floor request. |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT User ID of the floor participant being denied floor request. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0100010000000000 | Broadcast group call, queueing supported | BROADCAST-CALL |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.5	Floor Release

Table 5.5.6.5-1: Floor Release

| Derivation Path: 24.380 [10], Table 8.2.7-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | x0100 | Floor Release with x=0,1 depending on the UE implementation; x=0: Acknowledgment is not required x=1: Acknowledgment is required |  |
| SSRC | RTCP SSRC of the client |  |  |
|  | The SSRC of the message sender |  | OFF-NETWORK |
| name | MCPT |  |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT User ID of the floor participant releasing the floor. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 10000x0000000000 | Normal call x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise |  |
|  | 01000x0000000000 | Broadcast group call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | BROADCAST-CALL |
|  | 00010x0000000000 | Emergency call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | EMERGENCY-CALL |
|  | 00001x0000000000 | Imminent Peril call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | IMMPERIL-CALL |

#### 5.5.6.6	Floor Idle

Table 5.5.6.6-1: Floor Idle

| Derivation Path: 24.380 [10], Table 8.2.8-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00101 | Floor Idle with acknowledgment not required |  |
|  | 10101 | Floor Idle with acknowledgment required | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server |  |
|  | The SSRC of the message sender | The SSRC of the floor arbitrator | OFF-NETWORK |
| name | MCPT |  |  |
| Message Sequence Number |  |  |  |
| Message Sequence Number | The value sent in the previous Floor Idle message, if any, increased with 1 | Any value between '0' and '65535' When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0100010000000000 | Broadcast group call, queueing supported | BROADCAST-CALL |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.7	Floor Taken

Table 5.5.6.7-1: Floor Taken

| Derivation Path: 24.380 [10], Table 8.2.9-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00010 | Floor Taken with acknowledgment not required |  |
|  | 10010 | Floor Taken with acknowledgment required | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server |  |
|  | The SSRC of the message sender | The SSRC of the floor arbitrator | OFF-NETWORK |
| name | MCPT |  |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT user ID of the floor participant sending the Floor Taken message |  |
| Granted Party's Identity |  |  |  |
| Granted Party's Identity | px_MCPTT_ID_User_B | The MCPTT User ID of the floor participant being granted the floor. |  |
| Granted Party's Identity | Not Present |  | Multi-Talker |
| Permission to Request the Floor |  |  |  |
| Permission to Request the Floor | "1" | The receiver is permitted to request floor |  |
| Message Sequence Number |  |  |  |
| Message Sequence Number | The value sent in the previous Floor Taken message, if any, increased with 1 | Any value between '0' and '65535' When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0100010000000000 | Broadcast group call, queueing supported | BROADCAST-CALL |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |
| Floor Indicator |  |  | Multi-Talker |
| Floor Indicator | 1000010010000000 | Normal call, queueing supported, multi-talker |  |
|  | 0100010000000000 | Broadcast group call, queueing supported | BROADCAST-CALL |
|  | 0001010010000000 | Emergency call, queueing supported, multi-talker | EMERGENCY-CALL |
|  | 0001100010000000 | Imminent peril call, queueing supported, multi-talker | IMMPERIL-CALL |
| Audio SSRC of Granted Participant | Audio SSRC of a remote client (Client B) | The SSRC of the granted floor participant. |  |
| Audio SSRC of Granted Participant | Not present |  | Multi-Talker |
| Functional Alias | Not present |  |  |
|  | px_MCPTT_ID_FA_B | Functional Alias = URI | FA AND NOT Multi-Talker |
| List of Granted Users | Not present |  |  |
| List of Granted Users |  |  | Multi-Talker |
| No of users | '10' |  |  |
| User ID | px_MCPTT_ID_User_A |  |  |
| User ID | px_MCPTT_ID_User_B |  |  |
| List of Audio SSRC of Granted Participants | Not present |  |  |
| List of Audio SSRC of Granted Participants |  |  | Multi-Talker |
| Number of SSRCs | '10' |  |  |
| SSRC | Audio SSRC of the client (Client A) |  |  |
| SSRC | Audio SSRC of a remote client (Client B) |  |  |
| List of Functional Aliases | Not present |  |  |
| List of Functional Aliases |  |  | FA AND Multi-Talker |
| No of FAs | '10' |  |  |
| Functional Alias | px_MCPTT_ID_FA_A |  |  |
| Functional Alias | px_MCPTT_ID_FA_B |  |  |
| Location |  |  | NOT Multi-Talker |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |
| Location | Not present |  | Multi-Talker |
| List of Locations | Not present |  | NOT Multi-Talker |
| List of Locations |  | The location information shall be maintained in the same order as the users in the List of Granted Users to allow location information to be matched to the correct user. | Multi-Talker |
| Number of Locations | '10' |  |  |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |

#### 5.5.6.8	Floor Revoke

Table 5.5.6.8-1: Floor Revoke

| Derivation Path: 24.380 [10], Table 8.2.10.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00110 | Floor Revoke |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server |  |
|  | The SSRC of the message sender | The SSRC of the floor arbitrator | OFF-NETWORK |
| name | MCPT |  |  |
| Reject Cause |  |  |  |
| Reject Cause | "4" | Cause#4 - Media Burst pre-empted |  |
| Reject Phrase | "Media Burst pre-empted" | a text string encoded the text string in the SDES item CNAME as specified in IETF RFC 3550 [76], clause 6.5.1. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0100010000000000 | Broadcast group call, queueing supported | BROADCAST-CALL |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.9	Floor Queue Position Request

Table 5.5.6.9-1: Floor Queue Position Request

| Derivation Path: 24.380 [10], Table 8.2.11-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 01000 | Floor Queue Position Request |  |
| SSRC | RTCP SSRC of the client |  |  |
|  | The SSRC of the message sender |  | OFF-NETWORK |
| name | MCPT |  |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT ID of the floor participant requesting the information. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |

#### 5.5.6.10	Floor Queue Position Info

Table 5.5.6.10-1: Floor Queue Position Info

| Derivation Path: 24.380 [10], Table 8.2.12-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 01001 | Floor Queue Position Info with acknowledgment not required |  |
|  | 11001 | Floor Queue Position Info with acknowledgment required | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server |  |
|  | The SSRC of the message sender | The SSRC of the floor arbitrator | OFF-NETWORK |
| name | MCPT |  |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_B | the MCPTT ID of the floor participant sending the Floor Queue Position Info message |  |
| SSRC of queued floor participant | Not present |  |  |
|  | The SSRC of the message recepient | The SSRC field carries the SSRC of the queued floor participant | OFF-NETWORK |
| Queued User ID | Not present |  |  |
| Queued User ID |  |  | OFF-NETWORK |
| Queued User ID | px_MCPTT_ID_User_A | the MCPTT ID of the queued floor participant |  |
| Queue Info |  |  |  |
| Queue Position Info | "1" |  |  |
| Queue Priority Level | "0" |  |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0100010000000000 | Broadcast group call, queueing supported | BROADCAST-CALL |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.11	Floor Ack

Table 5.5.6.11-1: Floor Ack

| Derivation Path: 24.380 [10], Table 8.2.13-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 01010 | Floor Ack |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server for on-network and floor arbitrator for off-network. | DOWNLINK |
|  | RTCP SSRC of the client |  | UPLINK |
| name | MCPT |  |  |
| Source |  |  |  |
| Source | "2" | The controlling MCPTT function is the sender of the message see TS 24.380[10] cl 4.2.1 and cl. 8.2.3.12 | DOWNLINK |
| Source | "0" | The Floor participant is the sender of the message see TS 24.380[10] cl 6.2 and cl. 8.2.3.12 | UPLINK |
| Message Type |  |  |  |
| Message Type | ’0000xxxx’ with ‘xxxx’ being the lower four bits of the subtype of the message to be acknowledged | Message Type of the Floor Control message which requested the acknowledgment |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Location | Not present | Rel-16 | DOWNLINK |
| Location | If present | Rel-16 | UPLINK |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |

| Condition | Explanation |
| --- | --- |
| UPLINK | The message is sent from the UE |
| DOWNLINK | The message is sent from the SS |
| For further conditions see table 5.5.6.1-1 |  |

#### 5.5.6.11A	Floor Release Multi Talker

Table 5.5.6.11A-1: Floor Release Multi Talker

| Derivation Path: 24.380 [10], Table 8.2.14-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 01111 | Floor Release Multi Talker |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor participant sending the message. |  |
|  | The SSRC of the message sender |  | OFF-NETWORK |
| name | MCPT |  |  |
| User ID |  |  |  |
| User ID | px_MCPTT_ID_User_B | The MCPTT User ID of the floor participant releasing the floor. |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010010000000 | Normal call, queueing, multi-talker |  |
|  | 0100010000000000 | Broadcast group call, queueing supported | BROADCAST-CALL |
|  | 0001010010000000 | Emergency call, queueing supported, multi-talker | EMERGENCY-CALL |
|  | 0001100010000000 | Imminent peril call, queueing supported, multi-talker | IMMPERIL-CALL |

#### 5.5.6.12	Connect

Table 5.5.6.12-1: Connect

| Derivation Path: 24.380 [10], Table 8.3.4-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00000 | Connect with acknowledgment required |  |
|  | 10000 | Connect with acknowledgment required | ACK |
| SSRC | RTCP SSRC of the SS |  |  |
| name | MCPC |  |  |
| MCPTT Session Identity field |  |  |  |
| Session Type | “00000000” | No session type |  |
|  | “00000001” | private | PRIVATE-CALL |
|  | "00000011" | prearranged | GROUP-CALL |
|  | “00000100” | chat | CHAT-GROUP-CALL |
| MCPTT Session Identity | tsc_MCX_SessionID_B | SIP URI, which identifies the MCPTT session between the MCPTT client and the controlling MCPTT function |  |
| MCPTT Group Identity field | Not Present |  | PRIVATE-CALL |
| MCPTT Group Identity field |  |  | GROUP-CALL |
| MCPTT Group Identity | px_MCPTT_Group_A_ID | a URI, which identifies the MCPTT group |  |
| Media Streams |  |  |  |
| Media Stream field | "1" | 8 bit parameter giving the number of the" m=audio" m-line negotiated in the pre-established session |  |
| Control Channel | "2" | 8 bit parameter giving the number of the "m=application" m-line negotiated in the pre-established session |  |
|  | "0" | no floor control | WITHOUT_FLOORCONTROL |
| Warning Text field | Not Present |  |  |
| Answer State field |  |  |  |
| Answer State | "1" | confirmed |  |
| Inviting MCPTT User Identity field |  |  |  |
| Inviting MCPTT User Identity | px_MCPTT_ID_User_B | URI, which identifies the inviting MCPTT user |  |
| Invited MCPTT User Identity | Not Present | Rel-18 |  |
| PCK I_MESSAGE field | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| WITHOUT_FLOORCONTROL | There shall be no floor control during the call (e.g. in case of private or first-to-answer call) |
| For further conditions see table 5.5.1-1 |  |

#### 5.5.6.13	Disconnect

Table 5.5.6.13-1: Disconnect

| Derivation Path: 24.380 [10], Table 8.3.5-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00001 | Disconnect with acknowledgment not required |  |
|  | 10001 | Disconnect with acknowledgment required | ACK |
| SSRC | RTCP SSRC of the SS |  |  |
| name | MCPC |  |  |
| MCPTT Session Identity field | Same MCPTT Session Identity as used in the connect message at call establishment | TS 24.380 [10] clause 9.3.2.4.5 |  |
| Reason Cause | Not Present | Rel-17 |  |

#### 5.5.6.14	Acknowledge

Table 5.5.6.14-1: Acknowledge

| Derivation Path: 24.380 [10], Table 8.3.6-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00010 | Acknowledge |  |
| SSRC | RTCP SSRC of the client |  |  |
| name | MCPC |  |  |
| Reason Code |  |  |  |
| Reason Code | "0" | Accepted |  |

#### 5.5.6.15	Map Group To Bearer

Table 5.5.6.15-1: Map Group To Bearer

| Derivation Path: 24.380 [10], Table 8.4.4-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00000 | Map Group To Bearer |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server |  |
| name | MCMC |  |  |
| MCPTT Group ID | px_MCPTT_Group_A_ID | The group ID of the call |  |
| TMGI |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [69] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |
| MBMS Subchannel |  |  |  |
| Audio m-line Number | "1" | The number of the "m=audio" m-line in the SIP MESSAGE request announcing the MBMS bearer |  |
| Floor m-line Number | "2" | The number of the "m=application" m-line in the SIP MESSAGE request announcing the MBMS bearer. The <Floor m-line Number> value is set to "0" when the same subchannel is used for media and for floor control. |  |
| IP version | "0" | '0' = IP version 4 '1' = IP version 6 All other values are reserved for future use |  |
| Floor control Port Number | "9" | The port to be used if the<Floor m-line Number> value is greater than '0'. If the <Floor m-line Number> value is equal to '0', the <Floor control Port Number> value is not included in the MBMS Subchannel field |  |
| Media Port Number | "9" |  |  |
| IP Address | "0.0.0.0" |  |  |

#### 5.5.6.16	Unmap Group To Bearer

Table 5.5.6.16-1: Unmap Group To Bearer

| Derivation Path: 24.380 [10], Table 8.4.5-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00001 | Unmap Group To Bearer |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the floor control server |  |
| name | MCMC |  |  |
| MCPTT Group ID | px_MCPTT_Group_A_ID | The group ID of the call |  |

#### 5.5.6.17	Application Paging

Table 5.5.6.17-1: Application Paging

| Derivation Path: 24.380 [10], Table 8.4.6-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00010 | Application Paging |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the participating MCPTT function. |  |
| name | MCMC |  |  |
| MCPTT Group ID | px_MCPTT_Group_A_ID | The group ID of the call |  |

#### 5.5.6.18	Bearer Announcement

Table 5.5.6.18-1: Bearer Announcement

| Derivation Path: 24.380 [10], Table 8.4.7-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00011 | Bearer Announcement |  |
| name | MCMC |  |  |
| TMGI |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [69] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |
| Alternative TMGI | Not present |  |  |
| Monitoring State | '1' | The <Monitoring State> value is a binary value where the following values are defined: '0' Monitoring is inactive '1' Monitoring is active |  |

<End of modification>

<Start of modification>

#### 5.5.8.4	MCPTT Service Configuration

The structure of a user profile document is specified in TS 24.484 [14] clause 8.4, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 7.2.

Table 5.5.8.4-1: MCPTT Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 8.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| service configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| common |  |  |  |  |
| min-length-alias | "2" | Indicates minimum length of an alphanumeric identifier (i.e., alias) | TS 24.483 [13] clause 7.2.9 |  |
| broadcast-group |  |  |  |  |
| num-levels-group-hierarchy | "1" | Indicates the number of levels of group hierarchy for group-broadcast groups | TS 24.483 [13] clause 7.2.7 |  |
| num-levels-user-hierarchy | "1" | Indicates the number of levels of user hierarchy for user-broadcast groups | TS 24.483 [13] clause 7.2.8 |  |
| on-network |  |  |  |  |
| emergency-call |  |  |  |  |
| private-cancel-timeout | "PT30M" | 30 minutes |  |  |
| group-time-limit | "PT20M" | 20 minutes |  |  |
| private-call |  |  |  |  |
| hang-time | "PT30S" | 30 seconds |  |  |
| max-duration-with-floor-control | "PT30S" | 30 seconds |  |  |
| max-duration-without-floor-control | "PT20M" | 20 minutes |  |  |
| num-levels-priority-hierarchy | 10 |  |  |  |
| transmit-time |  |  |  |  |
| time-limit | "PT30S" | 30 seconds |  |  |
| time-warning | "PT20M" | 20 minutes |  |  |
| hang-time-warning | "PT20M" | 20 minutes |  |  |
| floor-control-queue |  |  |  |  |
| depth | 5 |  |  |  |
| max-user-request-time | "PT20M" | 20 minutes |  |  |
| fc-timers-counters |  |  |  |  |
| T1-end-of-rtp-media | "PT4S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T3-stop-talking-grace | "PT3S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T7-floor-idle | "PT2S" | Depends on the characteristic of the radio access network | TS 24.380 [10] clause 11 |  |
| T8-floor-revoke | "PT1S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T11-end-of-RTP-dual | "PT4S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T12-stop-talking-dual | "PT30S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T15-conversation | "PT30S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T16-map-group-to-bearer | "PT0.5S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T17-unmap-group-to-bearer | "PT0.2S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T20-floor-granted | "PT1S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T25-mbs-conversation | "PT30S" | Rel-18 Default value (30s) | TS 24.380 [10] Table 11.1.4-3 |  |
| T26-map-group-to-session-stream | "PT0.5S" | Rel-18 Default value (500ms) | TS 24.380 [10] Table 11.1.4-3 |  |
| T27-unmap-group-from-session-stream | "PT0.2S" | Rel-18 Default value (200ms) | TS 24.380 [10] Table 11.1.4-3 |  |
| T55-connect | "PT2S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T56-disconnect | "PT2S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| C7-floor-idle | 10 | Default value | TS 24.380 [10] clause 11 |  |
| C17-unmap-group-to-bearer | 3 | Default value | TS 24.380 [10] clause 11 |  |
| C20-floor-granted | 3 | Default value | TS 24.380 [10] clause 11 |  |
| C27-unmap-group-from-session-stream | 3 | Rel-18 Default value | TS 24.380 [10] Table 11.2.4-3 |  |
| C55-connect | 3 | Default value | TS 24.380 [10] clause 11 |  |
| C56-disconnect | 3 | Default value | TS 24.380 [10] clause 11 |  |
| signalling-protection |  |  |  |  |
| confidentiality-protection | true |  |  |  |
| integrity-protection | true |  |  |  |
| protection-between-mcptt-servers |  |  |  |  |
| allow-signalling-protection | true |  |  |  |
| allow-floor-control-protection | true |  |  |  |
| emergency-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" |  | RFC 8101 [45] |  |
| resource-priority-priority | "8" |  | RFC 8101 [45] |  |
| imminent-peril-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" |  | RFC 8101 [45] |  |
| resource-priority-priority | "5" |  | RFC 8101 [45] |  |
| normal-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" |  | RFC 8101 [45] |  |
| resource-priority-priority | "1" |  | RFC 8101 [45] |  |
| anyExt |  |  |  |  |
| functional-alias-list |  |  |  |  |
| functional-alias-entry[1] |  |  |  |  |
| functional-alias | px_MCPTT_ID_FA_A |  |  |  |
| max-simultaneous-activations | "1" |  |  |  |
| allow-takeover | "true" |  |  |  |
| mcptt-user-list |  |  |  |  |
| entry[1] |  |  |  |  |
| uri-entry | px_MCPTT_ID_User_A |  |  |  |
| off-network |  |  |  |  |
| emergency-call |  |  |  |  |
| private-cancel-timeout | "PT5S" | 5 seconds; Indicates timeout value for the cancellation of an in progress emergency for an MCPTT private call. Values: : 0-65535 s | TS 24.483 [13] clause 7.2.14 |  |
| group-time-limit | "PT5S" | 5 seconds; Indicates time limit for an in progress MCPTT emergency call related to an MCPTT group. Values: 0-65535 s | TS 24.483 [13] clause 7.2.16 |  |
| private-call |  |  |  |  |
| hang-time | "PT5S" | 5 seconds; Indicates hang timer for private calls (with floor control). Values: 0-65535 s | TS 24.483 [13] clause 7.2.13 |  |
| max-duration-with-floor-control | "PT60S" | 60 seconds; Indicates max private call (with floor control) duration. Values: 0-65535 s | TS 24.483 [13] clause 7.2.12 |  |
| num-levels-priority-hierarchy | "4" | Indicates the number of levels of hierarchy for floor control override in off-network. Values: 4-256 | TS 24.483 [13] clause 7.2.17 |  |
| transmit-time |  |  |  |  |
| time-limit | "PT60S" | 60 seconds; Indicates transmit time limit from a single request to transmit in a group or private call. Values: 0-65535 s | TS 24.483 [13] clause 7.2.18 |  |
| time-warning | "PT50S" | 50 seconds; Indicates configuration of warning time before time limit of transmission is reached (off-network). Values: 0-255 s | TS 24.483 [13] clause 7.2.19 |  |
| hang-time-warning | "PT4S" | 4 seconds; Indicates configuration of warning time before hang time is reached (off-network). Values: Values: 0-255 s | TS 24.483 [13] clause 7.2.20 |  |
| default-prose-per-packet-priority |  |  |  |  |
| mcptt-private-call-signalling | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 23.303 [68] TS 24.483 [13] clause 7.2.22 |  |
| mcptt-private-call-media | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 23.303 [68] TS 24.483 [13] clause 7.2.23 |  |
| mcptt-emergency-private-call-signalling | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 23.303 [68] TS 24.483 [13] clause 7.2.24 |  |
| mcptt-emergency-private-call-media | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 23.303 [68] TS 24.483 [13] clause 7.2.25 |  |
| allow-log-metadata | "true" | Indicates whether an MCPTT emergency group call is permitted on the MCPTT group | TS 24.483 [13] clause 7.2.26 |  |
| default-pqi | not present | Rel-18 |  |  |

<End of modification>

<Start of modification>

#### 5.5.8.8	MCVideo Service Configuration

The structure of a service configuration document is specified in TS 24.484 [14] clause 9.4. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 14.2.

Table 5.5.8.8-1: MCVideo Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 9.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| service configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| Common |  |  |  |  |
| min-length-alias | "2" | Indicates minimum length of an alphanumeric identifier (i.e., alias) |  |  |
| broadcast-group |  |  |  |  |
| num-levels-group-hierarchy | "1" | Indicates the number of levels of group hierarchy for group-broadcast groups |  |  |
| num-levels-user-hierarchy | "1" | Indicates the number of levels of user hierarchy for user-broadcast groups |  |  |
| on-network |  |  |  |  |
| signalling-protection |  |  |  |  |
| confidentiality-protection | "true" |  |  |  |
| integrity-protection | "true" |  |  |  |
| protection-between-mcvideo-servers |  |  |  |  |
| allow-signalling-protection | "true" |  |  |  |
| allow-transmission-control-protection | "true" |  |  |  |
| emergency-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" | MCVideo uses the MCPTT namespace values of RFC 8101 [45] | RFC 8101 [45] |  |
| resource-priority-priority | "7" |  | RFC 8101 [45] |  |
| imminent-peril-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" | MCVideo uses the MCPTT namespace values of RFC 8101 [45] | RFC 8101 [45] |  |
| resource-priority-priority | "4" |  | RFC 8101 [45] |  |
| normal-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" | MCVideo uses the MCPTT namespace values of RFC 8101 [45] | RFC 8101 [45] |  |
| resource-priority-priority | "0" |  | RFC 8101 [45] |  |
| off-network |  |  |  |  |
| default-prose-per-packet-priority |  |  |  |  |
| mcvideo-private-call-signalling | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-private-call-media | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-emergency-private-call-signalling | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-emergency-private-call-media | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| private-call |  |  |  |  |
| mcvideo-max-duration | "600" | Value in seconds | TS 24.483 [13] clause 14.2.17 |  |
| num-levels-priority-hierarchy | "4" |  | TS 24.483 [13] clause 14.2.18 |  |
| default-pqi | not present | Rel-18 |  |  |

<End of modification>

<Start of modification>

#### 5.5.8.12	MCData Service Configuration

The structure of a service configuration document is specified in TS 24.484 [14] clause 10.4. Single MCData group configuration parameters are defined in TS 24.483 [13] clause 11.2.

Table 5.5.8.12-1: MCData Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 10.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| service configuration |  |  |  |  |
| domain attribute | px_MCData_User_A_Organization | Mandatory attribute: domain name of the mission critical organization |  |  |
| on-network |  |  |  |  |
| tx-and-rx-control |  |  |  |  |
| max-data-size-sds-bytes | "10000000" | The maximum data that the originating client can send in an SDS message |  |  |
| max-payload-size-sds-cplane-bytes | "1000" | The maximum payload data that the originating client can send in an SDS message over C-plane |  |  |
| max-data-size-fd-bytes | "100000000" | The maximum data that the originating client can send in an FD message |  |  |
| max-data-size-auto-recv-bytes | "10000000" | The maximum data that the server can send to the terminating client without requesting the user to indicate a present need for the data |  |  |
| signalling-protection |  |  |  |  |
| confidentiality-protection | "true" | Indicating whether confidentiality protection of MCData signalling is enabled or disabled between the MCData client and MCData server |  |  |
| integrity-protection | "true" | Indicating whether integrity protection of MCData signalling is enabled or disabled between the MCData client and MCData server |  |  |
| protection-between-mcdata-servers |  |  |  |  |
| allow-signalling-protection | "true" | Indicating whether protection of MCData signalling is enabled between MCData servers |  |  |
| file-availability |  |  |  |  |
| default-file-availability | "10000000" | The default time for which a file is available on the server for download, if a explicit time period is not requested by the originating client |  |  |
| max-file-availability | "10000000" | The maximum time for which a file can be made available on the server for download |  |  |
| anyExt |  |  |  |  |
| functional-alias-list |  |  |  |  |
| functional-alias-entry[1] |  |  |  |  |
| functional-alias | px_MCData_ID_FA_A |  |  |  |
| max-simultaneous-activations | "1" |  |  |  |
| allow-takeover | "true" |  |  |  |
| mcdata-user-list |  |  |  |  |
| entry[1] |  |  |  |  |
| uri-entry | px_MCData_ID_User_A |  |  |  |
| functional-alias-priority | "1" |  |  |  |
| off-network |  |  |  |  |
| default-prose-per-packet-priority |  |  |  |  |
| mcdata-one-to-one-call-signalling | "1" |  | TS 24.483 [13] clause 11.2.11 |  |
| mcdata-one-to-one-call-media | "1" |  | TS 24.483 [13] clause 11.2.12 |  |
| default-pqi | not present | Rel-18 |  |  |

<End of modification>

<Start of modification>

### 5.5.11	Default MCVideo Transmission Control Messages and other Information Elements

#### 5.5.11.0	General

The following conditions apply throughout clause 5.5.11:

Table 5.5.11.0-1: Conditions

| Condition | Explanation |
| --- | --- |
| FA | IE for when an active Functional Alias is used |
| ACK | Message requests a Transmission control Ack |
| UPLINK | The message is sent from the UE |
| DOWNLINK | The message is sent from the SS |
| NOTE: For further conditions see table 5.5.1-1 |  |

For MCVideo media plane control different SSRCs (Synchronization SouRCes) need to be distinguished. Table 5.5.11.0-2 specifies the SSRCs as used in the default MCVideo media plane control messages for the case that there is no multiplexing of media plane control channels.

NOTE 1:	Multiplexing of media plane control channels has been introduced in Rel-18 of TS 24.281 [86] and TS 24.581 [88] and may be specified in Rel-18 and above test cases.

NOTE 2:	In contrast to Rel-18 where there are distinct SSRCs for the audio and video stream, in Before-Rel-18 releases there is still only one SSRC identifying a media stream. In Transmission Control Messages this Media SSRC effectively is the same as the Audio SSRC of Rel-18 as the Field ID is the same, but in Before-Rel-18 releases there is no field for a Video SSRC.

NOTE 3:	In test cases Before-Rel-18 behaviour is applied unless specified otherwise in the test case.

Table 5.5.11.0-2: SSRCs in MCVideo media plane control messages
(No multiplexing of media plane control channels)

| SSRC (NOTE 1) | Description | Value |
| --- | --- | --- |
| Media/Audio SSRC of the client (NOTE 1) | Before-Rel18: SSRC identifying the media stream of the client (Client A) | Arbitrarily selected by the SS and assigned to the client when the transmission is granted by sending a Transmission Granted message (NOTE 2) |
|  | Rel-18 and later: SSRC identifying the audio stream of the client (Client A) |  |
| Video SSRC of the client (NOTE 1) | Before-Rel18: Not applicable - there is no Video SSRC in transmission control messages and no SDP fmtp attribute indicating such SSRC and the SSRC used in RTP packets of the video stream is not specified. | Before-Rel18: Not present in DL, not checked in UL |
|  | Rel-18 and later: SSRC identifying the audio stream of the client (Client A) | Rel-18 and later: Arbitrarily selected by the SS and assigned to the client when the transmission is granted (NOTE 2) |
| Media/Audio SSRC of a remote client (NOTE 1) | Before-Rel18: SSRC identifying the media stream of a remote client (Client B, C) | Arbitrarily selected by the SS (NOTE 2) |
|  | Rel-18 and later: SSRC identifying the audio stream of a remote client (Client B, C) |  |
| Video SSRC of a remote client (NOTE 1) | Before-Rel18: Not applicable - there is no Video SSRC in transmission control messages and no SDP fmtp attribute indicating such SSRC and the SSRC used in RTP packets of the video stream is not specified. | Before-Rel18: Not present in DL, not checked in UL |
|  | Rel-18 and later: SSRC identifying the video stream of a remote client (Client B, C) | Rel-18 and later: Arbitrarily selected by the SS (NOTE 2) |
| RTCP SSRC of the client (NOTE 1) | SSRC used by the client (Client A) in the RTCP header of the MCVideo media plane control messages sent to the SS | The client may use any value, value is not checked by the SS (NOTE 4). |
| RTCP SSRC of the SS (NOTE 1) | SSRC used by the SS in the RTCP header of the MCPTT media plane control messages sent to the client | Arbitrarily selected by the SS (NOTE 4) |
| NOTE 1: The terms "RTCP SSRC", "Audio SSRC" and "Video SSCR" have been introduced in Rel-18 of TS 24.581 [88]. "Media SSRC" is used as in Rel-14 .. Rel-17 there is no "Audio SSRC" and "Video SSRC" yet in media plane control messages, but only a single SSRC value identifying the transmitter of a media stream. NOTE 2: Different SSRC values shall be selected by the SS for media streams from different clients. Nevertheless it is not clear for Before-Rel-18 releases how the client uses the SSRC value provided by the server in the Transmission Granted message and there is no way to provide SSRC value(s) in case of implicit transmission grant In general collisions according to IETF RFC 3550 [76] may occur but collision resolution is out of scope of this document. NOTE 3: For Before-Rel-18 releases the Media SSRC can only be assigned by sending a Transmission Granted message as there are no "mc_ssrc", "mc_audio_ssrc" or "mc_video_ssrc" fmtp attributes in Before-Rel-18 releases (nevertheless there can still be implicit grants for Before-Rel-18 releases even though in this case the server has no control over the SSRC values used by the client). From Rel-18 onward the Audio and Video SSRCs can be assigned with an implicit grant using fmtp attributes "mc_audio_ssrc" and "mc_video_ssrc" or by sending a Transmission Granted message with Audio and Video SSRC ( The server has control over the Audio and Video SSRCs used by the client). NOTE 4: In clause 4.3.3.1 TS 24.581 [88] clarifies in Rel-18 that "the SSRC of the RTCP header is used to enable multiplexing of media plane control channels"; in clauses 14.2.7 and 14.3.8 it is clarified that the "mc_transmission_ssrc" fmtp attribute is used to indicate support of multiplexing and to exchange of the SSRC values to be used in the RTCP header. It is assumed that RTCP SSRC values have no meaning in case of no multiplexing. |  |  |

#### 5.5.11.1	Transmission Control Specific Messages Sent by the Transmission Participant

##### 5.5.11.1.1	Transmission Request

Table 5.5.11.1.1-1: Transmission Request

| Derivation Path: TS 24.581 [88] Table 9.2.4-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP-header |  |  |  |  |
| Subtype | “00000” | Transmission Request | TS 24.581 [88] clause 9.2.4 and Table 9.2.2.1-1 |  |
|  | “10000” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| Transmission Priority | If present |  | TS 24.581 [88] clause 9.2.3.2 |  |
| Transmission Priority Value | Any allowed value | If present, a value between '0' and '255' where '0' is the lowest priority and '255' is the highest priority. If the Transmission Priority field is not included in the message the default priority is used as the Transmission Priority value. The value of the default priority is '0'. The default priority is sometimes referred to as normal priority. |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the MCVideo ID of the transmission participant sending the Transmission Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88] clause 9.2.3.21 | FA |

##### 5.5.11.1.2	Transmission Release

Table 5.5.11.1.2-1: Transmission Release

| Derivation Path: TS 24.581 [88] Table 9.2.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP-header |  |  |  |  |
| Subtype | “00010” | Transmission Release | TS 24.581 [88] clause 9.2.7 and Table 9.2.2.1-1 |  |
|  | “10010” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the MCVideo ID of the transmission participant sending the Transmission Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Transmission Indicator |  |  |  |  |
| Transmission Indicator | "1000000000000000" | Normal call | TS 24.581 [88] clause 9.2.3.11 |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.1.3	Queue Position Request

Table 5.5.11.1.3-1: Queue Position Request

| Derivation Path: TS 24.581 [88] Table 9.2.11-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00011” | Queue Position Request | TS 24.581 [88] clause 9.2.11 and 9.2.2.1-1 |  |
|  | “10011” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76], |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  |  | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |

##### 5.5.11.1.4	Receive Media Request

Table 5.5.11.1.4-1: Receive Media Request

| Derivation Path: TS 24.581 [88] Table 9.2.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Receive Media Request | TS 24.581 [88] clause 9.2.14 and 9.2.2.1-1 |  |
|  | “10100” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field is used to carry the identity of the user who is requesting the reception of the media. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| User Id of the Transmitting User | any value if present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 |  |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of the Transmitting User (client B) as provided by the SS in the Media Transmission Notification |  | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | any value if present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Reception Priority | if present | Describes the level of reception priority requested in a Reception Request message or granted in a Reception Granted message. The max reception priority that can be requested in a Reception Request message is negotiated between the transmission control participant and the transmission control server | TS 24.581 [88] clause 9.2.3.19 and 6.2.5.3.3 |  |
| Reception Priority value | any allowed value | The reception priority (0 to 255) where 0 is the lowest reception priority and 255 is the highest reception priority. If the Reception Priority field is not included in the message the default reception priority is used as the Reception Priority value. The value of the default reception priority is 0. The default reception priority is sometimes referred to as normal reception priority. |  |  |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88]clause 9.2.3.21 | FA |

##### 5.5.11.1.5	Void

##### 5.5.11.1.6	Remote Transmission Request

Table 5.5.11.1.6-1: Remote Transmission Request

| Derivation Path: TS 24.581 [88] Table 9.2.22-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00111” | Remote Transmission Request | TS 24.581 [88] clause 9.2.22 and Table 9.2.2.1-1 |  |
|  | “10111” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| Remote ID |  | Carries the identity of the user who remotely initiated the media transmission of another user. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_B |  |  |  |
| User ID |  | Carries the identity of the user whose media transmission is requested. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A |  |  |  |

##### 5.5.11.1.7	Remote Transmission Cancel Request

Table 5.5.11.1.7-1: Remote Transmission Cancel Request

| Derivation Path: TS 24.581 [88] Table 9.2.24-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01000” | Remote transmission cancel request | TS 24.581 [88] clause 9.2.24 and Table 9.2.2.1-1 |  |
|  | “11000” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| User ID |  | Carries the identity of the user whose media transmission is requested for cancellation. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A |  |  |  |

#### 5.5.11.2	Transmission Control Specific Messages Sent by the Transmission Control Server

##### 5.5.11.2.1	Transmission Granted

Table 5.5.11.2.1-1: Transmission Granted

| Derivation Path: TS 24.581 [88] Table 9.2.5-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00000” | Transmission granted | TS 24.581 [88] clause 9.2.5 and 9.2.2.1-2 |  |
|  | “10000” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender | The SSRC of the transmission arbitrator |  | OFF-NETWORK |
| name | MCV1 | Transmission Control messages sent by the transmission control server and transmission control participant |  |  |
| Duration |  |  | TS 24.581 [88] clause 9.2.3.3 |  |
| Duration | "00000000 10000000" | 128 sec (an arbitrary value) |  |  |
| Audio SSRC of the Transmitting User | Media/Audio SSRC which should be used by the client in the header of RTP packets | Before-Rel-18: SSRC indentifying the media stream Rel-18 and later: SSRC to be used by the client in the audio stream | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Transmission priority | Not present | If the Transmission Priority field is not included in the message the default priority (='0') is used as the Floor Priority value | TS 24.581 [88] clause 9.2.3.2 |  |
| User ID | Not present |  |  |  |
| User ID |  |  | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Queue Size | Not present |  |  |  |
| Queue Size | "0" | the number of queued MCVideo clients in the MCVideo call | TS 24.581 [88] clause 9.2.3.15 | OFF-NETWORK |
| SSRC of queued floor participant | Not present |  |  |  |
|  | The SSRC of queued transmission participant |  | IETF RFC 3550 [76] | OFF-NETWORK |
| Queued User ID | Not present |  |  |  |
|  | px_MCVideo_ID_User_C | MCVideo ID of the transmission participant in the queue | TS 24.581 [88] clause 9.2.3.14 | OFF-NETWORK |
| Queue Info | Not present |  |  |  |
| Queue Info |  | queue position and granted transmission priority in the queue | TS 24.581 [88] clause 9.2.3.5 | OFF-NETWORK |
| queue position info | "00000001" |  |  |  |
| queue priority level | "00000000" |  |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] Table 9.2.3.11-2 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.2	Transmission Rejected

Table 5.5.11.2.2-1: Transmission Rejected

| Derivation Path: TS 24.581 [88] Table 9.2.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00001” | Transmission rejected | TS 24.581 [88] clause 9.2.6 and 9.2.2.1-2 |  |
|  | “10001” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| Reject Cause |  | Includes the reason for the rejecting the transmission request and can be followed by a text-string explaining why the transmission request was rejected. Therefore the length of the packet will vary depending on the size of the application dependent field. | TS 24.581 [88] clause 9.2.3.4 |  |
| Reject Cause | "255" | Th<Reject cause> value set to '255' indicates that the transmission control server does not grant the transmission request due to the transmission control server local policy. | TS 24.581 [88] clause 9.2.6.2 |  |
| Reject Cause Phrase | “Other reason” | A text string encoded the text string in the SDES item CNAME. | IETF RFC 3550 [76] |  |
| User ID | Not present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the requesting transmission participant to which the Transmission Rejected message is sent. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.3	Transmission Arbitration Taken

Table 5.5.11.2.3-1: Transmission Arbitration Taken

| Derivation Path: TS 24.581 [88] Table 9.2.8-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00010” | Transmission Arbitration Taken | TS 24.581 [88] clause 9.2.8 and 9.2.2.1-2 |  |
|  | “10010” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User Id of the Transmitting User |  | MCVideo Id of the user who has been granted the right to transmit media. | TS 24.581 [88] clause 9.2.3.6 |  |
| User Id of the Transmitting User | px_MCVideo_ID_User_A |  |  |  |
| Permission to Request the Transmission |  | Indicates whether receiving parties are allowed to request the transmission. | TS 24.581 [88] clause9.2.3.7 |  |
| Permission to Request the Transmission | "1" | Coded as follows: 0 The receiver is not permitted to request transmission. 1 The receiver is permitted to request transmission |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the transmission participant sending the Transmission Arbitration Taken message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Message Sequence Number |  |  | TS 24.581 [88] clause 9.2.3.9 |  |
| Message Sequence Number | The value sent in the previous Transmission Arbitration Taken message, if any, increased by 1 | The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again. |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call. |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of the Transmitting User (client B) | Notation in accordance with clause 5.5.11.0. | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |

##### 5.5.11.2.4	Transmission Arbitration Released

Table 5.5.11.2.4-1: Transmission Arbitration Released

| Derivation Path: TS 24.581 [88] Table 9.2.9-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00011” | Transmission Arbitration Release | TS 24.581 [88] clause 9.2.9 and 9.2.2.1-2 |  |
|  | “10011” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| Granted Party's Identity |  | Identifies the MCVideo user that is granted to send media. | TS 24.581 [88] clause 9.2.3.6 |  |
| Granted Party's Identity | px_MCVideo_ID_User_A |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the transmission participant sending the Transmission Arbitration Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Message Sequence Number |  |  | TS 24.581 [88] clause 9.2.3.9 |  |
| Message Sequence Number | The value sent in the previous Transmission Arbitration Release message, if any, increased by 1 | The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again. |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| SSRC of Granted Transmission Participant | Media SSRC of the intended recipient of the message | Notation in accordance with clause 5.5.11.0. | IETF RFC 3550 [76] |  |

##### 5.5.11.2.5	Transmission Revoked

Table 5.5.11.2.5-1: Transmission Revoked

| Derivation Path: TS 24.581 [88] Table 9.2.10-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Transmission Revoked | TS 24.581 [88] clause 9.2.10 and 9.2.2.1-2 |  |
|  | “10100” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| Reject Cause |  | Message includes <Reject Cause> cause value in the Reject Cause field explaining why the transmission control server wants the transmission participant to stop sending media and can be followed by additional information. Therefore the length of the packet can vary depending on the value of the rejection cause. | TS 24.581 [88] clause 9.2.3.4 |  |
| Reject Cause Value | 7 | The <Reject Cause> value set to 7 indicates that the MCVideo client's permission to send a media is being queued. No additional information is included. | TS 24.581 [88] clause 9.2.10.2 |  |
| Reject Cause Phrase | "Queue the transmission" | A text string encoded the text string in the SDES item CNAME. | TS 24.581 [88] clause 9.2.10.2 |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.6	Queue Position Info

Table 5.5.11.2.6-1: Queue Position Info

| Derivation Path: TS 24.581 [88] Table 9.2.12-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00101” | Queue Position Info | TS 24.581 [88] clause 9.2.12 and 9.2.2.1-2 |  |
|  | “10101” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User ID | Not present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the MCVideo user ID of the transmission participant sending the Queue Position Info message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| SSRC of Queued Transmission Participant | Not present |  |  |  |
| SSRC of Queued Transmission Participant | The SSRC of the queued transmission participant | Applicable only in off-network and shall carry the SSRC of the queued transmission participant. | IETF RFC 3550 [76]. | OFF-NETWORK |
| Queued User ID | Not present |  |  |  |
| Queued User ID | px_MCVIDEO_ID_User_B | Used in off-network only. The Queued User ID field carries the MCVideo ID of the queued transmission control participant. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| Queue Info |  | Defines the queue position and granted transmission control priority in the queue. | TS 24.581 [88] clause 9.2.3.5 |  |
| Queue Position Info | "1" | value is a binary value |  |  |
| Queue Priority Level | "0" | value consists of 8 bit parameter giving the transmission priority. The value of the default priority is ‘0’. The default priority is sometimes referred to as normal priority. | TS 24.581 [88] clause 9.2.3.2 |  |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |
| Transmission Control Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.7	Media Transmission Notification

Table 5.5.11.2.7-1: Media Transmission Notification

| Derivation Path: TS 24.581 [88] Table 9.2.13-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00110” | Media Transmission Notification | TS 24.581 [88] clause 9.2.13 and 9.2.2.1-2 |  |
|  | “10110” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User Id of the Transmitting User |  | User ID of the user transmitting the media | TS 24.581 [88] clause 9.2.3.6 |  |
| User Id of the Transmitting User | px_MCVideo_ID_User_B |  |  |  |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of remote client (Client B) | SSRC of the user transmitting the media | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Permission to Request the Transmission |  | Indicates whether receiving parties are allowed to request the transmission. | TS 24.581 [88] clause 9.2.3.7 |  |
| Permission to Request the Transmission value | 1 | The receiver is permitted to request transmission |  |  |
|  | 0 | The receiver is not permitted to request transmission |  | BROADCAST-CALL |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal Call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88]clause 9.2.3.21 | FA |
| Reception Mode |  |  | TS 24.581 [88]clause 9.2.3.22 |  |
| Reception Mode value | 1 | The receiver is not granted permission to automatically receive media |  |  |
|  | 0 | The receiver is granted permission to automatically receive media |  | EMERGENCY-CALL, IMMPERIL-CALL, BROADCAST-CALL |

##### 5.5.11.2.8	Receive Media Response

Table 5.5.11.2.8-1: Receive Media Response

| Derivation Path: TS 24.581 [88] Table 9.2.15-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00111” | Receive Media Response | TS 24.581 [88] clause 9.2.15 and 9.2.2.1-2 |  |
|  | “10111” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76], |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| Result |  | Indicates whether media reception is possible as per the request | TS 24.581 [88] clause 9.2.3.17 |  |
| Result | "1" | 0 - The receiver is not permitted (rejected) to receive the media transmission. 1 - The receiver is permitted (granted) to receive the media transmission. |  |  |
| Reject Cause | Not present | Includes the reason for the rejecting the media receive request and can be followed by a text-string explaining why the media receive request was rejected. Therefore the length of the packet will vary depending on the size of the application dependent field |  |  |
| User Id of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 |  |
| Audio SSRC of the Transmitting User | Same value as in the corresponding Receive Media Request | SSRC of the user transmitting the media Notation in accordance with clause 5.5.11.0. | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.9	Media Reception Notification

Table 5.5.11.2.9-1: Media Reception Notification

| Derivation Path: TS 24.581 [88] Table 9.2.16-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01000” | Media Reception Notification | TS 24.581 [88] clause 9.2.16 and 9.2.2.1-2 |  |
|  | “11000” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User ID |  | The User ID field is used to carry the identity of the user who is receiving the media | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_B |  |  |  |
| Audio SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88] clause 9.2.3.21 | FA |

##### 5.5.11.2.10	Void

##### 5.5.11.2.11	Transmission Cancel Request Notify

Table 5.5.11.2.11-1: Transmission Cancel Request Notify

| Derivation Path: TS 24.581 [88] Table 9.2.19-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01010” | Transmission Cancel Request Notify | TS 24.581 [88] clause 9.2.19 and 9.2.2.1-2 |  |
|  | “11010” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender | The SSRC of the transmission arbitrator |  | OFF-NETWORK |
| name | MCV1 | Transmission Control messages sent by the transmission control server and transmission control participant |  |  |

##### 5.5.11.2.12	Remote Transmission Response

Table 5.5.11.2.12-1: Remote Transmission Response

| Derivation Path: TS 24.581 [88] Table 9.2.23-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01011” | Remote Transmission Response | TS 24.581 [88] clause 9.2.23 and 9.2.2.1-2 |  |
|  | “11011” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |

##### 5.5.11.2.13	Remote Transmission Cancel Response

Table 5.5.11.2.13-1: Remote Transmission Cancel Response

| Derivation Path: TS 24.581 [88] Table 9.2.25-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01100” | Remote Transmission Cancel Response | TS 24.581 [88] clause 9.2.25 and 9.2.2.1-2 |  |
|  | “11100” |  |  | ACK |
| SSRC | RTCP SSRC of the SS |  | IETF RFC 3550 [76]. |  |
| name | MCV1 |  |  |  |

##### 5.5.11.2.14	Media Reception Override Notification

Table 5.5.11.2.14-1: Media Reception Override Notification

| Derivation Path: TS 24.581 [88] Table 9.2.28-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01101” | Media Reception Override Notification | TS 24.581 [88] clause 9.2.28 and 9.2.2.1-2 |  |
|  | “11101” |  |  | ACK |
| SSRC | RTCP SSRC of the SS |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User ID |  | Carries the identity of the user who is requesting the reception of the media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| SSRC of transmitter | Same value as sent to the client in the Receive Media Response (Media SSRC of client B) | The SSRC of transmitter field carries the SSRC of the user transmitting the media Notation in accordance with clause 5.5.11.0. | IETF RFC 3550 [76]. |  |
| Overriding ID |  | Carries the identity of the user of the overriding media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_C |  |  |  |
| Overridden ID |  | Carries the identity of the user of the overridden media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_B |  |  |  |

##### 5.5.11.2.15	Transmission End Notify

Table 5.5.11.2.15-1: Transmission End Notify

| Derivation Path: TS 24.581 [88] Table 9.2.29-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01110” | Transmission End Notify | TS 24.581 [88] clause 9.2.29 and 9.2.2.1-2 |  |
|  | “11110” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the transmission control server. | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User Id of the Transmitting User |  | Carries the identity of the user whose media transmission has been released | TS 24.581 [88] clause 9.2.3.6 |  |
| User Id of the Transmitting User | px_MCVideo_ID_User_B |  |  |  |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of remote client (Client B) | SSRC of the user transmitting the media | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |

##### 5.5.11.2.16	Transmission Idle

Table 5.5.11.2.16-1: Transmission Idle

| Derivation Path: TS 24.581 [88] Table 9.2.30-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01111” |  | TS 24.581 [88] clause 9.2.2.1-2 |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender | The SSRC of the transmission arbitrator. |  | OFF-NETWORK |
| name | “MCV1” | Transmission Control messages sent by the Transmission Control Server and the Transmission Control Participant. |  |  |
| Message Sequence Number |  |  | TS 24.581 [88] clause 9.2.3.9 |  |
| Message Sequence Number | The value sent in the previous Transmission Idle message, if any, increased with 1 | value is a binary value. The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

#### 5.5.11.3	Transmission control specific messages sent by both the transmission control server and transmission control participant

##### 5.5.11.3.1	Transmission End Request

Table 5.5.11.3.1-1: Transmission End Request

| Derivation Path: TS 24.581 [88] Table 9.2.20-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00000” | Transmission End Request | TS 24.581 [88] clause 9.2.20 and 9.2.2.1-3 |  |
|  | “10000” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. | IETF RFC 3550 [76]. | DOWNLINK |
|  | RTCP SSRC of the client | The SSRC of transmission control participant |  | UPLINK |
| name | MCV2 |  |  |  |
| User Id of the Transmitting User |  | identity of the user whose media transmission is requested to be terminated. | TS 24.581 [88] clause 9.2.3.6 |  |
| User Id of the Transmitting User | px_MCVideo_ID_User_A |  |  |  |
| Audio SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.16 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Reject Cause |  | Includes the reason explaining why the transmission control server wants the transmission participant to stop sending media | TS 24.581 [88] clause 9.2.3.4 | DOWNLINK |
| Reject Cause Value | 8 |  |  |  |
| Reject Cause Phrase | "No receiving participant" |  |  |  |
| Reject Cause | not present |  |  | UPLINK |

##### 5.5.11.3.2	Transmission End Response

Table 5.5.11.3.2-1: Transmission End Response

| Derivation Path: TS 24.581 [88] Table 9.2.21-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00001” | Transmission End Response | TS 24.581 [88] clause 9.2.21 and 9.2.2.1-3 |  |
|  | “10001” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. | IETF RFC 3550 [76]. | DOWNLINK |
|  | RTCP SSRC of the client |  |  | UPLINK |
| name | MCV2 |  |  |  |
| User Id of the Transmitting User |  | identity of the user whose media transmission is requested to be terminated. | TS 24.581 [88] clause 9.2.3.6 |  |
| User Id of the Transmitting User | px_MCVideo_ID_User_A |  |  |  |
| Audio SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.16 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |

##### 5.5.11.3.3	Media Reception End Request

Table 5.5.11.3.3-1: Media Reception End Request

| Derivation Path: TS 24.581 [88] Table 9.2.26-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00010” | Media Reception End Request | TS 24.581 [88] clause 9.2.26 and 9.2.2.1-3 |  |
|  | “10010” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the transmission control server | IETF RFC 3550 [76] | DOWNLINK |
|  | RTCP SSRC of the client | The SSRC of the transmission control participant |  | UPLINK |
| name | MCV2 |  |  |  |
| User Id of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of remote client as provided in Media transmission notification message sent to the UE | SSRC of the user transmitting the media Notation in accordance with clause 5.5.11.0. | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.3.4	Media Reception End Response

Table 5.5.11.3.4-1: Media Reception End Response

| Derivation Path: TS 24.581 [88] Table 9.2.27-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00011” | Media Reception End Response | TS 24.581 [88] clause 9.2.27 and 9.2.2.1-3 |  |
|  | “10011” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the transmission control server | IETF RFC 3550 [76] | DOWNLINK |
|  | RTCP SSRC of the client | The SSRC of the transmission control participant |  | UPLINK |
| name | MCV2 |  |  |  |
| User Id of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of remote client (same value as in the corresponding Media Reception End Request) | SSRC of the user transmitting the media | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |

##### 5.5.11.3.5	Transmission Control Ack

Table 5.5.11.3.5-1: Transmission Control Ack

| Derivation Path: TS 24.581 [88] Table 9.2.31-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Transmission Control Ack | TS 24.581 [88] clause 9.2.31 and 9.2.2.1-3 |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. | IETF RFC 3550 [76] | DOWNLINK |
|  | RTCP SSRC of the client | The SSRC of the transmission control participant |  | UPLINK |
| name | MCV2 |  |  |  |
| Source |  |  | TS 24.581 [88] clause 9.2.3.12 |  |
| Source | "2" | the controlling MCVideo function is the sender of the message |  | DOWNLINK |
|  | “0” | the transmission participant is the sender of the message |  | UPLINK |
| Message name |  |  | TS 24.581 [88] clause 9.2.3.18 |  |
| Message Name | Message Name of the transmission control messages which requested the acknowledgement | value is as coded as an ascii name field of the RTCP APP packet containing the message to be acknowledged |  |  |
| Message type |  |  | TS 24.581 [88] clause 9.2.3.10 |  |
| Message Type | ’0000xxxx’ with ‘xxxx’ being the lower four bits of the subtype of the message to be acknowledged | Message Type of the transmission control messages which requested the acknowledgement |  |  |

<End of modification>