3GPP TSG-RAN5 Meeting #104	R5-244448

Maastricht, Netherlands, 19th – 23rd Aug 2024

Agenda item:	5.4.20

Source:	Keysight Technologies

Title:	PC6 MUs based on antenna array assumptions

Document for:	Discussion and Endorsement

# Introduction

This contribution intends to provide an overview of the MU elements based on the PC6 antenna array assumptions.

# Discussion

In [1], it is recommended to consider a 6x6 antenna array for PC6 UEs. This array assumption matches the alternate array assumption for PC5 UEs based on a vendor declaration [2], [3]. The measurement grids (default and SE) for the 6x6 array assumptions were already provided in [4][5] and incorporated in [2]. It is proposed to leverage the beam peak search and TRP MUs and measurement grids of the PC5 alternate array assumption for PC6.

Proposal 1: Leverage the beam peak search and default and SE TRP measurement grids and MU from PC5 (alternate array assumption) for PC6, see [2][4]

This contribution intends to provide MUs for the elements that are dependent on the array assumptions, similar to what was previously presented in [6]. The spherical coverage side conditions for PC6 and PC5 are different and require separate investigations not presented here.

Proposal 2: Defer the spherical coverage test case MUs and measurements grids given the deviation in spherical coverage side conditions for PC6.

The antenna array assumptions are the same as those presented in [4] and will not be repeated here other than presenting the 6x6 antenna array pattern plot in Figure 1.

Figure 1: 6x6 Antenna Pattern.

# MU Elements: “Positioning misalignment” and “DUT repositioning”

The MU elements, other than those related to the measurement grid, depending on the antenna pattern have been identified in [7]:

Positioning misalignment

DUT repositioning (EIRP/EIS)

The approach to determine the positioning misalignment MU is based on the approach outlined in [8] with a maximum deviation from the beam peak direction due to positioning misalignment to be 0.25o. The simulation results shown in Figure 2 show that the error based on the positioning misalignment is negligible with maximum error between the theoretical beam peak and the pattern at the misaligned beam peak direction of less than 0.005dB.

Figure 2: Histogram of maximum beam peak errors for a maximum misalignment error of 0.25o.

It is therefore proposed to define the standard uncertainty of the “Positioning misalignment” MU element for PC1 to be 0dB.

Proposal 3: Define standard uncertainty of the “Positioning misalignment” MU element for PC6 to be 0dB

The approach to determine the DUT repositioning for EIRP/EIS metrics is based on the approach outlined in [9] with a maximum deviation from the beam peak direction due to DUT re-positioning to be 1o. The simulation results shown in Figure 3 show that the error based on the positioning misalignment is very small with maximum error between the theoretical beam peak and the pattern at the misaligned beam peak direction 0.08dB. As discussed in [9], this MU element should be considered to be 0dB for TRP and spherical coverage test cases.

Figure 3: Histogram of maximum beam peak errors for a maximum misalignment error of 1o.

Proposal 4: Define the standard uncertainty of the “DUT repositioning” MU element for PC6 to be 0.08dB for EIRP and EIS metrics and 0dB for TRP.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1: Leverage the beam peak search and default and SE TRP measurement grids and MU from PC5 (alternate array assumption) for PC6, see [2][4]

Proposal 2: Defer the spherical coverage test case MUs and measurements grids given the deviation in spherical coverage side conditions for PC6.

Proposal 3: Define standard uncertainty of the “Positioning misalignment” MU element for PC6 to be 0dB

Proposal 4: Define the standard uncertainty of the “DUT repositioning” MU element for PC6 to be 0.08dB for EIRP and EIS metrics and 0dB for TRP.

# References

- R5-245204, Discussion on PC6 HST FR2 Antenna Assumptions for measurement grids, Samsung Electronics Nordic AB, 3GPP TSG-RAN5 Meeting #104, August 2024

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- TS 38.508-2, User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS)

- R5-231774, PC5 measurement grids including the alternate antenna array assumptions, Keysight Technologies, 3GPP TSG-RAN5 Meeting #98, March 2023

- R5-232627, Spurious Emissions TRP Measurement Grids using Offset Approach, Keysight Technologies, 3GPP TSG-RAN5 Meeting #99, May 2023

- R5-211189, PC1 MUs based on the revised antenna array assumptions, Keysight Technologies, 3GPP TSG-RAN5 Meeting #90-e, March 2021

- R5-191628, MU Elements: documentation of justification and interdependence, Keysight Technologies, 3GPP TSG-RAN5 Meeting #82, February 2019

- R5-190389, On Positioning Misalignment MU, Keysight Technologies, 3GPP TSG-RAN WG5 5G NR AH #4, January 2019

- R5-190387, On MU Element for DUT Repositioning, Keysight Technologies, 3GPP TSG-RAN WG5 5G NR AH #4, January 2019