3GPP TSG-RAN5 Meeting #104	R5-244518

Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4553 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 8.1.6.1.2.14 for SON_MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update SON/MDT TC 8.1.6.1.2.14 Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism to enable Uplink data transmission in WLAN to cause IDC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update SON/MDT TC 8.1.6.1.2.14 Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism - Add some test procedures and test conditions to enable checking user plane connectivity with WLAN AP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The hard number for 4.9.X highlighted in yellow should be replaced by the assigned number in TS 38.508-1 CR 3271 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

8.1.6.1.2.14	Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism

8.1.6.1.2.14.1	Test Purpose (TP)

(1)

with { UE is performing measurement logging and T330 is running }

ensure that {

when { UE detects IDC problem }

then { UE suspend measurement logging and tag MDT report with inDeviceCoexDetected-r17 flag in logMeasInfoList-r16 in VarLogMeasReport }

}

(2)

with { UE has suspended measurement logging and T330 is running }

ensure that {

when { The IDC problems detected by the UE is resolved during the last logging interval }

then { UE resume measurement logging }

}

8.1.6.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.5a.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331 clause 5.5a.3.2]

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if not suspended, perform the logging in accordance with the following:

...

2>	when performing the logging:

3> if InterFreqTargetInfo is configured and if the UE detected IDC problems on at least one of the frequencies included in InterFreqTargetInfo during the last logging interval, or

3>	if InterFreqTargetInfo is not configured and if the UE detected IDC problems during the last logging interval:

4>	if measResultServingCell in the VarLogMeasReport is not empty:

5>	include inDeviceCoexDetected;

5>	suspend measurement logging from the next logging interval;

4>	else:

5>	suspend measurement logging;

...

8.1.6.1.2.14.3	Test description

8.1.6.1.2.14.3.1	Pre-test conditions

System Simulator:

-	NR Cell 14/NRf4 (2390-2400MHz for band n40), WLAN AP 1 (Cell 27) as specified in cl. 4.13 TS 38.508-1[4];

UE:

None;

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.14.3.2	Test procedure sequence

Table 8.1.6.1.2.14.3.2-1a for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration “C1” for "T0" indicates the initial conditions. Subsequent configurations marked "T1", “T2”, “T3” and "T4" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.2.14.3.2-1. The configuration “C1” for “T0”, “T2” and “T4”, and “C2” for “T1” and “T3” are specified in TS 38.508-1 [4] Table 4.13-1.

Table 8.1.6.1.2.14.3.2-1a: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Refer to “C1” in TS 38.508-1 [4] |  |  |  |  |
| T1 | Refer to “C2” in TS 38.508-1 [4] |  |  |  |  |
| T2 | Refer to “C1” in TS 38.508-1 [4] |  |  |  |  |
| T3 | Refer to “C2” in TS 38.508-1 [4] |  |  |  |  |
| T4 | Refer to “C1” in TS 38.508-1 [4] |  |  |  |  |

Table 8.1.6.1.2.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 14. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 1A | The SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.6.1.2.14.3.2-1a. | - | - | - | - |
| 1B | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.X indicate that the UE is capable of exchanging data with WLAN AP? | - | - | 1, 2 | P |
| 1C | The SS re-adjusts the SS/PBCH EPRE level according to row "T2" in Table 8.1.6.1.2.14.3.2-1a. |  |  |  |  |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 10s to allow UE to activate logging. | - | - | - | - |
| 3A | The SS re-adjusts the SS/PBCH EPRE level according to row "T3" in Table 8.1.6.1.2.14.3.2-1a. Note: UE shall detect IDC problem in RRC_IDLE with configuration “T3”. | - | - | - | - |
| 3B | SS starts timer T1=10s Note: To allow UE to detect IDC problem |  |  |  |  |
| 4 | Void | - | - | - | - |
| 4A | The SS re-adjusts the SS/PBCH EPRE level according to row "T4" in Table 8.1.6.1.2.14.3.2-1a. Note: UE shall not detect IDC problem in RRC_IDLE with configuration “T4” | - | - | - | - |
| 5 | SS starts timer T1=10s Note: To allow UE to resume logging. | - | - | - | - |
| 6-13 | Steps 1 to 8 of the generic procedure for NR RRC_CONNECTED specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. | - | - | - | - |
| 14-20 | Void | - | - | - | - |
| 21 | The SS transmits an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 22 | Check: Does the UE transmit an UEInformationResponse message including logMeasReport with a LogMeasInfoList consisting of at least three LogMeasInfo entries, and one of the LogMeasInfo entries, except the first one and the last one, is tagged with inDeviceCoexDetected-r17 and also there exists about 10s gap between the relativeTimeStamp of the LogMeasInfoList entry with inDeviceCoexDetected-r17 flag and the relativeTimeStamp of the next LogMeasInfoList entry without inDeviceCoexDetected-r17 flag? Note 1: The relativeTimeStamp is increased between the subsequent LogMeasInfoList entries. Note 2: About 10s gap (step 4A) between the relativeTimeStamp of the LogMeasInfoList entry with inDeviceCoexDetected-r17 flag and the relativeTimeStamp of the next LogMeasInfoList entry without inDeviceCoexDetected-r17 flag is used to check that the UE suspends measurement logging when IDC problem is detected. Note 3: After 10s gap (step 5), the next LogMeasInfoList entry without inDeviceCoexDetected-r17 flag is used to check that the UE resume logging when IDC problem is resolved. | --> | NR RRC: UEInformationResponse | 1, 2 | P |

8.1.6.1.2.14.3.3		Specific message contents

Table 8.1.6.1.2.14.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.14.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.14.3.3-2: UEInformationRequest (steps 21, Table 8.1.6.1.2.14.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.14.3.3-3: UEInformationResponse (step 22, Table 8.1.6.1.2.14.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 3 entries where at least 3 entry complies to the entries with index ‘x-1’, ‘x’, ‘x+1’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x-1] SEQUENCE { |  | entry x-1 |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x-1 |  |
| servCellIdentity-r16 | Same as NR Cell 14 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | Not present |  |  |
| } |  |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x, and relativeTimeStamp-r16_x-1 < relativeTimeStamp-r16_x, relativeTimeStamp-r16_x + 10s ≤ relativeTimeStamp-r16_x+1 |  |
| servCellIdentity-r16 | Same as NR Cell 14 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | true |  |  |
| } |  |  |  |
| LogMeasInfo-r16[x+1] SEQUENCE { |  | entry x+1 |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x+1 |  |
| servCellIdentity-r16 | Same as NR Cell 14 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>