3GPP TSG-RAN5 Meeting #104	R5-244565

# Maastricht, Netherlands, 19th– 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3306 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Test Case: TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TRS based SCell Activation of SSB-less SCell Test Case 6.5.3.15 has been missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Minimum conformance requirements, test case, and cell configuration mapping table have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work item cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.0.2(new), 6.5.3.15 (new), E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.5.3	SCell activation and deactivation delay

#### 6.5.3.0	Minimum conformance requirements

-------- Start of 1st change --------

##### 6.5.3.0.2	Minimum conformance requirements for TRS based SCell activation

For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled,

-	The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and

-	The [EPRE] difference at the UE is smaller than or equal to [12] dB, where [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and

-	The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

where the reference serving cell can be indicated by higher layer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Where Tactivation_time is

-	Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17

-	Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n (where n mode 20=1), defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in TS 38.133 [6] clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] clause 8.3, where Ninterruption is the interruption length given in TS 38.133 [6] clause 8.2.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The interruption length X2 on any activated serving cell are according to Table 6.5.3.0.2-1 and 6.5.3.0.2-2.

Table 6.5.3.0.2-1: Interruption length X2 for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |

Table 6.5.3.0.2-2 Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms; - the longest SMTC duration among all active servingNOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.3.2.

-------- End of 1st change --------

----- Unchanged Clauses Skipped ------

-------- Start of 2nd change --------

#### 6.5.3.12 to 6.5.3.14

#### 6.5.3.15	NR SA FR1 TRS based SCell Activation of SSB-less SCell in inter-band CA in non-DRX

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test procedure and message contents are FFS

- Test applicability statement in TS 38.522 is missing

- Some parameters are defined in brackets

- Test tolerance analysis and measurement uncertainty are missing

6.5.3.15.1 	Test purpose

The purpose of this test is to verify that the SSB-less SCell activation delay is within the requirements stated in TS 38.133 [6] clause 8.3.2, when the to be activated SCell in FR1 is provided with periodic CSI-RS for tracking instead of SSB. SCell does not provide neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration.

6.5.3.15.2	Test applicability

This test applies to all types of NR UE release 18 onwards that support scellWithoutSSB, and scellWithoutSSB-InterBandCA-r18.

6.5.3.15.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clauses 8.3.2 and 8.2.2.2.

6.5.3.15.4	Test description

6.5.3.15.4.1	Initial conditions

The supported test configurations are shown in Table 6.5.3.15.4.1-1 and 6.5.3.15.4.1-2 below.

Table 6.5.3.15.4.1-1: FR1 inter-band SSB-less SCell activation based on TRS for NR PCell in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table 6.5.3.15.4.1-2: FR1 inter-band SSB-less SCell activation based on TRS for NR SCell in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.3.15.4.1-3.

Table 6.5.3.15.4.1-3: Initial conditions for TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.7A.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2a | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. - Without Faders |  |  |

1. The general test parameter settings are set up according to Table 6.5.3.15.4.1-4.

2. Message contents are defined in clause 6.5.3.15.4.3.

3. The test scenario comprises of two NR carriers, each with one cell. Both cells have constant signal levels throughout the test and are configured according to Annex E.4.

Table 6.5.3.15.4.1-4: General test parameters for TRS based SCell activation of SSB-less SCell in FR1 inter-band CA in non-DRX for 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test, where NR radio channels 1 and 2 are in different NR bands. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell2 timing offset to cell1 | s | Length of CP of Cell 2 |  |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

6.5.3.15.4.2	Test procedure

FFS

6.5.3.15.4.3 	Message contents

FFS

6.5.3.15.5	Test requirement

Table 6.5.3.15.5-1 and Table 6.5.3.15.5-2 define the primary level settings including test tolerances for TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX.

Table 6.5.3.15.5-1: PCell test configuration parameters for TRS based SCell activation of SSB-less SCell in FR1 inter-band CA in non-DRX for 160 ms measurement cycle

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2,3 |  | TDD |  |
| TDD configuration | Config 1 |  | Not applicable |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |
|  | Config 3 |  | Note 7 |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |
|  | Config 3 |  | 106 Note 6 |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |
| TCI state |  |  | TCI.State.0 |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| OCNG Patterns | Config 1,2 |  | OP.1Note 5 |  |
|  | Config 3, |  | OP.1 Note 6 |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| Periodic CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |
| SMTC configuration |  |  | SMTC.1 |  |
| reportConfigType |  |  | periodic |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |
|  | Config 3 |  | 10 |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |
|  | Config 3 |  | 5 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |
|  | Config 3 |  | -101 |  |
|  |  | dB | 10+TT |  |
|  |  | dB | 10+TT |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -94+TT |  |
|  | Config 3 |  | -91+TT |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -91+TT |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -65.63+TT |  |
|  | Config 3 | dBm/ 38.16MHz | -59.54+TT |  |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: Io, SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |

Table 6.5.3.15.5-2: SCell test configuration parameters for TRS based SCell activation of SSB-less SCell in FR1 inter-band CA in non-DRX for 160 ms measurement cycle

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |
|  | ConfigSCell 2,3 |  | TDD |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |
| referenceCell | ConfigSCell 1,2,3 |  | Cell 1 |  |
| BWchannel | ConfigSCell 1,2 | MHz | Note 7 |  |
|  | ConfigSCell 3 |  | Note 7 |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |
| TCI state |  |  | TCI.State.0 Note 9 |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | - |  |
|  | ConfigSCell 2 |  | - |  |
|  | ConfigSCell 3 |  | - |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | - |  |
|  | ConfigSCell 2 |  | - |  |
|  | ConfigSCell 3 |  | - |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | - |  |
|  | ConfigSCell 2 |  | - |  |
|  | ConfigSCell 3 |  | - |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1Note 5 |  |
|  | ConfigSCell 3 |  | OP.1 Note 6 |  |
| SSB Configuration | ConfigSCell 1,2,3 |  | Not provided |  |
| Periodic CSI-RS configuration for CSI reporting (Note 8) | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |
| SMTC configuration |  |  | Not provided |  |
| reportConfigType |  |  | periodic |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | - |  |
|  | ConfigSCell 3 |  | - |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | - |  |
|  | ConfigSCell 3 |  | - |  |
| EPRE ratio of PSS to SSS |  | dB | N/A |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |
|  | Config 3 |  | -101 |  |
|  |  | dB | 19 + [ΔEPRENote 10] |  |
|  |  | dB | 19 + [ΔEPRENote 10] |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | - |  |
|  | Config 3 |  | - |  |
| SCH_RP Note 3 |  | dBm/15 kHz | N/A |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | Note 3 |  |
|  | Config 3 | dBm/ 38.16MHz | Note 3 |  |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: Io, SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. Note 9: The SSB in referenceSignal in the TCI state is configured as the SSB in Cell 1. |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (n+1+THARQ+3msNR slot length). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption. During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = Tfirst_TRS + TTRS + 5 ms, as defined in 6.5.3.0.2.

During T2 interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] clause 8.3.2.

The interruption on any activated serving cell shall not be more than the values specified as defined in Table 6.5.3.0.2-1 and Table 6.5.3.0.2-2.

All the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in TS 38.133 [6] clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

----- End of 2nd change ------

----- Skipped Unchanged Clauses ------

-------- Start of 3rd change --------

Annex E (normative):
 Cell configuration mapping

# E.0	General

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.8 | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.3 | NR SA FR1-FR1 Handover with unknown Target Cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | NR Cell 1 |  |  |  |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | NR Cell 1 |  |  |  |  |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | NR Cell 1 |  |  |  |  |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | NR Cell 1 |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.9 | SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | NR Cell 1 |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.2.2 | SA FR1 interruptions at NR SRS carrier based switching | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.4 | NR SA FR1 direct SCell activation at SCell addition of known SCell | NR Cell 1 | NR Cell 6 |  |  | intra-band |
| 6.5.3.5 | NR SA FR1 direct SCell activation at handover with known SCell | NR Cell 1 | NR Cell 6 | NR Cell 2 |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.10 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.11 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.15 | NR SA FR1 TRS based SCell Activation of SSB-less SCell in inter-band CA in non-DRX | NR Cell 1 | NR Cell 10 |  |  | Inter-band |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 33 |  |  |  |
| … |  |  |  |  |  |  |

------- Start of 3rd change --------