3GPP TSG RAN WG5 Meeting #104	R5-244674

# Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1483 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ServingSatelliteInfo-v1820 to SystemInformationBlockType31 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT, MTK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ServingSatelliteInfo-v1820 in SystemInformationBlockType31 is needed for IoT NTN enh signaling test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ServingSatelliteInfo-v1820 was added to SystemInformationBlockType31. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IoT NTN enh signaling test cases cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

8.1.4.3.3	NB-IoT Common contents of system information blocks

-	SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

Table 8.1.4.3.3-1: SystemInformationBlockType2-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 SEQUENCE {} | RadioResourceConfigCommonSIB-NB-DEFAULT | See subclause 8.1.6 |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t300-r13 | ms10000 |  |  |
| t301-r13 | ms10000 |  |  |
| t310-r13 | ms1000 |  |  |
| n310-r13 | n2 |  |  |
| t311-r13 | ms1000 |  |  |
| n311-r13 | n1 |  |  |
| } |  |  |  |
| freqInfo-r13 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r13 | Not present | Default UL EARFCN applies | Standalone |
|  | See subclause 8.1.3.1 |  | Inband_Same, Inband_Different, Guardband |
| additionalSpectrumEmission-r13 | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4F -1. |  |
|  | 4 (NS_04) |  | NS_04 |
| } |  |  |  |
| timeAlignmentTimerCommon-r13 | infinity |  |  |
| multiBandInfoList-r13 | Not present |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  | FDD |
| freqInfo-v1530 SEQUENCE { |  |  | TDD |
| tdd-UL-DL-AlignmentOffset-r15 | khz0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Inband_Same | In-band with same PCI test environment |
| Inband_Different | In-band with different PCI test environment |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |
| NS_04 | When using the NS_04 frequency table variants in subclause 8.1.3.1 |

<< Unchanged contents omitted >>

-	SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information for the serving cell. SystemInformationBlockType31-NB is only signalled in an NTN cell.

Table 8.1.4.3.3-10: SystemInformationBlockType31-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors | See cl. 6.3.5.1 |  | GSO |
|  | See cl. 6.3.5.2 |  | NGSO |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 8243100 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDrift-r17 | 0 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 550 |  | GSO |
|  | See cl. 6.3.5.2 |  | NGSO |
| k-Mac-r17 | Not present |  | GSO |
|  | See cl. 6.3.5.2 |  | NGSO |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| servingSatelliteInfo-v1820 SEQUENCE { |  |  | NTN enh. |
| referenceLocation-r18 CHOICE { |  |  |  |
| fixedReferenceLocation-r18 | '2273BA566355'H |  | GSO |
| movingReferenceLocation-r18 | [FFS] |  | NGSO |
| } |  |  |  |
| distanceThresh-r18 | 500 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |
| NTN enh. | For NB-IoT NTN enhancement test case |

< End of changes >>