3GPP TSG-RAN5 Meeting #104	R5-244723

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4583 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MAC test case 7.1.1.6.1-Semi-persistent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In step16/Step17, SS schedules two overlapped PDSCH: one PDSCH with PDCCH CS-RNTI and another PDSCH without PDCCH, and the PDCCH does not end 14 symbols before PDSCH. According to 38.214 clause 5.1,step16 is not expected by UE. In addition, SPS PDSCH transmitted in slot"P+nX" has already been activated in step 6, SS does not need to reactivate it in step16. Therefore, Step16 is useless. [38.214 5.1] The UE is not expected to decode a PDSCH in a serving cell scheduled by a PDCCH with C-RNTI, CS-RNTI, MCS-C-RNTI, G-RNTI, G-CS-RNTI or MCCH-RNTI and one or multiple PDSCH(s) required to be received according to this Clause in the same serving cell without a corresponding PDCCH transmission if the PDSCHs partially or fully overlap in time except if the PDCCH scheduling the PDSCH ends at least 14∙2max⁡(0,μ-3) symbols before the earliest starting symbol of the PDSCH(s) without the corresponding PDCCH transmission, where and the symbol duration are based on the smallest numerology between the scheduling PDCCH and the PDSCH, in which case the UE shall decode the PDSCH scheduled by the PDCCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Void step16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.6	Semi-Persistent Scheduling

7.1.1.6.1	Correct handling of DL assignment / Semi-persistent case

7.1.1.6.1.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and sps-Configuration in DL is enabled }

ensure that {

when { UE receives a DL assignment addressed to its stored CS-RNTI in slot y and with NDI set as 0 and PDCCH content indicates activation }

then { UE starts receiving DL MAC PDU in slots y+n*[semiPersistSchedIntervalDL] where ‘n’ is positive integer starting at zero }

}

(2)

with { UE in RRC_Connected state with DRB established and stored DL SPS assignment to receive MAC PDU in slot y+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives a DL assignment addressed to its CS-RNTI in slot p and with NDI set as 0, where p!= y+n*[semiPersistSchedIntervalDL] }

then { UE starts receiving DL MAC PDU in slots p+n*[semiPersistSchedIntervalDL] and stops receiving DL MAC PDU at slots y+n*[semiPersistSchedIntervalDL]where ‘n’ is positive integer starting at zero }

}

(3)

with { UE in RRC_Connected state with DRB established and stored DL SPS assignment to receive MAC PDU at slot p+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives a DL assignment [for retransmission] addressed to its CS-RNTI in Slot z and with NDI set as 1, where z!= p+n*[semiPersistSchedIntervalDL] }

then { UE receives MAC PDU as per the retransmission grant for CS-RNTI }

}

(4)

with { UE in  RRC_Connected state with DRB established and stored DL SPS assignment to receive MAC PDU at slot y+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives a DL assignment addressed to its C-RNTI in Slot p, such that p= y+n*[semiPersistSchedIntervalDL] }

then { UE receives MAC PDU as per assignment addressed to its C-RNTI }

}

(5)

with { UE in RRC_Connected state with DRB established and stored DL SPS grant to receive MAC PDU at slot z+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives a RRC Message including sps-Configuration with sps-ConfigurationDL set as ‘disable’ and hence resulting in DL SPS grant deactivation }

then { UE deletes the stored sps-Configuration DL parameters and stops receiving DL MAC PDU’s as per stored SPS assignment in slot z+n*[semiPersistSchedIntervalDL] }

}

(6)

with { UE in RRC_Connected state with DRB established and sps-Configuration in DL is enabled }

ensure that {

when { UE receives a DL assignment addressed to its stored CS-RNTI in slot p and with NDI set as 0 and PDCCH content indicates deactivation }

then {UE stops receiving DL MAC PDU’s as per stored SPS assignment }

}

7.1.1.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 38.321, clause 5.3.1, 5.8.1 TS 38.300, clause 10.2 and TS 38.213 clause 102. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.3.1]

Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity's C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or a configured downlink assignment:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this Serving Cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted,is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

3>	else if PDCCH content indicates SPS activation:

4>	store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment;

4>	initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in subclause 5.8.1;

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell:

2>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

2>	consider the NDI bit for the corresponding HARQ process to have been toggled;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame ×periodicity ))] modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

[TS 38.321, clause 5.8.1]

Semi-Persistent Scheduling (SPS) is configured by RRC per Serving Cell and per BWP. Activation and deactivation of the DL SPS are independent among the Serving Cells.

For the DL SPS, a DL assignment is provided by PDCCH, and stored or cleared based on L1 signalling indicating SPS activation or deactivation.

RRC configures the following parameters when SPS is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	nrofHARQ-Processes: the number of configured HARQ processes for SPS;

-	periodicity: periodicity of configured downlink assignment for SPS.

When SPS is released by upper layers, all the corresponding configurations shall be released.

After a downlink assignment is configured for SPS, the MAC entity shall consider sequentially that the Nth downlink assignment occurs in the slot for which:

(numberOfSlotsPerFrame × SFN + slot number in the frame) =
[(numberOfSlotsPerFrame × SFNstart time + slotstart time) + N × periodicity × numberOfSlotsPerFrame / 10] modulo (1024 × numberOfSlotsPerFrame)

where SFNstart time and slotstart time are the SFN and slot, respectively, of the first transmission of PDSCH where the configured downlink assignment was (re-)initialised.

[TS 38.300, clause 10.2]

In the downlink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible assignments when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

The gNB may pre-empt an ongoing PDSCH transmission to one UE with a latency-critical transmission to another UE. The gNB can configure UEs to monitor interrupted transmission indications using INT-RNTI on a PDCCH. If a UE receives the interrupted transmission indication, the UE may assume that no useful information to that UE was carried by the resource elements included in the indication, even if some of those resource elements were already scheduled to this UE.

In addition, with Semi-Persistent Scheduling (SPS), the gNB can allocate downlink resources for the initial HARQ transmissions to UEs: RRC defines the periodicity of the configured downlink assignments while PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the downlink assignment can be implicitly reused according to the periodicity defined by RRC, until deactivated.

NOTE:	when required, retransmissions are explicitly scheduled on PDCCH(s).

The dynamically allocated downlink reception overrides the configured downlink assignment in the same serving cell, if they overlap in time. Otherwise a downlink reception according to the configured downlink assignment is assumed, if activated.

When CA is configured, at most one configured downlink assignment can be signalled per serving cell. When BA is configured, at most one configured downlink assignment can be signalled per BWP. On each serving cell, there can be only one configured downlink assignment active at a time, and multiple configured downlink assignment can be simultaneously active on different serving cells only. Activation and deactivation of configured downlink assignments are independent among the serving cells.

[TS 38.213, clause 10.2]

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

-	the new data indicator field for the enabled transport block is set to '0'.

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

Table 10.2-1: Special fields for DL SPS and UL grant Type 2 scheduling activation PDCCH validation

|  | DCI format 0_0/0_1 | DCI format 1_0 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' | For the enabled transport block: set to '00' |

Table 10.2-2: Special fields for DL SPS and UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0 | DCI format 1_0 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '1's | set to all '1's |

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after {} symbols from the last symbol of a PDCCH providing the SPS PDSCH release. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release,  for ,  for , and  for , otherwise,  for ,  for ,  for , and  for , wherein  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

7.1.1.6.1.3	Test description

7.1.1.6.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.6.1.3.2	Test procedure sequence

Table 7.1.1.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | SS transmits NR RRCReconfiguration message to configure DL SPS. (Note 4) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 0B | The UE transmits NR RRCReconfigurationComplete message. (Note 5) | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 1 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘Y’, NDI=0. | <-- | (DL SPS Grant) | - | - |
| 2 | The SS transmits in Slot ‘Y’, a DL MAC PDU containing a RLC PDU (DL-SQN=0) on UM DRB. | <-- | MAC PDU | - | - |
| 3 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 4 | The SS transmits in Slot ‘Y+X’, a DL MAC PDU containing a RLC PDU (DL-SQN=1) on DRB. (Note 1) | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 6 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘P’, NDI=0; (Where Y+X<P<Y+2X) | <-- | (DL SPS Grant) | - | - |
| 7 | The SS transmits in Slot ‘P’, a DL MAC PDU containing a RLC PDU (DL-SQN=2) on UM DRB. | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2 | P |
| 9 | The SS transmits in Slot ‘Y+2X’, a DL MAC PDU containing a RLC PDU (DL-SQN=3) on UM DRB. | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK/NACK | 2 | F |
| - | EXCEPTION: Steps 11a1-11b2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on UE support of K0 greater than 0 for PDSCH mapping type A. | - | - | - | - |
| 11a1 | IF pc_dl_SchedulingOffset_PDSCH_TypeA THEN the SS transmits a DL assignment using UE’s C-RNTI in Slot ‘P+X-4’, NDI=0, where TDRA indicates the 2nd entry in pdsch-TimeDomainAllocationList (K0=4), and K1=8. | <-- | (DL Grant) | - | - |
| 11a2 | The SS transmits in Slot ‘P+X’, a DL MAC PDU containing a RLC PDU (DL-SQN=3) on UM DRB. (Note 2) | <-- | MAC PDU | - | - |
| 11a3 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 4 | P |
| 11b1 | IF NOT pc_dl_SchedulingOffset_PDSCH_TypeA THEN the SS transmits in Slot ‘P+X’, a DL MAC PDU containing a RLC PDU (DL-SQN=3) on UM DRB. | <-- | MAC PDU | - | - |
| 11b2 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 12-13 | Void | - | - | - | - |
| 14 | The SS transmits in Slot ‘P+2X’, a DL MAC PDU containing a RLC PDU (DL-SQN=4) on UM DRB. | <-- | MAC PDU | - | - |
| 15 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 16 | Void | - | - | - | - |
| 17 | The SS transmits in Slot ‘P+3X’, a DL MAC PDU containing 1 RLC PDU’s (DL-SQN=5) on UM DRB; CRC is calculated in such a way will result in CRC error in UE. | <-- | MAC PDU | - | - |
| 18 | Check: Does the UE transmit a HARQ NACK? | --> | HARQ NACK | - | - |
| - | EXCEPTION: Step 19 and 20 shall be repeated until HARQ retransmission count = 3 is reached for MAC PDU at step 17.(Note 3) | - | - | - | - |
| 19 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘Z’, NDI=1; Where (P+3X < Z <P+4X); The DL HARQ process is same as in step 18. | <-- | (DL SPS Grant) | - | - |
| 20 | The SS re-transmits in Slot ‘Z’, a DL MAC PDU containing a RLC PDU (DL-SQN=5) on UM DRB. | <-- | MAC PDU | - | - |
| - | EXCEPTION: Up to 3 HARQ NACK from the UE should be allowed at step 21(Note 3). | - | - | - | - |
| 21 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 3 | P |
| 22 | The SS Transmits a PDCCH [for DL SPS deactivation] using UE’s CS-RNTI in slot ‘Q’, NDI=0; Where (P+3X< Q <P+4X). | <-- | PDCCH [for DL SPS explicit release] | - | - |
| 23 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 6 | P |
| 24 | The SS transmits in Slot ‘P+5X’, a DL MAC PDU containing 1 RLC PDU’s (DL-SQN=6)on UM DRB; | <-- | MAC PDU | - | - |
| 25 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK/NACK | 6 | F |
| 26 | The SS Transmits a DL assignment using UE’s CS-RNTI in SF-Num ‘P+6X’, NDI=0 | <-- | (DL SPS Grant) | - | - |
| 27 | The SS transmits in SF-Num ‘P+6X’, a DL MAC PDU containing a RLC PDU (DL-SQN=6)on UM DRB | <-- | MAC PDU | - | - |
| 28 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 29 | SS transmits NR RRCReconfiguration to disable SPS-ConfigurationDL.(Note 4) | <-- | RRCReconfiguration | - | - |
| 30 | The UE transmits NR RRCReconfigurationComplete.(Note5) | --> | RRCReconfigurationComplete | - | - |
| 31 | The SS transmits in Slot ‘P+5X’, a DL MAC PDU containing 1 RLC PDU’s (DL-SQN=7) on UM DRB; | <-- | MAC PDU | - | - |
| 32 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK/NACK | 5 | F |
| Note 1: X is equal to semiPersistSchedIntervalDL in this document. Note 2: The DL assignment for C-RNTI and hence the size of MAC PDU is different in size than stored CS-RNTI DL assignment in step 6. This assures UE is receiving DSCH data as per DL assignment for C-RNTI and not as per stored grant for CS-RNTI. Note 3: The value 4 for the maximum number of HARQ retransmissions has been chosen based on an assumption that, given the radio conditions used in this test case, a UE soft combiner implementation should have sufficient retransmissions to be able to successfully decode the data in its soft buffer. Note 4: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 5: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 6: As per TS 38.508-1[4], the default value for PDSCH slot offset (K0) is 0, hence the DL MAC PDU’s associated with DL SPS grant in Slot X are sent in same slot X. |  |  |  |  |  |

7.1.1.6.1.3.3	Specific message contents

Table 7.1.1.6.1.3.3-1: RRCReconfiguration (Step 0A, Table 7.1.1.6.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE{ |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-2: CellGroupConfig (Table 7.1.1.6.1.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| servCellIndex |  | 1 |  |  |
|  |  | Not present |  | NR |
| reconfigurationWithSync SEQUENCE { |  |  |  | EN-DC AND pc_dl_SchedulingOffset_PDSCH_TypeA |
| spCellConfigCommon |  | ServingCellConfigCommon |  |  |
| } |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |  |
| sps-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| periodicity |  | ms80 |  |  |
| nrofHARQ-Processes |  | 8 |  |  |
| n1PUCCH-AN SEQUENCE { |  |  |  |  |
| pucch-ResourceId |  | 6 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |  |
| cs-RNTI CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| RNTI-Value |  | ‘FFE0’H |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.6.1.3.3-2A: ServingCellConfigCommon (Table 7.1.1.6.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommon |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-2B: DownlinkConfigCommon (Table 7.1.1.6.1.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-52: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 7.1.1.6.1.3.3-5C |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-3: RRCReconfiguration (step 29, Table 7.1.1.6.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-4: CellGroupConfig (Table 7.1.1.6.1.3.3-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| servCellIndex |  | 1 |  |  |
|  |  | Not present |  | NR |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |  |
| sps-Config CHOICE { |  |  |  |  |
| release |  | Null |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.6.1.3.3-5: SIB1 (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| } |  |  |  |  |

Table 7.1.1.6.1.3.3-5A: ServingCellConfigCommonSIB (Table 7.1.1.6.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-5B: DownlinkConfigCommonSIB (Table 7.1.1.6.1.3.3-5A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-5C: BWP-DownlinkCommon (Tables 7.1.1.6.1.3.3-2B and 7.1.1.6.1.3.3-5B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-5D: PDSCH-ConfigCommon (Table 7.1.1.6.1.3.3-5C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-101 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-5E: PDSCH-TimeDomainResourceAllocationList (Table 7.1.1.6.1.3.3-5D)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | 0 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | 4 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>