3GPP TSG-RAN5 Meeting #104	R5-244802

# Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Title:							Testability analysis on Transmit OFF power for FR2b

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the beginning of Release 15, testability of Transmit OFF power for FR2b (n260) was discussed, and relaxation value was defined with [ ]. However, further discussion on whether to test or not, or whether to remove [ ], was not progressed since there were no industry demands. Now, this paper restarts the discussion to finish the pending work on Transmit OFF power for FR2b. Unless otherwise commented, observations and proposals in this paper are for FR2b, PC3 and PC1, CBW  400 MHz, NTC, IFF, max device size  40 cm.

# 2.	Discussion

The relaxation value for FR2b in Transmit OFF power is defined as [33.1] dB for CBW 400 MHz in the current TS 38.521-2 [1]. Historically, this value was analyzed in [2] along with relaxation values for other test cases and/or other frequency ranges. However, it is an early analysis during the beginning of Release 15, and many of the relaxation values proposed in it have been revised later. For example, the relaxation value for FR2a in Transmit OFF power was analyzed as 23.9 dB for CBW 400 MHz in [2], but the current agreed value is 30.4 dB in [1]. Therefore, the relaxation value for FR2b should also be reanalyzed.

Observation 1: The current relaxation value for FR2b in Transmit OFF power of [33.1] dB for CBW 400 MHz should be reanalyzed based on the current testability assumption.

For testing Transmit OFF power for FR2b, we assume -5.5 dBm/400 MHz for TE noise floor as already used in Minimum output power [3]. Considering the minimum requirement of -35 dBm/CBW, 35.5 dB is required for relaxation in CBW = 400 MHz to test with influence of noise of 1.0 dB. Finally, MTSU is calculated as 5.67 dB by using influence of noise = 1.0 dB.

Proposal 1: For FR2b Transmit OFF power, adopt 26.5 dB for CBW = 50 MHz, 29.5 dB for CBW = 100 MHz, 32.5 dB for CBW = 200 MHz, 35.5 dB for CBW = 400 MHz to relaxation value.

Proposal 2: Adopt 5.67 dB to MTSU for FR2b Transmit OFF power.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals for FR2b, PC3 and PC1, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Proposal 1: For FR2b Transmit OFF power, adopt 26.5 dB for CBW = 50 MHz, 29.5 dB for CBW = 100 MHz, 32.5 dB for CBW = 200 MHz, 35.5 dB for CBW = 400 MHz to relaxation value.

Proposal 2: Adopt 5.67 dB to MTSU for FR2b Transmit OFF power.

# 4.	References

[1] TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V18.3.0 (2024-06)

[2] R5-188229, “SNR estimation for priority 1 and 2 FR2 Tx test cases”, Anritsu, RAN5#81

[3] TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, V18.3.0 (2024-06)