3GPP TSG-RAN5 Meeting #104	R5-244804

# Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Title:							MU discussion on FR2d

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

For FR2d (n262), MU analysis except QoQZ is missing in TR 38.903 [1], and test cases are incomplete in TS 38.521-2 [2]. This discussion paper starts MU discussion on FR2d. Unless otherwise commented, observations and proposals in this paper are for FR2d, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

# 2.	Discussion

Quality of Quiet Zone

The uncertainty values for quality of quiet zone for FR2d have been discussed [3][4] and defined in Table B.2.2.3-1 [1] as below.

- QZ size ≤ 30 cm: QoQZ (stage 2) = 0.7 dB

- QZ size ≤ 30 cm: QoQZ (stage 1) = 0.5 dB

- QZ size = 40 cm: QoQZ (stage 2) = 0.8 dB

- QZ size = 40 cm: QoQZ (stage 1) = 0.5 dB

- Note: For uncertainty assessment and MTSU calculation for FR2d, QoQZ uncertainty value for QZ size of 40cm is used also for QZ size ≤ 30cm.

Uncertainty of the RF power measurement equipment

According to our analysis on the RF power measurement equipment, the uncertainty value for FR2d is 0.1 dB larger than that for FR2c and is 3.70 dB.

Other MU elements

We assume that the same uncertainty values as FR2c are applicable to FR2d for other MU elements except for those which require test case-specific analysis, such as influence of noise, modulated interferer uncertainty, and values for supurious testing.

Observation 1: In MTSU calculation for FR2d, 0.8 dB and 0.5 dB are used for QoQZ of stage 2 and stage 1, respectively.

Proposal 1: Adopt 3.70 dB to uncertainty of the RF power measurement equipment for FR2d.

Proposal 2: Adopt the same uncertainty values as FR2c to FR2d for MU elements except for those requirering test case-specific analysis.

Observation 2: Further testability analysis is required for influence of noise and other test case-specific MU elements.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals for FR2d, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Proposal 1: Adopt 3.70 dB to uncertainty of the RF power measurement equipment for FR2d.

Proposal 2: Adopt the same uncertainty values as FR2c to FR2d for MU elements except for those requirering test case-specific analysis.

# 4.	References

[1] TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, V18.3.0 (2024-06)

[2] TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V18.3.0 (2024-06)

[3] R5-240605, “On 30cm QoQZ MU Topics”, Keysight Technologies, RAN5#102

[4] R5-242729, “On the QoQZ MU for n262”, Rohde & Schwarz, RAN5#103